3GPP TSG RAN WG1 #105-e	R1-2105610

e-Meeting, May 10th - 27th, 2021

Title:	About the LS on Status Update on XR Traffic Model

Source: 	ZTE, Sanechips

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

SA4 sent to RAN1 an LS [1] together with the XR traffic PD encompassing the latest progress on XR Traffic Model, where  following aspects are emphasized

-  Traces and test channels are used to derive the quality evaluation from SA perspective given statistical model could not reflect time correlation or priority level between packets

-  latest development on VR1 traffic models-

-  Considerations on the additional information such as priority levels in the trace models

These actions are mentioned therein.

ACTION:

To RAN1

- To take into account the information in this document.

- To inform SA4 in case support would be needed for defining and/or verifying statistical models based on P-Traces.

- To kindly support SA4 in the definition of appropriate and representative test channels and provide feedback and comments on the initial definition in clause 7.6 of the PD.

- To inform SA4 in case support would be needed for providing simulation and evaluation results for different test channels in order to identify the benefit of specific radio/QoS settings.

This contribution shares our views on the LS from SA4.

Discussion

During RAN1#104b-e,  the traffic for single stream is finalized with several options featuring different (std,max, min)/mean ratio. What remains to be settled is the traffic for multiple streams. The statistics from the SA contribution [2] is synthesized in Table 1.

Table 1 Traffic Pattern for Different I-P Frame Based Multi-Stream Modelling

| Application | AR/CG/VR |  |
| --- | --- | --- |
| Two stream data | Stream#1: I-stream Stream#2: P-stream |  |
|  | Option 1: Sliced-based | Option 2: Frame-based (GoP) |
| Structure | Number of stream #1: 1 Number of stream #2: N - 1 Recommand: N = 8. | Number of stream #1: 1 Number of stream #2: K - 1 Recommand: K = 8. |
| Frame per second | Stream #1 : 60FPS Stream #2: 60FPS | Stream #1 + Stream #2 = 60FPS |
| Average packet size ratio | Stream#1 : Stream #2 = 2:1 | Stream#1 : Stream #2 = 2:1 |

| - Frame based Traffic model | - Slice based Traffic model |
| --- | --- |

Figure. 1 Illustrations of Different Traffic Models under I/P Frame Based Multiple Stream

Given the interest in RAN1 for multiple streams reflecting the I  P frame characteristics, considerations on QoS requirements such as PDB and reliability aspects are mentioned during the previous RAN1 meetings. In the meantime, according to the  illustrations of  the different models of I P frame setting, the GoP based model shall experience a different QoS requirement in terms of both latency and reliability considering:

- I frame transmission is supposed to be satisfied in a more stringent manner due to the content it conveys

- I frame decoding is supposed to more consuming in terms of latency and thus the PDB requirement is more stringent..

While for slice based model, the PDB requirement is supposed to be aligned given I P fractions are aggregated and the arrival is supposed to be simultaneous. The above considerations lead to the 	QoS settings in Table 2.

Table 2 QoS Settings for Different I-P Frame Based Multi-Stream Modelling

| Application | AR/CG/VR |  |
| --- | --- | --- |
| Two stream data | Stream#1: I-stream Stream#2: P-stream |  |
|  | Option 1: Sliced-based | Option 2: Frame-based (GoP) |
| (PSR,PDB) | AR/VR Stream#1:(99%, 20ms) Stream#2:(90%, 20ms) CG: Stream#1: (99%, 30ms) Stream#2: (90%, 30ms) | Option1: Stream#1:(99%, 10ms) Stream#2:(90%, 10ms) Option2: Stream#1:(99%, 15ms) Stream#2:(99%, 10ms) |

With the above analysis, we propose,

In addition to the reliability and latency requirement from RAN1 as in Table 3 , SA4 provides input, if any, regarding the QoS Settings for the I-P Frame Based Multi-Stream Modelling.

Table 3 QoS Settings for Different I-P Frame Based Multi-Stream Modelling

| Application | AR/CG/VR |  |
| --- | --- | --- |
| Two stream data | Stream#1: I-stream Stream#2: P-stream |  |
|  | Option 1: Sliced-based | Option 2: Frame-based (GoP) |
| (PSR,PDB) | AR/VR Stream#1:(99%, 20ms) Stream#2:(90%, 20ms) CG: Stream#1: (99%, 30ms) Stream#2: (90%, 30ms) | Option1: Stream#1:(99%, 10ms) Stream#2:(90%, 10ms) Option2: Stream#1:(99%, 15ms) Stream#2:(99%, 10ms) |

Frame level modelling is adopted in RAN1 simulation where 1% reliability requirement is set as baseline. Thus RAN1 could report, under different radio settings the frame loss rate, which could not meet the requirement of SA due to the fact that the packet level simulation is done to generate subsequent slice loss rate and pSNR etc. Though SA evaluation has the benefit of modelling the timing correlation of the packets as well as the priority levels in terms of slice/frame re-construction, the outcome could be flawed with a frame level loss rate information from RAN1. Thus in case SA evaluations are performed based on RAN1 input, PER should be derived based on a RAN1 agreed FER to PER mapping model.

In case SA evaluations are performed based on RAN1 input, PER should be derived based on a RAN1 agreed FER to PER mapping model.

The latest development in SA on VR1 provides input on data rate and the streaming model captured FoV using per-tiled segments. Given the data rate is covered by the agreed data rate requirement, the distinctive feature is to reflect the FoV part of the streaming model. This could be done as well under the multiple stream discussion of FoV/non-FoV modelling. SA4 is expected to provide corresponding QoS setting for the simulation in terms of PDB/reliability requirement.

SA4 is expected to provide the PER and PDB for multi stream modelling including FoV/non-FoV

During previous RAN1 meetings, the 3% and 4% std/mean ratio in the agreement on the packet size distribution parameters well captured the SA raw data in the sense that both PDF/CDF curve fitting results of the data sample and the model aligns reasonably well [3]. In case some further input is needed e.g. to reflect the potential VR1 statistics or the setting of slice/GoP based traffic characteristics, the methodology therein could be re-used to obtain the characteristic parameters reflecting the curve.

The methodology in [3] could be re-used to evaluate raw data and the traffic characteristics for additional SA input.

Some sentence on the appendix.

Conclusion

Observation 1:	The methodology in [3] could be re-used to evaluate raw data and the traffic characteristics for additional SA input.

Proposal 1: In addition to the reliability and latency requirement from RAN1 as in Table 3 , SA4 provides input, if any, regarding the QoS Settings for the I-P Frame Based Multi-Stream Modelling.

Table 3 QoS Settings for Different I-P Frame Based Multi-Stream Modelling

| Application | AR/CG/VR |  |
| --- | --- | --- |
| Two stream data | Stream#1: I-stream Stream#2: P-stream |  |
|  | Option 1: Sliced-based | Option 2: Frame-based (GoP) |
| (PSR,PDB) | AR/VR Stream#1:(99%, 20ms) Stream#2:(90%, 20ms) CG: Stream#1: (99%, 30ms) Stream#2: (90%, 30ms) | Option1: Stream#1:(99%, 10ms) Stream#2:(90%, 10ms) Option2: Stream#1:(99%, 15ms) Stream#2:(99%, 10ms) |

Proposal 2: In case SA evaluations are performed based on RAN1 input, PER should be derived based on a RAN1 agreed FER to PER mapping model.

Proposal 3: SA4 is expected to provide the PER and PDB for multi stream modelling including FoV/non-FoV

Reference

- R1-2104023, LS on Status Update on XR Traffic, 3GPP TSG SA WG4, RAN1#104b-e

- S4-210071. “[FS_XRTraffic] Proposed Additional Configurations, Traces and Traffic Models.” Qualcomm Incorporated. 3GPP TSG-SA4 Meeting #112e

- R1-2103278, Further Discussion on Traffic Model for XR evaluations, 3GPP TSG SA WG2, RAN1#104b-e

- Appendix

According to SA4 incoming LS [1], RAN1 should kindly support SA4 in the definition of appropriate and representative test channel and provide feedback and comments on the initial definition. Moreover, based on Permanent Document therein, the test channel is aligned with the QoS model, whose characteristics include, e.g., resource type, priority level, packet delay budget, packet error rate, averaging window and maximum data burst volume. Therefore, RAN1 would like to provide the initial settings for the QoS model in Permanent Document, based on agreed RAN1 simulation assumptions and 5QI values in 23.501.

QoS models from RAN1 perspective are shown as follow, where two models are for Downlink and two models are for Uplink. And some questions or explanations are below the tables. - For Downlink AR/VR traffic, RAN1 has agreed the traffic model, where the bit rate is 30Mbps/45Mbps and air interface PDB = 10ms, with [STD, MAX, MIN] = [10.5, 150, 50]%*mean packet size baseline relationship. Resource type; Delay-critical GBR. Priority Level [1]; N/A. Packet Delay Budget [2]; 10 ms. Packet[3] Error Rate; N/A. Averaging window[4]; 2000ms or N/A. Maximum Data Burst Volume [5]; 30Mbps@60fps: 93750 Bytes 45Mbps@60fps: 140625 Bytes. Note 1: In our opinion, the priority level is introduced for multi-stream simulation. We would like to know views of SA4 rapporteur on multiple streams. According to RAN1's views on multi-stream models, the priority level can be used for identify two different streams. And we have two ways for identification: > inter-stream identification (video stream vs. audio stream) > intra-stream identification (I-frame and P-frame for a video stream) Note 2: From RAN1’s perspective, core network packet delay budget cannot be measured. Hence, the packet delay budget represents air interface packet delay budget. Note 3:From RAN1’s perspective, packet error rate cannot be provided. In current RAN1 simulation, granularity of traffic is frame-level. Hence, only FER could be provided by RAN1. For a stringent mapping relationship between FER and PER, saying that, if one packet in a frame occur error, the frame is regarded as a wrong frame, FER=10-2 for RAN1 simulation can achieve PER = 10-4 required in SA4 new 5QI items. Note 4:In RAN1 simulation statistic model rather than P-trace is considered. As a result, the averaging window is not captured in RAN1 simulation. The default averaging window we provided is based on the 5QI table. Note 5: For 30Mbps@60fps, mean packet size is 62.5kBytes. According to agreement in RAN1#104b-e, the ratio between maximum packet size and mean packet size is 150%. Hence, the maximum packet size is 93.75kBytes.. For 45Mbps@60fps, mean packet size is 93.75kBytes. The ratio is same as the first bullet. Hence, the maximum packet size is 140.625kBytes.. - For Downlink CG traffic, RAN1 has also agreed the traffic model, where the bit rate is 30Mbps/8Mbps and air interface PDB = 15ms, with the same relationship between STD/MAX/MIN and mean packet size as AR/VR traffic model. Resource type; Delay-critical GBR. Priority Level [1]; N/A. Packet Delay Budget [2]; 15 ms. Packet[3] Error Rate; N/A. Averaging window[4]; 2000ms or N/A. Maximum Data Burst Volume [5]; 30Mbps@60fps: 93750 Bytes 8Mbps@60fps: 25000 Bytes. Note 1: In our opinion, the priority level is introduced for multi-stream simulation. We would like to know views of SA4 rapporteur on multiple streams. According to RAN1's views on multi-stream models, the priority level can be used for identify two different streams. And we have two ways for identification: > inter-stream identification (video stream vs. audio stream) > intra-stream identification (I-frame and P-frame for a video stream) Note 2: From RAN1’s perspective, core network packet delay budget cannot be measured. Hence, the packet delay budget represents air interface packet delay budget. Note 3: From RAN1’s perspective, packet error rate cannot be provided. In current RAN1 simulation, granularity of traffic is frame-level. Hence,only FER could be provided by RAN1. For a stringent mapping relationship between FER and PER, saying that, if one packet in a frame occur error, the frame is regarded as a wrong frame, FER=10-2 for RAN1 simulation can achieve PER = 10-4 required in SA4 new 5QI items. Note 4: In RAN1 simulation statistic model rather than P-trace is considered. As a result, the averaging window is not captured in RAN1 simulation. The default averaging window we provided is based on the 5QI table. Note 5: For 30Mbps@60fps, mean packet size is 62.5kBytes. According to agreement in RAN1#104b-e, the ratio between maximum packet size and mean packet size is 150%. Hence, the maximum packet size is 93.75kBytes. For 8Mbps@60fps, mean packet size is 16.67kBytes. The ratio is same as the first bullet. Hence, the maximum packet size is 25.00kBytes.. - For Uplink, the pose / control traffic is agreed, where periodicity is 4ms, packet size is fixed 100 Byte and air interface PDB is 10ms. Resource type; Delay-critical GBR. Priority Level [1]; N/A. Packet Delay Budget [2]; 10 ms. Packet[3] Error Rate; N/A. Averaging window[4]; 2000ms or N/A. Maximum Data Burst Volume [5]; 300 Bytes. Note 1: In our opinion, the priority level is introduced for multi-stream simulation We would like to know views of SA4 rapporteur on multiple streams. According to RAN1's views on multi-stream models, the priority level can be used for identify two different streams. And we have two ways for identification: > inter-stream identification (video stream vs. pose/control) > intra-stream identification (I-frame and P-frame for a video stream) Note 2: From RAN1’s perspective, core network packet delay budget cannot be measured. Hence, the packet delay budget represents air interface packet delay budget. Note 3: From RAN1’s perspective, packet error rate cannot be provided. In current RAN1 simulation, granularity of traffic is frame-level. Hence,only FER could be provided by RAN1. For a stringent mapping relationship between FER and PER, saying that, if one packet in a frame occur error, the frame is regarded as a wrong frame, FER=10-2 for RAN1 simulation can achieve PER = 10-4 required in SA4 new 5QI items. Note 4: In RAN1 simulation statistic model rather than P-trace is considered. As a result, the averaging window is not captured in RAN1 simulation. The default averaging window we provided is based on the 5QI table. Note 5: The pakcet size of pose/ control traffic is fixed 100 Byte. The periodicity of pose/control traffic is 4ms, In 10ms PDB, there are maximum 3 packets transmitting. Hence, the MDBV is 300 Bytes.. - For Uplink, the AR video traffic is agreed, where the bit rate is 10Mbps/20Mbps, air interface PDB is 60ms with the same relationship between STD/MAN/MIN and mean packet size as Downlink traffic model. Resource type; Delay-critical GBR. Priority Level [1]; N/A. Packet Delay Budget [2]; 60 ms. Packet[3] Error Rate; N/A. Averaging window[4]; 2000ms or N/A. Maximum Data Burst Volume [5]; 10Mbps@60fps: 125000 Bytes 20Mbps@60fps: 250000 Bytes. Note 1:In our opinion, the priority level is introduced for multi-stream simulation We would like to know views of SA4 rapporteur on multiple streams. According to RAN1's views on multi-stream models, the priority level can be used for identify two different streams. And we have two ways for identification: > inter-stream identification (video stream vs. audio stream) > intra-stream identification (I-frame and P-frame for a video stream) Note 2: From RAN1’s perspective, core network packet delay budget cannot be measured. Hence, the packet delay budget represents air interface packet delay budget. Note 3: From RAN1’s perspective, packet error rate cannot be provided. In current RAN1 simulation, granularity of traffic is frame-level. Hence,only FER could be provided by RAN1. For a stringent mapping relationship between FER and PER, saying that, if one packet in a frame occur error, the frame is regarded as a wrong frame, FER=10-2 for RAN1 simulation can achieve PER = 10^-4 required in SA4 new 5QI items. Note 4: In RAN1 simulation statistic model rather than P-trace is considered. As a result, the averaging window is not captured in RAN1 simulation. The default averaging window we provided is based on the 5QI table Note 5: For 10Mbps@60fps, mean packet size is 20.8kBytes. According to agreement in RAN1#104b-e, the ratio between maximum packet size and mean packet size is 150%. Hence, the maximum packet size is 31.2kBytes. In 60ms air interface PDB, there are maximum 4 packets transmitting. Hence, the MDBV is 125000 Bytes. For 20Mbps@60fps, mean packet size is 41.7kBytes. The ratio is same as the first bullet. Hence, the maximum packet size is 62.5kBytes. In 60ms air interface PDB, there are maximum 4 packets transmitting. Hence, the MDBV is 250000 Bytes..

Secondly, we consider traces to represent time correlations. - this could be resolved by defining second order matrix for the Thirdly, traces can be used to assign application packets and importance and relevance on the application impact. There are many reasons why traces can provide significantly better information. In summary, for proper quality evaluation, SA4 needs P’-traces to identify application quality.