3GPP TSG RAN WG1 #105-e		R1-2105611

e-Meeting, May 10th – 27th, 2021

Title: 	Remaining issues on mode 1

Source: 	ZTE, Sanechips

Agenda item:	7.2.4

Document for:	Discussion and decision

Introduction

In this contribution, some remaining issues of mode 1 are discussed.

Corrections for Type-1 codebook generation

In this section two points for corrections for type-1 codebook are given respectively, and lastly a final TP is provided.

- Issue 1: Actual PSSCH transmission occasions corresponding to a PSFCH slot

In sidelink physical procedure , it is a common understanding that the number of logical slots in a resource pool within 10240ms period may not be an integer multiple of the PSFCH resource period N, and for the slot location of PSFCH , it is the logical slot with the index #0, #N, #2N, …, within 10240 ms period [1]. When the number of logical slots in a resource pool within 10240ms is not an integer multiple of the PSFCH resource period N, the PSSCH transmission occasions corresponding to the first PSFCH slot within 10240ms is smaller than PSFCH resource period N. In this case to generate type-1 codebook, it should be generated according to the actual corresponding PSSCH transmission occasions with the PSFCH slot, rather than PSFCH resource period.

For Type 1, the number of PSSCH slots associated with the same PSFCH slot could be smaller than the PSFCH resource period.

Type 1 codebook should be generated according to the actual PSSCH transmission occasions corresponding to the PSFCH slot, rather than PSFCH resource period.

- Issue 2: Type-1 HARQ-ACK codebook generation for multiple resource pools

As we know a UE can be configured by higher layers with one or more sidelink resource pools. For each grant, by the indication of “resource pool index” field in DCI or CG, a UE knows which resource pool is used for PSCCH/PSSCH transmission and the corresponding PSFCH reception.

For Type-1 HARQ-ACK codebook determination, some of the relevant descriptions in current TR38.213[2] are as follows:

For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1 b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period

According to the above description, we can see, in a PUCCH transmission in slot nU, the HARQ-ACK information of a set of MA occasions for candidate PSSCH transmissions can be multiplexed in a Type-1 HARQ-ACK codebook. Those candidate PSSCH transmissions may be scheduled by more than one DCI format 3_0 or sl-PSFCH-ToPUCCH-CG-Type1, which would be associated to more than one resource pools. The intention of bullet c) in above description seems to reflect the bitmap configuration for multiple resource pools.

For a Type-1 codebook in a PUCCH transmission in slot nUthe candidate PSSCH transmissions corresponding to a set of MA occasions would be associated to more than one resource pools.

In current TR38.213[2], the pseudo-code for determining the set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions reflects only one resource pool configured, and it is not clear that which resource pool is used during the procedure in current specification.

When multiple resource pools are configured for a SL BWP on a serving cell c, in current 38.213 specification, only one but unknown which one resource pool is used to generate the set MA in the pseudo-code and the HARQ-ACK information bits.

All the configured resource pools should be taken into Type-1 codebook generation.

- Proposed TP:

From all the analysis in this section above, a text proposal can be provided for type-1 codebook correction.

For corrections for Type-1 codebook, a corresponding 38.213 TP in section 16.5.1.1 is given below:

- 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged parts omitted> For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a the period of PSFCH transmission occasion resources for a each sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set NResourcePool to the number of resource pool configured Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH,l NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l Set NPSFCH,l,Actual to the actual corresponding PSSCH transmission occasions with the PSFCH slot for the sidelink resource pool l, where NPSFCH,l,Actual≤NPSFCH,l as specified in clause 16.3 Set l=0-index of sidelink resource pool(s) While l<NResourcePool Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool l and includes PSFCH resources as indicated by a the sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH nF<NPSFCH,l,Actual MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while <Unchanged parts omitted>

TP for SL HARQ-ACK reporting to the gNB

In RAN1#104-e meeting, RAN1 sent a LS(R1-2102176) to ask RAN2 whether to introduce a solution that is compatible with the RAN1 agreements, if the parameter of sl-N1PUCCH-AN is not configurable for SL CG type 2. In this meeting, a reply LS (R1-2104160) from RAN2 is received, and a new parameter for sidelink configured grant type 2 is introduced[3]:

- On Issue 1: RAN2 defines a new parameter sl-N1PUCCH-AN-Type2-r16 to indicate the HARQ resource for PUCCH for PSCCH/PSSCH transmissions without a corresponding PDCCH on sidelink configured grant type 2.

SL-ConfiguredGrantConfig field descriptions

sl-N1PUCCH-AN-Type2 This field indicates the HARQ resource for PUCCH for PSCCH/PSSCH transmissions without a corresponding PDCCH on sidelink configured grant type 2. The actual PUCCH-Resource is configured in sl-PUCCH-Config and referred to by its ID.

Based on RAN2’s reply LS, the corresponding modification of TS38.213 is proposed as below:

16.5 UE procedure for reporting HARQ-ACK on uplink <Unchanged parts omitted> For a PSSCH transmission by a UE that is scheduled by a DCI format, or for a SL configured grant Type 2 PSSCH transmission activated by a DCI format, the DCI format indicates to the UE that a PUCCH resource is not provided when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 2 PSSCH transmission, a PUCCH resource can be provided by sl-N1PUCCH-AN-Type2 and sl-PSFCH-ToPUCCH-r16.For a SL configured grant Type 1 PSSCH transmission, a PUCCH resource can be provided by sl-N1PUCCH-AN and sl-PSFCH-ToPUCCH-CG-Type1. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions. <Unchanged parts omitted>

In TS38.213, capture the parameter sl-N1PUCCH-AN-Type2-r16 to indicate the HARQ resource for PUCCH for PSCCH/PSSCH transmissions without a corresponding PDCCH on sidelink configured grant type 2.

Corrections of TS 38.214, section 8.1.2

In TS 38.212, for DCI format 3_0, the descriptions of DCI field “Configuration index” cannot be found in clause 8.1.2 of [6, TS 38.214].

- 7.3.1.4.1 Format 3_0 DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell. The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI: ... - Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI. ...

To complete the field description of “Configuration index” in DCI format 3_0, the following TP of TS38.214 is proposed:

<Unchanged parts omitted> - 8.1.2.1 Resource allocation in time domain ... In sidelink resource allocation mode 1: - For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0. - For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0. - For sidelink dynamic grant and sidelink configured grant type 2: - The "Time gap" field value m of the DCI format 3_0 provides an index m + 1 into a slot offset table. That table is given by higher layer parameter timeGapFirstSidelinkTransmission and the table value at index m + 1 will be referred to as slot offset KSL. - The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than TDL-TTA2+KSL×Tslot where TDL is starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value and KSL is the slot offset between the slot DCI and the first sidelink transmission scheduled by DCI, Tc is as defined in 38.211, and Tslott is the SL slot duration. - The "Configuration index" field , if provided, in the DCI format 3_0 indicates an index of a configured SL grant provided by high layer. - For sidelink configured grant type 1: - The slot of the first sidelink transmissions follows the higher layer configuration according to [10, TS 38.321]. <Unchanged parts omitted>

Conclusion

This paper concludes with the following proposals and observations:

Observation 1: For Type 1, the number of PSSCH slots associated with the same PSFCH slot could be smaller than the PSFCH resource period.

Observation 2: For a Type-1 codebook in a PUCCH transmission in slot nUthe candidate PSSCH transmissions corresponding to a set of MA occasions would be associated to more than one resource pools.

Observation 3: When multiple resource pools are configured for a SL BWP on a serving cell c, in current 38.213 specification, only one but unknown which one resource pool is used to generate the set MA in the pseudo-code and the HARQ-ACK information bits.

Proposal 1: Type 1 codebook should be generated according to the actual PSSCH transmission occasions corresponding to the PSFCH slot, rather than PSFCH resource period.

Proposal 2: All the configured resource pools should be taken into Type-1 codebook generation.

Proposal 3: For corrections for Type-1 codebook, a corresponding 38.213 TP in section 16.5.1.1 is given below:

- 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel <Unchanged parts omitted> For a SL BWP on a serving cell c and an active UL BWP on the primary cell, as described in Clause 12, a UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-FeedbackToUL-ACK for DCI format 3_0 or by sl-ACKToUL-ACK b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a set of sidelink resource pool bitmaps d) a value of a the period of PSFCH transmission occasion resources for a each sidelink resource pool provided by a respective periodPSFCHresource For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set NResourcePool to the number of resource pool configured Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH,l NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l Set NPSFCH,l,Actual to the actual corresponding PSSCH transmission occasions with the PSFCH slot for the sidelink resource pool l, where NPSFCH,l,Actual≤NPSFCH,l as specified in clause 16.3 Set l=0-index of sidelink resource pool(s) While l<NResourcePool Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool l and includes PSFCH resources as indicated by a the sidelink resource pool bitmap and periodPSFCHresource, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH nF<NPSFCH,l,Actual MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while <Unchanged parts omitted>

Proposal 4: In TS38.213, capture the parameter sl-N1PUCCH-AN-Type2-r16 to indicate the HARQ resource for PUCCH for PSCCH/PSSCH transmissions without a corresponding PDCCH on sidelink configured grant type 2.

Proposal 5: To complete the field description of “Configuration index” in DCI format 3_0, the following TP of TS38.214 is proposed:

<Unchanged parts omitted> - 8.1.2.1 Resource allocation in time domain ... In sidelink resource allocation mode 1: - For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0. - For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0. - For sidelink dynamic grant and sidelink configured grant type 2: - The "Time gap" field value m of the DCI format 3_0 provides an index m + 1 into a slot offset table. That table is given by higher layer parameter timeGapFirstSidelinkTransmission and the table value at index m + 1 will be referred to as slot offset KSL. - The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than TDL-TTA2+KSL×Tslot where TDL is starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value and KSL is the slot offset between the slot DCI and the first sidelink transmission scheduled by DCI, Tc is as defined in 38.211, and Tslott is the SL slot duration. - The "Configuration index" field , if provided, in the DCI format 3_0 indicates an index of a configured SL grant provided by high layer. - For sidelink configured grant type 1: - The slot of the first sidelink transmissions follows the higher layer configuration according to [10, TS 38.321]. <Unchanged parts omitted>

References

- Chairman note RAN1#102-e

- TS 38.213 g50

- R1-2104160(R2-2104463), RAN2, May 10th - 27th, 2021