3GPP TSG RAN WG1 #105-e			R1-2105616

e-Meeting, May10th – 27th, 2021

Agenda item:	8.11.1.2

Source: 	Hyundai Motors

Title:	Discussion on inter-UE coordination for Mode 2 enhancements

Document for:	Discussion and Decision

Introduction

In the Rel-17 WID for NR sidelink enhancement [1], an objective on resource allocation enhancement in mode2 is as follows:

2. Resource allocation enhancement: - Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#90. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#90 is to be decided in RAN#90. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after RAN#89.

In RAN1 #104bis-e [2], the following agreements were made:

Agreement: - Support the following schemes of inter-UE coordination in Mode 2: - Inter-UE Coordination Scheme 1: - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information - FFS condition(s) in which Scheme 1 is used - Inter-UE Coordination Scheme 2: - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict - FFS condition(s) in which Scheme 2 is used Agreements: - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination: - Details include applicable scenario(s)/inter-UE coordination scheme(s) - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc. - Including the possibility of being subject to certain conditions and/or capability Agreement: - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission - For scheme 1: - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information - For scheme 2: - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

In this contribution, we discuss resource allocation enhancement in mode 2 based on inter-UE coordination.

Discussion

In RAN1 #104bis-e meeting, RAN1 supported two schemes of inter-UE coordination in Mode 2:

  - Inter-UE Coordination Scheme 1:

    - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

  - Inter-UE Coordination Scheme 2:

    - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

For the scheme 1, after UE-B receives the inter-UE coordination information from UE-A, UE-B can determine candidate resources for its own PSCCH/PSSCH transmission using the set of resources preferred and/or non-preferred for UE-B’s transmission . For example, UE-B can determine candidate resources based only on the set of resources provided by UE-A. Alternatively, UE-B determines candidate resources based on both the set of resources provided by UE-A and UE-B’s own sensing results.

Proposal 1: For inter-UE coordination scheme 1, UE-B can determine candidate resources based only on received coordination information or based on both received coordination information and UE-B’s sensing results.

For the inter-UE coordination, the coordination information can also include the (parts of) source ID / (parts of) destination ID and RSRP results. Using these information provided by UE-A, UE-B could determine its candidate resources.

Proposal 2: For inter-UE coordination, UE-B can also determine candidate resources based on other coordination information such as (parts of) source / (parts of) destination ID and RSRP.

For the inter-UE coordination, the payload size of coordination information is different between scheme 1 and scheme 2. For scheme 1, since large number of payload size is needed to indicate the set of resources preferred and/or non-preferred for UE-B’s transmission, 1st SCI or 2nd SCI is more suitable for the container. Meanwhile, 1-bit information is needed to indicate the presence of resource conflict. Therefore, PSFCH (or PSFCH-like format) can be used for the container of the inter-UE coordination scheme 2

Proposal 3: SCI (e.g., 1st SCI or 2nd SCI) can be used for the container of inter-UE coordination scheme 1 and the PSFCH (or PSFCH-like format) can be used for the container of inter-UE coordination scheme 2.

For the inter-UE coordination, the coordination information can be sent from one UE-A to multiple UE (e.g., UE-B1, UE-B2, …) or sent from multiple UE (e.g., UE-A1, UE-A2, …) to one UE-B. In these case, it is necessary to investigate how to receive coordination information from UE-A(s), and how UE-B(s) operates after receiving coordination information.

Proposal 4: For inter-UE coordination, RAN1 should further study the cases of multiple UE-As and/or UE-Bs.

Conclusion

In this contribution, the following conclusions were made:

Proposal 1: For inter-UE coordination scheme 1, UE-B can determine candidate resources based only on received coordination information or based on both received coordination information and UE-B’s sensing results.

Proposal 2: For inter-UE coordination, UE-B can also determine candidate resources based on other coordination information such as (parts of) source / (parts of) destination ID and RSRP.

Proposal 3: SCI (e.g., 1st SCI or 2nd SCI) can be used for the container of inter-UE coordination scheme 1 and the PSFCH (or PSFCH-like format) can be used for the container of inter-UE coordination scheme 2.

Proposal 4: For inter-UE coordination, RAN1 should further study the cases of multiple UE-As and/or UE-Bs.

Reference

[1] RP-201385, “WID revision: NR sidelink enhancement,” LG Electronics.

[2] R1-2104103, “Feature lead summary for AI 8.11.2.2 Inter-UE coordination for Mode 2 enhancements,” RAN1 #104bis-e, Moderator (LG Electronics).