3GPP TSG RAN WG1 #105-e	R1-2105635

e-Meeting, May 10th – 27th, 2021

Source:	Sharp

Title:	Discussion on reduced maximum UE bandwidth

Agenda Item:	8.6.1.1

Document for:	Discussion and Decision

Introduction

In RAN1#104bis-e, following agreements and conclusion were obtained [1].

Working assumption: - During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth. - The bandwidth and location of the initial DL BWP for RedCap UEs can be the same as the bandwidth and location of the MIB-configured initial DL BWP for non-RedCap UEs. - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth. - This does not preclude separate or additional bandwidth and location for initial DL BWP for RedCap UEs (FFS). Working assumption: After initial access, at least for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - FFS: BWP#0 configuration option 2 (as in 38.331, Appendix B2) Agreement: - During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Agreement: - After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e: - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Working assumption: A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

In this contribution, we discuss some views on the reduced maximum UE bandwidth features.

Discussions

In RAN1#104bis-e, the operation of initial DL BWP, initial UL BWP and non-initial DL/UL BWP for RedCap UEs and for non-RedCap UEs was mainly discussed, and current situation seems to be as following table 1. Therefore, in this meeting, yellow highlight parts should be discussed.

Table 1: current situation of initial DL BWP, initial UL BWP

|  |  | During initial access | After initial access |
| --- | --- | --- | --- |
| Initial DL BWP | Non-RedCap | MIB-configured not expected to exceed the maximum RedCap UE bandwidth | FFS: can be wider bandwidth than the maximum RedCap UE bandwidth |
|  | RedCap | MIB-configured not expected to exceed the maximum RedCap UE bandwidth SIB-configured FFS: separate or additional bandwidth and location | BWP#0 configuration option 1 (SIB-configured) not expected to exceed the maximum RedCap UE bandwidth BWP#0 configuration option 2 FFS |
| Initial UL BWP | Non-RedCap | Option 1, 2 can be wider bandwidth than the maximum RedCap UE bandwidth Option 3 not expected to exceed the maximum RedCap UE bandwidth | Option 1, 2 can be wider bandwidth than the maximum RedCap UE bandwidth Option 3 not expected to exceed the maximum RedCap UE bandwidth |
|  | RedCap | Option 1 can be wider bandwidth than the maximum RedCap UE bandwidth Option 2, 3 not expected to exceed the maximum RedCap UE bandwidth | Option 1 can be wider bandwidth than the maximum RedCap UE bandwidth Option 2, 3 not expected to exceed the maximum RedCap UE bandwidth |

Initial DL BWP

During initial access, the bandwidth and the location of initial DL BWP is defined by MIB-configured CORESET0 and the bandwidth never exceed the maximum RedCap UE bandwidth (i.e. 20MHz in FR1 and 100MHz in FR2) even for non-RedCap UEs. On the other hand, for off-loading and better scheduling flexibility purpose, the introduction of separate SIB-configured initial DL BWP for RedCap UEs has been considered. In our view, separate SIB-configured initial DL BWP for RedCap UEs is useful also for the case of BWP#0 configuration option 2 after initial access and the specification impact is limited. If the SIB overhead should be limited, some BWP parameters other than location and bandwidth can be common with SIB-configured initial DL BWP for non-RedCap UEs. Regarding the location of separate SIB-configured initial DL BWP for RedCap UEs, if it does not require to contain CORESET0, the location can be out of bandwidth of MIB-configured initial UL BWP and it need to be further discussed.

Proposal 1: During initial access, separate SIB-configured initial DL BWP which contains out of bandwidth of MIB configured initial BWP is supported

For the case after initial access, as mentioned above, if separate initial DL BWP for RedCap UEs is introduced, there is no impact on the bandwidth of initial DL BWP for non-RedCap UEs. Therefore, the RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

Proposal 2: After initial access, separate SIB-configured initial DL BWP is configured/defined for RedCap UEs to allow the scenario where the initial DL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth

Proposal 3: After initial access, for BWP#0 configuration option 2, a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth

Initial UL BWP

In last meeting, for both cases during initial access and after initial access, following 3 options for down-selection were caputered as agreement and each Pros/Cons were discussed.

- Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

Table 2: pros/cons of each option

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 (The scenario is allowed, and a RedCap UE can use the same UL BWP) | - No impact on initial UL BWP configuration for non-RedCap UEs - No impact on resource utilization | - Requiring RF re-tuning time in symbol level - Requiring RAN4 evaluations - More power consumption due to RF re-tuning |
| Option 2 (The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs) | - No impact on initial UL BWP configuration for non-RedCap UEs - No impact on current BWP definition | - New SIB information is needed - Resource fragmentation by PUCCH with frequency hopping may be occurred |
| Option 3 (The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth) | - Minimum impact on specification - No additional SIB information | - Bandwidth limitation of initial UL BWP for non-RedCap UEs |

As mentioned in [2], at least, current flexibility of initial UL BWP for non-RedCap UE should not be restricted by the introduction of RedCap devices. Therefore, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth should be allowed. (i.e. option 3 should not be supported).

Proposal 4: During initial access and after initial access, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth is allowed

Between option 1 and option 2, both options may work for the introduction of RedCap devices. However, many companies have concern to use RF-retuning in option 1 due to the RF re-tuning time in symbol levels and power consumption by frequent RF re-tuning. Although it can be evaluated by RAN 4 with corresponding LS, the straightforward is to apply separate initial UL BWP no wider than the RedCap UE maximum bandwidth for RedCap UEs for both cases during initial access and after initial access. Regarding the drawback of option 2, increase of signalling overhead for separate initial UL BWP can be minimized by using common parameters between initial UL BWP for non-RedCap UEs and for RedCap UEs. For example, the parameters other than locationAndBandwidth can be same between non-RedCap UEs and RedCap UEs. About fragmentation by PUCCH frequency hopping, disabling hopping for RedCap UEs can be used if the PUCCH performance is guaranteed.

Proposal 5: During initial access, separate SIB-configured initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs

Proposal 6: After initial access, separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs

Conclusion

In this contribution, we have the following observations and proposals:

Proposal 1: During initial access, separate SIB-configured initial DL BWP which contains out of bandwidth of MIB configured initial BWP is supported

Proposal 2: After initial access, separate SIB-configured initial DL BWP is configured/defined for RedCap UEs to allow the scenario where the initial DL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth

Proposal 3: After initial access, for BWP#0 configuration option 2, a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth

Proposal 4: During initial access and after initial access, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth is allowed

Proposal 5: During initial access, separate SIB-configured initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs

Proposal 6: After initial access, separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs

References

”RAN1 Chairman’s Notes,” RAN1 #104bis-e, April, 2021.

R1-2103476, “Discussion on reduced maximum UE bandwidth,” Sharp, RAN1 #104bis-e, April, 2021