3GPP TSG RAN WG1 #105-e			R1-2105681

e-Meeting, May 10th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Maintenance for sidelink physical layer procedure

Agenda Item:	7.2.4

Document for: 	Discussion and Decision

- Introduction

At RAN1#104bis-e meeting [1], we discussed maintenance for 5G V2X with NR-SL WID and agreed several CRs.

In this contribution, we share our views for maintenance of SL physical layer procedure.

- Discussions

  - Collision between PUCCH with SL HARQ-ACK and Uu PUCCH/PUSCH

- Discussion/Reason for change

RAN1#98bis Agreements - SL HARQ-ACK is reported in PUSCH when reporting in PUCCH overlaps with a PUSCH transmission. - The Rel-15 procedures and signaling for multiplexing DL HARQ-ACKs in PUSCH are reutilized. RAN1#99 Conclusion: - No support of multiplexing of SL HARQ and Uu UCI on PUCCH or PUSCH in Rel-16 - Note: this reverts the agreements made during RAN1#98b email discussion RAN1#101-e Agreements: - When PUCCH with SL HARQ overlaps with one or more UL TXs, the processing order of addressing UCI multiplexing is reused, i.e. the prioritization between PUCCHs is performed first, then followed by multiplexing/prioritization with PUSCH. RAN1#104-e Agreements: - UE does not expect the case when a PUSCH with no UCI overlaps with two non-overlapping PUCCHs each of which contains SL HARQ-ACK and Uu UCI. - No spec change is needed. Agreement: In terms of prioritization between PUCCH with SL HARQ-ACK and PUCCH with UCI, the principle specified in 9.2.5.0 are used only. CRs? 2/2 latest draft CR is endorsed. Final CR in R1-2102069 (38.213, CR0194) RAN1#104bis-e Conclusion: UE does not expect that PUCCH carrying SL HARQ-ACK overlaps with PUSCH with aperiodic or semi-persistent CSI reports. Agreement: - After prioritization with other PUCCH transmissions, when PUCCH carrying SL HARQ-ACK information is prioritized and overlaps with a PUSCH of priority index 1, - PUCCH carrying SL HARQ-ACK information is dropped if it overlaps with a PUSCH of priority index 1. - After prioritization with other PUCCH transmissions, when PUCCH carrying SL HARQ-ACK information is prioritized and overlaps with a PUSCH of priority index 0, - the SL HARQ-ACK information is multiplexed on the overlapping PUSCH of priority index 0 following Clause 9.2.5 and 9.2.6 if the PUSCH does not contain Uu UCI. - The PUSCH with SL HARQ-ACK is subject to the prioritization with any other UL transmissions as per the existing specifications. - The corresponding CR is to be prepared in RAN1#105-e, - Including checking whether or not any other essential correction is necessary based on the above agreement

At the last meeting, RAN1 have discussed SL HARQ-ACK multiplexing on / prioritization with PUSCH and agreed the above. In this meeting, further discussion is necessary to conclude whether/how spec is updated.

Based on discussions in Rel-16 eURLLC WI and Rel-16 V2X WI, the following processing order is assumed for Uu multiplexing, Uu prioritization, and Uu/SL prioritization in our understanding. (Note that LP means Lower Priority, and HP means Higher Priority.)

Step 1: solve LP PUCCH vs SL PUCCH, and HP PUCCH vs SL PUCCH

Step 2: solve LP PUCCH vs LP PUCCH/PUSCH

Step 3: solve LP PUCCH/PUSCH vs HP PUCCH/PUSCH

Step 4: solve HP PUCCH vs HP PUCCH/PUSCH

Step 5: solve LP PUCCH/PUSCH vs HP PUCCH/PUSCH (redo)

Step 6: solve LP PUSCH vs SL PUCCH, or solve HP PUSCH vs SL PUCCH

Then, reviewing current 38.213 specification, it seems that there are two required corrections as follows:

- PUCCH multiplexing pseudo-code vs section 9.2.5.0

In current spec, PUCCH multiplexing pseudo-code in section 9.2.5 includes section 9.2.5.0, which is describing Uu PUCCH vs SL PUCCH. However, if our understanding of the processing order is correct, the current specification is incorrect. In the above steps, Step 1 is performed as section 9.2.5.0, but the pseudo-code is applied in Step 2 and Step 4. Therefore, 9.2.5.0 should not be included in the pseudo-code. Meanwhile, spec is carefully updated so that the UE processing timeline in section 9.2.5 is applied with 9.2.5.0.

- SL PUCCH vs PUSCH

In current spec, it seems that SL HARQ-ACK can be multiplexed on any PUSCH regardless of presence of Uu UCI. Based on above agreements, SL HARQ-ACK can be multiplexed only on LP PUSCH without UCI. Corresponding correction is required.

- Summary of change

Section 9.2.5.0 of 38.213 is applied independent of PUCCH multiplexing pseudo-code in section 9.2.5.

The above agreements of SL HARQ-ACK multiplexing on / prioritization with PUSCH is described in section 9 of 38.213.

- Consequences if not approved

UE does not handle collision case between PUCCH with SL HARQ-ACK and other PUCCH/PUSCH as agreed.

Text proposal 1:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> # 9 UE procedure for reporting control information ### <Unchanged parts omitted> If a UE - would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and - the PUSCH and PUCCH transmissions fulfill the conditions in Clause 9.2.5 for UCI multiplexing, the UE - multiplexes only HARQ-ACK information in response to a PDSCH reception or a SPS PDSCH release, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic or semi-persistent CSI reports in the PUSCH; - multiplexes only HARQ-ACK information in response to a PDSCH reception or a SPS PDSCH release and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic or semi-persistent CSI reports in the PUSCH.; - multiplexes SL HARQ-ACK reports in the PUSCH transmission and does not transmit the PUCCH if the UCI includes only SL HARQ-ACK reports, the PUSCH transmission has smaller priority index, and the UE does not multiplex any UCI other than SL HARQ-ACK reports in the PUSCH; - cancels the PUCCH transmission if the UCI includes only SL HARQ-ACK reports the PUSCH transmission has larger priority index, the PUSCH transmission has larger priority index, and the UE does not multiplex any UCI other than SL HARQ-ACK reports in the PUSCH; - does not expect that SL HARQ-ACK reports are multiplexed with any UCI other than SL HARQ-ACK reports in the PUSCH transmission. ### <Unchanged parts omitted> When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in Clause 9.2.5.0 and 9.2.6 subject to the processing timelines in Clause 9.2.5. ### <Unchanged parts omitted> ### 9.2.5 UE procedure for reporting multiple UCI types ### <Unchanged parts omitted> while if and resource overlaps with resource else if determine a single resource for multiplexing UCI associated with resources as described in Clauses 9.2.5.0, 9.2.5.1 and 9.2.5.2 set the index of the single resource to % start from the beginning after reordering unmerged resources at next step % function that re-orders resources in current set Set to the cardinality of else end if end if end while ### <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

  - SL/UL prioritization – reference of UL multiplexing/prioritization

RAN1#101-e Agreements: - When one SL transmission overlaps with one or more UL transmissions overlapping with any UL transmission in time, - multiplexing/prioritization defined on Uu, if any applicable, apply to the overlapping UL transmissions before the prioritization with the SL transmission. - If the SL transmission still overlaps with more than one UL transmission, - the UE transmits SL transmission if the SL transmission is prioritized over all the UL transmissions subject to both UE multiplexing and processing timeline w.r.t. the first SL/UL transmission. - the UE transmits UL transmission if at least one UL transmission is prioritized over the SL transmission subject to both UE multiplexing and processing timeline w.r.t. the first SL/UL transmission. - UE processing timeline w.r.t. the first SL/UL transmission is the same as in proposal 8.

- Reason for change

At the previous meeting, RAN1 agreed that UL channel overlap is solved before SL/UL prioritization. In the current specification for this procedure, there is a description that prioritization between SL/UL is performed after doing clause 9.2.5 of 38.213. However, for UL multiplexing/prioritization, UE firstly follows clause 9 including prioritization by priority index and then 9.2.5 (multiplexing procedure for the same priority index and no repetition case) or 9.2.6 (repetition case). Referring only 9.2.5 is not sufficient.

- Summary of change

Referred sections are added.

- Consequences if not approved

Prioritization/multiplexing specified in section 9 and 9.2.6 is not included in procedures performed before SL/UL prioritization. This is not intended UE behavior in RAN1 agreements.

Text proposal 2:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> ##### 16.2.4.3.1 Prioritizations for sidelink and uplink transmissions A UE performs prioritization between SL transmissions/receptions and UL transmissions after performing the procedures described in Clause 9, Clause 9.2.5, and Clause 9.2.6, and in Clause 6.1 of [6, TS 38.214]. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

  - SL/UL prioritization – carriers

- Reason for change

At the current specification, UE behavior for prioritization of SL and UL at the same serving cell is described for UE incapable of the simultaneous transmissions. However, from two reasons, the current text should be updated. Firstly, ‘serving cell’ is not valid terminology for SL. Secondly, description of the corresponding UE capability in 38.306 is the following. In other words, UL and SL may be at the same band/CC, may not.

supportedTxBandCombListPerBC-Sidelink-r16, supportedRxBandCombListPerBC-Sidelink-r16 Indicates, for a particular Uu band combination, the PC5 band combination(s) on which the UE supports simultaneous transmission/reception. The leading / leftmost bit (bit 0) corresponds to the first band combination included in BandCombinationListSidelinkEUTRA-NR, the next bit corresponds to the second band combination included in BandCombinationListSidelinkEUTRA-NR and so on. with value 1 indicating simultaneous transmission/reception is supported.

Similarly, UE behavior of SL and UL for UE capable of the simultaneous transmissions should be updated as well; i.e., ‘serving cell’ is not used and any combinations of Uu BC and PC5 BC are covered.

- Summary of change

A: ‘Serving cell’ is not used. Instead, ‘carrier’ is used as reception part.

B: UL and SL on both the same carrier or different carriers are covered.

- Consequences if not approved

RAN1 spec. is not aligned with UE capability.

Text proposal 3:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> #### 16.2.4.3 Simultaneous SL and UL transmissions/receptions If a UE - would simultaneously transmit on the UL and on the SL of a serving cell in a carrier or in two respective carriers, and - the UE is not capable of simultaneous transmissions on the UL and on the SL of the serving cell in the carrier or in the two respective carriers the UE transmits only on the link, UL or SL, with the higher priority. If a UE - would simultaneously transmit on the UL and receive on the SL in a carrier, the UE transmits on UL or receives on SL, with the higher priority. If a UE - is capable of simultaneous transmissions on the UL and on the SL of two respective carriers of a serving cell, or of two respective serving cells in a carrier or in two respective carriers, - would transmit on the UL and on the SL of the two respective carriers of the serving cell, or of the two respective serving cells in the carrier or in the two respective carriers, - the transmission on the UL would overlap with the transmission on the SL over a time period, and - the total UE transmission power over the time period would exceed PCMAX the UE - reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX - reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

- Conclusion

In this contribution, we discussed maintenance of SL physical layer procedure. Proposals are summarized as following:

Text proposal 1:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> # 9 UE procedure for reporting control information ### <Unchanged parts omitted> If a UE - would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and - the PUSCH and PUCCH transmissions fulfill the conditions in Clause 9.2.5 for UCI multiplexing, the UE - multiplexes only HARQ-ACK information in response to a PDSCH reception or a SPS PDSCH release, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic or semi-persistent CSI reports in the PUSCH; - multiplexes only HARQ-ACK information in response to a PDSCH reception or a SPS PDSCH release and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic or semi-persistent CSI reports in the PUSCH.; - multiplexes SL HARQ-ACK reports in the PUSCH transmission and does not transmit the PUCCH if the UCI includes only SL HARQ-ACK reports, the PUSCH transmission has smaller priority index, and the UE does not multiplex any UCI other than SL HARQ-ACK reports in the PUSCH; - cancels the PUCCH transmission if the UCI includes only SL HARQ-ACK reports the PUSCH transmission has larger priority index, the PUSCH transmission has larger priority index, and the UE does not multiplex any UCI other than SL HARQ-ACK reports in the PUSCH; - does not expect that SL HARQ-ACK reports are multiplexed with any UCI other than SL HARQ-ACK reports in the PUSCH transmission. ### <Unchanged parts omitted> When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in Clause 9.2.5.0 and 9.2.6 subject to the processing timelines in Clause 9.2.5. ### <Unchanged parts omitted> ### 9.2.5 UE procedure for reporting multiple UCI types ### <Unchanged parts omitted> while if and resource overlaps with resource else if determine a single resource for multiplexing UCI associated with resources as described in Clauses 9.2.5.0, 9.2.5.1 and 9.2.5.2 set the index of the single resource to % start from the beginning after reordering unmerged resources at next step % function that re-orders resources in current set Set to the cardinality of else end if end if end while ### <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

Text proposal 2:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> ##### 16.2.4.3.1 Prioritizations for sidelink and uplink transmissions A UE performs prioritization between SL transmissions/receptions and UL transmissions after performing the procedures described in Clause 9, Clause 9.2.5, and Clause 9.2.6, and in Clause 6.1 of [6, TS 38.214]. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

Text proposal 3:

-------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> #### 16.2.4.3 Simultaneous SL and UL transmissions/receptions If a UE - would simultaneously transmit on the UL and on the SL of a serving cell in a carrier or in two respective carriers, and - the UE is not capable of simultaneous transmissions on the UL and on the SL of the serving cell in the carrier or in the two respective carriers the UE transmits only on the link, UL or SL, with the higher priority. If a UE - would simultaneously transmit on the UL and receive on the SL in a carrier, the UE transmits on UL or receives on SL, with the higher priority. If a UE - is capable of simultaneous transmissions on the UL and on the SL of two respective carriers of a serving cell, or of two respective serving cells in a carrier or in two respective carriers, - would transmit on the UL and on the SL of the two respective carriers of the serving cell, or of the two respective serving cells in the carrier or in the two respective carriers, - the transmission on the UL would overlap with the transmission on the SL over a time period, and - the total UE transmission power over the time period would exceed PCMAX the UE - reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX - reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------

References

- 3GPP RAN1#104bis-e meeting,	chairman’s notes