3GPP TSG RAN WG1#105-e	 	 	R1-2105701

e-Meeting, May 10th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on DL-AoD positioning enhancements

Agenda Item:	8.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#91-e meeting, updated WID on NR positioning was agreed [1]. The work item includes RAN1-led objectives as follows:

The objective of this work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. The objective of this work item also includes the support of GNSS enhancements.

The specific objectives of this work are:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

Notes:

Solutions for RAT-dependent positioning enhancements are designed to operate in both frequency ranges (i.e. FR1 & FR2)

RAN working groups will take into account the work from other working group SA/CT which involves same objectives.

- The WID is subject to further update in RAN #91 for RAN1/2/3/4 scoping.

In this contribution, we present our view on DL-AoD enhancements for Rel-17 NR positioning.

- Potential enhancements for improving accuracy

  - RSRP measurement reporting

Regarding enhancements related to RSRP measurement reporting, the following agreement was made at the last meeting [2].

Agreement:

For UE-assisted DL AOD, select one of the following options for reporting of RSRP measurements per TRP

- Option 1: Up to 8 measurements in a measurement report (as in release 16)

- Option 2: Up to 8 measurements in a measurement report, for the same Rx beam index

- Option 3: Up to N>=8 measurements

  - Note: Multiple measurements corresponding to different Rx Beam index may be reported for a given PRS resource.

  - FFS: value for N.

Rel-16 NR supports multiple RSRP measurements within a DL-AoD reporting. However, whether the multiple RSRP measurements correspond to the same Rx beam or not is up to UE implementation. If the multiple RSRP measurements correspond to the same Rx beam index, we think it will be helpful to improve positioning accuracy. Therefore, we prefer Option 2 compared with Option 1.

If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is enough to achieve sufficient positioning accuracy, Option 2 would be enough. On the other hand, if additional reporting based on different Rx beams is beneficial to improve the positioning accuracy, Option 3 (i.e., Option 2 plus additional measurement(s) based on different Rx beam(s)) can be considered.

Observation 1:

- Rel-17 can consider the following two alternatives for further study

  - Alt 1: If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is enough to achieve sufficient positioning accuracy, Option 2 is adopted

  - Alt 2: If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is not enough to achieve sufficient positioning accuracy, Option 3 is adopted and it includes Option 2 like measurement results (i.e., based on same Rx beam) and additional measurement result(s) (i.e., based on different Rx beam(s))

  - Adjacent beam reporting

Regarding enhancements related to adjacent beam reporting, the following agreement was made at the last meeting [3].

Agreement:

Support the following enhancements under UE capability for both UE-B and UE-A DL-AOD positioning method

- Enhancing the signaling to UE for the purpose of PRS resource(s) measurement and (for UE-A) report

  - FFS: The detailed signaling (e.g, the boresight direction for UE-A DL-AoD, further spatial information of PRS resources, processing prioritization of PRS resources)

- FFS: The following options

  - Option 1: Enhancing the reporting to include the measurements of adjacent beams PRS resources that related with each other indicated by the assistance data.

  - Option 2: UE can be requested to measure and report on specific PRS resources

Rel-16 NR supports multiple RSRP measurements (up to 8 measurements) within a DL-AoD reporting. However, the UE may not report adjacent beams of the best beam in a measurement reporting since the UE does not understand correspondence between DL-PRS resources and Tx beams. If the UE can be indicated to measure and report these adjacent beams in addition to the best beam, it may be beneficial to improve DL-AoD positioning accuracy. In order to support the UE to measure the adjacent beams, we can consider adding adjacent DL-PRS resource IDs to the DL-PRS configuration so that UE understands DL-PRS resources for the adjacent beams of each beam and can include measurement results on adjacent beams in addition to that on the best beam (as in Option 1 in above agreement).

Observation 2:

- We can consider adding adjacent DL-PRS resource IDs to the DL-PRS configuration

  - Indication of expected departure/arrival angle

Regarding enhancements related to angle measurement and reporting, the following agreement was made at the last meeting [3].

Agreement:

For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, study further whether to support at most one of the following options:

- Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

  - Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP]

- Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

  - Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP]

- Option 3: Indication of expected AoD/ZoD or AoA/ZoA value and uncertainty is not introduced.

- FFS: details of signaling

FFS: Applicability of this agreement to other Positioning methods

In order to improve accuracy of UL-AoA measurement, RAN1 agreed supporting expected AoA/ZoA signaling from LMF to gNB/TRP. Similarly, in case of DL-AoD measurement, indication of expected departure/arrival angle value for DL-AoD purpose would be helpful to determine Rx beam direction at the UE side. Therefore, we support introducing indication of expected angle value for DL-AoD measurement.

In Option 1, the overhead of expected angle information may be smaller than the overhead required in Option 2 since DL-PRS resource ID based information can be considered. The relation between DL-PRS resource IDs and DL-AoD/ZoD may need to be clear in the specifications. In addition, how to determine Rx beam direction is up to the UE. On the other hand, Option 2 can indicate Rx beam direction(s) for the UE, however, the overhead of expected angle information may be larger than the overhead in Option 1. We may need to consider the impact on the required additional assistance information.

Proposal 1:

- We support one of the following options

  - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

  - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

- Conclusion

In this contribution, we discussed on potential enhancements for Rel-17 DL-AoD positioning. Based on the discussion, we made following observations and proposal.

Observation 1:

- Rel-17 can consider the following two alternatives for further study

  - Alt 1: If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is enough to achieve sufficient positioning accuracy, Option 2 is adopted

  - Alt 2: If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is not enough to achieve sufficient positioning accuracy, Option 3 is adopted and it includes Option 2 like measurement results (i.e., based on same Rx beam) and additional measurement result(s) (i.e., based on different Rx beam(s))

Observation 2:

- We can consider adding adjacent DL-PRS resource IDs to the DL-PRS configuration

Proposal 1:

- We support one of the following options

  - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

  - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

References

- RP-210903, 3GPP RAN#91-e, “Revised WID on NR Positioning Enhancements”, Electronic Meeting, Mar. 2021.

- 3GPP RAN1#104-e, Chairman’s Notes, Feb. 2021.

- 3GPP RAN1#104b-e, Chairman’s Notes, Apr. 2021.