3GPP TSG RAN WG1 #105-e		　　　　　　　　　                R1-2105708

e-Meeting, May 10th - 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on paging enhancements

Agenda Item:	8.7.1.1

Document for: 	Discussion and decision

- Introduction

At the RAN1#104b meeting, the following were agreed. [1]

Agreement: Observation 1a: For the evaluation and comparison of PEI candidate designs, the following observations for coexistence with legacy PDSCH are identified: - For coexistence with legacy PDSCH, semi-static resouce sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering PEI REs is supported for all PEI candidate designs. - For coexistence with legacy PDSCH, dynamic resource sharing can be realized for all PEI candidates if PDSCH is scheduled by DCI format 1_1 - For PDCCH based PEI, CORESET-level rate matching can be realized for the PDSCH as per mandatory capability - For SSS-based PEI, CORESET-level rate matching may be realized for the PDSCH as per mandatory capability, depending on the design of SSS-based PEI and UE capability regarding number of supported CORESETs - For TRS/CSI-RS based PEI, RE-level rate matching can be realized for the PDSCH as per mandatory capability - When PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how PEI is transmitted in PDSCH resource Observation 2a: For the evaluation and comparison of PEI candidate designs, the following summarize the identified configurations of PEI candidate designs, including pairs of the minimum required resource and maximum UE (sub)group indication capacity per PEI, that can comply with the mandatory performance metrics agreed in RAN1 #104-e: - If Behv-A is assumed, Paging Setting; PEI candidate design; Physical-layer configuration and resource; UE (sub)group indication capcity; Number of companies providing performance results. PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload; PDCCH-based PEI; AL4 PDCCH with 12-bit payload, occupying 288 REs; 12 bits; 5 (HW/HiSi, OPPO, ZTE, CATT, MTK). AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo). AL8 PDCCH with 41-bit payload, occupies 576 REs; 41 bits; 1 (CATT). SSS-based PEI; 1-symbol SSS, occupying 132 REs (11 RB x 1 symbol); 3 bits; 1 (IDCC). 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols); 1 bit; 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung). 3 bits; 1 (IDCC). 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 4 bits; 1 (MTK). TRS/CSI-RS-based PEI; 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols); ≥ 8 bits; 1 (Intel). 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (HW/HiSi). 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (Samsung). 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 1 bit; 3 (vivo, ZTE, Ericsson). 6 bits; 1 (CATT). 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 2 (OPPO, QC). 4 bits; 1 (MTK). PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload; PDCCH-based PEI; AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 4 (OPPO, ZTE, MTK, Intel). SSS-based PEI; 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 4 bits; 1 (MTK). TRS/CSI-RS-based PEI; 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols); 3 bits; 1 (Intel). 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols); 8 bits; 1 (Intel). 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO). 4 bits; 1 (MTK). - If Behv-B is assumed, Paging Setting; PEI candidate design; Physical-layer configuration; UE (sub)group indication capcity; Number of companies providing performance results. PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload; PDCCH-based PEI; AL4 PDCCH with 12-bit payload, occupying 288 REs; 12 bits; 4 (HW/HiSi, OPPO, ZTE, MTK). AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 2 (vivo, Samsung). SSS-based PEI; 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols); 1 bit; 3 (HW/HiSi, vivo, ZTE). 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 1 bit; 1 (MTK). TRS/CSI-RS-based PEI; 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (HW/HiSi). 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 1 bit; 2 (vivo, ZTE). 6 bits; 1 (CATT). 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO). 2 bits; 1 (MTK). PDSCH: MCS0, TB scaling 0.5 PDCCH: AL16, 41-bit payload; PDCCH-based PEI; AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 3 (OPPO, ZTE, MTK). SSS-based PEI; 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 1 bit; 1 (MTK). TRS/CSI-RS-based PEI; 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO). 2 bits; 1 (MTK). Observation 3a: For the evaluation and comparison of PEI candidate designs, the following summarize average resource overheads per PO for PEI candidate designs, considering the configurations identified from performance observation. - The average overhead results are based on PO settings without impact from UE sub-grouping indication within the PO. - Note: For comparison purpose, single-beam transmission for PEI is assumed, and results with multi-beam transmission for PEI is scaled. This doesn’t preclude any beam-forming related design for PEI. - If Behv-A is assumed: Paging Setting; PEI candidate design; Physical-layer configuration and resource; UE (sub)group indication capacity; Number of companies providing performance results; Average resource overhead per PO (REs); PO and PEI related assumptions; Resource sharing assumption. PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload; PDCCH-based PEI; AL4 PDCCH with 12-bit payload, occupying 288 REs; 12 bits; 5 (HW/HiSi, OPPO, ZTE, CATT, MTK); 17.2; OPPO; 1 PEI for up to 12 PO's; PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged. 17.2; ZTE; 1 PEI for up to 12 PO's. 17.6; HW/HiSi; 1 PEI for up to 12 PO's. 21.8; MTK; 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle. 28.8; CATT; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 288.0; CATT; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo); 49.5; vivo; 1 PEI for 4 PO; PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged. 57.6; vivo; 1 PEI for 1 PO. 57.6; QC; 1 PEI for 1 PO. 57.6; Samsung; 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam. AL8 PDCCH with 41-bit payload, occupies 576 REs; 41 bits; 1 (CATT); 57.6; CATT; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 576.0; CATT; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. SSS-based PEI; 1-symbol SSS, occupying 132 REs (11 RB x 2 symbols); 3 bits; 1 (IDCC). 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols); 1 bit; 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung); 25.4; Samsung; 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam; Dynamic rate-matching in PDSCH. 25.4; vivo; 1 PEI for 1 PO. 26.4; ZTE; 1 PEI for 1 PO. 28.8; CATT; 1 PEI for 1 PO. 28.8; QC; 1 PEI for 1 PO. 254.0; HW/HiSi; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 264.0; ZTE; 1 PEI for 1 PO. 288.0; QC; 1 PEI for 1 PO. 3 bits; 1 (IDCC). 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 4 bits; 1 (MTK); 34.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; Dynamic rate-matching in PDSCH. 437.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms; Semi-static rate-matching in PDSCH. TRS/CSI-RS-based PEI; 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols); ≥ 8 bits; 1 (Intel); 14.4; Intel; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (HW/HiSi); 123.4; HW/HiSi; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 168.0; HW/HiSi; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (Samsung); 21.6; Samsung; 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam; Dynamic rate-matching in PDSCH. 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 1 bit; 3 (vivo, ZTE, Ericsson); 28.8; vivo; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 28.8; ZTE; 1 PEI for 1 PO. 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 6 bits; 1 (CATT); 28.8; CATT; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 288.0; CATT; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 2 (OPPO, QC); 30.0; OPPO; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 30.0; QC; 1 PEI for 1 PO. 300.0; QC; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 4 bits; 1 (MTK); 26.0; MTK; 1 PEI for up to 4 PO's; Dynamic rate-matching in PDSCH. PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload; PDCCH-based PEI; AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 4 (OPPO, ZTE, MTK, Intel); 34.4; OPPO; 1 PEI for up to 12 PO's; PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged. 43.6; MTK; 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle. 57.6; Intel; 1 PEI for 1 PO; PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged. SSS-based PEI; 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 4 bits; 1 (MTK); 34.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; Dynamic rate-matching in PDSCH. 437.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms; Semi-static rate-matching in PDSCH. TRS/CSI-RS-based PEI; 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols); 3 bits; 1 (Intel); 14.4; Intel; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols); 8 bits; 1 (Intel); 21.6; Intel; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO); 30.0; OPPO; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 4 bits; 1 (MTK); 26.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; Dynamic rate-matching in PDSCH. - If Behv-B is assumed: Paging Setting; PEI candidate design; Physical-layer configuration; UE (sub)group indication capacity; Number of companies providing performance results; Average resource overhead per PO (REs); PO and PEI related assumptions; Coexistence assumption. PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload; PDCCH-based PEI; AL4 PDCCH with 12-bit payload, occupying 288 REs; 12 bits; 4 (HW/HiSi, OPPO, ZTE, MTK); 24.0; HW/HiSi; 1 PEI for up to 12 PO's; PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET. 24.0; OPPO; 1 PEI for up to 12 PO's. 24.0; ZTE; 1 PEI for up to 12 PO's. 51.0; MTK; 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle. AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 2 (vivo, Samsung); 518.4; vivo; 1 PEI for 1 PO; PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET. 518.4; Samsung; 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam. SSS-based PEI; 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols); 1 bit; 3 (HW/HiSi, vivo, ZTE); 228.6; vivo; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 228.6; ZTE; 1 PEI for 1 PO. 254.0; HW/HiSi; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 264.0; ZTE; 1 PEI for 1 PO. 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 1 bit; 1 (MTK); 561.0; MTK; 1 PEI for 1 PO; average over all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms; Semi-static rate-matching in PDSCH. TRS/CSI-RS-based PEI; 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (HW/HiSi); 168.0; HW/HiSi; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 1 bit; 2 (vivo, ZTE); 259.2; ZTE; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 259.2; vivo; 1 PEI for 1 PO. 6 bits; 1 (CATT); 288.0; CATT; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO); 279.0; OPPO; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 2 bits; 1 (MTK); 150.0; MTK; 1 PEI for 2 PO's; Semi-static rate-matching in PDSCH. PDSCH: MCS0, TB scaling 0.5 PDCCH: AL16, 41-bit payload; PDCCH-based PEI; AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 3 (OPPO, ZTE, MTK); 48.0; OPPO; 1 PEI for up to 12 POs; PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET. 102.0; MTK; 1 PEI for up to 12 POs. SSS-based PEI; 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 1 bit; 1 (MTK); 561.0; MTK; 1 PEI for 1 PO; RB-symbol rate-matching pattern period up to 40 ms; Semi-static rate-matching in PDSCH. TRS/CSI-RS-based PEI; 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO); 270.0; OPPO; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 2 bits; 1 (MTK); 150.0; MTK; 1 PEI for 2 PO's; Semi-static rate-matching in PDSCH.

- Discussion

In this contribution, we will mainly describe paging early indication.

  - Features of paging early indication

In NR paging, UE determines PO (Paging Occasion) for the UE based on the calculation by e.g., UE ID, and the PO includes PDCCH monitoring occasion(s) which corresponds to each SSB. A PO can consist of a set of monitoring occasions corresponding to different beams in multi-beam operations. A large number of UEs share the same PO. These UEs wake up to monitor paging PDCCH in the same PO and have to receive corresponding PDSCH including paging message. If the paging message does not have the information for UEs, power consumption increases by unnecessary paging reception. In previous meeting, it was agreed to support PEI (paging early indication) before paging occasion in order to resolve such problem. As for PEI design, the following candidates can be considered:

- DCI-based approach

- RS/Sequenced-based approach

The characteristics of each PEI design are summarized in the table1 below.

Table1．PEI design characteristic

| PEI Design | Advantages | Disadvantages |
| --- | --- | --- |
| DCI-based | - Low false alarm due to CRC - Information other than Wake UP notifications can be easily notified | - sensitive to synchronization error |
| RS/Sequence-based | - Not sensitive to synchronization error | - Trade-off between false alarm rate and miss detection rate |

When introducing PEI, the impact on legacy paging functionality should be considered. One functionality of the paging DCI is to inform UEs of some legacy information (e.g., ETWS, SI update).

Legacy UE only monitors paging DCI by monitoring PO in every cycle. On the other hand, UE supporting PEI should monitor PO to check short message field for SI change and/or ETWS in addition to PEI. That is, UE supporting PEI might have more power consumption compared to legacy UE.

One of the solutions is to notify UE of such legacy information via PEI. The UE monitors the PEI first to know whether or not there is a paging message. Thus, it is reasonable to notify the legacy information via PEI so that additional power consumption can be avoided. It is not a significant issue for any PEI candidate to support 2bits indication for SI change and ETWS.

Observation 1: If UE needs to monitor PO for SI change and/or ETWS in the case of configuring PEI, it causes additional UE power consumption.

Proposal 1: It should be considered that ETWS and SI update is indicated via PEI.

Also, availability indication of TRS/CSI-RS for idle/inactive mode UE to acquire ACG and synchronization can be informed by PEI. In following section, we discuss how to inform about it. When there is connected mode UE in the cell, idle/inactive mode UE can use TRS/CSI-RS to acquire ACG and synchronization. Our previous paper [2] shows the evaluation results on the power saving gain obtained by availability indication in paging early indication. In case the group paging rate is 60%, the power saving gain is 12.1% when only DCI-based PEI is applied, and when TRS/CSI-RS is additionally applied, the power saving gain becomes 24.7%. In other word, the UE can get the additional power saving gain of 12.6% by using TRS/CSI-RS for synchronization.

But the UE needs to be informed of availability indication of TRS/CSI-RS by the NW to achieve power saving. If the UE doesn’t know availability of TRS/CSI-RS, UE has to perform Blind detection at the TRS/CSI-RS occasion(s), which will increase power consumption. One of the methods to inform UE of availability of TRS/CSI-RS is to adapt DCI-based PEI and include availability of TRS/CSI-RS in DCI-based PEI. In order to indicate availability of TRS/CSI-RS by PEI, it is necessary to notify more information by PEI, and it is DCI-based PEI that has this characteristic.

Observation 2: Availability indication of TRS/CSI-RS for idle/inactive mode UE to acquire ACG and synchronization can be informed by DCI-based PEI.

  - How to inform UE subgroup information

In the previous meetings, we discussed about how to inform UE of subgroup information. According to the simulation results provided by the companies in the previous meetings, when UE subgroups indication is carried by PEI, the power saving gain is higher than that obtained by UE subgroup indication in paging DCI. Also, the larger the number of subgroups becomes, the larger the power saving gain becomes. From this result, it is desirable that PEI can notify a lot of subgroup information. It is proposed that SSS-based also inform it, but there would be a little subgroup information that can be informed. On the other hand, DCI-based PEI can notify a lot of information as mentioned above.

In addition, the higher the group paging rate becomes, the higher the power saving gain for subgroups becomes. Also, TRS/CSI-RS for idle/inactive mode has similar property. The reason why power saving gain for subgroups and TRS/CSI-RS become the higher is that even when the UE is indicated to wake up by PEI, at least for some UEs among UEs with the same PO, the UE can skip some actions, e.g., SSB reception, paging PDCCH/PDSCH reception, and move into sleep earlier so that power saving gain can be obtained.

On the other hand, PEI has the opposite characteristics, and the higher the group paging rate becomes, the smaller the power saving gain becomes. Power saving gain of only PEI can be obtained to skip the some actions such as described above when the UE is indicated not to wake up by PEI, i.e., all UEs with the same PO are not paged.   Hence, when group paging rate is high, power saving gain of only PEI is not good because UE has to frequently wake up, and on the other hand subgroup indication and TRS/CSI-RS indication can provide better power saving gain.

Therefore, the combination of subgroups and PEI should be considered.

Observation 3: DCI-based PEI that can notify a lot of information is desirable for subgroups.

Observation 4: The power saving gain can be obtained at any grouping rate by combining PEI with subgroup indication and/or TRS/CSI-RS indication.

  - UE behavior

In the last meeting, we discussed the following Behavior A/B.

- Behv-A:

  - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged

  - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

- Behv-B:

  - PEI indicates whether or not UE should monitor a PO

  - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

As it was pointed out by some companies, Behv-A can have less overhead than Behv-B. On the other hand, even in the case of Behv-B, overhead is not big issue at least in some cases.

One example is the case of high group paging rate. The higher the GPR becomes, the more frequently PEI is transmitted even in Behv-A. In this case, there is not much difference on resource overhead between Behavior A and B. Also, in Behv-B, NW can have the flexibility to prioritize legacy channels/signals. For example, when the resource is full for legacy channels/signals and there is no room for PEI, NW can skip PEI transmission and UE just monitors a PO since UE does not detect PEI. When NW skip PEI in Behv-A, UE can’t receive paging DCI in PO and try to monitor PEI at the next paging cycle.

Thus, it is beneficial that NW can choose the default behavior between Behv-A and B, and Behv-A/B should be configurable, e.g., via SIB and/or Paging DCI.

Proposal 2: Behavior A or Behavior B should be configurable.

  - Coexistence

In the last meeting, we discussed how PEI coexist with legacy signals/channels. Regarding SSS-based PEI, we understand that CORESET-level rate matching may be realized for the PDSCH if a dedicated CORESET and Search Space set are configured for a rate matching. However, there may be impact for legacy UE since it causes additional power comsumption to do the blind detection in a dedicated CORESET and Search Space set. a dedicated CORESET used only for the rate matching wastes the configurable number of CORESET. When it comes to coexsistance SSS-based PEI with PDCCH, the current specification defines the UE behavior for coexistence only based on SSB  provided by SIB1 [section 10 in TS38.213] .

Observation 5: There are some restrictions in SSS-based PEI to avoid to impact legacy UE.

  - PEI design

As explained in section 2.1-2.4, DCI-based PEI is better from several points of view and should be supported. Compared with RS/sequence-based PEI, DCI-based PEI has less system impact and can provide more benefits by indicating additional information.

Proposal 3: DCI-based PEI should be supported.

  - Information notified by PEI

The following candidates can be considered as the information notified by PEI:

- Whether or not UE wakes up at PO

- Subgroup Information

- Availability of TRS/CSI-RS for idle/inactive mode

- legacy indication (ETWS,SI update)

In a paging cycle, the UE first receives the PEI in front of the PO. As explained above, further power consumption reduction can be achieved if availability of TRS/CSI-RS and subgroup information are notified to the UE by PEI. Also, if legacy indication, e.g., ETWS, SI update, can be indicated by PEI, the UE does not need to paging DCI only for the legacy indication.

Proposal 4: The following candidates can be considered as the information notified by PEI:

- Whether or not UE wakes up at PO

- Subgroup Information

- Availability of TRS/CSI-RS for idle/inactive mode

- legacy indication (ETWS, SI update)

- Conclusion

In this contribution, paging early indication was discussed. Based on the discussion, the following observations and proposals were made:

Observation 1: If UE needs to monitor PO for SI change and/or ETWS in the case of configuring PEI, it causes UE additional power consumption.

Observation 2: Availability indication of TRS/CSI-RS for idle/inactive mode UE to acquire ACG and synchronization can be informed by DCI-based PEI.

Observation 3: DCI-based PEI that can notify a lot of information is desirable for subgroups.

Observation 4: The power saving gain can be obtained at any grouping rate by combining PEI with subgroup indication and/or TRS/CSI-RS indication.

Observation 5: There are some restrictions in SSS-based PEI to avoid to impact legacy UE.

Proposal 1: It should be considered that ETWS and SI update is indicated via PEI.

Proposal 2: Behavior A or Behavior B should be configurable.

Proposal 3: DCI-based PEI should be supported.

Proposal 4: The following candidates can be considered as the information notified by PEI:

- Whether or not UE wakes up at PO

- Subgroup Information

- Availability of TRS/CSI-RS for idle/inactive mode

- legacy indication (ETWS, SI update)

References

- “RAN1 Chairman’s Notes”, RAN1#104b, April 12th –April 20th, 2021.

- R1-2101622, "Discussion on paging enhancements", NTT DOCOMO, INC., January 2021.