3GPP TSG-RAN WG1 Meeting #105-e											R1-2105745

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			FL summary on the maintenance of 2-step RACH

Document for:	Discussion

Introduction

This document contains the summary of issues related to the maintenance of Rel-16 2-step RACH WI in RAN1#105-e meeting.

Maintenance issues

The following 2 issues are identified based on the submitted contributions in RAN1#104b-e.

| Issue # | Description | Related TDoc # |
| --- | --- | --- |
| 1 | Editorial correction on the DMRS parameter for MsgA | R1-2104474 |
| 2 | Determination of power control parameter and PUSCH waveform in case of 2-step RACH only operation | R1-2105507 |

Views on the above issues, please fill in ‘Yes/No/Editorial’ to the following table.

| Company | Issue #1 | Issue #2 | Comments |
| --- | --- | --- | --- |
| Huawei | Y | Y |  |
| Ericsson | Editorial | Y |  |
| Qualcomm | Editorial | Y | In addition to the two issues above, we think it is necessary to clarify the dataScramblingIdentityPDSCH and nRNTI applicable to msgB PDSCH (and msg2 PDSCH). In Clause 7.3.1.1, TS 38.211, neither msgB-RNTI nor RA-RNTI is mentioned in the list of RNTIs when PDSCH (msg2 or msgB) is scheduled using DCI format 1_0 in a CSS. This will create ambiguity in the processing of msgB/msg2 in RACH. |
| CATT | Editorial | Y |  |
| SS | Editorial | Y(?) | Is this the issue #2 the same issue discussed last meeting? The common understanding was the same issue should be handled in Rel15 CR if needed, if I remember correctly. I wonder is there any progress on it. |
| Nokia | Editorial | Y(?) | As we read the contribution, the deployment scenario is a bit artificial, as the cell/BWP is configured with 2-step RACH only, meaning that all Rel-15 UEs are excluded from accessing the cell as well as all Rel-16 UEs that does not support 2-step RACH. We are not really sure that we need to account for this kind of mis-configuration. Probably OK to discuss the need, but our starting point is that we do not need this change. |
| Intel | Editorial | Y(?) | Share similar view as Nokia that we may need to first discuss the need for this configuration. If this is not needed, the CR is not necessary. |

Summary

The summary and scope for the potential email discussion will be updated later based on companies’ comments.

For issue#1, seems all the company are fine with the editorial change, so I think it can be proposed to the 211 editor to include it in the alignment CR directly.

For issue#2, based on the comments, it seems necessary to first clarify if the case that 4-step RACH is not configured for any of the BWPs exists or not for Rel-16. Not sure if this can be resolved in RAN1, so probably we can also ask for RAN2’s clarification together with the proposal 1 in Ericsson’s contribution (to mention 2-step RACH when describing p0-AlphaSets parameter in RRC specification).

Proposed outcome of the preparation phase:

- Endorse the draft CR in R1-2104474, and include it in the editors’ alignment CR;

- Discuss the potential LS to RAN2 on the description of RRC parameter p0-AlphaSets and whether 4-step RACH can be absent for any of the BWPs.

Any other comments?

| Company | Comment |
| --- | --- |

References

- R1-2104474 Correction on higher layer parameter for MsgA PUSCH DMRS		CATT

- R1-2105507 Discussion on corrections for 2-step RACH		Ericsson

Appendix

List of proposals in the submitted contributions.

| TDoc | Proposals |
| --- | --- |
| R1-2104474, CATT | Reason for change: Misalignment of higher-layer parameter name between 38.211 and 38.331. Summary of change: Alignment of higher-layer parameter name. Consequences if not approved: Misaligned parameter name. 6.4.1.1.1.1 Sequence generation when transform precoding is disabled If transform precoding for PUSCH is not enabled, the sequence shall be generated according to . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231 where is the OFDM symbol number within the slot, ns,fμ is the slot number within a frame, and - NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 or 0_2, or by a PUSCH transmission with a configured grant; - NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI; - NID0,NID1∈0,1,…,65535 are, for each msgA PUSCH configuration, given by the higher-layer parameters msgA-ScramblingID0 and msgA-ScramblingID1, respectively, in the msgA-DMRS-Config IE if provided and the PUSCH transmission is triggered by a Type-2 random access procedure as described in clause 8.1A of [5, TS 38.213]; - NIDnSCIDλ=NIDcell otherwise; < Unchanged parts are omitted > |
| R1-2105507, Ericsson | Observation 1 In case of non-standalone, rach-ConfigCommon is in dedicated RRC signaling and it’s required to be present since delta signaling is expected. Observation 2 In case of NR standalone, rach-ConfigCommon may be absent in SIB1 and not available according to the RRC signaling design, however it may have to be configured in Rel-15 since there’s only one random acces type. Observation 3 In NR Rel-16, 2-step RACH and 4-step RACH configurations are independent from each other, and rach-ConfigCommon may be not available when only 2-step RACH (i.e. msgA-ConfigCommon-r16) is configured. Proposal 1 Inform RAN2 that 2-step RACH was not considered when describing p0-AlphaSets parameter in RRC specification for Rel-16. Proposal 2 In case of 2-step RACH only operation, when p0-AlphaSets is not provided, for power control of normal PUSCH, alpha for msgA PUSCH is used, according to TP1, or alpha is fixed to be 1 according to TP2. Proposal 3 In case of 2-step RACH only operation, when transformPrecoder is not provided, waveform of normal PUSCH is determined based on msgA-transformPrecoder according to TP3 or is fixed to be CP-OFDM according to TP4. -------------------------------start of TP1 of 38.213 V16.5.0 -------------------------------------- 7.1.1 UE behaviour *** unchanged text omitted*** - For - For , - if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, is the value of msg3-Alpha - else, - For , is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a set of values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - For j=1 or j∈SJ, if P0-PUSCH-AlphaSet is not configured, the UE uses msg3-Alpha configured for msg3 PUSCH if a Type-1 random access is configured for the BWP or uses msgA-Alpha for msgA PUSCH if only a Type-2 random access procedure is configured for the BWP. *** unchanged text omitted*** -------------------------------- end of TP1 of 38.213 V16.5.0 ------------------------------------ ------------------------- start of TP2 of 38.213 V16.5.0 ----------------------------------- 7.1.1 UE behaviour *** unchanged text omitted*** - For - For , - if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, is the value of msg3-Alpha - else, - For , is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a set of values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - For j=1 or j∈SJ, if P0-PUSCH-AlphaSet is not configured, the UE uses msg3-Alpha configured for msg3 PUSCH if a Type-1 random access is configured for the BWP or UE assumes αb,f,cj=1 if only a Type-2 random access procedure is configured for the BWP. *** unchanged text omitted*** ------------------------------ end of TP2 of 38.213 V16.5.0 -------------------------------------- -------------------------------- start of TP3 of 38.214 V16.5.0 ---------------------------------- *** unchanged text omitted*** For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if only a Type-2 random access procedure is configured for the BWP. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if only a Type-2 random access procedure is configured for the BWP. For PUSCH transmission with a configured grant - If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if only a Type-2 random access procedure is configured for the BWP. *** unchanged text omitted*** ------------------------- end of TP3 of 38.214 V16.5.0 --------------------------------------- ------------------- start of TP4 of 38.214 V16.5.0 ------------------------------------- *** unchanged text omitted*** For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or consider the transform precoding always disabled if only a Type-2 random access procedure is configured for the BWP. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or consider the transform precoding always disabled if only a Type-2 random access procedure is configured for the BWP. For PUSCH transmission with a configured grant - If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or consider the transform precoding always disabled if only a Type-2 random access procedure is configured for the BWP. *** unchanged text omitted*** ------------------ end of TP4 of 38.214 V16.5.0 ----------------------------------- |