3GPP TSG RAN WG1 #105-e	R1-2105762

e-Meeting, May 10th – 27th, 2021

Source:	Lenovo, Motorola Mobility

Title:	CSI enhancements for multi-TRP and FDD reciprocity

Agenda Item:	8.1.4

Document for:	Discussion and Decision

# Introduction

In RAN1#104bis-e, several agreements were made for CSI enhancements under multi-TRP deployments, as well as CSI enhancements that exploit FDD channel reciprocity in FR1. For multi-TRP enhancements, agreements were made that refine the scope of enhancements for multi-TRP deployments, including CSI Report configuration, CSI-RS Resource grouping, and CSI report structure. For FDD reciprocity, companies have agreed on spatial aspects of the codebook structure. In this contribution we provide our views on different aspects of the CSI enhancements under multi-TRP/panel transmission, as well as on CSI enhancements under FR1 FDD reciprocity based on the class of Type-II Port Selection codebooks.

# CSI Reporting for DL multi-TRP/Panel Transmission

In RAN1#104bis-e [1], the following agreements were made for CSI enhancements under multi-TRP transmission:

Agreement Support the indication of following RI combinations by a joint RI field for a NCJT measurement hypothesis in CSI part 1, when the maximal transmission layers is less than or equal to 4: - {1, 1}, {1, 2}, {2,1}, {2,2} - FFS: CBSR and/or RI restrictions per TRP or across TRPs Agreement With regarding to the maximal values of Nmax for N, Ks,max for Ks: - Support of Nmax=2 is a UE optional feature - Support of Ks,max=X is a UE optional feature - X can be up to 8 and other candidate values can be discussed as part of UE features - FFS: Default value of Nmax, Ks,max - FFS: Which combinations of N<=Nmax, Ks<=Ks,max are supported Agreement With regarding to possible restriction between K1 and K2 - Alt 2: No restriction as long as K1+K2=Ks Agreement The UE may assume that QCL-Type D of CMRs associated with a NCJT measurement hypothesis are applied to the corresponding CSI-IM resource. Agreement For the UE be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses (i.e. Option 2), - Alt 1: Single CRI is reported whereas CRI bit size depends on total number of valid CMR pairs for NCJT measurement hypothesis and valid CMRs for single-TRP measurement hypotheses. - FFS further mapping mechanism between each CRI codepoint and Single-TRP/NCJT measurement hypothesis. Agreement A 2-part CSI report is supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis with following clarifications: - Within CSI part 1 - CRI, RI, WB CQI and SB CQI for the first CW are reported with consistent payload and zero padding (if needed). FFS further details - FFS whether RI can be shared between NCJT CSI and single-TRP CSIs to reduce CSI feedback overhead - FFS whether additional field is needed, at least for Option 2 - Within CSI part 2: - FFS further compression/omission/Sharing of PMI among Single-TRP and NCJT hypotheses Agreement Whether a NZP CSI-RS resource m can be referred by two CMR pairs (m, a) and (m, b) configured for NCJT measurement hypotheses, study following Alternatives and down-select one Alternative in RAN1#105-e: - Alt 1: It is feasible for FR1 but not for FR2. - Alt 2: It is feasible for both FR1 and FR2 but subject to further UE capability for FR2. Agreement Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis, study following Alternatives and down-select one Alternative in RAN1#105e: - Alt 2: It is feasible for FR1 but it is not for FR2. For FR2, the UE is expected to have different NZP CSI-RS resources configured for all CMRs of Single-TRP and NCJT measurement hypotheses respectively. - Alt 3: It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot. Agreement For the UE configured to report X CSIs (at least when X>0) associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis, study following issues for potential CSI omission/priority/updating rules: - Issue 1: Prioritize CSI with different measurement hypotheses within the single CSI report, when the UE is configured with CSI Option 1 with X=1 or 2. - Issue 2: Omission of NCJT CSI in CSI part 2 depending on the corresponding CRI or RI or CQI in CSI part 1. Agreement For the UE configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis (i.e. Option 1), - Alt 1: X+1 CRIs are reported, whereas X CRIs are for single-TRP measurement hypotheses and one CRI is for NCJT measurement hypothesis. Each CRI bit size depends on the corresponding number of either valid CMR pairs for NCJT measurement hypothesis or valid CMRs for single-TRP measurement hypotheses - FFS: Whether the X+1 CRIs are reported jointly as one CSI report or as separate CSI reports. Agreement For CSI measurement associated with a CSI-ReportConfig for NC-JT, study following aspects: - whether to support dynamic updating, e.g. by MAC-CE, for CMR pairs for NCJT measurement hypotheses, and/or CMRs for Single-TRP measurement hypotheses, and/or TCI states in CMRs, and/or the number of single-TRP CSIs (i.e. X=0/1/2) in a NCJT CSI report - whether additional high layer signalling is needed to configure M (M≤ Ks) CMRs from the CSI-RS resource set for CMR for Single-TRP measurement hypotheses - For CMRs configured in the CSI-RS resource set, whether support high layer signalling to enable/disable single-TRP measurement hypothesis using CMR configured within CMR pairs for NCJT measurement hypothesis Agreement Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM, study following Alternatives and down-select one Alternative in RAN1#105e: - Alt 1: Yes, it is supported, subject to limitations, e.g. N=1 CMR pair and Ks=2 CMR resources - Alt 2: No, it is not supported Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, an NCJT CSI hypothesis based on a pair of CMRs assumes to occupy two CPUs, two active NZP CSI-RS resources, and a number of active ports corresponding to both CMRs. - If a NZP CSI-RS resource is referred X times by CMR pairs for NCJT measurement hypothesis and CMR for Single-TRP measurement hypothesis, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times for active resources and active ports. - Note: For above CSI computation, UE assumes PDSCH transmission is single-DCI based multi-TRP scheme(s). FFS: Multi-DCI based multi-TRP scheme

## CSI-RS Resource Grouping

It was agreed in RAN1#104-e [2] that a UE can be configured with two CMR groups with Ks=K1+K2 CMRs for multi-TRP transmission, where a subset of the Ks CMRs can be used for single-TRP hypothesis. It was later agreed in RAN1#104bis-e that the number of CMRs per group, K1, K2 are selected freely as long as K1+K2=Ks. A few details still remain, including whether a CMR can be reused for more than one hypothesis, and the limits on the configured Ks,max, Nmax values, in addition to the CMR selection process for CSI reporting.

### CMR reuse across multiple hypotheses

It was discussed in prior RAN1 meetings whether a CMR corresponding to one TRP for NCJT transmission can be reused for single-TRP transmission from the same TRP. In RAN1#104bis-e, it was agreed that reusing a CMR associated with an NCJT hypothesis for a single-TRP hypothesis is allowed in FR1 without restriction, however for FR2 it was decided to down select in RAN1#105-e between restricting CMR reuse in FR2 or allowing CMR reuse in FR2 based on an optional UE capability. Our preference is the latter alternative: a CMR associated with NCJT hypothesis can be reused for a single-TRP hypothesis in FR2 based on a UE capability. In our understanding, the motivation to totally restrict NCJT CMR reuse for single-TRP transmission is not justified.

For FR2, a CMR associated with an NCJT hypothesis can be reused for a single-TRP hypothesis conditioned on an optional UE capability

- Another issue that was raised in RAN1#104bis-e is related to reusing the same CMR across two NCJT hypotheses, e.g., CMR1 corresponding to TRP1 can be coupled with either CMR2 or CMR3 for NCJT transmission of TRPs {1,2} or {1,3}, respectively. For FR2, some companies raised concerns that one-to-one mapping of CMR pairs across CMR groups is necessary for NCJT, otherwise it would cause ambiguity on UE side when coupling the receive beams under simultaneous panel reception. In our opinion, the restriction on CMR reuse across two NCJT hypotheses in FR2 may be needed under some UE implementations, however, it may not be necessary for other implementations, e.g., with UE panel selection. In RAN1#104bis-e, it was agreed that reusing a CMR across two NCJT hypotheses is allowed in FR1 without restriction, however, it was decided to down select in RAN1#105-e between restricting CMR reuse in FR2 or allowing CMR reuse in FR2 based on an optional UE capability. Our preference is the latter alternative: a CMR associated with NCJT hypothesis can be reused for another NCJT hypothesis in FR2 based on a UE capability.

For FR2, a CMR associated with an NCJT hypothesis can be reused for another NCJT hypothesis conditioned on an optional UE capability

### Bounds on Ks, N values

In addition, bounds on the total number of CMRs across CMR groups as well as the number of NCJT CMR pairs were also discussed to limit the UE complexity. In our opinion, the maximum value of Ks for a given UE should depend on the operating frequency range. For instance, since CMR reuse is allowed without restriction in FR1, the parameter pair (Ks, N) = (2,1) should be supported mandatorily for all UEs supporting multi-TRP transmission. For FR2, the parameter pair (Ks, N) = (2,1) should also be mandatory for UEs that allow reusing an NCJT CMR for single-TRP hypothesis. An additional pair value, (Ks, N) = (4,1) can also be made mandatory, at least for UEs that do not support reusing an NCJT CMR for a single-TRP hypothesis.

Support the pair (Ks, N) = (2,1) as mandatory parameter values for multi-TRP transmission. For UEs that do not support reusing an NCJT CMR for a single-TRP hypothesis, support the pair (Ks, N) = (4,1) as mandatory parameter values for FR2

- Other parameter pair values that can be considered for CMR grouping are as follows: (Ks, N) = {(3,1), (4,2), (6,2), (8,2)}, where the two pair values (Ks, N) = {(6,2), (8,2)} can be restricted to FR2. Further details, e.g., whether the parameter pair values are mandatory or optional, can be made FFS.

Study the following candidates for the parameter pair values (Ks, N) = {(3,1), (4,2), (6,2), (8,2)}, where the two pair values (Ks, N) = {(6,2), (8,2)} can be restricted to FR2

- Also, assume the following example: two CMR groups with K1=1 (corresponding to CMR0), K2=2 (corresponding to CMR1, CMR2) in FR1 in which reusing a CMR associated with NCJT hypothesis for either a single-TRP hypothesis or another NCJT hypothesis is allowed, i.e., a total of 3 single-TRP hypotheses and 2 NCJT hypotheses are possible. To the best of our knowledge, no restriction is imposed on this setup as of RAN1#104bis-e, at least for FR1. However, it is not clear what the associated value of the parameter N would be, since N is defined in pairs. Since it was agreed in RAN1#104bis-e that CMR reuse is not restricted in FR1, further clarification of the notation is needed for the aforementioned point.

The parameter pair (Ks, N) does not sufficiently characterize the CMR breakdown for a scenario with CMR reuse across more than one NCJT hypotheses, e.g., a case with K1=1, K2=2 where all CMR pairs corresponding to NCJT hypotheses are allowed

Modify the notation (Ks, N) to fit the scenario where a CMR is shared across more than one NCJT hypothesis

### Indication of selected CMRs for CSI Reporting

- One other aspect of the CMR grouping that is yet to be finalized is the mapping between the Ks CMRs associated with multi-TRP transmission and X+1 CSI Reports which the UE is configured to feed back to the network, where X=0,1,2. In RAN1#104bis-e, it was suggested to use dynamic signaling, e.g., MAC-CE signaling, to update the UE with the selected hypotheses for CSI reporting, as well as indicate the M CMRs (out of Ks) that can be used for single-TRP hypotheses. It is not clear to us why RRC signaling would not suffice to indicate the CMRs (and hence the hypotheses) of interest which the UE should report their corresponding CSI reports. In light of that, dynamic indication of the CMRs corresponding to the CSI reporting may not be necessary, at least as a mandatory feature.

No clear justification of using dynamic indication of the selected CMRs for CSI reporting

- In our opinion, the CMRs configured for multi-TRP transmission via the CSI Report Configuration should match the CMRs needed for CSI reporting computation, i.e., (Ks, N)= (2,1) for UEs capable of CMR reuse across NCJT and single-TRP hypotheses, and (Ks, N)= (4,1) for UEs without the capability of CMR reuse across NCJT and single-TRP hypotheses. For the latter case, whenever the UE is not capable of CMR reuse across single-TRP and NCJT hypotheses, the first CMR in each group, e.g., CMR corresponding to the lowest CSI-RS resource ID, can be assigned for single-TRP hypothesis. Dynamic signaling of the selected CMR can be limited to scenarios in which the number of configured CMRs in the CSI Report Configuration exceeds that of CMRs needed for CSI reporting, which can be made an optional UE capability. Further details can be made FFS.

At least as a mandatory feature of multi-TRP CSI Reporting, the number of configured CMRs in the CSI Report Configuration should match that of CMRs needed for CSI reporting

Study dynamic signaling of M selected CMRs for CSI Reporting out of the Ks configured CMRs as an optional UE feature. Further details are FFS

## CSI Reporting mechanism for NCJT

Different alternatives for CSI Reporting mechanism were proposed for NCJT, and a decision was reached in RAN1#104-e [2] to support two options, where the first option includes reporting one NCJT CSI Report and X single-TRP CSI reports (X=0,1,2), and the second option includes reporting one CSI report corresponding to the best hypothesis across single-TRP and NCJT hypotheses. Clearly, both options (as well as the sub-options under Option 1) vary in terms of UE complexity, CSI feedback overhead and network complexity. For instance, Option 1 with X=2 provides the best flexibility in terms of providing CSI feedback to all possible transmission hypotheses in case of two TRPs, however the overall CSI feedback has roughly triple the overhead size as conventional CSI feedback under single-TRP transmission. On the other hand, a UE under Option 2 would report CSI feedback for the best hypothesis among single-TRP and NCJT hypotheses, which requires roughly the same CSI overhead as Rel. 15/16 single-TRP transmission CSI reporting, at the expense of restricting the CSI feedback to one hypothesis only. One way to rationalize such large overhead is via reusing part of the CSI, e.g., PMI/RI for more than one hypothesis. An example of such algorithm is provided in Table 1 below, as follows.

Step 1: A UE is configured with two NZP CSI-RS Resources, CSI-RS0 and CSI-RS1 transmitted from TRP1 and TRP2, respectively Step 2: The UE computes PMI0, RI0 under NCJT hypothesis, where CSI-RS0 and CSI-RS1 are used for channel and interference measurements, respectively Step 3: The UE computes PMI1, RI1 under NCJT hypothesis, where CSI-RS1 and CSI-RS0 are used for channel and interference measurements, respectively Step 4: The UE computes CQI0 for single-TRP hypothesis from TRP1, based on PMI0, RI0 Step 5: The UE computes CQI1 for single-TRP hypothesis from TRP2, based on PMI1, RI1 Step 6: The UE computes CQI2 for NCJT hypothesis from TRP1 and TRP2, based on PMI0/RI0 and PMI1/RI1

Table 1: CSI computation for Option 1 with X=2 using two PMI quantities and three CQI quantities

From Table 1, the UE can compute CSI for three transmission hypotheses using two PMI quantities, where PMI0 and PMI1 for NCJT hypothesis that are associated with TRP1 and TRP2 are reused for single-TRP hypothesis with TRP1 and single-TRP hypothesis with TRP2, respectively. Although PMI0 and PMI1 are not optimized for single-TRP hypothesis, this approach would simultaneously reduce the CSI feedback overhead, and bring down the UE computational complexity since only 2 PMI and 2 RI values (rather than 4 PMI, 4 RI) would be computed, as well as provide enough flexibility to the network to support up to three transmission hypotheses. Note that the two reported CQI values CQI0, CQI1 are computed based on the two single-TRP hypotheses corresponding to TRP1 and TRP2, respectively, without taking the PMI/interference measurement from the other TRP into consideration. A similar mechanism can be adopted for Option 1 with X=1, where only 2 PMI, 2 RI and 2 CQI values are needed to support both NCJT and the single-TRP hypotheses. Further details on CSI feedback structure and CSI configuration signaling under the proposed approach can be discussed further.

CSI feedback overhead and UE complexity of Option 1 with X=1,2 can be reduced via adopting more concise CSI reporting approaches

Reuse PMI, RI across NCJT and single-TRP hypotheses for CSI reporting under Option 1 with X=1,2

## CSI Report Priority for multi-TRP Transmission

In Rel. 16 NR, CSI reports are associated with a priority value, where a lower CSI priority value indicates higher priority, as follows

PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s

s: CSI reporting configuration index, where Ms is the maximum number of CSI reporting configurations

c: Cell index, and Ncells is the number of serving cells

k: 0 for CSI reports carrying L1-RSRP or L1-SINR, and 1 otherwise

y: 0 for aperiodic reports, 1 for semi-persistent reports on PUSCH, 2 for semi-persistent reports on PUCCH, 3 for periodic reports

Obviously, the Rel. 16 CSI report priority function does not take into consideration the case where multiple CSI reports are associated with the same CSI reporting configuration index, which may require updating the CSI report priority function to include a parameter that represents the CSI report index within a CSI reporting configuration.

The Rel. 15/16 CSI report priority function is not compatible with the multi-TRP CSI framework where multiple CSI reports can be associated with the same CSI reporting configuration

A new CSI report priority function for Rel. 17 is needed to support multi-TRP CSI reporting framework with multiple CSI reports per CSI reporting configuration

- Note that up to four possible cases for CSI Reporting under multi-TRP CSI framework exist, as follows

- (i)   Option 1 with X=0: UE reports one NCJT CSI report

- (ii)  Option 1 with X=1: UE reports one NCJT CSI report + one single-TRP CSI report

- (iii) Option 1 with X=2: UE reports one NCJT CSI report + two single-TRP CSI reports

- (iv) Option 2: UE reports one CSI report corresponding to best hypothesis among NCJT and single-TRP hypotheses

- As shown above, both the first and fourth cases correspond to feeding back a single CSI report, and hence no update to the CSI report priority  function is needed. For the second setup with X=1, i.e., one single-TRP CSI report and one NCJT CSI report are fed back, the CSI report corresponding to single-TRP transmission can be given a higher priority. Finally, for the case where X=2 with two single-TRP CSI reports and one NJCT CSI report, the two CSI reports corresponding to single-TRP transmission can be given higher priority compared with NCJT CSI report, however, ties must be broken between the two single-TRP CSI reports, e.g., based on the CSI-RS resource ID corresponding to the CMR used for single-TRP hypothesis. Further details are FFS.

A CSI report corresponding to single-TRP hypothesis has higher priority compared with a CSI report corresponding to NCJT hypothesis

For a CSI reporting configuration with X=2 single-TRP CSI reports, ties must be broken between the two single-TRP CSI reports to prioritize one CSI report over the other, e.g., based on associated CSI-RS Resource ID

## CSI Omission for multi-TRP based CSI Reporting

In RAN1#104bis-e, it was proposed by some companies to study whether omission of NCJT CSI in CSI part 2 depending on the corresponding CRI, RI or CQI in CSI part 1 is needed. It is reasonable to associate more CSI feedback bits to CSI Reports corresponding to higher CQI values, however it is not clear to us how such omission can be based on the CRI or RI reported in CSI Part 1.

The motivation behind CSI omission based on RI or CRI values is not clear

- Considering the case where a CSI report is omitted based on CQI, a few details need to be settled. First, it is more convenient to make the CSI omission decision based on the wideband CQI value reported for each CSI report. Also, it should be clarified whether Part 2 CSI omission implies fully omitting CSI Part 2 or only a portion of CSI Part 2. Finally, the rule on which CSI omission is based upon should be clarified, e.g., the CSI report with the lowest CQI value in CSI Part 2.

For CSI omission based on reported CQI value, study the following aspects

- Whether Part 2 CSI is fully or partially omitted

- The rule which identifies the CSI report(s) whose Part 2 CSI is omitted

## CBSR and RI Restriction for multi-TRP based CSI Reporting

In RAN1#104bis-e, it was agreed to support the following RI pairs for NCJT: {1, 1}, {1, 2}, {2,1}, {2,2}, where the total number of layers is limited to 4, such that the difference between the number of layers corresponding to the two NCJT PMIs is no larger than one. It was also decided to further study whether CBSR and RI restriction should be configured jointly or separately for each UE. In our opinion, RI restriction should follow legacy definitions, where the RI restriction is applied to the total number of layers from both TRPs/Panels, if applicable.

Reuse legacy RI restriction format for NCJT, such that an RI restriction applies to the total number of layers transmitted from the TRP(s) for a given transmission hypothesis

- For CBSR under NCJT, the configuration should also follow legacy definition with a single CBSR configuration for all TRPs/Panels. Note that CBSR is needed to mitigate inter-cell interference, and hence the restriction is purely beam-based, and does not depend on the corresponding channels of the TRP(s) with the restricted UE.

Reuse legacy CBSR format for NCJT, such that a CBSR applies to all codebooks corresponding to a given CSI reporting configuration

## Interference measurement for multi-TRP based CSI Reporting

- It was discussed in RAN1#104bis-e whether  there is need to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis. In our opinion, this is not needed, since under NCJT hypothesis the UE can use the CMR corresponding to the second TRP for interference measurement when designing the codebook feedback parameters corresponding to the layers of the first TRP, and/or use the CMR corresponding to the first TRP for interference measurement when designing the codebook feedback parameters corresponding to the layers of the second TRP. Whether this implementation requires specification impact can be FFS.

Inter-TRP interference measurement for NCJT can be done with the CMR pair configured for NCJT measurement hypothesis via UE implementation

At least for single-DCI case, do not support inter-TRP interference measurement for NCJT based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis

- Note that the prior proposal does not preclude configuring the UE with an NZP CSI-RS resource for interference measurement, as long as this NZP CSI-RS resource corresponds to a TRP that is not a candidate for NCJT transmission for this particular UE.

Configuring the UE with an NZP CSI-RS for interference measurement transmitted from a TRP outside the set of candidate TRPs for NCJT is not precluded

## Support for multi-DCI based NCJT

It was decided in RAN1#104-e [2] to defer the down selection between explicit CSI reporting configuration (Option 1) and implicit CSI reporting configuration (Option 2) for multi-DCI based NCJT to RAN1#105-e meeting. In principle, NCJT can operate under systems with either ideal or non-ideal backhaul between TRPs, where scenarios with non-ideal backhaul are usually restricted to multi-DCI based multi-TRP transmission. Moreover, two alternatives were proposed in RAN1#104-e meeting, which discuss whether explicit, i.e., TRP specific, CSI Reporting Configuration or implicit, i.e., joint, CSI reporting configuration is adopted for multi-DCI based NCJT. Since the resources used by the TRPs under multi-DCI based multi-TRP may be either fully overlapping, or partially overlapping or non-overlapping, NCJT-based CSI reporting may not be always needed, e.g., under non-overlapping resources CSI reports can be computed independently.

Multi-DCI based multi-TRP may correspond to transmission scenarios where both TRPs use non-overlapping resources for transmission, and hence Rel. 15/16 CSI reporting configuration per TRP suffices

Also, one other important aspect of the problem is that CSI reporting and PDSCH transmission are separately triggered, and hence it is not always clear whether/how the appropriate PDSCH transmission and CSI reporting modes are seamlessly matched. Since two codewords are transmitted under multi-DCI based NCJT, two CQI values should be reported within the CSI report corresponding to each codeword, compared with one CQI value corresponding to a single codeword under single-DCI based NCJT.

Two CQI values should be reported within a multi-DCI based NCJT report, compared with one CQI value for a single-DCI NCJT report

One solution to address the aforementioned points is as follows. First, the CSI Reporting Configuration can indicate whether a single-DCI based NCJT report (with a single CQI value), or a multi-DCI based NCJT report (with two CQI values) is triggered. It can be left to network implementation whether multi-DCI based NCJT reporting is triggered when the network is planning to use overlapping resources for NCJT transmission. Another alternative that provides a unified solution would be triggering the UE with up to three CQI values, one CQI value for a single-DCI based NCJT codeword, and the other two  CQI values for the two multi-DCI based NCJT codewords. Note that the same RI, PMI values can be shared for both setups. A third alternative would be triggering multi-DCI based CSI Reporting with aperiodic reporting only. Finally, since single-DCI based multi-TRP CSI reporting is restricted to RI≤4, having an RI restriction that allows RI>4 implies supporting multi-DCI based multi-TRP CSI reporting. In our opinion, RAN1 should strive to support a unified CSI reporting framework for both single-DCI and multi-DCI based multi-TRP transmission. Thereby, we have the following proposal.

Under implicit CSI reporting for multi-DCI based NCJT, select one or more of the following alternatives

- Alt1: the CSI reporting configuration triggers the UE with either a single-DCI or a multi-DCI based NCJT CSI reporting

- Alt2: A CSI reporting configuration triggers the UE with an NCJT CSI report that includes up to three CQI values (one for single-DCI based NCJT and two for multi-DCI based NCJT)

- Alt3: A CSI reporting configuration corresponding to multi-DCI based NCJT reporting is triggered with aperiodic reporting only, where the multi-DCI based NCJT CSI report includes two CQI values, corresponding to two codeword transmissions

- Alt4: A CSI reporting configuration corresponding to multi-DCI based NCJT reporting is inferred from RI restriction, i.e., whether RI>4 is restricted

- Note: RAN1 should strive to support a unified CSI reporting framework for both single-DCI and multi-DCI based multi-TRP transmission

# Type-II Port Selection Codebook Enhancement

In RAN1#104bis-e [2], the following agreements were made for port-selection codebook enhancements under FDD channel reciprocity in FR1:

Agreement For rank=1, polarization-common based free-selection should be supported for W1W1. - FFS: Whether there is a need to restrict the number of CSI-RS ports for which this is supported Agreement At least for rank 1, combinatorial coefficient is used for port selection for W1. - FFS when Wf is turned off Agreement Confirm following working assumption of Wf for R17 PS CB - Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters. Agreement At least for rank 1, regarding the value(s) of K1 for port selection matrix W1 in NP*K1, study and down-select from the following candidate values of K1 and the maximal value of P in RAN1 105e - K1 in {2,4,8,12,16,24,32} with K1 <= P - The maximal value of P as Pmax, e.g. 32 - FFS: possible parameter combinations/dependence for K1 with other PS CB parameters, e.g. whether different candidate values of K1 should be configured for different ranks (if rank>1 is supported). - FFS: Whether any value of K1 up to P can be supported for some codebook parameters - Note: for Polarization-common based free-selection, it means to select the same L=K1/2 ports out of P/2 ports for both polarizations. Note: for polarization-specific based free-selection, it means select K1 ports out of P ports Note: P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study) Agreement A bitmap for indication non-zero coefficients should be supported for W2 with a compression coefficient beta<=1 whereas - FFS values of beta < =1, e.g. 1/8, 1/4, 1/2, 3/4, 1 - FFS: whether/how such a bitmap can be absent for specific codebook configuration parameters - FFS: whether a bitmap is polarization-common or polarization-specific whereas polarization-specific bitmap is the baseline - FFS: possible parameter combinations/dependence for beta with other PS CB parameters Agreement At least for rank 1, the FD bases used for Wf quantitation are limited within a single window/set with size N configured to the UE, study and down-select one Alternative in RAN1 105e: - Alt 1: FD bases in the window must be consecutive from an orthogonal DFT matrix - Alt 2: FD bases in the set can be consecutive/non-consecutive, and are selected freely by gNB from an orthogonal DFT matrix - FFS: Applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv - FFS: Whether this applies when Wf is turned OFF Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/not supporting additional alternatives for higher rank. Agreement At least for rank 1, for relationship between N and Mv, study and down-select one Alternative from following in RAN1#105e - Alt 1: N= Mv always - Alt 2: N >= Mv and FFS candidate value(s) of N, e.g. 2, 4 - FFS: applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv - FFS: Whether this applies when Wf is turned OFF Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/ not supporting additional alternatives for higher rank. Agreement At least for rank 1, regarding the value(s) of R for Rel-17 PS codebook enhancement, study and down-select one or more than one Alternative (or a subset of corresponding values) in RAN1 105e: - Alt 0: R < 1 (e.g. 1/4, 1/2) - Alt 1: R=1 - Alt 2: R=1 and 2 - Alt 3: R=1,2, 4, and 8 - Alt 4: R= {1,2,…, D*NPRBSB} whereas D is the density of CSI-RS in frequency domain - FFS: applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv - FFS: Whether this applies when Wf is turned OFF Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/not supporting additional alternatives for higher rank. Agreement For the quantization of W2 coefficient, study following Alternatives with Alt 1 as the baseline: - Alt1: Reusing Rel-16 quantization mechanism for Rank 1 at least, which can be summarized as following: - An indicator for the strongest coefficient - Two polarization-specific reference amplitudes: - for the polarization associated with the strongest coefficient, the reference amplitude is not reported - for the other polarization, reference amplitude is quantized to 4 bits - For coefficients other than the strongest coefficient - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits - phase is quantized to 16PSK - Alt1-1: the ref amplitude = 0 reserved in R16 can be replaced with a new value, e.g. (1/2)^(1/8), (1/2)^(3/8) - Alt2-0: Individual amplitude (e.g. 3 or 4 bits with Rel15/16 amplitude codebooks) and phase (e.g. 16PSK) quantization - FFS: amplitude codebook is uniform in db or linear scale - FFS: support a strongest coefficient indicator, and individual quantization for other non-zero coefficients. - Alt2-1: ref amp (e.g. 4 bits), Individual amplitude (e.g. 3 bits) and phase (e.g. 16PSK) quantization for each non-zero coefficient - FFS: amplitude codebook is uniform in db or linear scale - FFS: reference amplitude is polarization specific or polarization common, and corresponding codebook - Note: Other quantization schemes or enhancement on top of Alt 1 or Alt 2 are not precluded. Agreement For PS codebook enhancements utilizing DL/UL reciprocity of angle and/or delay, down-select ONE option for CSI-RS configurations associated with Rel-17 PS codebook, from Option 0 (No further enhancement), Option 1 (i.e. lower CSI-RS density) and Option 3 (i.e. configuring multiple CSI-RS resources) - If there is no consensus in RAN1#105e, Option 0 is by default.

## Reciprocity-based codebook structure

In RAN1#104-e [2], it was agreed that the Rel. 17 port-selection codebook would follow a similar structure to that of Rel. 16 Type-II port-selection codebook, with a DFT-based frequency compression matrix Wf. Note that when M=1, i.e., the frequency compression matrix Wf collapses to a scaled all-ones vector of length N3. Hence, including special considerations for the case where M=1, e.g., turning off Wf by gNB, is redundant and may be confusing. In light of that, the current format of the codebook structure (which includes the DFT-based frequency compression matrix Wf) suffices to represent the codebook for a generic value of M≥1, and hence no further discussion is necessary.

The Rel. 17 codebook structure W=W1W2WfH agreed in RAN1#104-e is generic and can support different frequency compression parameter values M≥1

No further modification is necessary on the Rel. 17 codebook structure

FDD channel reciprocity can be exploited to optimize both the spatial and frequency beamforming of the CSI-RS, such that the channel corresponding to the beamformed CSI-RSs has a flat channel response, i.e., the channel would be characterized with a single dominant tap in the delay domain for each beamformed spatial dimension. In such case, a Rel. 16 Type-II like codebook with M=1 (which resembles a wideband codebook) would realize most the performance of  sub-band based CSI reporting. In more detail, starting from a Rel. 16 Type-II port selection codebook, the design of the CSI-RS beamforming matrix G(n) is transparent. Assume a simplified model with a single antenna port at the UE and a ULA of Mt antennas with spacing d at the gNB, and wavelength λ, the UL and DL channel models with duplexing distance of ∆F for sub-band n can be rewritten as

- hmULn=p=0P-1gpULej2πn∆fτp + j2πmdλsinθp

- hmDLn=p=0P-1gpDLej2πn∆fτp + j2πmdλsinθp

where P is the number of paths,  gpUL, gpDL are the complex gains for path p in UL and DL channels, respectively, and m is the gNB antenna port index. τp, θp are the delay and AoA of path p, respectively, and ∆f is the sub-band spacing. In light of this model, the gNB would obtain the delays and angles of arrival of each path and can use it in CSI-RS beamforming matrix. One possible design of the CSI-RS beamforming matrix is as follows

G(n)=e-j2πΔF+nΔfτk + mFc+Δfdcsinθkm+1,k+1

For K≥P, such design would enable steering each CSI-RS port towards a given path. The received CSI-RS symbol sk corresponding to a simplified noiseless channel would then be as follows

ykDLn=1Mtm=0Mt-1hmDLnGm,k(n)sk= gkDLsk+ 1Mtskm=0Mt-1 p=0,p≠kP-1gpDLej2πΔF+nΔf(τp -τk)+ mdλ(sinθp-sinθk)

The received signal can then be averaged (across sub-bands) as follows

ykDL=1N3 n=0N3-1ykDL(n)=gkDLsk+zk ,

and

zk≜1Mt.N3skn=0N3-1m=0Mt-1p=0,p≠kP-1gpDLej2πΔF+nΔf(τp -τk)+ mdλ(sinθp-sinθk).

Note that  zk, denoted here by reciprocity-based error term, would vanish for richly scattered environments, i.e., large P. Additionally, the larger the values of Mt, N3, the smaller the expected value of zk. The UE can then estimate gpDL and report it to the network as part of the CSI feedback report, and hence construct precoder coefficients that match the beamformed downlink channel, given its knowledge of τp, θp that is obtained from the uplink/downlink channel reciprocity. For such scenario, reporting one amplitude and one phase value corresponding to each CSI-RS port suffices. The codebook design that corresponds to the aforementioned analysis can be realized with a slightly modified Rel. 16 port selection codebook with M=1, i.e., with wideband PMI reporting. Note that zk may not vanish for all channel conditions, which would in turn require reporting more than one magnitude/phase coefficient value per port, i.e., M >1. Based on the analysis of the enhanced Type-II codebooks of Rel. 16, significant performance gains were achieved for M >2, e.g., M =4, without the need to exploit UL/DL channel reciprocity. Hence, the motivation to support uplink/downlink channel-reciprocity based codebook for M >2 may not be justified. Thereby, we support M =1,2 for frequency compression of Rel. 17 Type-II port-selection codebook.

Support M=1, 2 for frequency compression parameter values for Rel. 17 Type-II port-selection codebook

It is worth mentioning that the previous derivation confirms the conjecture pointed out by some companies that the number of CSI-RS ports supported for M=2 can be less than that supported for M=1, since the reciprocity-based error term zk was shown to diminish as the number of ports Mt increases. However, in real-world scenarios, the channel reciprocity mismatch is expected to be even larger, since the downlink delays and angles of arrival are expected to slightly deviate from their uplink counterparts. For instance, when the uplink and downlink angles of arrivals are not identical, i.e., τkUL≠τkDL and θkUL≠θkDL, the modified reciprocity-based error term, zk', would become approximately as follows

zk'≈ zk+1Mt.N3skn=0N3-1m=0Mt-1gpDLej2πΔF+nΔf(τkDL-τkUL)+ mdλ(sinθkDL-sinθkUL).

In light of that, considering a large number of CSI-RS ports for the case with M=2 is preferrable, so as to ensure the robustness of the Rel. 17 port-selection codebook in real world scenarios, where the path delays and angles are not expected to be perfectly aligned in both uplink and downlink channels, as emphasized in the second term of the modified reciprocity-based error term zk'.

When the number of FD basis indices M exceeds one, the reciprocity-based error term is expected to diminish with the number of CSI-RS ports

The reciprocity-based error term would increase when taking channel imperfections in real-world scenarios into account, e.g., mismatch in path delays and angles of arrival values between uplink and downlink channels

Do not further restrict the supported number of CSI-RS ports for the Rel. 17 port-selection codebook configured with M=2

Regarding the mechanism for configuring and/or indicating the FD basis indices for the Rel. 17 Type-II port-selection codebook, we believe a similar framework to that of Rel. 16 Type-II codebook for the case with N3>19 should be utilized, where a window-based approach is utilized with a layer-common window that indicates the set of contiguous FD basis indices from which the FD basis indices for all layers are selected. The size (N’) and location (Minitial) of the window can be higher-layer configured.

A layer-common, window-based approach is used to configure the FD basis indices for all layers, where the set of FD basis indices is contiguous, and whose location and size are higher-layer configured

For the layer-specific FD basis indices, our preference is UE-based selection of the M FD basis indices out of the N’ window-based FD basis indices. The motivation behind our preference for UE-based selection of the layer-specific FD basis indices over network selection is that the FD basis selection in one layer depends on the codebook parameters corresponding to prior layers, i.e., for a precoder whose precoding vectors for different layer are quasi-orthogonal, codebook parameters of layer l depend on the parameters selected for layers l-1, l-2, …, 1, and hence should be selected by the UE, since the UE selects the codebook parameters of each layer based on the CSI configuration.

For a precoder whose per-layer precoders are quasi-orthogonal, since the codebook parameters of a given layer depend on the codebook parameters of the prior layers, the FD basis indices selection should be carried out by the UE

Support UE-based FD basis indices reporting for each layer from a layer-common network-configured window

## CSI-RS configuration enhancements

In RAN1#104-e [2], it was agreed to study different options of CSI-RS configuration associated with Rel. 17 port-selection codebook to ensure low CSI-RS overhead and CSI-RS processing complexity at the UE. While the CSI-RS configuration is a fundamental aspect of the CSI framework, we believe that studying the CSI-RS configuration enhancements is out of the scope of this work item. The scope of this work item in [3] is described as follows: “Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead”. In light of that, we believe the scope of the enhancements for this work item is limited to a new codebook configuration based on the class of Type-II port-selection codebooks. Additionally, associating a specific CSI-RS design to a given codebook may not be a good practice and may lead to restricting the network flexibility in selecting that codebook. Thereby, we prefer Option 0: No CSI-RS enhancements should be supported.

CSI-RS configuration enhancements is out of the scope of this work item and should not be discussed

Support Option 0: No further CSI-RS enhancement as the baseline

## Quantization scheme of codebook coefficients

- In RAN1#104bis-e [1], the quantization scheme for linear combination coefficients of Rel. 17 reciprocity-based codebook was discussed.  Two alternatives were proposed

- Alt1: Reuse the quantization scheme for Rel. 16 Type-II codebook with a polarization-common amplitude coefficient, a differential amplitude coefficient per codebook entry and a phase coefficient per codebook entry

- Alt2: Use an amplitude coefficient per codebook entry and a phase coefficient per codebook entry

- In our opinion, Alt1 is more helpful for the case of a large number of coefficients per polarization, which can be helpful in the case of large number of spatial and/or transformed frequency dimensions, i.e., large L, M values. Since the reciprocity-based codebook is expected to adopt a small number of spatial/frequency dimensions, e.g., L≤4, M≤2, the differences between both alternatives is expected to diminish. However, we do not believe either alternative would have a significant impact on performance overhead trade-off.  In light of that we support reusing Rel. 16 quantization scheme for the reciprocity-based codebook.

Reuse the Rel-16 coefficient quantization approach for the amplitudes and phases of the Rel. 17 port-selection codebook

- FFS: whether modified amplitude quantization values and/or bitwidth are considered

## Bitmap reporting for reciprocity-based codebook

In RAN1#104bis-e, using a bitmap was discussed for Rel. 17 reciprocity-based codebook similar to Rel. 16 Type-II codebook. In general, a bitmap can help reduce the CSI feedback overhead in scenarios where the UE quantizes a fraction of a large number of coefficient available for the codebook. However, when either the number of codebook coefficients diminishes, or the number of quantized coefficients approaches the total number of coefficients, the advantages of using a bitmap would diminish, and the overhead incurred by including the bitmap can dominate the CSI feedback overhead saving resulting from it. In light of that, we propose that a bitmap is not supported for specific codebook configurations where most coefficients are quantized, i.e., β=1, or the number of coefficients is small, e.g., M=1. Further details can be left FFS.

The advantages of using a bitmap diminishes when either the number of codebook coefficients diminishes, or the number of quantized coefficients approaches the total number of coefficients

A bitmap is not reported for Rel. 17 port selection codebook when most coefficients are quantized, i.e., β=1, or the number of coefficients is small, e.g., M=1

## Polarization-common vs. polarization-specific port selection

During the Rel. 16 eType-II codebook specification it was discussed whether the beam selection process at the UE should be polarization common or polarization specific. It was shown that the gains achieved due to polarization-specific beam selection is negligible, whereas reporting polarization-specific amplitude/phase values for the linear combination coefficients achieves notable gains. It is not clear why Rel. 17 codebook should deviate from that design. Thereby, the port selection and coefficient quantization processes should follow in the footsteps of Rel. 15 and Rel. 16 Type-II Port Selection Codebooks.

Based on analysis of Rel. 15/16 Type-II codebooks, using polarization-common spatial domain basis transformation and polarization-specific quantized linear-combination coefficients provides efficient performance compared with other alternatives related to polarization common/specific codebook design parameters

Polarization-common port selection and polarization-specific coefficient quantization should be supported for Rel. 17 Reciprocity-based Port Selection codebook

## Time-domain behavior of CSI Reporting for Rel. 17 Type-II codebook

Previously, CSI reporting corresponding to Rel. 15 and Rel. 16 Type-II codebooks was only allowed on PUSCH, with the exception of supporting semi-persistent on PUCCH reporting of Part 1 only of Type-II codebook. In our understanding, the motivation behind restricting PUCCH reporting of CSI corresponding to legacy Type-II codebook is due to the substantial CSI feedback overhead of Type-II codebooks. However, Rel. 17 Type-II port selection codebook seems to deviate from this norm due to its potentially lower CSI feedback overhead, thanks to exploiting the uplink-downlink channel reciprocity. For example, for a port-selection codebook with L=2, M=1, β=1/2, the UE would report up to 8 linear combination coefficients for RI=2 with 48 bits corresponding to coefficients quantization, indication of the strongest coefficients and the bitmap. In light of that, supporting CSI feedback of Rel. 17 port-selection codebook should be studied, at least for codebook parameter combinations that yield substantially low CSI feedback overhead.

Study PUCCH reporting of CSI feedback corresponding to Rel. 17 port-selection codebook for some codebook parameter combinations that yield low CSI feedback overhead

- FFS: codebook parameter combination values that support PUCCH reporting

# Conclusion

This contribution addressed CSI enhancements for NR Rel. 17, including enhancements for NCJT as well as CSI enhancements under FDD channel reciprocity in FR1.

For CSI enhancements for NCJT multi-TRP, we have the following observations:

- The parameter pair (Ks, N) does not sufficiently characterize the CMR breakdown for a scenario with CMR reuse across more than one NCJT hypotheses, e.g., a case with K1=1, K2=2 where all CMR pairs corresponding to NCJT hypotheses are allowed

- No clear justification of using dynamic indication of the selected CMRs for CSI reporting

- CSI feedback overhead and UE complexity of Option 1 with X=1,2 can be reduced via adopting more concise CSI reporting approaches

- The Rel. 15/16 CSI report priority function is not compatible with the multi-TRP CSI framework where multiple CSI reports can be associated with the same CSI reporting configuration

- A new CSI report priority function for Rel. 17 is needed to support multi-TRP CSI reporting framework with multiple CSI reports per CSI reporting configuration

- The motivation behind CSI omission based on RI or CRI values is not clear

- Inter-TRP interference measurement for NCJT can be done with the CMR pair configured for NCJT measurement hypothesis via UE implementation

- Multi-DCI based multi-TRP may correspond to transmission scenarios where both TRPs use non-overlapping resources for transmission, and hence Rel. 15/16 CSI reporting configuration per TRP suffices

- Two CQI values should be reported within a multi-DCI based NCJT report, compared with one CQI value for a single-DCI NCJT report

Based on the observations above, we have reached the following conclusions for CSI enhancements under NCJT:

- For FR2, a CMR associated with an NCJT hypothesis can be reused for a single-TRP hypothesis conditioned on an optional UE capability

- For FR2, a CMR associated with an NCJT hypothesis can be reused for another NCJT hypothesis conditioned on an optional UE capability

- Support the pair (Ks, N) = (2,1) as mandatory parameter values for multi-TRP transmission. For UEs that do not support reusing an NCJT CMR for a single-TRP hypothesis, support the pair (Ks, N) = (4,1) as mandatory parameter values for FR2

- Study the following candidates for the parameter pair values (Ks, N) = {(3,1), (4,2), (6,2), (8,2)}, where the two pair values (Ks, N) = {(6,2), (8,2)} can be restricted to FR2

- Modify the notation (Ks, N) to fit the scenario where a CMR is shared across more than one NCJT hypothesis

- At least as a mandatory feature of multi-TRP CSI Reporting, the number of configured CMRs in the CSI Report Configuration should match that of CMRs needed for CSI reporting

- Study dynamic signaling of M selected CMRs for CSI Reporting out of the Ks configured CMRs as an optional UE feature. Further details are FFS

- Reuse PMI, RI across NCJT and single-TRP hypotheses for CSI reporting under Option 1 with X=1,2

- A CSI report corresponding to single-TRP hypothesis has higher priority compared with a CSI report corresponding to NCJT hypothesis

- For a CSI reporting configuration with X=2 single-TRP CSI reports, ties must be broken between the two single-TRP CSI reports to prioritize one CSI report over the other, e.g., based on associated CSI-RS Resource ID

For CSI omission based on reported CQI value, study the following aspects

- Whether Part 2 CSI is fully or partially omitted

- The rule which identifies the CSI report(s) whose Part 2 CSI is omitted

- Reuse legacy RI restriction format for NCJT, such that an RI restriction applies to the total number of layers transmitted from the TRP(s) for a given transmission hypothesis

- Reuse legacy CBSR format for NCJT, such that a CBSR applies to all codebooks corresponding to a given CSI reporting configuration

- At least for single-DCI case, do not support inter-TRP interference measurement for NCJT based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis

- Configuring the UE with an NZP CSI-RS for interference measurement transmitted from a TRP outside the set of candidate TRPs for NCJT is not precluded

Under implicit CSI reporting for multi-DCI based NCJT, select one or more of the following alternatives

- Alt1: the CSI reporting configuration triggers the UE with either a single-DCI or a multi-DCI based NCJT CSI reporting

- Alt2: A CSI reporting configuration triggers the UE with an NCJT CSI report that includes up to three CQI values (one for single-DCI based NCJT and two for multi-DCI based NCJT)

- Alt3: A CSI reporting configuration corresponding to multi-DCI based NCJT reporting is triggered with aperiodic reporting only, where the multi-DCI based NCJT CSI report includes two CQI values, corresponding to two codeword transmissions

- Alt4: A CSI reporting configuration corresponding to multi-DCI based NCJT reporting is inferred from RI restriction, i.e., whether RI>4 is restricted

- Note: RAN1 should strive to support a unified CSI reporting framework for both single-DCI and multi-DCI based multi-TRP transmission

- For CSI enhancements under FDD channel reciprocity in FR1, we have the following observations:

- The Rel. 17 codebook structure W=W1W2WfH agreed in RAN1#104-e is generic and can support different frequency compression parameter values M≥1

- When the number of FD basis indices M exceeds one, the reciprocity-based error term is expected to diminish with the number of CSI-RS ports

- The reciprocity-based error term would increase when taking channel imperfections in real-world scenarios into account, e.g., mismatch in path delays and angles of arrival values between uplink and downlink channels

- For a precoder whose per-layer precoders are quasi-orthogonal, since the codebook parameters of a given layer depend on the codebook parameters of the prior layers, the FD basis indices selection should be carried out by the UE

- CSI-RS configuration enhancements is out of the scope of this work item and should not be discussed

- The advantages of using a bitmap diminishes when either the number of codebook coefficients diminishes, or the number of quantized coefficients approaches the total number of coefficients

- Based on analysis of Rel. 15/16 Type-II codebooks, using polarization-common spatial domain basis transformation and polarization-specific quantized linear-combination coefficients provides efficient performance compared with other alternatives related to polarization common/specific codebook design parameters

- Based on the observations above, we have reached the following conclusions for CSI enhancements under FDD channel reciprocity in FR1:

- No further modification is necessary on the Rel. 17 codebook structure

- Support M=1, 2 for frequency compression parameter values for Rel. 17 Type-II port-selection codebook

- Do not further restrict the supported number of CSI-RS ports for the Rel. 17 port-selection codebook configured with M=2

- A layer-common, window-based approach is used to configure the FD basis indices for all layers, where the set of FD basis indices is contiguous, and whose location and size are higher-layer configured

- Support UE-assisted FD basis indices selection for each layer from a layer-common network-configured window

- Support Option 0: No further CSI-RS enhancement as the baseline

- Reuse the Rel-16 coefficient quantization approach for the amplitudes and phases of the Rel. 17 port-selection codebook

- FFS: whether modified amplitude quantization values and/or bitwidth are considered

- A bitmap is not reported for Rel. 17 port selection codebook when most coefficients are quantized, i.e., β=1, or the number of coefficients is small, e.g., M=1

- Polarization-common port selection and polarization-specific coefficient quantization should be supported for Rel. 17 Reciprocity-based Port Selection codebook

- Study PUCCH reporting of CSI feedback corresponding to Rel. 17 port-selection codebook for some codebook parameter combinations that yield low CSI feedback overhead

- FFS: codebook parameter combination values that support PUCCH reporting

# References

| [1] | 3GPP TSG RAN WG1 #104bis-e, "Draft Meeting Report," E-meeting, Apr. 12-20, 2021. |
| --- | --- |
| [2] | 3GPP TSG RAN WG1 #104-e, "Draft Meeting Report," Jan. 25- Feb. 5, 2021. |
| [3] | RP-193133, Samsung, "Rel. 17 enhancements on MIMO for NR," Sitges, Spain, Dec. 9-12, 2019. |