3GPP TSG RAN WG1 #105-e		R1-2105773

e-meeting, May 10th – 27th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Enhancements on PUSCH repetition type A

Agenda Item:   8.8.1.1

Document for:	Discussion

# 1	Introduction

In RAN#91-e, revised work item for NR coverage enhancements in Rel-17 has been approved with following objectives for in RP-210855 [1]:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

In this contribution, we provide our views on following enhancements needed for PUSCH repetition type A for improved coverage:

- Determination of available slots

- Postponement mechanism

- Maximum number of repetitions

- PUSCH repetition mode configuration/indication

# 2	Determination of available slots

One of the PUSCH repetition type A mode that has been agreed is to count the number of repetitions based on available slots for PUSCH transmission. The key aspect related to this mode is how to determine if the slot is counted as an available slot.  In RAN1#104-e [2], following two alternatives have been discussed:

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

-        Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

-        Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

In our view, to be able to guarantee that the required number of repetitions are transmitted, the most accurate counting of available slots is necessary. From this perspective, the availability of a slot should be determined based on dynamic signalling in addition to RRC configuration. Therefore, based on the above agreement, Alt2 should be supported. Further, both the SFI as well as cancellation indication should be accounted for determining the available slots.

Proposal 1: For PUSCH type A repetition coverage enhancements in NR Rel-17, support the determination of available slots for PUSCH repetition based on following configuration/indication:

- semi-static TDD configuration

- dynamic SFI

- dynamic cancellation indication

Another aspect that has been discussed in RAN1#104-e is when the availability of slot for PUSCH repetition determined. In RAN1#104-e [2], following two alternatives have been discussed, but not yet agreed:

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

In our view, Alt-a is a simple solution where the determination of available slots is done before the start of first actual transmission by taking into account both the semi-static TDD configuration and dynamic signalling including SFI and cancellation indication. No additional slots or postponement will be applied in this case. However, the actual number of available slots might still not be determined completely in this case, if UE receives SFI and/or CI after the start of first transmission. In this case, there might still be a potential that the required number of repetitions for desired coverage is not available. From this perspective, Alt-b might be better since the actual availability of the scheduled slots could be determined even after the first transmission has started. However, one issue with this alternative is that once the UE determines that one or more slots are not available after the transmissions have started, then postponement would be needed to reach the required number of actual repetitions. If there is no restriction on the postponement, then this will led to additional latency depending upon availability of UL symbols on slots that are initially not scheduled for repetitions. We further discuss the postponement mechanism in section 3 where we propose controlled postponement. In our view, if controlled postponement can be supported, then Alt-b is preferable, otherwise Alt-a should be supported.

Proposal 2: For PUSCH type A repetition coverage enhancements in NR Rel-17, the determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions, if controlled postponement is supported, otherwise, the determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

# 3	Postponement mechanism

For the postponement mechanism, one possibility is to limit the repetitions within a maximum repetition duration such that the UE is not allowed to go beyond the span duration even if the indicated number of repetitions are not transmitted. This would guarantee at least the maximum duration beyond which no repetitions are transmitted. In fact, if both the mechanisms being considered are applied together, that is the UE is configured with number of repetitions (consecutive slots) and number of actual repetitions (available UL slots), then the number of repetitions can be implied maximum repetition span duration. For example, when number of repetitions (consecutive slots) is indicated to be 32 and number of actual repetitions is indicated to be 16, and if the UE is able to transmit 16 repetitions in 24 consecutive slots, then the transmission stops after 24th consecutive slots and doesn't continue till the end of 32nd consecutive slots. On the other hand, if the actual number of repetitions transmitted in 32 consecutive slots is less than 16, then UE will stop the repetitions at the end of 32nd slot and will not continue.

Proposal 3: For PUSCH type A repetition coverage enhancements in NR Rel-17, controlled postponement can be supported, where the indication of a repetition span duration is specified and beyond the duration of repetition span, no further UL repetitions are allowed.

- In case both the mechanisms of indicating number of repetitions and actual number of repetitions are simultaneously supported, then the number of repetitions can be used as a repetition span to transmit the actual number of repetitions

# 4	Maximum number of slots

For enhanced PUSCH type A repetition with increased number of repetitions, additional values beyond 16 need to be discussed and agreed. In RAN1#104-e [2], following agreements related to maximum number of repetitions and indication of maximum number of repetitions have been made:

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

In our view, the maximum increased number of repetitions should be 32 and additional values between 16 and 32 with same granularity can be supported such as 20, 24, and 28. Supporting these values should provide sufficient flexibility for required coverage enhancement. Furthermore, the increased number of repetitions should be configured/indicated with repetition factor in TDRA, PUSCH-Config and ConfiguredGrantConfig.

Proposal 4: For PUSCH type A repetition coverage enhancements in NR Rel-17, maximum number of repetitions is increased to 32 and additional values for number of repetitions 20, 24 and 28 should be supported for both DG-PUSCH and CG-PUSCH, where the indication/configuration should be supported with repetition factor in TDRA, PUSCH-Config and CofiguredGrantConfig.

# 5	PUSCH repetition mode configuration/indication

Another aspect is how to configure/indicate new PUSCH repetitions modes. One straightforward solution could be that for indicating more than 16 repetitions, no additional specification is done, except just supporting additional values for number of repetitions as discussed in section 4. For the mechanism to count number of repetitions as available UL slots, a semi-static indication could be configured for enhanced PUSCH repetition type A. Basically, if this field is configured by RRC, then the current indication for number of repetitions either in TDRA table or with aggregation factor or repetition factor is counted as available UL slots. However, this will allow only semi-static switching between the two modes for PUSCH repetition type A even for dynamic grant or PUSCH CG type 2. Other possibility could be to have a more dynamic switching between the two modes by DCI field, but this would require new 1-bit field. Therefore, a more suitable method to dynamic switch between the two modes for PUSCH repetition type A would be to have implicit indication with the TDRA table. In our view, it is critical for relative lower repetition values to ensure the indicated number of repetitions are actually transmitted (i.e., count the actual repetitions) while supporting a very large number of actual repetition (actual available UL slots) is not necessary from coverage point. Thus, we suggest that if the number of repetitions indicated with the numberofRepetitions in the TDRA is higher than the currently supported maximum value i.e. 16, then the value is interpreted as consecutive number of slots (including UL/DL/flexible symbols), otherwise if 16 or less number of repetitions are indicated, then the value is interpreted as number of available UL slots. An example of the TDRA table is illustrated in Table 1.

Enh-Type A with 8 repetitions (actual, since less than 16)

Current Type A behaviour with 20 repetitions (not actual, since more than 16)

Enhanced Type A indicate up to 16 repetitions (actual repetitions, since equal to 16)

Table 1: Illustration of TDRA table with implicit indication for two modes of PUSCH repetition type A

| Row index | PUSCH mapping type | K2 | S | L | numberofRepetitions |
| --- | --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 | 2 |
| 2 | Type A | j | 0 | 4 | 16 |
| 3 | Type A | j | 0 | 10 | 4 |
| 4 | Type B | j | 2 | 10 | 1 |
| 5 | Type B | j | 4 | 4 | 8 |
| 6 | Type B | j | 4 | 8 | 2 |
| 7 | Type B | j | 4 | 6 | 4 |
| 8 | Type A | j+1 | 0 | 2 | 32 |
| 9 | Type B | j+1 | 2 | 4 | 20 |
| 10 | Type A | j+1 | 0 | 10 | 1 |
| 11 | Type A | j+2 | 0 | 4 | 16 |
| 12 | Type A | j+2 | 0 | 12 | 1 |
| 13 | Type A | j+2 | 0 | 10 | 1 |
| 14 | Type B | j | 8 | 6 | 2 |
| 15 | Type A | j+3 | 0 | 14 | 8 |
| 16 | Type A | j+3 | 0 | 10 | 2 |

Proposal 5: For PUSCH type A repetition coverage enhancements in NR Rel-17, both dynamic switching and semi-static configuration should be supported for switching between two modes of PUSCH repetition Type A (number of repetitions counted as total consecutive slots and number of repetitions counted as available UL slots):

- At least implicit switching based on the numberofRepetitions indicated in TDRA table should be considered – if the numberofRepetitions is below a certain threshold (for example <= 16), then numberofRepetitions is counted as number of available slots, otherwise numberofrepetitions is counted as consecutive number of total slots (current behaviour)

- If numberofRepetitions is not included in TDRA, then repetition factor repK is applied for CG type 1 and type 2 and pusch-AggregationFactor for dynamic grant PUSCH

# 6	Conclusion

In this section, we summarize the observation/proposals from above section:

Proposal 1: For PUSCH type A repetition coverage enhancements in NR Rel-17, support the determination of available slots for PUSCH repetition based on following configuration/indication:

- semi-static TDD configuration

- dynamic SFI

- dynamic cancellation indication

Proposal 2: For PUSCH type A repetition coverage enhancements in NR Rel-17, the determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions, if controlled postponement is supported, otherwise, the determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

Proposal 3: For PUSCH type A repetition coverage enhancements in NR Rel-17, controlled postponement can be supported, where the indication of a repetition span duration is specified and beyond the duration of repetition span, no further UL repetitions are allowed.

- In case both the mechanisms of indicating number of repetitions and actual number of repetitions are simultaneously supported, then the number of repetitions can be used as a repetition span to transmit the actual number of repetitions

Proposal 4: For PUSCH type A repetition coverage enhancements in NR Rel-17, maximum number of repetitions is increased to 32 and additional values for number of repetitions 20, 24 and 28 should be supported for both DG-PUSCH and CG-PUSCH, where the indication/configuration should be supported with repetition factor in TDRA, PUSCH-Config and CofiguredGrantConfig.

Proposal 5: For PUSCH type A repetition coverage enhancements in NR Rel-17, both dynamic switching and semi-static configuration should be supported for switching between two modes of PUSCH repetition Type A (number of repetitions counted as total consecutive slots and number of repetitions counted as available UL slots):

- At least implicit switching based on the numberofRepetitions indicated in TDRA table should be considered – if the numberofRepetitions is below a certain threshold (for example <= 16), then numberofRepetitions is counted as number of available slots, otherwise numberofrepetitions is counted as consecutive number of total slots (current behaviour)

- If numberofRepetitions is not included in TDRA, then repetition factor repK is applied for CG type 1 and type 2 and pusch-AggregationFactor for dynamic grant PUSCH

# 7	References

[1]	3GPP RP-210855, “Revised WID on NR coverage enhancements”, China Telecom

[2] 	3GPP RAN1#104-e. “Chairman Notes”