3GPP TSG RAN WG1 #105-e		R1-2105788

e-Meeting, May 10th – 27th, 2021

Source:	Ericsson

Title:	Maintenance for Rel-16 UE power savings

Agenda Item:	7.2.7

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss one correction related to condition under which CSI-RS triggering offset is fixed to zero (in 38.214-g50, 5.2.1.5.1).

# Discussion

The following sentence is in 38.214-g50, subclause 5.2.1.5.1, and describes  the condition when CSI-RS triggering offset is fixed to zero.

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states, the CSI-RS triggering offset is fixed to zero.

The highlighted text can cause potential confusion as the condition becomes unclear i.e. whether it is an “or” or “and”. In our understanding, the condition should be “and” condition i.e. “the UE is not configured minimumSchedulingOffsetK0 for any DL BWP and the UE is not configured with minimumSchedulingOffsetK2 for any UL BWP”.

This can be clarified by adopting below TP.

Proposal 1 : Adopt below TP for 38.214-g50, subclause 5.2.1.5.1

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or and if the UE is not configured with minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states, the CSI-RS triggering offset is fixed to zero.

# Conclusions

- Proposal : Adopt below TP for 38.214-g50, subclause 5.2.1.5.1

<omit unchanged text>

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or and if the UE is not configured with minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states, the CSI-RS triggering offset is fixed to zero.

<omit unchanged text>

# References

[1] 38.214-g50