3GPP TSG RAN WG1 #105-e	                                         	R1-2105790

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.11

Source:	Ericsson

Title:	Remaining details of Rel-16 NR UE features

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss a description update for FG 22-10.

The discussion is based on the latest RAN1 agreed feature list in [1].

# 2	Discussion

## 2.1	FG 22-10

Below agreements were made in RAN1#104-e meeting.

Agreement

Regarding the interpretation of UE capabilities in case of cross-carrier operation, for cross-carrier scheduling with the same SCS in the scheduling cell and the scheduled cell, RAN1 clarifies that support of pdcch-MonitoringAnyOccasionsWithSpanGap is based on both the support of this capability for the band of the scheduled/triggered/indicated cell and the support of this capability for the band of the scheduling/triggering/indicating cell.

- Note: For pdcch-MonitoringAnyOccasionsWithSpanGap, the supported set (set1, set2 or set 3) for cross-carrier scheduling with the same SCS in the scheduling cell and the scheduled cell is still based on the indicated value for the band of the scheduling cell.

Agreement

For cross-carrier scheduling with different SCSs in the scheduling cell and the scheduled cell, RAN1 introduces new Rel-16 UE capability to clarify the interpretation of pdcch-MonitoringAnyOccasionsWithSpanGap in case of cross-carrier operation.

- The detailed design of this new Rel-16 UE capability is to be discussed under Rel-16 UE feature session.

Agreement

LS to RAN2 on interpretation of UE features in case of cross-carrier operation is endorsed in R1-2102085

FG 22-10 was introduced to reflect the second agreement listed above. However, one minor issue to clarify for the FG is whether the number of spans/span-gap follow those of the scheduling cell or scheduled cell. We propose to follow the same principle as CCS with same SCS case (like the note in first agreement above).

Proposal : For FG 22-10, add below sentence to the note column:

For pdcch-MonitoringAnyOccasionsWithSpanGap, the supported set (set1, set2 or set 3) for cross-carrier scheduling with the different SCSs in the scheduling cell and the scheduled cell is still based on the indicated value for the band of the scheduling cell.

# 3	Conclusions

In this contribution we discussed the remaining details of Rel-16 NR UE features and make the following proposal:

- Update FG 22-10 description as described in section 2.

# References

- R1-2104120, "Updated RAN1 UE features list for Rel-16 NR after RAN1#104bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#104bis-e, April 2021.