3GPP TSG RAN WG1 #105-e	Tdoc R1-2105795

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.7.3

Source:	Ericsson

Title:	Evaluation results for UE power saving schemes

Document for:	Discussion and Decision

# 1	Introduction

In this paper, additional simulation results for PDCCH monitoring adaptation are presented, for SSSG-switching in relation with DRX functionality.

# 2	Performance evaluations

In the previous meeting, it was agreed that the PDCCH monitoring adaptation agenda item AI in the UE power-saving (UEPS) work item (WI) will focus on PDCCH-skipping and SSSG-switching. In PDCCH-skipping, the UE is configured with a certain skipping duration. The NW then command the UE to skip PDCCH monitoring during that duration when the NW has no data in the buffer. In SSSG-switching, the UE is configured with two SSSGs. In the simulation, the UE is configured with dense monitoring (i.e., monitoring PDCCH in every slot) and sparse monitoring (monitoring a certain duration in every certain period). The UE switches to dense monitoring when there is no data in the NW buffer.

## 2. 1.	SSSG-switching + DRX functionality

The performance comparison of SSSG-switching for the cases of the DRX on-duration using the sparse PDCCH monitoring (or sparse SSSG) and dense PDCCH-monitoring (dense SSSG) is shown below.

The timeline of these scenarios can be seen in the following figures.

Figure 1. Sparse monitoring is used in the DRX on-duration. Only long-DRX is configured.

Figure 2. Dense monitoring is used in the DRX on-duration. Only long-DRX is configured.

The parameter of each method can be seen in below table.

Table 1. Parameter set-up for the case of long-DRX only scenario

| No | Methods | Parameter / Remark |
| --- | --- | --- |
| 1 | Long-DRX only | Long-DRX [160ms; 8ms]; IAT [100ms] |
| 2 | Long-DRX + SSSG switching DRX on-duration uses dense monitoring | Long-DRX [160ms; 8ms]; IAT [100ms] Once the UE receive PDSCH, the UE monitors PDCCH in every slot. If there is no PDSCH for 4ms, the UE monitors PDCCH with sparse SS, i.e., using a duration of 2ms within 8ms periodicity. DRX on-duration uses dense monitoring. |
| 3 | Long-DRX + SSSG switching DRX on-duration uses sparse monitoring | Long-DRX [160ms; 8ms]; IAT [100ms] Once the UE receive PDSCH, the UE monitors PDCCH in every slot. If there is no PDSCH for 4ms, the UE monitors PDCCH with sparse SS, i.e., using a duration of 2ms within 8ms periodicity. DRX on-duration uses sparse monitoring. |

The simulation results can be seen in the below table.

Table 2. Power consumption and UPT performance in long-DRX only case

| Methods | UPT (Mbps) | UPT Loss (%) | Power (Relative Units) | Power saving (%) |
| --- | --- | --- | --- | --- |
| Long-DRX only | 35.10 |  | 41.32 |  |
| Long-DRX + SSSG switching DRX on-duration uses dense monitoring | 34.70 | 1.2% | 30.61 | 25.9% |
| Long-DRX + SSSG switching DRX on-duration uses sparse monitoring | 33.28 | 5.2% | 28.71 | 30.5% |

From the above simulation results, it can be observed that for the case of only long-DRX is configured, the power-saving gain by using sparse SS during the DRX on-duration is only about 5% at the cost of 4% of UPT loss. In other words, the gain of using/maintaining sparser SS during the DRX on-duration in this scenario is not clear.

Using dense monitoring during DRX on-duration provides better UPT vs UE power saving tradeoff.

# 3	Conclusion

In section 2, the following observation is made:

Observation 1	Using dense monitoring during DRX on-duration provides better UPT vs UE power saving tradeoff.