3GPP TSG-RAN WG1 #105e		R1-2105797

eMeeting, May 10- 27, 2021

Source:	Ericsson

Title:	Study on single DCI scheduling PDSCH on multiple cells

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction

In this document, we discuss further discuss the study on “PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI”.

# Discussion

Evaluation results for the study were discussed in RAN1#104-e and the observations were submitted to RAN in [1]. This was further discussed in RAN1#91e (in [91E][26][DSS_scope] email thread [2]) and outcome of RAN discussion was that “…there is no agreement to start the normative work on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI. There is no agreement to stop studying this either”

In our RAN1#104-e contribution [3], we provided our evaluation results for the study and our observations from [3] are given in Annex A as a reference. In Annex B, we provide additional simulation results for agreed Combination 1 (i.e., 20MHz carrier at 2GHz used for scheduling PCell PDSCH/PUSCH on another low-band DSS carrier).

Overall, our evaluations indicate that single DCI scheduling PDSCH on two cells (mc-DCI) provides marginal or no performance gains under the agreed simulation framework.

Observation 1

- Evaluations under the agreed simulation framework indicate that single DCI scheduling PDSCH on two cells (mc-DCI) provides marginal or no performance gains.

Below we list some alternatives on how to handle the work for this Objective (some of these were also discussed in RAN#91e)

- Conclude that there is no consensus on performance gains for “PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI” under the agreed simulation framework and stop further work on this Objective for Rel17.

- Continue Rel17 work with modified WID objectives, e.g.,

  - Remove “The increase in DCI size should be minimized” from the objectives and add “Allow configuration of DCI fields for scheduling the 2 PDSCHs”

  - Limit the work to same SCS case with single DCI

  - Limit the work to intra-band case

- Stop further work on the Objective for Rel17 with the understanding that “PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI” can be further studied for Rel18 targeting other scenarios e.g. intra-band CA with 4/8 CCs with same SCS, including FR2.

In our view, discussions related to point 2 above (i.e., updating the WID objectives) are out of RAN1 scope and should be handled in RAN.

Our preference is to conclude based on point 1. If there is no consensus to agree on point 1, we would be OK with a conclusion along the lines of point 3 above, with the understanding that other PDCCH load reduction features such as multi-PDSCH scheduling (being specified in NR_ext_to_71GHz WI) are also considered when evaluating the benefits of this feature.

# Conclusion

In this document we discuss the study on “PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI,” making the following observations and proposals:

- Evaluations under the agreed simulation framework indicate that single DCI scheduling PDSCH on two cells (mc-DCI) provides marginal or no performance gains.

- Select one of below two alternatives for concluding the study related to the objective on “PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI”

  - Alt1 - Conclude that there is no consensus on performance gains for “PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI” under the agreed simulation framework and stop further work on this Objective for Rel17.

  - Alt2 – Conclude to stop further Rel17 work for this objective with the understanding that

    - “PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI” can be further studied for Rel18 targeting other scenarios e.g. intra-band CA with 4/8 CCs with same SCS, including FR2, and

    - other PDCCH load reduction features such as multi-PDSCH scheduling (being specified in NR_ext_to_71GHz WI) are also considered when evaluating the benefits of this feature

# References

[1] RP-210433 - “Status report for WI: Core part: NR Dynamic spectrum sharing (DSS)”, RAN#91e, March 2021.

[2] RP-210883 – “ Moderator's summary for email discussion [91E][26][DSS_scope]”, Nokia, RAN#91e, March 2021.

[3] R1-2101562 – “Study on single DCI scheduling PDSCH on multiple cells”, Ericsson, RAN1#104-e, February 2021.

# Annex A

Here we provide the observations from previous evaluations submitted in our RAN1#104-e contribution [3]. Detailed simulation results are given in our RAN1#104-e contribution [3].

Observation 1

- For mc-DCI to have no loss of scheduling flexibility compared to independent DCI per cell, at least the following assumptions should be satisfied:

  - Numerologies of the two scheduled cells are independent

  - Channel BWs (and BWP BW) of the two scheduled cells are independent

  - MIMO configuration of the two scheduled cells are independent

  - HARQ processes/MCSs used for each scheduled cell are independent

  - FDRA/TDRA (including type) used for each scheduled cell are independent

  - mc-DCI should support the functionality of all the DCI fields specified for existing DCI formats

Observation 2

- For a CA scenario with e.g. two serving cells, the NW may choose to schedule any of following cases for a given UE based on available slots for NR scheduling on a given carrier, data in the buffer, channel conditions, NW loading and HARQ retransmission activity of each serving cell

  - PDSCH on cell 1 only

  - PDSCH on cell 2 only

  - PDSCH on cell 1 and cell 2

  - No PDSCH scheduled

- Gains from 1 DCI scheduling 2 PDSCHs (i.e., mc-DCI) are only available for Case 3 above

Observation 3

- For scenario 1 (i.e., 20MHz carrier at 2GHz used for scheduling PCell PDSCH/PUSCH on another low-band DSS carrier),

  - in slots where PDSCH is scheduled on both cell1 and cell 2, mc-DCI can achieve similar blocking performance as baseline case with reduced CCE allocation. The amount of possible CCE reduction depends on loading, i.e., 8 CCEs for low load and smaller for higher loads. If CCE allocation is reduced any further, performance of mc-DCI is worse.

  - Assuming 50% of slots have two-PDSCH scheduling with cell1 scheduling PDSCH on both cell1 and cell2 (optimistic assumption for scenario 1 if one of the scheduled carriers is shared with LTE), and 10 symbols available for data scheduling on scheduling cell (2 DMRS symbols), an overhead reduction of < 2.5% is expected with other optimistic assumptions that rate-matching of PDSCH around PDCCH can reclaim all the saved resources (which is unlikely when there are other DCIs in the Coreset), and that there is no performance loss due to lower flexibility when scheduling with mc-DCI. Under realistic assumptions, no gains are expected.

Observation 4

- For scenario 2 (i.e., 100MHz mid-band 4GHz carrier used for scheduling PCell PDSCH/PUSCH on a low-band DSS carrier),

  - using mc-DCI is not expected to provide performance gains as the blocking performance for scheduling up to 10 UEs is close to zero even for baseline case of two legacy DCIs.

Observation 5

- Evaluations indicate that single DCI scheduling PDSCH on two cells (mc-DCI) provides marginal or no performance gains.

# Annex B

Figure B1 below shows UE throughput comparison between using legacy DC and mc-DCI assuming the agreed full-buffer traffic model. Results are shown for Combination 1/Scenario 1 (i.e., 20MHz carrier at 2GHz used for scheduling PCell PDSCH/PUSCH on another low-band carrier). The same optimistic assumptions for mc-DCI as assumed in our RAN1#104-e contribution [3] were also used in the below evaluation. i.e.,

- For mc-DCI the CCE savings for low load as estimated in [3] (and captured in observation 3 in Annex A) are used and it is (optimistically) assumed that rate-matching of PDSCH around PDCCH can reclaim all the saved CCE resources (this is unlikely as there will be other DCIs in the Coreset and it should be considered as upper bound)

- 50% of slots have two-PDSCH scheduling with cell1 scheduling PDSCH on both cell1 and cell2

Figure B1. UE throughput for Scenario 1 (20MHz CC @2GHz used for scheduling PCell PDSCH/PUSCH on another low-band carrier)

As seen from the results the performance difference between mc-DCI and legacy DCI is marginal (+2.5% mean tput gain for MC-DCI) even under the optimistic assumptions.

# Annex C, Simulation assumptions

Table A-2 Link level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | Option 1: Inter-band CA (700MHz + 4GHz) Intra-band CA (2GHz) Option 2: Only 4GHz is considered |
| SCS | 15 kHz for 700MHz/2GHz 30 kHz for 4GHz |
| Bandwidth | Option 1: Baseline: PCell 10MHz + SCell 10/40MHz Optional: PCell 20MHz + SCell 20/40/100MHz Option 2: Baseline: Scheduling cell 100 MHz Optional: Scheduling cell 20 MHz |
| Channel model | TDL-C |
| Delay spread | 300 ns |
| Number of symbols for CORESET | [1], 2 or 3 |
| CORESET BW (contiguous PRB allocation) | 24/48/96 RBs depending on the bandwidth |
| CCE-to-REG mapping | Interleaved, [non-interleaved] |
| REG bundle size | 6 |
| Interleaver size | 2 |
| DCI payload size (excluding CRC) | Single PDSCH scheduling: 60 bits as baseline payload size Multi-cell PDSCH scheduling: 72/84/96/104 bits |
| BLER target for multi-cell scheduling DCI | Option 1: 1% Option 2: 0.5% |
| Number of BS antennas | 2 Tx for 700MHz/2GHz carrier frequency 4 Tx for 4GHz |
| Number of UE antennas | 2 Rx for 700MHz/2GHz carrier frequency 4 Rx for 4GHz carrier frequency |
| Modulation | QPSK |
| Channel coding | Polar code |
| UE speed | 3km/h |
| Aggregation level | 1/2/4/8/16 |
| Tx Diversity | One port precoder cycling |

Note 1: For two-cell scheduling via a single DCI, PDCCH transmitted on SCell schedules one PDSCH on the SCell and another PDSCH on PCell.

Note 2: For comparison, for single-cell scheduling, one PDCCH transmitted on SCell schedules one PDSCH on the SCell via self-scheduling and another PDCCH transmitted on the SCell schedules another PDSCH on PCell via cross-carrier scheduling.

Further discussion which rows are applicable to the scheduling cell/the scheduled cell for PDCCH

Agreements:

Further study with below simulation assumptions:

Simulation scenarios:

- For two-cell scheduling via a single DCI, PDCCH transmitted on a first cell schedules one PDSCH on the first cell and another PDSCH on a second cell.

- For single-cell scheduling (baseline), one PDCCH transmitted on a first cell schedules one PDSCH on the first cell via self-scheduling and another PDCCH transmitted on the first cell schedules another PDSCH on a second cell via cross-carrier scheduling.

  - Companies can optionally compare to the case of PDCCH transmitted on each of the two cells via self-scheduling. In this case, company should provide details on how to calculate the PDCCH blocking rate.

Simulation assumptions on carrier frequency, SCS, antenna configuration, carrier bandwidth as well as CORESET configuration

- Combination 1: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW, 2-symbol CORESET with 96RBs

- Combination 2: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz carrier BW, 1-symbol CORESET with 270RBs

- [Combination 3: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW, 3-symbol CORESET with 48RBs]

- [Combination 4: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz carrier BW, 2-symbol CORESET with 96RBs]

Payload size of two-cell scheduling DCI (excluding CRC):

- 60 for single-cell scheduling DCI (baseline).

- 72/84/96/108 for two-cell scheduling DCI.

- Companies are encouraged to report how the values are obtained, e.g., via separate or shared fields in DCI format.

Target BLER for two-cell scheduling DCI: 1% (baseline), 0.5%(optional)

- Option 1: 1%.

- Supported by OPPO, vivo, Nokia, Qualcomm, CATT, Ericsson, Huawei, Lenovo, Intel, MediaTek

- Option 2: 0.5%.

- Supported by Samsung, LG

Regarding the CCE-to-REG mapping, based on the agreed interleaved CCE-to-REG mapping, whether to adopt non-interleaved CCE-to-REG mapping is up to the proponent.

Agreements:

- Further study with below simulation assumptions:

Table A-3 System level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | For scheduling cell, follow agreed link level simulation assumptions For scheduled cell, consider 700MHz/2GHz with 10/20MHz BW (LTE overhead on DSS carrier can be optionally provided, up to proponent) |
| SCS |  |
| Simulation bandwidth |  |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 46 dBm for 10MHz |
| Scenario | Urban Macro |
| ISD | 500m |
| TRP antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 700MHz (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) for 2GHz (M,N,P,Mg,Ng;Mp,Np)= (8,4,2,1,1;1,1) for 4GHz |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) for 700MHz/2GHz (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 4GHz |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds of interest | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer(baseline), FTP model 1 or 3 up to company |
| Macro sites | 19 |
| Number of UEs per cell | 10/15/20 UEs |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |