3GPP TSG-RAN WG1 Meeting #105-e	R1-2105810

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.6

Source:	Ericsson

Title:	Maintenance for single-DCI based multi-TRP in Rel-16

Document for:	Discussion

# Introduction

In this paper, we address an open issue related to single-DCI based multi-TRP PDSCH transmission.

# Discussion

In RAN1#104bis-e, the following issue was discussed in [1]:

Although the issue has been proposed by multiple companies over multiple meetings, the issue hasn’t been discussed in an email thread and there hasn’t been any conclusion.  Although some companies claim that this is an optimization or a non-essential issue.

We would like to clarify that this, in fact, is a feature already supported by the specifications.  According to current RAN1 specifications, the single DCI based multi-TRP PDSCH repetition schemes can be scheduled with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2.  Note that there are no restrictions on which RNTIs can be used when scheduling single DCI based multi-TRP PDSCH repetition schemes.

According to current RAN1 specifications, there are no restrictions on which RNTIs can be used when scheduling single DCI based multi-TRP PDSCH repetition schemes.

The single DCI based multi-TRP PDSCH repetition schemes can be scheduled with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2.

However, for DL SPS, the RV field in the activating DCI is reserved and is used for activation validation according to clause 10.2 of TS 38.213.  Hence, the RV values to be assumed for DL SPS scheduled with single DCI based multi-TRP PDSCH repetition schemes is not defined in 38.214, and hence the specifications are incomplete.

For DL SPS, the RV field in the activating DCI is reserved and is used for activation validation.

The RV values to be assumed for DL SPS scheduled with single DCI based multi-TRP PDSCH repetition schemes is not defined in 38.214, and hence the specifications are incomplete.

Hence, we suggest to discuss the issue of DL SPS based PDSCH repetitions for at least S-DCI mTRP case in RAN1#105-e and make a conclusion.  We have provided the corresponding draft CR in [2].

Discuss the issue of DL SPS based PDSCH repetitions for at least S-DCI mTRP case in RAN1#105-e.

# Conclusion

Based on the discussion in the previous section we propose the following:

Observation 1	According to current RAN1 specifications, there are no restrictions on which RNTIs can be used when scheduling single DCI based multi-TRP PDSCH repetition schemes.

Observation 2	The single DCI based multi-TRP PDSCH repetition schemes can be scheduled with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2.

Observation 3	For DL SPS, the RV field in the activating DCI is reserved and is used for activation validation.

Observation 4	The RV values to be assumed for DL SPS scheduled with single DCI based multi-TRP PDSCH repetition schemes is not defined in 38.214, and hence the specifications are incomplete.

Proposal 1	Discuss the issue of DL SPS based PDSCH repetitions for at least S-DCI mTRP case in RAN1#105-e.

# References

[1]	R1-2103217, Summary for Rel-16 NR eMIMO maintenance, RAN1#104b-e, April 12th – 20th, 2021.

[2]	R1-2105809, Draft CR on DL SPS based PDSCH repetitions, Ericsson, RAN1#105-e, May 10th – 27th, 2021.