3GPP TSG RAN WG1#105-e                                	R1-2105822

e-Meeting, May 10th – May 27th, 2021

Source: 	Asia Pacific Telecom, FGI, ITRI, III

Title: 	Enhancements on HARQ in NTN

Agenda item:	8.4.3

Document for:	Discussion and Decision

# Introduction

In RAN1#104-e, the following agreements were achieved.

Agreement in RAN1#104-e For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. - Working assumption: X = T_proc,1 - FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0 - Note: The TB of the two PDSCHs can be either same or different Agreement in RAN1#104-e For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes - FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

Based on the FFS in the above agreements, the following discussions were needed.

- Whether X should be changed to X = max (T_proc,1, K1) where K1 is the minimum k1 if it is configured

- For Type-2 HARQ codebook, the details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

# Discussion

## PDSCH/PUSCH scheduling restriction

In RAN1#104-e, the following agreement has made.

Agreement in RAN1#104-e For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. - Working assumption: X = T_proc,1 - FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0 - Note: The TB of the two PDSCHs can be either same or different

The intention of FFS is to associated X with the scheduling offset K1 for relaxing the UE processing time requirement.

However, the goal for X is to find the minimum PDSCH processing time, e.g., N1, as a guideline for NW scheduling. This restriction of X will be used for UE to tell whether a PDSCH scheduling is valid. Therefore, X shall not be linked to a scheduling offset provided by NW.

As pointed out by the moderator in R1-2102143, we quote as follows: As the majority’s view, reusing of T_(proc,1) can be accepted as the compromise solution to minimize the spec impact with relative relaxed requirement for UE.

RAN1 shall confirm the working assumption: X = T_proc,1.

Conform the working assumption: X = T_proc,1.

## HARQ codebook enhancement

In RAN1#104-e, the following agreement has been made.

Agreement in RAN1#104-e For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

In NR Rel-16, new DCI format 1_2 for DL scheduling is introduced. The DCI format 1_2 has the DAI field that supports a 0-bit configuration by RRC IE downlinkAssignmentIndexDCI-1-2.

3GPP TS 38.331 V16.3.1 (2021-01), downlinkAssignmentIndexDCI-1-2 Configures the number of bits for "Downlink assignment index" in DCI format 1_2. If the field is absent, then 0 bit for "Downlink assignment index" in DCI format 1_2. Note that 1 bit and 2 bits are applied if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic. 4 bits is applied if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook is set to dynamic (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

In NR Rel-16, the corresponding UE behavior is also introduced. If a DCI format does not include a counter DAI field, then A UE does not expect to multiplex its HARQ-ACK in a Type-2 HARQ-ACK codebook.

3GPP TS 38.213 V16.4.0 (2020-12), 9.1.3 Type-2 HARQ-ACK codebook determination A UE does not expect to multiplex in a Type-2 HARQ-ACK codebook HARQ-ACK information that is in response to a detection of a DCI format that does not include a counter DAI field.

To minimize spec impacts, reusing Rel-16 DCI format 1_2 to support reducing Type-2 HARQ-ACK codebook (CB) shall be considered. However, DCI format 1_2 applicability to NR NTN shall be concluded.

Reuse Rel-16 DCI format 1_2 with 0-bit DAI field to support reducing codebook size with Type-2 HARQ-ACK CB only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes.

The applicability of DCI format 1_2 to NR NTN shall be concluded in RAN1.

In Rel-15 NR, for some cases, even if the TB is correctly decoded in the physical layer of a UE, the UE may not generate any HARQ-ACK information, as captured in TS 38.321.

In Rel-15 NR, for some cases, even if the TB is correctly decoded in the physical layer of a UE, the UE may not generate any HARQ-ACK information.

However, as shown in R1-1911401, no RAN1 CR has been agreed because RAN1 believed that UE and NW shall have the same understanding of the expected HARQ-ACK codebook based on TS 38.321.

R1-1911401, Summary of Draft CRs on CA and HARQ-ACK Codebook for Rel-15 NR, Samsung The reason for change is: HARQ-ACK information is generated in the physical layer in the UE according to the instruction of the higher layer in the UE, as described in the specification of TS 38.321. Hence, for some cases, even if the TB is correctly decoded in the physical layer of a UE, the UE may not generate any HARQ-ACK information, as captured in TS 38.321 provided also below: - if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or - if the HARQ process is equal to the broadcast process; or - if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired: - not instruct the physical layer to generate acknowledgement(s) of the data in this TB. The current description for HARQ-ACK information generation in TS 38.213 contradicts with the above cases, hence should be corrected. The suggested change is: Adding “subject to the conditions in subclause 5.3.2 of [TS 38.321]” for the completion of conditions of HARQ-ACK information generation Comment: It is mentioned in “Consequences if not approved”, that a UE and gNB may not have the same understanding of the expected HARQ-ACK codebook. However, TS 38.321 captures when a UE does not generate HARQ-ACK information. Recommendation: Conclude whether the CR is not needed or whether the suggested change can be included in an alignment CR.

According to R2-2102052, the stage-3 running CR for TS 38.321 may add a new condition for HARQ-ACK disabling.

R2-2102052, Stage-3 running CR for TS 38.321 for Rel-17 NTN - if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or - if the HARQ process is associated with a transmission indicated with a MSGB-RNTI and the Random-Access procedure is not yet successfully completed (see clause 5.1.4a); or - if the HARQ process is equal to the broadcast process; or - if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired; or: - if HARQ feedback is disabled for the HARQ process: - not instruct the physical layer to generate acknowledgement(s) of the data in this TB.

If we follow the legacy UE behavior in Rel-15 and Rel-16, then HARQ-ACK CB enhancement may not be needed.

Whether to enhance HARQ-ACK CB in RAN1 shall be discussed, regarding TS 38.321 may capture when a UE does not generate HARQ-ACK information.

# Conclusion

In this contribution, we have the following observations

Observation 1	In Rel-15 NR, for some cases, even if the TB is correctly decoded in the physical layer of a UE, the UE may not generate any HARQ-ACK information.

Based on observations, the following proposals are made

Proposal 1	Conform the working assumption: X = T_proc,1.

Proposal 2	Reuse Rel-16 DCI format 1_2 with 0-bit DAI field to support reducing codebook size with Type-2 HARQ-ACK CB only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes.

Proposal 3	The applicability of DCI format 1_2 to NR NTN shall be concluded in RAN1.

Proposal 4	Whether to enhance HARQ-ACK CB in RAN1 shall be discussed, regarding TS 38.321 may capture when a UE does not generate HARQ-ACK information.

# Reference

Chairman’s Notes, 3GPP TSG RAN WG1 #103 e-meeting

Chairman’s Notes, 3GPP TSG RAN WG2 #112 e-meeting

3GPP TR 38.821 V16.0.0, Solutions for NR to support non-terrestrial networks, December 2019.

R1-2102143, Summary#4 of AI 8.4.3 for HARQ for NTN, ZTE

R2-2102052, Stage-3 running CR for TS 38.321 for Rel-17 NTN, InterDigital

R1-1911401, Summary of Draft CRs on CA and HARQ-ACK Codebook for Rel-15 NR, Samsung

R1-1911270, Correction on HARQ-ACK codebook determination Huawei, HiSilicon