3GPP TSG RAN WG1 Meeting #105-e	R1-2105857

e-Meeting, May 10th - May 27th, 2021

Agenda Item:	8.5.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	UL-AoA positioning enhancements

Document for:	Discussion & Decision

# Introduction

In RAN1#104bis the following agreement was made providing assistance signaling from LMF to gNB with an expected UL-AoA:

- Uncertainty range for expected UL AoA/ZoA is defined as follows - Expected azimuth angle of arrival as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) - φAOA - expected azimuth angle of arrival, ΔφAOA – uncertainty range for expected azimuth angle of arrival - Expected zenith angle of arrival as (θAOA - ΔθAOA/2, θAOA + ΔθAOA/2) - θAOA - expected zenith angle of arrival, ΔθAOA – uncertainty range for expected zenith angle of arrival - Select one of the following coordinate system alternatives for signaling UL AoA/ZoA assistance information - Alt.1: Only GCS is supported for AoA/ZoA assistance information indication - Alt.2: Both GCS and LCS are supported for AoA/ZoA assistance information indication - FFS: Additional signaling for AoA/ZoA assistance information (expected value and uncertainty range)

In this contribution we further discuss the aspects related to the UL-AoA TRP assistance data.

# UL-AoA TRP assistance data

Typically, the LMF is aware of relevant information related to a UE which is not (or only partly) available at the TRPs. To provide a TRP with information on the expected AoA/ZoA range; two options are applicable:

- Opt.1: the LMF derives directly the azimuth and zenith angle of arrival and uncertainties based on coarse UE-position estimate

- Opt.2: the LMF indirectly the azimuth and zenith angle of arrival and uncertainties with respect to a DL-PRS.

Option 1 was agreed in RAN1#104b, however the discussion on option 2 was not concluded. The TRP will in both options use this information to determine a more accurate AoA/ZoA estimate in a reduced search range. Option 2 is complementary to option 1 for scenarios when the DL-PRS beam indication is more accurate than an expected UL AoA/ZoA estimated based on a UE position (Figure 1). In addition, we see both options can be supported for the following reasons and scenarios:

- The TRP will not be able to derive the information from the SRS-configuration if the UE is not configured with SRS-spatial-relation towards a given TRP (or the UE is not configured with an SRS-spatial-relation).

- The LMF doesn’t necessarily know the beam direction of the DL-PRS resources (e.g. in case DL-AOD is not supported)

- Option 2 has minor spec impact:

In NRRPa, the TRP can provide the LMF with Measurement-Beam-Information on the TRP-Rx-beam direction used for UL-SRS measurement by indicating a DL-PRS or an SSB.

Observation 1: It is beneficial to use TRP Tx beam and Rx beam relation(s) to indicate the expected AoA/ZoA.

Proposal 1: For facilitating TRP UL-AoA measurements, support that LMF indicates DL-PRS resource for the expected AoA/ZoA and uncertainty range.

Figure 1 - UL-AoA uncertainty information for options 1 and 2 for a scenario with two TRPs and a single SRS-Spatial-Relation configuration.

# Conclusions

Based on the discussion in this document, we propose:

Proposal 1: For facilitating TRP UL-AoA measurements, support that LMF indicates DL-PRS resource for the expected AoA/ZoA and uncertainty range.