3GPP TSG RAN WG1 #105-e					       R1-2105870

e-Meeting, May 10th – 27th, 2021

Source:	WILUS Inc.

Title:	Discussion on multi-PDSCH/PUSCH scheduling for NR from 52.6GHz to 71GHz

Agenda item:	8.2.5

Document for:	Discussion/Decision

- Introduction

A new Rel-17 WI on supporting NR from 52.6 GHz to 71 GHz was approved in RAN#90-e meeting and revised WID was approved in RAN#91-e meeting. The objectives are listed as following [1]:

- Physical layer aspects including [RAN1]: - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timeing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively. - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS(480kHz, 960kHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum - Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS. …

In this contribution, we discuss enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI.

- Discussion on multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI

Issue 1: Type-2 HARQ-ACK codebook for multi-PDSCH scheduling with a single DCI

Agreement at RAN1 #104-e: [2] For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e. - Alt 1: C-DAI/T-DAI is counted per DCI. - Alt 2: C-DAI/T-DAI is counted per PDSCH. - Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …). - FFS: Codebook generation details - FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3) - FFS: Whether to apply time domain bundling of HARQ-ACK feedback

Since a single DCI schedules more than one PDSCHs with different TBs, it is not clear how to interpretate C-DAI and T-DAI in the DCI. In the RAN1#104-e meeting, three alternatives were identified [2]. The three alternatives are illustrated in Figure 1.

Figure 1. DAI counting rules. (a) Alt 1, (b) Alt 2, and (c) Alt 3. oi represents HARQ-ACK information of TBi.

In the last RAN1#104b-e meeting, RAN1 reached conclusions for each alternative. Based on these conclusions, we compare three alternatives below.

- Alt 1: C-DAI/T-DAI is counted per DCI.

Conclusion: (RAN1#104b-e [3]) The following is observed for alternative 1 from prior agreement. - For Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - C-DAI/T-DAI in DL DCI: Same DAI overhead with Rel-16 single-PDSCH DCI - T-DAI in UL DCI: - In case of single codebook handling feedback for both single and multi-PDSCH scheduling, same DAI overhead with Rel-16 UL DCI - In case of separate sub-codebooks, need additional DAI field (with same bit-width of DAI with Rel-16 UL DCI), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - Note that DAI field increment for this case is similar for the case in Rel-15 where CBG is configured - HARQ-ACK codebook generation: - A separate sub-codebook can be generated when multi-PDSCH DCI is configured for a serving cell, similar to the way as 2nd sub-codebook is defined to handle CBG-based scheduling - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell - FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s) - HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-)codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group. - The number of HARQ-ACK bits for multi-PDSCH DCI in case of separate sub-codebooks, or for all DL DCIs in case of single codebook, does not depend on the number of actually scheduled PDSCHs, rather, it is fixed as the maximum configured number of PDSCHs. - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

In Alt 1, C-DAI and T-DAI are counted per DCI and each increment represents the maximum number of PDSCHs scheduled by a single DCI. In Figure 1-(a), the maximum number of PDSCHs is assumed to 4. The merit of the Alt 1 is that the definition of C-DAI and T-DAI in Rel-15/16 is not changed and the number of bits for C-DAI and T-DAI can be kept as 2 bits. However, it may be result in increasing some overhead. For example, even if the DCI in CC#0 with (C-DAI,T-DAI)=(2,3) schedules 3 PDSCHs and the DCI in CC#2 with (C-DAI,T-DAI)=(4,4) schedules 2 PDSCHs, but a UE assumes that each of these DCIs schedules 4 PDSCHs to avoid Type-2 CB size ambiguity between UE and gNB. Therefore, some filler bits, ‘x’ would be inserted to the Type-2 CB. As a result, 13 TBs are scheduled, but the Type-2 CB includes 16 bits (with 3 filler bits).

In the conclusion of Alt 1, there are the following three FFS points.

- Q1) FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell.

  - As similarly in CBG based PDSCH reception, two sub-codebooks can be constructed, where the first sub-codebook is for the first type of transmission (i.e., M bit(s) for each DAI increment, for example single PDSCH scheduled by a single DCI) and the second sub-codebook is for the second type of transmission (i.e., N bits for each DAI increment, for example multi PDSCHs scheduled by a single DCI or CBG based transmission), where M<N. Instead, if single codebook is constructed, a UE regards the first type of transmission as the second type of transmission by padding N-M dummy bits, which results in high HARQ-ACK bit overhead. To make Type-2 HARQ-ACK as compact as possible, two sub-codebook approach would be better.

- Q2) FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s)

  - As we mentioned in Q1, the multi-PDSCH DCI and the CBG DCI can be considered as the second type of transmission, where a UE generates N bits for each DAI increments. Note that the value N can be determined by the maximum number of CBGs and the maximum number of SLIVs in a row in TDRA table.

- Q3) FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - If time domain bundling of HARQ-ACK feedback is configured, then the second type of transmission can be regarded as the first type of transmission, i.e., the second sub-codebook is dropped and the first sub-codebook is constructed based on the first type of transmissions.

- Alt 2: C-DAI/T-DAI is counted per PDSCH.

Conclusion: (RAN1#104b-e [3]) The following is observed for alternative 2 from prior agreement. - For Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - C-DAI/T-DAI in DL DCI: Bit-width can be increased (FFS: by how much), in DL DCI not only for multi-PDSCH DCI but also for single-PDSCH DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - T-DAI in UL DCI: Bit-width can be increased (FFS: by how much), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - C-DAI/T-DAI in DL DCI and T-DAI in UL DCI shall be designed such that at most 3 consecutive DCI missing can be resolved, same as in Rel-15/16 NR. - FFS: details on increment of DAI field size - FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI - HARQ-ACK codebook generation: - The number of HARQ-ACK bits depends on the number of scheduled PDSCHs. - FFS: ordering of the PDSCHs for DAI counting - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

In Alt 2, C-DAI and T-DAI are counted per PDSCH and each increment represents single PDSCH. Contrary to Alt 1, Alt 2 has no overhead in Type-2 CB. That is, if 13 TBs are scheduled, then the Type-2 CB includes 13 bits without any filler bits. Therefore, in terms of Type-2 CB size, Alt 2 would be the best among the three alternatives. However, it requires a greater number of bits for C-DAI and T-DAI than the number of bits as in Rel-15 to keep the same DTX capability as in Rel-15/16. Note that in Rel-15/16 up to 3 consecutive DTXs can be detected by 2-bit C-DAI and 2-bit T-DAI. For example, if 3-bit C-DAI and 3-bit T-DAI are used, then (C-DAI,T-DAI)=(4,4) in the DCI of CC#1, (C-DAI,T-DAI)=(7,3) in the DCI of CC#0, (C-DAI,T-DAI)=(3,3) in the DCI of CC#3, and (C-DAI,T-DAI)=(5,5) in the DCI of CC#2. If the DCIs of CC#0/1/3 are missed (consecutive 3 DTXs), then the UE generates 5-bit Type-2 CB since (C-DAI,T-DAI)=(5,5) in the DCI of CC#2 is detected. Note that 4-bit C-DAI and 4-bit T-DAI can address this issue.

In the conclusion of Alt 2, there are the following four FFS points.

- Q1) FFS: details on increment of DAI field size

  - In Figure 1, to detect up to 3 consecutive DTXs, 4-bit C-DAI and 4-bit T-DAI are used. In general, the number of additional bits for C-DAI and T-DAI, respectively, are ceil(log2(the maximum number of SLIVs in a row in TDRA)) to detect up to 3 consecutive DTXs. However, the small number of additional bits may be also acceptable in some cases, where the maximum number of SLIVs in a row in TDRA is much larger than other rows. So, rather than fixing the number of additional bits or deriving from the maximum number of SLIVs in a row in TDRA, it would be better to configure the number of additional bits by a gNB for more flexiblity.

- Q2) FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI

  - For example, DCI format 1_0 has 2-bit C-DAI, of which range is 1, 2, 3, or 4, but DCI format 1_1 may have N-bit C-DAI, of which range is 1, 2, …, or 2N. Since it is hard to repurpose some bits or add new bits in the DCI format 1_0, the range of 2-bit C-DAI in the DCI format 1_0 should be extended/interpreted to N-bit C-DAI in the DCI format 1_1. The details should be further discussed.

- Q3) FFS: ordering of the PDSCHs for DAI counting

  - The ordering of DCIs is based on PDCCH monitoring occasion index and cell index. The order of PDSCHs is based on the slot index corresponding to the PDSCH.

- Q4) FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - The time domain bundling of HARQ-ACK feedback is configured, the Alt 2 is same as the Alt 1 (time domain bundling), i.e., C-DAI/T-DAI is counted per DCI.

- Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4,…).

Conclusion: (RAN1#104b-e [3]) The following is observed for alternative 3 from prior agreement. - For Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - If M equals to the maximum configured number of PDSCHs, Alt 3 is the same with Alt 1, if the same number of codebooks is assumed. - Else if M equals to 1, Alt 3 is the same with Alt 2. - Otherwise (i.e., 1<M<the maximum configured number of PDSCHs), Alt 3 is similar to Alt 2, except that - The number of HARQ-ACK bits corresponding to each DAI increases by M times. - NACK bits may be padded if the number of scheduled PDSCHs is not an integer multiple of M. - FFS: details on DAI field size - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell - In addition, new RRC parameter to configure M needs to be introduced. - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

In Alt 3, C-DAI and T-DAI are counted per M scheduled PDSCH(s), where M is configurable. The Alt 3 is a generalization of Alt 1 and Alt 2 by introducing a configurable parameter M. That is, if M=1 is configured, then the Alt 3 is same as the Alt 1, and if M is configured to the maximum number of PDSCHs scheduled by a single DCI, then the Alt 3 is same as the Alt 2. Figure 2 illustrates M=2 case, where each increment represents 2 PDSCH(s). Even if the DCI in CC#0 schedules 3 PDSCHs, then 4 bits (including 1 filler bit) are inserted in Type-2 CB. Therefore, the Type-2 CB size is 14 bits, which is middle of Alt 1 and Alt 3.

In fact, similar issues were discussed in case of CBG-based transmission in Rel-15, where the number of scheduled CBGs can be varied across different DCIs so that the number of HARQ-ACK bits for the scheduled CBGs can be also varied. Also, the number of CBGs can be configured as 2/4/8 per PDSCH and it can be different in different CCs. The pros and cons of each alternatives were intensively discussed. Finally, RAN1 agreed to support Alt 1-like approach for CBG-based transmission, i.e., C-DAI and T-DAI are counted per DCI and each increment corresponds to the maximum number of CBGs. For this issue, the same approach can be reused.

- Proposal 1: We propose to support Alt 1 as DAI counting for Type-2 HARQ-ACK CB when scheduling multiple PDSCHs in single DCI.

  - Alt 1: C-DAI/T-DAI is counted per DCI.

Issue 2: Type-1 HARQ-ACK codebook for multi-PDSCH scheduling with a single DCI

Agreement: (RAN1#104b-e [3]) For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered, - Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set - Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table - Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table - FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table

In the last RAN1#104bis-e meeting, RAN1 made the agreements on Type-1 HARQ-ACK CB for multi-PDSCHs scheduling by a single DCI.

- Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set

In Rel-15/16, Type-1 HARQ-ACK codebook is generated based on a SLIV in a row in a TDRA table and K1 values. The difference of multi-PDSCH scheduling by a single DCI is the followings:

- 1) the K1 value in a single DCI represents the slot offset between the slot of the last PDSCH scheduled by the singe DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs. So, the corresponding effective(extended) K1 values, defined by the slot offset between the slot of the PDSCH and the slot carrying the HARQ-ACK information, can be derived.

- 2) More than one SLIVs for a row in the TDRA table can be configured and the SLIVs is allocated to the consecutive or non-consecutive slots. Note that the TDRA table may not include K0 values for each SLIV, but a UE can know which slot is assigned to a SLIV in the TDRA table by a pre-determined way.

According to the above two points, when a PUCCH carrying HARQ-ACK information of multi-PDSCH scheduling is transmitted at slot n, the UE can identify which SLIVs are associated to a slot n-K1eff, where K1eff is an effective K1 value. The pseudo-code for Rel-15/16 Type-1 HARQ-ACK CB construction rule can be reused by replacing with 1), 2) as follows:

- the set K1 including the configured K1 values to the set K1 including the effective(extended) K1 values and

- the set R including all SLIVs in a TDRA table to the set R including each SLIV corresponding to the effective(extended) K1 value.

The Figure 2 illustrates option 1, where a TDRA table in a single DCI is given in Table 1 and K1 values in the single DCI are configured to {1,2}. Here, the effective(extended) K1 values are {1,2,3,4,5} due to the single can schedule up to 4 SLIVs in 4 consecutive slots, and the UE identifies that

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-5 (where the effective K1 value is 5),

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-4 (where the effective K1 value is 4),

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-3 (where the effective K1 value is 3),

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-2 (where the effective K1 value is 2),

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-1 (where the effective K1 value is 1).

Figure 2. Type-1 HARQ-ACK CB construction (Option 1)

Therefore, the pseudo-code for Type-1 HARQ-ACK CB can be performed with K1={1,2,3,4,5} and R={(0,14), (0,7), (7,7)} for all the effective(extended) K1 value {1,2,3,4,5}. For example, if K1,k=3 is selected, two HARQ-ACK occasions, one for SLIV (0,14) and SLIV (0,7) and another for SLIV (7,7) can be derived. In this example, 10 HARQ-ACK occasions (index of 10 HARQ-ACK occasions are shown in Figure 2) can be derived. Since each HARQ-ACK occasion have one SLIV, then total size of the Type-1 HARQ-ACK CB is 10 bits (assuming 1 bit per SLIV, i.e., no CBG transmission and 1-TB per PDSCH).

- Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table

Contrary to option 1, where each SLIV is used to construct Type-1 HARQ-ACK CB, option 2 is to use the last SLIV in a row in a TDRA table. It may be a simple solution with minor specification impacts because the Type-1 HARQ-ACK CB construction rule defined in Rel-15/16 is directly applicable without considering multiple SLIVs in a TDRA table. Figure 3 shows the last SLIV in each row of the TDRA in Table 1.

- the last SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-2 (where the K1 value is 2),

- the last SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-1 (where the K1 value is 1),

Note that the SLIVs other than the last SLIVs are colored as ‘gray’ and these SLIVs are not used to generate HARQ-ACK occasions in option 2.

Based on the last SLIVs, we can generate 4 HARQ-ACK occasions, where

- the HARQ-ACK occasion 0 includes TDRA row indexes 0, 1, 3, and 4 and K1=2,

- the HARQ-ACK occasion 1 includes TDRA row indexes 2, and 5 and K1=2,

- the HARQ-ACK occasion 2 includes TDRA row indexes 0, 1, 3, and 4 and K1=1,

- the HARQ-ACK occasion 3 includes TDRA row indexes 2, and 5 and K1=1.

The next step is to determine how many bits are needed for each HARQ-ACK occasion. For example, for the HARQ-ACK occasion 0, TDRA row indexes 0 and 1 include two SLIVs so that 2 bits (assuming 1 bit per SLIV, i.e., no CBG transmission and 1 TB per PDSCH) are required, while TDRA row indexes 3 and 4 include four SLIVs so that 4 bits are required. In this end, the HARQ-ACK occasion 0 needs to contain 4 bits (the maximum number of SLIVs among rows in TDRA corresponding to the HARQ-ACK occasion 0). Similarly, the HARQ-ACK occasion 1, 2, and 3 need to contain 4 bits. The total bit size of Type-1 HARQ-ACK CB is 12 bits.

Figure 3. Type-1 HARQ-ACK CB construction (option 2)

As comparing option 1 and option 2, option 1 may provide much smaller size of Type-1 HARQ-ACK codebook. In this sense, we support option 1.

- Proposal 2: For Type-1 HARQ-ACK codebook construction for multi-PDSCH scheduling by a single DCI, we support option 1.

  - Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set.

It is worth noting that the Type-1 HARQ-ACK CB construction rule in option 1 is not optimal in terms of CB size. In fact, with the TDRA table in Table 1, up to 8 HARQ-ACK occasions are enough. It is because a single DCI schedules multi-PDSCHs so that SLIVs in slots are not independent. For example, if a UE detects a DCI with TDRA index of 4 and K1=2, then the UE is, additionally, able to detect a DCI with TDRA index of 4 and K1=2 or TDRA index of 4 and K1=1. In this case, the UE receives 8 PDSCHs so that 8 HARQ-ACK occasions are needed, but the proposed Type-1 HARQ-ACK CB construction rule generates 10 HARQ-ACK occasions. However, such an overhead can be controlled by gNB’s scheduler and optimization of Type-1 HARQ-ACK CB may cause an additional UE complexity. Also, Type-2 HARQ-ACK CB can be used by configuration if the size of HARQ-ACK CB is crucial.

- Conclusion

In this contribution, we discussed enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI. The followings are proposed:

- Proposal 1: We propose to support Alt 1 as DAI counting for Type-2 HARQ-ACK CB when scheduling multiple PDSCHs in single DCI.

  - Alt 1: C-DAI/T-DAI is counted per DCI.

- Proposal 2: For Type-1 HARQ-ACK codebook construction for multi-PDSCH scheduling by a single DCI, we support option 1.

  - Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set

References

- RP-210862, Revised WID: Extending current NR operation to 71 GHz, CMCC

- Final Report of 3GPP TSG RAN WG1 #104e v1.0.0

- Draft Report of 3GPP TSG RAN WG1 #104bis-e v0.2.0