3GPP TSG RAN WG1 #105	R1-2105885

e-Meeting, May 10th – 27th, 2021

Agenda item:		8.6.2

Source:	Nordic Semiconductor ASA

Title:					On RedCap UE early identification

Document for:		Discussion and Decision

# Introduction

RAN2 did not discuss in RAN2#103b-e these topics at all:

#### 8.12.2.1 Definition of RedCap UE type and reduced capabilities

#### 8.12.2.2 Identification, access and camping restrictions

Nevertheless, we provide our initial view on Early identification.

# Early RedCap identification and barring

Regarding early indication, WID clearly states the following

- Specify definition of one RedCap UE type including capabilities for RedCap UE identification and for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. [RAN2, RAN1]

  - The existing UE capability framework is used; changes to capability signalling are specified only if necessary.

- Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1]

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

First of all, our understanding is that WID description does not explicitly preclude early identification of number of Rx branches. Early indication sub-bullet is not related to RedCap UE type per se.

Observation-1: WID itself does not preclude early identification of number of Rx branches.

When considering whether number of Rx branches should be early identified there are pros and cons. On one side, this would provide more flexibility at the gNB side, on the other side, it creates more complexity and segmentation of available resources. A gNB may want to identify RedCap UEs by separate ROs and differentiation of Rx antennas would require two separate sets of ROs. Furthermore, if different MSG3 repetition factor would be used for different UEs, four set of ROs would be needed. Therefore, we think that if separate ROs are introduced for RedCap UEs, they shall be bundled with MSG3/A repetitions and or RedCap UE type rather than Rx antennas.

Proposal-1: Support early identification of RedCap UE through separate ROs that are used to identify need for MSG3/A repetitions and/or RedCap UE type rather than to indicate the number of Rx antennas/branches by the UE.

# Conclusions

We briefly discussed early identification of RedCap UEs and we have the following observation and proposal:

Observation-1: WID itself does not preclude early identification of number of Rx branches.

Proposal-1: Support early identification of RedCap UE through separate ROs that are used to identify need for MSG3/A repetitions and/or RedCap UE type rather than to indicate the number of Rx antennas/branches by the UE.