3GPP TSG-RAN WG1 Meeting #105-e	R1-2105891

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.9.3

Source:	Ericsson

Title:	On the L2 Buffer Size for NB-IoT and LTE-M UEs

Document for:	Discussion

# 1	Introduction

The Work Item (WI) on “Additional enhancements for NB-IoT and LTE-MTC” includes the following objectives [1]:

| - Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4] - Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL, for HD-FDD Cat M1 UEs. [LTE-MTC] [RAN1] - Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4]. | - Introduce support for NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.). [NB-IoT] [RAN2, RAN3] - For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power. [LTE-MTC] [RAN4] - Add a Rel-17 optional UE capability to support a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only. [LTE-MTC] [RAN1, RAN2] - Determine soft buffer size [RAN1] - Capability signaling without introducing a new UE category [RAN2] - There shall be no changes to: DCI formats, TBS tables, CQI tables - This objective begins work from RAN#90, i.e. December 2020 |
| --- | --- |

This contribution is for information purposes only and aims at communicating the views in [2] about the “L2 Buffer Size” for NB-IoT and LTE-M UEs. The paper includes views on the “L2 Buffer Size” as stated in [2] related to the support of “NB-IoT: 16-QAM for unicast in DL”, the support of “LTE-MTC: 14 HARQ processes in DL” and the support of “LTE-MTC: a max DL TBS of 1736 bits”.

# 2	NB-IoT: L2 Buffer Size for the support of 16-QAM for unicast in DL

In RAN1 #103-e, the following agreement was reached for the support of 16-QAM for unicast in DL [3]:

Agreement At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits with ISF=7.

In terms of the Layer 2 buffer size requirements for 16-QAM, the following has been proposed in [2]:

- “For Cat NB2 UE supporting 16-QAM, the total L2 buffer size is 16000 bytes”.

# 3	LTE-MTC: L2 buffer size for the support of 14 HARQ processes in DL for HD-FDD Cat M1 UEs

For the support of 14 HARQ processes in DL for HD-FDD Cat M1 UEs the following has been proposed in [2]:

In terms of the Layer 2 buffer size requirements for the support 14 HARQ processes in DL, the following has been proposed in [2]:

- “For HD-FDD Cat M1 UEs supporting 14 HARQ processes in DL, no change is required to the current L2 buffer sizes”.

# 4	LTE-MTC: L2 Buffer Size for the support of a max DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A.

In RAN1 #104-bis-e, the following agreement was reached [4]:

Agreement The soft channel bits for UEs supporting maximum DL TBS of 1736 bits is 43008 bits.

In terms of the Layer 2 buffer size requirements for the support of a max DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A, the following has been proposed in [2]:

- For Cat M1 UE supporting max DL TBS of 1736 bits, the L2 buffer size is [24000] bytes when maximum number of UL-SCH transport block bits transmitted within a TTI is 1000 bits.

- For Cat M1 UE supporting max DL TBS of 1736 bits, the L2 buffer size is 30000 bytes when maximum number of UL-SCH transport block bits transmitted within a TTI is 2984 bits.

# 5	Overview

This document summarizes the views in [2], related to the L2 Buffer size requirements for both NB-IoT and LTE-MTC Rel-17 features.

This paper is for information purposes only. The discussion related to the L2 Buffer Size for NB-IoT and LTE-M features in Rel-17 are to take place in RAN2.

# 6	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	In terms of the Layer 2 buffer size requirements for 16-QAM, the following has been proposed in [2]:

	“For Cat NB2 UE supporting 16-QAM, the total L2 buffer size is 16000 bytes”.

Observation 2	In terms of the Layer 2 buffer size requirements for the support 14 HARQ processes in DL for HD-FDD UEs, the following has been proposed in [2]:

	“For HD-FDD Cat M1 UEs supporting 14 HARQ processes in DL, no change is required to the current L2 buffer sizes”.

Observation 3	In terms of the Layer 2 buffer size requirements for the support of a max DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A, the following has been proposed in [2]:

	For Cat M1 UE supporting max DL TBS of 1736 bits, the L2 buffer size is [24000] bytes when maximum number of UL-SCH transport block bits transmitted within a TTI is 1000 bits.

	For Cat M1 UE supporting max DL TBS of 1736 bits, the L2 buffer size is 30000 bytes when maximum number of UL-SCH transport block bits transmitted within a TTI is 2984 bits.

Observation 4	This paper is for information purposes only in relation to L2 Buffer Size for NB-IoT and LTE-M UEs in Rel-17. The discussion related to the L2 Buffer Size are to take place in RAN2.

# References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.

R2-2106158, “Total L2 Buffer Size for NB-IoT and LTE-M UEs,” Ericsson, 3GPP TSG RAN WG2 Meeting #114-e, e-Meeting, May 19th – 27th, 2021.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #103-e, e-Meeting, October 26th – November 13th, 2020.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #104-bis-e, e-Meeting, April 12th – 20th, 2021.