3GPP TSG RAN WG1 #105-e	R1-2105896

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.4

Source:	Ericsson

Title:	Corrections to Mode 1

Document for:	Discussion, Decision

# 1	Discussion

In this contribution we discuss one issue related to SL HARQ-ACK reporting in uplink in SL Mode 1.

# 2	Correction on Mode 1

During the preparatory phase of RAN1#104bis-e, the following issue was identified:

M1-3-2. SL HARQ-ACK reporting in UL when the SL transmission (scheduled by DG/CG) does not use SL HARQ feedback

Due to limitations in the e-mail thread budget, the issue was not discussed during the meeting.

Related to this issue, the following agreement was made in RAN1#100bis-e:

Agreements: - The working assumption (as in proposal 4 in the summary) from RAN1#100-e is confirmed. - If the SL transmission does not use SL HARQ feedback (if supported by RAN2), the UE reports NACK to request further resources for blind retransmission and ACK otherwise.

In the RAN2 specification (TS 38.321), the agreement has been implemented as follows:

##### 5.22.1.3.2 PSFCH reception The MAC entity shall for each PSSCH transmission: 1> if an acknowledgement corresponding to the PSSCH transmission in clause 5.22.1.3.1a is obtained from the physical layer: 2> deliver the acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process; 1> else: 2> deliver a negative acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process; 1> if the PSSCH transmission occurs for a pair of Source Layer-2 ID and Destination Layer-2 ID corresponding to a PC5-RRC connection which has been established by upper layers: 2> perform the HARQ-Based Sidelink RLF Detection procedure as specified in clause 5.22.1.3.3. If sl-PUCCH-Config is configured by RRC, the MAC entity shall for a PUCCH transmission occasion: 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB. 1> else if a MAC PDU has been obtained for a sidelink grant associated to the PUCCH transmission occasion in clause 5.22.1.3.1, the MAC entity shall: 2> if the most recent transmission of the MAC PDU was not prioritized as specified in clause 5.22.1.3.1a: 3> instruct the physical layer to signal a negative acknowledgement on the PUCCH according to clause 16.5 of TS 38.213 [6]. 2> else if HARQ feedback has been disabled for the MAC PDU and next retransmission(s) of the MAC PDU is not required: 3> instruct the physical layer to signal a positive acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [6]. 2> else if HARQ feedback has been disabled for the MAC PDU and no sidelink grant is available for next retransmission(s) of the MAC PDU, if any: 3> instruct the physical layer to signal a negative acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [6]. 2> else: 3> instruct the physical layer to signal an acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [6] 1> else: 2> instruct the physical layer to signal a positive acknowledgement on the PUCCH according to clause 16.5 of TS 38.213 [6].

However, there is currently no corresponding text in the RAN1 specifications (i.e., Clause 16.5 of TS 38.213). To correct this, we provide the following TP.

Adopt the following TP for TS 38.213 dealing with reports of SL HARQ-ACK information on uplink for the case that SL HARQ feedback is disabled.

## ============================= Start of text proposal for 38.213 ========================== ## 16.5 UE procedure for reporting HARQ-ACK on uplink <Unchanged parts omitted> The UE generates a NACK when, due to prioritization, as described in Clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH transmission. The UE generates a NACK when, due to prioritization as described in Clause 16.2.4, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to prioritization. The UE generates an ACK if the UE does not transmit a PSCCH with a SCI format 1-A scheduling a PSSCH in any of the resources provided by a configured grant in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the ACK is same as the largest priority value among the possible priority values for the configured grant. <Unchanged parts omitted> For reporting HARQ-ACK information on uplink corresponding to one ore multiple PSSCH transmissions with a corresponding SCI format with the field ‘HARQ feedback enabled/disabled indicator’ set to disabled, the UE generates HARQ-ACK information with the contents instructed by higher layer. <Unchanged parts omitted> ## ============================= End of text proposal ==========================

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the following TP for TS 38.213 dealing with reports of SL HARQ-ACK information on uplink for the case that SL HARQ feedback is disabled.

## ============================= Start of text proposal for 38.213 ========================== ## 16.5 UE procedure for reporting HARQ-ACK on uplink <Unchanged parts omitted> The UE generates a NACK when, due to prioritization, as described in Clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH transmission. The UE generates a NACK when, due to prioritization as described in Clause 16.2.4, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to prioritization. The UE generates an ACK if the UE does not transmit a PSCCH with a SCI format 1-A scheduling a PSSCH in any of the resources provided by a configured grant in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the ACK is same as the largest priority value among the possible priority values for the configured grant. <Unchanged parts omitted> For reporting HARQ-ACK information on uplink corresponding to one ore multiple PSSCH transmissions with a corresponding SCI format with the field ‘HARQ feedback enabled/disabled indicator’ set to disabled, the UE generates HARQ-ACK information with the contents instructed by higher layer. <Unchanged parts omitted> ## ============================= End of text proposal ==========================