3GPP TSG-RAN WG1 Meeting #105-e	R1-2105912

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.5.5

Source:	Ericsson

Title:	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

Document for:	Discussion, Decision

# Introduction

The updated WID for NR positioning enhancements includes the following objective

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

This contribution is towards the above objective. In this contribution, we propose a few ways to enable LOS/NLOS detection in the network.

# NLOS detection – required measurements and methods

We will have following discussions subsequently:

- Motivation for LOS/NLOS detection

- Signature based LOS/NLOS classification (based on real world measurements)

- Algorithmic LOS/NLOS detection

- Machine learning for LOS/NLOS detection

- Selecting right Channel Impulse Response (CIR) to detect NLOS using first path/peak.

- LOS detection based on rich reporting of multiple propagation paths

- Rich reporting for LOS/NLOS detection

## Motivation for LOS/NLOS detection

Other features being specified in Release 17 such as DL-AoD and UL-AoA based positioning depends heavily on LOS/NLOS detection. Many of these features being specified may remain futile or not useful in the absence of information on LOS/NLOS nature of the link.  Unlike any timing estimation which may have bounded variability of say tens of meters around its mean estimate in NLOS channel, angle estimates may have unbounded uniformly distributed variability from 0 to 360 degrees in NLOS channel.

Hence while working on angle-based positioning in this Release its very important that LOS/NLOS detection mechanism of a link must be specified too.

A NLOS channel may result in bounded timing delay variation. However, it can result in unbounded angle spread. Hence, angle-based positioning methods are more prone to errors than timing-based methods in NLOS channel. And Release 17 focuses a lot on angle-based methods.

NLOS/LOS detection mechanism are beneficial to NR positioning enhancements since:

  - It will strengthen other features being specified in Release 17. Particularly the angle-based methods

  - It will help meeting stringent positioning accuracy requirement in Release 17.

Support NLOS/LOS detection mechanisms in release 17

Even in all timing-based positioning methods, additional random bias due to non-line-of-sight condition deteriorates performance of position estimation. For example, in the InF models defined in 38.901, the mean NLOS excess delay is 48.3ns corresponding to 14.5m. NLOS links will thus rarely contribute constructively to meter or submeter accuracy positioning. With stringent requirement on positioning accuracy for IIOT scenario in SID [1] of 20 cm, LOS detection will be very crucial in meeting the requirements.

The mean NLOS excess delay in the InF models is 48.3ns corresponding to 14.5m. NLoS links will thus rarely contribute constructively to meter or submeter accuracy positioning.

To show the potential for performance improvements based on LOS detection, we have evaluated DL TDOA positioning based on genie LOS detection. In addition, we have evaluated a simple realistic LOS detection mechanism which detects a link as LOS if the first peak in the channel impulse response is also the strongest peak. In either case, position estimation is done by including only the detected LOS links. In Figure 1 and  Table 1, we show the positioning performance in the InF-DH scenario for genie LOS detection, strongest peak LOS detection and without LOS detection.

The results for strongest peak LOS detection show promising gains compared to no LOS detection, a factor 2 improvement at the 90 percentile, a factor 4 improvement at the 80 percentile and a factor 37 improvement at the 50 percentile. This comes with a method with very low complexity. To detect the strongest peak is obviously never harder than detecting the first peak and is most likely already done by UEs for other purposes. We therefore propose that RAN1 should specify reporting of the strongest peak in rel. 17.

Strongest peak LOS detection gives fantastic gains compared to no LOS detection for the InF-DH scenario, a factor 2 improvement at the 90 percentile, a factor 4 improvement at the 80 percentile and a factor 37 improvement at the 50 percentile.

The results for genie LOS detection show enormous gains in positioning accuracy compared to no LOS detection, a 47 fold improvement at the 90 percentile and a 47 fold improvement at the 50 percentile. This shows the great potential of LOS detection techniques. It also shows that there is a big potential to improve beyond the use of strongest peak based LOS detection. No LOS detection method is fool proof and to get closer to the genie LOS detection potential a combination of multiple LOS indicators will be needed. We therefore propose that RAN1 should study LOS detection techniques and reporting of LOS indicators for potential specification in Rel. 17.

Genie LOS detection give enormous gains in positioning accuracy in the InF-DH scenario compared to no LOS detection, a 69 fold improvement at the 90 percentile and a 47 fold improvement at the 50 percentile. This shows the great potential of LOS detection techniques.

Figure 1 Comparison of DL TDOA performance for the InF-DH scenario with genie LOS detection, with LOS detection based on whether the first peak is also the strongest peak, and without LOS detection. The figure to the right is zoomed in at small positioning errors.

Table 1 Comparison of DL TDOA performance for the InF-DH scenario with genie LOS detection, with LOS detection based on whether the first peak is also the strongest peak, and without LOS detection at different percentiles.

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Genie LOS detection | 0.07m | 0.11m | 0.16m | 0.26m |
| Strongest peak LOS detection | 0.09m | 0.18m | 1.9m | 8.4m |
| No LOS detection | 3.3m | 5.4m | 8.2m | 18m |

## Signature based LOS/NLOS classification (based on real world measurements)

In signature-based LOS/NLOS classification, we propose detecting specific signatures(patterns) in the measurements to detect LOS/NLOS. Below we motivate a LOS/NLOS detection method inspired from a real-world measurement dataset in LOS/NLOS scenarios Error! Reference source not found.. Figure 2 shows power delay profile in indoor factory like scenario in LOS/NLOS conditions. The zoomed version of the power delay profile is shown on the right in Fig.2 in relevant regions of the measurement.

Figure 2 Power delay profile in LOS/NLOS scenarios;  zoomed version of the PDP is shown on the right

It can be seen from above figures that in LOS scenario the highest peak of the measurement is very prominent. It is the first very clear peak arising from the measurement noise floor. Whereas, in the NLOS case, the highest peak of the measurement data is very gradual. The nature of LOS peak can be attributed to the fact that in the LOS conditions, all scattered signals arrive after the LOS component of the delay profile. Magnitudes of arriving scattered signals are always lower than the LOS reception due to higher path losses and other loss attributing effects. In NLOS cases, the highest peak can arrive possibly after many other scattering peaks. Even when the NLOS peak is prominent, typically it may be received after diffuse scattering components. Receiving diffuse scattering components along and before the prominent NLOS peak makes the rise of the peak very gradual, as evident from. Based on above discussion, following observations can be made.

LOS peak is always the first peak in power delay profile of the received signal for a LOS link.

A NLOS peak can also be a first prominent peak in a delay profile. But it appears along with smaller surrounding peaks and other components of delay profile generated from diffusely scattered signal before and after the NLOS prominent peak.

Rise of LOS highest peak is very sharp. Rise of NLOS highest peak is typically very gradual.

Pattern-matching type of NLOS identification methods are suitable when limited number of LOS TRPs are visible to the UE, i.e. when outliers dominate the dataset.

Figure 3 shows a possible implementation for detecting LOS/NLOS based on analyzing power delay profile of the received signal.

Figure 3 LOS/NLOS detection algorithm based on analyzing power delay profile of the received signal.

Based on the above discussion, we make the following observation:

Following measurements should be specified in Rel-17 to support signature-based methods. These measurements can be part of rich reporting.

  - Location and magnitude of the first peak.

  - Location and magnitude of the highest peak.

  - Components of PDP/CIR around first/highest peak.

## Algorithmic LOS/NLOS classification

Classification of measurements as LOS or NLOS and mitigation of the effects of NLOS can sometimes be done algorithmically with methods for robust estimation. The input to this kind of algorithms is typically a set of observations, like for instance TOA or TDOA measurements between different TRPs and a UE. NLOS measurements are considered as contaminated observations or outliers. While e.g. estimation with a least-squares method is highly sensitive to outliers, robust estimation methods are not. The methods work best when there is an excess of observations from which outliers can be pruned, hence algorithmic classification is particularly interesting for positioning in e.g. industrial scenarios where there is a dense deployment of TRPs, but not so interesting in sparse deployments.

In this section we show results for an adaptation of the RANdom SAmple Consensus (RANSAC) algorithm proposed in (Bolles, 1981) to the TOA/TDOA positioning method. The performance of robust estimation methods degrades as the fraction of contaminated observations increases. RANSAC can cope with a relatively large fraction of contaminated data. Algorithmic LOS/NLOS classification can benefit from rich reporting and be used in combination with e.g. signature-based LOS/NLOS detection to obtain even better performance. Here we show one such example where RANSAC is combined with a LOS/NLOS detector that compares the strength of different peaks in the power delay profile: If the first peak is the strongest, then it is likely LOS.

We evaluate the positioning performance on the InF-DH scenario specified by 3GPP in TR 38.901 with model parameters as given in TR 38.857. There are 18 TRPs in a 120x60 room which are mounted on a high level and 800 UEs are uniformly distributed over the deployment area. Each TRP broadcast DL-PRS that the UEs perform TOA measurements on by peak detection in an estimated channel impulse response. The TRPs are assumed synchronized with each other but not with the UE. The bandwidth and carrier frequency of the reference signal is 100 MHz respectively 3.5 GHz. With 800 UEs and 18 TRPs there in total 14400 TOA measurements. Figure 4 shows the TOA error distribution for LOS resp. NLOS measurements. The UE/TRP clock synchronization errors are not included in these figures, meaning that what is shown are the measurement errors and NLOS excess delays.

For LOS measurements the error distribution is well approximated by a Student´s t distribution with location, scale and shape parameters μ= 0, σ= 0.05 and ν= 1, respectively. The accuracy of these measurements in the order of decimeters, likely thanks to the high bandwidth of the DL-PRS signal. The NLOS excess delay is modeled with a log-normal distribution in the InF-DH scenario. This distribution is also plotted in Figure 4 (b) and its agreement with the simulated data is very good. Notice that the NLOS TOA errors are generally much larger than the LOS TOA errors, this is an important property for the performance of an algorithmic LOS/NLOS classifier.

Figure 4

Figure 5 shows the UE positioning error CDFs that are obtained with two different RANSAC algorithms. The figure also includes the results obtained with a least squares estimator, serving as a baseline performance reference. Moreover, the "LOS Genie'' results are obtained with a least-squares estimation based on the actual LOS measurements. Hence, these results indicate an upper limit for the accuracy that can theoretically be achieved by an estimator with a non-ideal LOS classifier.

The performance of the least-squares estimator is bad because of the large fraction of NLOS measurements in this scenario. Already the basic RANSAC algorithm gives very good improvements as compared to the LS estimator. The RANSAC+SFP algorithm combines RANSAC with the output of a signature-based LOS/NLOS classifier. The combination gives an even better positioning performance, close to the performance of “LOS Genie”.

Robust estimation methods can drastically improve the positioning performance in dense deployments with many NLOS measurements.

Robust estimation methods can be combined with signature-based LOS/NLOS detectors to achieve even better positioning performance.

Figure 5

In Figure 6 we group the total 800 UEs according to how many LOS measurements there are for each of them and show how many of the UEs that different algorithms locate to a position within 1 m from the true position.

We observe a gradually reduced performance for both the robust estimation methods as the number of LOS measurements reduce. However, the combined RANSAC+SFP method performs better when there are very few LOS measurements.

The performance of robust estimation methods reduces gradually as the fraction of LOS measurements decrease.

The use of signature-based LOS/NLOS detectors becomes more important when there are only a few LOS measurements.

Figure 6

The next table summarizes the LOS/NLOS classification performance of the methods. SFP is the detector that compares the strengths of the pdp-peaks.

The SFP classifier has a much larger fraction of false LOS classifications than the other methods. In this dense deployment scenario, algorithmic LOS/NLOS classification with RANSAC works well. However, the best performance is obtained with the combined algorithmic and signature-based detector RANSAC+SFP.

Note: In our implementation, RANSAC+SFP will always “inherit” the NLOS classifications of the SFP detector and possibly also add more. However, this need not be the case depending on how the detectors are combined.

The LOS/NLOS classification performance of robust estimation methods is good in dense deployments.

The LOS/NLOS classification performance can be further improved by combining signature-based classifiers with robust estimation techniques.

Table 2 LOS/NLOS detector statistics. In total 14400 TOA measurements are simulated according to the Inf-DH scenario, 7524 of them are in LOS conditions and the other 6876 are in NLOS conditions

| Detector | LOS |  | NLOS |  |
| --- | --- | --- | --- | --- |
|  | Correct | False | Correct | False |
| SFP | 7136 | 2250 | 4626 | 388 |
| RANSAC | 7197 | 249 | 6627 | 327 |
| RANSAC+SFP | 7016 | 32 | 6844 | 508 |
| LOS Genie | 7524 | 0 | 6876 | 0 |

When performing TOA measurements on an estimated PDP, it is not always obvious which peak to report for, for instance if the first peak is not the strongest. In this situation the first peak could be a noise peak or it can correspond to the LOS path which is attenuated from e.g. penetration of a wall. However, since algorithmic LOS/NLOS classification can resolve such confusions, it is best to always report both the TOA for the first peak and additionally the strongest peaks.

To support outlier detection types of methods (e.g. RANSAC) The UE could report TOA for the first (earliest) detected peak and additionally the strongest detected peaks.

## Machine learning based NLOS detection

We think machine learning can be a very useful tool for NLOS detection. In the following we illustrated how machine learning can be used for positioning.

### Test cases

10 test cases were evaluated, with increasing number of reported peaks:

For case 1 to 7:

- Using only the highest peaks in the PDP for LOS/NLOS detection:

  - Peak values and the delay time when the peak occurs are used as ML features

  - Number of peaks evaluated: 5, 10, 15, 20, 30, 40 or  50 (Test case 1 to 7)

For case 8 to 10:

- Using the the measured PDP first X number of samples:

  - Using X= 100 (case 8), X=150 (case 9), and all samples in the PDP (case 10)

- K-nearest neighbor method is used as it gives the best performance

### Results

Table 3 summarize the results of the evaluation.

Table 3 ML results for test cases 1-10. the true LOS/NLOS status of the channel is on the vertical axis, and the ML-estimate is on the horizontal axis.

From the results, it can be observed that the ML algorithm can match the channel LOS/NLOS status with very high accuracy.

For enabling good accuracy in Machine learning based LOS/NLOS detection, timing information of as many peaks as possible should be reported.

## Selecting right Channel Impulse Response (CIR) to detect NLOS using first path/peak.

We have seen from previous discussions the importance of first path in detecting NLOS links. The first path can be missed due to using an FFT window which are typically used for communication purposes also being used for computing CIR for positioning purposes. It is important to ensure that right CIR is used in doing NLOS detection using CIR peaks.

Fig.7 shows a NLOS impulse response where the start of impulse response, the first path falls within the FFT window. However, Fig.8 shows the case when the first path is missed as it may fall out of the FFT window. This can happen when a FFT window is placed to capture maximum energy of an impulse response, which is typically the case while placing FFT windows for communication purposes. Higher numerologies can be more prone to such issue as the length of cyclic prefix shrinks while the geometries of scenarios and the corresponding time of arrival delay remains same.

Figure 7   A typical CIR where the first path is included within the cyclic prefix. A NLOS link will be correctly detected as first peak falls within the FFT window.

Figure 8  A CIR where first path is  missed due to wrong placement of FFT window. In this case a NLOS channel will be detected as LOS channel. This can happen when the FFT windows are placed in order to capture maximum energy within the window.

In higher numerologies, cyclic prefix shrinks but delays in channel remains same. Hence, it is more probable to miss the first path in higher numerologies.

It is important to ensure that CIR used in NLOS detection is not missing the first peak.

Channel reciprocity between the network and UE can be used for avoiding the mis-detection of LOS/NLOS nature of the link.

Corroboration of CIRs of the two ends based on reciprocity principle would improve reliability of the NLOS detection and hence the position estimation.

The CIR generated at both gNB and at the UE should be corroborated using reciprocity principle for ensuring correct NLOS/LOS detection.

The UE and the gNB report the impulse responses used in NLOS detection with many peaks to the LMF to validate or improve the detection.

The LMF can provide configuration for FFT window placement while doing positioning measurements or generating measurements for NLOS detection.

NLOS/LOS detection should be done by the LMF by using CIRs from the UE and gNBs.

UEs and gNBs should indicate the method they are using to detect NLOS/LOS nature of the links to the LMF.

## Rich reporting

While release 16 provides a good suite of reference signals for positioning, the measurements specified in release 16 needs to be enriched. The enrichment of measurements in release 17 can be towards incorporating additional useful information for positioning or it can be for improving measurement quality of existing measurements.  As can be seen in the previous observations in this contribution, a detailed reporting of the channel impulse response is beneficial to many algorithms used to establish whether a line of sight link is available.

Rich multipath reporting have been discussed in release 16, and there is support for up to two additional paths to be reported with the timing measurements (RSTD, UE RxTX Time Diff, gNB RxTx Time Diff and RTOA). However, in release 17, to make rich reporting useful and to meet stringent accuracy requirements, we need to increase the number of additional paths reported and unambiguously define what additional paths and what metrics the UE shall report.

Rich multipath reporting can include many parameters of the received signal which provides additional information helpful for positioning. Such parameters can be magnitudes of peaks SNRs of the peaks, Doppler frequencies, angle of arrival etc. of every reported multipath.

In order for rich reporting of additional paths to be at all useful, it’s necessary to define unambiguously what additional paths the UE should report. Different paths can be useful for different purposes, e.g.

- First path. Gives the TOA of the LOS path in case of LoS and the TOA closest to LOS in case of NLOS.

- Strongest path. Gives information that can be used for LOS detection (If the first path isn’t also the strongest path it can be assumed to be NLOS). It can be measured with best precision, and is useful for fingerprinting techniques.

- N strongest paths with shorter delay than the strongest path. Gives useful information in case the first path is misdetected with noise, interference or an out of range path. This is useful also for fingerprinting techniques.

- N strongest paths. This is useful for fingerprinting techniques.

- Alternatively, a first path based on lower noise and interference threshold.

Based on the above discussion, we make the following proposal:

Following measurements should be specified in Rel-17 to support LOS identification methods. These measurements can be part of rich reporting.

  - Location and magnitude of the first and additional paths.

  - Location and magnitude of the highest peak.

  - Components of PDP/CIR around first/highest peak.

Magnitude, SNR, Doppler frequency, angle of arrival of every path should be reported.

It shall be unambiguously defined what additional paths a UE shall report.

The UE shall always report both the first path and the strongest path

- For enabling good accuracy in Machine learning based LOS/NLOS detection, information of as many peaks as possible should be reported.

# Conclusion

In the previous sections we made the following observations:

Observation 1	A NLOS channel may result in bounded timing delay variation. However, it can result in unbounded angle spread. Hence, angle-based positioning methods are more prone to errors than timing-based methods in NLOS channel. And Release 17 focuses a lot on angle-based methods.

Observation 2	NLOS/LOS detection mechanism are beneficial to NR positioning enhancements since:

a.	It will strengthen other features being specified in Release 17. Particularly the angle-based methods

b.	It will help meeting stringent positioning accuracy requirement in Release 17.

Observation 3	The mean NLOS excess delay in the InF models is 48.3ns corresponding to 14.5m. NLoS links will thus rarely contribute constructively to meter or submeter accuracy positioning.

Observation 4	Strongest peak LOS detection gives fantastic gains compared to no LOS detection for the InF-DH scenario, a factor 2 improvement at the 90 percentile, a factor 4 improvement at the 80 percentile and a factor 37 improvement at the 50 percentile.

Observation 5	Genie LOS detection give enormous gains in positioning accuracy in the InF-DH scenario compared to no LOS detection, a 69 fold improvement at the 90 percentile and a 47 fold improvement at the 50 percentile. This shows the great potential of LOS detection techniques.

Observation 6	LOS peak is always the first peak in power delay profile of the received signal for a LOS link.

Observation 7	A NLOS peak can also be a first prominent peak in a delay profile. But it appears along with smaller surrounding peaks and other components of delay profile generated from diffusely scattered signal before and after the NLOS prominent peak.

Observation 8	Rise of LOS highest peak is very sharp. Rise of NLOS highest peak is typically very gradual.

Observation 9	Pattern-matching type of NLOS identification methods are suitable when limited number of LOS TRPs are visible to the UE, i.e. when outliers dominate the dataset.

Observation 10	Following measurements should be specified in Rel-17 to support signature-based methods. These measurements can be part of rich reporting.

a.	Location and magnitude of the first peak.

b.	Location and magnitude of the highest peak.

c.	Components of PDP/CIR around first/highest peak.

Observation 11	Robust estimation methods can drastically improve the positioning performance in dense deployments with many NLOS measurements.

Observation 12	Robust estimation methods can be combined with signature-based LOS/NLOS detectors to achieve even better positioning performance.

Observation 13	The performance of robust estimation methods reduces gradually as the fraction of LOS measurements decrease.

Observation 14	The use of signature-based LOS/NLOS detectors becomes more important when there are only a few LOS measurements.

Observation 15	The LOS/NLOS classification performance of robust estimation methods is good in dense deployments.

Observation 16	The LOS/NLOS classification performance can be further improved by combining signature-based classifiers with robust estimation techniques.

Observation 17	To support outlier detection types of methods (e.g. RANSAC) The UE could report TOA for the first (earliest) detected peak and additionally the strongest detected peaks.

Observation 18	For enabling good accuracy in Machine learning based LOS/NLOS detection, timing information of as many peaks as possible should be reported.

Observation 19	In higher numerologies, cyclic prefix shrinks but delays in channel remains same. Hence, it is more probable to miss the first path in higher numerologies.

Observation 20	It is important to ensure that CIR used in NLOS detection is not missing the first peak.

Observation 21	Channel reciprocity between the network and UE can be used for avoiding the mis-detection of LOS/NLOS nature of the link.

Observation 22	Corroboration of CIRs of the two ends based on reciprocity principle would improve reliability of the NLOS detection and hence the position estimation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support NLOS/LOS detection mechanisms in release 17

Proposal 2	The CIR generated at both gNB and at the UE should be corroborated using reciprocity principle for ensuring correct NLOS/LOS detection.

Proposal 3	The UE and the gNB report the impulse responses used in NLOS detection with many peaks to the LMF to validate or improve the detection.

Proposal 4	The LMF can provide configuration for FFT window placement while doing positioning measurements or generating measurements for NLOS detection.

Proposal 5	NLOS/LOS detection should be done by the LMF by using CIRs from the UE and gNBs.

Proposal 6	UEs and gNBs should indicate the method they are using to detect NLOS/LOS nature of the links to the LMF.

Proposal 7	Following measurements should be specified in Rel-17 to support LOS identification methods. These measurements can be part of rich reporting.

a.	Location and magnitude of the first and additional paths.

b.	Location and magnitude of the highest peak.

c.	Components of PDP/CIR around first/highest peak.

Proposal 8	Magnitude, SNR, Doppler frequency, angle of arrival of every path should be reported.

Proposal 9	It shall be unambiguously defined what additional paths a UE shall report.

Proposal 10	The UE shall always report both the first path and the strongest path

Proposal 11	For enabling good accuracy in Machine learning based LOS/NLOS detection, information of as many peaks as possible should be reported.

# References

[1] Bolles, R. C. (1981). A RANSAC-based approach to model fitting and its application to finding cylinders in range data. IJCAI Citeseer.

[2] 3GPP. (2019). TR 38.901. Study on channel model for frequencies from 0.5 to 100 GHz.