3GPP TSG RAN WG1 Meeting #105-e                                                                                  R1-2105929

E-meeting, May 10th – 27th, 2021

Agenda Item:	8.2.3

Source:	Huawei, HiSilicon

Title:	Enhancement on PUCCH formats

Document for:	Discussion and Decision

Introduction

We address the open issues for enhancing PUCCH format 0, 1 and 4 to multiple physical resource blocks (PRBs). This includes the supported number of PRBs and the resource element (RE) mapping as discussed in Sec. 2. Furthermore, the alternatives for sequence length is treated in Sec. 3 together with extensive simulation results in the Appendix and based on observations from these sections, the proposals are contained in the concluding Sec. 4.

Arrangement of PRBs

Number of PRBs

In [1], the maximum transmit power per device and the maximum power spectral density (PSD) per MHz are defined as shown in Table 1. Using the PSD values of this table, it can be found that a device can achieve both the transmit power limit and PSD limit on frequency band 75, 75a and 75b, if the transmission bandwidth is 50 MHz.

Table 1. Frequency bands with corresponding harmonized technical conditions and implementation deadlines for short-range devices.

For the scenario where large coverage might be necessary, the UE may not be able to reach the transmit power limit if the one-PRB Rel-15 PUCCH formats 0/1/4 are used, especially for small SCS.  For example, 1 PRB with 120 kHz SCS occupies 1.44 MHz bandwidth. There is approximately a 17 dB margin to the transmit power limit if the UE transmits at the PSD limit. So in order to provide additional coverage/reliability for PUCCH formats 0/1/4, it suffices to have at most 32/8/4 PRBs for 120 kHz, 480 kHz and 960 kHz respectively, considering the number of allocated PRBs should be a product of 2, 3 or 5 from the requirement of DFT-S-OFDM and 50 MHz transmission bandwidth. These values do not consider the antenna gain and thus are an upper limit on the maximum number of PRBs.

Observation 1: The maximum number of PRBs could be limited to 32, 8, and 4, for 120 kHz, 480 kHz and 960 kHz SCS, respectively.

Simulation results are given in Appendix by Table 4-7 and Figure 4-10. Generally, we can make the following observation.

Observation 2: Based on computed Maximum Isotropic Loss (MIL), the following can be concluded:

- In most cases, larger MIL can be achieved with larger PUCCH bandwidth.

- The gain of larger PUCCH bandwidth depends on the regulations and is typically larger for USA than for EU.

Granularity of the number of PRBs

The number of PRBs is presumably going to be configured by RRC signaling. Thus, in terms of RAN1 aspects, there does not appear to be any reason to use a coarse granularity of the configurable number of PRBs. The PUCCH formats scale with the number of PRBs and it does not involve any extra standardization effort to have a finer granularity.

Observation 3: There is no obvious technical issue that motivates to use a coarse granularity for the configurable number of PRBs.

Sub-PRB interlacing

For 120 kHz SCS, it has been proposed to introduce using a subset of REs within each PRB in order to meet the effective isotropic radiate power (EIRP) limit with less REs occupied, i.e., to improve the spectral efficiency. However, this would be an optimization with large consequences on the PHY layer. It basically results in a new PUCCH and DMRS design. Orthogonality of both PUCCH sequences and DMRSs still need to be provided. For example, the DMRS is currently orthogonal over 12 consecutive REs, i.e., orthogonal on any PRB, but that will not be the case with sub-PRB interlacing if the same DMRS sequence is used. If a shorter DMRS sequence is specified in order to provide orthogonality on less than 12 REs, fewer cyclic shifts become available. Thus the multiplexing capacity may be compromised. Furthermore, in order for sub-PRB interlacing to be useful, consequently also the PUSCH may need to be modified accordingly, as it otherwise would result in large power differences between these two channels.

Observation 4: Sub-PRB interlacing is an optimization to increase the spectral efficiency which implies large changes to the PHY layer.

- New DMRS sequences with fewer cyclic shifts

- New PUCCH sequences

- Potentially introduction of sub-PRB interlacing for the PUSCH

Enhancements of PUCCH formats to multiple PRBs

Applicability of enhanced PUCCH formats

PUCCH format 0/1 can be used prior to dedicated PUCCH resource configuration through Table 9.2.1-1 in TS38.213. The merits of the enhanced PUCCH format 0/1 in terms of wider bandwidth are applicable also in this case. However, it suffices that the enhanced PUCCH format 4 is only applicable for dedicated PUCCH resource.

Observation 5: The enhanced PUCCH format 0/1 would offer performance benefits also when used prior to dedicated PUCCH resource configuration.

PUCCH format 0 and PUCCH format 1

The main issue is whether to select a single long sequence (Alt-1) or a repeated short sequence (Alt-2), for the DMRS and the PUCCH. Both sequence alternatives are supported in the NR specifications, thus there should not be any concern on complexity. Alt. 1 and Alt. 2 could multiplex PUCCH resources with different number of PRBs but Alt. 2 could additionally handle different starting PRB positions, as shown in Figure 1.

Figure 1. Example of DMRS multiplexing using 4 different cyclic shifts (CSs) and allocations of 1, 2, 3  and 4 PRBs. The starting PRB needs to be the same when single DMRS sequence (Alt-1) is used (left), while more DMRSs can be multiplexed with a repeated DMRS sequence, since different starting PRB positions are feasible (Alt-2).

The CM at the 95-percentile for Alt. 2 is up to 2 dB higher when the number of PRBs is less than 9 as shown in Figure 2, which is computed from all specified low-PAPR Type 1 sequences and 6-times upsampling.

Figure 2. CM at 95-percentile for different number of PRBs for the DMRS, using either a single sequence without repetition, or a repeated length-12 sequence multiplied with a phase ramp in each RB.

In these evaluations, we have for the repetition case (as was used in Rel-16 NR-U), multiplied the base sequence with a phase ramp rn=ejαnr(n) with α=2πNscRB5nRB where nRB is the resource block index within the PUCCH resource.

Observation 6: Using a repeated DMRS sequence of length-12 with phase ramp for CM/PAPR reduction offers better UE multiplexing than a single sequence. The 95-percentile CM is at most 2 dB worse for allocations below 9 PRBs.

When considering the regional constraints on maximum transmit power and computing it according to [1], the difference becomes small for the two sequence alternatives. For 120 kHz SCS, there is no difference at all for the USA, while there is at most 1 dB difference for 3-6 PRB bandwidth. Similarly for 480 kHz SCS, there is at most 1 dB difference.

Observation 7: When considering the regional limitations, there is no or very small (< 1 dB) difference in maximum transmit power between using a single sequence or a repeated sequence of length-12 with phase ramp for CM/PAPR reduction.

Table 4 includes a comparison of the link budgets for a single long sequence and a repeated sequence with CM/PAPR reduction. It shows that the detection performance is similar for both schemes.

Figure 3. Maximum transmit power according to the regulations in the USA and the EU, using a long sequence or a short sequence with repetition and phase ramp for CM/PAPR reduction, for 120 kHz SCS (left) and 480 kHz SCS (right).

PUCCH format 4

Rate matching

In Rel-15/Rel-16, the rate matching of UCI is limited by one PRB for PUCCH format 4. It is also noted in [2] that a UE is not expected to report CSI with a total number of UCI bits and CRC bits larger than 115 bits when configured with PUCCH format 4. As it is agreed that the actual number of PRBs used for enhanced PUCCH format 4 does not vary dynamically based on UCI payload, how to make full use of the additional resources to achieve further performance gain is open.

On one hand, without changing UCI payload size limitation in [2], the following rate matching options can be considered:

- One way is to do rate matching to N RBs. Thus coverage gain can be obtained by a lower code rate.

- Another way is to copy UCI on each configured RB and keep the legacy rate matching to 1 RB. In this case the reliability of UCI transmission is enhanced by repetition.

- The third way is to repeat UCI several times before rate matching to N RBs, considering both low code rate and repetition benefits.

On the other hand, if UCI payload size limitation can be removed, more UCI bits can be carried on PUCCH format 4. It means that CSI reported by a UE can be delivered with a smaller latency. Accordingly rate matching to N RBs can be adopted.

Observation 8: For enhanced PUCCH format 4, different rate matching options can be supported to achieve desired benefits, i.e. coverage, reliability and latency.

Conclusions

Based on the observations made above, we arrive at the following proposals.

Our evaluations show that the following maximum bandwidths for the PUCCH are reasonable.

Proposal 1: The maximum number of PRBs for the PUCCH is:

- For 120 kHz SCS: 32

- For 480 kHz SCS: 8

- For 960 kHz SCS: 4

We have not seen any compelling reason to have a coarse granularity for the configurable number of PRBs and, thus, we prefer Alt. 1, i.e., allow all values.

Proposal 2: Adopt Alt. 1 for the granularity of the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4.

Sub-PRB interlacing is an optimization which has too large impact to the other signals/channels.

Proposal 3: Sub-PRB interlaced mapping is not introduced for 120 kHz SCS.

The PUCCH enhancements are beneficial and should apply for both common and dedicated PUCCH resources.

Proposal 4: The enhanced PUCCH formats 0/1 are applicable both with common and dedicated PUCCH resource. The enhanced PUCCH format 4 is applicable only with dedicated PUCCH resource.

Proposal 5: For PUCCH format 4, the following rate matching options can be considered:

- Alt-1: Rate matching to N PRBs, without changing UCI limitation

- Alt-2: Copy UCI on each configured PRB and keep the legacy rate matching to 1 RB, without changing UCI limitation

- Alt-3: Repeat UCI serval times and rate matching to N PRBs, without changing UCI limitation

- Alt-4: Rate matching to N PRBs and remove UCI payload limitation

References

- EU 2019/1345, “Amending Decision 2006/771/EC updating harmonised technical conditions in the area of radio spectrum use for short-range devices”.

- 3GPP TS 38.214, “NR; Physical layer procedures for data”.

- Appendix

Table 2. Evaluation assumptions for PUCCH.

| Parameter | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz |
| Subcarrier spacing, SCS [kHz] | 120, 480, 960 kHz |
| Number of usable RBs per carrier | 256 for 120 kHz SCS (corresponds to ~400 MHz carrier) 256 for 480 kHz SCS (corresponds to ~1600 MHz carrier) 160 for 960 kHz SCS (corresponds to ~2000 MHz carrier) |
| PUCCH Frequency Hopping | On |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) |
| Waveform | CP-OFDM for PF0/1 DFT-s-OFDM for PF4 |
| CP Type | Normal CP |
| Channel Model | TDL-A, Delay spread (DS) = 10ns |
| BS Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,2} |
| UE Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,1} |
| Mobility | 3 km/hr |
| PA Model | None |
| gNB TRP PN Model | Zero phase noise |
| UE PN Model | Zero phase noise |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |

Table 3. Reporting metrics for PUCCH.

| Parameter | Value |
| --- | --- |
| PUCCH Format | PF0, PF1, PF4 |
| Subcarrier spacing, SCS [kHz] | 120, 480, 960 kHz |
| Frequency hopping details | Frequency offset 46 RBs |
| Number of RBs used per hop (N_RB) | {1, 2, 4, 8} RBs |
| PUCCH bandwidth per hop, BW [MHz] | N_RB*SCS MHz |
| Number of OFDM symbols used for PUCCH resource | 2 for PF0 14 for PF1/4 |
| Sequence construction details | Long sequence for PF0/1 Long sequence for DMRS of PF4 |
| OCC configuration details | N.A. |
| Cyclic shift configuration details | Initial cyclic shift 0 for PF0/1/4 |
| Number of multiplexed users, e.g., by code division, if applicable | 1 user |
| PUCCH payload encoder type | Reed Muller for PF4 |
| PUCCH payload size(s) (bits) | For PF4 : 11 bits |
| PUCCH encoding rate(s) | Rate matching according to Reed-Muller code |
| Required SNR (dB) | Based on link level simulation results. |
| Cubic Metric, CM (dB) | The 95-percentile, based on simulation results. |
| UE Tx Beamforming gain (dBi) | TxBF = 6dBi |
| BS Rx Beamforming gain (dBi) | RxBF = 20dBi |
| UE Power Limitations | Maximum EIRP: UE_EIRP = 25 dBm Maximum conduced power (prior to consideration of backoff): UE_P = 21 dBm |
| Pmax (dBm) | Maximum allowed conducted power considering limit per region |
| Backoff (dB) | Power backoff is equal to the cubic metric |
| Transmit power, P_TX (dBm) | Maximum allowed transmit power including UE power limitation and backoff P_TX = min(Pmax, UE_EIRP – TxBF, UE_P – Backoff) |
| Noise power, P_N (dBm) | BS Noise Figure, NF = 7 dB Noise PSD = -174 dBm/Hz P_N = Noise PSD + 10*log10(BW * 1e6) + NF |
| Maximum Isotropic Loss, MIL (dB) | MIL = P_TX – P_N – Required SNR + TxBF + RxBF |

Table 4. Comparison for PUCCH format 0 between a long sequence and a repeated short sequence with CM/PAPR reduction method.

| Sequence | Regulation | NRB | SCS [kHz] | SNR [dB] | PN [dBm] | 95% CM [dB] | PTX [dBm] | MIL [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long | EU | 4 | 120 | -4.5 | -99.4 | 1.78 | 19.00 | 148.9 |
| Long | EU | 8 | 120 | -7.4 | -96.4 | 1.59 | 19.00 | 148.8 |
| Long | USA | 4 | 120 | -4.5 | -99.4 | 1.78 | 14.60 | 144.5 |
| Long | USA | 8 | 120 | -7.4 | -96.4 | 1.59 | 17.61 | 147.4 |
| Rep. | EU | 4 | 120 | -4.4 | -99.4 | 3.30 | 19.00 | 147.5 |
| Rep. | EU | 8 | 120 | -7.4 | -96.4 | 2.37 | 19.00 | 148.4 |
| Rep. | USA | 4 | 120 | -4.4 | -99.4 | 3.30 | 14.60 | 144.4 |
| Rep. | USA | 8 | 120 | -7.4 | -96.4 | 2.37 | 17.61 | 147.4 |

Table 5. Results for PUCCH format 0 and 1 for 120 kHz SCS.

| Format | Regulation | NRB | SCS [kHz] | SNR [dB] | PN [dBm] | 95% CM [dB] | PTX [dBm] | MIL [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | EU | 1 | 120 | 1.5 | -105.4 | 0.67 | 18.58 | 148.5 |
| PF0 | EU | 2 | 120 | -1.7 | -102.4 | 0.78 | 19.00 | 149.1 |
| PF0 | EU | 4 | 120 | -4.5 | -99.4 | 1.78 | 19.00 | 148.9 |
| PF0 | EU | 8 | 120 | -7.4 | -96.4 | 1.59 | 19.00 | 148.8 |
| PF1 | EU | 1 | 120 | -6.0 | -105.4 | 0.67 | 18.58 | 156.0 |
| PF1 | EU | 2 | 120 | -8.0 | -102.4 | 0.78 | 19.00 | 155.4 |
| PF1 | EU | 4 | 120 | -11.5 | -99.4 | 1.78 | 19.00 | 155.9 |
| PF1 | EU | 8 | 120 | -14.8 | -96.4 | 1.59 | 19.00 | 156.2 |
| PF0 | USA | 1 | 120 | 1.5 | -105.4 | 0.67 | 8.58 | 138.5 |
| PF0 | USA | 2 | 120 | -1.7 | -102.4 | 0.78 | 11.59 | 141.7 |
| PF0 | USA | 4 | 120 | -4.5 | -99.4 | 1.78 | 14.60 | 144.5 |
| PF0 | USA | 8 | 120 | -7.4 | -96.4 | 1.59 | 17.61 | 147.4 |
| PF1 | USA | 1 | 120 | -6.0 | -105.4 | 0.67 | 8.58 | 146.0 |
| PF1 | USA | 2 | 120 | -8.0 | -102.4 | 0.78 | 11.59 | 148.0 |
| PF1 | USA | 4 | 120 | -11.5 | -99.4 | 1.78 | 14.60 | 151.5 |
| PF1 | USA | 8 | 120 | -14.8 | -96.4 | 1.59 | 17.61 | 154.8 |

Table 6. Results for PUCCH format 0 and 1 for 480 kHz SCS.

| Format | Regulation | NRB | SCS [kHz] | SNR [dB] | PN [dBm] | 95% CM [dB] | PTX [dBm] | MIL [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | EU | 1 | 480 | 0.3 | -99.4 | 0.67 | 19.00 | 144.1 |
| PF0 | EU | 2 | 480 | -3.0 | -96.4 | 0.78 | 19.00 | 144.4 |
| PF0 | EU | 4 | 480 | -5.3 | -93.4 | 1.78 | 19.00 | 143.7 |
| PF1 | EU | 1 | 480 | -5.9 | -99.4 | 0.67 | 19.00 | 150.3 |
| PF1 | EU | 2 | 480 | -9.0 | -96.4 | 0.78 | 19.00 | 150.4 |
| PF1 | EU | 4 | 480 | -11.6 | -93.4 | 1.78 | 19.00 | 150.0 |
| PF0 | USA | 1 | 480 | 0.3 | -99.4 | 0.67 | 14.60 | 139.7 |
| PF0 | USA | 2 | 480 | -3.0 | -96.4 | 0.78 | 17.61 | 143.0 |
| PF0 | USA | 4 | 480 | -5.3 | -93.4 | 1.78 | 19.00 | 143.7 |
| PF1 | USA | 1 | 480 | -5.9 | -99.4 | 0.67 | 14.60 | 145.9 |
| PF1 | USA | 2 | 480 | -9.0 | -96.4 | 0.78 | 17.61 | 149.0 |
| PF1 | USA | 4 | 480 | -11.6 | -93.4 | 1.78 | 19.00 | 150.0 |

Table 7. Results for PUCCH format 0 and 1 for 960 kHz SCS.

| Format | Regulation | NRB | SCS [kHz] | SNR [dB] | PN [dBm] | 95% CM [dB] | PTX [dBm] | MIL [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | EU | 1 | 960 | 0.4 | -96.4 | 0.67 | 19.00 | 141.0 |
| PF0 | EU | 2 | 960 | -1.4 | -93.4 | 0.78 | 19.00 | 140.0 |
| PF1 | EU | 1 | 960 | -5.5 | -96.4 | 0.67 | 19.00 | 146.9 |
| PF1 | EU | 2 | 960 | -8.5 | -93.4 | 0.78 | 19.00 | 146.9 |
| PF0 | USA | 1 | 960 | 0.4 | -96.4 | 0.67 | 17.61 | 139.6 |
| PF0 | USA | 2 | 960 | -1.4 | -93.4 | 0.78 | 19.00 | 139.8 |
| PF1 | USA | 1 | 960 | -5.5 | -96.4 | 0.67 | 17.61 | 145.5 |
| PF1 | USA | 2 | 960 | -8.5 | -93.4 | 0.78 | 19.00 | 146.9 |

- (a) 4 PRBs                                                    (b) 8 PRBs

Figure 4. PF0 at 120 kHz with different number of PRBs and repeated sequence.

- (a) 1 PRB                                                       (b) 2 PRBs

- (c) 4 PRBs                                                    (d) 8 PRBs

Figure 5. PF0 at 120 kHz with different number of PRBs and long sequence.

- (a) 1 PRB                                                       (b) 2 PRBs

- (c) 4 PRBs                                                    (d) 8 PRBs

Figure 6. PF1 at 120 kHz with different number of PRBs.

- (a) 1 PRB                                                       (b) 2 PRBs

- (c) 4 PRBs

Figure 7. PF0 at 480 kHz with different number of PRBs.

- (a) 1 PRB                                                      (b) 2 PRBs

- (c) 4 PRBs

Figure 8. PF1 at 480 kHz with different number of PRBs.

- (a) 1 PRB                                                      (b) 2 PRBs

Figure 9. PF0 of 960 kHz with different number of PRBs.

- (a) 1 PRB                                                      (b) 2 PRBs

Figure 10. PF1 at 960 kHz with different number of PRBs.