3GPP TSG RAN WG1 Meeting #105-e	R1-2105941

E-meeting, May 10th – 27th, 2021

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on RSS based RSRQ for LTE-MTC

Document for:	Discussion and Decision

Introduction

RAN2 has sent an reply LS [1] to RAN1 and RAN4, as a response to a previous RAN4 LS [2] regarding the RSS based RSRQ for LTE-MTC.

This contribution discusses the potential impact of the reply LS from RAN2.

Discussion

In the LS [2], RAN4 observed that RSS based RSRQ measurement is not defined in 36.214. Then two options regarding cell selection and cell reselection for cells measured based on RSS are proposed.

- Option 1: Remove RSRQ from the cell selection and cell re-selection criterion when a cell is measured using RSS.

- Option 2: Define RSRQ for RSS measurements.

And RAN4 has following statement in the LS:

RAN4 will update the RSS based measurement(s) based on RAN1/RAN2 decision, if needed.

In the RAN2 reply LS [1], RAN2 concluded that option 1 is not preferred, and leave the decision to RAN1 and RAN4.

From RAN1 point of view, we don’t see any problem if option 2 is supported. Therefore, we have the following proposal:

Proposal 1: Send RAN4/RAN2 LS to confirm that option 2 (define RSRQ for RSS measurements) is supported.

If option 2 is supported, then the RSRQ for RSS measuremnts needs to be captured in 36.214. Hence, we have the following proposal:

Proposal 2: Endorse the following text proposal to TS 36.214.

===============================Text Proposal to TS 36.214========================

- 5.1.3	Reference Signal Received Quality (RSRQ)

| Definition | Reference Signal Received Quality (RSRQ) is defined as the ratio N×RSRP/(E-UTRA carrier RSSI), where N is the number of RB's of the E-UTRA carrier RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks. E-UTRA Carrier Received Signal Strength Indicator (RSSI), comprises the linear average of the total received power (in [W]) observed only in certain OFDM symbols of measurement subframes, in the measurement bandwidth, over N number of resource blocks by the UE from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. Unless indicated otherwise by higher layers, RSSI is measured only from OFDM symbols containing reference symbols for antenna port 0 of measurement subframes. If higher layers indicate all OFDM symbols for performing RSRQ measurements, then RSSI is measured from all OFDM symbols of the DL part of measurement subframes. If higher-layers indicate certain subframes for performing RSRQ measurements, then RSSI is measured from all OFDM symbols of the DL part of the indicated subframes. If higher layers indicate measurements based on discovery signals, RSSI is measured from all OFDM symbols of the DL part of the subframes in the configured discovery signal occasions. If higher layers indicate measurements based on resynchronization signals (RSS), RSSI is measured from OFDM symbols containing the resynchronization signal in the subframes in the configured resynchronization signal occasions. The measurement bandwidth is the RB’s containing the resynchronization signal. The reference point for the RSRQ shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding RSRQ of any of the individual diversity branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

============================End of Text Proposal to TS 36.214=====================

Conclusions

We discussed the RSS based RSRQ for LTE-MTC regarding the reply LS[1], and have the following proposals.

Proposal 1: Send RAN4/RAN2 LS to confirm that option 2 (define RSRQ for RSS measurements) is supported.

Proposal 2: Endorse the text proposal to TS 36.214 in section 2.

- References

R1-2104159 (R2-2104392), Reply LS related to RSS based RSRQ for LTE-MTC, From: RAN2, To: RAN4, RAN1, May, 2021, RAN1#105-e.

R4-2103728, LS related to RSS based RSRQ for LTE-MTC, From: RAN4, To: RAN2, CC: RAN1, January, 2021, RAN4#98-e