3GPP TSG RAN WG1 Meeting #105-e	R1-2105954

e-Meeting, May 10– May 27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR |  | rev | - | Current version: | V13.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UL index for LTE-M |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS36.212, 5.1.1.1, 7.2.1, 8 and 8.4 of TS36.213 are the citation for the UL index. However, the UL index related description for BL/CE UE is not found and not needed in 7.2.1, 8 and 8.4 of TS36.213. Moreover, the UL index description for BL/CE UE can be found in 8.0 of TS36.213 and the corresponding citation is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the citation for UL index from ‘5.1.1.1, 7.2.1, 8 and 8.4’ to ‘5.1.1.1 and 8.0’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The citation 7.2.1, 8 and 8.4 for the UL index is not correct and the citation 8.0 for the UL index is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.3.1.10	Format 6-0A

<Unchanged parts are omitted>

DCI format 6-0A is used for the scheduling of PUSCH in one UL cell.

The following information is transmitted by means of the DCI format 6-0A:

-	Flag format 6-0A/format 6-1A differentiation – 1 bit, where value 0 indicates format 6-0A and value 1 indicates format 6-1A

-	Frequency hopping flag – 1 bit, where value 0 indicates frequency hopping is not enabled and value 1 indicates frequency hopping is enabled as defined in clause 5.3.4 of [2]

-	Resource block assignment – +5 bits for PUSCH as defined in [3]:

-	 MSB bits provide the narrowband index as defined in clause 5.2.4 of [2]

-	5 bits provide the resource allocation using UL resource allocation type 0 within the indicated narrowband

-	Modulation and coding scheme – 4 bits as defined in clause 8.6 of [3]

-	Repetition number – 2 bits as defined in clause 8.0 of [3]

-	HARQ process number – 3 bits

-	New data indicator – 1 bit

-	Redundancy version – 2 bits

-	TPC command for scheduled PUSCH – 2 bits as defined in clause 5.1.1.1 of [3]

-	UL index – 2 bits as defined in clauses 5.1.1.1, and 8.0 of [3] (this field is present only for TDD operation with uplink-downlink configuration 0)

-	Downlink Assignment Index (DAI) – 2 bits as defined in clause 7.3 of [3] (This field is present only for cases with TDD primary cell and either TDD operation with uplink-downlink configurations 1-6 or FDD operation. This field is reserved when the configured maximum repetition number is larger than 1 for MPDCCH, or when the higher layer parameter csi-NumRepetitionCE-r13 indicates more than one subframe)

<Unchanged parts are omitted>