3GPP TSG RAN WG1 #105-e	                                              R1-2105963

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.14.1

Source: 	Intel Corporation

Title:	Initial results for XR

Document for:        Discussion/Decision

- Introduction

In RAN1#104bis-e, substantial progress is achieved on evaluation assumptions for XR evaluations [1]. In this paper, we provide initial evaluation results.

- Evaluation assumptions

| Parameter | Value |
| --- | --- |
| Layout | Dense Urban, 21cells with wraparound ISD: 200m |
| UE distribution | 80% indoor, 20% outdoor |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25m |
| UE height | hUT = 3(nfl – 1) + 1.5 Outdoor UEs: nfl = 1 Indoor UEs: nfl ~ uniform(1,N) with N ~ uniform(4,8) |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |
| BS Antenna parameters | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |
| BS Tx Power | 44dBm for 20 MHz simulated |
| UE antenna pattern | Omni-directional, 0 dBi, |
| UE Antenna parameters | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Downtilt | 102 |
| TDD configuration | DDDSU |
| System BW | 20 MHz |
| Scheduler | MU-MIMO PF Scheduler |
| CSI | SRS based |
| Traffic | 30 Mbit/s video [6Mbits/s per 20 MHz simulated], 60 fps, [STD, Max, Min]: [10.5, 150, 50]% of mean packet size, truncated Gaussian |
| Jitter | STD = 2ms, Range [+/-4ms], truncated Gaussian |

- Initial evaluation results

Initial evaluation results for Dense Urban scenario is shown in the following figures. For all figures in this section, each marker corresponds to the following number of UEs dropped per cell: [1, 3, 5, 7, 10, 15]

Cell throughput/RU

| Figure 1: DU, 30 Mbps VR, Presence of jitter leads to lower cell throughput for a given RU, equivalently same cell throughout results in a larger resource utilization. y-axis corresponds to a 20 MHz equivalent system (5x smaller throughput than a 100 MHz system) | Figure 2: Jitter applied in all cases. Average cell throughout is proportional to the offered load. 45 Mbps VR creates 1.5x higher load compared to 30 Mbps VR given a particular number of UEs per cell (scheduler is delay unaware). y-axis corresponds to a 20 MHz equivalent system (5x smaller throughput than a 100 MHz system) |
| --- | --- |

Average Delay

| Figure 3: DU, 30 Mbps VR, Average delay measured across all UEs across all packets is negatively affected in the presence of jitter, however the difference is small | Figure 4: Jitter applied in all cases. Average delay measured across all UEs across all packets is a strong function of the offered load that a network can handle. Clearly 45 Mbps VR with more than 5 UEs leads to dramatic degradation in latency |
| --- | --- |

Average PER

| Figure 5: DU, 30 Mbps VR, Average PER (fraction of packets exceeding PDB) degrades significantly in the presence of jitter | Figure 6: Jitter applied in all cases. for the same PDB, a larger load leads to larger PER |
| --- | --- |

Fraction of successful UEs

| Figure 7: DU, 30 Mbps VR, the capacity of a system with 10 ms PDB is 8.5 UEs and 13.5 UEs with and without jitter, and that of a system with 15 ms PDB is 12.5 UEs and 15+ UEs with and without jitter. | Figure 8: Jitter applied in all cases. 45 Mbps VR with 10 ms PDB leads to quite low capacity of 3 UEs in the presence of jitter. |
| --- | --- |

# 4. Conclusions

In this contribution, we provide initial set of simulation results with the agreed traffic model and simulation assumptions for capacity KPI. Initial results show that for DU, 30 Mbps VR, the capacity of a system with 10 ms PDB is 8.5 UEs and 13.5 UEs with and without jitter, and that of a system with 15 ms PDB is 12.5 UEs and 15+ UEs with and without jitter. 45 Mbps VR with 10 ms PDB leads to quite low capacity of 3 UEs in the presence of jitter and needs further study.

# References

[1] "Draft Report of 3GPP TSG RAN WG1 #104bis-e v0.2.0"

[2] "R1-2101765, LS to on XR-Traffic Models, SA4"