3GPP TSG RAN WG1 #105-e	                                              																																		R1-2105997

e-Meeting, May 10th – 27th, 2021

Source:	Moderator (vivo)

Title:	Summary #1 of evaluation methodology for XR

Agenda Item:	8.14.2

Document for:	Discussion and Decision

- Introduction

This contribution is a summary on the email discussion on other evaluation methodology and assumptions for XR and Cloud Gaming in the contributions [1-17] submitted under AI 8.14.2.

Discussion

Template to collect evaluation results

The template to collect evaluation results are updated in the following. Companies are encouraged to check the template and update the evaluation results accordingly.

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_105-e/Inbox/drafts/8.14.2/Template%20for%20collecting%20evaluation%20results/Updated_XR_evaluation_result_template_v001.xlsx

Following is the brief summary of modifications for the template. Please check the excel sheet for more details.

- For capacity sheet

  - Remove the column of avg # UEs/cell = N, since it is not related to the capacity.

  - Add some notes for column of capacity (#UEs…) for clarification, to make sure companies have common understanding when filling the results.

  - Remove the data validation for column BR : Additional Assumptions, BB : aggregate bit rates, and AD : HARQ initial target. Companies can freely input the information for these columns.

  - A column in the beginning is added to provide some simple strings for identifying each row. Companies can put any simple string specific to the row

  - Merge App and traffic direction in Capacity tab, make a single column with following choices: DL-AR/VR, DL-CG, UL-VR/CG, UL-AR. Please do not use other custom values, e.g., XR.

- For power sheet

  - Change column FA of capacity to report C1 value (same as the definition of the column CY for C1 in capacity sheet)

  - Add some notes to column EY, FA, FC, FD and FO to make the template clearer.

  - Change the columns of PSG (%) to % mode, to make sure the input from companies is the number in % format.

  - Keep one list of data validation for the direction, i.e. DL, UL, DL+UL

Coverage evaluation

Companies’ views on coverage evaluation for XR are summarized as below.

| Ericsson [17] | Proposal 4 Reuse the end-user satisfaction criteria agreed for the capacity evaluations also for the coverage evaluations. Proposal 5 Coverage is defined as the probability that a user is satisfied when the number of users in the system is very low. |
| --- | --- |
| vivo [3] | Proposal 1: For XR/Cloud Gaming coverage evaluation, support the SLS approach as the baseline, and the LLS approach can be optional. - Step 1: Run system-level simulation(s) for capacity evaluation and collect the corresponding intermediate results, such as PER, coupling loss and distance between the UE and serving gNB, etc. for each UE dropped in the simulation(s). - Step 2: Select the satisfied UEs for each of which the requirement of PER > X% is met (i.e., more than X (%) packets are successfully transmitted in the given air interface PDB), e.g., X=99. - Step 3: Find the maximum value of coupling loss or distance between UE and serving gNB among the satisfied UEs, for UL and DL respectively. - Step 4: Determine the bottleneck of data channel, e.g., which transmission direction is more limited in coverage, DL or UL, and how far a satisfied UE can locate from its serving gNB in the more limited transmission direction. Taking AR (DL-45Mbps, UL-10Mbps) traffic in Dense Urban in FR1 as an example, the CDF curve of coupling loss is shown as below: Figure 1. Coupling loss of satisfied UEs for AR traffic in FR1 Dense Urban |
| CATT [4] | Proposal 2: RAN1 focus on capacity and power consumption evaluation for XR first and de-prioritize mobility and coverage. |
| Nokia [5] | Proposal 4: The coverage can simply be defined as the probability that an XR user is satisfied under low load conditions. Proposal 5: Rely on the agreed system-level simulation methodology for assessing the XR coverage performance without the need for new link-level performance studies. Proposal 6: The coverage for the agreed XR scenarios (i.e. Dense Urban, Indoor Hotspot) is defined as the probability of XR user satisfaction under low load conditions with only one UE per cell. Applying the same user satisfaction criteria as used for the Capacity evaluations. Proposal 7: Companies are encouraged to report the SINR CDF curves/quantiles (e.g., 5%, 50%, and 95%), and/or the average SINR values to facilitate analysis of their individual observations and conclusions. |
| QC [6] | Proposal 7: Define XR coverage via system level simulations of which results are mostly already available. XR coverage can be defined to be X-percentile (e.g., X = 95) of distance in meters from the serving cell and/or MCL for the satisfied UEs for different XR applications (AR/VR/CG) and different system loading: - High system load: # of UEs per cell is close to system capacity - Low load: # of UEs per cell is 1-3 depending on XR applications (AR, VR, CG) and deployment scenarios (InH, UMa, Dense Urban) - Noise limited scenario: Only one UE in the entire network, e.g., a single UE only in a center cell |
| OPPO [7] | Proposal 4: The evaluation on coverage of XR/CG is de-prioritized. Companies can evaluate it via link-level simulation optionally and the detailed parameter setting is up to companies. |
| Samsung [10] | Proposal 2 System-level evaluation of coverage for XR devices is de-prioritized; coverage analysis for selected code streams can be added to TR 38.838 using link-level evaluations. |
| Xiaomi [14] | Proposal 4: Link level evaluation is the baseline for coverage evaluation of XR services. |

[3] [4] [5][6] [7] [10] [14] [17] discussed the coverage evaluation for XR.

Coverage is one the potential KPIs identified in the XR SID. For XR application, coverage is an important factor that affect the user experience. It is very helpful to study the coverage performance for XR.

The main concern on coverage evaluation is that it may need a lot of effort to determine how to evaluate coverage but the time budget is limited.

For coverage evaluation, two approaches can be considered. One approach is based on system-level simulation (SLS), which is similar to capacity evaluation for XR. The other approach is based on link-level simulation (LLS), which is similar to the evaluation methodology adopted in coverage enhancement SI.

For XR coverage evaluation, it is necessary to take into account the XR traffic characteristics with certain QoS requirements. For capacity and power evaluation, XR performance is evaluated by system level simulation with the agreed traffic model and evaluation methodologies. System level simulation is able to accurately evaluate the effect of XR traffic on coverage.

Besides, to define XR coverage, it would be desirable to reuse the SLS results for capacity/power evaluation that are already available as much as possible.  In this regard, FL makes the following proposal.

FL Proposal 1: XR coverage is defined to be X-percentile (e.g., X = 95) of Coupling Loss (CL) for the “satisfied” UEs for different XR applications (AR/VR/CG) with the following:

- Option 1: High system load, i.e. # of UEs per cell is system capacity

- Option 2: Low load, i.e. # of UEs per cell is Y (e.g., Y=1).

- Option 3: Noise limited scenario, i.e. Only one UE in the entire network, a single UE only in a center cell.Note: CDF of CL for the satisficed UEs can be obtained from multiple drops, e.g., a large number of drops for Option 3.

- Please share your comment on the FL Proposal 1. Please share your view on values of X and Y. Also please share your view on which option should be baseline (note: more than one options can be baseline).

| Company | Comment |
| --- | --- |

Remaining issue for capacity evaluation

### Issue 1. Confirm the number of symbol gap for option 2 TDD configuration

| OPPO [7] | Proposal 2: Confirm the clarification: for option 2 for FR1/FR2, there is [2]-symbol gap at the end of third “D” slot of DDDUU. |
| --- | --- |
| QC [6] | Proposal 1: Confirm 2 symbol gap at the end of third “D” slot of DDDUU. |

[6][7] proposed to confirm the 2-symbol gap at the end to third “D” slot of DDDUU for TDD configuration.

It seems reasonable to adopt the 2-symbol gap at the end to third “D” slot of DDDUU. So the following proposal is suggested.

FL Proposal 2: confirm the 2-symbol gap at the end to third “D” slot of DDDUU for FR1/FR2.

- Please indicate whether you agree the FL Proposal 2 and share your comment if any.

| Company | Comment |
| --- | --- |

### Issue 2. How to determine the DL/UL capacity when DL and UL performance are evaluated independently

| FUTUREWEI [1] | Proposal 1: For DL and UL performances, only separate capacities are reported. |
| --- | --- |
| vivo [3] | Proposal 2: If a set of capacity evaluations will provide the baseline capacity performances for a corresponding set of power consumption evaluations, then these two sets of evaluations should adopt the same dependencies between DL and UL evaluations, i.e. evaluated independently or together. Proposal 3: When DL and UL performances are evaluated independently, the joint capacity for DL and UL can be the minimum between the DL capacity and UL capacity corresponding to the same set of simulation parameter values. Proposal 4: When DL and UL performances are evaluated together, a UE is declared as a satisfied UE if the UE is regarded as satisfied for both DL and UL based on the corresponding metrics collected during a simulation. |
| CATT [4] | Proposal 1: Capturing the DL and UL capacity separately is sufficient for XR performance study, and the joint capacity for DL and UL is misleading. |
| OPPO [7] | Proposal 1: For XR/CG capacity evaluation, when DL and UL performances are evaluated independently, no need to introduce the joint capacity |
| Apple [9] | Proposal 2: when DL and UL performances are evaluated independently, to determine the joint capacity for DL and UL, at least the UE dropping, pathloss, shadowing fading, and indoor status etc. for DL evaluation and UL evaluation should be consistent/identical. |
| ZTE [15] | Proposal 1: When DL and UL performances are evaluated independently, joint capacity for DL and UL is obtained by min{C_DL,C_UL}. |

[1][3][4][7][9][15] discussed the remaining issue on DL and UL evaluation for capacity.

It should be noted that in RAN1 #104-e meeting, it was agreed that at least for XR/CG capacity evaluation, for DL and UL

DL and UL performances are evaluated independently as baseline, while DL and UL performance are evaluated together optionally. So it is moderator’s understanding that no more discussion on whether to adopt joint DL and UL performances evaluation is needed.

For DL and UL evaluation independently, the remaining issue is how to obtain the joint capacity for DL and UL.

In [9], it is proposed that at least the UE dropping, pathloss, shadowing fading, and indoor status etc. for DL evaluation and UL evaluation should be consistent/identical to evaluate the joint capacity. From moderator’s understanding, it is good to align the assumptions for DL and UL evaluation if possible. However, since it is related to simulation setup issue, the simulation details can be left to company.

On the determination of DL and UL capacity, [15] proposed the joint capacity for DL and UL is obtained by min{C_DL,C_UL}, while [1] [7] think that no need to introduce joint capacity. From moderator’s understanding, whether and how to determine the joint capacity is related to the evaluation result output, i.e. how to draw observation based on the DL and UL performance evaluated independently. So, it is suggested to be discussed together with the evaluation results.

- On the determination of DL and UL capacity when DL and UL performance are evaluated independently, it will be discussed together with the evaluation results. Please share your comment if any.

| Company | Comment |
| --- | --- |

### Issue 3. Per UE KPI for multi-streams

| Huawei [2] | Proposal 1: For per UE KPI of DL multiple streams, RAN1 should evaluate multiple PSR and PDB values. Proposal 2: For per UE KPI of DL multiple streams, RAN1 agrees on the following multiple combinations of (PSR, PDB) for evaluation, other combinations of (PSR, PDB) can be optionally evaluated; Table 1. (PSR, PDB) values for DL multiple streams Index; (Packet success rate X%, PDB (ms)) of {I-stream, P-stream}. VR/AR; CG. 4; { (99.5, 7), (95, 7) }; { (99.5, 12), (95, 12) }. 3; { (99.5, 10), (95, 10) }; { (99.5, 15), (95, 15) }. 2; { (99, 15), (99, 9) }; { (99, 20), (99, 14) }. 1; { (95.5, 13), (90, 13) }; { (95.5, 18), (90, 18) }. Proposal 4: For per UE KPI of AR UL, multiple combinations of (PSR, PDB) should be evaluated. Proposal 5: For per UE KPI of AR UL, RAN1 agrees on the following multiple combinations of (PSR, PDB): - For Option 2, and Stream 2 in Option 1/3 - (PSR, PDB) pairs: (99, 10), (99, 15), (95, 60) - For Stream 2 and 3 in Option 4 Index; (Packet success rate X%, PDB (ms)) of {I-stream, P-stream}. 3; { (99.5, 10), (95, 10) }. 2; { (99.5, 15), (95, 15) }. 1; { (95.5, 60), (90, 60) }. - Additional combinations of (PSR, PDB) values can be optionally evaluated |
| --- | --- |
| vivo[3] | Proposal 5: When DL and UL performances are evaluated together, a UE is declared as a satisfied UE if the UE is regarded as satisfied for both DL and UL based on the corresponding metrics collected during a simulation. Proposal 6: A UE with multiple streams is declared as a satisfied UE if each stream from the multiple streams has been satisfied, i.e. for each stream more than X (%) of packets are successfully transmitted within a given air interface PDB, where the X value and the given air interface PDB can be set per stream. |
| IDC [13] | Proposal 5: Optionally, when evaluating capacity of a UE supporting M data flows, all the data flows can have common PDB which is set to be the lowest of all the PDB values. |
| MTK [11] | Observation 1: The evaluation methodology of DL two-stream traffic model has been discussed in our contribution R1-2105376 [2] in 8.14.1. Observation 2: For UL capacity evaluation, since the UL data rate of video is much larger than the pose/control, Option 1 and Option 2 should provide similar results. For UL power evaluation, since the arrival period of pose/control (4ms) is smaller than video (16.7ms), Option 1 should provide considerably larger power consumption than Option 2. |

[2][3][11][13] discussed the per UE KPI for multi-stream evaluation. Since per UE KPI is related to multi-stream traffic modelling, it will be handled in the traffic model discussion.

Moderator’s comment: Per UE KPI for multi-stream is to be discussed in traffic model session.

Remaining issue for UE power consumption evaluation

### Issue 4. How to model UE with less than 0 dBm transmit power

| FUTUREWEI[1] | Proposal 2: UE with transmit power less than 0 dBm is considered for power consumption evaluation. It is up to companies to choose/report the transmit model used in this case. |
| --- | --- |
| ZTE[15] | Proposal 2: Adopt the power model of 0 dBm for UE with transmit power less than 0 dBm. |
| vivo[3] | Observation 1: The probability that UL transmit power is lower than 0dBm is up to 90% in Indoor hotspot scenario with 20Mbps and 60fps for uplink video traffic, which cannot be ignored. Proposal 7: The case where UE transmits with power less than 0dBm can be considered, and the linear interpolation method can be extended with extrapolation. |
| Nokia[5] | Observation 4: The overall UE device power consumption with the Tx power less than 0 dBm is primarily dominated by other factors than the actual transmit power of less than 1 mW. Observation 5: The overall UE device power consumption with the Tx power less than 0 dBm, when following Option 1 or Option 2, differs by no more than 1%, which is a marginal difference. These two models are effectively equivalent for our particular case. Proposal 8: Agree on a unified UE power model for the UEs with the Tx power less than 0 dBm. Proposal 9: Adopt either Option 1 or Option 2 for the UEs with the Tx power less than 0 dBm, as these two give almost identical results (less than 1% difference). Option 2 here may be slightly preferable, as it is simpler in terms of implementation complexity. |
| Samsung [10] | Proposal 3 UEs with observed Tx power < 0 dBm should be modeled at system-level by reusing the power model of 0 dBm. |

In RAN1 #104b-e, there was discussion on whether and how to model UE with transmit power less than 0 dBm.

Based on the previous discussion, it is common understanding that UE with transmit power less than 0 dBm should be considered. On the modeling of UE power less than 0 dBm, following options were identified.

    - Option 1: Extrapolation is adopted for UE with transmit power less than 0 dBm

    - Option 2: Adopt the power model of 0 dBm for UE with transmit power less than 0 dBm

In RAN1 #105-e, [1] [3] [5] [10] [15] discussed how to model UE with transmit power less than 0 dBm.

In [5], it is pointed out that there is less than 1% difference between these two options.

Since option 2 is simple in terms of implementation, so option 2 is recommended.

FL Proposal 3: UE with transmit power less than 0 dBm is considered for power consumption evaluation, adopt option 2 as baseline, i.e. the power model of 0 dBm for UE with transmit power less than 0 dBm.

- Other option, e.g. option 1, can be optionally evaluated

- Please indicate whether you agree on the FL Proposal 3 and share your comment if any.

| Company | Comment |
| --- | --- |

### Issue 5. UE power model for FR2

| QC [6] | For FR2 1TX configuration, we propose that the UE UL power consumption model is a linear function of the EIRP in the linear scale. The linear function, P(X) is given as P(X) = 0.07*X + 350, 0dBm <= 10*log10(X) <= 35dBm. Proposal 5: For FR2 2TX configuration, we propose a scaled of the 1TX power model. The scaling factor is 1.10. |
| --- | --- |
| ZTE [15] | - For FR2, re-use the 350 relative power in case no new power state is further defined for 23dBm. |
| Samsung [10] | Proposal 4 The UE power model for FR2 should use linear interpolation based on EIRP instead of Tx power. |

[6][10] [15] discuss the UE power model for FR2. For FR2, the UL UE power consumption for the PUSCH/PUCCH in TR 38.840 is specified as one value, i.e. 350 relative power and there is no corresponding transmit power. To evaluate the UE power consumption in FR2 more accurately, a model to cover a wider range of UE transmit power would be needed. In [6], a power consumption model based on a linear function of EIRP in the linear scale is provided for UE power model in FR2.

FL Proposal 4: For FR2 1TX configuration, the UE UL power consumption model is a linear function of the EIRP in the linear scale. The linear function, P(X) is given as P(X) = 0.07*X + 350, 0dBm <= 10*log10(X) <= 35dBm

- For FR2 2TX configuration, the power model is scaled by the 1TX power model, where the scaling factor is [1.10]

- Please indicate whether you agree on the FL Proposal 4 and share your comment if any.

| Company | Comment |
| --- | --- |

### Issue 6. Other issues on power evaluation

There are other aspects related to power evaluation discussed by a few companies in the following.

- Details of Linear interpolation based power consumption estimate method for FR1

| QC [6] | Linear interpolation based power consumption estimate method for FR1: We have used following two points (1mW, A=250) and (200mW, B=700) defined in 38.840 as two end points and applied linear interpolation or extrapolation for tx power other than 1mW (0dBm) and 200mW (23dBm) in linear domain as shown in Figure 2 (left). Figure 2 (right) shows the representation of dBm domain. Figure 2 Linear interpolation in linear domain (left) and representation in dBm domain (right) |
| --- | --- |

- Recommended number of UE per cell for XR UE power consumption evaluation in lightly load

| QC [6] | Proposal 2: Use the number of UEs/cell N=3 for lightly loaded case for power evaluation. |
| --- | --- |

- UL power state

| ZTE | Proposal 4: For XR evaluation, the following two alternatives for UL power states can be considered. Alt 1: Use the original power states and do not add more power states. Alt 2: Power of PUCCH or PUSCH with different number of symbols can be obtained by power of long PUCCH or PUSCH multiplied by a coefficient. The coefficient can be calculated by C = 0.3 + (N-1)/13*0.7, N is the number of symbols the UL power state occupied. Proposal 5: Power of “PDSCH+PUCCH” should be 450 at 23dBm, and power of “PDCCH+PDSCH+PUCCH” should be 470 at 23 dBm. |
| --- | --- |

- Power modelling of S slot

| ZTE [15] | - Proposal 5: Power of “PDSCH+PUCCH” is 450 units at 23dBm, and power of “PDCCH+PDSCH+PUCCH” is 470 units at 23 dBm. |
| --- | --- |

- Please share your comments on the other issues of power evaluation, if any.

| Company | Comment |
| --- | --- |

Mobility evaluation

Companies’ views on mobility evaluation for XR are summarized as below.

| Ericsson[17] | In the XR evaluation SI, RAN1 should treat cloud gaming with first priority, AR use cases with second priority, and VR use cases with third priority. Proposal 2 Inter-cell mobility is evaluated analytically by describing the currently specified mobility procedures from an XR service point of view, relying on the agreed traffic models and user satisfaction criteria. Proposal 3 Further consider if some restricted simulation setup could complement the analytical evaluation. |
| --- | --- |
| IDC[13] | Proposal 4: As a start prioritize low mobility scenarios with pedestrian walking speed, i.e., maximum of 3 km/h – 5 km/h. |
| Intel[8] | - Proposal-1: For impact of mobility events on XR performance, the L1-mobility EVM from Release-17 MIMO can be adopted as a baseline. - Proposal-2: Consider defining the following KPIs for mobility evaluations: - KPIs for capacity evaluation - RSRP Distribution - Beam Switching latency (FR2) |
| DCM[16] | Proposal 1: It is suggested that mobility evaluation is conducted in this study item to see the performance and whether any enhancement on mobility is needed for XR services. Proposal 2: - The following mobility speed can be considered for XR mobility evaluations: - Pedestrian (e.g. 3 km/h), vehicular (e.g. 60 km/h), and HST (e.g. 300 km/h or 500 km/h) Proposal 3: - Further discuss whether analytical evaluation or system level evaluation based on Rel-17 MIMO mobility study is used for XR mobility evaluation. |
| vivo [3] | Proposal 8: For XR/Cloud Gaming mobility evaluation, capacity performance changes or impacts due to handover procedures can be considered firstly. Proposal 9: For XR/Cloud Gaming mobility evaluation, study the method for evaluating capacity performance loss due to handovers. |
| CATT[4] | Proposal 2: RAN1 focus on capacity and power consumption evaluation for XR first and de-prioritize mobility and coverage. |
| Samsung [10] | Proposal 1 System-level evaluation of mobility for XR devices is de-prioritized and captured analytically in TR 38.838. |
| Xiaomi [14] | Proposal 5: Mobility events, e.g. handover and RLF, should be considered for the evaluation of XR services. |
| Nokia [5] | Proposal 1: There no need for RAN1 to conduct advanced dynamic system-level simulations to assess the XR handover performance at this point of time. Note also that it is typically RAN2 and RAN4 conducting detailed handover system-level performance analysis. Proposal 2: Conduct simple analytical analysis of the number effected XR frames for the different agreed XR traffics, adopting the requirements for the interruption time from 3GPP TS 38.133, considering traditional HO, CHO, and DAPS (FR1 only). Based on that, simple conclusions can be drawn on how this will impact the XR QoS/QoE, including potential pointers for possible enhancements. Proposal 3: When later concluding on the recommended CDRX setting to best leverage the tradeoffs between XR service QoE (or XR system capacity) and UE power consumption, it shall be checked by means of simple analytical calculation how this influence on the timing of the HO. |

In [3] [4] [5] [8] [10] [13] [14] [17], mobility for XR is discussed. As mobility is one of the potential KPI identified in the SID, it is helpful to evaluate the mobility performance for XR.

In [8], it is proposed to study the mobility for XR by System level mobility evaluations using Rel-17 MIMO mobility study as the starting point.

In [5] [10] [17], it is proposed that analytical evaluation based on the mobility procedures and from XR service’s perspective is used for mobility evaluation for XR. For example, conduct simple analytical analysis of the number effected XR frames for the different agreed XR traffics, adopting the requirements for the interruption time from 3GPP TS 38.133, considering traditional HO, CHO, and DAPS (FR1 only)

In [3], a simplified evaluation method with 2 steps is proposed for XR mobility evaluation, Step 1: Collect critical statistics based on system-level simulation(s) and Step 2: Analyze capacity performance loss due to handovers based on critical statistics.

- Regarding the evaluation method for XR mobility, please share your views on the mobility evaluation based on the system level mobility evaluation or the above analytical evaluation. Also, please share your views on the performance metrics (KPI) for XR mobility evaluation.

| Company | Comment |
| --- | --- |

Other issues

### Issue 7.  Prioritization of use cases/scenarios for XR capacity and power consumption evaluations

5 companies discuss the prioritization of use cases for XR evaluation []. Companies’ views on prioritization for XR evaluation are summarized as follows.

We have discussed the potential prioritization of use cases for XR evaluation in RAN1 #104b-e. There is no consensus in RAN1 #104b-e. It can be seen that there are different preferences on the interested use cases among the companies. On the other hand, there also was comment by some companies that no prioritization is needed. From moderator’s understanding, companies can choose the use cases and scenarios from their own interest.

| Ericsson | - In the XR evaluation SI, RAN1 should treat cloud gaming with first priority, AR use cases with second priority, and VR use cases with third priority. |
| --- | --- |
| Xiaomi | Proposal 1: In-door deployment can be prioritized for the evaluation of VR Proposal 2: Both indoor and outdoor deployment scenarios should be considered for AR and cloud gaming services Proposal 3: Both AR and CG should be prioritized for power consumption evaluation |
| IDC | Proposal 1: Prioritize Indoor deployment scenario for VR evaluations Proposal 2: Prioritize Dense urban deployment scenario for AR and CG evaluations Proposal 3: Prioritize FR1 in XR/CG evaluations and FR2 should be optional |
| LG | Proposal 1: - For VR1 and VR2 applications, Indoor hotspot is prioritized - For AR1 and AR2, Dense urban and Urban macro are prioritized - For CG, Dense urban [and Indoor hotspot] is[/are] prioritized - FR1 is prioritized for some of combinations of deployment scenarios and applications, e.g., AR1 and AR2 Proposal 2: Prioritize the following combinations of TDD configurations and XR/CG applications. - DDDUU for AR applications for both FR1 and FR2 - DDDSU for VR/CG applications for both FR1 and FR2 Observation 1: XR power evaluation based on Option 1 (all UEs are considered) is more realistic while not requiring additional simulation work. Proposal 3: For power consumption evaluation, prioritize AR in Dense urban and Urban macro deployment scenarios - FR1 is prioritized over FR2 |
| OPPO | Proposal 5: For XR/CG evaluation for NR - FR1: Dense urban and indoor hotspot are prioritized - FR2: Dense urban is prioritized Proposal 6: Prioritization of AR/VR/CG is not needed - If prioritization of AR/VR/CG is needed, AR/CG should be prioritized. |

- Please share any other comments if any.

| Company | Comment |
| --- | --- |

Summary

List of contributions in RAN1 #105-e

- R1-2104208	XR evaluation methodology	FUTUREWEI

- R1-2104239	Evaluation methodology for XR and Cloud Gaming	Huawei, HiSilicon

- R1-2104396	Discussion on evaluation methodologies for XR	vivo

- R1-2104499	Evaluation methodology and performance index for XR	CATT

- R1-2104556	Development of the Evaluation Methodology for XR Study	Nokia, Nokia Shanghai Bell

- R1-2104702	Remaining Issues on Evaluation Methodology for XR	Qualcomm Incorporated

- R1-2104746	Discussion on the XR evaluation methodology	OPPO

- R1-2104935	Evaluation Methodology for XR	Intel Corporation

- R1-2105135	Remaining issues in XR evaluation methodology	Apple

- R1-2105343	Evaluation methodology and KPIs for XR	Samsung

- R1-2105377	On Evaluation Methodology for XR and CG	MediaTek Inc.

- R1-2105444	Discussion on evaluation methodologies for XR	LG Electronics

- R1-2105500	Discussion on additional issues on XR Evaluations Methodology and KPI	InterDigital, Inc.

- R1-2105548	Discussion on remaining issues of evaluation methodology for XR services	Xiaomi

- R1-2105604	Further Discussion on XR Evaluation Methodology	ZTE, Sanechips

- R1-2105727	Discussion on evaluation methodology for XR	NTT DOCOMO, INC.

- R1-2105830	Evaluation methodology for XR	Ericsson

Appendix (previous agreements)

- RAN1 #103-e

Agreement: XR applications

RAN1 confirms that diverse applications of VR1/2, AR1/2, (XR conference FFS), CG are of interest for study. Potential prioritization/down selection of these applications for evaluation is to be discussed after detailed traffic models and relevant evaluation assumptions are stable.

- FFS: other applications, e.g., XR conferencing

Agreement: Traffic model

Traffic model for DL and UL should reflect various aspects, e.g., various bit rates, variable frame/packet (definition of frame/packet to be clarified with traffic model as necessary) size, and periodicity (how to model jitter is FFS).  RAN1 will strive to conclude on detailed traffic models in the next RAN1 meeting (104-e) where SA4 outcome on traffic model is expected to be available.

- Statistical model is preferred.

- It is preferred traffic model for both UL and DL have a certain degree of variability so thatand the total number of traffic models can be reduced.

- Note: Taking into account the fact that the decision on traffic models may hold many other crucial decisions, discussion on traffic model in the next RAN1 meeting is prioritized from the beginning.

Agreement:

Adopt the following deployment for XR/CG evaluations

- Indoor hotspot: FR1 and FR2

  - Detailed definition of Indoor hotspot refers to TR 38.913.

  - Channel model: InH. Detailed definition of InH refers to TR 38.901.

- Dense urban: FR1 and FR2

  - Detailed deployment refers to TR 38.913, where single layer with Marco layer is assumed.

  - Channel model: UMi. Detailed definition of UMi refers to TR 38.901.

FFS: Whether to prioritize FR1 for evaluation.

Note 1: When selecting the deployment and evaluation assumptions for XR/CG evaluations, it is up to company to evaluate FR1 or FR2 or both for the frequency range.

Note 2: It does not mean that all applications are evaluated for all the deployment scenarios.

Agreement:

Urban Macro can be optionally reported for XR/CG evaluations only for FR1.

- FFS: whether Uma is optional or not

- Following parameters can be assumed.

| Parameter | Proposed value |
| --- | --- |
|  | Urban Macro (FR1) |
| Layout | 21cells with wraparound ISD = 500 m |
| BS Tx power | FR1: 49 dBm/20 MHz |

Agreement:

It is to be further discussed how to prioritize the combinations of deployment scenarios and applications after traffic models for each application are stable.

Agreement:

System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

FFS: how to calculate the percentage of satisfied users across multiple drops of simulations

Agreement:

- Adopt the simulation assumptions in table 1 as below

Table 1: Simulation assumptions for XR evaluation (Part 1) (updated)

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor hotspot FR1/FR2 | Dense urban FR1/FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21cells with wraparound ISD: 200m |
| Carrier frequency | FR1: 4 GHz FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz FR2: 120 kHz |  |
| BS height | 3m | 25m |
| UE height | hUT=1.5 m |  |
| BS noise figure | FR1: 5 dB FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB FR2: 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic FFS:Ideal(optional) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, FR2: UE antenna radiation pattern model 1, 5dBi |  |

Agreement:

Adopt the following UE distribution for XR/CG evaluation for outdoor scenario

- For outdoor scenario:

  - FR1: 80% indoor, 20% outdoor

  - FR2: 100% outdoor

Other UE distribution can be evaluated optionally.

Agreement:

Adopt the following TDD configuration for XR/CG evaluation

- FR1:

  - Option 1: DDDSU

  - Option 2: DDDUU

- FR2:

  - Option 1: DDDSU

FFS detailed S slot format

Note: Other TDD configuration or FDD can be optionally evaluated.

Agreement:

Adopt the following BS antenna parameters for indoor scenario for XR/CG evaluation

- FR1:

  - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4)

  - (dH, dV) = (0.5, 0.5)λ

- FR2:

  - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1)

  - (dH, dV) = (0.5, 0.5)λ

Other BS antenna parameters can be optionally evaluated

Agreement:

For XR/CG evaluation, adopt the following assumptions for downtilt

- Dense Urban

  - FFS: 6 or 12 degree

  - Other downtilt can be optionally evaluated.

- Indoor hotspot

  - 90° (pointing to the ground)

Other downtilt can be optionally evaluated

Agreement:

- Adopt the simulation assumptions in table 3 as below

Table 3: Simulation assumptions for XR evaluation (Part 3)

| Power control parameter | Companies should report |
| --- | --- |
| Transmission scheme | Companies should report, such as Type I/II codebook, rank assumption |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, Assumptions on SRS: periodicity, processing gain, processing delay, etc and etc. |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |

Agreement:

The following aspects are to be discussed after traffic model is stable.

- For the system capacity definition, how to determine whether a UE is satisfied or not is to be deferred until the exact traffic model along with how to measure E2E user experience is available. Additional metrics to be collected will be further discussed after traffic model is stable.

- Various options for traffic arrival offset among UEs per cell were proposed by companies, e.g., even offset, random offset, no offset. It will be discussed after traffic model is determined.

Agreement:

System bandwidth for XR/CG evaluations are as follows.

- For FR1,

  - Baseline: 100 MHz

  - Optional: 20/40 MHz (FFS: 200 MHz)

- FFS FR2

Agreement:

For outdoor scenarios, the baseline BS antenna parameters are as follows.

- FFS FR1,

  - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

  - Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2)

  - Option 3: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1,4,4)

(dH, dV) = (0.5λ, 0.85λ)

- FR2:

  - TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

(dH, dV) = (0.5λ, 0.5λ)

Other configurations can be optionally evaluated.

Agreement:

UE antenna parameters for XR/CG evaluations are as follows

- FR1:

  - Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ

  - Optional: 4T/4R, 1T/2R, 2T2R

- FFS FR2: down-selection between the next two options. Please indicate if you have preference.

  - Option 1 (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151)

    - (M, N, P)=(1, 4, 2), 3 panels (left, right, top)

    - (Mp, Np) is up to company. Need to be reported with simulation result.

  - Option 2 (from TR 38.802 – developed in Rel-14)

    - 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

Agreement:

BS Tx power for XR/CG evaluations are as follows

- For Indoor hotspot:

  - FR1:

    - 24 dBm per 20 MHz

  - FR2:

    - 23 dBm per 80 MHz. EIRP should not exceed 58 dBm

- For Dense urban:

  - FR1:

    - 44 dBm per 20 MHz

  - FR2:

    - 40 dBm per 80 MHz. EIRP should not exceed 73 dBm

For system BW larger than above, Tx power scales up accordingly.

Agreement:

UE max Tx power for XR/CG evaluations are as follows

- FR1: 23 dBm

- FR2: 23 dBm, maximum EIRP 43 dBm

Agreement: Baseline power evaluation methodology

If UE power consumption is agreed as a KPI for evaluation of XR performance over NR,TR38.840 is the baseline methodology potentially with some modifications if necessary.  RAN1 aim to minimize modeling effort. For example, the following aspects can be considered for further discussion but not limited to.

- FFS whether/how to model UE power consumption for UE tx power other than 0dBm and 23dBm,

- FFS whether/how to model UE power consumption for UL slots that are not defined in TR38.840

- FFS whether/how to model UE power consumption for ‘S’ slot

- FFS whether/how to model UE power consumption for 400MHz in FR2 including scaling rule for FR2 BWP adaption.

- FFS whether/how to model UE consumption for the corresponding number of Tx antennas

- FFS whether/how to model the UE power consumption for UE tx power under FR2

Agreement:

- RAN1 continues to discuss evaluation methodologies for UE power consumption and system capacity.

- RAN1 is to discuss whether/how to study/evaluate mobility and coverage at a later stage, e.g., starting from Q1 2021.

- RAN1 #104-e

Agreements: RAN1 adopts a parameterized statistical traffic model for evaluation of XR and CG, and KPI with details as shown below (RAN1 strives to agree on the remaining details during RAN1 #104e, based on SA4 input):

- There are M1 and M2 streams in DL and UL respectively

  - At least adopt the case where M1=1 & M2=1

  - FFS the values of M1 and M2, including the possibility of being application-dependent

- DL

  - Bitrate for video streaming

    - VR/AR: [60 Mbps (mandatory), 30 Mbps (optional)]

    - CG: [30 Mbps (mandatory), 45 Mbps (optional)]

      - FFS: other optional values

  - Air interface Packet Delay budget (PDB)

    - Air interface delay is measured from the point when a packet arrives at gNB to the point when it is successfully delivered to UE

    - Air interface PDB for video streaming

      - VR/AR: [10ms (mandatory), 20ms (optional)]

      - CG: [15ms (mandatory), 30ms (optional)]

        - FFS: other optional values

  - FFS: Frame-level/IP packet-level modeling for packet arrival, latency measure, etc.

  - FFS: Packet size, including the possibility of varying packet sizes

  - FFS: Packet Inter arrival time including the possibility of modeling jitter

- UL

  - FFS: Bitrate

  - FFS: Air interface Packet Delay budget (PDB)

  - FFS: Frame-level/IP packet-level modeling for packet arrival, latency measure, etc.

  - FFS: Packet size

- Per UE KPI

  - Baseline: A UE is declared a satisfied UE if more than X (%) of packets are successfully transmitted within a given air interface PDB. The exact value of X is FFS.

  - FFS: In addition to the baseline, the following additional method is FFS

    - When determining a XR/CG user is satisfied or not, the following factors are considered. FFS how to use those factors.

      - Packet loss information

      - Packet delay information

      - Some XR/CG source related information if they can be available within RAN, e.g. the mapping between packet and slices or frames and the packet importance

      - Multiple data streams traffic model

  - FFS if there are multiple streams (if adopted)

- FFS additional aspects not addressed above.

- Note 1: Companies are encouraged to provide details such as parameters (e.g., mean, STD, etc.), distributions, etc., by analyzing SA4 input, e.g., V/S/P traces

- Note 2: All FFS points above are to be further discussed in RAN1 #104e

Agreements

- Statistical traffic model for a single DL video stream for a single UE

  - The statistical traffic model for a single UE for a single DL video stream in Figure 1 is adopted, where a packet is assumed to represent multiple IP packets corresponding to a single video frame for modelling/evaluation purposes, e.g., traffic arrival, packet size, evaluation of latency and reliability.

- Frame per second (fps) for DL video stream for a single UE

  - 60 fps (baseline)

  - 120 fps (optional)

  - Other values, e.g., 30, 90 fps can be also optionally evaluated.

- Average data rate for DL video stream:

  - VR/AR: 30, 45 Mbps @60fps (baseline)

    - 30, 60 Mbps @60fps (optional)

    - Note: this is the aggregated data rate when applicable

  - CG: 8, 30 Mbps @60fps (baseline)

    - 8, 45 Mbps @60fps (optional)

  - Other values (in combination with fps) can be also optionally evaluated.

- Truncated Gaussian distribution is used for the packet size distribution of video stream for AR/VR/CG.

  - Other distribution is not precluded.

- (Working assumption) Parameters of Truncated Gaussian distribution for Packet size (note: these parameter values are those before the truncation)

  - Mean: Derived from average data rate and fps as follows.

    - (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes]

  - STD

    - TBD

  - Max packet size

    - TBD

  - Min packet size

    - TBD

    - FFS whether or not to use this parameter

- Per UE KPI

  - Baseline: A UE is declared a satisfied UE if more than X (%) of packets are successfully transmitted within a given air interface PDB.

    - The exact value of X is FFS, e.g., 99, 95

      - FFS different values for I-frame and P-frame if evaluation of them is agreed.

      - Other values can be optionally evaluated

- DL traffic model: video stream

- (Working assumption) Parameters of Truncated Gaussian distribution for Packet size (note: these parameter values are those before the truncation)

  - Mean: Derived from average data rate and fps as follows.

    - (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes]

  - STD

    - [15% of Mean packet size derived above]

    - Note: The above value is an example for further investigation, and is to be revisited potentially with more inputs from companies in RAN1#104-bis-e

  - Max packet size

    - [1.5 x Mean packet size derived above]

    - Note: The above value is an example for further investigation, and is to be revisited potentially with more inputs from companies in RAN1#104-bis-e

  - Min packet size

    - TBD

    - FFS whether or not to use this parameter

    - Note: This is to be revisited potentially with more inputs from companies in RAN1#104-bis-e.

- Jitter for DL video stream for a single UE

  - (Already agreed) Per the agreed statistical traffic model, arrival time of packet k is k/X1000 [ms] + J [ms], where X is the given fps value and J is a random variable.

  - (Newly proposed agreement) J is drawn from a truncated Gaussian distribution:

    - Mean: [0]

    - STD: [2 ms]

    - Range: [[-4, 4]ms]

      - Note: The values ensure that packet arrivals are in order (i.e., arrival time of a next packet is always larger than that of the previous packet)

    - Note: The above values for mean, STD and Range are working assumption for initial simulations, and is to be revisited potentially with more inputs from companies in RAN1#104-bis-e

- Air interface PDB for DL video stream

  - VR/AR:

    - 10ms

    - Other values, e.g., 5ms, 20 ms can be optionally evaluated.

  - CG:

    - 15ms

    - Other values, e.g., 10ms, 30ms can be optionally evaluated.

  - FFS whether or not to have more than one mandatory value

Working assumption: On UL Traffic model and QoS parameters

- CG/VR: single stream (pose/control)

- Traffic model for Pose/control

  - Periodic: 4ms (no jitter)

    - Other values can be optionally evaluated.

  - Fixed: 100 bytes (SA4 input)

  - PDB: 10 ms

- AR

  - FFS

Agreements: On evaluation of multiple streams/flows:

- FFS the following in RAN1#104-bis-e

  - Whether/how to model and evaluate I-frame and P-frame for both DL and UL, e.g., separate definition of fps, packet size, QoS requirements (e.g., PER, PDB), etc.

  - Whether/how to separately model and evaluate two streams of video and audio/data for both DL and UL

  - Whether/how to model and evaluate FOV (high-resolution) and non-FOV (lower-resolution omnidirectional) streams, e.g., separate definition of fps, packet size, QoS requirements (e.g., PER, PDB), etc

Agreement: Adopt following update for TDD configuration for XR/CG evaluation

- FR1:

  - Option 1: DDDSU

  - Option 2: DDDUU

- FR2:

  - Option 1: DDDSU

  - Option 2: DDDUU

Detailed S slot format is 10D:2F:2U. Other S slot format(s) can also be optionally evaluated.

Further clarify that for option 2 for FR1/FR2, there is [2]-symbol gap at the end of third “D” slot of  DDDUU.

FFS whether or not to differentiate the two options (e.g., mandatory vs. optional)

Agreement: For XR evaluation, ideal channel estimation can be optionally evaluated.

Agreements: System bandwidth for XR/CG evaluations are as follows.

- For FR1,

  - Baseline: 100 MHz

  - Optional: 20/40 MHz, 2*100 MHz with CA

- FR2

  - Option 1: 100 MHz

  - Option 2: 400 MHz

Companies should report the CA setting if CA is adopted.

Other system bandwidth can also be optionally evaluated.

Agreements:For outdoor scenarios, the BS antenna parameters are as

- Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

- Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2)

Company to report the BS antenna parameters for XR/CG evaluation.

Other BS antenna parameters can also be optionally evaluated.

Agreements:For FR2, UE antenna parameters for XR/CG evaluations are as follows.

- Option 1 (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151)

  - (M, N, P)=(1, 4, 2), 3 panels (left, right, top)

- Option 2 (from TR 38.802 – developed in Rel-14)

  - 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

Company to report the UE antenna parameters for XR/CG evaluation.

Other UE antenna parameters can also be optionally evaluated.

Agreements: For XR/CG evaluation, adopt following assumptions for BS height for Urban Macro

| Parameter | Proposed value |
| --- | --- |
|  | Urban Macro (FR1) |
| BS height | 25m |

Agreements: For Dense urban and Urban Macro, the UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873.

|  |  | Urban Micro/Macro cell with high UE density (3D-UMi) /(3D-UMa) |
| --- | --- | --- |
| UE height (hUT) in meters | general equation | hUT=3(nfl – 1) + 1.5 |
|  | nfl for outdoor UEs | 1 |
|  | nfl for indoor UEs | nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |

Agreements: At least for XR/CG capacity evaluation, for DL and UL

- Baseline: DL and UL performances are evaluated independently

- Optional: DL and UL performance are evaluated together

- FFS details both the baseline and the optional evaluations

Agreements: For Dense urban for XR/CG evaluation, update the agreement in RAN1 #103e for channel model as follows.

- Dense urban: FR1 and FR2

  - Channel model: UMi UMa. Detailed definition of UMi UMa refers to TR 38.901.

Agreements: For XR/CG evaluation, adopt 12 degree for downtilt for Dense Urban in FR1.

- Other downtilt value can also be optionally evaluated

Agreements: To facilitate further discussion on evaluation of power saving effect of different power saving schemes, the following references are defined.

- Case 1 (baseline): UE power consumption assuming UE is always ON, i.e., UE is always available for gNB scheduling.

- Case 2 (FFS optional or baseline): UE power consumption assuming Rel-15/16 CDRX configuration

  - FFS CDRX configuration details

- Company can also optionally evaluate for other cases, e.g.

  - Genie: UE power consumption assuming that UE is in a sleep state (e.g., micro/light/deep sleep as defined in TR38.840) whenever there is neither DL data reception nor UL transmission. From the gNB scheduling perspective, UE is always available for scheduling, i.e., there is no difference from Baseline in gNB scheduling and corresponding UE Tx/Rx. It is noted that Genie is not a power saving scheme but the result may serve as an upper bound of power saving gain of power saving techniques, which may potentially motivate development of new power saving techniques that can approach the Genie performance.

  - R15/16/17 power saving techniques for connected mode, e.g., BWP, PDCCH skipping, search space switching, etc.

Decision: As per email posted on Feb 5th,

Agreements:

UE power consumption (i.e., power saving gain of the evaluated scheme) for XR is evaluated in conjunction with impact on latency, user experience, and capacity.  In this regard, the following table is used to collect results for system level simulation from companies as a starting point.

- FFS all UEs or only satisfied UEs are included for obtaining the PS gain

Table 1 Evaluation of UE power saving schemes for e.g., {dense urban, AR, FR1}

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 | - | - | - | - | K1 / N |
| Case 2 | X1 % | Y1 % | Z1 % | U1% | K2/ N |
| Case X | X2 % | Y2 % | Z2 % | U2% | K3 / N |

Note 1: CDF of power saving gains of each UE

Note 2: # of satisfied UEs per cell among # of UEs per cell (=N).

Note 3: # of dropped UEs per cell (=N) that needs to be the same for all power saving schemes to be evaluated.

Note 4: company to provide the detailed simulation assumptions including parameter values for each case, e.g. CDRX parameters

Note 5: company can report one or more power saving gain metrics (i.e. mean PS gain or PS gain of 5%/50%/95%/-tile UE in PSG CDF) for each power saving scheme

Agreements: For UL UE power consumption evaluation for UE with transmit power X [0,23] dBm, adopt the following

- Option 1 (Baseline): Consider only two Tx power values as defined in TR 38.840

  - Power number is given as A for X= [0, M)dBm and B for X =[M, 23]dBm, where A and B (defined in 38.840) correspond to power consumption numbers for a given uplink slot for 0dBm and 23dBm respectively.

    - M = [20]

    - Other value(s) of M can be optionally evaluated

  - Companies to provide detailed assumptions on UE power consumption for Tx power values other than 0 and 23 dBm

    - E.g. Power number is given as A for X= [0, 20)dBm and B for X =[20, 23]dBm, where A and B (defined in 38.840) correspond to power consumption numbers for a given uplink slot for 0dBm and 23dBm respectively.

- Option 2 (FFS mandatory or optional): Linear interpolation method in linear scale for Tx power values other than 0 dBm and 23 dBm

- FFS whether or not to differentiate the two options (e.g., mandatory vs. optional)

- FFS whether or not to consider UE with transmit power less than 0 dBm