3GPP TSG RAN WG1 Meeting #105-e	R1-2106072

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon (Moderator)

Title:	Summary of CSI enhancements for MTRP and FDD (Round 2)

Document for:	Discussion and Decision

# Agreements

Agreement

For Rel-17 port selection codebook, the maximal value of CSI-RS port number P as Pmax is 32.

Conclusion

At least for rank 1, no further restriction or condition is applied for polarization-common based free-selection and combinatorial coefficient based port selection for W1.

Working Assumption

At least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1

- FFS: Further dependence/restriction, e.g. conditioned on N3 or the number of CSI-RS ports, can be applied to above design. If does, how to support a non-consecutive FD bases used for Wf quantization

- FFS: Whether to introduce thresholds for N3 and/or P

Agreement

A polarization-specific bitmap for indication non-zero coefficients should be supported for W2.

Agreement

For the quantization of W2 coefficient, reusing following Rel-16 quantization mechanism for Rank1 at least:

- Two polarization-specific reference amplitudes:

  - for the polarization associated with the strongest coefficient, the reference amplitude is not reported

  - for the other polarization, reference amplitude is quantized to 4 bits

    - The alphabet is{1, 1/2)^(1/4), (1/4)^(1/4), (1/8)^(1/4), …, (1/2^14)^(1/4), [Reserved]} (-1.5dB step size)

- For coefficients other than the strongest coefficient

  - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - The alphabet is {1, 1/sqrt(2), 1/2, 1/(2*sqrt(2)), 1/4, 1/(4*sqrt(2)), 1/8, 1/(8*sqrt(2))} (-3dB step size)

  - phase is quantized to 16PSK

- For the reserved state for reference amplitude, down-select one Alt

  - Alt 1: it is kept to be reserved

  - Alt 2: it is replaced as (1/2)^(15/4)

  - Alt 3: it is replaced as (1/2)^(3/8)

Note: whether/how SCI is supported for R17 codebook will be discussed separately

Agreement

Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis:

- It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot.

Conclusion

Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM

Alt 2: No, it is not supported

Agreement

For a CSI-RS resource set with Ks NZP CSI-RS resources configured for CMR and N NZP CSI-RS resource pairs configured for NCJT measurement hypotheses, study following default value of Ks, max,

- Alt 1: Ks, max = 4

- Alt 2: Ks, max = 2

- Alt 3: Ks, max = 4 for FR2, and Ks, max = 2 for FR1

- Note that default value means the minimal supported value for Ks,max in UE capability reporting, if UE support this feature.

Agreement

For CSI measurement associated with a CSI-ReportConfig for NC-JT, study whether/how to support following dynamic updating on, e.g. by MAC-CE

- Alt 1: CMR pairs for NCJT measurement hypotheses

- Alt 2: CMRs for Single-TRP measurement hypotheses

- Alt 3: TCI states in CMRs

- Alt 4: the number of single-TRP CSIs (i.e. X=0/1/2) in a NCJT CSI report

Conclusion:

There is no consensus to go with either of the following options in RAN1 #105e:

- Option 1: Confirm the Working Assumption from RAN1 103e

- Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT

Agreement

For Rel-17 Multi-TRP CSI enhancement, companies are encouraged to study following potential specification impact:

- CRI codepoint mapping order with CMRs and CMR pairs

- Whether/how to configure RI restriction/CBSR configuration for NCJT CSI measurement

- Whether/how to enhance the CSI updating rule to address CPU overbooking

- Whether/how to introduce new CSI computation delay requirement for NCJT CSI calculation

- Whether/how to support wideband CSI report

Agreement

At least for rank 1 and for Mv>1, Minit for the single window with size N is fixed to be 0

Conclusion

For PS codebook enhancements utilizing DL/UL reciprocity of angle and/or delay, there is no consensus of further enhancement for CSI-RS configurations associated with Rel-17 PS codebook.

# Summary of CSI enhancement for FDD

Proposal 2: At least for rank 1, candidate values of K1 for port selection matrix W1 in NP*K1 are {2, 4, 8, 12, 16, 24, 32}.

- Note: for polarization-common based free-selection, it means to select the same L=K1/2 ports out of P/2 ports for both polarizations

Proposal 2-1: Further reduction for possible parameter combinations among codebook parameters of Rel-17 port selection codebook, e.g. {K1, Mv, Beta}, will be discussed jointly once candidate values are determined

- based on trade-off among UPT performance, feedback overhead, and complexity

- based on all supported ranks

- Limit total number of parameter combinations comparable to Rel-16 eType II, e.g. no more than 8

- Exact parameters (e.g. with 2 or 3 parameters) within each combination are FFS

- Other parameterizations of codebook parameter (e.g. alpha with K1= Alpha*# of CSI-RS ports and Alpha <=1) are not excluded

Proposal 10: At least for rank 1 and 2, for the compression coefficient Beta for non-zero coefficients of W2, values of Beta are {[1/4], [1/2], 3/4, 1}

Possible way forwards:

- WF1: No change.

- WF2: remove bracket over 1/2 and with following note:

  - Note: [1/4] means that 1/4 is also a candidate value for the discussion on reduction of parameter combinations, but has a lower priority compared to other beta values

Proposal 3: For Wf in CN3*Mv, Mv=2 is supported for R17 PS codebook, at least by one parameter combination

- FFS: whether further dependence/restriction, i.e. conditioned on the number of CSI-RS ports, can be applied to Mv=2

- FFS: Whether Mv=4 can be supported for # of CSI-RS ports, e.g. 4 or 8

Proposal 11: Studying whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting

- FFS: applicable conditions of being absent, .e.g. Mv=1 and Beta =1 for rank 1 or higher ranks

- FFS: additional impact for reporting mechanism when/how the bitmap is absent

- Note: the principle of UE determining the real number of NZC (same as Rel-15 and Rel-16) is unchanged in Rel-17

- based on trade-off among UPT performance, feedback overhead and complexity

# Summary of CSI enhancement for MTRP

Proposal 17: Whether a NZP CSI-RS resource m can be referred by two CMR pairs (m, a) and (m, b) configured for NCJT measurement hypotheses, down-select one Alternative in RAN1#105e:

- Alt 1: It is feasible for FR1 but not for FR2.

- Alt 2: It is feasible for both FR1 and FR2 but subject to further UE capability for FR2.

Alt 1 (12): ZTE, MTK, DOCOMO, CMCC, QC, Vivo, LG, Apple, Samsung, CATT, Nokia/NSB

Alt 2 (6): Oppo, Mot/Lenovo, Futurewei, QC, Ericsson

Proposal 19:

Alt 2: A CSI-IM resource is configured to be associated with either a CMR for Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis:

- One-to-one mapping between M+N CSI-IM resources versus M NZP CSI-RS resources for single-TRP measurement hypothesis and N NZP CSI-RS resource pairs for NCJT measurement hypothesis configured in a CSI-RS resource set.

  - FFS the value/definition of M

Note: it is possible to configure the same value of CSI-IM resource ID for both NCJT and Single-TRP measurement hypotheses in FR1 and FR2, subject to QCL-Type D consistency between measurement hypotheses of the shared CMR in FR2