3GPP TSG RAN WG1 #105-e	R1-2106113

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.12.2

Source:	Moderator (Huawei)

Title:	FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

This summary summarizes the contributions submitted in AI 8.12.2 to discuss how to improve the reliability for MBS for RRC_CONNECTED UEs.

This summary includes HARQ-ACK feedback specific issues, NACK-only specific issues, HARQ-ACK feedback common issues, HARQ-ACK feedback for SPS multicast, PDSCH repetition, and CSI feedback. In each of high level issue, a sub-level list of issues are organized.

For each of listed issue, proposal(s) is/are suggested from moderator’s perspective according to the submitted individual company’s proposal(s). Companies are welcome to make comments in the table “collect views”. The proposals may be updated in subsequent rounds according to the comments collected in previous rounds so as to strive to converge to consensus. Note that moderator may only tend to collect concerns when time is right for some specific rounds, for which companies are expected to only provide concerns in the table “collect concerns” if any instead of inputting views again and again to alleviate efforts.

People can use “navigation pane” to quickly overview the organization of the summary and proposal(s) for each issue for discussion and provide views/comments into the table of “collect view”/“collect concerns” under each proposal.

Note: for all proposals FL suggests, companies are encouraged to input views, situation can be known better, so as to progress fruitfully.

# ACK/NACK-based feedback specific

## Separate HARQ-ACK codebook

Submitted Proposals

(Huawei) Proposal 1:

  - For the cases of unicast and multicast scheduled from the same TRP,

    - UE can support two non-overlapping PUCCHs for multicast and unicast, respectively, with different priorities in the same slot.

    - UE can be configured to generate two separate codebooks for unicast and multicast, respectively.

      - Unicast and multicast can be with the same or different priorities.

(vivo) Proposal 3:

  - The HARQ-ACK codebook type for multicast and unicast should be separately configured.

(CATT) Proposal 9:

  - Regardless of the codebooks’ priorities between unicast and multicast, when PUCCH-ConfigurationList for multicast and PUCCH-ConfigurationList for unicast are separately configured, UE is expected to construct separate codebooks for multicast and unicast corresponding to PUCCH-Config for multicast and PUCCH-Config for unicast, respectively

    - Note: UE expects the PUCCH resources for multicast and for unicast configured/indicated by gNB are non-overlapping.

(CMCC) proposal 4:

  - A separate pdsch-HARQ-ACK-CodebookList can be configured for multicast service.

(CMCC) proposal 5:

  - At least at the following conditions, separate HARQ-ACK codebooks for multicast and unicast are generated

    - HARQ-ACK for unicast and for multicast have different priorities.

    - HARQ-ACK for unicast and for multicast have the same priority and unicast PUCCH and multicast PUCCH for HARQ-ACK do not overlap in time, and two PUCCHs for HARQ-ACK for different services can be transmitted in one slot(sub-slot).

(CMCC) proposal 6:

  - At least at the following conditions, HARQ-ACK for multicast and unicast are multiplexed in one PUCCH

    - HARQ-ACK for unicast and for multicast have the same priority and unicast PUCCH and multicast PUCCH for HARQ-ACK overlap in time.

    - HARQ-ACK for unicast and for multicast have the same priority, unicast PUCCH and multicast PUCCH for HARQ-ACK do not overlap in time, but only one PUCCH for HARQ-ACK can be transmitted in one slot(sub-slot).

(OPPO) Proposal 3:

  - If PUCCH-config/PUCCH-ConfigurationList for unicast is configured for MBS, HARQ-ACK bits of unicast and MBS with same priority are multiplexed and transmitted in the PUCCH of the same priority level

(OPPO) Proposal 5:

  - If separate PUCCH-config/PUCCH-ConfigurationList is configured for MBS, HARQ-ACK codebook for MBS and unicast are constructed separately and transmitted in PUCCH resource dedicated for MBS or unicast.

(Samsung) Proposal 5:

  - A UE is configured by higher layers whether to separately or jointly construct Type-1 HARQ-ACK codebooks for unicast or multicast.

(Samsung) Proposal 6:

  - For separately constructed unicast and multicast HARQ-ACK codebooks of same priority in a PUCCH/PUSCH, the multicast HARQ-ACK codebook is appended to the unicast HARQ-ACK codebook.

### Round-1 (closed)

FL’s comment:

In URLLC introduced in Rel-16, UE can be configured with a codebooklist to generate two codebooks with different priorities. For HARQ-ACK feedback for multicast, last meeting has agreed to configure a PUCCH-ConfigurationList to correspond to a low priority codebook and a high priority codebook, respectively.

Regarding whether to generate separate codebooks for unicast and multicast, it was discussed in the last meeting without achieving any conclusion. 5 companies proceeded the discussion in this meeting. Views are as follows:

If separate PUCCH-config/PUCCH-ConfigurationList is configured, generate separate codebook:

- CATT, OPPO

If HARQ-ACK for unicast and multicast have different priorities, generate separate codebooks

- CMCC

UE can be configured to generate separate codebooks,

- Huawei, Samsung

At least UE can be configured to generate separate codebooks for the different priority cases and separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively, to correspond the two separate codebooks with different priorities, which is the mechanism introduced in URLLC and is the intention to introduce a PUCCH-ConfigurationList to correspond the separate codebooks with different priority cases. The same priority case can be FFS.

FL’s Proposal:

#### Proposal 2.1.1

- UE can be configured to generate separate codebooks for unicast and multicast, respectively, at least for the case of different priorities.

- separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively.

- FFS for the case of the same priority.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| LG | We think that separate codebooks could be generated even if PUCCH-config for multicast is not configured, noting that we previously agreed as follows: Agreement: For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies. Thus, we propose to change to: #### Proposal 2.1.1 - UE can be configured to generate separate codebooks for unicast and multicast, respectively, at least for the case of different priorities. - separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively. - FFS for the case of the same priority. |
| CATT | Generally OK with it. |
| ZTE | For the main bullet, for the case of different priorities, UE must generate separate codebooks for different priorities, there is no need to have such a configuration process. In other words, even if without the configuration, UE still needs to generate separate codebooks for different priorities. Based on our understanding, even if separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively, UE still needs to generate joint codebook for unicast and multicast at least for the type-1 codebook for TDM case. In this sense, the first bullet is not correct. Furthermore, the details of how to construct semi-static codebook for different priorities are still open and not clear. The semi-static codebook is based on the TDRA, however the priorities are not associated with the TDRA table. More discussion is needed. Thus, we suggest to focus on dynamic codebook for the above proposal. Thus, we would propose the following proposal. Updated proposal from ZTE: - For type2 codebook, UE can be configured to generate separate codebooks for unicast and multicast, respectively, at least for the case of different priorities. - separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively. - FFS for the case of the same priority. - FFS for the case of type1 codebook. |
| CMCC | Partially support. We think for shared PUCCH configuration, if multicast and unicast have different priorities, the codebooks are also generated separately. |
| Nokia, NSB | Although we understand the intention of the proposal, we do not agree with it. We believe that the proposal can be misleading. In our view, in addition to Rel-16 unicast PUCCH-ConfigurationList that includes two different PUCCH-configs (one for high priority and one for low priority), we have already agreed that a separate PUCCH-ConfigurationList for multicast can be configured optionally for ACK/NACK based HARQ-ACK feedback: Agreement: For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively. - FFS other configurations In addition to that, we believe that NACK-only feedback may also have a distinct PUCCH-ConfigurationList including two different PUCCH-config configurations. As in Rel-16, the UE should generate 1 codebook per PUCCH-config, which may include a concatenation of different sub-codebooks (e.g. 1 sub-codebook for unicast and 1 sub-codebook for multicast) in the case unicast and multicast services are to be ACK/NACK-ed in the same slot. The exact sub-codebook / concatenation / unification mechanism has already been discussed in specific proposals for Type-1 / Type-2 HARQ-ACK and multiplexing / prioritization. So, one codebook per PUCCH-config (which may include a concatenation of different sub-codebooks (e.g. 1 sub-codebook for unicast and 1 sub-codebook for multicast) in case unicast and multicast services are to be ACK/NACK-ed in the same slot) should be transmitted by the UE in a (sub-)slot. |
| Spreadtrum | Generally we are fine. |
| vivo | Not agree. The construction of separate codebooks for unicast and multicast has no relation with separate configuration of PUCCH-config. Even when only one PUCCH-config is configured for unicast and multicast, HARQ-ACK codebook can be separately constructed. In addition, in URLLC introduced in Rel-16, different codebook types can be configured by codebooklist for different priorities, for unicast and multicast, the HARQ-ACK codebook type should also be separately configured. |
| Apple | We understand the intention of this proposal. maybe it could be clearer if the priority combinations between unicast and multicast are provided with the related PUCCH-config/PUCCH-ConfigurationList (and/or default configuration). |
| Samsung | Agree with previously made comments (e.g. from LG, CMCC, Nokia, Vivo, …). For same priority, a single HARQ-ACK codebook can be generated as in Rel-16 and an additional UE capability is not needed (note that, even for M-TRP, separate codebooks were introduced to account for backhaul latency which is not the case of multicast-unicast). |
| OPPO | Fine with us basically, however, the relationship between the main bullet and first sub-bullet are not clear. If our understanding correct, the first sub-bullet should be a prerequisite of the main bullet, if so, we suggest following modification: - If separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast respectively, UE can be configured to generate separate codebooks for unicast and multicast, respectively, at least for the case of different priorities. - separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively. - FFS for the case of the same priority. |
| Qualcomm | Not sure why ‘UE can be configured’ in the main bullet. If different priorities, separate codebooks will be generated. Do we need to additional configuration/signaling? The first subbullet is not needed, which is not a necessary condition. |
| Ericsson | Support. For clarification, is it the feature lead intention that there should always be two separate configuration lists, or could multicast and unicast share a list? |
| NTT DOCOMO | We are fine with the proposal. |

### Round-2 (closed)

FL’s comment:

- The following is agreed on the GTW session:

Agreement:

- The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

- Regarding how UE is configured to generate two codebooks, what is specified for URLLC is as follows:

“If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE can be indicated by pdsch-HARQ-ACK-CodebookList to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks

-	a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1

-	the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCH-ConfigurationList, UCI-OnPUSCH-ListDCI-0-1, PDSCH-CodeBlockGroupTransmissionList} or {PUCCH-ConfigurationList, UCI-OnPUSCH-ListDCI-0-2, PDSCH-CodeBlockGroupTransmissionList}, respectively, for use with the first and second HARQ-ACK codebooks, respectively”

- From 38.331,

PDSCH-HARQ-ACK-CodebookList-r16 ::=     SEQUENCE (SIZE (1..2)) OF ENUMERATED {semiStatic, dynamic}

- Hence, from 38.331, the pdsch-HARQ-ACK-CodebookList parameter can indicate UE to generate one or two separate codebooks and if generate two separate codebooks, they can be the same or different codebook types. Also, if the UE is indicated to generate two HARQ-ACK codebooks, UE is provided first and second for each of PUCCH-Config by PUCCH-ConfigurationList.

- As agreed, the signalling for URLLC can be reused to configure separate codebooks for unicast and multicast for the case of different priorities. Following up the agreement to solve the issue regarding the FFS points:

FL’s Proposal:

#### Proposal 2.1.2

- When UE is configured to generate two separate codebooks for unicast and multicast with different priorities,

- the separate codebooks can be the same type and different types of {Type-1 or Type-2}.

- UE is provided separate PUCCH-Config/ PUCCH-ConfigurationList for multicast from that for unicast, for use with unicast and multicast HARQ-ACK codebooks.

Collect views:

| Company | Comments |
| --- | --- |
| CMCC | Still have concern why to exclude shared PUCCH-config case. As the spec of Rel-16 URLLC, the UE is provided pdsch-HARQ-ACK-CodebookList to generate one or two codebooks, which the codebook with priority index 0 uses the first PUCCH-config, and codebook with priority index 1 uses the second PUCCH-config. For Rel-17 multicast, let’s assume the PUCCH-ConfigurationList is shared for unicast and multicast, and UE is configured to generate two codebooks both for unicast and multicast. That is unicast codebook with priority index 0 uses the first PUCCH-config, unicast codebook with priority index 1 uses the second PUCCH-config, multicast codebook with priority index 0 uses the same first PUCCH-config, multicast codebook with priority index 1 uses the same second PUCCH-config. If the priorities are different for unicast and multicast, UE generates separate codebooks which corresponding to different PUCCH-configs. Is this case right? |
| FL’ comment | @CMCC, It depends on whether UE is configured to generate separate codebooks with the same priority that is FFS. Let’s check other views also, if separate codebooks is not applied to the same priority, we can work on more to improve the proposal for the case you concerned. |
| Nokia, NSB | We support the proposal with minor editing. - When a UE is configured to generate two separate codebooks, one for unicast and one for multicast with different priorities, - the separate codebooks can be the same type and different types of {Type-1 or Type-2}. - UE is provided separate PUCCH-Config/ PUCCH-ConfigurationList for multicast from that for unicast, for use with unicast and multicast HARQ-ACK codebooks. - FFS: details of separate PUCCH-Config/PUCCH-ConfigurationList for multicast. The last FFS for us, is to cover scenarios where a UE is configured with separate MCast PUCCH configs for NACK-only and ACK-NACK feedback options for the same/different MCast service(s). |
| CATT | The direction of this proposal is reasonable, but The main bullet confuses us. Further clarification is needed. - Regarding ‘the two separate codebooks’, whether it means one codebook is for unicast and another separate codebook is for multicast (Case 1) or two separate codebooks is configured for unicast and two separate codebooks is configured for unicast, respectively (Case 2). - Regarding ‘with different priorities’, in our understanding it is not proper no matter which case is. - For Case1, according to ‘If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0.’, so these two separate codebooks should have the same priority with priority index 0. Thus, we think that the ‘with different priorities’ is not right. - For Case 2, regardless of the multicast or unicast, once the corresponding PUCCH-ConfigurationList indicates to generate two codebooks, these two codebooks will have different priorities (0 and 1). Thus, we think that the ‘with different priorities’ is redundant. Hope my views can help us to make progress, and please also correct me if my understanding is not right. |
| vivo | For the main bullet, Agree with CATT, it is not clear. For case 1, both unicast and multicast is low priority. For case 2, unicast can be low or high priority and multicast and be low or high priority. In addition, it can be one codebook is for unicast and two separate codebooks are for multicast (Case 3) or two separate codebooks is configured for unicast and one codebook is configured for multicast (Case 4). In case 2/3/4, it is possible that HARQ-ACK for unicast and multicast have different priorities. For the first bullet, we think “of {Type-1 or Type-2}” can be removed. For unicast HARQ-ACK, a UE can be configured with pdsch-HARQ-ACK-Codebook-r16 and/or pdsch-HARQ-ACK-OneShotFeedback-r16 when UE is not configured with pdsch-HARQ-ACK-CodebookList. In that case, other types of CB (enhanced type 2 and type 3) can be configured for unicast. Even though we haven’t agreed to support enhanced type 2 and type 3 CB for multicast, we shouldn’t mandate a UE to be configured with Type-1 or Type-2 CB for unicast when the UE also has multicast to receive. |
| Lenovo, Motorola Mobility | Generally OK with us. Two small revisions from our side on the main bullet to address companies’ concern and better align the standard. - When UE is indicated to generate two separate codebooks for unicast and multicast with different priorities, respectively, |
| Qualcomm2 | Need more time to consider different cases as listed by companies. In last GTW, we spend a lot of time on this issue. Does FL plan to discuss the new proposal in next GTW? |
| LG | Considering ongoing discussion on separate codebooks with LP/HP for unicast in R17 URLLC WI, it would be better to focus on same priority case in MBS WI for the time being to avoid complicated discussion consuming time. Moreover, we also think that we would need more time to consider different cases as listed by companies. So, we could discuss different priority case later after how R17 URLCC works becomes clear. |
| OPPO | We tend to agree with analysis of CATT and vivo, based on Rel-16 URLLC mechanism, HARQ codebook with priority 1 cannot be configured along, hence, if priority for unicast and multicast are not same, then at least one of them are configured with both priority 0 and priority 1, under this condition UE may need to generate at least 3 codebooks if the intension is no HARQ-ACK bits multiplexing between unicast and multicast in one codebook. The agreement achieve on Wed GTW is aligned with Rel-16 principle that “UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index”. To move forward, we need to discuss if separate PUCCH-config/PUCCH-configlist are configured for unicast and multicast, whether UE can multiplex HARQ-ACK information for unicast and multicast with same priority in one codebook or not. On top of that, we can further discuss how to generate codebooks in various cases. |
| NTT DOCOMO | Support the modified version by Nokia. |
| ZTE | We are ok with the proposal in principle. However, we want to point out there are some more complicated scenarios, which may need further discussion for this issue. 1. Unicast with low priority only + Multicast with low priority only; 2. Unicast with low priority and high priority + Multicast with low priority only; 3. Unicast with low priority only + Multicast with low priority and high priority; 4. Unicast with low priority and high priority + Multicast with low priority and high priority; |

### Round-3 (closed)

FL’s comment:

Addressing the comments from Round-2:

@Nokia, DOCOMO, the proposal here mainly focuses on codebook based feedback, so ACK/NACK based is more  relevant than NACK-only. It would be better to focus on ACK/NACK based firstly and NACK-only specific issues will be discussed anyway in the other section.

@CATT, the original intention was one U-cast and one M-cast with different priorities, as the case of two unicast with different priorities. Now based on others comments (vivo, OPPO, ZTE) as well, we may need to separate all different cases for further discussion.

@QC, I don’t plan to spend more time on GTW for this proposal and hope at least we can identify all different cases and possible solutions. From FL’s perspective, I see this issue is important as we need to figure out whether UEs generate separate codebooks according to the separate PUCCH-config (regardless the same or different priorities) or just according to the different priorities to facilitate the codebooks discussion easier.

@LG, confused by the comment that “separate codebooks with LP/HP for unicast in R17 URLLC”, you mean generating separate codebooks with LP/HP but will be multiplexed in the same PUCCH that is being discussed in R17 URLLC? If your answer is yes, I would say the proposal here is not taking about the same thing. Rather, it is just about under what condition UE generates two separate codebooks for unicast and multicast, respectively.

Based the comments received earlier, the proposals cover the case 1 (low codebook for unicast) and case 2 (low and high codebooks for unicast) as follows:

#### Proposal 2.1.3-1

- When UE is indicated to generate one HARQ-ACK codebook for unicast with low priority corresponding to PUCCH-config_ucast,

- If a separate PUCCH-config_mcast is configured for multicast, the codebook for multicast is low priority, and

- Alt1: UE is expected to generate a single codebook with low priority, including the HARQ-ACK feedback for unicast and multicast, and then determines the PUCCH resources for transmission across PUCCH-config_ucast and PUCCH-config_mcast.

- Alt2: UE is expected to first generate two separate codebooks with low priority for unicast and multicast, respectively, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast and PUCCH-config_mcast, respectively.

- Down-select between the two alternatives.

- If a separate PUCCH-ConfigurationList_mcast is configured for multicast, the two separate codebooks for multicast are low and high priority, respectively.

  - Alt1:

    - UE is expected to generate a single codebook with low priority, including the HARQ-ACK feedback for unicast and multicast, and then determines the PUCCH resources for transmission across PUCCH-config_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

    - UE is expected to generate a single codebook with high priority for multicast, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

  - Alt2:

    - UE is expected to first generate two separate codebooks with low priority for unicast and multicast, respectively, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast, respectively.

    - UE is expected to generate a single codebook with high priority for multicast, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

  - Down-select between the two alternatives.

- If a separate PUCCH-config/ PUCCH-ConfigurationList is NOT configured for multicast

  - UE is expected to generate a single codebook with low priority, including the HARQ-ACK feedback for unicast and multicast, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast.

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | Separate or joint generation of HARQ-ACK codebooks is independent of PUCCH_Config. For Type-2 CB, the generation for multicast and unicast can only be separate. If there are separate PUCCH_Config for unicast and multicast and the priority is same, the UE can use the PUCCH_Config for unicast to multiplex the multicast Type-2 CB if both Type-2 CBs are multiplexed in a same PUCCH; otherwise, it can use the PUCCH_Config for multicast. For Type-1 CB, joint generation can always apply for same priority – no reason for separate generation and no relevance again to whether or not separate PUCCH_Config is provided for multicast (no apparent reason for the gNB to provide in case of same priority since PDCCH MOs for unicast/multicast are not considered for the generation of the Type-1 CBs). Separate generation of course applies in case of ‘NACK-only’ for multicast or in case of different priorities (and, again, whether or not separate PUCCH_Config is provided is not relevant, or is a consequence of the required PUCCH resources depending on other aspects). In general, RRC aspects for configuration of PUCCH resources can wait for later and will/should become clear once all the overall design is in place. |
| ZTE | First of all, this proposal mainly consists two parts, i.e., (1) the number of codebooks, (2) PUCCH resource determination. Considering that we have separate section to discuss the PUCCH resource determination, we suggest to focus on the number of codebooks in this proposal and discuss other issues separately. Otherwise, it may be difficult to reach consensus with such a huge proposal. Besides, we have the following comments: 1. Regarding the term “codebook” used in Alt.1 and Alt.2 above, from our perspective, if UE generates one type-2 sub-codebook for unicast and generates another type-2 sub-codebook for multicast, and then UE appends them together. This still belongs to one “codebook”, is this the correct understanding? 2. Does the above proposal cover the case when different types of codebook are generated for unicast and multicast? For example, if UE generates type-1 (sub)CB for unicast and type-2 (sub)CB for multicast, then can we append them together into one codebook? |
| Lenovo, Motorola Mobility | We share same concern with Samsung and ZTE. |
| FL’s comments | To all, I’d like to clarify the intention of these two proposals in this section. Earlier we have agreed “Concatenation of Type-2 … is supported” and “construction of Type-1 … based on union TDRA sets… is supported”. However, it is unclear or undecided yet when or in which cases such concatenating or basing on union TDRA is supported esp. when others also propose different Type-1 schemes from “union TDRA” but rather similar to “concatenating of Type-1 sub-codebook of unicast and that of multicast ”. This is the reason I suggest this proposal. With this proposal, we or at least me as FL is confused how the codebook is exactly constructed in different cases as others commented in Round-1 and as I proposed here. I assume companies should agree with me this is important for further more discussion on detailed issues later and this is why I suggested as the first issue in this section. Surely, if companies have different views, please comment. To Samsung, With above clarified, for Type-2, you say if separate PUCCH-config and CB is same priority and UE can use the PUCCH_Config for unicast to multiplex, and the issue I see is why PUCCH_Config for multicast to multiplex cannot be used. Maybe the answer is simple and straightforward but for now it is open and does seem to be considered in submitted contributions. For Type-1, joint CB based on union has been agreed to be supported but it is for the same priority. For different priorities, it is probably to generate separate codebooks as what is specified for URLLC, maybe it is straightforward but at least still open right now, and it is part of discussion in this proposal. Also, what if UE is configured with low CB for unicast but configured with low/high CBs for multicast, what the CB should be constructed is also what intended to be discussed in this proposal. Regarding the two alternatives, both are proposed by some companies. If we can down-select to a single one in this meeting, that could be totally perfect from FL’s perspective. To me, it is not purely RRC aspects for configuration of PUCCH resources but rather what codebooks should be constructed in different cases esp. for low/high for unicast and low/high for multicast. The proposal is indeed corresponding to PUCCH-config also as proposed by companies as reflected in the two alternatives. Down-selection will be done anyway. To ZTE, At least this proposal applies to both Typ-1 unicast/multicast and Type-2 unicast/multicast. Regarding your first question, the “single codebook ” include the case you described “if UE generates one type-2 sub-codebook for unicast and generates another type-2 sub-codebook for multicast, and then UE appends them together.”, so yes it is a single codebook in the proposal. When I draft the proposal, as least type1 and type1 and type2 and type2 should be intended in this proposal. Whether type1 and type2 or even other types for unicast and FFS. Maybe applied as well but let’s fix the same type first for simplicity. |
| OPPO | - We think the current status is as below: - Following Rel-16 principle and the agreement achieved in the latest GTW, HARQ-ACK information of unicast or multicast with different priorities should not be multiplexed in the same PUCCH. - RAN1 has also agreed that “Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot”. - And also “For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config/ PUCCH-ConfigurationList for multicast”. - Given that, we think the key point now is to discuss and decide whether separate codebooks can be configured for unicast and multicast HARQ information with same priority. Thereafter, we can discuss further details, including whether separate PUCCH-Config/ PUCCH-ConfigurationList should be configured for multicast if separate codebooks are configured for unicast and multicast. - Furthermore, we share similar view as ZTE that we need to align the interpretation on terms “codebook” and “sub-codebook”. According to the following agreement, codebook is a bit string to be transmitted in PUCCH, it could be concatenation of multiple sub-codebooks. Agreement: For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource, - The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast. - FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast. - Note: The case of SPS PDSCH will be discussed separately. |
| CATT | Before considering the how to construct the CBs for unicast and multicast, we think some clarifications are needed for the followings: - In current URLLC mechanism, the number of the PUCCHs in a slot which carrying HARQ-ACK is different based on UE capability. For example, when UE is constructed with one CB, more than one PUCCH for HARQ-ACK can be transmitted within a slot; when two CBs is constructed with two priority, two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook simultaneously is constructed for supporting HARQ-ACK codebooks with different priorities at a UE. Thus, with the introducing of the multicast, will the UE capability will be change or not? - Whether HARQ-ACK PUCCH for unicast and for multicast is overlap in time or not can also affect the way to construct the separate or joint CBs. Furthermore, we share similar view as ZTE that we need to align the interpretation on terms “codebook” and “sub-codebook”. |
| Spreatrum | - The proposal is very huge, and it seems that opinions are still diverse. Maybe we should forward step by step, starting with codebook issue for one unicast with one priority and one multicast with one priority. Then proceed to more complex cases. - Share our views on the codebook issue for the case where one unicast is with one priority and one multicast is with one priority below: - For type-2 codebook, given that independent DAI/TAI for unicast and multicast, only separate codebooks for unicast and multicast shall be supported, no matter whether their priority is same or different, and no matter whether PUCCH-Config is separately configured. - For type-1 codebook, when unicast and multicast with different priority, Rel-16 URLLC signalling could also be reused to configure to generate two separate codebooks as we agreed for type-2 codebook - For type-1 codebook, when unicast and multicast with same priority, if following Rel-16 URLLC, single codebook shall be generated. |
| MTK | - The wording of “single codebook” is not clear for us in above two Alt 1. Does it mean that generating the codebook for multicast and unicast with Type-1 mechanism? |
| NTT DOCOMO | - We share similar view as ZTE and OPPO. We would like to focus on the number of codebooks in this proposal. |
| LG | - We generally share the same view with Samsung and ZTE. We think that separate or joint generation of HARQ-ACK codebooks is independent of PUCCH_Config. |
| Nokia, NSB | - We need to have the right understanding first within the RAN1 group. In our understanding, NR does not support 2 slot-based PUCCH transmissions in a slot with HARQ-ACK information, based on Rel-16 specifications below: 38.213 S.9.2.3: "A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot”. - Assuming that a separate PUCCH-ConfigurationList_mcast is configured for multicast, and the - two separate codebooks for multicast are low and high priority, respectively, we believe that the following rules apply: - If the PUCCH transmission in unicast low priority PUCCH-config is to be made in the same slot with a PUCCH transmission in multicast low priority PUCCH-config, then these should be multiplexed and sent as a one codebook (including possibly multiple sub-codebooks). Multiplexing can be done as we discuss in the separate sections (i.e., for Type-1, TDRA unification and possibly concatenation of unicast and multicast sub-codebooks; for Type-2 concatenation of sub-codebooks). - Which PUCCH resource to use to send the multiplexed feedback is also discussed in different sections (unicast / multicast DCI’s PRI field). - If the PUCCH transmission in unicast low priority PUCCH-config is to be made in the same slot with a PUCCH transmission in multicast high priority PUCCH-config, then the low priority should be dropped (and vice-versa). - If the PUCCH transmission in unicast high priority PUCCH-config is to be made in the same slot with a PUCCH transmission in multicast high priority PUCCH-config, then these should be multiplexed and sent as a one codebook (including possibly multiple sub-codebooks as in case of first bullet). - If no multicast PUCCH-config is configured: - If the PUCCH transmission of unicast low priority is to be made in the same slot with a PUCCH transmission of multicast low priority, then these should be multiplexed and sent as a one codebook, as above. Similar for the case when both are high priority. - If the PUCCH transmission of unicast low priority is to be made in the same slot with a PUCCH transmission of multicast high priority, then the low priority should be dropped (and vice-versa). |
| vivo | - We generally share the same view with Samsung and ZTE. We think that separate or joint generation of HARQ-ACK codebooks is independent of PUCCH_Config. We share similar view as ZTE that we need to align the interpretation on terms “codebook” and “sub-codebook”. If it is the common understanding “if UE generates one type-2 sub-codebook for unicast and generates another type-2 sub-codebook for multicast, and then UE appends them together.”, so yes it is a single codebook in the proposal. - We don’t think there is any additional complexity when the CB types for unicast and multicast are different. - For the case with one low priority CB for unicast and one low priority for multicast, we think it depends on whether we support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with the same priority, respectively. If it is supported, it should be Alt 2. If it is not supported, it is Alt1. |

#### Proposal 2.1.3-2

- When UE is indicated to generate two HARQ-ACK codebooks for unicast with low and high priority, respectively, corresponding to PUCCH- ConfigurationList_ucast,

- If a separate PUCCH-config_mcast is configured for multicast, the codebook for multicast is low priority, and

- Alt1: UE is expected to generate a single codebook with low priority, including the HARQ-ACK feedback for unicast and multicast, and then determines the PUCCH resources for transmission across PUCCH-config_ucast of PUCCH- ConfigurationList_ucast and PUCCH-config_mcast.

- Alt2: UE is expected to first generate two separate codebooks with low priority for unicast and multicast, respectively, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast of PUCCH- ConfigurationList_ucast and PUCCH-config_mcast, respectively.

- Down-select between the two alternatives.

- UE is expected to generate a single codebook with high priority for unicast, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast of PUCCH- ConfigurationList_ucast.

- If a separate PUCCH-ConfigurationList_mcast is configured for multicast, the two separate codebooks for multicast are low and high priority, respectively.

  - Alt1:

    - UE is expected to generate a single codebook with low priority, including the HARQ-ACK feedback for unicast and multicast, and then determines the PUCCH resources for transmission across PUCCH-config_ucast of PUCCH-ConfigurationList_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

    - UE is expected to generate a single codebook with high priority, including the HARQ-ACK feedback for unicast and multicast, and then determines the PUCCH resources for transmission across PUCCH-config_ucast of PUCCH-ConfigurationList_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

  - Alt2:

    - UE is expected to first generate two separate codebooks with low priority for unicast and multicast, respectively, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast of PUCCH-ConfigurationList_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast, respectively.

    - UE is expected to first generate two separate codebooks with high priority for unicast and multicast, respectively, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast of PUCCH-ConfigurationList_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast, respectively..

  - Down-select between the two alternatives.

- If neither a separate PUCCH-config_mcast nor a PUCCH-ConfigurationList_mcast is configured for multicast

- UE is expected to generate a single codebook with low priority, including the HARQ-ACK feedback for unicast and multicast, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast of PUCCH- ConfigurationList_ucast.

- UE is expected to generate a single codebook with high priority, including the HARQ-ACK feedback for unicast and multicast, and then determines the PUCCH resources for transmission corresponding to PUCCH-config_ucast of PUCCH- ConfigurationList_ucast.

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | The motivation for some aspects of the proposal is not understood. For example, why should the CB for multicast be LP if there are HP/LP CBs for unicast? In general, the proposal has the RRC configurations for PUCCH resources determine whether a UE jointly or separately generates CBs. However, those aspects are not necessarily related and, when they do, the former (PUCCH Configs) is a consequence of the latter (CB generation and corresponding PUCCH transmission aspects) instead of the reverse. Similar to the above, discussion on required PUCCH configurations can happen at a later time, as usual. |
| ZTE | Similar comments as in previous proposal. |
| Lenovo, Motorola Mobility | We share same concern with Samsung and ZTE. |
| FL’s comments | To all, I’d like to clarify the intention of these two proposals in this section. Earlier we have agreed “Concatenation of Type-2 … is supported” and “construction of Type-1 … based on union TDRA sets… is supported”. However, it is unclear or undecided yet when or in which cases such concatenating or basing on union TDRA is supported esp. when others also propose different Type-1 schemes from “union TDRA” but rather similar to “concatenating of Type-1 sub-codebook of unicast and that of multicast ”. This is the reason I suggest this proposal. With this proposal, we or at least me as FL is confused how the codebook is exactly constructed in different cases as others commented in Round-1 and as I proposed here. I assume companies should agree with me this is important for further more discussion on detailed issues later and this is why I suggested as the first issue in this section. Surely, if companies have different views, please comment. To Samsung, To answer your question, if a separate PUCCH-config for multicast is configured, I assume it is corresponding the CB with low priority for multicast, as what is specified for unicast. Again, it is not purely RRC aspects for configuration of PUCCH resources but rather what codebooks should be constructed in different cases esp. for low/high for unicast and low/high for multicast. The proposal is indeed corresponding to PUCCH-config also as proposed by companies as reflected in the two alternatives. Down-selection will be done anyway. |
| LG | We generally share the same view with Samsung, ZTE and Lenovo. |
| Nokia, NSB | Please see our reply under previous proposal. |
| vivo | We generally share the same view with Samsung and ZTE. |

### Round-4

#### FL’s suggestion:

- The point of this proposal is right the attempt to refrain us from going into a circle that whether UE should generate the codebooks first and then check the PUCCH resources is overlapping or not including both the same and different priorities, or UE should by default construct a single codebook including HARQ-ACK bits for unicast and multicast when they are the same priority especially and then determine the PUCCH resources for transmission.

- Also, there are different opinions whether generating separate codebooks depends on whether separate PUCCH-config or depends on whether different the codebook priorities. The two open alternatives are to reflect both.

- It is not possible to converge in this meeting, so I suggest companies further discuss this issue in the contribution taking this proposal into account as well as others comments/views.

- As explained, it is important and should be solved or at least keep people on the same page when discussing the details of HARQ-ACK codebooks. Surely, if you have different assessment, welcome to express as well.

## Non-overlapping PUCCH in the same slot

Submitted Proposals

“sub-slot based PUCCH”

(ZTE) Proposal 6:

  - Regarding the case of non-overlapping PUCCHs resources for HARQ-ACK in the same UL slot:

- HARQ-ACK PUCCH overriding mechanism should be reused for multicast with the same priority.

- if HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast are determined in the same UL slot, then

•	The overriding/multiplexing should be used for the two HARQ-ACK PUCCHs for the same priority (regardless of whether they overlap in the time domain).

•	The final PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

- sub-slot based PUCCH transmission for HARQ-ACK is supported.

(Spreadtrum) proposal 2:

  - Sub-slot based PUCCH transmission for HARQ-ACK is supported.

(Nokia) proposal 4:

  - Rel-16 sub-slot based PUCCH configurations and mechanisms are supported also for at least UE-specific ACK / NACK based feedback for MBS.

(CMCC) proposal 1:

  - Support sub-slot based PUCCH transmission for multicast HARQ-ACK.

“Non-overlapping slot-based PUCCH”

(Huawei) Proposal 1:

  - For the cases of unicast and multicast scheduled from the same TRP,

    - UE can support two non-overlapping PUCCHs for multicast and unicast, respectively, with different priorities in the same slot.

    - UE can be configured to generate two separate codebooks for unicast and multicast, respectively.

      - Unicast and multicast can be with the same or different priorities.

(Huawei) Proposal 4:

  - For the cases of unicast and multicast scheduled from different TRPs,

    - UE can support two non-overlapping PUCCHs for unicast and multicast, respectively, with same priorities in the same slot.

(Huawei) Proposal 5:

  - For the cases of UE being configured with mTRP and for the TRP configured to schedule both unicast and multicast,

    - UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot.

(vivo) Proposal 9:

  - For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast,

    - For the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot, separate transmission of HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast is supported.

(Spreadtrum) proposal 1:

  - Not support multiple non-overlapping PUCCH resources for HARQ-ACK in the same slot

(Nokia) proposal 5:

  - As in Rel-16 unicast operation, the UE receiving MBS services, based on UE capability, support multi-TRP operation configured with ackNackFeedbackMode-r16 = separate, i.e., the UE can transmit two non-overlapping PUCCHs including HARQ-ACK feedback per (sub-)slot, i.e., one for each TRP.

(CMCC) proposal 2:

  - For UEs with M-TRP capability, support transmitting two slot based PUCCHs for multicast HARQ-ACK in a slot.

(CMCC) proposal 3:

  - If separate PUCCH(s) is configured for HARQ-ACK for multicast, the PUCCH structure for PUCCH-Config of multicast and unicast HARQ-ACK with the same priority index should be the same. The PUCCH structure for PUCCH-Config for multicast and unicast HARQ-ACK with different priority index can be different.

(Ericsson) Proposal 17:

  - When PUCCH resource for multicast and unicast is not overlapped in time, if UE UE has capability to transmit more than one PUCCH with HARQ per slot, then UE transmit HARQ feedback for both multicast and unicast traffic. If UE can only transmit one PUCCH with HARQ, multiplex multicast and unicast HARQ feedback into one PUCCH resource if they are configured with same priority. Otherwise, low priority PUCCH is dropped.

### Round-1 (closed)

“sub-slot based PUCCH”

FL’s comment:

Four companies clearly proposed to support sub-slot based PUCCH. Since we have agreed to support low/high priority for multicast and the meaning of low/high is very likely to be the same as that for unicast introduced in URLLC, if UE unicast is configured with sub-slot based PUCCH but if UE multicast does not support slot-based PUCCH, UE has to be configured with a separate PUCCH-config with slot-based PUCCH for multicast and we also have to face the issue what if the slot-based PUCCH for multicast collides sub-slot based PUCCH for unicast in the same slot but occasion is TDM-ed in different symbols. not supporting sub-slot based PUCCH does not seem less spec efforts apparently.

Considering UE may have the capability of support sub-slot based PUCCH for unicast and be configured by network to use it, it seems safer to support it as well for multicast. The capability can be discussed separately later. For example, whether supporting it for multicast relies on whether UE supports it for unicast or could be an additional capability for multicast.

FL’s Proposal:

#### Proposal 2.2.1-1

Support sub-slot based PUCCH for ACK/NACK based HARQ-ACK feedback for multicast subject to UE capability.

- UE capability can be discussed separately later.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We don’t see the strong motivation to support sub-slot PUCCH for multicast since multicast has no urgent timing requirements as URLLC. Support slot-based PUCCH for multicast is sufficient for multicast. FL’s response: I worry people are concerning URLLC UE is configured with sub-slot based PUCCH but multicast only supports slot based PUCCH, then we may have to solve the issue that the slot-based PUCCH for multicast collides sub-slot based PUCCH for unicast in the same slot but occasion is TDM-ed in different symbols. |
| CATT | Agree. |
| ZTE | Ok with the proposal in principle. We would suggest to delete the example in the first bullet for now. Updated proposal from ZTE: Support sub-slot based PUCCH for ACK/NACK based HARQ-ACK feedback for multicast subject to UE capability. - UE capability can be discussed separately later, e.g., whether supporting it for multicast relies on whether UE supports it for unicast or could be an additional capability for multicast. FL’s response: fine with the change as long as it clarifies the motivation. |
| CMCC | Support |
| Nokia, NSB | We support the proposal. |
| Spreadtrum | The motivation is not clear. For MBS, there is no such low latency requirement as Rel-16 URLLC. FL’s response: multicast could be used for V2X in Uu which may require low latency as well. |
| vivo | Ok in principle. One question for the main bullet, it says” PUCCH for ACK/NACK based HARQ-ACK feedback for multicast”, does it mean for “PUCCH for NACK-only based HARQ-ACK feedback for multicast”, separate discussion is needed? FL’s response: I prefer to separate the discussion from NACK-only. This issue is put under ACK/NACK specific |
| Apple | General ok with this proposal. we are just curious about the MBS use case with sub-slot PUCCH feedback. FL’s response: multicast could be used for V2X in Uu which may require low latency as well |
| Samsung | Do not support the proposal. There is no motivation for the proposal as there is no multicast service in the Rel-17 WID that requires the strictest URLLC latency. Further, short latencies can also be achieved by gNB configuration of slot-based PUCCH resources. FL’s response: multicast could be used for V2X in Uu which may require low latency as well |
| OPPO | Fine with the proposal. |
| Qualcomm | Fine with the main bullet. We prefer to delete the subbullet. |
| Ericsson | Support |
| NTT DOCOMO | We are fine with the proposal. |
| MTK | Support sub-slot based PUCCH subject to UE capability. |
| Qualcomm2 | The subbullet is not needed. Anyway, the UE capability will be discussed later. |
| Lenovo 2 | Are you referring to a URLLC UE receiving unicast and multicast? Still, we think it is better to focus on the basic feature for a MBS UE at the first step. |

“Non-overlapping slot-based PUCCH”

FL’s comment:

There is statement from company’s tdoc that only one HARQ-ACK PUCCH for the same priority can be transmitted in one UL slot for unicast, which does not seem to consider the case that UE being configured with mTRP can transmit two non-overlapping PUCCH in the same slot with the same priority, in which case UE is configured to generate “separate” codebooks. If UE supports mTRP and multicast is scheduled from the two TRPs, it is straightforward to support two non-overlapping slot-based PUCCH with the same priority in the same slot. If it is the case, it is also nature for UE to support two non-overlapping slot-based PUCCH with the same priority for multicast and unicast respective, e.g., unicast from TRP0 and multicast from TRP1.

Regarding the case of unicast and multicast scheduled from the same TRP, UE supports two non-overlapping slot-based PUCCH with different priorities for unicast. From this sense, it is also nature for UE to support two non-overlapping slot-based PUCCH with different priorities for multicast and multicast and unicast.

Two separate proposals are suggested for discussion to address the case of multicast only and the case of multicast and unicast respectively corresponding to the two non-overlapping slot-based PUCCH.

FL’s Proposal:

#### Proposal 2.2.1-2

Support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast subject to UE capability.

- The two PUCCHs can be the same or different priorities.

  - If the multicast is scheduled from different TRPs, the two non-overlapping slot-based PUCCHs can be with the same priority.

  - If the multicast is scheduled from the same TRP, the two non-overlapping slot-based PUCCHs are with different priorities.

- UE capability can be discussed separately later.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We are OK with two PUCCHs with different priorities. But for two PUCCHs having same priorities, why do we need support two non-overlapping slot-based PUCCH for multicast? FL’s response: in the case that multicast are scheduled from two different TRPs. |
| LG | Considering support of multiplexing HARQ-ACK feedbacks, we wonder why we additionally need to support two non-overlapping slot-based PUCCHs which seem not supported even for URLLC. FL’s response: in the case that multicast are scheduled from two different TRPs. |
| ZTE | Overall, we are fine with the proposal. However, the note is not very clear from our perspective. We suggest the following updates. Updated proposal from ZTE: Support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast subject to UE capability. - The two PUCCHs can be the same or different priorities. - If the multicast is scheduled from different TRPs, the two non-overlapping slot-based PUCCHs can be with the same priority. - If the multicast is scheduled from the same TRP, the two non-overlapping slot-based PUCCHs are with different priorities. - Note: The same priority case corresponds to the case of multicast scheduled from mTRP. The different priorities case corresponds to the case of multicast scheduled with different priorities from the same TRP. - UE capability can be discussed separately later. FL’s response: Ok with this suggestion. |
| Nokia, NSB | We do not support the proposal. First, we need to have a right understanding as the RAN1 group. In our view, based on Rel-16 specifications below: 38.213 S.9.2.3: "A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot”. Thus, if there is only one TRP, then the UE can transmit one PUCCH with HARQ-ACK feedback in the same (sub-)slot. Thus, the UE can only transmit more than one PUCCH with HARQ-ACK feedback for multicast in the same slot with different priorities, in case one multicast service is low and the other multicast service is high priority, and PUCCH-configs for those services are configured with a sub-slot based architecture, so that each sub-slot can contain a PUCCH with HARQ-ACK. One slot can include 2 or 7 sub-slots, therefore the UE is capable of transmitting 7 HARQ-ACK feedback per slot. The UE can transmit PUCCH including HARQ-ACK for each slot, so the UE can also transmit up to 7 PUCCHs per slot for a service. Thus, at least for Rel-16 specifications, in our view, the below statement is not correct, since the UE can transmit only one PUCCH with HARQ-ACK in the same (sub-)slot: “Regarding the case of unicast and multicast scheduled from the same TRP, UE supports two non-overlapping slot-based PUCCH with different priorities for unicast.” FL’s response: the explanation you gave above is sub-slot based PUCCH, here the proposal is saying slot-based PUCCH Our explanation was both for slot and sub-slot based PUCCH. So, if there is only one TRP, then the UE can transmit one PUCCH with HARQ-ACK feedback in the same slot. With current specs, our understanding is, the UE is not allowed to send two slot-based PUCCHs including HARQ-ACK feedback per slot. Therefore, we think that the following sub-bullet below intends to modify the specs. We do not see motivation to do that. - If the multicast is scheduled from the same TRP, the two non-overlapping slot-based PUCCHs are with different priorities. In Rel-16 multi-TRP operation, the UE can be configured with ackNackFeedbackMode-r16 = separate, so that the UE can transmit two non-overlapping PUCCHs including HARQ-ACK feedback per (sub-) slot, i.e., one for each TRP. In our understanding, priority does not play a role here, so two transmissions can be either same or different priorities. FL’s response: spec does not restrict it has to be the same so theoretically yes can be the same or different. My point is it implies at least the same because actually R16 mTRP discussion does not consider URLLC and URLCC does not consider mTRP. Along with the proposal 2.2.1-1 above, we believe that for the current mechanisms to be kept also for multicast (allowing us to avoid a separate proposal 2.2.1-3), we propose the following proposal: Support two non-overlapping same-slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast/multicast and multicast/unicast subject to UE capability. - The two PUCCHs can be the same or different priorities. - Note: Both cases only apply when mTRP. - UE capability can be discussed separately later. |
| vivo | We think the different priorities case can correspond to the case of multicast scheduled with different priorities from the same TRP or different TRPs. FL’s response: spec does not restrict it has to be the same so theoretically yes can be the same or different. My point is it implies at least the same because actually R16 mTRP discussion does not consider URLLC and URLCC does not consider mTRP. |
| Apple | Does this proposal propose to support non-overlapping between sub-slot based and slot based PUCCHs? FL’s response: No. The proposal says what it literally says: support two non-overlapping slot-based PUCCHs |
| Samsung | Do not support the proposal for the case of same priority – that case can be treated as Rel-16 M-TRP with ideal backhaul (multiplexing applies). If a UE does not support 2 priorities, it should not have to support multiple PUCCHs with HARQ-ACK in a slot. The proposal may be considered only for the case of different priorities. FL’s response: What if the multicast are scheduled from two different TRPs? |
| OPPO | Fine with the proposal. |
| Ericsson | Support |
| NTT DOCOMO | Not support. Current specification is based on the principle that only one PUCCH with HARQ-ACK can be sent per TRP in a slot. The principle should be kept in Rel-17. We see no benefit in changing the principle in Rel-15/16. FL’s response: Current specification supports non-overlapping PUCCHs in the same slot for different priorities. |
| MTK | What is the meaning of “UE capability” in the main bullet? Does it mean that if UE support mTRP, the two PUCCH corresponds to each TRP with the same priority of if UE has the single TRP, it still support two non-overlapping PUCCH with different priority (which means that UE not supporting m-TRP still needs to transmit two PUCCH with HARQ-ACK per slot)? FL’s response: Ue capability is anyway needed but how the capability is formulated can be further discussed, .e.g., support URLLC or mTRP etc… |
| Qualcomm2 | 2.2.1-2 and 2.2.1-3 can be merged together. The intention here is to limit two slot-based PUCCHs from same TRP (for unicast+unicast, for multicast+multicast, or for unicast+multicast) need to have different priorities? Then the proposal can be simplified as: Support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast or two multicasts, respectively, subject to UE capability. - If from the same TRP, the two non-overlapping slot-based PUCCHs are with different priorities. - UE capability can be discussed separately later. |
| OPPO1 | In our view, same priority is possible only when multicast is scheduled form different TRPs and non-ideal backhaul: Support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast subject to UE capability. - The two PUCCHs can be the same or different priorities. - If the multicast is scheduled from different TRPs and non-ideal backhaul (if supported for multicast), the two non-overlapping slot-based PUCCHs can be with the same priority. - If the multicast is scheduled from the same TRP Otherwise, the two non-overlapping slot-based PUCCHs are with different priorities. - UE capability can be discussed separately later. |
| NTT DOCOMO2 | We correct our previous comment. Current specification is based on the principle that only one PUCCH with HARQ-ACK can be sent per priority index per TRP in a slot/sub-slot. The principle should be kept in Rel-17. For clarification, we prefer to change as below. If the multicast is scheduled from different TRPs and if the UE is provided ackNACKFeedbackMode = separate, the two non-overlapping slot-based PUCCHs can be with the same priority. |
| CMCC2 | DCM’s update version seems clearer. |
| vivo2 | For the case with the same priority, DCM’s update version seems better. |

#### Proposal 2.2.1-3

Support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast, respectively, subject to UE capability.

- The two PUCCHs can be same or different priorities.

  - If the multicast and unicast are scheduled from different TRPs, the two non-overlapping slot-based PUCCHs can be with the same priority.

  - If the multicast and unicast are scheduled from the same TRP, the two non-overlapping slot-based PUCCHs are with different priorities.

- UE capability can be discussed separately later.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We are OK with two PUCCHs with different priorities. But for two PUCCHs having same priorities, why do we need support two non-overlapping slot-based PUCCH for multicast and unicast? FL’s response: in the case that unicast is scheduled from TPR0 and multicast is scheduled from multicast. |
| LG | Considering support of multiplexing HARQ-ACK feedbacks, we wonder why we additionally need to support two non-overlapping slot-based PUCCHs which seem not supported even for URLLC. FL’s response: in the case that unicast is scheduled from TPR0 and multicast is scheduled from multicast. |
| ZTE | Similar comments here. Updated proposal from ZTE: Support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast, respectively, subject to UE capability. - The two PUCCH can be same or different priorities. - If the multicast and unicast are scheduled from different TRPs, the two non-overlapping slot-based PUCCHs can be with the same priority. - If the multicast and unicast are scheduled from the same TRP, the two non-overlapping slot-based PUCCHs are with different priorities. - Note: The same priority case corresponds to the case of multicast and unicast scheduled from different TRPs. The different priorities case corresponds to the case of multicast and unicast scheduled with different priorities from the same TRP. - UE capability can be discussed separately later. FL’s response: fine with change. |
| Nokia, NSB | We do not support the proposal. Please see our response for 2.2.1-2. |
| Vivo | Agree with the main bullet and the first bullet. For the note, we think the same priority case can correspond to the case of multicast and unicast scheduled from same or different TRPs. The different priorities case can also correspond to the case of multicast and unicast scheduled with different priorities from the same TRP or different TRPs. FL’s response: spec does not restrict it has to be the same so theoretically yes can be the same or different. My point is it implies at least the same because actually R16 mTRP discussion does not consider URLLC and URLCC does not consider mTRP. |
| Samsung | Do not support the proposal. Relates to proposal 2.2.1-2. Also, the note is not correct. For M-TRP, the case of 2 PUCCHs was introduced to support non-ideal backhaul – that is not applicable to the present case. FL’s response: what if unicast is from TRP0 but multicast is from TRP1? |
| OPPO | Fine in principle. However, the Note seems not consistent with RAN1 agreement achieved before, where separate PUCCH-config could be configured for multicast and unicast even they are same priority. FL’s response: don’t follow how this proposal is related to the separate PUCCH-config |
| Ericsson | Support |
| NTT DOCOMO | Not support. The reason is described above. FL’s response: see the response above. |
| MTK | The similar question as we commented in Proposal 2.2.1-2. FL’s response: see the response above. |
| Qualcomm2 | See comments for 2.2.1.2. |
| OPPO1 | If separate PUCCH-config is configured for unicast and multicast, non-overlapping PUCCH can be configured for unicast and multicast respectively, regardless of the priority and TRP. Any restriction on gNB from doing that? |
| NTT DOCOMO2 | We want the same clarification as described in Proposal 2.2.1-2. |
| CMCC | Same question as vivo and OPPO, if multicast and unicast is scheduled from different TRPs, two non-overlapping PUCCHs can be transmitted regardless same priority or different priorities. Support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast, respectively, subject to UE capability. - The two PUCCHs can be same or different priorities. - If the multicast and unicast are scheduled from different TRPs, the two non-overlapping slot-based PUCCHs can be with the same priority. - If the multicast and unicast are scheduled from the same TRP, the two non-overlapping slot-based PUCCHs are with different priorities. - UE capability can be discussed separately later. |

### Round-2 (closed)

The discussion on the “large question” regarding whether supporting m-TRP and whether supporting URLLC features are directly written in emails. The views are summarized in the following table.

Discussion on whether support “M-TRP”or “URLLC features (sub-slot based)”:

| Company | Comments |
| --- | --- |
| Samsung | Not sure how to continue the discussion in response to “FL comments”. It is a very challenging work environment but GTW time is also too short, especially to open relatively ‘large’ questions. There are several comments I would like to continue discussion on, but updating the document may not be the most efficient way in a group setting. Maybe it is worth considering a direct email discussion. A couple of quick clarifications/comments to make. - Regarding whether or not to introduce support of sub-slots, the FL comment is “multicast could be used for V2X in Uu which may require low latency as well“. However, V2X is not the scope of the WID and V2X does not support sub-slots either. - Regarding why multiplexing of same priority HARQ-ACK should be in the same codebook, the FL suggestion to not do that is because “what if unicast is from TRP0 but multicast is from TRP1?“. But that does not matter. Separate codebooks/PUCCHs were introduced in Rel-16 MIMO only to deal with non-ideal backhaul for M-TRPs – otherwise, multiplexing and a joint CB applies as for CA. Clearly, there is no backhaul issue for multicast and there is no need to expect M-TRP capabilities from a UE in order to support multicast. |
| FL’s responses | - FL’s responses: we have agreed multicast has low and high priority, V2X could be a case for high reliability. My point is not V2X supporting sub-slot. Rather, the point is considering high priority is probably as the same priority as URLCC, for example, multicast used for V2X, supporting sub-slot has the same motivation as what is motivated for URLLC. - FL’s responses: I am not suggesting not to do that from feature lead perspective. Instead, as FL I see the arguments from others that unicast could be from TRP0 but multicast is from TRP1. For the “large” question esp. whether multicast can be scheduled from different TRP, we never directly discussed this question yet to my knowledge, including both ideal or non-ideal backhaul cases. From FL’s perspective, I would like or appreciate to see companies’ views on this, but currently I don’t follow why no backhaul issue for multicast from your point of view? |
| Samsung2 | [Aris2]: HP (or high reliability) does not mean strictest latency. Even for the strictest latency in URLLC, only HARQ-ACK may be sub-slot based and even that did not need to happen – it complicated the specifications and, if another layer (multicast) of “sub-slot” is now allowed, that will further increase. Even if V2X was in scope, there is no identified V2X application that requires sub-slot based PUCCH for HARQ-ACK. The objective of the (“small”) WID is to have basic specifications enabling fast MBS deployment in the market without impacting UE HW complexity. There should not be features that do not have an association with a requirement from the WID (with a possible exception of simple ones or of ones with identifiable benefit). [Aris2]: HP (or high reliability) does not mean strictest latency. Even for the strictest latency in URLLC, only HARQ-ACK may be sub-slot based and even that did not need to happen – it complicated the specifications and, if another layer (multicast) of “sub-slot” is now allowed, that will further increase. Even if V2X was in scope, there is no identified V2X application that requires sub-slot based PUCCH for HARQ-ACK. The objective of the (“small”) WID is to have basic specifications enabling fast MBS deployment in the market without impacting UE HW complexity. There should not be features that do not have an association with a requirement from the WID (with a possible exception of simple ones or of ones with identifiable benefit). |
| FL’s responses2 | Thanks Aris for the response and I added more inline. Btw, @all, I appreciate other companies can join the discussion also, esp. the proponents of sub-slot based PUCCH and mTRP for multicast. What I responded to Aris is more purely from FL’s perspective though I tend to agree with Aris on some points from Huawei point of view. - FL’s responses2: firstly, from Huawei point of view, I agree with you that we prefer to see multicast functions works in Rel-17 for fast deployment without need to over-specify unnecessary complex features. However, from FL’s perspective, for my understanding, would you see problems or much spec impact if unicast supports/configured with sub-slot PUCCH but multicast only supports slot-based PUCCH esp. when multiplexing the HARQ with the same priority within the same PUCCH? - FL’s responses2: Now I see what you mean by “no backhaul” issue. For better understanding your point and asking the question from FL’s perspective, multiple TRP does not necessarily mean SFN which is out of scope but rather for high capacity or just say SFN is only one of cases for multiple TRP, please correct me if I am wrong. Also, if mTRP is supported by UE and deployed by network assuming non-ideal backhaul, does it motivate UE to support two PUCCHs with the same priority with one for unicast and the other for multicast? |
| Samsung3 | Thank you again for the follow up. I hope the discussion is useful although I understand email is not very convenient. I continue the discussion inline. [Aris3]: There are two aspects – specification and UE implementation requirements. Rel-16 UEs support 1 PUCCH with HARQ-ACK per slot as baseline (and there can be more PUCCHs with HARQ-ACK per slot if the UE support certain features such as M-TRP). Assuming no additional requirements for the UEs supporting multicast, it is not reasonable to assume that Rel-17 MBS is targeting only UEs supporting M-TRP (or sub-slot PUCCH) – there are actually no such UEs currently in the field. So, MBS has to support UEs that can transmit only 1 PUCCH with HARQ-ACK per slot. The specification impact may vary (probably from small to moderate) depending on discussions/decisions that would need to be made. For example, if allowed, would HP+HP HARQ-ACK multiplexing (multicast + URLLC) be conditioned on the sub-slot of multicast being same (e.g. not larger) as the sub-slot of URLLC? (same for LP + LP (eMBB + multicast) HARQ-ACK multiplexing if the sub-slot is smaller than the slot). Will sub-slot LP SPS HARQ-ACK and CSI/SR multiplexing be allowed and is it OK if the resulting PUCCH resource is not in the sub-slot and overlaps with another sub-slot with other LP SPS HARQ-ACK? … [Aris3]: Yes, I agree about SFN (out of scope). I also agree that for non-ideal backhaul between unicast and multicast TRPs, separate corresponding PUCCHs are needed. But that will require M-TRP capabilities for all served UEs to support MBS. It is then also not clear whether having no CSI enhancements is OK (and several other issues will need to (re)open). |
| FL’s responses3 | The discussion is definitely useful at least to me (as feature lead). Still for better understanding your points and asking the questions from FL perspective, see responses inline. - FL’s responses3: At least from FL perspective, when I draft the proposal, I suppose the R15 feature should be the baseline esp. the mandatory feature and additionally consider the optional features supported in R16 (M-TRP, sub-slot PUCCH) esp. for UE with capability to support such R16 features, so it is not targeting only UEs supporting such R16 features. The angle I am thinking the problem is whether it cause trouble or additional efforts (including spec and implementation) if UE supports and being configured with the R16 optional features for unicast but not support that for multicast. - FL’s responses3: Thanks for the answer firstly. When I asked this question, I am not sure about the answer frankly. However, I’d like to hear the views from editor’s perspective and also views from other companies for better judging the situation as FL. - FL’s responses3: If I understand you correctly, it does not necessarily require M-TRP capability for all served UEs to support multicast. For example, UE1 and UE2 in the group for multicast, UE1 with capability of M-TRP but UE2 not, network transmits multicast from TRP0 to both UE1 and UE2 and transmit unicast from TRP1 to UE1, it is still possible works, right? |
| Lenovo | Thank you all for the insightful discussion. For the reliability improvement of MBS, I kind of feel that almost all the proposals on HARQ-ACK feedback from FL are closely relevant to M-TRP and URLLC. Especially when unicast and multicast are transmitted from different TRPs, or with different priorities, the HARQ-ACK codebook determination for slot-based PUCCH or sub-slot PUCCH is quite complex, which delays the progress of this item. Meanwhile, for fast deployment of MBS, combining the basic feature of MBS with URLLC and M-TRP is not necessary. We don’t see a need for a MBS UE to support low latency requirements like URLLC and joint HARQ-ACK codebooks as M-TRP. So one suggestion from my side, can we discuss HARQ-ACK feedback mechanism just from single TRP case for time being and consider multi-TRP/URLLC in the next step? Thanks. |
| LGE | Thank you all for good discussion. We generally have similar view with Lenovo on how to make progress on HARQ-ACK. As we previously commented, we could focus on unicast and multicast in a slot with a same priority for a single TRP for the time being. Considering tight relation with URLCC and MTRP, we concern inefficient discussion on some issues which stem from other WIs. So, we prefer to consider the issues on multie TRP, differnet priorities, and sub-slot as the next step which could start later after some progress on R17 MTRP/URLCC. |
| CMCC | Thank you for the discussion about the relationship between multicast and URLLC/MTRP. From our perspective, as we have the agreement to support different priorities for multicast service, it is natural to support sub-slot based PUCCH to reduce HARQ-ACK feedback latency for high priority multicast service. As the complexity issue, we think the multiplexing/prioritization rule is the same for slot/sub-slot if we restrict the PUCCH structure of multicast and unicast is the same in Rel-17. That is we don’t need to discuss how to multiplex or prioritize for overlapped sub-slot based PUCCH for unicast and slot based PUCCH for multicast. As the MTRP feature, we think we can postpone the discussion consider the first step deployment. Hope this suggestion can make a progress for the group. |

FL’s Comments

Based on the comments for these “large” questions, at least we can focus on the case that UE multicast or multicast and unicast are scheduled from the same TRP and the cases regarding different TRPs can be discussed later. Since we have agreed multicast support low/high priorities, at least some features from URLLC are expected to be considered for multicast.

#### Proposal 2.2.2

At least NR supports the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively.

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | Support |
| Qualcomm | Suggest modifying the wording as: - UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability. - UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively in a slot subject to UE capability. |
| CMCC | Support |
| ZTE | OK |
| Lenovo, Motorola Mobility | We prefer Qualcomm’s version. |
| OPPO | We are wondering whether the proposal has already been implied by the following agreement. If the proposal intends to clarify that PUCCH resources configured by up to 2 PUCCH-Config could be non-overlapping in time domain, then it seems that UE also needs to support up to 4 non-overlapping slot-based PUCCH, i.e. when separate PUCCH-ConfigurationList are configured for unicast and multicast and PUCCH resource configured by any PUCCH-Config is non-overlapping with another. Agreement: For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively. - FFS other configurations |
| CATT | Agree. |
| MTK | Qualcomm’s version is fine for us. |
| NTT DOCOMO | Support |
| LG | We are generally fine with QC’s change if majority companies consider TDMed PUCCH would be necessary to support multicast. BTW, we wonder if RAN1 already concluded whether to support two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or multicast/unicast with same priorities for same TRP. Could you clarify the current status on the same priority for same TRP in RAN1? |
| Apple | OK with Qualcomm’s updates. |
| Nokia, NSB | First, we need to have a right understanding as the RAN1 group. In our view, based on Rel-16 specifications below: 38.213 S.9.2.3: "A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot”. So, if there is only one TRP, then the UE can transmit one slot-based PUCCH with HARQ-ACK feedback in the same slot. With current specs, our understanding is, the UE is not allowed to send two slot-based PUCCHs including HARQ-ACK feedback per slot. Therefore, we think that proposal intends to modify the specs. We do not see motivation to do that. |
| vivo | Generally fine with the proposal. We have the same question as LG |

### Round-3

The proposal in this round is the updated to address the comments in previous rounds.

In particular,

@OPPO, the agreement you referred to is only for multicast and it corresponds to the first sub-bullet of the proposal and the second sub-bullet is targeting the case of unicast and multicast. Even though two PUCCH-config are configured, it does not mean UE is implied to support two non-overlapping PUCCH. They could be different issues. @LGE,vivo, if multicast or unicast and multicast are from the same TRP, at least support two non-overlapping PUCCHs with different priority, which seems the middle ground we have. Whether supporting that for the same priority is still controversial for the time being.

@Nokia, I may have responded to your comment somewhere in previous rounds or last meeting. Never mind, I can repeat it. What you referred to the spec is for R15. Supporting non-overlapping PUCCHs with different priorities has been supported and was introduced by R16 URLLC, the spec I referred to is from 38213 section 9.2.3 as follow:

“A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot per priority index, if the UE is not provided ackNackFeedbackMode = separate.”

This is our understanding that no more than one PUCCH is allowed per priority index and given UE supports two codebooks with different priorities, so UE can support two non-overlapping slot-based PUCCH in the same slot. Based on the discussion of context, I think others are on the same page.

#### Proposal 2.2.3

At least NR supports the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK |
| Nokia, NSB | Thank you FL for your answer. In our view, ackNackFeedbackMode = separate is configured when HARQ-ACK codebooks for different TRPs are intended to be separately sent, and it is not related to different priorities. So, even codebooks with same priorities can have separate transmissions in the same slot from different TRPs. We do not see any limitation there in terms of priorities. However, we believe that in case of single TRP, which is the intention of this proposal, only 1 slot-based PUCCH is allowed in single TRP. We kindly ask for the clarification from the companies, and point the group where the following statement is provided in the specs: “This is our understanding that no more than one PUCCH is allowed per priority index and given UE supports two codebooks with different priorities, so UE can support two non-overlapping slot-based PUCCH in the same slot. Based on the discussion of context, I think others are on the same page.” |
| Ericsson | We would like to clarify the second bullet, in total how many PUCCH are supported by UE? If the second bullet means that there will be one pucch to transmit a common low priority codebook for unicast and multicast, and another pucch to transmit a common high priority codebook for unicast and multicast, then it seems the proposal should be agreed as a conclusion, as we cannot see a specification impact. |
| FL’s responses | @Nokia, I’d like to point another statement in the spec 38213 as follows, : The paragraphs are more directly answering your question. The first sentence says UE may transmit two PUCCHs in different symbols within a slot. Then what the two PUCCHs are in separate paragraphs divided into the case 1 “when UE is not provided ackNackFeedbackMode = separate” including the cases (case 1.1) that UE is provided “ackNackFeedbackMode = joint” and the case (case 1.2)) that UE is not provided “ackNackFeedbackMode” at all which means the case for a single TRP but URLLC , and the case 2 “when UE is provided ackNackFeedbackMode = separate” “A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot. When the UE transmits two PUCCHs in a slot and the UE is not provided ackNackFeedbackMode = separate, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2. If a UE is provided ackNackFeedbackMode = separate, the UE may transmit up to two PUCCHs with HARQ-ACK information in different symbols within a slot.” From 331: ackNackFeedbackMode-r16 ENUMERATED {joint, separate} OPTIONAL, -- Need R @Ericsson, From what is stated in spec, UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot, the total is two. The spec impact is incorporating multicast and multicast/unicast as cases of UE supporting two non-overlapping. |

## Type-1 HARQ codebook

Submitted Proposals

“FDM-ed cases”

(Huawei) Proposal 2:

  - For FDM-ed unicast and multicast from the same TRP, support Opt 1 for Type-1 HARQ-ACK codebook construction:

    - Opt 1: HARQ-ACK bits for all the PDSCH occasions over all the slots for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for multicast, for a given serving cell.

(Huawei) Proposal 3:

    - UE generates Type-1 HARQ-ACK codebook per the option specified for the FDM-ed cases only when UE is configured with FDM-ed unicast and multicast.

(ZTE) Proposal 2:

  - Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast or FDM-ed unicast and multicast, for a PDSCH SLIV group, the number of HARQ-ACK bits generated by the UE is min (N, M), where N is the number of FDM-ed PDSCHs in the PDSCH SLIV group, and M is the UE receiving capability for FDM PDSCHs.

(ZTE) Proposal 4:

  - For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast in the same slot with the same TRP to be multiplexed in the same PUCCH resource, Alt.3 is supported.

Alt. 3:

- the SLIV group is determined based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (regardless of whether it is FDMed between unicast and multicast).

- Each SLIV group can generate multiple HARQ-ACK bitss and specify the order of these HARQ-ACKs based on predefined rules, for example, unicast HARQ-ACK precedes multicast HARQ-ACK.

(vivo) Proposal 5:

  - For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, and Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

    - Construction of Type-1 HARQ-ACK codebook based on the concatenation of the HARQ-ACK codebooks of the FDMed unicast and multicast, or FDMed multicast and multicast, is supported.

(vivo) Proposal 6:

  - For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast in the same slot with the same TRP to be multiplexed in the same PUCCH resource, it is proposed to support

- -	to construct HARQ-ACK sub-codebook for unicast PDSCH and multicast PDSCH separately, and then concatenate the two HARQ-ACK sub-codebooks together (i.e., option 4)

(CATT) Proposal 10:

  - To support the FDMed unicast and multicast scenarios, the order of PDSCH reception between unicast and multicast when constructing Type-1 HARQ-ACK codebook can be decided as the principle that the unicast feedback information followed by multicast feedback information. The order between multiple MBS Type-1 HARQ-ACK sub-codebooks can be further studied.

(Nokia) Proposal 26:

  - The UE constructs separate Type-1 HARQ-ACK sub-codebooks using Rel-15 / 16 mechanisms for each MBS service and one sub-codebook for unicast services, when FDM-ed transmissions of unicast are multicast are allowed.

(Nokia) Proposal 27:

  - The UE concatenates the constructed Type-1 sub-codebooks and sends them in the same PUCCH resource in case their HARQ-ACK feedback is scheduled for the same time instance (slot or sub-slot).

(Nokia) Proposal 28:

  - The PHY identification of PDSCH HARQ-ACK to MBS Type-1 sub-codebook mapping is the group-common RNTI value.

(Nokia) Proposal 29:

  - The UE maps the PDSCH HARQ-ACK of unicast services scrambled with a UE-specific RNTI to the unicast Type-1 sub-codebook.

(Nokia) Proposal 30:

  - The order of concatenation of the MBS sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, follows the increasing order of the G-RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook. MBS sub-codebooks are preceded by unicast sub-codebook, as agreed for Type-2 HARQ-ACK codebook.

(Nokia) Proposal 32:

  - In a resource limited system, construction of semi-static HARQ-ACK sub-codebooks per PTM service can be avoided. Instead, for the FDM-ed PDSCH occasions, one unified bit can be included in the HARQ-ACK codebook that is to be constructed using the Rel-15 / 16 methods. This unified bit can be produced based on a logical “OR” or “AND” operation of the HARQ-ACK feedback for the FDM-ed TBs.

    - Enabling / disabling of this unification mechanism at the UE can be done via RRC signaling or DCI.

(CMCC) Proposal 10:

  - For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast in the same slot with the same TRP to be multiplexed in the same PUCCH resource, HARQ-ACK bits for all PDSCH occasions for unicast, precede, HARQ-ACK bits for all PDSCH occasions for multicast, within the same slot.

(Qualcomm) Proposal 5:

  - Proposal 5: For multiplexing of ACK/NACK feedback for unicast + multicast, or multicast + multicast

    - For Type-1 codebook, the HARQ-ACK information bits are concatenated in increasing order of

      - PDSCH reception occasion index at first

      - and then serving cell index

      - and X-RNTI (C-RNTI before G-RNTIs, and G-RNTI1, G-RNTI2… for different multicast)

      - and CORESETPoolIndex (if configured for mTRP)

    - For Type-2 codebook, the number of HARQ-ACK bit(s) and the bit order for multicast are according to the DAI of DL DCI per multicast G-RNTI.

(OPPO) Proposal 6:

  - Type-1 HARQ-ACK codebook for MBS and unicast are constructed separately;

  - No optimization on Type-1 HARQ-ACK codebook is needed for payload size reduction;

  - Same HARQ-ACK codebook type is used for MBS and unicast.

(Lenovo) Proposal 9:

  - For HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, two separate Type-1 HARQ-ACK codebooks are generated for unicast and multicast, respectively, according to the respective K1 set and TDRA table.

(Lenovo) Proposal 10:

  - For HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, Type-1 HARQ-ACK codebook for unicast is placed firstly then followed by Type-1 HARQ-ACK codebook for multicast.

(Intel) Proposal 7:

  - If MBS and unicast PDSCH are FDMed, the Type 1 HARQ codebook can be generated by concatenating two sub-codebooks, each generated by considering separately the PDSCH TDRA tables of unicast and multicast where the HARQ bits for all PDSCH occasions over all slots for all serving cells for unicast precede the HARQ-ACK bits for multicast.

(Apple) Proposal 4:

  - For FDM between unicast PDSCH and a CFR in a slot and type 1 HARQ-ACK feedback, HARQ-ACK bits over all the slots for all serving cells for unicast, precede, HARQ-ACK bits over all the slots for all serving cells for multicast.

(MediaTek) Proposal 6:

  - For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, the HARQ-ACK bit for multicast can be constructed after that of unicast.

(DOCOMO) Proposal 6:

  - For Type-1 HARQ-ACK codebook, if RAN1 is going to use different codebook construction methods for FDM case and TDM case, RAN1 should also consider how to select either.

(Ericsson) Proposal 13:

  - When multicast and unicast or multicast and multicast traffic can be FDMed in a slot, multicast traffic scheduled by one G-RNTI is treated as coming from a virtual carrier, and the HARQ codebook construction rule before R-17 can be reused for this joint type-1 HARQ-ACK codebook.

(Ericsson) Proposal 14:

  - The index of virtual carrier associated with multicast traffic can be either explicitly configured via RRC signaling or implicitly determined by predefined rules. The predefined rule to determine virtual carrier index can be FFS.

“TDM-ed cases”

(ZTE) Proposal 3:

  - For Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is for slot timing values K_1 in the union of K_1 set for unicast and K_1 set for multicast, based on the union of the PDSCH TDRA sets.

(vivo) Proposal 4:

  - For ACK/NACK based feedback for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the K1 sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

    - Candidate of PDSCH reception occasions are determined based on the union of PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured) only in the slots corresponding to the intersection of K1 set of unicast service and multicast service.

(CATT) Proposal 11:

  - To reduce the HARQ-ACK feedback payload size, the mechanism for optimizing the Type-1 codebook construction should be studied and can be based on the UE’s capability.

(CMCC) Proposal 9:

  - For Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is:

    - for slot timing values K_1 in the intersection of K_1 set for unicast (termed set A) and K_1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

    - for slot timing values K_1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

    - for slot timing values K_1 in set B but not in set A, based on PDSCH TDRA set for multicast.

(Qualcomm) Proposal 4:

  - For HARQ-ACK codebook construction of ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, prefer Alt1 to determine PDSCH reception candidate occasions.

    - Alt 1 assuming K1 set for unicast termed as set A and K1 set for multicast termed as set B

      - for slot timing values K1 in the intersection of A and B, based on union of the PDSCH TDRA sets,

      - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

      - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

(OPPO) Proposal 6:

  - Type-1 HARQ-ACK codebook for MBS and unicast are constructed separately;

  - No optimization on Type-1 HARQ-ACK codebook is needed for payload size reduction;

  - Same HARQ-ACK codebook type is used for MBS and unicast.

(Ericsson) Proposal 12:

  - When MBS traffic is configured with its own dl-DataToUl-Ack in PUCCH configuration, the number of bits in joint HARQ codebook is determined by the union of elements in the sets of K1 of both multicast and unicast where K1 of multicast is provided by dl-DataToUl-Ack in multicast PUCCH configuration and K1 of unicast is provided by dl-DataToUl-Ack in unicast PUCCH configuration or is predefined as {1,2,3,4,5,6,7,8}. The union of TDRA sets is considered at the DL slots given by the intersection of both K1 sets in multicast and unicast.

### Round-1 (closed)

“FDM-ed cases”

FL’s Comments

14 companies submitted proposals regarding Type-1 HARQ codebook construction for FDM-ed unicast and multicast from the same TRP and 13 companies tend to agree to generate sub-codebooks for unicast and multicast respectively and concatenating them by unicast sub-codebook preceding  multicast sub-codebook, i.e., option 4 as summarized in the last meeting as follows, though phrasing the proposals in different styles. Only one company (ZTE) proposed option 3, for which the main reason is for compatibility with the union of TDRA table for TDM-ed cases.

The proposal from last meeting for FDM-ed Type-1 codebook construction:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast in the same slot with the same TRP to be multiplexed in the same PUCCH resource, consider the following options

- Opt 1: HARQ-ACK bits for all the PDSCH occasions over all the slots for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for multicast, for a given serving cell.

- Opt 2: HARQ-ACK bits for all PDSCH occasions for unicast, precede, HARQ-ACK bits for all PDSCH occasions for multicast, within the same slot.

- Opt 3: HARQ-ACK bits for unicast precedes HARQ-ACK bits for multicast within one SLIV group where PDSCH occasions are overlapping.

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- Other options are not precluded.

- FFS for FDM-ed multicast and multicast.

Since the Type-1 codebook construction for TDM-ed and FDM-ed are fundamentally different and probably UE needs to be instructed from network in which manner the codebook should be generated that could be a separate issue, the compatibility of TDM-ed and FDM-ed cases does not seem essentially necessary.

FL’s Proposal:

#### Proposal 2.3.1-1

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS how UE is indicated the codebook is generated for FDM-ed cases.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Ok. |
| LG | We are fine with Opt 4 considering that this is similar to the joint Type-1 codebook for mTRP. |
| CATT | Ok with it. |
| ZTE | More discussion is needed for this issue at least for the following aspects. 1. How to determine whether it is “TDM-ed” or “FDM-ed” case. If we go with Opt 4, different mechanisms are applied for the “TDM-ed” case and “FDM-ed” case. Do we need a RRC parameter to differentiate these two cases? Or do we determine the case based on the actual scheduled PDSCHs? FL’s response: I suppose we may need to configure UE to generate the Type-1 as FDMed, otherwise, UE does not expect network will schedule via FDM. That is why the FFS is added. 2. If UE receives more than one multicast service in one cell, how to construct the codebook for these multicast? FL’s response: I guess it could be in the order of e.g., unicast Type1 subcodebook, multicast 1 Type1 subcodebook, multicast 2 Type1 subcodebook, …, which seems easily extendable than option 3 3. If more than one MBS service is configured for the UE, the UE capability of simultaneous reception needs to be considered in generating the codebook. Otherwise, there will be overhead issue. Compared with Opt 4. Opt 3 can resolve the above issues. Thus, we suggest to go with Opt 3. FL’s response: more than one multicast could be discussed later and wait RAN2 LS but as least option 4 seems easily extendable than option 3 for this case. |
| CMCC | We can accept opt 4. |
| Nokia, NSB | We support Opt 1. In our view, most companies’ proposals are closer to Opt 1 then Opt 4, since the difference of Opt 1 and Opt 4 is small and most companies haven’t mentioned the difference directly within their proposals, but in the description part of their contributions. FL’s response: if concatenating unicast sub-codebook followed by multicast sub-codebook is favorable in general, it should be option4. Note opt1 is “for a given serving cell.” |
| Spreadtrum | We are fine with Opt4 We are also fine with Opt4 applied for inter-TDMed, and intra-TDMed multiplexing case for Type-1 codebook. Unified solution is preferred as we have done in Rel-16 M-TRP for Type-1 codebook. |
| vivo | Support in principle. |
| Apple | We support this Proposal. |
| Samsung | Support the proposal. No need to change the Rel-16 HARQ-ACK codebook generation. Proponents of other options need to provide a motivation for introducing an additional mechanism over Rel-16. Also, only “Opt.4” can apply for Type-2 or, in general, if one of the codebooks is not Type-1. |
| OPPO | Support the proposal. |
| Qualcomm | We are fine with Opt 4. But confused by the following comment from FL. What about the case that unicast and multicast is FDM-ed in slot#1 and TDMed in slot#2? We think a unified solution to determine the order of HARQ-ACK bits is necessary. What is the problem to use Opt 4 for TDMed case as well? “Since the Type-1 codebook construction for TDM-ed and FDM-ed are fundamentally different and probably UE needs to be instructed from network in which manner the codebook should be generated that could be a separate issue, the compatibility of TDM-ed and FDM-ed cases does not seem essentially necessary. ” FL’s response: The point is we have agreed to use union TDRA table for Type-1 TDM-ed, at least it is a nature way for the same priority as URLLC. On top of that, what is the motivation to introduce additional different mechanism for TDM-ed case esp. for the same priority case? |
| Ericsson | Support Opt.4. Regarding FFS, we think it needs to be clarified why UE needs an indication. FL’s response: As Type-1 codebook constructions are different for TDM-ed and FDM-ed cases, whether TDM or FDM is normally network scheduling manner but Type-1 is essentially semi-statically determined codebook size, so UE should decide whether Type-1 is constructed as TDM-ed or FDM-ed case. |
| NTT DOCOMO | How the codebook construction method is selected will affect the UE processing complexity. We prefer to discuss how to select codebook construction method as a priority. FL’s response: Not sure I understand the comment. If how the codebook is generated is unclear, what ground should be based for the selection? |
| Qualcomm2 | To FL, RAN1 only agreed to use a union TDRA table, which means to consider both unicast PDSCH occasions and multicast PDSCH occasions, but no agreement on the order of HARQ information bits. We don’t support different mechanism for TDM-ed and FDM-ed cases, neither support additional signaling to indicate which mechanism is used. Repeat the question again: What about the case that unicast and multicast is FDM-ed in slot#1 and TDMed in slot#2? FL’s response: since the union TDRA tables only works for TDM-ed which cannot be directly used for FDM-ed, so the mechanism for TDM-ed can FDM-ed cases are definitely different. For the question you asked, the solution would be if UE is configured to generate the codebook assuming FDM, and then two bits are expected for each SLIV even though a single PDSCH may actually schedule on that SLIV. |
| NTT DOCOMO2 | To FL, sorry for confusion. Our concern is the same as ZTE’s concern 1. If the construction method is configured to the UE, the UE processing will be simple. But if the UE has to choose dynamically, e.g., based on actually assigned PDSCH, the UE processing will be more complex. FL’s response: Not sure if I get the point. Since the Type-1 construction for TDM and FDM are different, it is worth being discussed further whether UE generates it for FDM or TDM. It is likely UE is configured semi-statically for this; otherwise FDM should be always assumed for each SLIV. |
| CMCC2 | It seems companies are discussing about whether the TDM-ed and FDM-ed scheduling pattern is semi-statically configured or dynamically switched. From our perspective, the semi-statically scheduling is preferred if we take Opt 4 for FDM-ed case, otherwise, we may need new design for mixed unicast and multicast simultaneously scheduling pattern as Qualcomm concerned. |
| ZTE2 | Let’s take the following example from FL as an example. 1. In this example, UE needs to receive more than one multicast service (let’s say N) in one cell. If Option4 is adopted, UE needs to generate N subcodebooks for multicast. However, if UE doesn’t support FDM case, then Opt 4 will end up with huge overhead for HARQ-ACK bits. Opt 3 can reduce the overhead by taking the UE capability into account for each SLIV, which is the same as what we have for Rel-15. FL’s response: If UE does not support FDM, network should naturally not schedule in FDM way, right? 2. In this example, UE needs to generate N subcodebooks for multicast. If the DCIs for one of the sub-codebook has been missed, then there will be ambiguity issue. Opt3 can address this issue by always assume there is PDSCH transmitted in the SLIV group. FL’s response: If UE is to generate the codebook as FDM-ed matter, UE will always assume two bits FDMed PDSCH in each SLIV, Type-1 codebook size should not depend on dynamic scheduling or whether DCI is missed or not, which is the essential difference from Type-2. ----- Example ---- 2. If UE receives more than one multicast service in one cell, how to construct the codebook for these multicast? FL’s response: I guess it could be in the order of e.g., unicast Type1 subcodebook, multicast 1 Type1 subcodebook, multicast 2 Type1 subcodebook, …, which seems easily extendable than option 3 |
| Qualcomm3 | For the case with mixed TDMed and FDMed, agree with the comments of CMCC. We think the only way to make it work is to semi-statically configure the FDMed codebook (similar as mTRP by configuring the CORESET Pool), instead of depending on dynamic actual FDMed transmission. If keeping FFS, it is hard to agree with this proposal for FDM-ed case. So we suggest to revise the proposal as: #### Proposal 2.3.1-1 For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast from the same TRP, support - Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP). - If UE is capable of receiving the FDMed unicast and multicast, UE is indicated semi-statically to use FDM-ed codedbook or not. |

“TDM-ed cases”

FL’s Comments

7 companies submitted proposals regarding whether optimizing Type-1 codebook and 4 (vivo, CMCC, Qualcomm, Ericsson) from which companies propose to base on the intersection of K1 sets for Type-1 codebook construction. 2 companies (ZTE and OPPO) propose to base on the union of K1 sets.

ZTE raised the issue that for basing on the intersection of K1 sets may cause codebook size ambiguity due to the omission of DCI because UE does not generate the Type-1 codebook if UE does not receive any DCI for scheduling, so proposes using the union of  the K1 sets.

Based on this situation, feature lead suggests Alt2 for Round-1, and encourages proponents to address the related concerns of Alt1.

FL’s Proposal:

#### Proposal 2.3.1-2

For Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on Alt 2 from the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Alt 1 is preferred for overhead reduction. |
| LG | We are fine with Alt 2. |
| ZTE | We support the FL proposal. As also analyzed in our tdoc, Alt.1 may leads to size ambiguity issue if UE miss all the unicast DCIs. For example, if there are 2 DCIs for unicast and 2 DCIs for multicast and they indicate that the corresponding HARQ-ACKs are transmitted on the same PUCCH, if the UE misses the 2 unicasts DCI, then according to Option 1, the UE only generates HARQ-ACK for multicast according to the K_1 of the multicast service based on PDSCH TDRA set for multicast. Obviously, the HARQ-ACK codebook size is not expected by the base station, but according to Option 2, the above problem will not occur. FL’s response: per my understanding of the concern, it may only happen when Type-1 codebook for unicast and multicast are separately generated but multiplexed again. Yes? But generating separate codebooks for the same priority does not seem agreeable right now. |
| CMCC | Not support, prefer alt 1. Regarding the concern from ZTE’s tdoc as the following, “For example, when the UE only receives some unicast (or multicast) DCI, the UE does not know whether any multicast (or unicast) DCI is missed. Therefore, the UE thinks that only unicast HARQ-ACKs are in a PUCCH, that is, unicast and multicast HARQ-ACKs are not in the same PUCCH, so in this case, does the UE generate Type-1 codebook based only on unicast K1 set?. If the UE generates a Type-1 subcodebook for multicast, then it does not comply with the above-mentioned current specification of the Type-1 codebook when unicast and multicast independently construct the Type-1 subcodebook. Otherwise, then if the base station actually sends the multicast DCI and the UE does miss the multicast DCI, then the Type-1 codebook size ambiguity will occur between the base station and the UE. “ If UE miss all unicast DCIs (or multicast DCIs), UE will only generate independent multicast HARQ-ACK codebook (or unicast HARQ-ACK codebook) as the rule in Rel-15/16 regardless Alt 1 or Alt 2 because UE considers it is not the “multiplexing case”. That is the codebook size ambiguity issue is the same for two alternatives if UE miss all unicast DCIs (or multicast DCIs). But in “multiplexing case” (UE receives both unicast DCI and multicast DCI), Alt 1 is more beneficial than Alt 2 to reduce redundant codebook bits. |
| Nokia, NSB | We support Alt 1. We believe that the size ambiguity that ZTE mentioned is anyway also there for Alt 2, in case the DCI for one service is not received at all. In addition, we see that our proposal 32, which targets further optimizations to Type-1 HARQ-ACK codebook is not treated in the summary. We kindly would like it to be considered under Type-1 HARQ codebook item. FL’s response: I moved proposal 32 here. |
| Spreadtrum | Support Alt.2. As what we have said for FDMed case, we prefer one unified solution for all multiplexing cases as we have done in Rel-16 M-TRP for Type-1 codebook. Optimization is not necessary. |
| Vivo | Support Alt 1. For the concern of ZTE. We think there is no ambiguity regarding the codebook size. Regarding the DCI omission issue, we think it is same for Alt 1 and Alt 2. |
| Apple | Both options are ok for us. |
| Samsung | Support the proposal. We do not support changes to the Rel-16 Type-1 codebook generation – there has to be a very strong motivation for such changes. Alt.2 is mandatorily supported (and will remain applicable for G-RNTI as is for any other RNTI associated with PDSCHs). Alt.1 intends to introduce an optimization for an optional feature (FDMed receptions). Even if introduced, that optimization will not result to any meaningful reduction of the Type-1 codebook size for any conceivable deployment scenario (e.g. why should the TDRA for multicast be very different than for unicast?). There are optimizations that can be made for Type-1, but they are mostly for CA and for unicast. |
| OPPO | Fine with the proposal. |
| Qualcomm | Slightly prefer Alt1. Same understanding as some companies: the DCI missing issue is common for Alt1 and Alt2. |
| Ericsson | We prefer alt1. |
| NTT DOCOMO | As noted above, we prefer to discuss how to select codebook construction method as a priority. FL’s response: see responses there. |
| ZTE2 | We understand companies’ comments that the DCI missing issue is common for Alt.1 and Alt.2. Then it seems we may need to discuss this issue separately in next meeting. Between the two alternatives, we still prefer a solution with less spec impact. If companies prefer a solution to reduce the codebook size, some proposals in Proposal 2.3.1-1 (e.g., Opt 3) can also reduce the codebook size, all these solutions can be considered together. |
| CATT | We prefer alt1. |
| Qualcomm3 | It is ok for us to compromise on Alt2. But need to make it clear Alt2 is for union TDRA table of TDMed ACK/NACK-based unicast and multicast. For FDMed unicast and multicast, no need to discuss the union TDRA table. #### Proposal 2.3.1-2 For Type-1 HARQ-ACK codebook construction for TDMed ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on Alt 2 from the two alternatives as follows: - Alt 1: - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets. |

### Round-2 (closed)

FL’s Comments

Based on the comments in the last round, companies suggested making it clear in the proposal that UE  generating the Type-1 codebook in TDM-ed or FDM-ed  manner is semi-statically configured when agreeing the FDM-ed Type-1 codebook. The proposal is updated as follows:

FL’s Proposal:

#### Proposal 2.3.2-1

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- If UE is capable of receiving the FDMed unicast and multicast, UE is indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner.

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We prefer to remove the newly added part or change it to FFS as follows: #### Proposal 2.3.2-1 For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast from the same TRP, support - Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP). - FFS: If UE is capable of receiving the FDMed unicast and multicast, UE is indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner. - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed. |
| Samsung | Support the first bullet. For the second bullet, it is not clear why it is needed. For example, for same (low) priority, the UE should always multiplex unicast and multicast CBs (the Rel-16 UE capability is anyway for one PUCCH transmission with HARQ-ACK per slot). For different priorities, the UE should not multiplex (assuming that the Rel-17 URLLC feature for supporting multiplexing of UCI with different priorities in a same PUCCH will not be supported). We prefer to remove the second bullet but OK with FFS if several companies prefer to have it. |
| Qualcomm | ok |
| CMCC | OK, fine to add FFS in second bullet |
| ZTE | Although our preference is Opt.3, we can live with the current proposal. |
| Lenovo, Motorola Mobility | We are OK with the main bullet and first sub-bullet. For the last sub-bullet, we can accept it with minor modification: - If UE reports the capability of supporting the FDMed unicast and multicast in same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner. - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed. |
| OPPO | OK |
| CATT | Generally OK with this proposal. |
| MTK | Ok with the proposal. |
| NTT DOCOMO | Support |
| Apple | OK with this proposal. |
| Nokia, NSB | We support the proposal. |
| vivo | We support the first bullet. For the second bullet, we can accept to add FFS in second bullet. |

FL’s Comments

According to the first round comments and arguments from both sides, both alternatives can work and if any concern regarding the DCI missing issue, it is applied to both alternatives. Hence, the issue essentially is whether to optimize Type-1 codebook for TDM-ed cases due to introducing multicast.

The following are the survey to each alternative:

Supportive to Alt1: (8)

- Lenovo, CMCC, Nokia, vivo, Apple, Qualcomm, Ericsson, CATT

Supportive to Alt2: (6)

- LGE, ZTE, Spreadtrum, Apple, Samsung, OPPO,

Given this situation, FL may either suggest Alt1 or further study.

FL’s Proposal:

#### Proposal 2.3.2-2

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on one of the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this update. |
| Samsung | OK with the proposal. We can have a more focused/detailed discussion next time. We would like to request from proponents of Alt. 1 to provide estimates for the Type-1 CB size reduction for few exemplary cases with single-cell operation for multicast and single cell or CA operation for unicast (for ‘typical’ TDRAs and UL/DL configurations) – particularly with CA for unicast as that is when the Type-1 size can mostly benefit from a reduction. It should also be considered that, if supported, Alt. 1 will be optional while Alt. 2 is already mandatorily implemented (e.g. for DCIs with unicast RNTI - DCIs with G-RNTI are not any different from the Type-1 CB generation perspective) and needs to be anyway supported. |
| Qualcomm | ok |
| CMCC | OK |
| ZTE | It seems the proposal itself is almost the same as what achieved in last meeting. |
| Lenovo, Motorola Mobility | OK. |
| CATT | Fine with this update. |
| MTK | OK. |
| NTT DOCOMO | Support |
| Apple | Share the views with ZTE, hope we can have agreements in this meeting. |
| Nokia, NSB | Ok |
| vivo | Ok. It would be good if we can have agreements in this meeting. |

### Round-3

The proposals are the updated to reflect the comments received.

FL’s Proposal:

#### Proposal 2.3.3-1

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK |
| Nokia, NSB | Ok. |
| Ericsson | We are OK to accept this proposal in principle. However, now FDM and TDM case are discussed separately. The codebook construction rule here is for FDM. It seems that the codebook construction rule for TDM follow the legacy. As FDM together with TDM can happen, we would like to make sure the rule to construct codebook mentioned here is extendable to FDM together with TDM case. |
| FL’s responses | @Ericsson The FFS in the proposals is right to solve the issue whether TDM-ed or FDM-ed case is assume by UE to generate Type-1 codebook. I can understand people may need more time for this sub-bullet and that is why we put FFS and the first sub-bullet should be the converged/compromised solution for FDM-ed |

Regarding the TDM-ed cases, we have no progress unfortunately in this meeting right now but strictly we have not achieved anything in the last meeting officially. I doubt we can converge given 8 vs 6 in this meeting but I may report this proposal for GTW if time allows.

#### Proposal 2.3.3-2

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on one of the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK |
| Nokia, NSB | Ok. |
| Ericsson | OK to select the alternative later. Support Alt 1. |

## Type-2 HARQ codebook

Submitted Proposals

(ZTE) Proposal 5:

  - Type-2 HARQ-ACK codebook construction for MBS:

    - The DAI count is performed per MBS service.

    - If the UE receives multiple MBS services, multiple sub-codebooks is generated separately for each MBMS service, then UE cconcatenates these sub-codebooks based on the G-RNTI of each MBS service in ascending order to form a type-2 codebook for the multiple MBS services.

      - If the UE receives unicast and multicast, the sub-codebooks for unicast and multicast are generated separately, and then the multicast sub-codebook is concatenated after the unicast sub-codebook.

(vivo) Proposal 7:

  - For ACK/NACK based HARQ-ACK feedback, for type 2 HARQ-ACK codebook for RRC_CONNECTED Ues receiving multicast.

    - If UE is configured with multiple g-RNTIs

      - If PTM transmission scheme 1 is used for group-common PDSCH, separate DAI counting for different g-RNTIs is used.

        - Concatenating more than one Type-2 HARQ-ACK codebook for multicast is supported.

    - If PTM transmission scheme 2 is used for group-common PDSCH, the existing mechanism can be reused to construct a HARQ-ACK codebook for different PDSCHs.

(CATT) Proposal 12:

  - Support concatenating no more than 3 Type-2 HARQ-ACK codebooks for multicast scheduling.

(CATT) Proposal 13:

  - For concatenating more than one Type-2 HARQ-ACK codebooks for multicast, the order between sub-codebooks can be based on the increasing order of G-RNTI.

(Nokia) Proposal 23:

  - When Type-2 HARQ-ACK codebook is used the PHY identification of PDSCH HARQ-ACK to MBS sub-codebook mapping is the group-common RNTI value.

(Nokia) Proposal 24:

  - When Type-2 HARQ-ACK codebook is used the UE maps the PDSCH HARQ-ACK of unicast services scrambled with a UE-specific RNTI to the unicast sub-codebook.

(Nokia) Proposal 25:

  - When Type-2 HARQ-ACK codebook is used, the order of concatenation of the sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, follows the increasing order of the RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook.

(CMCC) Proposal 11:

  - The concatenation order of Type-2 HARQ-ACK codebook for multicast can be in ascending order of G-RNTI values for sub-codebooks for different multicast services.

(OPPO) Proposal 4:

  - At most one Type 2 MBS HARQ-ACK sub-codebook is concatenated with type 2 unicast HARQ-ACK sub-codebook.

(Lenovo) Proposal 8:

  - For multicast PDSCHs, don’t support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

(Apple) Proposal 3:

  - Whether to support more than one Type-2 codebook for MBS  is waiting for the outcome of the CFR design.

(Ericsson) Proposal 11:

  - For type-2 HARQ codebook, support more than one Type-2 HARQ-ACK sub-codebook for multicast. When there are more than one type-2 codebooks for multicast, the codebook associated with a smaller G-RNTI precedes the one associated with a larger G-RNTI

### Round-1

FL’s Comments

Since DAIs for unicast and multicast cannot be jointly counted, the Type-2 codebook is constructed to concatenate the sub-codebooks for unicast preceding the sub-codebook for multicast, which has been agreed in previous meeting. The proposals submitted for this issue in this meeting mainly talk about more than one multicast services. Whether UE supports more than one has been sent to RAN2 by LS. Without the LS reply, it is better not to spend time on it in this meeting, esp. when some companies support to consider more than one but others not (OPPO and Lenovo) in constructing Type-2 codebook.

FL will not suggest a proposal but wait and see more discussion later in future meetings.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| CATT | OK. |
| CMCC | OK |
| Nokia, NSB | Considering the time limitations and support from the companies on the same view, we believe that we can have agreements as such: Proposal i: When Type-2 HARQ-ACK codebook is used, the DAI counter is separate for each multicast service, if multiple multicast services are supported, since the Ues in the PTM group may be interested in different services. Separate DAI counters naturally lead to construction of separate sub-codebooks. - PHY identification of PDSCH HARQ-ACK to MBS sub-codebook mapping is the group-common RNTI value. Proposal ii: The order of concatenation of the sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, if multiple multicast services are supported, follows the increasing order of the RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook. |
| Apple | OK |
| Samsung | OK |
| NTT DOCOMO | We agree to wait for the LS reply from RAN2. |
| Vivo | OK |

## Enh Type-2 / Type 3 HARQ codebook

Submitted Proposals

(Nokia) Proposal 15:

  - For ACK / NACK based feedback for RRC_CONNECTED Ues receiving multicast, not support

    - Enhanced Type-2 HARQ-ACK codebook.

    - Type-3 HARQ-ACK codebook

(Qualcomm) Proposal 6:

  - For RRC_CONNECTED Ues, also support Type 3 HARQ-ACK codebook for multicast ACK/NACK based HARQ-ACK feedback.

(Ericsson) Proposal 15:

  - Enhanced Type 2 or Type 3 HARQ-ACK codebooks are not supported for PTM traffic feedback

### Round-1

FL’s Comments

Only three submissions for discussion of this issue and diverse views from the submitted proposals.

FL will not suggest a proposal but will see possible more discussions later in future meetings.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Ok. |
| CATT | OK. |
| Nokia, NSB | OK |
| Apple | OK |
| Samsung | OK |
| NTT DOCOMO | We prefer to preclude enhanced type-2 and type 3. They are required for unlicensed band, which is not included in this WI scope. |
| MTK | OK |
| vivo | Ok. Clarification for enhanced type-2 and type 3, they are not required for unlicensed band. They can also be applied to licensed band. This has been agreed in the UE feature part. And the corresponding TP is discussed in email thread of [105-e-NR-NRU-03] in issue of HARQ1. |

## UCI multiplexing/prioritizing

Submitted Proposals

“last unicast DCI”

(vivo) Proposal 8:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast.

(Nokia) Proposal 22:

  - Alt. 1 is supported, since a later MBS scheduling at the time of unicast scheduling is not expected with high probability.

(CMCC) Proposal 8:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission should be based on the PRI indicated in the last DCI for unicast.

(Samsung) Proposal 1:

  - When a UE multiplexes HARQ-ACK associated with unicast DCIs and HARQ-ACK associated with multicast DCIs in a PUCCH, the UE determines a PUCCH resource from the PRI value in the last unicast DCI.

(DOCOMO) Proposal 7:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PUCCH resource is determined by the last unicast DCI.

    - The order is first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and is then indexed in an ascending order across PDCCH monitoring occasion indexes.

(Ericsson) Proposal 18:

    - The PRI from last DCI for unicast is used to indicate the PUCCH resource for joint transmission of HARQ-ACK from unicast and multicast.

“last across DCI”

(Spreadtrum) Proposal 3:

  - For ACK/NACK based HARQ-ACK feedback, determining the PUCCH resources for transmission is based on the PRI indicated in the last DCI across unicast and multicast.

(CATT) Proposal 18:

  - For multiplexing the HARQ-ACKs of multicast and unicast, the last DCI refers to the last DCI across unicast and multicast.

(Qualcomm) Proposal 3:

  - For multiplexing of ACK/NACK feedback for unicast and multicast, determining the PUCCH resources for transmission is based on the PRI indicated in the last DCI across unicast and multicast.

(Apple) Proposal 2:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PRI in last DCI across the unicast and multicast is used for PUCCH resource indication.

(LGE) Proposal 17:

  - For multiplexing HARQ-ACKs for dynamically scheduled multicast and dynamically scheduled unicast or for dynamically scheduled multicast PDSCHs only, UE determines a PUCCH resource based on the PRI of the last DCI scheduling either multicast or unicast.

- “HARQ with other UCI”

(CATT) Proposal 17:

  - The multiplexing / prioritization rules in R16 for unicast can be used on prioritizing/multiplexing of the multicast and other UCI.

(Nokia) Proposal 20:

  - Rel-15 / 16 handling rules are followed for multiplexing / prioritization of HARQ-ACK with other UL transmissions.

(CMCC) Proposal 13:

  - The multiplexing/prioritization rule between the HARQ-ACK for multicast and other UCIs for unicast can reuse Rel-16 multiplexing/prioritization rule between the HARQ-ACK for unicast and other UCIs for unicast.

(Intel) Proposal 5:

  - When a PUCCH resource carrying HARQ ACK for MBS overlaps in time domain with a PUCCH resource carrying other UCI types, a default priority order for PUCCH dropping can be defined in specification where HARQ-ACK feedback of unicast transmission > HARQ-ACK feedback of MBS transmission > SR > CSI report.

(Samsung) Proposal 3:

  - Multiplexing multicast HARQ-ACK (of priority 0) and CSI report in a PUCCH resource is enabled by RRC configuration; if not provided, the UE drops the CSI report.

(Samsung) Proposal 4:

  - Multiplexing multicast HARQ-ACK and SR (of same priority) in a PUCCH resource is default UE behavior.

- “multiplexing/multiplexing for overlap or not”

(Nokia) Proposal 18:

  - Multiplexing of HARQ-ACK feedback of same priority and prioritizing of HARQ-ACK feedback of different priorities are supported in case PUCCH transmissions are in the same (sub-)slot, not only when the corresponding PUCCH resources physically overlap.

(DOCOMO) Proposal 8:

    - When multicast transmission is performed from a single TRP and HARQ-ACK feedback for unicast and multicast is indicated as transmitted at the same slot/sub-slot, the UE multiplexes HARQ-ACK with the same priority regardless whether PUCCH resources are overlapped or not.

(Ericsson) Proposal 16:

  - When multicast and unicast traffic has same priority and their PUCCH resource for HARQ feedback is overlap in time, the HARQ feedback from multicast and unicast traffic can be multiplexed. The PUCCH resource set is determined according to the total number of HARQ feedback bits of both multicast and unicast traffic, and the specific PUCCH resource is determined by the PRI in DCI associated with unicast traffic. If the capacity of PUCCH is not enough to transmit all bits from unicast and multicast, then HARQ feedback bits has higher priority compared to CSI feedback.

(CATT) Proposal 16:

  - If a UE does not support multiplexing / prioritization capability when the unicast PUCCH resource overlaps with the multicast PUCCH resource, gNB shall void scheduling PUCCHs overlapping in time domain.

Others

(Huawei) Proposal 13:

  - For the cases of HARQ-ACK feedback is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

    - support multiplexing for the same priority and prioritizing for different priorities, when the corresponding PUCCH resources overlap in time in a sub-slot.

    - UE is not expected to be configured with the following cases:

    - slot-based PUCCH for multicast and sub-slot based PUCCH for unicast

    - slot-based PUCCH for unicast and sub-slot based PUCCH for multicast

    - different length configuration for sub-slot based PUCCH for multicast and unicast

(Nokia) Proposal 19:

  - UE capabilities on multiplexing / prioritization are discussed at the end of the WI by RAN1 group, given that there may be multiple dependencies with as yet undefined features.

(CATT) Proposal 16:

  - If a UE does not support multiplexing / prioritization capability when the unicast PUCCH resource overlaps with the multicast PUCCH resource, gNB shall void scheduling PUCCHs overlapping in time domain.

### Round-1(closed)

“last DCI”

FL’s Comments

Regarding the issue that last DCI is unicast (Alt1) or across unicast and multicast (Alt2), 11 companies submitted proposals with 6 supporting Alt1 and 5 supporting Alt2. The arguments from both sides from submitted contributions are summarized as follows:

From proponents of Alt1, the issues for Alt2 include:

  - No closed-loop power control for unicast PUCCH (CMCC).

  - one-to-one mapping between PUCCH resources for any resource set of multicast HARQ-ACK and PUCCH resources for any resource set of unicast (and multicast) HARQ-ACK, across all PUCCH resource sets (CMCC, Samsung).

  - PUCCH resource allocation flexibility of Alt.1 is higher than Alt.2 (vivo, Nokia, DOCOMO).

  - PUCCH configuration for multicast may only include configuration for PUCCH formats 0/1 (Ericsson).

From proponents of Alt2, the issues for Alt1 include:

  - Less spec impact (Spreadtrum, CATT).

  - Alt.1 requires a unicast PDSCH always to be scheduled and the number of unicast DCIs shall be more or equal to four (CATT, Apple).

  - Alt.1 has codebook size ambiguity due to UE missing last DCI for multicast coming after unicast DCI (QC)

Based on this situation, for Round-1 discussion, feature lead suggest keeping two options open and encourage companies in the table to address the arguments from the other side.

FL’s Proposal:

#### Proposal 2.6.1-1

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, continue discussing the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

- This meeting aims to down select one alternative.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Alt 2 is supported. |
| LG | We assume that this proposal can be only applied to the case when both multicast and unicast are dynamically scheduled, because if multicast/unicast SPS PDSCH is scheduled, we do not schedule PDSCH based on DCI. Thus, we propose to change to: #### Proposal 2.6.1-1 For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast when neither multicast nor unicast are semi-statically scheduled, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, continue discussing the following alternatives: - Alt.1: the last DCI for unicast; - Alt.2: the last DCI across unicast and multicast; - This meeting aims to down select one alternative. FL’s response: Not sure about the intention of this change. This issue is specifically for ACK/NACK based feedback and feedback for SPS is separately discussed section 5. |
| CATT | Alt 2 is supported. It has been agreed in #104-e meeting that it is up to network to configure orthogonal PUCCH resources among Ues within the same group for ACK/NACK based HARQ-ACK feedback for multicast. When the last DCI is multicast, gNB will guarantee the PUCCH resources are orthogonal among different Ues in a same group. If it is difficult for gNB to assign the orthogonal PUCCH resource, based on gNB’s implementation, gNB can make unicast DCI as last DCI. We did not see a strong need to support t Alt.1. |
| ZTE | We support Alt.1 To address the comment raised by the opponents. - Regarding the spec impact, actually we think Alt.1 has less spec impact as it is essentially the same as the current mechanism. - Regarding that Alt.1 requires a unicast PDSCH, as we can see in the main bullet of this proposal “For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast”, the scenario is there are both multicast and unicast, there is anyway unicast PDSCH to be scheduled in this case. “Missing the last DCI” is a generic issue for all solutions here. |
| CMCC | Support alt 1 |
| Nokia, NSB | We prefer Alt.1 We agree that less spec impact is there when Alt 2, however, Alt 2 is not the best technical solution, and Alt 1 requires only small amount of spec impact. Naturally, when we are discussing multiplexing feedback for multicast and unicast, a unicast PDSCH should have been scheduled already, therefore, we do not understand the intention of concern 2 (also, what is the reason behind more than 3-4 unicast DCIs to be present?). We believe that size ambiguity will anyway be there and should be separately treated, regardless which resource is selected based on which DCI. |
| Spreadtrum | Support alt 2 |
| vivo | Alt 1 is supported. The payload for different UE can be various, it is difficult to indicate a suitable PUCCH resource for all Ues in the MBS group by PRI in group-common DCI. |
| Apple | Alt 2 is supported. |
| Samsung | Support Alt. 1. Alt. 2 is not feasible in practice and will create large complications in existing network deployments (basically, Alt.2 disables PRI and operates unicast without PRI). |
| OPPO | Alt 2 should be supported. From our perspective, the main issue of Alt 1 is that if the last unicast is scheduled earlier than multiple multicast, as gNB may not be able to predict the upcoming multicast when it transmit the last unicast, consequently the PRI indicated in the last unicast DCI may not be applicable. |
| Qualcomm | We prefer Alt2, which is aligned with the legacy scheme. Similar understanding as CATT. |
| Convida | Support Alt 1. |
| Ericsson | Support the proposal |
| NTT DOCOMO | We are generally fine with the proposal. We would like to clarify that the existing DCI indexing mechanism specified in 38.213 is reused, i.e., the order is first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and is then indexed in an ascending order across PDCCH monitoring occasion indexes. |
| MTK | Slightly prefer Alt 2. Similar view as CATT/QC. |
| CATT2 | @ Nokia Regarding the comment ‘what is the reason behind more than 3-4 unicast DCIs to be present?’ from Nokia, if the last DCI has unicast, gNB should guarantee that at least one unicast DCI can be detected. Thus, to avoid the case where three consecutive DCI are missing, the number of unicast DCIs shall be more or equal to four. This is too restricting for network. Regarding the concern on ‘PUCCH configuration for multicast may only include configuration for PUCCH formats 0/1’, we do not understand. Can the proponents of Alt1 explain it? Per our understanding, this proposal is related about ACK/NACK-based feedback, which can include not only PUCCH formats 0/1 but also other PUCCH formats. |

- “HARQ with other UCI”

FL’s Comments

5 companies discussed the multiplexing/prioritizing issues for HARQ-ACK for multicast and other UCI for unicast and three companies propose to reuse the Rel-16 rule defined for unicast. Maybe the rule defined for unicast should be the starting point for the support and identify whether there is issue for this support which can be further discussed if any.

FL’s Proposal:

#### Proposal 2.6.1-2

- For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and other UCIs for unicast can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and other UCIs for unicast.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Ok. |
| LG | We are fine with this proposal. |
| ZTE | Support the above proposal. |
| CMCC | OK |
| Nokia, NSB | We support the proposal, but for ACK/NACK feedback. NACK-only needs special treatment. Thus, we propose: The multiplexing/prioritizing rule between the HARQ-ACK for multicast and other UCIs for unicast, at least for ACK/NACK based HARQ-ACK feedback, can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and other UCIs for unicast. |
| Vivo | If it is for ACK/NACK based feedback for multicast. We think the proposal is ok. For NACK-only feedback, we think separate discussion is needed. FL’s response: this issue is ACK/NACK based specific, I can clarify it in main bullet if it helps. |
| Apple | OK |
| Samsung | OK. Also fine to separately discuss the “NACK-only” case. |
| OPPO | OK |
| Qualcomm | Fine to focus on ACK/NACK feedback here. |
| Ericsson | Support the proposal |
| NTT DOCOMO | We are fine with the proposal. We don’t see any motivation to introduce new rules. |
| MTK | We are fine with the proposal. |
| Qualcomm2 | The ‘other UCIs for unicast’ does not include the ‘UCIs for multicast’? |
| CMCC2 | As Qualcomm’s 2rd round comment, at least for now we don’t agree any “UCIs for multicast” |
| Qualcomm3 | Can we say ‘other UCIs’? |

- “multiplexing/multiplexing for overlap or not”

FL’s Comments

We have agreed HARQ-ACK bits for unicast and multicast with the same priority is to be multiplexed in the same slot when the corresponding PUCCH resources overlap. DOCOMO and Nokia further proposed multiplexing/prioritizing the HARQ-ACK should not be only when the PUCCH resources are overlapped. In other words, when the PUCCH resources are not overlapped in the same slot, the HARQ-ACK feedback for unicast and multicast with the same priority should be multiplexed or prioritized.

This issue is related with the issue discussed in section 2.2 whether UE supports two non-overlapping PUCCH in the same slot. If UE does not support two non-overlapping PUCCH in the same slot, even though the PUCCH resources does not overlap, the HARQ-ACK for unicast and multicast will be multiplexed/prioritized.

FL suggests focusing on the discussion in section 2.2 first.

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | OK to discuss the aspects in 2.2. first. The above can be part of the discussion in 2.2. We support the proposal from DOCOMO and Nokia (there is no other way in our opinion). |

### Round-2(closed)

FL’s Comments

The arguments from both sides include spec impact and DCI missing issue. Both sides are not convinced by the other regarding the spec impact and the DCI missing issues.

The following are the survey to each alternative:

Supportive to Alt1: (6)

- ZTE, CMCC, Nokia, vivo, Samsung, Convida,

Supportive to Alt2: (7)

- Lenovo,  CATT, Spreadtrum, Apple, OPPO, Qualcomm, MTK,

Given this situation, FL is not sure this issue can converge in this meeting though it is always hoped. On the other hand, some arguments may need time for more consideration. Hence, FL suggests further discussion and companies are encouraged to address the arguments for more discussion. We have agreed to down-select between the two alternatives, so no need to agree anything more if down-selection is not going to happen in this meeting.

Any comments or suggestions from companies? Welcome to fill the following table.

Collect views:

| Company | Comments |
| --- | --- |
| LG | Considering FL’s response, we propose to change to: Proposal 2.6.1-1 For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast scheduled with corresponding PDCCH, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, continue discussing the following alternatives: - Alt.1: the last DCI for unicast; - Alt.2: the last DCI across unicast and multicast; - This meeting aims to down select one alternative. - FFS: SPS PDSCH for multicast/unicast |
| Samsung | OK to defer further discussion to the next meeting. A request to companies supporting Alt. 2 would be to describe how a network can operate without unicast PRI - e.g. how PUCCH resources are configured to different UEs and how different UEs reporting both multicast and unicast HARQ-ACK in corresponding PUCCHs in a same slot can determine the respective PUCCH resources in a first/second/third/… slot. |

FL’s Proposal:

#### Proposal 2.6.2-2

- For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and other UCIs can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and other UCIs.

Collect views:

| Company | Comments |
| --- | --- |
| Qualcomm | can we say ‘other UCIs for unicast’? |
| FL’s responses | @Qualcomm, I think the change should be fine, though we haven’t agreed to have other UCI for multicast and UCI may not differential Unicast or multicast eventually. |
| LG | Considering Rel-16 i.e. dropping for different priorities, we propose to change to: #### Proposal 2.6.2-2 - For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and other UCIs with a same priority can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and other UCIs with a same priority. |
| Samsung | OK with the updated proposal (based on the suggestion by Qualcomm) |
| CMCC | Ok |
| ZTE | OK |
| OPPO | OK |
| CATT | NOT OK with this proposal. Per our understating, the ‘other UCIs’ in ‘HARQ-ACK for multicast and other UCIs ’ and ‘the HARQ-ACK for unicast and other UCIs.’ has different meanings. In ‘the HARQ-ACK for unicast and other UCIs.’, the other UCIs belongs to unicast, but in ‘HARQ-ACK for multicast and other UCIs ’ the other UCIs may belong to unicast and multicast. Thus, we prefer FL’s previous version. |
| MTK | OK. |
| NTT DOCOMO | Support. We would like to change the wording for clarification. “other UCI” -> “SR/CSI” |
| Apple | OK |
| Nokia, NSB | Ok |
| vivo | ok |

### Round-3

FL’s Comments

@CATT, not sure if replacing “other UCI” by “SR/CSI” can address your concern. As I comment, so far they are equivalent because currently no SR/CSI for multicast. The current wording is generic enough.

#### Proposal 2.6.3-2

- For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Either “other UCIs” or “SR/CSI” is OK with us. |
| Nokia, NSB | Ok with both current and previous version. |
| Ericsson | OK with the proposal |

## PUCCH resource

Submitted Proposals

“Resource sets/resources”

(ZTE) Proposal 1:

  - Regarding configuration of PUCCH resource sets for multicast (reuse unicast mechanism):

- A maximum of 4 PUCCH resource sets are configured for multicast for the UE.

- NACK-only and ACK/NACK feedback share the same PUCCH resource set.

- UCI size limit for each PUCCH resource sets is configured, except for the PUCCH resource set 0, which only supports no more than 2 UCI bits.

(Nokia) Proposal 2:

  - A new mechanism is defined for the gNB to configure which PUCCH-config to use for each MBS service, or even for the feedback of each transport block, if needed. The mechanism is down-selected from one or more of the following options:

    - RRC configuration of the PUCCH-config ID to be used for all MBS services with a specific priority level (i.e., priority 0 or 1).

    - RRC configuration of the PUCCH-config ID to be used for a specific MBS service, where the configuration can be an addition to the G-RNTI configuration.

    - DCI indication of the PUCCH-config ID to be used for the HARQ-ACK feedback of the corresponding scheduled PDSCH TB.

(Nokia) Proposal 3:

  - Which PUCCH-config to utilize upon PTP based retransmission of an MBS service by the UE to send its HARQ-ACK feedback is down-selected from one or more of the following options:

    - A new mechanism similar to the one in previous proposal, which aims at initial transmissions, is defined.

    - The same PUCCH-config with the one used upon initial transmission of the same TB is used.

    - The UE uses the unicast PUCCH-config.

(Lenovo) Proposal 4:

  - For ACK/NACK based HARQ-ACK feedback, each UE in the group of UEs receiving multicast is configured with a specific PUCCH resource for support of PTP-based retransmission.

(Lenovo) Proposal 7:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

(LGE) Proposal 1:

  - PUCCH-config for multicast can be configured per CFR or per UE’s active UL BWP associated to the CFR.

(LGE) Proposal 5:

  - For PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

- “shared/separate resources”

(OPPO) Proposal 11:

  - For ACK/NACK based HARQ-ACK feedback for multicast, both shared and separate PUCCH resources among UEs within the group are supported.

(OPPO) Proposal 12:

  - For ACK/NACK based feedback and NACK only based feedback for multicast, RSRP based PUCCH resource configuration is supported.

(OPPO) Proposal 13:

  - Whether shared or separate PUCCH resources are used can be up to gNB configuration or scheduling.

(Lenovo) Proposal 3:

  - For ACK/NACK based HARQ-ACK feedback, it is up to network to configure a PUCCH resource shared by two or more UEs or a specific PUCCH resource for each UE in the group of UEs receiving multicast.

- others

(LGE) Proposal 3:

  - For PTM scheme 1, group common DCI indicates a single PUCCH resource indicator and a single PDSCH-to-HARQ_feedback timing indicator at least for ACK/NACK based HARQ-ACK.

(LGE) Proposal 4:

  - For UE specific PUCCH resource allocation for ACK/NACK based HARQ-ACK to group common DCI, different UEs in the group can be configured with different values of at least PUCCH-Resource and dl-DataToUL-ACK in UE dedicated PUCCH-config for multicast or for unicast (unless PUCCH-config for multicast is configured).

    - Different UEs can be allocated with different PUCCH resources by the same PUCCH resource indicator and the same PDSCH-to-HARQ_feedback timing indicator of the group common DCI.

(DOCOMO) Proposal 5:

  - A list of k1 values for DCI format 1_0 for multicast is configurable.

### Round-1(closed)

FL’s comment:

“Resource sets/resources”

7 companies submitted proposals and the issues discussed commonly by 4 companies are regarding the resource sets or resource including number of resources sets/resources for multicast, PUCCH-config for specific MBS services, and PUCCH-config for PTP retransmission.

The agreements achieved include UE can be optionally configured a separate PUCCH-Config for multicast and PUCCH-Config for unicast applies otherwise. However, whether a separate PUCCH-Config means different resource sets from that for unicast can be an issue to be discussed.

It is noted that different PUCCH Resource IDs are configured in different PUCCH-Config within the pucch-ConfigurationList if configured according to TS 38.331 for two simultaneously constructed HARQ-ACK codebooks for URLLC. From this sense per FL’s view, supporting multicast does not have to be configured with additional PUCCH resources sets in the separate PUCCH-Config for multicast but it is open for discussion.

FL’s Proposal:

#### Proposal 2.7.1-1

For the separate PUCCH-ConfigurationList configured for multicast for two simultaneously constructed HARQ-ACK codebooks with different priorities,

- different PUCCH Resource IDs are configured in the two PUCCH-Config within the PUCCH-ConfigurationList

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Ok. |
| LG | We wonder why we need to avoid configuring same PUCCH resource ID under same PUCCH resource set ID in separate PUCCH-config/PUCCH-ConfigurationList for multicast. FL’s response: the reason is that per my understanding network can indicate two non-overlapping PUCCHs in the same slot for the two separate codebooks with different priorities. |
| CATT | OK. |
| ZTE | We are not sure whether this issue has any RAN1 spec impact. It seems this is an implementation issue. Network can configure same or different PUCCH Resource IDs in the two PUCCH-Config. FL’s response: the reason is that per my understanding network can indicate two non-overlapping PUCCHs in the same slot for the two separate codebooks with different priorities. This is the same as URLLC. If separate codebooks, then two PUCCH-config, so that the PUCCH resource ID are different so as to have two non-overlapping PUCCHs in the same slot. |
| CMCC | OK |
| Nokia, NSB | We believe that such a proposal and agreement is not needed. In our view, different PUCCH resource sets and consequently different PUCCH resources are configured in a different PUCCH-config. Otherwise, the unicast PUCCH-config can be used, anyway. In addition, one PUCCH-config for multicast high priority can be sub-slot based, whereas the other can be slot-based. So, we expect that the PUCCH resource sets and consequently PUCCH resources are different. FL’s response: different resources sets can have the same resource ID. different PUCCH-config thematically can have the same resource set ID and resource ID. One is sub-sot based and the other is slot-base case is a bit complex and at this moment I am not sure whether this case is going to be supported. In our view, which multicast PUCCH-config to use (low priority / high priority) will be reflected by DCI priority indicator (which is also the same for URLLC). So, resource set IDs (which resource set to use is anyway selected by the UE based on size of the UCI) and PUCCH resource IDs can be configured the same/reused in different PUCCH-configs. Our understanding is that the priority indicator will guide UE to the correct PUCCH-config -> PUCCH resource. If we introduce the limitation implied by the bullet point, then considering PRI is only 3 bits, we are reducing the configurable PUCCH resources in a multicast PUCCH-config, which seems not the best practice. Because of those reasons, we do not think that an agreement as such is needed. |
| Spreadtrum | The motivation is not clear to us. We don’t understand why the PUCCH resource ID should be different. FL’s response: the reason is that per my understanding network can indicate two non-overlapping PUCCHs in the same slot for the two separate codebooks with different priorities. This is the same as URLLC. If separate codebooks, then two PUCCH-config, so that the PUCCH resource ID are different so as to have two non-overlapping PUCCHs in the same slot. |
| vivo | Support in principle. We think same mechanism as that in Rel-16 for URLLC and eMBB should be used. i.e., different PUCCH Resource IDs are configured in the two PUCCH-Config within the PUCCH-ConfigurationList FL’s response: yes. The intention here is reusing what has been supported for URLLC. |
| Apple | OK |
| Samsung | Share a same opinion with some previous comments – no apparent need for the sub-bullet. |
| OPPO | We do not understand the motivation either, why we need to restrict gNB configuration ? FL’s response: the reason is that per my understanding network can indicate two non-overlapping PUCCHs in the same slot for the two separate codebooks with different priorities. This is the same as URLLC. If separate codebooks, then two PUCCH-config, so that the PUCCH resource ID are different so as to have two non-overlapping PUCCHs in the same slot. |
| Qualcomm | It seems this proposal is not combing from the listed companies’ proposals. Not understand the intention of the proposal. FL’s response: the proposals under ““Resource sets/resources”” motivates me to suggest this proposal which follows what has been introduced for URLLC and this seems being what we have been doing when defining the HARQ mechanism for multicast, i.e., referring to URLLC or mTRP. |
| Ericsson | Support the proposal |
| NTT DOCOMO | We are fine with the proposal. |
| MTK | We are OK with the proposal since the proposal is aligned with legacy mechanism. |
| CMCC2 | Rel-15/16 spec only supports configure maximum 128 PUCCH resources per BWP. As we know, the maximum PUCCH resource number of the first PUCCH resource set is 16, and maximum PUCCH resource number of other PUCCH resource set is 8, that is maximum 40 PUCCH resources are configured for HARQ-ACK for one PUCCH-config. If separate PUCCH-ConfigurationList is configured for multicast, and both unicast and multicast support two priorities, the total PUCCH resource number will be 4*40=160 which extends current PUCCH resource configuration number. |
| FL’s comments | @CMCC, Resources IDs are different does not mean resource sets have to be different as well. Also, multicast does not seem to need other PUCCH resources sets addition to the first set for HARQ-ACK. Right? |
| Qualcomm2 | Do it mean we don’t allow same PUCCH resource ID can be shared by TDMed multicast traffic? |
| ZTE2 | We still don’t see the motivation here. Let’s ask a question for clarification. What would be the issue if network configures the same ID for in the two PUCCH-Config within the PUCCH-ConfigurationList? |
| CATT | We share the views with Spreadtrum/LG/ZTE, we don’t understand why the PUCCH resource ID should be different and what is the issues if the PUCCH resource ID is the same. |

FL’s Proposal:

#### Proposal 2.7.1-2

When separate PUCCH-config/PUCCH-ConfigurationList is configured for multicast ACK/NACK-based HARQ-ACK feedback from that for unicast at least for separate codebooks with different priorities, down-select one from the following alternatives:

- Alt1: PUCCH resources set IDs for multicast are different from that for unicast.

- Alt2: PUCCH resources IDs for multicast are different from that for unicast but the resource set ID can be the same.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| LG | We wonder why we need to avoid configuring same PUCCH resource ID under same PUCCH resource set ID in separate PUCCH-config/PUCCH-ConfigurationList for multicast and unicast. FL’s response: the reason is that per my understanding network can indicate two non-overlapping PUCCHs in the same slot for the two separate codebooks with different priorities. |
| ZTE | Similar comments here. We are not sure whether this issue has any RAN1 spec impact. It seems this is an implementation issue. Network can configure same or different PUCCH Resource (set) IDs in the two PUCCH-Config. FL’s response: the reason is that per my understanding network can indicate two non-overlapping PUCCHs in the same slot for the two separate codebooks with different priorities. This is the same as URLLC. If separate codebooks, then two PUCCH-config, so that the PUCCH resource ID are different so as to have two non-overlapping PUCCHs in the same slot. |
| CMCC | OK, prefer Alt 2 |
| Nokia, NSB | We believe that such a proposal and agreement is not needed. In our view, different PUCCH resource sets and consequently different PUCCH resources are configured in a different PUCCH-config. Otherwise, the unicast PUCCH-config can be used, anyway. In addition, one PUCCH-config for multicast high priority can be sub-slot based, whereas the other can be slot-based. So, we expect that the PUCCH resource sets and consequently PUCCH resources are different. FL’s response: different resources sets can have the same resource ID. different PUCCH-config thematically can have the same resource set ID and resource ID. One is sub-sot based and the other is slot-base case is a bit complex and at this moment I am not sure whether this case is going to be supported. Please see our answer for 2.7.1-1. |
| Spreadtrum | The motivation is not clear to us. We don’t understand why the PUCCH resource ID should be different. FL’s response: the reason is that per my understanding network can indicate two non-overlapping PUCCHs in the same slot for the two separate codebooks with different priorities. This is the same as URLLC. If separate codebooks, then two PUCCH-config, so that the PUCCH resource ID are different so as to have two non-overlapping PUCCHs in the same slot. |
| vivo | We think the PUCCH resources for unicast and multicast can be shared. FL’s response: I changed the main bullet to sync-up with what has been agreed for separate codebooks for multicast and unicast. |
| Apple | OK |
| Samsung | Same comment for 2.7.1-1 (no apparent need for the proposal). The configuration of PUCCH resources is a gNB implementation issue. FL’s response: the reason is that per my understanding network can indicate two non-overlapping PUCCHs in the same slot for the two separate codebooks with different priorities. This is the same as URLLC. If separate codebooks, then two PUCCH-config, so that the PUCCH resource ID are different so as to have two non-overlapping PUCCHs in the same slot. |
| OPPO | Similar comment as above. |
| Ericsson | Support the proposal, but an FFS on NACK-only is needed. |
| NTT DOCOMO | We are fine with the proposal. We prefer Alt 2. We prefer to use existing resource set IDs (i.e. 0-3) for multicast as well. |
| MTK | OK. |
| Qualcomm2 | Similar question here, do it mean we don’t allow same PUCCH resource ID can be shared by TDMed unicast and multicast traffic? |
| ZTE2 | We still don’t see the motivation here. Let’s ask a question for clarification. What would be the issue if network configures the same ID for the PUCCH resource or PUCCH resource set? |
| CATT | We have same concern with the P 2.7.1-2 as discussed above. |

### Round-2

- As commented by others in Round-1, the motivation to configure different resoruces ID is unclear though the same is used for URLLC is understood.

- After further checking the URLLC discussion in history, the information I obtained is that it is determined by RAN2. The reason is that RAN2 did not want to change the MAC-CE for PUCCH spatial relation activation/deactivation which uses PUCCH Resource ID, although URLLC introduced low and high priorities for HARQ-ACK codebook on PUCCHs. In other words, if network just wants to, for example, activate PUCCH spatial relation for high priority or deactivate the PUCCH spatial relation for low priority, MAC-CE can be unchanged by indicating the corresponding PUCCH resources ID. At least, the motivation is applicable to the following proposal for multicast.

- Companies are encouraged to check internally also and back to check this proposal.

#### Proposal 2.7.2-1

For the separate PUCCH-ConfigurationList configured for multicast for two simultaneously constructed HARQ-ACK codebooks with different priorities,

- different PUCCH Resource IDs are configured in the two PUCCH-Config within the PUCCH-ConfigurationList

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK |
| Nokia, NSB | Ok. |
| Ericsson | OK |

# NACK-only based feedback specific

## PUCCH format

Submitted Proposals

“Support format 0 and format 1”

(Huawei) Proposal 6:

  - For the group-common PUCCH resource for NACK-only based HARQ-ACK feedback,

    - PUCCH format 0 is supported.

    - PUCCH format 1 can also be considered.

(vivo) Proposal 1:

  - Support PUCCH format 0 and PUCCH format 1 for NACK-only based HARQ-ACK feedback.

(CATT) proposal 5:

  - PUCCH format 0/1 can be configured by gNB for NACK-only feedback for MBS.

(Nokia) proposal 6:

  - For NACK-only based feedback for RRC_CONNECTED UEs receiving multicast, support PUCCH format 0 and PUCCH format 1.

(CMCC) proposal 14:

  - PUCCH format 0 and PUCCH format 1 can be used for NACK-only based HARQ-ACK feedback.

(OPPO) Proposal 7:

  - Both PUCCH format 0 and PUCCH format 1 should be supported for NACK-only based HARQ feedback mechanism.

(Lenovo) Proposal 1:

  - For NACK-only based HARQ-ACK feedback, both PUCCH format 0 and PUCCH format 1 can be configured for UEs receiving multicast.

(Intel) Proposal 6:

  - For NACK-only feedback, PUCCH formats 0 and 1 are supported.

(LGE) Proposal 8:

  - Support PUCCH format 0 and 1 for NACK based HARQ feedback.

(DOCOMO) Proposal 9:

  - Support PUCCH format 0 and 1 for NACK-only based HARQ-ACK feedback for multicast.

(Ericsson) Proposal 5:

  - PUCCH formats 0 and 1 can be used for semistatic codebook as a basis for NACK-only signaling

- “Support format 0”

(ZTE) Proposal 7:

  - On NACK-only feedback for MBS,

    - PUCCH resource configuration per MBS service is supported.

    - PUCCH format 0 is supported for NACK-only feedback.

    - PUCCH format 0 supports PUCCH repetition.

(Spreadtrum) Proposal 7:

  - At least PUCCH format 0 is supported for NACK-only based feedback.

- others

(OPPO) Proposal 8:

  - In NACK-only based HARQ feedback scheme, only 1 HARQ-ACK bit is transmitted in one PUCCH.

### Round-1 (closed)

FL’s Comments

13 companies submitted proposals discussing the PUCCH format for NACK-only and 11 companies propose to support both PUCCH format 0 and format 1. ZTE proposed to support PUCCH format 0 and repetition for format 0 if coverage is a concern. Spreadtrum proposed to at least support format 0.

FL’s Proposal:

#### Proposal  3.1.1

Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast.

- FFS: support of PUCCH repetition for PUCCH format 0.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| LG | We are fine with P3.1.1 |
| CATT | Agree. Both PUCCH format 0 and PUCCH format 1 should be supported for NACK-only feedback to meet the different scenarios. |
| ZTE | Ok with the above proposal. We propose to add an FFS for PUCCH repetition. Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast. FFS: support of PUCCH repetition for PUCCH format 0. |
| CMCC | Ok |
| Nokia, NSB | We support the proposal. |
| Spreadturm | Support. We are also fine with format 1. |
| vivo | Support |
| Apple | OK |
| Samsung | OK with the proposal. No apparent need for PUCCH format 0 but that can be left to gNB implementation. |
| OPPO | Support the proposal. |
| FUTUREWEI | OK |
| Qualcomm | Ok |
| Ericsson | Support the proposal |
| NTT DOCOMO | We are fine with the proposal. |
| MTK | We are fine with the proposal. |
| FUTUREWEI2 | Ok with updated proposed. |
| CMCC2 | From our understanding, support of slot-based PUCCH repetition for PUCCH format 0 for unicast is under the discussion of R17 URLLC/IIOT, we think the FFS can wait R17 URLLC/IIOT’s progress. |

## Overlapping PUCCH/PUSCH

Submitted Proposals

“Overlap with other PUCCH/PUSCH”

- “multiplexing”

(Huawei) Proposal 7:

  - When the number of HARQ-ACK bits for multicast or multicast and unicast after multiplexing are more than two bits, the NACK-only based automatically transforms to the ACK/NACK based feedback.

(ZTE) Proposal 9:

  - For the time domain overlapping between NACK-only PUCCH and unicast PUCCH/PUSCH, RAN1 study PUCCH multiplexing for the following scenes.

    - Unicast ACK/NACK/SR PUCCH F0/F1 overlaps with NACK-only PUCCH.

    - Unicast ACK/NACK/CSI PUCCH F2/F3/F4 overlaps with NACK-only PUCCH; unicast PUSCH and NACK-only PUCCH overlap.

(ZTE) Proposal 10:

  - If HARQ-ACK PUCCH for unicast and NACK-only PUCCH for multicast are determined in the same UL slot, then they should be multiplexed for the same priority (regardless of whether they overlap in the time domain).

    - The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

(Nokia) Proposal 21:

  - If for a UE a scheduled group-common PUCCH resource for PTM NACK-only feedback overlaps in time with a UE-specific PUCCH resource for other UCIs or a PUSCH transmission with the same priority, this UE should multiplex the PTM HARQ-ACK feedback with the other UCIs on the UE-specific PUCCH resource or with the PUSCH transmission, by treating NACK-only feedback as a 1-bit ACK / NACK feedback.

(Nokia) Proposal 31:

  - When group-common NACK-only HARQ-ACK feedback is used as the HARQ-ACK scheme, in case the UE has UE-specific HARQ-ACK resource for unicast services along with group-common NACK-only resource for PTM, the UE utilizes the UE-specific PUCCH resource by constructing separate HARQ-ACK sub-codebooks, as if ACK / NACK based approach is being used for PTM.

(Samsung) Proposal 9:

  - A UE reports ACK/NACK when the UE multiplexes HARQ-ACK for multicast PDSCH receptions with other UCI in a PUCCH or in a PUSCH.

(Convida) Proposal 3:

  - When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUCCH resource allocated for a transmission (2nd transmission) using ACK/NACK HARQ feedback, the UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission.

(Convida) Proposal 4:

  - When a UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUCCH.

(Convida) Proposal 5:

  - When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUSCH transmission, the UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource.

(Convida) Proposal 6:

  - When a UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUSCH.

- “not multiplexing”

(CATT) Proposal 8:

  - UE expects that PUCCH resource for NACK-only based feedback is non-overlapping with other PUCCHs/PUSCH, and the multiplexing NACK-only information on the other PUCCH/PUSCH is not allowed.

(CMCC) Proposal 16:

  - If there is a collision between NACK-only feedback PUCCH for multicast and other UCIs, NACK-only feedback PUCCH can be dropped to solve the multiplexing issue.

“more than two bits for NACK-only”

(ZTE) Proposal 8:

  - RAN1 supports PUCCH multiplexing for the following scenarios for NACK only PUCCH.

    - One NACK-only PUCCH overlaps with another NACK-only PUCCH;

    - More than 2 NACK-only PUCCHs overlap with each other.

    - One NACK-only PUCCH overlaps with multiple non-overlapped NACK-only PUCCHs.

(Huawei) Proposal 7:

  - When the number of HARQ-ACK bits for multicast or multicast and unicast after multiplexing are more than two bits, the NACK-only based automatically transforms to the ACK/NACK based feedback.

(Nokia) Proposal 9:

  - For a proper operation of NACK-only HARQ-ACK feedback for MBS, a UE can be optionally configured to support more than one PUCCH carrying HARQ-ACK feedback per slot with a method that is to be down-selected from the list below:

    - Option 1:  Allowing multiple HARQ-ACK feedback carrying PUCCHs per slot.

    - Option 2:  Based on Rel-16 sub-slot PUCCH mechanism.

    - Option 3:  DL-heavy slot configuration.

(CMCC) Proposal 15:

  - Considering the following two group-common PDSCH HARQ-ACK information associations with PUCCH for NACK-only based HARQ-ACK feedback:

    - NACK inforamtion of different group-common PDSCHs are feedbacked in different PUCCHs respectively.

    - Different HARQ-ACK information of a number of group-common PDSCHs are feedbacked in different PUCCHs respectively.

(Samsung) Proposal 7:

  - A UE is provided, by UE-specific RRC, PUCCH resources to select from according to PDSCH/TB receptions for which the UE reports NACKs.

(DOCOMO) Proposal 10:

  - For NACK-only based HARQ-ACK feedback for multicast, support multiplexing multiple HARQ-ACK bits in a PUCCH resource.

(DOCOMO) Proposal 11:

  - For multiple HARQ-ACK bits of NACK-only based HARQ-ACK feedback, associate a bit sequence of HARQ-ACK codebook to a PUCCH resource.

(Ericsson) Proposal 6:

  - Denoting the number of bits in the NACK-only codebook by N, downselect from the following variants to create up to N NACK-only signals in the same uplink slot:

    - a) Use N PUCCH resources in the same slot, where each PUCCH resource represents one bit in the NACK-only codebook of N bits and the UE needs to transmit multiple NACK signals, one on each PUCCH resource corresponding to a bit in the codebook for which the UE has to signal a NACK.

    - b) Use multiple PUCCH resources in the same slot, M=2^N-1 PUCCH resources for N is the total number of PDSCHs for which the UE needs to provide feedback, each UE transmits on one of the resources according to the subset of PDSCHs for which the UE needs to signal NACK.

      - FFS: Use the PUCCH format 0 or format 1 phase rotations and for format 1 the DFT spreading as dimension in addition to OFDM-symbol and PRB.

      - FFS: Associate each NACK signal with a subset of bits in the NACK-only codebook, where multiple UEs use the same PUCCH resource for the NACK-only signal relating to the same subset and the subset size may reduce to 1. A UE transmits the NACK signal if at least one bit of the associated subset of bits in the NACK-only codebook is cleared, i.e. indicates a PDSCH decoding failure. The gNB accordingly retransmits the transport blocks of all HARQ processes of the subset. This enhancement reduces the number of required PUCCH resources on the expense of unnecessary retransmissions.

- Others

(DOCOMO) Proposal 12:

  - RAN1 should prioritize specifying ACK/NACK based feedback procedure and NACK-only based feedback procedure for when multiplexing/prioritizing is not required.

### Round-1 (closed)

“Overlap with other PUCCH/PUSCH”

FL’s Comments

9 companies submitted proposals regarding the issues when NACK-only based feedback collides with HARQ-ACK feedback for unicast and other UCI and when more than one bits are needed to be feedback for NACK-only for the same PUCCH occasion.

5 companies support to multiplex NACK-only feedback with HARQ-ACK feedback for unicast and other UCI but 2 companies propose not to support the multiplexing.

When discussing multiplexing/prioritizing NACK-only feedback with HARQ-ACK feedback, we imply priority for NACK-only based feedback as well and this is the motivation for proposal 3.2.1-0 given the similar thing has been agreed for ACK/NACK based HARQ-feedback.

FL’s Proposal:

#### Proposal  3.2.1-0 (newly added)

For a separate PUCCH-ConfigurationList for multicast that is configured for NACK-only based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively.

- FFS other configurations

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| Nokia, NSB | We support the proposal. |
| Qualcomm2 | It is not clear ‘separate PUCCH-ConfigurationList for multicast’ means - Opt1: separate from unicast one but shared with multicast ACK/NACK-based one - Opt2: separate from unicast one and separate from multicast ACK/NACK-based one |
| OPPO | OK |
| NTT DOCOMO | Support |
| CMCC | Similar concern as Qualcomm, in addition, we think this proposal is also related to Proposal 4.2.1. For Alt 1 in proposal 4.2.1 the PUCCH for A/N and N-only is separated configured by RRC, does it corresponds to Opt 2 in Qualcomm’s comment. Similarly Alt 2 in proposal 4.2.1 allows sharing PUCCH between A/N and N-only which corresponds to Opt 1. |
| CATT | Similar concern as Qualcomm. |
| vivo | Similar concern as Qualcomm. In addition, for multicast, for ACK/NACK feedback and NACK-only feedback, we are fine to configure PUCCH resources separately, we are just wondering whether a separate PUCCH-config is needed for NACK-only. One alternative is to configure PUCCH resources for ACK/NACK feedback and NACK-only feedback in the same PUCCH-config. The PUCCH resources configured in PUCCH resource set(s) are for ACK/NACK, and another parameter, such as PUCCHResourceForNACKonly, can be used to configure one or multiple resources for NACK only. |
| Qualcomm3 | Follow up the discussion, we don’t have a strong preference for Opt1 and Opt2. But, if PUCCH-Config is jointly configured for A/N and N-only (Opt1), RAN1 needs to discuss how to configure the parameters of PUCCH-PowerControl, which is currently shared by the PUCCH resources in a PUCCH-Config. We think A/N and N-only should have separate PC chains with different PUCCH-PowerControl. |
| Nokia, NSB | We think that Opt2 from Qualcomm’s options is needed. This would in turn solve the problem Qualcomm mentioned in their last response. Also, it would introduce more flexibility to gNB on how to configure the actual PUCCH resource sets and resources. Therefore, we suggest: For a separate PUCCH-ConfigurationList for multicast that is configured for NACK-only based HARQ-ACK feedback that is separate from the PUCCH-ConfigurationList for ACK/NACK based HARQ-ACK feedback, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively. - FFS other configurations |

#### Proposal  3.2.1-1

When PUCCH resource for NACK-only feedback for multicast collides with PUCCH resources for HARQ-ACK feedback for unicast or other UCI, down-select from the following alternatives:

- Alt1: support multiplexing for the same priority and prioritizing for different priorities.

  - FFS details about multiplexing/prioritizing

- Alt2: NACK-only feedback for multicast is always dropped.

- Alt3: UE expects that PUCCH resource for NACK-only based feedback is non-overlapping with other PUCCHs/PUSCH.

- Alt 4: NACK-only feedback for multicast is dropped for the same priority and prioritizing is used for different priorities.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| LG | We are fine with P3.2.1-1 |
| CATT | Support Alt3. Regarding Alt1, before applying the current multiplexing/prioritization rules, the NACK-only feedbacks have to change to ACK/NACK based HARQ-ACK feedbacks, which will lead significant specification efforts. Regarding Alt2, it is not fair for multicast services if the NACK-only feedback for multicast is always dropped. Thus, neither Alt.1 nor Alt.2 can work well when the NACK-only PUCCH overlaps with PUCCH resources for HARQ-ACK feedback for unicast or other UCI. Thus, we recommend UE expects that PUCCH resource for NACK-only based feedback is non-overlapping with other PUCCHs or PUSCH. FL’s response: companies may worry about the scheduling flexibility for Alt3. |
| ZTE | Ok to list the above alternatives. |
| CMCC | Ok to list all the potential methods. For Alt 2, since the NACK-only based PUCCH are shared among UEs, even the NACK-only based PUCCH is dropped by one UE, another UE which does not decode the PDSCH correctly can also transmits the PUCCH. From gNB’s perspective, it only needs to know whether there is UE feedback NACK but don’t care which UE feedbacks NACK. FL’s response: I suppose you support Alt2? The concern may be NACK-only with high priority but is dropped per Alt2 hard decision. |
| Nokia, NSB | We support Alt 1. In addition, priority for NACK-only (such as a PUCCH-ConfigurationList including high priority and low priority PUCCH-config) needs to be defined for multiplexing/prioritization rules. We propose: For a separate PUCCH-ConfigurationList for multicast that is configured for NACK-only based HARQ-ACK feedback, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively. - FFS other configurations FL’s response: This could be a new proposal to be added as 3.2.1-0. |
| Spreadtrum | fine |
| vivo | We want to add another alternative for discussion. Alt 4: NACK-only feedback for multicast is dropped for the same priority and prioritizing is used for different priorities. |
| Apple | Ok with this proposal, slight prefer Alt2. |
| Samsung | OK with the proposal as it is a good step to capture the options. We support Alt. 1. |
| OPPO | Support the proposal |
| Qualcomm | For Alt2, why ‘NACK-only’ is always dropped? Does it mean the NACK-only will be dropped, even if it collides with CSI in UCI? It is not clear what is the meaning of ‘prioritizing for different priorities’ in Alt1, since we haven’t discussed the priority of ‘NACK-only’ yet. FL’s response: as Nokia commented as well, there could be a new proposal to be added as 3.2.1-0. |
| Convida | Fine with the proposal. We support Alt 1. |
| Ericsson | Support the proposal. we prefer alt1. |
| NTT DOCOMO | We are fine with the proposal. We prefer to prioritize specifying non-overlapping cases because which method is best will depend on the procedures for the non-overlapping cases. |
| MTK | We are OK with the proposal. We prefer Alt 3. |
| Qualcomm2 | Before listing the alternatives, it seems RAN1 need to discuss the priority between NACK-only for multicast and ACK/NACK for unicast/multicast if using same priority index, first. |

“more than two bits for NACK-only”

FL’s Comments

Regarding the issue when more than one bits are needed to be feedback for NACK-only for the same PUCCH occasion, 7 companies discussed many different options. It could be good progress if all options are clearly summarized in this meeting and down-selection could be further discussed later if not possible in this meeting.

FL’s Proposal:

#### Proposal  3.2.1-2

When more than one bits for NACK-only are to be transmitted for the same PUCCH slot, down-select from the following alternatives:

- Alt1: Support multiplexing the HARQ-ACK bits.

- Alt2: Support UE transmitting more than one PUCCHs in the same PUCCH slot.

- Alt3: NACK-only bundling

- Alt4: support sub-slot based PUCCH for NACK-only based feedback.

- other alternatives are not precluded.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| LG | We are not sure how Alt-1 works. UE could also drop some of them. FL’s response: per my understanding of the proposals, when to be multiplexed in Alt1, NACK-only will transform to ACK/NACK based to generate codebook. |
| ZTE | The PUCCH occasion here is not very clear. It seems the issue occurs when more than one bits for NACK-only are to be transmitted in the same slot. Thus, we update the proposal as below. Updated proposal from ZTE: When more than one bits for NACK-only are to be transmitted in the same slotfor the same PUCCH occasion, down-select from the following alternatives: - Alt1: Support multiplexing the HARQ-ACK bits. - Alt2: Support UE transmitting more than one PUCCHs in the same PUCCH occasion slot. - Alt3: Don’t support this case. - Note: the PUCCH occasion means at least slot-based PUCCH. FFS sub-slot based PUCCH. FL’s response: Fine. I wonder what Alt3 actually means, UE is not expected more than one bits for NACK-only are to be transmitted for the same PUCCH slot? |
| CMCC | Ok to list all the potential methods. For Alt 1, different PUCCH resource/cyclic shift/.. can reflect different HARQ-ACK bits without additional UE capability to support transmitting more than one PUCCHs in alt 2. |
| Nokia, NSB | Alt 1 is not possible, since the NACK-only resource will be group-common. Either Alt 2 with some enhancements or sub-slot based PUCCH with some enhancements are needed to increase the capacity of NACK-only feedback. We have provided a detailed analysis in our contribution regarding advantages / spec impact for both methods. We propose: For a proper operation of NACK-only HARQ-ACK feedback for MBS, a UE can be optionally configured to support more than one PUCCH carrying HARQ-ACK feedback per slot with a method that is to be down-selected from the list below: Option 1: Allowing multiple HARQ-ACK feedback carrying PUCCHs per slot. - FFS details. Option 2: Based on Rel-16 sub-slot PUCCH mechanism. - FFS details. |
| Spreadtrum | Fine |
| vivo | We think for the case with multiple PDSCHs to be feedback in one PUCCH occasion, HARQ-ACK bundling can also be considered. For example, if any PDSCH is NACK, NACK PUCCH is transmitted. |
| Apple | We believe more than one or two bits case need to be studied further. We are not fully sure Alt1 or Alt2 can work well. So we propose sub-bullet: other alternatives are not precluded. |
| Samsung | Alt. 2 is not feasible (not supported in Rel-16 and will certainly not be supported in Rel-17 – not even PUSCH + PUCCH on the same cell will be supported). At a minimum, an LS to RAN4 would be needed for Alt. 2 but we think further consideration for Alt.2 is pointless. |
| Ericsson | We think Alt1 is unclear: what does “multiplex” mean for NACK-only? We agree that Alt2 is one variant to down-select from. We miss however our proposed method where one PUCCH resource represents multiple bits. We suggest thus to add this as a Alt3. FL’s response: per my understanding of the proposals, when to be multiplexed in Alt1, NACK-only will transform to ACK/NACK based to generate codebook. |
| NTT DOCOMO | We are fine with the proposal. We prefer Alt1. In Alt2, the number of symbols that can be used for one PUCCH will be limited to 14/N symbols for N HARQ-ACK bits, it is not desirable. |
| MTK | The motivation of the proposal is not clear for us. Why more than one bits for NACK-only will be transmitted for the same PUCCH occasion? FL’s response: The multiple PDSCHs’ NACK-only feedback are indicated to be transmitted in the same slot. |
| OPPO1 | OK, prefer Alt 3. |
| Qualcomm3 | suggest to say ‘When more than one bits for NACK-only are to be transmitted for the same PUCCH slot, if supported,’. For further study, we are fine to list up the alternatives. But Alt1 needs more clarification. Based on FL’s comment, Alt1: Support multiplexing the NACK-only bits in the way of ACK/NACK codebook, is it correct understanding? Is it dynamically or semi-statically? The dynamic way based on actual schedule GC-PDSCH may not work. Since now Alt4 is added, Alt2 means more than one slot-based PUCCH? |

### Round-2

#### Proposal  3.2.1-2

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

- Alt1: Support multiplexing the HARQ-ACK bits.

- Alt2: Support UE transmitting more than one PUCCHs in the same slot-based PUCCH slot.

- Alt3: NACK-only bundling

- Alt4: support sub-slot based PUCCH for NACK-only based feedback.

- other alternatives are not precluded.

Collect views:

| Company | Comments |
| --- | --- |
| Qualcomm | suggest to say ‘When more than one bits for NACK-only are to be transmitted for the same PUCCH slot, if supported,’. For further study, we are fine to list up the alternatives. But Alt1 needs more clarification. Based on FL’s comment, Alt1: Support multiplexing the HARQ-ACK bitsNACK-only bits in the way of ACK/NACK codebook, is it correct understanding? Is it dynamically or semi-statically? The dynamic way based on actual schedule GC-PDSCH may not work. Since now Alt4 is added, Alt2 means more than one slot-based PUCCH? |
| FL’s responses | @Qualcomm, The intention was to list all alternatives for further discussion to the generic question “ when more than one NACK-only based feedback are available for transmission in the same PUCCH slot”, I can modify the proposal like this so “if supported” is not needed to be added. Alt1 is listed in a generic way also, I didn’t explicitly describe all multiplexing is done so as to be more inclusive and include the way as you understood. It is dynamic as whether/when it happens depending on gNB scheduling. The feasibility issue is part of the further study/discussion of this alternative. Fine to “slot-based” added to the intended generic Alt2 is expected to include slot/sub-slot based. |
| LG | We are generally fine with this proposal. For Alt 1, we could add a sub-bullet as follows considering FL’s response: - Alt1: Support multiplexing the HARQ-ACK bits. - NACK-only will transform to ACK/NACK based to generate codebook |
| Samsung | OK with the proposal. We understand Alt.1 to be PUCCH resource selection as for PUCCH format 0 in NR, or as for PUCCH resource/channel selection in LTE. |
| Qualcomm | The reason to add ‘if supported’ is that whether a UE can support more than one bit NACK-only feedback or not. If not supported, the UE may regard it as an error case or keep 1bit and drop other bit(s) of the NACK-only feedback based on some rule. If we say “When more than one NACK-only based feedback are available”, it would be better to add other alternatives, such as - Alt5 UE regards it as an error case - Alt6 UE only sends 1bit and drops other bits of NACK-only based feedback Considering some of Alternatives are not clear for now, we prefer to say ‘further study’ instead of ‘down-select from the following alternatives’ |
| CMCC | Ok in principle. From our understanding, there may be several ways in Alt1, e.g., different PUCCH resources present different HARQ-ACK bits values as Samsung says, or different cyclic shifts in a single PUCCH resource present different HARQ-ACK bits values. |
| ZTE | Ok to list all the potential solutions for now. |
| Lenovo, Motorola Mobility | We are generally OK to list the possible alterantives. However, for sub-bullets, the listed alternatives are not clear to us. For Alt 1: What does “the HARQ-ACK bits” in the main bullet mean? Are they referring to only NACK-only based feedback? For Alt 2: Transmitting more than one sub-slot PUCCHs in same slot is against existing framework. For Alt 3: Since multiple NACK feedback are to be transmitted, how to perform bundling within NACKs? For Alt 4: Even if the sub-slot based PUCCH is allowed for MBS, when multiple sub-slot PUCCHs are overlapped, how can UE report the feedback? |
| OPPO | Fine to list all options, however, for the main bullet, we suggest changes as below. Otherwise, it seems UE would generate one NACK-only based feedback for each un-decoded TB, this has already precluded some options, e.g. UE generate at most one NACK-only based feedback. “When more than one bits for NACK-only are to be transmitted for When more than one NACK-only based feedback is used and more than one HARQ-ACK bits are available for transmission in the same PUCCH slot, down-select from the following alternatives:” |
| CATT | Ok in principle. May I ask what is the exactly difference between Alt 1 and Alt 3? Do they will result in different number of HARQ-ACK bits when more than one bits for NACK-only are to be transmitted in a PUCCH slot? |
| MTK | We are fine to list so many options. However, Alt 1 is not clear for us. Does the “HARQ-ACK bits” refer to only NACK bits? |
| NTT DOCOMO | We are generally fine with the proposal. |
| Nokia, NSB | Ok. |
| vivo | We are generally fine with the proposal. It would be better to clarify Alt1. For Alt 3, it can be implemented in the way as: if more than one bits for NACK-only based feedback are available for transmission are to be transmitted for in the same PUCCH slot, UE transmits 1-bit NACK on a PUCCH when any of the corresponding PDSCH is NACK. |
| Ericsson | The proposal from Ericsson that configuring 2^N-1 PUCCH resource and transmit one of them to represent the decoding results of N PDSCH is missing. We do not see our proposal can be mapped to any of the alternatives of P3.2.1-2. We have already made this comments in in round 1. From Samsung's contribution R1-2105337 in the description preceeding Proposal 7 we understand a similar scheme is intended, correct? We believe Alt 1 in the FL's understanding "when to be multiplexed in Alt1, NACK-only will transform to ACK/NACK based to generate codebook" is too constraining: It would constrain the dl-DataToUL-ACK to contain only 1 element and support only 1 PTM-PDSCH per slot to prevent UE has to fall-back to ACK/NACK based feedback. Maybe a clarification to our contribution: Where we talk about NACK-only "codebook" we just mean the "more than one NACK-only based feedback are available for transmission are to be transmitted for in the same PUCCH slot" as formulated in P3.2.1-2. We do not mean the NACK-only "codebook" is channel coded and mapped to a single PUCCH resource as in the ACK/NACK feedback mode. |

## PUCCH resource

Submitted Proposals

- “shared by UE”

(Lenovo) Proposal 2:

  - For NACK-only based HARQ-ACK feedback, a common PUCCH resource for transmitting the NACK is configured to the group of UEs receiving multicast.

(Apple) Proposal 1:

  - For NACK-only based feedback, PUCCH resources are shared among users in the group and are configured in a common frequency region.

(LGE) Proposal 2:

  - Group common PUCCH resources for NACK only based HARQ-ACK are configured within UL CFR configured within UE’s active UL BWP.

    - Group common PUCCH resources are only configured by PUCCH-config for multicast.

    - UE specific PUCCH resources are configured on UE’s active BWP, as currently specified.

(LGE) Proposal 6:

  - NACK only based HARQ-ACK is transmitted on group common PUCCH resource.

“Separate from ACK/NACK based”

(vivo) Proposal 14:

  - For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for NACK-only based HARQ-ACK feedback.

(Spreadtrum) Proposal 6:

  - For NACK-only based feedback for RRC_CONNECTED UEs receiving multicast, UE can be configured with a separate PUCCH-config for NACK-only based PUCCH resource.

(CATT) proposal 3:

  - For NACK-only based HARQ-ACK feedback, different from the PUCCH-config for unicast, a separate PUCCH-config for multicast can be configured.

(CATT) proposal 6:

  - Up to 32 PUCCH resources can be configured for the PUCCH resource set for NACK-only feedback in MBS.

(CATT) proposal 7:

  - The PUCCH resource set can be used by all MBS services which applying NACK-only based HARQ-ACK feedback.

(Nokia) proposal 7:

  - For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback PUCCH-config for HARQ-ACK feedback from per UE perspective is separate from PUCCH-config for HARQ-ACK feedback for unicast and from optional PUCCH-config for ACK / NACK feedback for MBS.

(Nokia) proposal 8:

  - For a separate PUCCH-ConfigurationList for multicast that is configured for NACK-only based HARQ-ACK feedback,

    - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively.

    - FFS other configurations

(Ericsson) Proposal 4:

  - PUCCH resources for NACK-only are configured separately from PUCCH resources for ACK/NACK feedback.

- “K1/PRI for NACK-only”

(CATT) proposal 4:

  - NACK-only based feedback framework is designed based on Rel-15 NR ACK/NACK-based feedback mechanism by considering k1, PRI and DCI CCE index to indicate the PUCCH resource.

(Ericsson) Proposal 7:

  - The PRI in the DCI is interpreted by UEs in NACK-only mode to select one of the PUCCH resource configurations for NACK-only.

- others

(LGE) Proposal 7:

  - Different group common PUCCH resources can be related to different DL RS e.g. in terms of symbol/slot and/or PRB and/or sequence for PUCCH.

### Round-1 (closed)

- “shared by UE”

FL’s Comments

For NACK-only based HARQ-ACK feedback, UEs with NACK are expected to transmit the HARQ-ACK feedback in the same resources for low PUCCH overhead. However, network should not be enforced to configure a single common PUCCH resources for all NACK-only based UE. PUCCH resources configuration is always up to network. Instead, from UE perspective, NACK-only based UEs can share the same PUCCH resources for the NACK-only based feedback.

FL’s Proposal:

#### Proposal  3.3.1-1 (for conclusion)

The same PUCCH resource can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Ok. |
| LG | We are fine with this proposal. |
| CATT | Agree. |
| ZTE | Ok with the proposal though it is an implementation issue. |
| CMCC | Ok in general, but it seems there is no spec impact. |
| Nokia, NSB | Ok. |
| Spreadtrum | fine |
| vivo | Ok in general, but it seems no spec impact. |
| Apple | OK |
| Samsung | Is there any specification impact on the UE procedure? If not, no need to agree to the proposal (no need for RAN1 to say what a NW can/should or cannot/should not do). |
| OPPO | OK |
| FUTUREWEI | OK but this is an implementation decision. |
| Qualcomm | Seem no spec impact for this proposal. |
| Ericsson | support |
| NTT DOCOMO | PUCCH resource configuration can be up to gNB implementation. We think no agreement is necessary. |
| MTK | Support. |
| FL’s comments | Agree with others, I assume no spec impact but it seems the base we design for NACK-only, i.e., sequence based but not codebook based because other UEs may use the same resources as well. From this sense, we need this but can be captured as conclusion. |

“Separate from ACK/NACK based”

FL’s Comments

Since the same PUCCH resource can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback, if network configures PUCCH resources that are shared by a group of UEs for NACK-only, then the PUCCH resources should be separate from that for ACK/NACK-based feedback for unicast or multicast.

FL’s Proposal:

#### Proposal  3.3.1-2 (for conclusion)

If PUCCH resource(s) is(are) configured to be shared by a group of UEs for NACK-only based feedback, from UE perspective, the PUCCH resource(s) should be separate from that for ACK/NACK based feedback for unicast or multicast if configured.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We support separate PUCCH resource for NACK-only based feedback for multicast and ACK/NACK based feedback for unicast. For PUCCH resource for NACK-only feedback and ACK/NACK feedback for multicast, we think the PUCCH resource for NACK can be shared. |
| LG | We are fine with this proposal. |
| ZTE | We don’t support this proposal. This is an implementation issue. Network can configure PUCCH for HARQ-ACK and PUCCH for NACK-only in the same PUCCH resource set or in different PUCCH resource sets. Even the same PUCCH resource can be used for both ACK/NACK-based and NACK-only codebook in a TDM manner. Thus, we don’t see the need to have such a limitation. FL’s response: I agree it is up to configuration eventually (so I propose it for conclusion) but RAN1 should set up the common understanding that at least if the PUCCH is shared by other UEs, PUCCH resources for ACK/NACK based should be separate in general. |
| Nokia, NSB | We agree with the proposal. However, more clarifications are needed for NACK-only feedback. The following agreement was made in RAN1#103-e meeting: Agreement: For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast. - FFS PUCCH format We believe that the definition of the PUCCH resource configuration as stated in the agreement could be understood differently by different companies (one of PUCCH-config / PUCCH resource set / PUCCH resource) and we raised our concern in RAN1#104-e and RAN1#104bis-e meetings during e-mail discussions. Although FL has clarified in e-mail discussions that the agreement reflects a new PUCCH-config for NACK-only, we think that this issue should be further clarified for a better understanding of the RAN1 group. We propose: For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback, PUCCH-config for NACK-only HARQ-ACK feedback from the UE perspective is separate from the PUCCH-config for HARQ-ACK feedback for unicast and from the optional PUCCH-config for ACK / NACK HARQ-ACK feedback for MBS. |
| vivo | Ok in principle. We think “If PUCCH resource(s) is(are) configured to be shared by a group of UEs for NACK-only based feedback” can be remove, since it is discussed in the above proposal. FL’s response: I agree with other comments it is up to configuration anyway (so I propose it for conclusion) but RAN1 should set up the common understanding that at least if the PUCCH is shared by other UEs, PUCCH resources for ACK/NACK based should be separate in general. |
| Apple | OK with this proposal. |
| Samsung | Do not support the proposal. Same as for 3.3.1-1 - it is a gNB implementation issue. It is not even clear how a UE can know that a resource is “shared”. It is OK to have separate configuration of PUCCH resources for “NACK-only” and for “ACK/NACK” but that is different from mandating the gNB to configure different resources (they don’t have to be different for UCI payloads up to 2 bits). FL’s response: The question from me is if NACK-only is shared by UE, whether UE can use the resource to feedback ACK/NACK based codebook. I propose this is because it is unclear whether the separate PUCCH resource(s) can be shared by ACK/NACK based and NACK-only based, esp. when considering how UE choose ACK/NACK based or NACK-only based for the feedback. |
| FUTUREWEI | OK in principle but some revisions may be needed. The intent of the proposal seems to be supporting separate configurations for NACK-only and ACK-NACK. FL’s response: at least if NACK-only is shared by a group of UE. I suppose PUCCH resource for ack/nack based should be separate in general. |
| Qualcomm | The ‘if…’ should be deleted. Besides the PUCCH resource(s), we think separate power control is needed for NACK-only and ACK/NACK. #### Proposal 3.3.1-2 The PUCCH configuration is separate from that(those) of ACK/NACK based feedback for unicast or multicast if configured. FL’s response: I am not sure it is “is separate” or “can be”. I agree with other comments it is up to configuration anyway (so I propose it for conclusion) but RAN1 should set up the common understanding that at least if the PUCCH is shared by other UEs, PUCCH resources for ACK/NACK based should be separate in general. |
| Ericsson | support |
| NTT DOCOMO | We are fine with the proposal. It is desirable to configure PUCCH resources separately from unicast to handle multiple HARQ-ACK bits in NACK-only feedback. |
| MTK | Support. |

### Round-2

#### Proposal  3.3.2-1 (for conclusion)

Support PUCCH resource for NACK-only that can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

Collect views:

| Company | Comments |
| --- | --- |
| Qualcomm | is it a conclusion? |
| FL’s responses | proposed for conclusion. Yes. |
| LG | We are fine with this proposal. |
| Samsung | We don’t see a need/benefit to have that conclusion – it is a gNB choice - even for ACK/NACK a resource can be shared, an individual UE does not care. However, if companies prefer to have it, OK to have it. |
| CMCC | Ok |
| Lenovo, Motorola Mobility | We are fine with this proposal. |
| OPPO | The“HARQ-ACK feedback” should be associated with same TB(s) in our understanding. |
| CATT | OK |
| MTK | We are fine with the peoposal. |
| NTT DOCOMO | Support |
| Apple | OK for conclusion. |
| Nokia, NSB | Ok. |
| vivo | Ok. Agree with Samsung’s view, it is a gNB choice. |
| Ericsson | Ok. |

FL’s Proposal:

#### Proposal  3.3.2-2 (for conclusion)

If PUCCH resource(s) is(are) configured to be shared by a group of UEs for NACK-only based feedback, from UE perspective, the PUCCH resource(s) should be separate from that for ACK/NACK based feedback for unicast or multicast if configured.

Collect views:

| Company | Comments |
| --- | --- |
| Qualcomm | ok |
| LG | We are fine with this proposal. |
| Samsung | Do not support that conclusion. There is no need for it and, unlike the previous one in 3.3.2-1, this one forces the NW behavior. Is there something that a UE should care about if the resources are not separate (i.e are UE procedures affected)? If yes, the conclusion should actually be an agreement. If not, there is no need for any conclusion. |
| CMCC | ok |
| ZTE | We have indicated our concern for this proposal via email and copied our previous comments below. We don't support the following proposal (Proposal 3.3.1-2). We don't understand why the PUCCH resouce for NACK-only should be separate from that for ACK/NACK. It is a totally network implementation issue. For example, if network activates NACK-only feedback, then PUCCH resouce#x can be used. Later, if network activates ACK/NACK feedback for the same UE (and disable the NACK-only feedback), network can of course reuse the same PUCCH resouce#x for ACK/NACK feedback. We don't see any issue with that. |
| Lenovo, Motorola Mobility | We don’t agree with this proposal. For a given UE, the PUCCH resource for NACK transmission in case of NACK-only based feedback mode can be shared for NACK transmission in case of ACK/NACK-based feedback mode. We don’t why the PUCCH resources for NACK should be separate for the two feedback modes for multicast. On the other hand, this is totally gNB implementation issue. So we don’t need such proposal. |
| OPPO | OK |
| CATT | OK |
| MTK | We are OK with the proposal even though even though this is a network implementation behavior. |
| NTT DOCOMO | Support |
| Apple | OK for conclusion. NACK-only PUCCH resource is group common, so it could be configured in UL CFR, so PUCCH resources for NACK-only feedback are most likely separated from PUCCH resources for unicast. |
| Nokia, NSB | Ok. We believe that multicast PUCCH-config for NACK-only based feedback is different than that of ACK/NACK-based multicast feedback, which would lead to a separate PUCCH resource configuration. |
| vivo | ok |
| Ericsson | OK. |

## Others

Submitted Proposals

(Qualcomm) Proposal 2:

  - For RRC_CONNECTED UEs, support separate PUCCH power control configuration for ACK/NACK-based and NACK-based multicast HARQ feedback.

    - FFS power control parameters

    - FFS signaling details

(OPPO) Proposal 10:

  - For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

(Ericsson) Proposal 2:

  - For NACK-only transmission of HARQ feedback for group scheduling, both Type-I and Type-II codebooks are supported

(Ericsson) Proposal 3:

  - The NACK-only codebook for a G-RNTI is constructed independently from codebooks constructed for other G-RNTIs that the UE may also receive and from the codebook that includes feedback for PDSCH scheduled by C-RNTI.

# HARQ-ACK feedback common

## Feedback priority

Submitted Proposals

“priority across Unicast and M-cast”

(Huawei) Proposal 9:

  - The priority across unicast and multicast is up to network configuration.

    - The comparison of the priority index of unicast and multicast is configured per UE.

(ZTE) Proposal 13:

  - Regarding priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast:

    - The unicast priority indication field in the DCI is reused for the multicast PDSCH;

    - The high/low priority of multicast is equal to high/low priority of unicast, respectively

(vivo) Proposal 10:

  - For the priority for HARQ-ACK feedback for multicast.

    - HARQ-ACK with priority index 0 for multicast has the same priority with HARQ-ACK with priority index 0 for unicast

    - HARQ-ACK with priority index 1 for multicast has the same priority with HARQ-ACK with priority index 1for unicast

(Spreadtrum) Proposal 5:

  - The priorities between multicast and unicast with the same priority index are equal.

(CATT) Proposal 14:

  - The same priority index value has same priority across multicast and unicast priority indication.

(Nokia) Proposal 16:

  - Multicast priorities are equal to their unicast counterparts, e.g., low-priority unicast eMBB transmission has the same priority with low-priority multicast eMBB transmission.

(CMCC) Proposal 7:

  - The priority comparison between multicast and unicast with the same priority index is not supported.

(OPPO) Proposal 1:

  - The priority of MBS HARQ-ACK feedback and unicast HARQ-ACK feedback are the same if they are configured to the same priority level.

(MediaTek) Proposal 5:

  - Unicast and multicast services have the same priority if the configured priority indicator field is the same value.

(LGE) Proposal 23:

  - The following rules of prioritization between MBS HARQ-ACK and unicast HARQ-ACK are applied:

    - The high L1 priority of MBS HARQ-ACK feedback has a lower priority than the high L1 priority of unicast HARQ-ACK feedback.

    - Any high L1 priority of HARQ-ACK feedback has a higher priority of any low L1 priority of HARQ-ACK feedback, regardless of multicast and unicast.

    - The low L1 priority of MBS HARQ-ACK feedback and the low L1 priority of unicast HARQ-ACK feedback have an equal priority.

(DOCOMO) Proposal 2:

  - The priority of unicast and the priority of multicast with the same priority index are the same.

“priority across M-cast and other UCI”

(Nokia) Proposal 17:

  - Priority between HARQ-ACK feedback for multicast and other UCI (SR, CSI) / PUSCH for unicast follow the same rules between HARQ-ACK feedback for unicast and those channels.

(CMCC) Proposal 12:

  - The priority of multicast HARQ-ACK can be lower, higher than or equal to the priority of other UCIs for unicast.

    - Multicast HARQ-ACK and other UCIs with the same priority index have the same priority.

(OPPO) Proposal 2:

  - Same priority rule as that in unicast is used for the priority between HARQ-ACK for MBS and other unicast UCI.

(DOCOMO) Proposal 3:

  - For relations between the priority of HARQ-ACK feedback for multicast and HARQ-ACK/SR/CSI for unicast or PUSCH, reuse existing mechanisms.

“priority indication”

(Huawei) Proposal 8:

  - Regarding the priority index indication for DCI formats scheduling multicast,

    - the priority index is included in the DCI format which takes DCI format 1_1/1_2 as the baseline;

    - the priority index is configured by RRC dedicated signalling for the DCI format which takes DCI format 1_0 as the baseline.

(ZTE) Proposal 13:

  - Regarding priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast:

    - The unicast priority indication field in the DCI is reused for the multicast PDSCH;

    - The high/low priority of multicast is equal to high/low priority of unicast, respectively

(Spreadtrum) Proposal 4:

  - Priority index can be included in DCI formats 1_1 /DCI format 1_2.

(CATT) Proposal 15:

  - In addition to priority index included in DCI, the indication of the priority for multicast by RRC configuration is also supported.

(MediaTek) Proposal 4:

  - The priority indicator field in DCI format 1_1 and 1_2 if supported for scheduling group common PDSCH can be reused to indicate multicast services priority.

(LGE) Proposal 22:

  - The high or low L1 priority of HARQ-ACK feedback is indicated by group common DCI for PTM scheme 1 or UE specific DCI for PTP retransmission.

(DOCOMO) Proposal 1:

  - Introduce a priority indicator field to DCI format 1_0 for multicast.

### Round-1 (closed)

“priority across Unicast/other UCI and M-cast”

FL’s Comments:

- The issues discussed about priority related include the priority comparison between unicast and multicast regarding the same priority index, priority index indication, and priority comparison between  HARQ-ACK and other UCI.

- 11 companies submitted proposals for the priority comparison and 9 companies proposes the priority is the same for the same priority index for unicast and multicast. The same priority rule as that in unicast reused for the priority between HARQ-ACK for MBS and other unicast UCI is also proposed similarly by four companies.

FL’s Proposal:

#### Proposal 4.1.1-1

The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK.

  - The multiplexing rule for the same priority rule defined for unicast HARQ-ACK and other unicast UCI is reused for the multiplexing of the same priority of multicast HARQ-ACK and other unicast UCI.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| CATT | Agree. |
| ZTE | We support the main bullet of this proposal. The sub-bullet is not very clear. It is not clear whether it is for multiplexing issue between HARQ-ACK with the same priority or for multiplexing issue between HARQ-ACK and other UCI. We suggest to make the sub-bullet clearer. FL’s response: The intention of this proposal is multiplexing rule of the same priority of unicast HARQ and other UCI is reused for the case of the same priority of multicast HARQ-ACK and other unicast UCI. The proposal is updated accordingly. |
| CMCC | Ok |
| Nokia, NSB | Agree |
| Spreadtrum | Fine |
| vivo | Ok |
| Apple | OK |
| Samsung | Agree |
| OPPO | Agree |
| Qualcomm | Does HARQ-ACK for multicast include both ACK/NACK and NACK-only? FL’s response: with proposal 3.2.1-0 (newly added) for introducing different priorities for NACK-only agreeable, this proposal can be generic for both. Otherwise, applied to ACK/NACK only. |
| Ericsson | support |
| NTT DOCOMO | We are fine with the proposal. If unicast and multicast with the same priority index have different priorities, UCI transmission procedure will be more complicated. |
| MTK | We are Ok with the proposal |
| Qualcomm2 | The newly added 3.2.1-0 seem not addressing the priority between NACK-only and ACK/NACK. Does the other unicast UCI include the UCI for multicast? |

“priority indication”

FL’s Comments:

- 7 companies submitted proposals discussing the priority index indication and the indication is included at least in the DCI format that takes DCI format 1_1 or 1_2 as the baseline, which is the same mechanism as defined in Rel-16 for URLLC.

FL’s Proposal:

#### Proposal 4.1.1-2

The priority index is,

- for the second DCI format with CRC scrambled with G-RNTI which takes DCI format 1_1 or 1_2 as the baseline, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

  - Note: this is the same as what is specified for URLLC.

- for the first DCI format with CRC scrambled with G-RNTI which takes DCI format 1_0 as the baseline, Opt1 is supported  from the following options:

  - Opt1: optionally configured to be included in the DCI format

  - Opt2: configured by RRC

  - Opt3: hard-coded in specification.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| LG | We are fine with this proposal. |
| ZTE | Ok to list the potential options. However there is one option missing for the last bullet. Updated proposal from ZTE: The priority index is, - for the second DCI format with CRC scrambled with G-RNTI which takes DCI format 1_1 or 1_2 as the baseline, included in the DCI format. - for the first DCI format with CRC scrambled with G-RNTI which takes DCI format 1_0 as the baseline, down-select from: - Opt1: included in the DCI format - Opt2: configured by RRC - Opt3: predefined by the specification FL’s response: reflected. |
| CMCC | Ok |
| Nokia, NSB | OK |
| Spreadtrum | Support the updated version from ZTE |
| vivo | We think for the second DCI format with CRC scrambled with G-RNTI which takes DCI format 1_1 or 1_2 as the baseline, it can be optional to configured the priority index indicator in the DCI format, like that of unicast. If the DCI has no priority index indicator, it can be low priority as that in Rel-16 for unicast, too. FL’s response: reflected accordingly. |
| Apple | OK |
| Samsung | Do not support the proposal. The reason is that the proposal allows potential differentiation between a “DCI format 1_0” baseline and a “DCI format 1_1/1_2” baseline. There is no apparent reason for that (the “fallback” nature of DCI 1_0 for unicast is not applicable for multicast – just the size needs to be same for the DCI size budget purposes). Although agree with the first sub-bullet, it would be preferable to first discuss why a “DCI format 1_0” baseline needs to be treated differently than a “DCI format 1_1/1_2” baseline (that discussion anyway needs to occur even if the proposal is agreed). FL’s response: we can try to down-select to opt1 and see other’s views. |
| Qualcomm | The priority index is optional for unicast DCI format 1_1 and 1_2. The first subbullet means the priority index is mandatory for multicast DCI format 1_1 and 1_2? FL’s response: same comments as vivo, reflected accordingly. |
| Ericsson | Support, but also ok to delay until the DCI discussion is more mature. |
| NTT DOCOMO | We are fine with the proposal. We prefer Opt1. It is possible to introduce a priority indicator to DCI format 1_0 for multicast while maintaining the size of the existing DCI format 1_0 by using unused bits for multicast(e.g., UL/DL identifier). |
| MTK | We have the similar question with QC. FL’s response: same comments as vivo, reflected accordingly. |
| Qualcom2 | We prefer to further study the alternatives for DCI 1_0. |
| OPPO1 | OK |
| vivo2 | We prefer to reuse the priority indication mechanism defined in R16. For DCI 1_0, it is low priority (unless opt1 is adopted). |

## Feedback mode selection

Submitted Proposals

“RRC configured”

(vivo) Proposal 12:

  - Support to configure the HARQ-ACK feedback option by RRC signalling.

(vivo) Proposal 13:

  - Not support to switch between NACK-only and ACK/NACK based HARQ-ACK feedback option for the same MBS service.

(Intel) Proposal 4:

  - For RRC_CONNECTED UEs, NR MBS supports both ACK/NACK based and NACK-only HARQ feedback. The configuration of ACK/NACK and NACK only mode can be done using the following options

    - Option 1: Semi-static RRC configuration of ACK/NACK or NACK only mode

    - Option 2: The configured PUCCH resource can contain additional indication that the UE is expected to transmit only NACK on the configured resource

    - Option 3: If UE has no dedicated PUCCH resource configuration, the UE uses cell-specific PUCCH resource and is expected to only transmit NACK

(ETRI) Proposal 1:

  - Support at least RRC configuration for UEs to decide which HARQ-ACK feedback scheme to use.

“Dynamic switching”

(ZTE) Proposal 11:

  - If NACK-only feedback is supported, consider the following method to determine the feedback mechanism for the UE between ACK/NACK feedback and NACK-only feedback:

    - PUCCH resource sets containing ACK/NACK PUCCH resources and NACK-only PUCCH resources is configured for multicast for the UE.

    - PRI in DCI is used to indicate the feedback mechanism and PUCCH resources from the PUCCH resource set.

(Nokia) Proposal 14:

  - A dynamic mechanism, such as switching via MAC-CE, is introduced to configure which HARQ-ACK feedback scheme is used by the UE.

(Lenovo) Proposal 5:

  - Option 1 (NACK-only based HARQ-ACK feedback) and Option 2 (ACK/NACK-based feedback) can be dynamically switched based on gNB’s scheduling policy and PUCCH resource capacity.

(Samsung) Proposal 8:

  - Support indication for a UE to report NACK-only or ACK/NACK for multicast PDSCHs by enabling the functionality via UE-specific RRC and indicating the report type by DCI formats scheduling multicast PDSCHs.

(MediaTek) Proposal 1:

  - Network can flexibly choose the HARQ-ACK mode and the HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

(Ericsson) Proposal 1:

  - We propose that the UEs can be dynamically reconfigured between NACK-only mode, ACK/NACK mode, and no feedback mode.

- others

(vivo) Proposal 11:

  - UEs can be defined into three types in terms of HARQ-ACK feedback option:

    - Type 1 UE: does not support any feedback

    - Type 2 UE: supports NACK-only based HARQ-ACK feedback

    - Type 3 UE: supports ACK/NACK based HARQ-ACK feedback

(Qualcomm) Proposal 1:

  - For RRC_CONNECTED UEs, support selection of UE-specific ACK/NACK and NACK-only for different UEs in the same group.

(Intel) Proposal 1:

  - For RRC_CONNECTED UEs receiving multicast supporting both ACK/NACK based and NACK-only HARQ feedback:

    - ACK/NACK based feedback is used for delivery mode with PTP or PTM Scheme 2 (if supported)

    - NACK only feedback is used for delivery modes 1 and 2 with PTM Scheme 1

    - UEs within a group receiving multicast transmission can be configured with different HARQ feedback modes.

(LGE) Proposal 9:

  - Select one of the following options to configure HARQ-ACK feedback scheme in PUCCH-config for multicast.

    - Option 1: Only one of NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in a PUCCH-config for multicast.

      - FFS: Whether/how to support ACK/NACK based HARQ-ACK if NACK only based HARQ-ACK is configured in PUCCH-config for multicast

      - FFS: Whether to support more than one PUCCH-config for multicast

    - Option 2: Both NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in PUCCH-config for multicast.

      - FFS: Whether NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK are can be jointly configured within a same PUCCH resource set or separately configured for different PUCCH resource sets.

(DOCOMO) Proposal 13:

  - HARQ-ACK feedback scheme for multicast is configured per priority index.

### Round-1 (closed)

FL’s Comments

Now that we have agreed to support both ACK/NACK based and NACK-only based HARQ-ACK feedback for multicast, whether it is ACK/NACK or NACK-only based feedback for a given PUCCH transmission is a question needs to be discussed.

13 companies submitted proposals regarding this issues, and wherein 3 companies support RRC configuring which mode is to be used and 6 companies propose both modes can be configured and DCI/MAC-CE can indicate which mode is to be used for a given transmission, e.g., via PRI in the DCI indicating the PUCCH resources for the feedback.

The first question is whether UE can be configured with PUCCH resources used for ACK/NACK based feedback and PUCCH resources used for NACK-only based feedback.

FL’s Proposal:

#### Proposal 4.2.1

For UE supports both ACK/NACK based and NACK-only based feedback for multicast, UE can be configured

- Alt1: either PUCCH resources for ACK/NACK based feedback or PUCCH resources for NACK-only based feedback.

- Alt2: both PUCCH resources for ACK/NACK based feedback and PUCCH resources for NACK-only based feedback.

  - Opt1: PRI in the DCI indicates the PUCCH resource of ACK/NACK based or of NACK-only based.

  - Opt2: a new field added in DCI indicates ACK/NACK based or NACK-only based.

- Down-select from the two alternatives and options.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Alt 2 is supported. Should it be modified as? - Alt2: both PUCCH resources for ACK/NACK based feedback and PUCCH resources for NACK-only based feedback. FL’s response: updated. |
| LG | We are fine with change from ‘or’ to ‘and’ as Lenovo commented. |
| CATT | We thanks the FL’s summary and the intention of this proposal is clear for us. But the proposal needs more clarification. Based on Lenovo’s comments, we suggest the proposal can be changed as: For UE supports both ACK/NACK based and NACK-only based feedback for multicast, for a given PUCCH transmission UE can be configured - Alt1: either PUCCH resources for ACK/NACK based feedback or PUCCH resources for NACK-only based feedback. - Alt2: both PUCCH resources for ACK/NACK based feedback and PUCCH resources for NACK-only based feedback. - Down-select from the two alternatives. FL’s response: updated “and” but not sure about the intention of “for a given PUCCH transmission”. Rather the intention is starting from the configuration, whether it is ACK/NACK or NACK-only for a given transmission, depends on which alterative is supported. See my response to Samsung below. |
| ZTE | We understand the intention of this proposal. However, it seems the PUCCH resource configuration is a separate issue from whether UE can be configured with ACK/NACK and NACK-only based feedback. We propose to reconstruct the proposal. FL’s response: the intention is clarified a bit more in the updated, please check whether it is fine now. |
| CMCC | Does Alt 2 conflicts with proposal 3.3.1-2? #### Proposal 3.3.1-2 If PUCCH resource(s) is(are) configured to be shared by a group of UEs for NACK-only based feedback, for UE perspective, the PUCCH resource(s) should be separate from that for ACK/NACK based feedback for unicast or multicast if configured. FL’s response: They are different things. 3.3.1-2 is saying if NACK-only is configured, ACK/NACK resources should be separate, it could be ACK/NACK for unicast or multicast. Here the proposal is actually discussing the first step for RRC based or DCI based switching. See responses to Samsung. |
| Nokia, NSB | We support Alt 2. We believe that the UEs in worse channel conditions (outlier UEs) can be configured from NACK-only feedback to ACK/NACK feedback to not to disrupt NACK-only operation. Moreover, different services may also have different conditions (e.g. one service has a large PTM group, whereas the other has relatively a smaller one), so different feedback mechanisms may be needed. |
| Spreadtrum | Maybe we should firstly discuss whether UE could be simultaneously configured/eneble ACK/NACK based feedback and NACK-only feedback, then discuss the PUCCH resource issue. FL’s response: the intention of this proposal is to address the issue you are saying. |
| vivo | We also think the proposal should be postponed and how to select/configure HARQ-ACK modes should be discussed first. |
| Apple | OK with this proposal with wording update and in Alt2. |
| Samsung | OK with the proposal. But the real intention is to decide whether to have DCI-based switching or RRC-based configuration between “ACK/NACK” based and “NACK-only” based HARQ-ACK. It would be better to directly discuss that aspect instead of listing related consequences. FL’s response: it is a bit complex per my review of the contributions. I’d like to take it step by step originally. To me, the proposal is not consequences of DCI based switching or RRC based switching but rather the condition. If Alt1 is supported, it is only to be RRC-based. If alt2 is supported, it is basically DCI based but there are two options how this is going to work. I updated the proposal to move forward a bit further. |
| OPPO | Agree with the proposal, we prefer Alt 1, from our perspective, the motivation of Alt 2 is to support dynamic change between ACK/NACK based feedback and NACK-only feedback, however, we did not see the need for that. |
| FUTUREWEI | Alt 2 with the correction suggested by Lenovo. Both ACK/NACK and NACK-only feedback can be configured at the same time. |
| Qualcomm | Prefer Alt2. |
| Ericsson | Agree, prefer Alt 1 |
| NTT DOCOMO | We are fine with the proposal. We prefer Alt2. Both may be configured regardless of whether or not they are actually used. |
| MTK | Lenovo’s version is fine for us. |
| Nokia, NSB | We prefer an updated version of Alt 2, with an additional Opt.3 for consideration: Alt2: both PUCCH resources for ACK/NACK based feedback and PUCCH resources for NACK-only based feedback. - Opt1: PRI in the DCI indicates the PUCCH resource of ACK/NACK based or of NACK-only based. - Opt2: a new field added in DCI indicates ACK/NACK based or NACK-only based. - Opt3: MAC-CE indicates the ACK/NACK based or NACK-only based. Regarding Opt1 of Alt2, (considering PRI is only 3 bits), we do not need to introduce such kind of a limitation to separate PUCCH resources as ACK/NACK and NACK-only based on PUCCH resource ID. In our view, PRI will be used to indicate a PUCCH resource in a specific PUCCH resource set, as in current specs. The PUCCH resource set and even PUCCH-config can be different for multicast ACK/NACK and NACK-only feedback mechanisms. Thus, a new mechanism is needed to choose which feedback scheme to use. Dynamic configuration of which scheme to use can be done with MAC-CE, hence our suggested Opt3. |
| Qualcomm2 | Need more clarification on the newly added Options. The Opt1 seem related with PUCCH-config for ACK/NACK and NACK-only. Does Opt1 implicitly mean a joint PUCCH-config is configured and PUCCH resources are split for A/N and N-only? If separate PUCCH-configs are configured for A/N and N-only, can PRI differentiate A/N and N-only? |
| ETRI | Fine with the proposal in principle. Down-selection can be done after reaching a common understanding on the proposal itself. |
| ZTE | We are ok with the updated proposal in principle. But the updated proposal is only discussing whether PUCCH resource for ACK/NACK and NACK-based need to be configured. Another issue is whether the PUCCH resource for them can be configured in the same PUCCH resource set or not. This will impact the opt.1 and opt.2 listed under Alt.2 here. Thus, we suggest to add an FFS point to address this issue. FFS: whether PUCCH resource for ACK/NACK based feedback and NACK-only based feedback can be configured in the PUCCH resource set. |
| Lenovo 2 | The updated proposal is generally OK with us. Just for clarification: In both alternatives, does it imply PUCCH resource for A/N based feedback and PUCCH resource for NACK-only based are orthogonal or not? E.g., whether the PUCCH resource for NACK in case of A/N feedback can be shared with PUCCH resource for NACK-only feedback? |
| vivo | Considering the QoS for different services are different, we think HARQ-ACK feedback mode can be configured per g-RNTI for DG PDSCH or per SPS-config for SPS PDSCH. Then both if PUCCH resources for ACK/NACK based and NACK-only based feedback are separate, UE can determine the PUCCH resource for feedback based on the RNTI of PDCCH/PDSCH or SPS configuration index. We think the following additional opt3 can also be considered. - Alt2: both PUCCH resources for ACK/NACK based feedback and PUCCH resources for NACK-only based feedback. - Opt1: PRI in the DCI indicates the PUCCH resource of ACK/NACK based or of NACK-only based. - Opt2: a new field added in DCI indicates ACK/NACK based or NACK-only based. - Opt 3: g-RNTI or SPS configuration index is used to determine ACK/NACK based or NACK-only based. |

### Round-2

FL’s Comments

Based on the comments on GTW, this issue will resume once we progress on the discussion of PUCCH resources.

## Enable/disable HARQ-ACK feedback

Submitted Proposals

“RRC based”

(vivo) Proposal 2:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, RRC indicates enabling/disabling (i.e. option 2) is supported.

(Spreadtrum) Proposal 9:

  - One new RRC signaling is introduced to indicate whether/which HARQ-ACK feedback is enabled for RRC_CONNECTED UE receiving multicast.

(Nokia) Proposal 37:

  - RRC-based enabling / disabling (Option 2) of HARQ-ACK feedback is used for MBS and Option 3 is not supported.

(Qualcomm) Proposal 7:

  - For RRC_CONNECTED UEs, HARQ-ACK feedback can be enabled/disabled by RRC signaling.

    - The configuration of HARQ-ACK feedback can be configured for a given G-RNTI (corresponding to a service) or for a UE receiving a service.

(Apple) Proposal 5:

  - HARQ-ACK feedback is only enabled or disabled by RRC signaling for PTM scheme 1.

“RRC+DCI based”

(Huawei) Proposal 12:

  - Support group-common DCI indicating enabling/disabling HARQ-ACK feedback.

(CATT) Proposal 1:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, Option 3 (i.e. RRC signaling configures the enabling/disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback) is supported.

    - For DCI indicating, a special state of an existing DCI field (i.e. k1 or PRI) can be pre-configured or pre-defined to indicate the disabling HARQ-ACK feedback.

    - When RRC signaling does not configure the function, enabling the feedback is the default mode.

(OPPO) Proposal 9:

  - For HARQ feedback enabling/disabling, only Option 3 is supported.

    - if RRC signalling does not configure the function, HARQ feedback is disabled in default;

    - if RRC signalling configures the function, 2 bits in DCI are used to indicate whether/which HARQ feedback is enabled;

(Lenovo) Proposal 6:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast, RRC signaling configures a non-numerical value in the K1 set and PDSCH-to-HARQ_timing indicator in the DCI indicates a numerical value or the non-numerical value for enabling or disabling the HARQ-ACK feedback.

(Samsung) Proposal 2:

  - Support enabling/disabling of HARQ-ACK reports for multicast PDSCHs from a UE by enabling the functionality via UE-specific RRC and indicating enabling/disabling by DCI formats scheduling multicast PDSCHs.

(MediaTek) Proposal 2:

  - RRC signalling configures the enabling/disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback is supported for multicast services.

(ETRI) Proposal 2:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, RRC signalling configuration for the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback is supported.

(Convida) Proposal 1:

  - For MBS HARQ-ACK feedback enabling/disabling, option 3 (i.e., RRC configures the enabling/ disabling function and DCI indicates enabling /disabling) is supported.

- “MAC-CE based”

(Spreadtrum) Proposal 8:

  - Not support enabling/disabling HARQ-ACK feedback by MAC-CE.

(Nokia) Proposal 38:

  - If use cases that require dynamic enabling / disabling are found, MAC-CE is preferred over the methods proposed in Option 3.

(Ericsson) Proposal 8:

  - Enabling/disabling HARQ-ACK feedback for MBS is supported by both RRC configuration and MAC CE.

- “Support more than one”

(ZTE) Proposal 12:

  - Regarding enabling/disabling HARQ-ACK feedback for MBS, option 2, 3 and 4 are supported together.

    - Option 2: RRC indicates enabling/disabling

    - Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

    - Option 4: MAC-CE indicates enabling/disabling.

(CMCC) Proposal 17:

  - Option 2 and Option 3 can be supported for enabling/disabling HARQ-ACK feedback for multicast.

(LGE) Proposal 25:

  - Support at least both Option 2 and/or 3 for enabling/disabling HARQ-ACK feedback. Which option is used is up to gNB configuration.

- (DOCOMO) Proposal 4:

  - Support both Option 2 and Option 3 for enabling/disabling HARQ-ACK feedback for multicast.

    - Introduce an RRC parameter with 3 states {enabled, disabled, dynamic}.

      - When ‘enabled’, HARQ-ACK is enabled.

      - When ‘disabled’, HARQ-ACK is disabled.

      - When ‘dynamic’, if PRI is zero and PDSCH-to-HARQ feedback timing indicator is zero, HARQ-ACK feedback is disabled. Otherwise, HARQ-ACK feedback is enabled.

- (Ericsson) Proposal 8:

  - Enabling/disabling HARQ-ACK feedback for MBS is supported by both RRC configuration and MAC CE.

- “default mode”

- “default enabled”

(CATT) Proposal 1:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, Option 3 (i.e. RRC signaling configures the enabling/disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback) is supported.

    - For DCI indicating, a special state of an existing DCI field (i.e. k1 or PRI) can be pre-configured or pre-defined to indicate the disabling HARQ-ACK feedback.

    - When RRC signaling does not configure the function, enabling the feedback is the default mode.

- “default disabled”

(OPPO) Proposal 9:

  - For HARQ feedback enabling/disabling, only Option 3 is supported.

    - if RRC signalling does not configure the function, HARQ feedback is disabled in default;

    - if RRC signalling configures the function, 2 bits in DCI are used to indicate whether/which HARQ feedback is enabled;

(MediaTek) Proposal 3:

  - Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

(ETRI) Proposal 4:

  - The default operation when RRC configuration on HARQ-ACK feedback scheme is not configured is HARQ-ACK feedback off.

- “default enabled if no DCI indication”

(Intel) Proposal 3:

  - Proposal 3: For NR MBS, if RRC configures enabling and disabling of HARQ, the default mode is HARQ ON if no DCI indication is provided.

- others

(CATT) Proposal 2:

  - Mechanism of enabling/disabling HARQ-ACK feedback can be used for both ACK/NACK and NACK-only based HARQ-ACK feedback.

(ETRI) Proposal 3:

  - RRC configuration on HARQ-ACK feedback scheme includes configuration for the enabling/disabling function of DCI indicating the enabling/disabling HARQ-ACK feedback.

(Ericsson) Proposal 9:

  - If enabling/disabling HARQ-ACK feedback for MBS is supported by MAC CE, the MAC CE message shall include a PUCCH resource indicator so that the UE knows which PUCCH resource to use. The mapping between PUCCH resource indicator and PUCCH resource is RRC configured.

(Ericsson) Proposal 10:

  - Since NACK-only is supported, enabling/disabling HARQ feedback can be extended to include NACK-only case. That is, UE can be configured with UE dedicated ACK/NACK PUCCH resource, shared NACK-only PUCCH resource and no PUCCH at all and switch among the three modes via RRC signaling or MAC CE command.

### Round-1 (closed)

FL’s comments:

The agreement achieved earlier is as follows:

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

18 companies submitted proposals for this issue in this meeting with diverse views (still) as follows:

- 5 companies support option 2 only

- 8 companies support option 3

- 2 companies support MAC-CE based and 1 company propose not to support MAC-CE based

- 4 companies support option 2+3 and 1 company support option2+MAC-CE

There are also a couple of proposals support the mechanism can be used for enabling/disabling NACK-only based feedback.

This issue has been discussed for a couple of meetings without progress. Hopefully we can close this issue in this meeting though the views are still diverse right now.

I would point out the arguments from Samsung for option 3 as follows for people to consider:

Possible reasons can only relate to dynamic system-wide considerations. Such reason can be to enable the gNB to control PUCCH resource overhead for multicast HARQ-ACK on a slot basis, for example in order to maximize resource availability for unicast PUSCH/PUCCH/SRS in a slot. For example, in order to prioritize resource allocation for unicast UL transmissions, the gNB can decide to disable HARQ-ACK reporting for multicast PDSCH and possibly reschedule. In general, given the potential large overhead of PUCCH transmissions with HARQ-ACK for multicast PDSCH, it is beneficial to allow the serving gNB to dynamically control that overhead.

From FL’s perspective, the option 4 in FL’s proposal aims to reflect the proposals of option 2+option 3.

Companies are not necessary to repeat your own views but are encouraged to address the comments/arguments from the other side. We can focus on how to improve the FL proposal so as to make decision on GTW if necessary.

FL’s Proposal:

#### Proposal 4.3.1

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, support one from the follow options:

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- Option 4: in option 3, if RRC signalling does not configure the function, the default mode (HARQ-ACK feedback is on or off) can be configured in the RRC signalling.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| LG | We are fine with this proposal. |
| CATT | We are not OK with the proposal and some rewording may be needed. For our understanding, the Option 4 is the FFS in the Option 3 when the RRC signalling does not configure the function. It is not clear for us why Option 4 is added. FL’s responses: FFS in option 3 is hard-coded mode as the default if RRC does not configure the function. Opt4 is the mode is also configured if RRC does not configure the function which is actually opt2+opt3 as proposed by some companies. |
| ZTE | After being discussing this issue for multiple meeting, we still believe the following Opt.5 would be the possible compromised solution. Updated proposal from ZTE: For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, support one from the follow options: - Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback. - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled - FFS details on RRC signalling and DCI indicating. - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback. - FFS whether enabling or disabling the feedback is the default mode. - Option 2: RRC indicates enabling/disabling. - Option 4: in option 3, if RRC signalling does not configure the function, the default mode (HARQ-ACK feedback is on or off) can be configured in the RRC signalling. - Option 5: RRC configures the following modes {enable, disable, enable&&disable} - If enable is configured, HARQ-ACK feedback is always enabled. - If disable is configured, HARQ-ACK is disabled. If enable&&disable is configured, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled FL’s responses: option 5 as suggested is essentially option 4 added. The only difference if any is the parameters which can be up to RAN2, but the functions are essentially the same. |
| CMCC | Ok |
| Nokia, NSB | We support Opt 2. To limit PUCCH overhead, dynamic switching from ACK/NACK to NACK-only can be used. We have shown that PUCCH overhead of NACK-only feedback is significantly small (down to 5%) compared to ACK/NACK based mechanism. We do not think that NACK-only feedback should be disabled due to uplink resource constraints, since anyway 1 PRB may be sufficient for that. If the contention is that severe, than the system could go for a more semi-static long term disabling (by RRC). DCI based enabling / disabling would incur significant amount of overhead, if unicast DCI is used. Moreover, if multicast DCI is used, then the HARQ-ACK feedback of individual UEs cannot be enabled/disabled, rather the group will behave the same. We have identified several use cases where individual UE’s HARQ feedback may be enabled/disabled (e.g. outlier UEs). Thus, we believe that Opt 3 is not capable to address those issues. |
| Spreadtrum | fine |
| vivo | We support Opt 2. We think the enabling/disabling should be per UE, group-common DCI can’t do in such way. |
| Apple | Option 2 is preferred. |
| Samsung | “Option 4” seems to be a separate issue and can be discussed regardless of “Option 2 or 3”. “Option 2 vs. Option 3” may need to be discussed together with dynamic switching between “ACK/NACK” and “NACK-only” as that also intends to address PUCCH resource overhead (UE power consumption is a marginal issue). FL’s responses: I don’t follow why opt4 is separate. The reason I added option 4 is option4=opt2+opt3, and we end up only selecting a single option then. I tend to separate this from “ACK/NACK” and “NACK-only” because for each proposal people still have diverse views |
| OPPO | OK |
| FUTUREWEI | OK with the proposal. |
| Qualcomm | We prefer Option 2. Agree with CATT/Samsung that Option 4 is not in the same level as Option 2/3. FL’s responses: I don’t follow why opt4 is separate. The reason I added option 4 is option4=opt2+opt3, and we end up only selecting a single option then. |
| Convida | Fine with the proposal. We support option 3. |
| Ericsson | Proposed answer: Option 4 is interesting, but we think there are some aspects missing: - With support for NACK-only, there is a need for the general enable/disable functionality to include all three states: ACK/NACK, NACK-only and no HARQ feedback. - When the DCI function is not RRC configured, the default mode can be RRC configured to be any of these three states. This means the gNB can, via RRC reconfiguration change the HARQ-ACK state of individual UEs between these three states. - We miss however the proposed functionality to address individual UEs with MAC-CE to indicate state and resources to use, based on RRC configuration. - When the DCI function is configured, DCI indicates whether UE should transmit ACK/NACK. When the DCI bit indicates no ACK/NACK feedback the UE may send no feedback or NACK-only feedback, depending on individual RRC configuration. FL’s responses: I know there is proposal of MAC-CE but less support and we have to down-select anyway. I tend to close this issue in this meeting since it has been discussed a couple of meetings without seeing the trend of convergence nor more new arguments, to do so I may need guidance from chair on GTW. |
| NTT DOCOMO | We think the relation between RNTI and service is unclear. Our current understanding is that a group-common RNTI can provide multiple multicast services. So we think that dynamic indication by DCI will be useful when providing multiple MBS services with different QoS levels. However, some companies that support semi-static approach seem to think a separate group-common RNTI will be configured for each multicast service. |
| MTK | OK with the proposal. We prefer Option 3. |
| Qualcomm2 | Reply to NTT DOCOMO, we think G-RNTI can provide multiple multicast services with similar QoS requirements (mapping between services and G-RNTI is up to RAN2 discussion). The RRC configuration per G-RNTI (Opt2) is sufficient. A related question, the function of enable/disable in Opt3 and Opt4 is per G-RNTI, per DCI format, or combo? FL’s responses: taking opt3 as an example, DCI indicates it, DCI format is still being in discussion, and the DCI is scrambled by G-RNTI, so not sure whether we should fix the issue as you asked regarding per G-RNTI, or per DCI format. Or rather it is up to network. |
| ETRI | Fine with the proposal. We prefer Option 3. |
| NTT DOCOMO2 | To Qualcomm, thanks for the question. We prefer per G-RNTI. We believe that feedback enable/disable should be related to the QoS level of the multicast service. When a G-RNTI provides a single service or multiple services with similar QoS levels, we think Opt2 is appropriate. However, we think Opt3 is appropriate when a G-RNTI provides multiple services with different QoS levels. |
| CATT2 | We thank the FL’s response. May I understand that unlike Opt3 where the default mode will be settled as enabling or disabling when the the function is not configured, in Opt 4, both the enabling and disabling mode can be used as by the default mode, and further discussion will be needed to determine which mode is the final mode? FL’s responses: opt4, the default mode is up to network for configuration. |
| FL’s comments, | Based on the above discussion, though I know companies still have the own preference, FL may suggest supporting option 4 for compromise. |

### Round-2

Based on the above discussion in Round-1, though I know companies still have the own preference, FL suggests supporting option 4 for compromise for progress:

#### Proposal 4.3.2

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, support Option 4 from the follow options:

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- Option 4: in option 3, if RRC signalling does not configure the function, the default mode (HARQ-ACK feedback is on or off) can be configured in the RRC signalling.

## Retransmission

Submitted Proposals

“CBG-based retransmission”

(CATT) Proposal 21:

  - CBG based PTP retransmission can be supported in multicast retransmission when a UE is configured with CBG transmission for unicast.

(Nokia) Proposal 1:

  - CBG-based re-transmissions are not supported for PTM re-transmissions.

(Intel) Proposal 8:

  - For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

- “retransmission vs. HARQ-ACK feedback mode”

(FUTUREWEI) Proposal 1:

  - NACK-only feedback should be used when PTM mode 1 is the current transmission mode and a retransmission is scheduled using PTM mode 1.

(MediaTek) Proposal 7:

  - PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

(MediaTek) Proposal 8:

  - PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

(Convida) Proposal 2:

  - PTM transmission scheme 2 should be supported for MBS retransmission.

(Chengdu TD Tech) Proposal 3:

  - For the ACK/NACK based HARQ-ACK feedback for an SPS MRB of the PTM bearer of the MBS session, both the PTP bearer and the PTM bearer with PTM scheme 1 can be used for the retransmission of the NACKed TB.

(Chengdu TD Tech) Proposal 4:

  - For the NACK-ONLY based HARQ-ACK feedback for an SPS MRB of the PTM bearer of the MBS session, the PTM bearer with PTM scheme 1 can be used for the retransmission of the NACKed TB, where PTM scheme 1 can use beam sweeping or partial beam sweeping.

### Round-1 (closed)

FL’s Comments

Whether UE supports simultaneously PTM scheme 1 an PTP for retransmission is currently discussed in AI 8.12.1 and whether UE can dynamic switch between ACK/NACK based and NACK-only based HARQ-ACK feedback is also to be discussed. The issues related in this section can be discussed later.

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| LG | We are fine with this proposal. |
| CATT | OK. |
| CMCC | Ok |
| Nokia, NSB | OK |
| Apple | OK |
| Samsung | OK |
| OPPO | OK |
| Convida | OK. |

# HARQ-ACK feedback for multicast SPS

Submitted Proposals

“configuration for SPS multicast”

(FUTUREWEI) Proposal 1:

  - For support of HARQ-ACK feedback for SPS multicast, use SPS-config configuration as a baseline for SPS multicast.

(Huawei) Proposal 10:

  - The priority for SPS multicast is configured in SPS-config for multicast.

(Huawei) Proposal 11:

  - When the separate PUCCH-config for multicast is configured, it can include the specific SPS-PUCCH-AN-List for multicast. Otherwise, the SPS-PUCCH-AN-List included in PUCCH-config for unicast is shared between unicast and multicast.

(Spreadtrum) Proposal 10:

  - For ACK/NACK based feedback for RRC_CONNECTED UEs receiving multicast SPS PDSCH, UE can be configured with a separate SPS-PUCCH-AN-list for multicast SPS PDSCHs.

(Qualcomm) Proposal 8:

  - For ACK/NACK-based feedback of RRC_CONNECTED UEs receiving SPS GC-PDSCH,

    - UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS GC-PDSCH configurations.

    - Otherwise, a common SPS-PUCCH-AN-List applies to all SPS PDSCH and SPS GC-PDSCH configurations.

- “HARQ-ACK mode for SPS multicast”

(ZTE) Proposal 14:

  - Regarding HARQ-ACK feedback for SPS-based MBS transmission, if NACK-only feedback is configured:

    - The NACK-only feedback mode applies to ‘PDSCH without scheduling PDCCH’ only

    - The feedback mode for ‘PDSCH with scheduling PDCCH’ can be,

- Option 1: fixed to ACK/NACK feedback

- Option 2: follow the configuration of feedback mode for DG-PDSCH

    - FFS: the feedback mode for deactivation PDCCH

(Spreadtrum) Proposal 12:

  - NACK-only based feedback for SPS PDSCH reception can be considered with low priority for study.

(Nokia) Proposal 39:

  - Both ACK / NACK and NACK-only based HARQ-ACK feedback schemes are supported also in case of SPS.

(Qualcomm) Proposal 9:

  - Support UE-specific ACK/NACK-based feedback for activation/release for SPS GC-PDSCH

  - Support no feedback, ACK/NACK-based or NACK-based feedback for SPS GC-PDSCH without PDCCH

(Lenovo) Proposal 11:

  - For group-common SPS configuration, a UE-specific PUCCH resource is configured for each UE to transmit ACK upon reception of activation/deactivation DCI.

(Lenovo) Proposal 12:

  - For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is also used for the UE to transmit ACK for the SPS PDSCH.

(LGE) Proposal 10:

  - For group common SPS, UE specific confirmation to group common SPS (de-)activation can be supported by PUCCH A/N.

    - UE specific PUCCH resource is allocated by DCI indicating SPS (de-)activation.

(LGE) Proposal 14:

  - For group common SPS, group common PUCCH resources used for NACK only based HARQ-ACK is semi-statically configured per SPS configuration for SPS PDSCH transmissions.

(Convida) Proposal 7:

  - Both ACK/NACK based HARQ-ACK feedback and NACK-only based HARQ-ACK feedback should be considered for MBS SPS PDSCH.

(Convida) Proposal 8:

  - Consider to support HARQ-ACK feedback for MBS SPS activation DCI and deactivation DCI.

(Chengdu TD Tech) Proposal 1:

  - Support the following HARQ-ACK feedback methods for each SPS MRB of the PTM bearer of an MBS session:

    - ACK/NACK based HARQ-ACK feedback with the different SPS PUCCH resources for the different RRC_CONNECTE UEs.

      - FFS: details for the ACK/NACK based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

    - NACK-ONLY based HARQ-ACK feedback with the different RRC_CONNECTED UEs sharing the same SPS PUCCH resources

      - FFS: details for the NACK-ONLY based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

      - FFS: whether or not other information can be fed back with the NACK-ONLY information.

(Chengdu TD Tech) Proposal 2:

  - The HARQ-ACK feedback method for each SPS MRB of the PTM bearer of an MBS session can be set independently. The HARQ-ACK feedback method for each SPS MRB can be signalled to UE by

    - Option 1: Use RRC signalling to enable one of the HARQ-ACK feedback method

    - Option 2:

      - Use RRC signalling to enable one of the HARQ-ACK feedback method

      - Use DCI format to enable/disable the HARQ-ACK feedback method dynamically

    - FFS: Which option to use

- “HARQ-ACK codebook”

(Spreadtrum) Proposal 11:

  - For codebook concatenation to be multiplexed in the same PUCCH resource for multicast and unicast SPS PDSCHs,

    - The HARQ-ACK bits are generated jointly across multicast and unicast SPS PDSCHs

- others

(Nokia) Proposal 41:

  - For NACK-only HARQ operation a mechanism should be used, in which UEs are made aware via RRC signalling that SPS might be used for an MBS and request retransmission of an SPS activation PDCCH only if they have not received it in a certain amount of time.

(Nokia) Proposal 42:

  - While gNBs can send SPS deactivation commands, that are in NACK-only mode not acknowledged by UEs, UEs can assume that SPS has been deactivated if they have not been able to decode a PDSCH for a certain period of time.

(Nokia) Proposal 43:

  - In NACK-only HARQ operation, a method is supported for UEs to check with the gNB whether an SPS (re-)activation has been sent by the gNB but missed by the UE. Options include:

    - (a) Option 1: Using a group-common uplink resource

    - (b) Option 2: Using UE-specific signalling (MAC-CE or RRC message)

    - (c) Other methods are not precluded.

(LGE) Proposal 15:

  - For group common SPS retransmission, PUCCH resource is allocated by DCI of which CRC is scrambled by G-CS-RNTI

(LGE) Proposal 16:

  - Either NACK only based HARQ-ACK or UE specific ACK/NACK based HARQ-ACK is used for SPS PDSCH retransmission.

(LGE) Proposal 18:

  - For multiplexing HARQ-ACKs for dynamically scheduled multicast and unicast SPS, for multicast SPS and dynamically scheduled unicast, or for dynamically scheduled multicast and multicast SPS, UE determines a PUCCH resource based on one of the following options:

    - Option 1: The PRI of the last DCI with dynamic scheduling.

    - Option 2: SPS (e.g. if SPS is unicast)

    - Option 3: priority index

(LGE) Proposal 19:

  - For multiplexing HARQ-ACKs for multicast SPS and unicast SPS or for multicast SPS PDSCHs, UE determines a PUCCH resource based on one of the following options:

    - Option 1: the lowest SPS configuration index

    - Option 2: the highest priority of SPS between SPS configurations

    - Option 3: unicast SPS configuration (or multicast SPS configuration)

## Round-1 (closed)

“configuration for SPS multicast”

FL’s Comments

9 companies submitted proposals regarding HARQ-ACK for SPS multicast and wherein 4 companies discuss the SPS-config for SPS multicast including the configuration of the priority and PUCCH resources and 8 companies discuss the HARQ-ACK feedback modes for SPS multicast.

FL’s Proposal:

#### Proposal 5.1-1

For support of ACK/NACK based HARQ-ACK feedback for SPS multicast,

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast.

  - UE determines a priority index from the HARQ-ACK codebook index

  - Note: this is the same for SPS unicast for URLLC.

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

Collect views:

| Company | Comments |
| --- | --- |
| LG | RAN1 previously agreed to support HARQ-ACK feedback for SPS group-common PDSCH for MBS. However, RAN1 did not discuss whether to support ACK/NACK based HARQ-ACK for SPS PDSCH not scheduled by DCI. So, we first need to discuss whether HARQ-ACK for group common SPS PDSCH is based on NACK only and/or ACK/NACK. FL’s response: we agreed to support HARQ-ACK feedback for SPS and I suppose both PDSCH with and without PDCCH scheduling should naturally support at least ACK/NACK as it is the same as SPS unicast. How NACK-only is supported for SPS, it is new for this meeting and needs more discussion but I tend to make the discussion for dynamic scheduling regarding ACK/NACK and NACK-only clearer in previous sections before discussing NACK-only for SPS multicast, I think it should be understandable to companies. |
| CATT | OK with it. |
| ZTE | Ok with this proposal. |
| CMCC | Ok |
| Nokia, NSB | We agree with the purpose of the proposal. However, HARQ-ACK codebook index seems unclear. In our view, a multicast service can use the unicast PUCCH-config (thus different HARQ codebooks) for some services, and multicast PUCCH-config for some other. This gives gNB significant flexibility. We do not see that previous proposals prevent this. Moreover, some UEs can use NACK-only, and some others ACK / NACK. Those may be from different PUCCH-configs. Thus, for example, HARQ-ACK codebook with low priority can mean multicast codebook to be transmitted in ACK / NACK PUCCH-config or unicast PUCCH-config or NACK-only PUCCH-config with low priority. In our view, PUCCH-config index should be indicated to clarify the issue. FL’s response: “HARQ-ACK codebook index” please refer to 38213 section 9 and 38331 in sps-config, the same thing is used for URLLC SPS unicast. I updated the proposal to make the intention clearer to address you comment. We have the same understanding for unicast URLLC SPS. Low priority codebook (and corresponding PUCCH-config) would be index 0, whereas high priority would be 1. However, how do we now index a multicast codebook (and optional new PUCCH-config) with low priority for ACK/NACK and a unicast codebook (PUCCH-config) with low priority? |
| Spreadtrum | Generally we are fine |
| Vivo | Generally fine. Note that we have agreed that for multicast, UE can be optionally configured a separate PUCCH-config different from that of unicast. SPS-PUCCH-AN-List is included in PUCCH-config. We think it has been agreed that SPS-PUCCH-AN-List for multicast SPS can be separately configured. the proposal also include the case that PUCCH-config for multicast is configured, but SPS-PUCCH-AN-List is not configured in the PUCCH-config, UE will use the SPS-PUCCH-AN-List in PUCCH-config for unicast for all SPS. FL’s response: PUCCH-config is separately configured does not mean all fields have to be separately configured. You have different views or any suggestion how I should modify the proposal? |
| Samsung | OK with the proposal |
| OPPO | OK |
| FUTUREWEI | OK |
| Qualcomm | ok |
| Convida | Fine with the proposal. |
| Ericsson | Support. |
| NTT DOCOMO | We are fine with the proposal. |
| Lenovo, Motorola Mobility | OK. |
| FUTUREWEI2 | ok |

FL’s Comments

8 companies discuss the HARQ-ACK feedback modes for SPS multicast. ACK/NACK based feedback has been agreed for SPS multicast and then the question is whether NACK-only feedback is supported for SPS multicast. Since the HARQ-ACK feedback mode selection for dynamic scheduling is to be discussed, the NACK-only based feedback or the feedback mode selection for SPS multicast can be further studied.

FL’s Proposal:

#### Proposal 5.1-2

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS PDSCH with PDCCH scheduling and SPS deactivation.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with this proposal. |
| CATT | We prefer to postpone it. |
| ZTE | OK |
| CMCC | Ok |
| Nokia, NSB | Do not agree. We do not see any reason not to support NACK-only feedback for SPS (no opposition from any other company, too), since we support it for dynamic transmissions. NACK-only feedback is even better suited for SPS than for dynamic scheduling, since DCI missing occasions are not there for SPS PDSCHs (which is one of the drawbacks of NACK-only for dynamic scheduling). Therefore, we propose: NACK-only feedback is supported for SPS multicast. |
| Spreadtrum | Fine |
| vivo | We think for SPS PDSCH activation/deactivation, NACK only feedback should be supported considering that gNB can’t know it is NACK or DCI miss detection. For the SPS PDSCH rather than the first PDSCH after activation, NACK-only can be supported for saving PUCCH resource. |
| Samsung | Share a same opinion as Nokia – SPS PDSCH is a stronger use-case for NACK-only. |
| OPPO | OK |
| FUTUREWEI | Similar view as other companies: SPS PDSCH is a better use-case for NACK-only. |
| Qualcomm | We think NACK-only feedback is supported SPS GC-PDSCH without PDCCH, but not supported for SPS GC-PDSCH activation/deactivation. |
| Convida | We agree with Nokia that we can try to agree on supporting NACK-only feedback for SPS multicast instead of further studying it to make some progress. |
| Ericsson | Support. |
| NTT DOCOMO | NACK-only based feedback should be supported for SPS. |
| FL’s response | The proposal is updated to reflect basically the same comments above. |
| Lenovo, Motorola Mobility | OK. |
| Nokia, NSB | We support the proposal. |
| FUTUREWEI2 | Ok with updated proposal |
| OPPO1 | OK |
| Lenovo 2 | The update is not clear to us. What does ” SPS PDSCH with PDCCH scheduling” mean? |

## Round-2

#### Proposal 5.2-1

For support of ACK/NACK based HARQ-ACK feedback for SPS multicast,

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast.

  - UE determines a priority index from the HARQ-ACK codebook index

  - Note: this is the same for SPS unicast for URLLC.

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

Collect views:

| Company | Comments |
| --- | --- |
| Qualcomm | ok |
| LG | RAN1 recently agreed to support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling. Thus, we propose to change to: #### Proposal 5.2-1 For support of If ACK/NACK based HARQ-ACK feedback is supported and configured for SPS multicast, - the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast. - UE determines a priority index from the HARQ-ACK codebook index - Note: this is the same for SPS unicast for URLLC. - UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations. FL’s response: confused by the modification. ACK/NACK based for SPS has been agreed to support. The point of this proposal is for support of ACK/NACK, the following is xxx |
| Samsung | Support |
| CMCC | Ok |
| ZTE | OK |
| Lenovo, Motorola Mobility | OK |
| OPPO | OK |
| CATT | OK |
| NTT DOCOMO | Support |
| Apple | OK. |
| Nokia, NSB | We agree with the purpose of the proposal. However, HARQ-ACK codebook index still seems unclear. Thank you FL for your initial answer. We have the same understanding for unicast URLLC SPS. Low priority codebook (and corresponding PUCCH-config) would be index 0, whereas high priority would be 1. However, how do we now index a multicast codebook (and optional new PUCCH-config) with low priority for ACK/NACK and a unicast codebook (PUCCH-config) with low priority? So, assume that UE determines a priority index from the HARQ-ACK codebook index, as stated in the proposal, and HARQ-ACK codebook index 0 is indicated. How does the UE understand that the feedback should be sent in multicast low priority PUCCH-config (if configured) and which multicast PUCCH-config to use (ACK/NACK or NACK-only if separately configured), but not in the unicast low priority PUCCH-config? FL’s responses: Not sure I get the point. The proposal itself aims to solve how to indicate the priority index of HARQ-ACK for SPS, i.e., it is included in the SPS configuration. You can imagine it should at least include the case that only SPS multicast is configured but no SPS unicast. What you described here is more like how the codebook is generated or which resources is used for the codebook of SPS, which should be separately discussed. You also noted that even for dynamic scheduling, the discussion is still ongoing in section 2.1 |
| Ericsson | OK |

# PDSCH repetition

Submitted Proposals

(vivo) Proposal 1:

  - For PDSCH repetition of group-common PDSCH, if a UE can be configured with multiple g-RNTIs,

- The PDSCH aggregation factors for group-common PDSCHs with different g-RNITs should be separately configured.

(Nokia) Proposal 33:

  - For Config A, pdsch-AggregationFactor is per MBS service.

(Qualcomm) Proposal 10:

  - Support semi-static and dynamic slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

    - Repetition configuration for SPS GC-PDSCH can be different than that of dynamic GC-PDSCH.

(LGE) Proposal 20:

  - For a group common SPS configuration, UE can be optionally configured with either pdsch-AggregationFactor or TDRA table with repetitionNumber as part of the TDRA table.

## Round-1

FL’s Comments

Two proposals submitted in this section are regarding the repetition configurations for different multicast services and two proposals are regarding repetition of SPS multicast.

FL suggests discussing these issues later when RAN2 confirms supporting more than one multicast services and more companies are interested in the repetition of SPS multicast.

Collect views:

| Company | Comments |
| --- | --- |

# CSI feedback

Submitted Proposals

(ZTE) Proposal 15:

  - UE supports reporting multiple candidate {CQI, PMI, RI} sets in one CSI report for MBS

(ZTE) Proposal 16:

  - RAN1 further discusses the issues on CSI subband determination for MBS transmission.

    - If common frequency resource is defined as an MBS frequency region within the unicast BWP, mechanism to align different UE’s CSI subband size is needed.

    - If common frequency resource is defined as an MBS BWP, no additional mechanism is needed (i.e., just reusing the previous MBS mechanism).

(CATT) Proposal 22:

  - CSI feedback enhancement for MBS can be further studied and discussed but with low priority.

(Nokia) Proposal 34:

  - When using NACK-only based HARQ-ACK feedback along with CSI reporting, CQI measurements are done based on actual (time-averaged) BLER measurements at the UEs, rather than (instantaneous) CSI-RS based measurements.

(Nokia) Proposal 35:

  - New compact CSI report formats are defined for multicast transmission, where only a CQI or CQI along with an RI can be reported, and these formats are used in CSI reporting when NACK-only based HARQ-ACK feedback on group-common PUCCH resources is used.

(Nokia) Proposal 36:

  - The configuration for CQI reporting for PTM is extended to include not only the reliability target but also the number of HARQ transmissions per transport block after which the reliability target should be met.

(Qualcomm) Proposal 11:

  - For RRC_CONNNECTED UES, configure the CSI-RS resource per MBS CFR.

    - CSI-RS bandwidth is limited within the MBS CFR.

    - CSI-RS power is associated with GC-PDSCH power.

(Qualcomm) Proposal 12:

  - Support GC-PDCCH to trigger A-CSI-RS transmission in MBS CFR.

(Qualcomm) Proposal 13:

  - For RRC_CONNECTED UEs, support beam management for multicast assisted by unicast connection.

(Qualcomm) Proposal 14:

  - Consider SRS configuration for CSI measurement of multicast transmission in MBS CFR.

(Samsung) Proposal 10:

  - Support triggering of CSI reports by DCI scheduling MBS PDSCH or by MAC CE in MBS PDSCH.

(Ericsson) Proposal 19:

  - The existing Rel. 15/16 framework of CSI feedback is reused for multicast/PTM.

## Round-1

FL’s Comments

Since we have agreed that existing CSI feedback can be used for multicast transmission and whether enhancement is needed needs further study, only four companies have been proposing enhancements but what needs to be enhanced are quite different from the submitted proposals.

FL suggests continuing further study these issues for companies that are interested for the enhancements.

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | It will be good to discuss now – again FFS at this point practically means “no support”. In our opinion, new feedback modes/quantities are not needed, but triggering enhancements are needed (cannot be trigger CSI feedback on a per-UE basis and cannot have P-CSI configurations for all UEs with multicast service). |

# Other miscellaneous proposals

Submitted Proposals

(FUTUREWEI) Proposal 4:

  - For the same service, all RRC_CONNECTED UEs receiving multicast are configured with the same MCS table.

(FUTUREWEI) Proposal 5:

  - For MBS, FBRM should be used to improve performance for small sized TBs.

(CATT) Proposal 19:

  - To support multi-beam transmission in MBS, gNB can transmit same MBS data on all SSB beams.

(CATT) Proposal 20:

  - UE can receive MBS data from neighbor SSB-beam, and the soft-combination is used to improve the reliability of MBS receptions.

(Nokia) Proposal 10:

  - A new mechanism is adopted to disable HARQ-ACK feedback (and optionally CSI reporting) of the outlier UEs. This is down-selected from the following:

    - The gNB detects the outlier UEs and disables the HARQ-ACK feedback mechanism of those UEs.

    - The UE detects itself that it is an outlier UE (e.g. if the reliability criteria cannot be met for a specific amount of time) and disables its own HARQ-ACK feedback mechanism.

    - The UE detects itself that it is an outlier UE and sends a request to the gNB to disable its feedback (e.g. using a specific CQI value such as CQI 0).

(Nokia) Proposal 11:

  - The outlier UE re-activates its group-common HARQ-ACK feedback by using one of the following methods:

    - The gNB can assign dedicated ACK / NACK resources to the UE and / or the UE can keep reporting CSI feedback, so that the gNB can decide when to re-activate the NACK-only based HARQ-ACK feedback of the UE.

    - The UE can be configured with a QoS criterion, such as an average BLER, and by satisfying such a criterion, the UE can either directly re-activate its feedback mechanism, or request gNB to allow the UE to utilize the group-common HARQ-ACK resource.

(Nokia) Proposal 12:

  - When NACK-only group-common HARQ-ACK feedback is used, UEs can be configured to report NACKs that they sent but that were not honoured by the respective requested retransmission (e.g., proportion of missing DL retransmissions to corresponding number of NACKs sent by the UE).

    - This reporting is done separately per HARQ transmission index,

    - Sent periodically, only upon request from the gNB or event-triggered on the UE side, i.e., a UE sends a report when the NACK-miss rate for some HARQ transmission index is above a certain maximum tolerated error rate or when it crosses that error rate in either direction.

(Nokia) Proposal 13:

  - To reduce possible high inter-cell interference levels when using group-common NACK-only feedback, the UEs can be configured with a mechanism, where the UEs that satisfy the QoS criteria of the MBS service even though dropping of the corresponding transport block may refrain from sending a NACK feedback for that transport block, although they cannot decode the transmission successfully.

(Intel) Proposal 2:

  - For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

(LGE) Proposal 11:

  - For group common SPS activation/deactivation to multiple UEs in a group, (de)activation DCI can be repeated on multiple CORESETs with same TCI state or different TCI states.

(LGE) Proposal 12:

  - For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

(LGE) Proposal 13:

  - After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

    - The pre-determined time is determined by RRC and/or DCI.

(LGE) Proposal 21:

  - Discuss whether different TCI states can be configured for group common SPS received by different UE, e.g. different slots of group common SPS PDSCH repetitions or different SPS configurations can be associated to different TCI states for the same group of UEs.

(LGE) Proposal 24:

  - UE may not need to send HARQ-ACK feedback to all multicast transmissions within a CFR because UE is typically interested in only a few services. Further discuss whether/how to consider UE’s interest in multicast services for construction of HARQ-ACK codebook.

FL’s Comments

The proposals summarized in this section are either interested by only a single company or can be discussed in other section/agenda. Hence, FL does not plan to handle the proposals in this section in this meeting.

Collect views:

| Company | Comments |
| --- | --- |

# Suggested proposals for GTW

#### Proposal 2.2.3

At least NR supports the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability.

#### Proposal 2.3.3-1

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

#### Proposal  3.3.2-1 (for conclusion)

Support PUCCH resource for NACK-only that can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

#### Proposal 4.3.2

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, support Option 4 from the follow options:

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- Option 4: in option 3, if RRC signalling does not configure the function, the default mode (HARQ-ACK feedback is on or off) can be configured in the RRC signalling.

#### Proposal 2.3.3-2

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on one of the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection.

#### Proposal 5.2-1

For support of ACK/NACK based HARQ-ACK feedback for SPS multicast,

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast.

  - UE determines a priority index from the HARQ-ACK codebook index

  - Note: this is the same for SPS unicast for URLLC.

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

#### Proposal 2.6.3-2

- For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI.

# References

R1-2104196	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2104249	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2104337	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2104388	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2104443	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2104492	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

R1-2104551	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2104633	Discussion on reliability improvement	CMCC

R1-2104696	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2104760	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2104866	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2104929	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2105129	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2105337	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2105382	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2105438	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2105601	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2105648	Discussion on HARQ-ACK feedback for multicast of RRC_CONNECTED UEs	ETRI

R1-2105721	Discussion on HARQ-ACK feedback for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2105843	Discussion on reliability for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2105915	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

# Appendix Agreements summary for AI 8.12.2

## 102e

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

## 103e

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS details of HARQ-ACK codebook design.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

## 104e

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be,

- Lower, higher than or equal to the HARQ-ACK feedback for unicast

  - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE

  - FFS: The total number of priorities across multicast and unicast

- FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on   the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, support the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

- FFS: NACK-only based HARQ-ACK feedback for multicast,

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- FFS details.

Agreement:

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot.

  - FFS whether it is subject to UE capability.

- FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot.

- FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported.

- FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast.

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

- FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size.

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor.

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.

## 104be

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Agreement:

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;