3GPP TSG- RAN WG1 Meeting #105-e                         R1-2106120

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.3.3

Source:	Apple Inc.

Title:	Design of Rel-17 intra-UE multiplexing/prioritization

Document for:	Discussion/Decision

1	Introduction	2

2	Discussion on Rel-17 UCI multiplexing flow	3

3	PUCCH resource selection	9

3.1	Review on Rel-15/16 PUCCH resource set selection, PUCCH resource selection, PRB number adjustment, and CSI omission rule	9

3.2	PUCCH resource set selection procedure	12

3.3	PUCCH resource selection procedure	12

3.4	PRB number adjustment procedure	12

3.5	CSI omission procedure	19

4	PUCCH resource set selection in Rel-17	20

4.1	Determination of r1 and r2	21

4.2	Summary	21

5	Rel-17 PRB number adjustment/interlace number adjustment design	21

5.1	PRB number adjustment	21

5.1.1	Multiplexing of different UCI parts on the same modulation symbol	22

5.2	Interlace number adjustment	22

5.3	Summary	23

6	Omission rules in Rel-17	23

6.1	Design for UCI omission	23

6.2	Summary	24

7	Ordering of UCIs in UCI part 1 and UCI part 2	24

7.1	Discussion on CSI Part I and CSI Part II	25

8	Solution to fix inconsistent design in PUCCH resource set selection and UCI omission rule	26

8.1	Inconsistent design in Rel-15/16	26

8.1.1	Issue 1	26

8.1.2	Issue 2	26

8.2	Joint PUCCH resource set selection and PUCCH resource selection	27

8.2.1	Joint PUCCH resource set selection and PUCCH resource selection tackling Issue 1 and Issue 2	27

8.2.2	Joint PUCCH resource set selection and PUCCH resource selection without tackling Issue 1 and Issue 2	27

9	Discussion on UCI multiplexing over PUCCH with more than 2 UCI bits	28

9.1	Separate encoding HP HARQ-ACK and LP HARQ-ACK	30

10	Discussion on HARQ/SR multiplexing on PUCCH with a small payload size	32

10.1	HARQ/HARQ multiplexing: unified solution	32

10.2	SR/HARQ multiplexing: unified solution	33

10.3	Alternative solution for PUCCH format 1	35

10.3.1	Review on Rel-15 PUCCH Format 1 design	35

10.3.2	Potential modification on PUCCH format 1	36

10.3.3	HARQ-ACK and SR multiplexing with modified PUCCH Format 1 design	36

11	UCI multiplexing over PUSCH	37

11.1	Mapping of UCIs to UCI Parts over PUSCH	38

12	PUSCH resource determination (α and β) for UCI multiplexing	40

12.1	Rel-15/16 design review	40

12.1.1	BetaOffsets	40

12.1.2	BetaOffsets for CG PUSCH and DG PUSCH	41

12.1.3	Summary on Rel-15/16 design for BetaOffsets	42

12.1.4	Summary on Rel-15/16 design for alpha	42

12.2	Rel-17 design	43

13	PUCCH resources for SPS HARQ-ACK	46

14	Discussion on PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH	48

15	Simultaneous PUCCH and PUSCH transmissions	48

16	Conclusion	50

17	References	52

# Introduction

At RAN1 #104bis, the following agreements were agreed:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs.

- It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

In this contribution, we provide our views on UCI multiplexing design and related design from HARQ/CSI enhancements.

On UCI multiplexing over PUCCH:

- In Section 2, we discuss the UCI multiplexing flow in Rel-16 and Rel-17.

- In section 3, we review the Re-15/Rel-16 design in PUCCH resource set selection, PUCCH resource selection, PRB number adjustment and CSI omission. In Sections 4/5/6, we give our proposals on PUCCH resource set selection, PRB number adjustment and UCI omission in Rel-17. Some design issues are identified in Section 8 also, a joint PUCCH resource set/PUCCH resource selection is proposed overt here to address the raised issues.

- The order UCIs in UCI parts are treated in Section 7. And separate encoding design for PF2 is treated in Section 9. UCI multiplexing with a small payload size is treated in Section 10.

- SPS HARQ related issues are discussed in Section 13.

On UCI multiplexing over PUSCH:

- In Section 11, we discuss alternatives in where to map LP-HARQ ACK over PUSCH.

- In Section 12, We discuss how beta/alpha are loaded for PUSCH.

In Section 14, we discuss CG/DG PUSCH cancellation, and in Section 15, we discuss simultaneous PUCCH/PUSCH transmission.

# Discussion on Rel-17 UCI multiplexing flow

From the past RAN1 meetings on Rel-17 eIIoT/URLLC, we can see there can be some UE features coming out of the Rel-17 WI. As UCI multiplexing is complicated even in Rel-15, we need to ensure decision taken on each of them won’t in the end lead to big problems.

- In terms of the transmission of UCIs

  - sim Tx of PUCCH/PUSCH

  - PUCCH carrier switching

- In terms of UCI multiplexing

  - inter-priority multiplexing

  - PHY cancellation of DG PUSCH/CG PUSCH

- In terms of HARQ codebook construction

  - SPS HARQ deferral

  - retransmission of cancelled HARQ

  - Type 1 sub-slot HARQ codebook enhancement

  - SPS HARQ skipping

- In terms of the physical channel design of PUCCH:

  - PUCCH sub-slot repetition

  - PUCCH F0/F2 inter-sub-slot repetition

Fundamental to UCI multiplexing in Rel-17 is the decision whether the Rel-16  L1 priority-specific UCI multiplexing design is used as the baseline or a L1 priority-agnostic UCI multiplexing design similar to that in Rel-15 is used as the baseline.

Considering L1 priority based UCI prioritization and inter-L1 priority UCI multiplexing may need to be supported at the same time:

- The current agreements on intra-UE multiplexing cover HARQ/SR multiplexing only, CSI is excluded in the consideration hence UCI-type specific treatment with prioritization or multiplexing is assumed to be necessary

- Under some condition inter-L1 priority multiplexing may be infeasible due to processing time

- If the processing flow for multiplexing is to a large degree shared with that of prioritization, then both specification effort and implementation/test effort can be reduced.

With these considerations, we propose to study the UCI processing flow in Rel-17. Note some items included there (e.g. PUCCH carrier switching, Type 1 codebook enhancement, etc) are for inspection on their effect on the UCI multiplexing processing flow, their inclusion here does not suggest we find them to be essential to the Rel-17 eIIoT/URLLC work.

Figure 1 Rel-17 UCI multiplexing flow

Figure 2 details of Rel-16 UCI multiplexing flow

Figure 3 Alt. 1 of Rel-17 UCI multiplexing flow

In a first alternative (Alt. 1 as shown in Figure 3), only Step 3 in Figure 2 is replaced by new rules for inter-physical layer priority UCI multiplexing. It seems much complexity can arise with Alt. 1, and 4 examples (Case 1- Case 4) are provided below.

Figure 4 Problematic cases with Alt. 1

- In Case 1:

  - two HP PUCCHs (two HP PUCCH resources Z, which do not overlap with any HP PUSCH, PUCCH-2 and PUCCH-4 survive PUSCH multiplexing) overlap with a LP PUCCH ( a PUCCH resource Z, which does not overlap with any LP PUSCH, i.e. PUCCH-1 survives PUSCH multiplexing).

  - One possible rule is that LP UCI is multiplexed over the latest HP PUCCH. However, as the combined HP & LP UCI bits may exceed the capacity of PUCCH-4, it may result in choosing another HP PUCCH (4A) for multiplexing. However, allowing HP PUCCH to move around can be problematic:

    - PUCCH-4A may overlap with a HP PUSCH now, which would lead re-run PUSCH multiplexing procedure (Step 2 in Figure 3).

- In case 2/3/4:

When there are two choices for UCI multiplexing (PUCCH-2 or PUSCH-5), from the point of view of LP UCI, which one should be chosen becomes a question.

With the proposed flow, Case 1 can still arise, hence it is necessary to develop rules to handle case 1. It may be possible to include all PUCCHs, irrespective of their physical layer priorities, in Set Q, as proposed by some companies. Essentially the Rel-15 Q set to PUCCH resource Z procedure is modified to support inter-physical layer priority multiplexing. However, handling of PUCCH with LP CSI only and PUCCH with HP CSI only may arise quite early on in the multiplexing process, which can be avoided if LP PUCCH carrying the LP CSI (one of the LP PUCCH resource Z) and HP PUCCH carrying the HP CSI (one of the HP PUCCH resource Z) don’t overlap. Also considering the interaction between the inter-physical layer priority multiplexing feature and the simultaneous PUCCH/PUSCH transmission feature, if HP UCIs can be carried on one carrier, and LP UCIs can be carried on another carrier, it is beneficial to support Rel-17 inter-physical layer priority multiplexing as an add-on to the Rel-16 UCI multiplexing procedure, which handles PUCCH multiplexing separately for different physical layer priorities. From these considerations, we propose to consider the processing flow in Figure 5.

Note in Rel-16, periodic CSI over PUCCH and semi-persistent CSI over PUCCH are both at LP, hence they won’t appear in the HP PUCCH multiplexing flow. Considering Rel-17 URLLC CSI feedback enhancement is still under discussion, we can include its potential presence in the processing flow (marked in yellow in the flowchart).  PUSCH(s) with SP-CSI is of LP in the Rel-16 URLLC design. Also out of the consideration of Rel-17 URLLC CSI feedback enhancement, we consider PUSCH(s) with SP-CSI can be associated with HP in Rel-17 (also marked by yellow in the flowchart).

Some considerations on Alt. 2 as shown in Figure 5 are provided below:

- Consider it is not desirable to re-run PUSCH UCI multiplexing (Step 2 in Figure 3), it is important PUSCH UCI multiplexing at each priority is executed at most once, PUCCHs should be frozen when PUSCH UCI multiplexing starts.

- To achieve that, the UCI multiplexing flow from Rel-16 is changed by introducing “inter-physical layer priority PUCCH multiplexing” between regular PUCCH resource Z generation and PUSCH multiplexing (we call this “modified procedure for {Set Q —> resource Z}”

With the proposed flow, overlapping of PUCCHs at different priorities are handled already:

- The output of the procedure may generate a number of LP PUCCHs and a number of HP PUCCHs (i.e. no overlapping HP/LP PUCCHs in Set Q from the very beginning), there is no overlap among them.

- Just as in regular {Set Q —> Resource Z} procedure, no overlapping among PUCCHs is used as the stop criterion.

Figure 5 Alt. 2 of Rel-17 UCI multiplexing flow

Now more details are provided for Alt. 2:

- Alt. 2A: modifying the 38.213 Set Q —> PUCCH resource Z procedure:

  - Scan HP PUCCHs according to starting symbols:

    - For the earliest starting HP PUCCH (PUCCH-A):

      - Collect overlapping PUCCHs

      - Perform inter-L1 priority multiplexing which may result in a HP PUCCH (PUCCH-A’ ) different from PUCCH-A

    - Insert the new HP PUCCH (PUCCH-A’) back to Set Q.

    - Repeat the above procedure until there is no overlap among PUCCHs.

select the LP PUCCH with HARQ-ACK (or {HARQ-ACK, SR}, or {HARQ-ACK, SR and CSI}) if any for UCI multiplexing

If LP PUCCH has more than one UCI type, some can be multiplexed with HP PUCCH, some may be dropped by design (according to specification. E.g. LP PUCCH HARQ+CSI, CSI is dropped)

- Alt. 2B: modifying the 38.213 Set Q —> PUCCH resource Z procedure:

  - Scan HP PUCCHs according to starting symbols:

    - For the earliest starting HP PUCCH (PUCCH-A):

      - Collect overlapping PUCCHs

      - Perform inter-L1 priority multiplexing, if it may result in a UCI payload which exceeds the capacity of PUCCH-A, then the LP UCI is discarded (best effort treatment for LP UCI), alternatively discarding rule is defined in the UCI omission part, to ensure the capacity of PUCCH-A is never exceeded.

      - Repeat the above procedure until there is no overlap among PUCCHs.

We have

Proposal 2-1: Study alternatives with introducing an inter-L1 priority PUCCH multiplexing with PUCCH resources Z as inputs.

# PUCCH resource selection

## Review on Rel-15/16 PUCCH resource set selection, PUCCH resource selection, PRB number adjustment, and CSI omission rule

In both Rel-15 and Rel-16, multiple procedures are involved in PUCCH resource selection and PUCCH resource size determination. In the figure below, the PUCCH resource set selection, PUCCH resource selection, PRB number adjustment/interlace number adjustment, and CSI omission are shown.

Figure 6 Rel-15/16 NR design

Example of RRC configuration is shown in Tables to help discussion below.

Table 1 Example of PUCCH resource set configuration

|  | PUCCH resource set 0 | PUCCH resource set 1 | PUCCH resource set 2 | PUCCH resource set 3 |
| --- | --- | --- | --- | --- |
| PUCCH resource set index | pucch-ResourceSetId = 0 | pucch-ResourceSetId = 1 | pucch-ResourceSetId = 2 | pucch-ResourceSetId = 3 |
| PUCCH resources in a PUCCH resource set | resourceList (0,1,2,…,7) | resourceList (32,33,..,39) | resourceList (40,41,..,47) | resourceList (48,49,…,55) |
| maxPayloadSize |  | N2=maxPayloadSize1 | N3= maxPayloadSize2 |  |
| Applicable range | OUCI≤2 | 2<OUCI≤N2 | N2<OUCI≤N3 | N3<OUCI≤1706 |

Table 2 Example of PUCCH resources configurations indicated by PRI=0.

|  | PUCCH resource 0 | PUCCH resource 32 | PUCCH resource 40 | PUCCH resource 48 |
| --- | --- | --- | --- | --- |
| pucch-ResourceId | 0 | 32 | 40 | 48 |
| startingPRB | 10 | 0 | 2 | 6 |
| intraSlotFrequencyHopping |  | enabled | enabled | enabled |
| secondHopPRB |  | 80 | 50 | 76 |
| PUCCH format | PUCCH-format0: - initialCyclicShift: 6; - nrofSymbols: 2 - startingSymbolIndex: 11 | PUCCH-format4: - nrofSymbols: 4 - occ-Length: n4 - occ-Index: n3 - startingSymbolIndex: 0 | PUCCH-format2: - nrofPRBs: = 8 - nrofSymbols: 2 - startingSymbolIndex: 12 | PUCCH-format3: - nrofPRBs: = 10 - nrofSymbols: 4 - startingSymbolIndex: 9 |

Table 3 Example of PUCCH resource configurations indicated by PRI = 1

|  | PUCCH resource 1 | PUCCH resource 33 | PUCCH resource 41 | PUCCH resource 49 |
| --- | --- | --- | --- | --- |
| pucch-ResourceId | 1 | 33 | 41 | 49 |
| startingPRB | 11 | 1 | 10 | 16 |
| intraSlotFrequencyHopping |  | enabled | enabled | enabled |
| secondHopPRB |  | 81 | 58 | 86 |
| PUCCH format | PUCCH-format0: - initialCyclicShift: 6; - nrofSymbols: 2 - startingSymbolIndex: 11 | PUCCH-format4: - nrofSymbols: 4 - occ-Length: n4 - occ-Index: n3 - startingSymbolIndex: 0 | PUCCH-format3: - nrofPRBs: = 2 - nrofSymbols: 4 - startingSymbolIndex: 7 | PUCCH-format3: - nrofPRBs: = 8 - nrofSymbols: 5 - startingSymbolIndex: 8 |

Table 4 Example of maxCodeRate configurations at PF2/PF3/PF4 for HP PUCCH-Config

|  |  | PUCCH-Config (for HP PUCCH) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | format1 | format2 | format3 | format4 |
| With PUCCH-FormatConfig, Each parameter can be provided for each PUCCH format separately | interslotFrequencyHopping |  |  |  |  |
|  | additionalDMRS |  |  |  |  |
|  | maxCodeRate | N/A | zeroDot08 | zeroDot25 | zeroDot15 |
|  | nrofSlots |  |  |  |  |
|  | pi2BPSK |  |  |  |  |
|  | simultaneousHARQ-ACK-CSI |  |  |  |  |

In the example, PUCCH resource 40 and PUCCH resource 41 are under PUCCH resource set 2, note they can be configured with different maxCodeRate (0.08 and 0.25 respectively in the example).

Comparing Table 4 and Table 5, it is seen also that maxCodeRate for a given PUCCH format can be configured separately for each PUCCH-Config.

Table 5 Example of maxCodeRate at PF2/PF3/PF4 for LP PUCCH-Config

|  |  | PUCCH-Config (for LP PUCCH) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | format1 | format2 | format3 | format4 |
| With PUCCH-FormatConfig, Each parameter can be provided for each PUCCH format separately | interslotFrequencyHopping |  |  |  |  |
|  | additionalDMRS |  |  |  |  |
|  | maxCodeRate | N/A | zeroDot15 | zeroDot35 | zeroDot25 |
|  | nrofSlots |  |  |  |  |
|  | pi2BPSK |  |  |  |  |
|  | simultaneousHARQ-ACK-CSI |  | true | true | true |

## PUCCH resource set selection procedure

- Input: OUCI, maxPayloadSize for the second PUCCH resource set, maxPayloadSize for the third PUCCH resource set

Note: at a given physical layer priority, with PUCCH-Config (i.e. there is one PUCCH-Config for LP PUCCHs, and another PUCCH-Config for HP PUCCHs), A UE can be configured with up to 4 PUCCH resource sets for for UCI feedback including HARQ-ACK in response to a grant based PDSCH or a SPS release.

There are up to 32 PUCCH resources under the first PUCCH resource set, and up to 8 PUCCH resources under each of the configured PUCCH resource set. The second and the third PUCCH resource sets can be configured with a “maxPayloadSize".

Necessary simplification is made to make the NR design easier to understand, the full details can be found in Clause 9.2.1 of TS 38.213. We denote PUCCH format x as PFx, e.g. PF1 for PUCCH Format 1, PF3 for PUCCH format 3, etc.

- Process (refer to Clause 9.2.1 of T 38.213 for details):

For PUCCH resource set selection, OUCI bits (not including CRC bits) is compared with 2, maxPayloadSize for the 2nd PUCCH resource set, maxPayloadSize for the 3rd PUCCH resource set to select a PUCCH resource set. Essentially, 2, maxPayloadSize for the 2nd PUCCH resource set, maxPayloadSize for the 3rd PUCCH resource set and 1706 define 4 ranges of UCI bits: and each range corresponds to a PUCCH resource set.

- Output: selected PUCCH resource set

## PUCCH resource selection procedure

- Input: 1) Selected PUCCH resource set, 2) PRI included a latest DL DCI

- Process: the PRI included the DL DCI is used to select a PUCCH resource within the selected PUCCH resource set.

- Output: selected PUCCH resource

## PRB number adjustment procedure

Since in Rel-15, only PUCCH format 2 and PUCCH format 3 can have more than one PRB, to use PUCCH resource in a parsimonious a way, the number of PRBs in PUCCH format 2 and PUCCH format 3 can be adjusted according to the payload size, note the CRC bits are included in the consideration of PRB number adjustment, and further the allowable PRB number for PUCCH format 3 is limited to a number with prime factor(s) from 2, 3 and 5 as DFT-S-OFDM is used for PUCCH format 3.

- Input: the configured PRB number of the selected PUCCH resource (if it is at PF2 or PF3), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource

Note 1: all PUCCH resources of the same PUCCH format (PF2/PF3/PF4) under the same PUCCH-Config have the same maxCodeRate in Rel-15/16.

Note 2: OUCI, OCRC consist of two parts if two part CSI is reported. More details are provided in the next.

- Process: Choose the smallest PRB number which allows transmission of OUCI+OCRC with no greater coding rate than the PUCCH specific maxCodeRate according to the PUCCH format of the selected PUCCH resource. For PF3, the number of PRBs is restricted to have prime factor(s) out of 2, 3 and 5.

- Output: NRB,minPUCCH (the smallest PRB number meeting the transmission requirement over the selected PUCCH resource

To guide the Rel-17 design, we examine cases in Figure 6 one by one:

- Case 1:

  - The PRB number adjustment for HARQ-ACK only over PF2/PF3 procedure (Clause 9.2.3 of TS 38.213 v16.4.0) is captured below.

  - For interlaced transmission of PF2/PF3 as introduced in NR-U, interlace number adjustment is performed in a similar way to PRB number adjustment.

Interlace number adjustment for PF2/PF3 with interlaced transmission as introduced in NR-U.

- Case 2:

  - The PRB number adjustment procedure for HARQ-ACK+SR (Clause 9.2.5.1 of TS 38.213)  over PF2/PF3 is given below.

  - For interlaced transmission of PF2/PF3 as introduced in NR-U, interlace number adjustment is performed in a similar way to PRB number adjustment.

- Case 3:

  - For {HARQ-ACK+SR+WB/SB CSI over PF2, or HARQ-ACK+SR+WB CSI over PF3/PF4}, Clause 9.2.5.2 of TS 38.213, the description is more complex than Case 1 and Case 2.

  - First, parameters/notations are defined in Clause 9.2.5.2, which are common for Case 3 and Case 4. Note

    - OCSI and OCRC consist of two parts;

    - r  is the maxCodeRate associated with the PUCCH format of the selected PUCCH resource.

PRB number adjustment/interlace number adjustment (highlighted in yellow) and CSI omission rules (highlighted in cyan) are described in Clause 9.2.5.2:

- Case 4:

  - The text for HARQ-ACK+SR+SB CSI over PF3/PF4, Clause 9.2.5.2 of TS 38.213, is captured below.

  - Note the parameters are defined in the text excerpt under Case 3:

    - OCSI and OCRC consist of two parts;

    - r  is the maxCodeRate associated with the PUCCH format of the selected PUCCH resource.

In Rel-16, interlaced transmission of PUCCH is supported from the NR-U work item. For interlaced transmission of PF3, the interlace number adjustment and CSI omission are captured below:

From the review, we have

Observation 3-1: in Rel-16, for PRB number adjustment/interlace number adjustment, the configured PRB number of the  selected PUCCH resource (if it is at PF2 or PF3), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource,  OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, and the same coding rate is used for two CSI parts.

## CSI omission procedure

- Condition for applying CSI omission:

* Omission Case 1: If CSI consists of WB CSI or Part 1 only, CSI omission is performed for Part I for a PUCCH resource at PF2/PF3/PF4. The priority value formula in Clause 5.2.5 in TS 38.212 is used.

* Omission Case 2: If CSI consists of two Parts, then CSI omission is performed for Part I and Part II for a PUCCH resource for PF3/PF4. The priority value formula in Clause 5.2.5 in TS 38.212 is used, and Table 5.2.3-1 Priority reporting levels for Part 2 CSI, Clause 5.2.3 of TS 38.212 is used.

- Input: selected PUCCH resource (if it is at PF2 or PF3, PF4), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource

- Process:

* For Omission case 1: discard Part 1 of CSI report(s) until the the remaining payload and its corresponding CRC bits can be transmitted with no greater a coding rate than the PUCCH format specific maxCodeRate according to the PUCCH format of the selected PUCCH resource.

* For Omission case 2: discard Part 2 of CSI report(s), and if necessary, Part 1 of CSI report(s) until the the remaining payload and its corresponding CRC bits can be transmitted with no greater a coding rate than the PUCCH format specific maxCodeRate according to the PUCCH format of the selected PUCCH resource.

- Output:

* For Omission case 1: NCSIreported (the number of CSI reports surviving CSI omission).

* For Omission case 2: NCSI-part2reported (the number of CSI reports with un-omitted CSI part 2) or NCSI-part1reported (the number of CSI reports with un-omitted CSI part 1 when part 2 of all CSI reports are omitted.

For omission of Part II of CSI reports, the following two conditions are used in TS 38.213:

n=1NCSI-part2reported‍OCSI-part2,n+OCRC,CSI-part2,N≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH-⌈OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1/Qm⋅r⌉⋅Qm⋅rn=1NCSI-part2reported+1‍OCSI-part2,n+OCRC,CSI-part2,N+1>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH-⌈OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1/Qm⋅r⌉⋅Qm⋅r

Those two conditions can be reformulated as

n=1NCSI-part2reported‍OCSI-part2,n+OCRC,CSI-part2,N/(Qm⋅r)+⌈OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1/Qm⋅r⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NCSI-part2reported+1‍OCSI-part2,n+OCRC,CSI-part2,N+1/(Qm⋅r)+⌈OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1/Qm⋅r⌉>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

From the reformulated conditions, it can be seen roughly the same coding rate is required for part I and part II.

Observation 3-2:for CSI omission, selected PUCCH resource (if it is at PF2 or PF3, PF4), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, the same coding rate is used for two CSI parts.

# PUCCH resource set selection in Rel-17

From the previous review on Rel-16 design, it can be seen that 1) the coding rate of UCI payload plus CRC bits is not a factor considered in PUCCH resource set selection, 2) all PUCCH resources of the same PUCCH format (PF2 or PF3 or PF4) under the same PUCCH-Config have the same maxCodeRate in Rel-15/16.

Considering a key motivation to support separate encoding is to provide differentiated protection to HP HARQ-ACK and LP HARQ-ACK. With that, q1 HP HARQ-ACK pluse  q2  LP HARQ-ACK can require very different PUCCH resource in terms of number of coded bits compared to the case with q2 HP HARQ-ACK plus q1 LP HARQ-ACK. Hence the coding rates for them should be reflected in PUCCH resource set selection.

PUCCH resource set selection Alt. 1:

For PUCCH resource set selection, the following can be considered:

Ocombined=OUCI-part1+OUCI-part2⋅r1/r2,

where r1 is the maximum coding rate for UCI part I, and r2 is the maximum coding rate for UCI part II.

With the revised metric to look up PUCCH resource set, it is also clarified in Rel-17 the maxPayloadSize under PUCCH-ResourceSet corresponds to one of the configured coding rates, e.g. r1.

PUCCH resource set selection Alt. 2:

It is also possible to treat the configuration PUCCH resources/PUCCH resource sets as with a reference coding rate, e.g. rref, then depending on some condition(s), r1 and r2 are determined (e.g. r1 and r2 can be loaded with different values, e.g. depending on UCI combination, PUCCH format, number of PRBs in the PUCCH resource, PUCCH resource set, etc.), and assume the required numbers of coded bits don’t exceed the number of coded bits available at the PUCCH, i.e.

OUCI-part11r1+OUCI-part2⋅1r2≤Mref1rref,

where Mref is the maximum number of UCI bits supported over PUCCH resource set j, e.g. Mref is 2, N2=maxPayloadSize1, N3=maxPayloadSize2, 1706. Alternatively, Mref can be configured for all or some PUCCH resource set(s).

Ocombined=OUCI-part1rrefr1+OUCI-part2⋅rrefr2,

is used to look up a PUCCH resource set.

In one choice, the reference coding rate is configured for all or some PUCCH resource set(s).

In another choice, the reference coding rate is an order statistics of the maxCodeRate for PUCCH resources under a PUCCH resource set or PUCCH formats for PUCCH resources under a PUCCH resource set. In one example, the reference coding rate is the minimum/maximum/second smallest/second largest/... of all the maxCodeRate for PUCCH resources under a PUCCH resource set or PUCCH formats for PUCCH resources under a PUCCH resource set.

Then a generic formulation can be given as

Ocombined=OUCI-part1/r1+OUCI-part2/r2,

where r1=r1/rref, r2=r2/rref.

For both alternatives, Ocombined is checked with the range [1,2], [3,N2], (N2,N3], (N3,1706] for PUCCH resource set selection.

## Determination of r1 and r2

Now Multiple options are provided for determining r1 and r2:

Option 4-1) r1 and r2 can be configured for UCI Part 1 and UCI part 2 under the high priority PUCCH-Config;

Option 4-2a) r1 and r2 can be configured for UCI Part 1 and UCI part 2 for a PUCCH resource Set under the high priority PUCCH-Config; different PUCCH resource Sets may have different pairs of r1 and r2;

Option 4-2b) r1 and r2 can be configured for UCI Part 1 and UCI part 2 for a PUCCH format under the high priority PUCCH-Config;

Option 4-2c) r1 and r2 can be configured for UCI Part 1 and UCI part 2 for a PUCCH resource under the high priority PUCCH-Config;

## Summary

We have

Proposal 4-1: Ocombined=OUCI-part1+OUCI-part2⋅r1/r2  is used for PUCCH resource set selection.

# Rel-17 PRB number adjustment/interlace number adjustment design

## PRB number adjustment

In Rel-15, PRBs for a PUCCH are contiguous in the frequency domain. In Rel-16, under NR-U (NR Unlicensed Spectrum Access), interlaced transmission of PUCCH where PRBs for a PUCCH may not be contiguous. When two interlaces are configured for PUCCH for unlicensed spectrum access, the PRBs under one PUCCH may actually be contiguous if both interlaces are utilized. However, the resource adjustment is at interlace level, which is different from the PRB number adjustment as specified in Rel-15.

In Clause 9.2.5.2 of TS 38.213 (v16.5.0), two conditions (trigger condition & stop condition) are used for PRB number adjustment:

If (OACK+OSR+OCSI+OCRC)≤MRBPUCCH⋅Nsc,ctrlRBNsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the HARQ-ACk, SR and the NCSItotal CSI report bits by selecting the minium number MRB,minPUCCH of PRBs from the MRBPUCCH PRBs satifying (OACK+OSR+OCSI+OCRC)≤MRB,minPUCCH⋅Nsc,ctrlRBNsymb-UCIPUCCH⋅Qm⋅r as described in Clasuses 9.2.3 nand 9.2.5.1

According to the notations established in Clause 9.2.5.2 of TS 38.213,

• OCSI=n=1NCSItotal‍OCSI-part1,n+OCSI-part2,n, where OCSI-part1,n is a number of Part 1 CSI reports bits for CSI report with priority value n, OCSI-part2,n is a number of Part 2 CSI report bits, if any, for CSI report with priority value n [6, TS 38.214], and NCSItotal is a number of CSI reports that include overlapping CSI reports.

• OCRC=OCRC,CSI-part1+OCRC,CSI-part2, where OCRC,CSI-part1 is a number of CRC bits, if any, for encoding HARQ-ACK, SR and Part 1 CSI report bits and OCRC,CSI-part2 is a number of CRC bits, if any, for encoding Part 2 CSI report bits.

Then the condition to trigger PRB number adjustment in Rel-16 can be reformulated as

(OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1)/(Qm⋅r)+(n=1NCSItotal‍OCSI-part2,n+OCRC,CSI-part2)/(Qm⋅r)≤MRBPUCCH⋅Nsc,ctrlRBNsymb-UCIPUCCH, essentially the sum of the number of modulation symbols required for UCI Part I and the number of modulation symbols frequired for UCI Part II does not exceed exceed the number of modulation symbols with the maximum available number PRBs on the selected PUCCH.

We propose to consider different coding rates for UCI Part I and UCI Part II in Rel-17 for PRB number adjustment.

Then for Rel-17 design, the condition to trigger PRB number adjustment can be given by

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

And similarly, the stop condition for PRB number adjustment can be modified in Rel-17 as follows:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRB,minPUCCH⋅Nsc,ctrlRB,min⋅Nsymb-UCIPUCCH

### Multiplexing of different UCI parts on the same modulation symbol

In Rel-15/16, CSI parts are not multiplexed on the same modulation symbol.

In Rel-17, even though logically it is also possible to allow different UCI parts to be carried on the same modulation symbol, it is good to keep the same design principle such that different UCI parts are not multiplexed on the same modulation symbol.

To be more consistent with the UCI omission rules to be discussed in the next, the condition for PRB number adjustment, can be tightened a little bit by taking the ceiled value of the second item:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm

And the stop condition for PRB number adjustment can be modified in Rel-17 as follows:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/r1⌉≤MRB,minPUCCH⋅Nsc,ctrlRB,min⋅Nsymb-UCIPUCCH⋅Qm

## Interlace number adjustment

For interlace number adjustment, instead of performing PRB number adjustment for potentially many PRB number candidates, only two PRB number candidates are examined: one corresponds to the number of PRBs in the first interlace, another corresponds to the number PRBs in the first and second interlace. The same consideration provided in the previous section applies.

Further details can be found below:

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

A joint PUCCH resource set selection and PUCCH resource selection can be also conducted.

## Summary

In this section, we treat PRB number adjustment in Rel-17. We have

Proposal 5-1: considering coding rates of UCI parts in the PRB number adjustment/interlace number adjustment.

Proposal 5-2: The condition to trigger PRB number adjustment is given by

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

And  the stop condition for PRB number adjustment  is as follows:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRB,minPUCCH⋅Nsc,ctrlRB,min⋅Nsymb-UCIPUCCH

# Omission rules in Rel-17

## Design for UCI omission

Assume different coding rate targets for UCI part I and UCI part II, instead of a single coding rate target r, we have r1 for the maximum coding rate for UCI part I, r2 for the maximum coding rate for UCI part II, the following conditions are used for UCI omission:

n=1NUCI-part2reported‍OUCI-part2,n+OCRC,UCI-part2,N/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part2reported+1‍OUCI-part2,n+OCRC,UCI-part2,N+1/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

If all UCI part II is dropped, then the the following is examined to determine remaining UCIs in UCI part I:

n=1NUCI-part1reported‍OUCI-part1,n+OCRC,UCI-part1,,N/(Qm⋅r1)≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part1reported+1‍OUCI-part1,,n+OCRC,UCI-part1,N+1/(Qm⋅r1)>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

## Summary

We have

Proposal 6-1: Consider different coding rates for UCI parts in UCI omission rule.

Proposal 6-2: With two UCI parts, the omission rule is as follows:

the following conditions are used for UCI omission:

n=1NUCI-part2reported‍OUCI-part2,n+OCRC,UCI-part2,N/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part2reported+1‍OUCI-part2,n+OCRC,UCI-part2,N+1/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

If all UCI part II is dropped, then the the following is examined to determine remaining UCIs in UCI part I:

n=1NUCI-part1reported‍OUCI-part1,n+OCRC,UCI-part1,,N/(Qm⋅r1)≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part1reported+1‍OUCI-part1,,n+OCRC,UCI-part1,N+1/(Qm⋅r1)>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH.

# Ordering of UCIs in UCI part 1 and UCI part 2

Here we consider CSI reporting is for either HP or LP. The order of UCI in a UCI part defines the order by which UCIs are omitted.

For the case where HP CSI is present:

On UCI part 1:

HP HARQ-ACK (including potentially more than one HARQ codebook), HP SR, HP CSI-Part I

On UCI part 2:

HP CSI-part II, LP-HARQ-ACK, (LP-SR)

Hence on UCI Part 2, (LP-SR) if supported by specification and present, is omitted first, LP-HARQ-ACK (including potentially more than one HARQ codebook) is omitted second (if more than one HARQ codebook is present, then the last placed HARQ codebook is omitted/compacted first for example), then HP CSI-part II can be omitted in a similar way as CSI part II in Rel-16.

For the case LP CSI is present:

On UCI part 1:

HP HARQ-ACK (including potentially more than one HARQ codebook), HP SR, LP CSI-Part I

On UCI part 2:

LP-HARQ-ACK, (LP-SR), LP CSI-part II

## Discussion on CSI Part I and CSI Part II

In Rel-15, the introduction of two part CSI feedback was motivated by the payload size variation for the reported rank: CSI Part I provides sufficient information for gNB to determine the size of CSI part II, so blind detection with respect to different ranks is avoided. Note in Rel-15, single part CSI reports are included in CSI Part I even though for a single part CSI report there is no CSI part I in the CSI report. Considering in Rel-17 the coding rate for UCI part I can be different from that for UCI part II, hence there may be a motivation to move all non-essential UCIs to UCI part II. Specifically, if a CSI report consists of a single part, then it can be carried to UCI Part II. However, if the single part CSI report is at HP, then it may be still desirable to carry the HP CSI report in UCI Part I.

Assume there are cases where the UCI payload may exceed the capacity of a PUCCH, if the following priority rule is used (note not all UCI types may be present in a particular scenario, nor all the combinations need to be supported by specification):

HP HARQ-ACK > HP SR > (HP CSI) > LP HARQ-ACK > (LP SR) > (LP CSI)

With the above prioritization rule, then CSI omission rule as captured in TS 38.214 may need to run twice: it is first run for LP CSI, and after omitting or dropping all LP CSI, LP SR and LP HP-HARQ-ACK, part of HP CSI needs to be omitted secondly. In Figure 5-14, it shows that the CSI priority rule is applied separately to HP CSI and LP CSI, then (parts of) CSI reports within H-CSI and CSI reports within L-CSI are separately dropped, so the CSI priority rules are run twice, one for HP CSI another for LP CSI.

Alternatively, LP CSI is dropped when HP CSI is present to avoid running CSI omission rules twice: there are two choices: with choice 1, LP CSI is dropped on UCI Part II only, with choice 2, LP CSI is dropped from UCI part I and UCI II all together. Then for UCI Part II or for both UCI Parts I and II,

when HP CSI is present, only the following UCIs are carried in UCI part I and part II:

HP HARQ-ACK > HP SR > (HP CSI) > LP HARQ-ACK > (LP SR)

When HP CSI is absent, only the following UCIs are carried UCI part I and part II

HP HARQ-ACK > HP SR > LP HARQ-ACK > (LP SR) > LP CSI

Figure 8-1 separate CSI treatment to physical layer priority

Figure 8-2 separate CSI treatment

We have:

Proposal 7-1

when HP CSI is present, only the following UCIs are carried in UCI part I and part II:

HP HARQ-ACK > HP SR > (HP CSI) > LP HARQ-ACK > (LP SR)

When HP CSI is absent, only the following UCIs are carried UCI part I and part II

HP HARQ-ACK > HP SR > LP HARQ-ACK > (LP SR) > LP CSI

# Solution to fix inconsistent design in PUCCH resource set selection and UCI omission rule

## Inconsistent design in Rel-15/16

### Issue 1

It can be seen that in Rel-15/16, PUCCH resource set selection design and CSI omission design are not consistent: CRC bits are not used in PUCCH resource set selection but CRC bits are considered in the CSI omission rule.

Hence it may happen PUCCH resource set 2 is selected according to OUCI bits, then later it is found that CSI omission is triggered over the indicated PUCCH resource within PUCCH resource set 2; it may happen a counterpart PUCCH in PUCCH resource set 3 has enough capacity to carry the UCI bits plus CRC bits without triggering CSI omission. If the CRC bits were considered in the first place in PUCCH resource set selection, then PUCCH resource set 3 could have been selected and then un-necessary CSI omission could be avoided in Rel-15/16.

One could argue such a CSI omission issue does not amount to much problem in Rel-16: as aperiodic CSI can be also triggered if gNB needs fresh CSI. However, from the agreement reached in RAN1 #104bis-e, LP HARQ-ACK can be carried over UCI part II, it would be undesirable to drop/or compact LP HARQ-ACK if the inconsistent design from Rel-15/16 is inherited in Rel-17.

### Issue 2

UCI payload’s Coding rate is not a factor in PUCCH resource set, but it is a factor in PRB number adjustment and CSI omission.

The variation in PUCCH capacity within a PUCCH resource set is not accounted in PUCCH resource set selection.

## Joint PUCCH resource set selection and PUCCH resource selection

In Rel-15/16, PUCCH resource takes two steps: in the first step, OUCI bits is used to look up a PUCCH resource set. Actually the capacity of PUCCH resources within a PUCCH resource set varies, e.g. PUCCH-11 and PUCCH-12 are both under PUCCH resource set 1, but PUCCH-11 supports up to 10 bits for payload while PUCCH-12 supports up to 20 bits, such variation in capacity may not be reflected by the PUCCH resource set’s maxPayloadSize; in the second step, PRI from a DL DCI is used to select a PUCCH resource within the selected PUCCH resource set in the first step.

From two issues discussed above, and also from the consideration on rref and Mref under PUCCH resource set selection Alt. 2, where additional RRC signaling or rules to determine rref and Mref are needed, a simplified solution is proposed which does joint PUCCH resource set selection and PUCCH resource selection, and does not require introduction of new RRC parameters for rref and Mref.

A simpler approach, compared to the solution provided in Sections 4 to Section 6, would be to use the PRI to look up a PUCCH resource matching the PRI from each PUCCH resource set, then among those PUCCH resources with a capacity no smaller than OUCI or OUCI+OCRC, then the the PUCCH resource with the least capacity is chosen; in the case where none of those PUCCH resources has a capacity no smaller than OUCI or OUCI+OCRC, then the PUCCH resource with the largest capacity is chosen.

### Joint PUCCH resource set selection and PUCCH resource selection tackling Issue 1 and Issue 2

And for selecting a PUCCH resource from a number of PUCCH resource sets, in each PUCCH resource set, the PUCCH resource of an index as indicated by the PRI is checked one by one with the following condition:

⌈n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)⌉+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

or

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

Note in either case, the CRC bit length is considered on PUCCH resource determination. To avoid chicken-egg problem, it can be also assumed for information bits no smaller than 360, then L=11 is assumed.

### Joint PUCCH resource set selection and PUCCH resource selection without tackling Issue 1 and Issue 2

To be more consistent with the Rel-16 design, another choice is to exclude CRC bit length in PUCCH resource determination and the formulas can be modified by removing OCRC,UCI-part2 and/or OCRC,UCI-part1

⌈n=1NUCI-part2total‍OUCI-part2,n/(Qm⋅r2)⌉+⌈n=1NUCI-part1total‍OUCI-part1,n/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

or

n=1NUCI-part2total‍OUCI-part2,n/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

From the discussion above, it can be seen with joint PUCCH resource set selection and PUCCH resource selection, there is no need to introduce PUCCH resource set-specific coding rates for UCI part I and UCI Part II. As in Rel-15/16, the maxCodeRate is configured per PUCCH format, then the enhancement in Rel-17 can be the introduction of two maxCodeRates per PUCCH format or per PUCCH resource (i.e. the two options provided in Section 4: 4-2b and 4-2C).

We have

Proposal 8-1: consider joint PUCCH resource set selection and PUCCH resource selection.

Proposal 8-2: if joint PUCCH resource set selection and PUCCH resource selection is supported, r1 and r2 are configured per PUCCH format or per PUCCH resource.

# Discussion on UCI multiplexing over PUCCH with more than 2 UCI bits

In NR design, UCI multiplexing over PUCCH considers reliability of OFDM symbols in PUCCH, OFDM symbols are grouped in up to 3 groups, then depending whether the size of UCI part I and UCI part II, the resources are determined.

Figure 9-1 UCI mapping for UCI part I in Rel-15

Given the complex nature of the design, it is not desirable to reopen the design on all fronts, and then Rel-15 design can be should retained when possible.

Figure 9-2 Rel-15 design

In Rel-15:

- In no CSI report is not of two parts, UCI multiplexing for HARQ-ACK, SR and CSI are jointly encoded.

- If at least one CSI report is of two parts, then CSI part 2 is separated encoded, and the rest are jointly encoded.

We see the key design principle to follow for Rel-17 URLLC design is to keep the design as close as possible to the Rel-15 design.

Figure 9-3 UCI mapping over PUCCH for two-part CSI in Rel-15/16

The mapping rules for CSI Part I and CSI part II over PUCCH are specified in TS 38.212 in Rel-15/16. To facilitate the discussion in the following, we denote the resources taken by CSI Part I and CSI part II as UCI Part I and UCI part II respectively. Then by creating mapping rules as proposed subsequently, it is intended to a large degree the mapping rules for CSI Part I, CSI part II as captured in Clause 6.3.1 of TS 38.212 can be reused for UCI part I and UCI Part II.

Note in Clause 6.3.1.1.3 of TS 38.212, two sequences are defined for the case with two part CSI for the case shown in the red box in Figure 2:

“If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and , according to the following, where  and ”.

With proposed solutions below,  and  are populated differently.

Similarly, one UCI bit sequence is generated, for cases shown in the blue box in Figure 2, then the discussion is above how to populate the UCI bit sequence.

In NR, 5 PUCCH formats are specified, the support of joint encoding and separate encoding of UCIs is shown below.

Table 9-1 Separate encoding and joint encoding with PUCCH formats in Rel-16

|  | PUCCH Format 2 | PUCCH Format 3 | PUCCH Format 4 |
| --- | --- | --- | --- |
| Joint encoding of UCIs | X | X | X |
| Separate encoding of UCIs |  | X | X |

With PUCCH Format 0, joint encoding of HARQ-ACK and SR is supported.

With PUCCH Format 1, channel selection is used in Rel-15/16 to support the signaling of SR and HARQ-ACK, yet on the PUCCH itself, only HARQ-ACK is carried. Partly for that reason, there has been diverse view to support HP/LP UCIs over PUCCH 1.

Over PUCCH Format 2, two part CSI is not supported in Rel-15, consequently in Rel-15/16, only joint encoding of UCIs is supported over PUCCH format 2.

## Separate encoding HP HARQ-ACK and LP HARQ-ACK

With separate encoding, the following mapping can be considered.

Figure 9-4 Separate encoding for HP HARQ-ACK and LP HARQ-ACK

As noted previously, in Rel-15 UCI part I and UCI part II roughly have the same coding rate. One argument for separate encoding is that different coding rates can be applied to HP HARQ-ACK and LP HARQ-ACK.

Note in a particular case or the final design, not all the UCIs are present or can be present. CSI feedback can be assumed at LP. If Rel-17 inter-L1 priority UCI multiplexing is limited to HARQ-ACK and SR, then CSI part I and CSI part II are both absent in the UCI multiplexing as shown in the figure below.

Figure 9-5 Separate encoding for HARQ-ACK/SR only

Figure 9-6(a) UCI mapping for separate encoding of HP/LP HARQ-ACK (1), RUCImax=r1

Figure 9-6(b) UCI mapping for separate encoding of HP/LP HARQ-ACK (2), RUCImax=r1

Proposal 9-1: leverage the Rel-15 design, LP HARQ-ACK is  mapped to UCI Part II in separate encoding, adopt the UCI mapping in Figures 9-6a/9-6b.

At RAN1 #104bis, the following agreements were agreed:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

As in Rel-15/16, PUCCH Format 2 does not support two part CSI, only joint encoding of UCIs is supported.  We have the following proposal regarding the support of HP HARQ-ACK/LP HARQ-ACK multiplexing over PUCCH format 2:

Generating two encoded sequences for HP-ACK (with r1) and LP-ACK (with r2) separately and then concatenating those two encoded sequences into one encoded sequence , UCI mapping is not changed for PUCCH Format 2.

We have

Proposal 9-2:

Consider for PUCCH format 2 to support multiplexing of HP UCI(s) and LP UCI(s):

generating two encoded sequences for HP-ACK (with r1) and LP-ACK (with r2) separately and then concatenating those two encoded sequences into one encoded sequence, UCI mapping is not changed for PUCCH Format 2.

# PUCCH resources for SPS HARQ-ACK

Figure 10-1 Feedback resource determination for SPS HARQ-ACK in Rel-15 and Rel-16

- In Figure 10-1, SPS HARQ-ACK only multiplexing over PUCCH for CSI is shown, the flow chart is generated according to TS 38.213.

- The blue box shows there is no PRB # adjustment for PF2/PF3 when a single CSI report’s PUCCH is selected for SPS HARQ multiplexing.

- The yellow box shows there is no PRB # adjustment for PF2/PF3, but CSI omission rules are used.

Figure 10-2 Proposed design for PUCCH resource determination SPS HARQ-ACK

We can consider a few use cases for SPS HARQ-ACK feedback:

- Use case 1:

  - If S/SP CSI targeting URLLC traffic as discussed currently in Rel-17, can be motivated & specified, then HP S/SP CSI’s PUCCH can be also considered as container for cross-priority HARQ-ACK/SR multiplexing. In this case, cross-priority HARQ multiplexing over PUCCH for CSI can happen:

    - LP SPS HARQ-ACK + HP S/SP CSI

- User case 2:

  - Even if S/SP CSI is still always at LP in Rel-17, the payload size range may swing more widely in Rel-17 than in Rel-16, with SPS HARQ-ACK deferral.

  - There is no agreement yet in RAN1 on the PUCCH resource for SPS HARQ-ACK deferral:

    - Several options can be considered:

      - DG HARQ-ACK’s PUCCH resource sets/PUCCH resources, but the lack of PRI indication may make such a design less attractive from network operation of view;

      - SPS HARQ-ACK’s own PUCCH resources, then depending on the presence/absence of SPS HARQ-ACK(without deferral) and SPS HARQ-ACK with deferral due on the current slot, the combined payload size can swing widely.

      - Dedicated PUCCH resources for SPS HARQ-ACK deferral.

With the considered use cases, we propose to consider the flow chart in Figure 29.

- SPS HARQ deferral is treated as a new input for the UCI multiplexing.

- The blue box can be modified to use CSI omission rules and HARQ compaction rules.

- The yellow box already supports CSI omission rules:

  - Consider the payload size fluctuation in Rel-17 is much more than in Rel-15, over-provision of PF2/PF3 may be necessary, from that PRB # adjustment may become necessary also as when SPS HARQ-ACK deferral is not present, the payload can be smaller.

Proposal 10-1: consider PRB # adjustment and CSI omission/HARQ compaction for the PUCCH resource under multi-CSI-PUCCH-ResourceList and pucch-CSI-ResourceList.

# UCI multiplexing over PUSCH

UCI multiplexing over PUSCH in Rel-15 is quite complex already:

- Depending on the number of HARQ-ACK bits, puncturing or rate matching is used for HARQ-ACK bits.

- As PUSCH does not carry SR, the UCI types are limited to HARQ-ACK and CSI. When both HARQ-ACK and CSI feedback are present, then a three-part UCI multiplexing scheme is used.

Given the complexity of the existing NR design, it is beneficial to reuse the existing design as much as possible. One can treat the CSI encoding/multiplexing almost as functions in a programming language, then with proper adaption of the inputs to these functions, the Rel-17 design can be obtained. Note however, some details still need to be worked out.

Figure 11-1 Mapping of UCIs to 3 parts in UCI multiplexing over PUSCH in Rel-15/16

The mapping rules for HARQ-ACK, CSI Part I and CSI part II over PUSCH are specified in TS 38.212 in Rel-15/16, e.g. in Clause 6.2.7 (Data and control multiplexing, TS 38.212).

The inputs to the mapping rules are defined in Clause 6.2.7:

“Denote the coded bits for UL-SCH as .

Denote the coded bits for HARQ-ACK or jointly coded bits for HARQ-ACK and CG-UCI when the high layer parameter cg-UCI-Multiplexing is configured, if any, as .

Denote the coded bits for CSI part 1, if any, as .

Denote the coded bits for CSI part 2, if any, as .

Denote the coded bits for CG-UCI without HARQ-ACK, if any, as g0CG-UCI,  g1CG-UCI,  g2CG-UCI, g3CG-UCI, …, gGCG-UCI-1CG-UCI.

Denote the multiplexed data and control coded bit sequence as . “

We denote the resources taken by HARQ-ACK and/or CG-UCI ( or ) , CSI Part I () and CSI part II () as UCI Part 0, UCI Part I and UCI part II respectively. Then by creating mapping rules subsequently, it is intended to a large degree the mapping rules for HARQ-ACK, CSI Part I, CSI part II as captured as a 6 step procedure in Clause 6.2.7 of TS 38.212 can be reused for UCI Part 0, UCI part I and UCI Part II.

To avoid cumbersome description, we may drop mention of CG-UCI in the following. It is understood that CG-UCI is mapped to UCI Part 0.

With proposed solutions below, UCIs in UCI Part 0 populate a sequence in replacement of , UCIs in UCI Part I populate a sequence in replacement of , UCIs in UCI Part II populate a sequence in replacement of .

## Mapping of UCIs to UCI Parts over PUSCH

Figure 11-2 Separate encoding of HP HARQ-ACK and LP HARQ-ACK

With the goal to reuse the Rel-15 design, LP HARQ-ACK can be mapped to UCI Part I or UCI Part II:

- If mapped to UCI Part I (shown by the blue solid line), then the encoding sequence is CSI Part I > LP HARQ-ACK if CSI Part I contains HP CSI, otherwise the encoding sequence is LP HARQ-ACK> CSI Part I.

- If mapped to UCI Part II (shown by the red dashed line), then the encoding sequence is CSI Part II > LP HARQ-ACK if CSI Part II contains HP CSI, otherwise LP HARQ-ACK> CSI Part II can be used.

Note even on LP PUSCH, LP HARQ-ACK is bumped to Part I or Part II once HP HARQ-ACK is present.

To avoid complicated CI omission rules, with a consideration similar to that on UCI multiplexing on PUCCH, LP CSI can be dropped once HP CSI is present.

LP HARQ-ACK mapping Alt. 1:

Benefit of mapping LP HARQ-ACK to UCI part I:

It is reasonable to assume UCI part I tends to be better protected than UCI part II. Mapping LP HARQ-ACK to UCI Part I gives better protection to LP HARQ-ACK than mapping LP HARQ-ACK to UCI Part II.

LP HARQ-ACK mapping Alt. 2:

Benefit of mapping LP HARQ-ACK to UCI part II:

- Since in Rel-15 CSI omission is applied on UCI PART II, to minimize specification impact and change in implementation for Rel-17, LP HARQ-ACK can be mapped to Part II all the time irrespective of whether LP PUSCH or HP PUSCH carries mixed UCIs including LP-HARQ-ACK. With that, tasks of CSI omission and HARQ-ACK omission/compaction can be conducted on a single UCI part instead of being spread to multiple UCI parts. Hence it may happen HP HARQ-ACK is mapped to Part 0, Part I is empty, LP HARQ-ACK is over Part II, which should not be a problem from specification or implementation point of view.

- Considering there may be important HP AP CSI carried in UCI part I, mapping LP HARQ-ACK into UCI Part II may have less impact to HP UCIs than mapping LP HARQ-ACK into UCI Part I.

Table 11-1 UCI multiplexing over HP PUSCH

|  | HP CSI is present |  | HP CSI is absent |  |
| --- | --- | --- | --- | --- |
|  | HP PUSCH (Alt. 1) | HP PUSCH (Alt. 2) | HP PUSCH (Alt. 1) | HP PUSCH (Alt. 2) |
| UCI Part 0 | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK |
| UCI Part I | HP CSI Part I, LP HARQ-ACK | HP CSI Part I | LP HARQ-ACK |  |
| UCI Part II | HP CSI Part II | HP CSI Part II, LP-HARQ-ACK |  | LP-HARQ-ACK |
| Note 1: if HP CSI consists of a single part, then HP CSI part II is absent Note 2: If CG-UCI is present, it is carried in UCI Part 0 |  |  |  |  |

Table 11-2 UCI multiplexing over LP PUSCH

|  | HP CSI is present |  | Both HP CSI and LP CSI are absent |  | HP CSI is absent, and LP CSI is present. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LP PUSCH (Alt. 1) | LP PUSCH (Alt. 2) | LP PUSCH (Alt. 1) | LP PUSCH (Alt. 2) | LP PUSCH (Alt. 1) | LP PUSCH (Alt. 2) |
| UCI Part 0 | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK |
| UCI Part I | HP CSI Part I, LP HARQ-ACK | HP CSI Part I | LP HARQ-ACK |  | LP HARQ-ACK, LP CSI Part I | LP CSI Part I |
| UCI Part II | HP CSI Part II | HP CSI Part II, LP-HARQ-ACK (discard LP-HARQ-ACK first) |  | LP-HARQ-ACK | LP CSI Part II | LP-HARQ-ACK, LP CSI Part II, (discard LP CSI first) |
| Note 1: if HP CSI consists of a single part, then HP CSI part II is absent. Note 2: If CG-UCI is present, it is carried in UCI Part 0 |  |  |  |  |  |  |

Figure 11-3 Separate UCI multiplexing choice

Figure 11-4 Separate UCI multiplexing choice

We have

Proposal 11-1: Between two options of mapping LP HARQ-ACK into UCI Part I and UCI Part II, RAN1 select one of them consistently across multiplexing scenarios.

Proposal 11-2: Adopt Alt. 1 or Alt. 2 design from Tables 11-1 and 11-2.

# PUSCH resource determination (α and β) for UCI multiplexing

## Rel-15/16 design review

### BetaOffsets

The RRC configuration for BetaOffsets from TS 38.331 is shown below:

BetaOffsets ::= SEQUENCE {

betaOffsetACK-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetACK-Index2 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetACK-Index3 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index2 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index2 INTEGER(0..31) OPTIONAL-- Need S

}

BetaOffsets field descriptions:

betaOffsetACK-Index1

Up to 2 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index2

Up to 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index3

Above 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetCSI-Part1-Index1

Up to 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part1-Index2

Above 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index1

Up to 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index2

Above 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

### BetaOffsets for CG PUSCH and DG PUSCH

ConfiguredGrantConfig ::= SEQUENCE {

...

uci-OnPUSCH SetupRelease { CG-UCI-OnPUSCH } OPTIONAL, -- Need M

...

betaOffsetCG-UCI-r16 INTEGER (0.. 31) OPTIONAL, -- Need R

...

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic BetaOffsets

}

Note on “uci-OnPUSCH”

- Selection between and configuration of dynamic and semi-static beta-offset.

- For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic.

betaOffsetCG-UCI

Beta offset for CG-UCI in CG-PUSCH, see TS 38.213 [13], clause 9.3

PUSCH-Config ::= SEQUENCE {

...

uci-OnPUSCH SetupRelease { UCI-OnPUSCH} OPTIONAL, -- Need M

...

uci-OnPUSCH-ListForDCI-Format0-2-r16 SetupRelease

{ UCI-OnPUSCH-ListForDCI-Format0-2-r16} OPTIONAL, -- Need M

...

uci-OnPUSCH-ListForDCI-Format0-1-r16 SetupRelease

{ UCI-OnPUSCH-ListForDCI-Format0-1-r16 } OPTIONAL, -- Need M

}

Note on uci-OnPUSCH-ListDCI-0-1, uci-OnPUSCH-ListDCI-0-2:

- Configuration for up to 2 HARQ-ACK codebooks specific to DCI format 0_1/0_2.

- The field uci-OnPUSCH-ListDCI-0-1 applies to DCI format 0_1 and the field uci-OnPUSCHListDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

UCI-OnPUSCH ::= SEQUENCE {

betaOffsets CHOICE {

dynamic SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic BetaOffsets

} OPTIONAL, -- Need M

scaling ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

UCI-OnPUSCH-ListForDCI-Format0-2-r16 ::= SEQUENCE (SIZE (1..2))

OF UCI-OnPUSCH-ForDCI-Format0-2-r16

UCI-OnPUSCH-ListForDCI-Format0-1-r16 ::= SEQUENCE (SIZE (1..2))

OF UCI-OnPUSCH

UCI-OnPUSCH-ForDCI-Format0-2-r16 ::= SEQUENCE {

betaOffsetsForDCI-Format0-2-r16 CHOICE {

dynamicForDCI-Format0-2-r16 CHOICE {

oneBit-r16 SEQUENCE (SIZE (2)) OF BetaOffsets,

twoBits-r16 SEQUENCE (SIZE (4)) OF BetaOffsets

},

semiStaticForDCI-Format0-2-r16 BetaOffsets

} OPTIONAL, -- Need M

scalingForDCI-Format0-2-r16 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

### Summary on Rel-15/16 design for BetaOffsets

For DG PUSCH:

For the non-fallback DCIs (0-1/0-2), if physical layer priority is configured (priorityIndicatorDCI-0-1/priorityIndicatorDCI-0-2) , then there can be one set of beta offsets for semi-static signaling of beta offsets, and there can be up to 4 sets of betaoffsets for dynamic signaling of beta offsets. Note each set of beta offsets can include 3 beta offsets for HARQ-ACK (for 3 ranges: up to 2 bits, 3 bits to 11 bits, and more than 11 bits), 2 offsets for CSI Part I (for 2 cases: up to 11 bits, and more than 11 bits), and 2 offsets for CSI Part II (for 2 cases: up to 11 bits, and more than 11 bits).

For CG PUSCH:

Similar to DG PUSCH, UCIs over CG PUSCH can be supported, and its signaling design in Rel-15 is similar to that for DG’s. Further CG-UCI’s beta offsets can be configured through RRC signaling.

The coding rate for UCI Part 0 (the part taken by HARQ-ACK) is determined according to the configured beta offsets set(s) and dynamic signaling if present:

For UCI part 0, the determination of βoffsetPUSCH can be summarized as below:

• HARQ-ACK only over PUSCH with UL-SCH or without UL-SCH (Clause 6.3.2.4.1.1):  βoffsetPUSCH=βoffsetHARQ-ACK;

• CG-UCI over PUSCH with UL-SCH (6.3.2.4.1.4 CG-UCI ):  βoffsetPUSCH=βoffsetCG-UCI;

• HARQ-ACK and CG-UCI over PUSCH with UL-SCH (6.3.2.4.1.5 HARQ-ACK and CG-UCI)

βoffsetPUSCH=βoffsetHARQ-ACK;

For UCI part 1, the determination of βoffsetPUSCH is as follows:

• CSI part I over PUSCH with UL-SCH or without UL-SCH:(Clause 6.3.2.4.1.2):  βoffsetPUSCH=βoffsetCSI-part1;

For UCI part 2, the determination of βoffsetPUSCH is as follows:

• CSI part II over PUSCH with UL-SCH or without UL-SCH (6.3.2.4.1.3 CSI part 2):  βoffsetPUSCH=βoffsetCSI-part2;

note βoffsetPUSCH is NOT used for CSI part II over PUSCH without UL-SCH.

### Summary on Rel-15/16 design for alpha

The determination of α is according to  scaling  in the RRC signaling.

The information bits or payload for UCI part 0 (corresponding to HARQ-ACK or CG-UCI or HARQ-ACK+CG-CGI in Rel-16), UCI part I (corresponding to CSI part I in Rel-16), UCI part II (corresponding to CSI part II in Rel-16).

Figure 12-1 Example of UCI multiplexing over PUSCH

## Rel-17 design for loading beta offsets

If there is no inter-L1 priority UCI multiplexing, the Rel-16 behavior should be retained. If there is inter-L1 priority UCI multiplexing, α and/or β can be modified accordingly:

Now with the UCI mapping discussed in our contribution to RAN1 #104bis (R1-2103106), UCI part II may have only 1 bit (e.g. 1 bit for LP HARQ-ACK), in this case, it may be possible to develop a new coding scheme, e.g. by using the PCFICH code design from LTE, or through padding or repetition: with padding, [X00] for 1 bit [XY 0] for 2 bits (X and Y are HARQ-ACK bits) can be fed to the Reed-Muller code. Through repetition is also possible: [XXX], or [XX], or [X0X] or [...X0..X..] (mapping two occurrence of X’s to two positions for example) can be considered. A new channel coding scheme can be also considered.

In this case, irrespective of how 1 or 2 bits for HARQ-ACK is handled over UCI Part I and UCI Part II is handled, it may be necessary to introduce a new β offset for up to Z bits for UCI Part I and/or UCI part II (Z = 1 or Z= 2 for example), then an IE under BetaOffsets can be introduced for up to Z bits such as

betaOffsetCSI-Part1-Index3 INTEGER(0..31) OPTIONAL– Need S

for LP HARQ-ACK mapping Alt. 1; or

betaOffsetCSI-Part2-Index3 INTEGER(0..31) OPTIONAL– Need S

for LP HARQ-ACK mapping Alt. 2.

With LP HARQ-ACK mapping Alt. 1, then the revised BeatOffsets set is as follows:

BetaOffsets ::= SEQUENCE {

betaOffsetACK-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetACK-Index2 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetACK-Index3 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index2 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index3 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index2 INTEGER(0..31) OPTIONAL-- Need S

}

betaOffsetCSI-Part1-Index3

1-2 bits of UCI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

Note up to Rel-16, α or scaling is semi-statically configured (separately for HP PUSCH and LP PUSCH).

In the discussion below, we assume the encoding of 1 or 2 bits UCI in Part I or Part II is with a solution already.

Then the UCI multiplexing resources can be fully determined once (β0,β1,β2,α) and the UCI encoding sequences are known which is treated above, where the beta offset for HARQACK/CG-UGI/(HARQ-ACK+CG-UCI) or UCI part 0 in Rel-16 corresponds to β0, and β1 is the beta offset for UCI part I, and β2 is the beta offset for UCI part II.

To make it easier to incorporate the specification change, we can categorize HARQ-ACK into Group-1-HARQ-ACK and Group-2-HARQ-ACK, basically Group-1-HARQ-ACK is transmitted over UCI Part 0, Group-2-HARQACK is transmitted over UCI Part I or UCI part II.

Essentially, we need to the adapt beta offsets and alpha for different scenarios:

Legacy scenarios:

If there is no inter-L1 priority UCI multiplexing and the UCIs are of the same physical layer priority as PUSCH, the Rel-16 design is reused, which can be also formulated as

(β0,β1,β2) = (betaOffsetHARQ−ACK or betaOffsetCG−UCI, betaOffsetCSI−part1, betaOfsetCSI−part2)

HP UCI only over LP PUSCH:

for alpha, a separate alpha can be semi-statically configured for such a case. It is also possible directly to assume α = 1 in this case and using up all the resources for HP UCI should be a viable choice.

As for beta offsets, there are two options:

Option 1: (β0,β1,β2) for HP UCIs over HP PUSCH can be re-used

Option 2: up to 4 new beta offset sets can be specifically introduced for the case with HP UCI(s) over LP PUSCH.

If the LP PUSCH is a DG (dynamic Grant) PUSCH:

beta_offset indicator in the DCI scheduling the LP PUSCH can be used to look up the set in either case. HP HARQ-ACK is Group-1-HARQ-ACK.

LP UCIs only over HP PUSCH:

a separate set of (β0,β1,β2,α) which can be different from the set for HP UCIs over HP PUSCH can be used. for alpha, a separate alpha can be semi-statically configured for such a case. It is also possible directly to assume α = 0.5 or even a smaller value.

As for beta offsets, there are two options:

Option 1: (β0,β1,β2) for LP UCIs over LP PUSCH can be re-used

Option 2: up to 4 new beta offset sets can be specifically introduced for the case with LP UCI(s) over HP PUSCH

and beta_offset indicator can be used to look up the set in either case. LP HARQ-ACK is Group-2-HARQ-ACK, which can be mapped to UCI part I or UCI part II.

Mixed UCIs over LP PUSCH:

the consideration can be similar to what we have for the case with HP UCIs only over LP PUSCH. In addition, we can also consider updating betaOffsets for the UCI part(s) which carry HP UCIs, e.g. β0 is used for the HP HARQ-ACK over LP PUSCH, which is different from that configured for LP HARQ-ACK over LP PUSCH, but β1 and β2 remain the same as for LP CSIs over PUSCH if UCI part I and UCI part II don’t carry any HP UCI. HP-ACK is Group-1-HARQ-ACK for rate matching. LP-ACK is Group-2-HARQ-ACK for rate-matching.

Mixed UCIs over HP PUSCH: α can remain unchanged compared to the case with HP UCIs only over HP PUSCH, and beta offsets can remain unchanged also compared to the case with HP UCIs only over HP PUSCH. HP HARQ-ACK is Group-1-HARQ-ACK for rate matching.

LP HARQ-ACK is Group-2-HARQ-ACK, which can be mapped to UCI part I or UCI part II.

Figure 12-2 betaOffset set selection at a PUSCH with a given physical layer priority

Note beta_offset indicator, the physical layer priority, and the types of UCI multiplexing (HP UCIs or LP UCIs or mixed UCIs, HARQ-ACK’s presence, etc) can be used to look up the beta offset set so the PRI field size does not need to be expanded. With that, 4 groups of beta Offsets sets can be supported:

- Group 1: beta Offsets sets for LP UCIs (all UCIs are at LP) over LP PUSCH as in the Rel-16 legacy design

- Group 2: beta Offsets sets for HP UCIs (all UCIs are at HP) over HP PUSCH as in the Rel-16 legacy design

- Group 3: beta Offsets sets for UCI(s) with at least one  UCI at HP over LP PUSCH

- Group 4: beta Offsets sets for UCI(s) with at least one  UCI at LP over HP PUSCH

Then on a LP PUSCH, with the presence of a HP UCI, Group 3 instead of Group 1 is selected. Similarly, on a HP PUSCH, with the presence of a LP UCI, Group 4 instead of Group 2 is selected.

For DG PUSCH triggered by dynamic signaling with DCI format 0-2, two groups of beta offset sets are already supported in Rel-16 w.r.t. physical layer priority, then the number of groups can be extended to 4 in Rel-17:

UCI-OnPUSCH-ListForDCI-Format0-2-r17 ::= SEQUENCE (SIZE (1..4))

OF UCI-OnPUSCH-ForDCI-Format0-2-r17

A similar extension can be considered for DG PUSCH triggered by dynamic signaling with DCI format 0-1, and the number of groups is extended to 4:

UCI-OnPUSCH-ListForDCI-Format0-1-r17 ::= SEQUENCE (SIZE (1..4))

OF UCI-OnPUSCH-r17

Note that UCI-OnPUSCH-r17 and UCI-OnPUSCH-ForDCI-Format0-2-r17 can have beta value less than 1 compared to their Rel-16 counterparts.

The semi-static beta Offset set is shown as Set 0 and Set 0A in Figure 20.  For the β1 set,  3 values are given in Figure 20 to reflect the choice to map LP HARQ-ACK to UCI Part 1 and creating a beta offset for 1~2 LP HARQ-ACK bits as discussed above. The adaptation of mapping LP HARQ-ACK to UCI part II can lead to adding one value to the β2 set. To reflect the situation that no all UCIs mapped to UCI part I or UCI part II are CSIs, the IE names can be modified, e.g. from “betaOffsetCSI-Part1-Index1” to “betaOffsetUCI-Part1-Index1”, …, from “betaOffsetCSI-Part2-Index1” to “betaOffsetUCI-Part2-Index1”, etc.

However, if the beta_offset indicator field size can be expanded, then the selection of a beta set group can be explicitly indicated.

For a configured grant configuration, as the physical layer priority is configured as part of configured grant configuration, two groups of beta Offsets sets can be configured:

- One group for the case where all UCI(s) are of the same physical layer priority as the configured grant PUSCH

- One group for the case where at one UCI is of a different physical layer priority as the configured grant PUSCH

Proposal 12-1: a beta offset set can be looked up according to physical layer priority, beta offset selection, and the presence of mixed UCIs.

# Discussion on PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH

The WI has tasked RAN1 to “Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline”.

At RAN1 103-e, the following was agreed:

Agreements:

Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17.

- FFS the related cancelation behavior for the PUSCH of lower PHY priority and other details.

  - First clarify what is the scope of this feature, e.g. if overlapping between more than 2 channels is considered.

- FFS the timeline requirements.

  - First clarify what is the behavior of Rel-16 UE in case of DG/CG/UCI overlapping, with and without uplink skipping enabled.

- FFS UE capability for this feature.

- Note: The main bullet has been agreed in the WID by RAN Plenary.

In Rel-15, when timeline conditions are met, a DG PUSCH can be scheduled over a CG transmission occasion. While the specification for DG/CG with single slot transmission is rather clear, we note that the Rel-15 behavior concerning CG/DG with slot aggregation was only clarified at RAN1 101-e.

With the introduction of multiple CGs over a BWP, PUSCH repetition Type B, and physical layer priority, the Rel-16 behavior concerning CG/DG is quite complicated and in need of clarification, we have discussed them in a companion paper [3]. In our view, bringing clarity to the Rel-16 behavior is the first step to define the Rel-17 solution, and it is important to identify what transmission strategies are already supported by the Rel-16 specification and what are not; this also fulfills the requirement in WID “taking the solution developed during Rel-16 as the baseline”.

We have

Proposal 13-1: Clarify the Rel-16 UE behavior concerning DG/CG transmission.

# Simultaneous PUCCH and PUSCH transmissions

The following was agreed:

Agreements:

Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA.

- FFS how to trigger this function.

- FFS for intra-band CA.

Subject to UE capability, simultaneous PUCCH and PUSCH transmissions for inter-band CA can be enabled; the enabling mechanism should be through RRC configuration. Note at RAN1 #97, the UCI multiplexing behavior was clarified, the note below has been captured in the Chairman’s notes of RAN1 #97:

The Rel-15 and Rel-16 NR design mandates that UCI which would be carried over a PUCCH overlapping with PUSCH(s), be multiplexed over one of those PUSCHs. With simultaneous PUCCH/PUSCH transmissions on different cells for inter-band CA, Step 2 is executed for PUCCH and PUSCHs on for CCs in the same band as the PUCCH-residing CC, and PUSCHs on CCs at other bands are not considered in Step 2.

Simultaneous PUCCH/PUSCH transmission for intra-band CA leads to Tx discontinuity and large Tx power back-off. More specifically, phase discontinuity is a concern if different durations of the PUSCH and PUCCH are used.  To avoid phase discontinuity, then PUSCH duration and PUCCH duration have to be the same. However, for transmission at different physical layer priorities, mandating the same transmission duration for PUCCH/PUSCH severely limits cases where Simultaneous PUCCH/PUSCH transmission is used. With such strong restriction on PUSCH/PUCCH, it is not clear whether Simultaneous PUCCH/PUSCH transmission for intra-band CA provides much benefit.

Figure 16-1 Phase discontinuity at PUCCH

Proposal 14-1: Simultaneous PUCCH/PUSCH transmission for intra-band CA is not supported if phase discontinuity problem cannot be addressed.

In NR, PTRS (Phase Tracking Reference Signal) has been introduced for PDSCH and PUSCH to handle phase noise in FR2. Essentially by inserting reference symbols in the time domain, the receiver can track the phase change from one OFDM symbol to another OFDM symbol, and with the pre-DFT insertion of PTRS with DFT S-OFDM even tracking phase change at sub-OFDM symbol is also possible. PTRS can be also used to track frequency offset, e.g. front-load DMRS and PTRS can be used together as an alternative to inserting DMRS at multiple locations in PUSCH. Similarly, PTRS can be used to handle phase discontinuity for simultaneous PUCCH/PUSCH transmission:

- PTRS in PUSCH can be configured so phase jump due to the start or the end of PUCCH on another CC can be handled.

- PTRS in PUCCH can be introduced so phase jump due to the start or the end of PUSCH on another CC can be handled.

  - Note: an often-cited example for simultaneous PUCCH/PUSCH transmissions is a short PUSCH on CC1 and a long PUCCH on CC2.

For PUCCH format 3, PTRS can be introduced in a similar way as that for DFT-S-OFDM PUSCH.

Proposal 14-2: consider the feasibility of introducing PTRS for PUCCH to handle phase discontinuity problem in simultaneous PUCCH/PUSCH transmissions for intra-band CA.

Simultaneous PUCCH/PUSCH transmission on the same cell also leads to phase discontinuity issue. When the separation of PUCCH and PUSCH in the frequency domain is large, an even more serious issue is inter-modulation interference, which leads to a big challenge in UE implementation.  We have

Proposal 14-3: Simultaneous PUCCH/PUSCH transmission on the same CC is not supported.

# Conclusion

In this contribution, we provide our views on UCI multiplexing design and other design issues in Rel-17. We have

Proposal 2-1: Study alternatives with introducing an inter-L1 priority PUCCH multiplexing with PUCCH resources Z as inputs.

Observation 3-1: in Rel-16, for PRB number adjustment/interlace number adjustment, the configured PRB number of the  selected PUCCH resource (if it is at PF2 or PF3), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource,  OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, and the same coding rate is used for two CSI parts.

Observation 3-2:for CSI omission, selected PUCCH resource (if it is at PF2 or PF3, PF4), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, the same coding rate is used for two CSI parts.

Proposal 4-1: Ocombined=OUCI-part1+OUCI-part2⋅r1/r2  is used for PUCCH resource set selection.

Proposal 5-1: considering coding rates of UCI parts in the PRB number adjustment/interlace number adjustment.

Proposal 5-2: The condition to trigger PRB number adjustment is given by

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

And  the stop condition for PRB number adjustment  is as follows:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRB,minPUCCH⋅Nsc,ctrlRB,min⋅Nsymb-UCIPUCCH

Proposal 6-1: Consider different coding rates for UCI parts in UCI omission rule.

Proposal 6-2: With two UCI parts, the omission rule is as follows:

The following conditions are used for UCI omission:

n=1NUCI-part2reported‍OUCI-part2,n+OCRC,UCI-part2,N/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part2reported+1‍OUCI-part2,n+OCRC,UCI-part2,N+1/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

If all UCI part II is dropped, then the the following is examined to determine remaining UCIs in UCI part I:

n=1NUCI-part1reported‍OUCI-part1,n+OCRC,UCI-part1,,N/(Qm⋅r1)≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part1reported+1‍OUCI-part1,,n+OCRC,UCI-part1,N+1/(Qm⋅r1)>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH.

Proposal 7-1:

when HP CSI is present, only the following UCIs are carried in UCI part I and part II:

HP HARQ-ACK > HP SR > (HP CSI) > LP HARQ-ACK > (LP SR)

When HP CSI is absent, only the following UCIs are carried UCI part I and part II

HP HARQ-ACK > HP SR > LP HARQ-ACK > (LP SR) > LP CSI

Proposal 8-1: consider joint PUCCH resource set selection and PUCCH resource selection.

Proposal 8-2: if joint PUCCH resource set selection and PUCCH resource selection is supported, r1 and r2 are configured per PUCCH format or per PUCCH resource.

Proposal 9-1: leverage the Rel-15 design, LP HARQ-ACK is  mapped to UCI Part II in separate encoding, adopt the UCI mapping in Figures 9-6a/9-6b.

Proposal 9-2:

Consider for PUCCH format 2 to support multiplexing of HP UCI(s) and LP UCI(s):

generating two encoded sequences for HP-ACK (with r1) and LP-ACK (with r2) separately and then concatenating those two encoded sequences into one encoded sequence, UCI mapping is not changed for PUCCH Format 2.

Proposal 10-1: consider PRB # adjustment and CSI omission/HARQ compaction for the PUCCH resource under multi-CSI-PUCCH-ResourceList and pucch-CSI-ResourceList.

Proposal 11-1: Between two options of mapping LP HARQ-ACK into UCI Part I and UCI Part II, RAN1 select one of them consistently across multiplexing scenarios.

Proposal 11-2: Adopt Alt. 1 or Alt. 2 design from Tables 11-1 and 11-2.

Proposal 12-1: a beta offset set can be looked up according to physical layer priority, beta offset selection, and the presence of mixed UCIs.

Proposal 13-1: Clarify the Rel-16 UE behavior concerning DG/CG transmission.

Proposal 14-1: Simultaneous PUCCH/PUSCH transmission for intra-band CA is not supported if phase discontinuity problem cannot be addressed.

Proposal 14-2: consider the feasibility of introducing PTRS for PUCCH to handle phase discontinuity problem in simultaneous PUCCH/PUSCH transmissions for intra-band CA.

Proposal 14-3: Simultaneous PUCCH/PUSCH transmission on the same CC is not supported.

# References

- RP-193233, “New WID on enhanced Industrial Internet of Things (IoT) and URLLC support”, Nokia, Nokia Shanghai Bell, RAN #86, December 2019

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, RAN #88e, June 2020

- R1-2008434, “Maintenance of configured grant design in Physical Layer Enhancements for NR URLLC”, Apple, RAN1 #103-e, October 2020

- R1-2103083, UCI multiplexing and PUSCH skipping design in URLLC, Apple, RAN1 #104bis-e, April 2021