3GPP TSG-RAN WG1 Meeting #105-e	R1-2106141

# e-Meeting,  May 10th - 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0190 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on enabling configuration of time restriction over L1-SINR measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Apple Inc.), Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current specification has defined UE behavior for measurement restriction for L1-SINR. There is a conflict between the descriptions in TS 38.214 and 38.331. Specifically, - In 38.214, the higher layer parameters timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements are considered as optional. Whether to apply measurement restriction for L1-SINR is determined based on whether the associated higher layer parameter is configured or not. - In 38.331, the higher layer parameters timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements are mandatory to be present. The candidate values for the two parameters are ‘configured’ and ‘notConfigured’. When timeRestrictionForChannelMeasurements or timeRestrictionForInterferenceMeasurements is configured as “notConfigured”, if the UE determines whether to apply measurement restriction according to the description in 38.214, it may still be mist-interpreted as that the UE should apply measurement restriction, although the true intention of gNB is to disable measurement restriction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the operating conditions for applying measurement restriction for L1-SINR in 38.214 to be aligned with signalling design in 38.331, i.e., from “is not configured with” to “the value of … is configured as ‘notConfigured’”, and from “is configured with” to “the value of … is configured as ‘configured’”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between 38.214 and 38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR is to align the description in RAN1 specs with the signaling design in RAN2 specs. It is expected that both gNB and UE have been implemented accordig to this CR, so no impact is expected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.1.4.4	L1-SINR Reporting

< Unchanged parts are omitted >

When one or two resource settings are configured for L1-SINR measurement

-	If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to ‘notConfigured’, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the SSB or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting.

-	If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to ‘configured’, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SSB or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

-	If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to ‘notConfigured’, the UE shall derive the interference measurements for computing L1-SINR reported in uplink slot n based on only the CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement no later than the CSI reference resource associated with the CSI resource setting.

-	If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to ‘configured’, the UE shall derive the interference measurements for computing the L1-SINR reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement associated with the CSI resource setting.

< Unchanged parts are omitted >