3GPP TSG-RAN WG1 Meeting #105-e 	R1-2106329

# e-Meeting, 10th – 27th May 2021

Title:	LS on RAN1 agreements on RAN2-led features for RedCap

Response to:

Release:	Rel-17

Work Item:	Support of reduced capability NR devices (NR_redcap-Core)

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Shinya Kumagai, shinya.kumagai (at) docomo-lab.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN1 discussed RAN1 aspects on RAN2-led features for RedCap and agreed to send RAN2-related agreements/working assumptions/conclusions to RAN2 to facilitate their work on RAN2-led features for RedCap. RAN1 respectfully asks RAN2 to take the agreements/working assumptions/conclusions into account in their further work on RAN2-led features for RedCap and provide feedback, if any.

Followings are the agreements/working assumptions/conclusion on RAN1 aspects on RAN2-led features for RedCap made in RAN1#105-e meeting:

Working assumption: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled - FFS How to support enable/disable the early indication - FFS details e.g.: - separate initial UL BWP - separate PRACH resource - PRACH preamble partitioning - FFS the possibility of supporting Msg3 for the early indication Agreement: (if the above working assumption is confirmed) - Early indication of RedCap UEs in Msg1 can be enabled/disabled via SIB Agreement: - Support 2-step RACH for RedCap UEs as an optional feature - FFS details of early indication in MsgA, e.g.: - Separation of 2-step RACH resources or MsgA preambles - Separation of initial UL BWP - Using a new indication in MsgA PUSCH part - Note: Discussion on 4-step RACH for early indication should be prioritised Working assumption: - RedCap UE type is defined based on one of the following options - Option 2: Only include the reduced capabilities that the network needs to know during initial access, if any. - Option 4: The corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support - FFS: details of the set of reduced capabilities Conclusion: - RAN1 postpones the discussion on constraining of reduced capabilities, and if deemed necessary, RAN1 can come back

For reference, RAN1 also agreed following RAN2-related agreement/conclusion in RAN1#105-e meeting:

Agreement: For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. - Detailed signalling is up to RAN2 Conclusion - No consensus to support early identification of the number of Rx branches in Msg1/Msg3/MsgA for Redcap UE in Rel-17

2	Actions

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account in their further work on RAN2-led features for RedCap and provide feedback, if any.

3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #106-e 	16th – 27th August 2021		Online

TSG-RAN WG1 Meeting #106bis-e 	11th – 19th October 2021	Online