## NR Multicast and Broadcast Services

Please refer to RP-201038 for detailed scope of the WI

### Mechanisms to support group scheduling for RRC_CONNECTED UEs

R1-2104195	Group Scheduling Aspects for Connected UEs	FUTUREWEI

R1-2104248	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2104336	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

R1-2104387	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

R1-2104442	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2104491	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2104550	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

R1-2104632	Discussion on group scheduling mechanisms	CMCC

R1-2104695	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2104759	Group scheduling for NR Multicast and Broadcast Services	OPPO

R1-2104865	On group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

R1-2104928	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

R1-2104984	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105041	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105069	Discussion on Group Scheduling and Simultaneous MBS and Unicast Reception 	TCL Communication Ltd.

R1-2105128	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Apple

R1-2105179	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

R1-2105336	Support of group scheduling for RRC_CONNECTED Ues	Samsung

R1-2105381	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

R1-2105437	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

R1-2105600	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

R1-2105647	Discussion on common frequency resource for multicast of RRC_CONNECTED UEs	ETRI

R1-2105670	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Google Inc.

R1-2105720	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2105838	Discussion on group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2105844	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

R1-2105914	Mechanisms to support MBS group scheduling for RRC_CONNECTED Ues	Ericsson

R1-2105973	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

[105-e-NR-MBS-01] Email discussion/approval on mechanisms to support group scheduling for RRC_CONNECTED UEs with checkpoints for agreements on May 24, May 27 – Fei (CMCC)

Agreement:

For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, Alt 2 is supported:

- Alt 2: support a Type-x CSS

  - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS.

- FFS: Whether the Type-x CSS is a Type-3 CSS

### Mechanisms to improve reliability for RRC_CONNECTED UEs

R1-2104196	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2104249	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2104337	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2104388	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2104443	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2104492	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

R1-2104551	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2104633	Discussion on reliability improvement	CMCC

R1-2104696	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2104760	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2104866	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2104929	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2104985	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105042	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105129	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2105337	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2105382	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2105438	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2105601	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2105648	Discussion on HARQ-ACK feedback for multicast of RRC_CONNECTED UEs	ETRI

R1-2105721	Discussion on HARQ-ACK feedback for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2105843	Discussion on reliability for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2105915	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

R1-2106012	FL summary#1 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

[105-e-NR-MBS-02] Email discussion/approval on mechanisms to improve reliability for RRC_CONNECTED UEs with checkpoints for agreements on May 25, May 27 – Jinhuan (Huawei)

Agreement:

- The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

### Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

R1-2104197	MBS Support for RRC IDLE/INACTIVE UEs	FUTUREWEI

R1-2104250	Discussion on UE receiving broadcast in RRC IDLE/INACTIVE state	Huawei, HiSilicon, CBN

R1-2104338	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs		ZTE

R1-2104389	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			vivo

R1-2104444	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	Spreadtrum Communications

R1-2104493	Discussion on basic functions for MBS for RRC_IDLEINACTIVE UEs	CATT, CBN

R1-2104552	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

R1-2104634	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

R1-2104697	Views on group scheduling for Multicast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

R1-2104761	Discussion on support for IDLE and INACTIVE state Ues	OPPO

R1-2104867	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

R1-2104930	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

R1-2104986	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

Withdrawn

R1-2105043	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

Withdrawn

R1-2105130	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Apple

R1-2105180	Considerations on MBS functions for RRC_IDLE/RRC_INACTIVE UEs	Sony

R1-2105338	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

R1-2105383	Discussion on broadcast or multicast for RRC_IDLE or INACTIVE UEs	MediaTek Inc.

R1-2105439	Basic function for broadcast/multicast	LG Electronics

R1-2105602	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

R1-2105673	Discussion on MBS for RRC_IDLE/INACTIVE UE	Google Inc.

R1-2105722	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			NTT DOCOMO, INC.

R1-2105849	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs	CHENGDU TD TECH LTD.

R1-2105916	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

[105-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs with checkpoints for agreements on May 24, May 27 – David (BBC)

### Others

R1-2104339	Consideration on performance enhancement for RRC_IDLE/INACTIVE UEs	ZTE

R1-2104390	Other issues for Rel-17 MBS	vivo

R1-2104494	Discussion on other issues in Rel-17 MBS	CATT

R1-2104762	PUCCH resource allocation for  NACK-only based HARQ-ACK feedback in MBMS	OPPO

R1-2105440	Other aspects for MBS	LG Electronics

R1-2105526	Impact from MCCH and MTCH on broadcast reception	Huawei, HiSilicon

R1-2105853	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

Withdrawn

R1-2105854	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

Withdrawn

R1-2105855	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

R1-2105917	Assumptions for performance evaluations of NR-MBS	Ericsson