## NR Multicast and Broadcast Services

Please refer to RP-201038 for detailed scope of the WI

### Mechanisms to support group scheduling for RRC_CONNECTED UEs

R1-2104195	Group Scheduling Aspects for Connected UEs	FUTUREWEI

R1-2104248	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2104336	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

R1-2104387	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

R1-2104442	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2104491	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2104550	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

R1-2104632	Discussion on group scheduling mechanisms	CMCC

R1-2104695	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2104759	Group scheduling for NR Multicast and Broadcast Services	OPPO

R1-2104865	On group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

R1-2104928	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

R1-2104984	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105041	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105069	Discussion on Group Scheduling and Simultaneous MBS and Unicast Reception 	TCL Communication Ltd.

R1-2105128	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Apple

R1-2105179	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

R1-2105336	Support of group scheduling for RRC_CONNECTED Ues	Samsung

R1-2105381	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

R1-2105437	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

R1-2105600	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

R1-2105647	Discussion on common frequency resource for multicast of RRC_CONNECTED UEs	ETRI

R1-2105670	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Google Inc.

R1-2105720	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2105838	Discussion on group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2105844	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

R1-2105914	Mechanisms to support MBS group scheduling for RRC_CONNECTED Ues	Ericsson

R1-2105973	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

R1-2106093	Summary#3 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

[105-e-NR-MBS-01] Email discussion/approval on mechanisms to support group scheduling for RRC_CONNECTED UEs with checkpoints for agreements on May 24, May 27 – Fei (CMCC)

Agreement:

For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, Alt 2 is supported:

- Alt 2: support a Type-x CSS

  - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS.

- FFS: Whether the Type-x CSS is a Type-3 CSS

Agreement:

For PTP retransmission of SPS group-common PDSCH, CS-RNTI is used for CRC scrambling of PDCCH with the NDI bit set to 1.

Agreement:

As a baseline, reuse existing fields in DCI format 1_0 with CRC scrambled by C-RNTI for the fields of first DCI format with CRC scrambled with G-RNTI.

- FFS: how to determine the bitlength of FDRA field.

- FFS: Whether ‘Identifier for DCI formats’, ‘TPC command for scheduled PUCCH’ are needed.

- FFS: How to perform DCI size alignment

- FFS: Whether to include new DCI fields

- Note: All of the fields may not be reused and the size of the fields may not be the same

Working assumption:

Option 2B for CFR associated with UE active BWP other than initial BWP is supported at least for multicast of RRC-CONNECTED UEs.

- FFS: CFR associated with initial BWP

- FFS: CFR larger than initial BWP

Agreement:

For multicast of RRC_CONNECTED UEs, further study

- How the LBRM (Limited buffer rate-matching) for GC-PDSCH TBS is determined.

- how the xOverhead for GC-PDSCH TBS determination is configured.

- whether MAC-CE over GC-PDSCH is needed for activation/deactivation of semi-persistent ZP CSI-RS resource set if the semi-persistent ZP CSI-RS resource set is configured in PDSCH-Config in CFR.

Agreement:

Confirm the working assumption:

Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

- FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

Agreement:

For Rel-17 MBS UE, the UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.

- Note:   Group-common PDSCH(s) are counted as unicast PDSCH(s).

### Mechanisms to improve reliability for RRC_CONNECTED UEs

R1-2104196	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2104249	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2104337	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2104388	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2104443	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2104492	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

R1-2104551	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2104633	Discussion on reliability improvement	CMCC

R1-2104696	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2104760	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2104866	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2104929	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2104985	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105042	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105129	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2105337	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2105382	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2105438	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2105601	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2105648	Discussion on HARQ-ACK feedback for multicast of RRC_CONNECTED UEs	ETRI

R1-2105721	Discussion on HARQ-ACK feedback for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2105843	Discussion on reliability for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2105915	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

R1-2106012	FL summary#1 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

R1-2106064	FL summary#2 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

[105-e-NR-MBS-02] Email discussion/approval on mechanisms to improve reliability for RRC_CONNECTED UEs with checkpoints for agreements on May 25, May 27 – Jinhuan (Huawei)

Agreement:

- The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

- Agreement:

- Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS activation/deactivation.

Agreement:

The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.

Proposal:

When UE supports both ACK/NACK based and NACK-only based feedback for multicast, UE can be configured

- Alt1: either PUCCH resources for ACK/NACK based feedback or PUCCH resources for NACK-only based feedback.

- Alt2: both PUCCH resources for ACK/NACK based feedback and PUCCH resources for NACK-only based feedback.

  - Opt1: PRI in the DCI indicates the PUCCH resource of ACK/NACK based or of NACK-only based.

  - Opt2: A new field added in DCI indicates ACK/NACK based or NACK-only based.

- Down-select from the two alternatives and options.

### Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

R1-2104197	MBS Support for RRC IDLE/INACTIVE UEs	FUTUREWEI

R1-2104250	Discussion on UE receiving broadcast in RRC IDLE/INACTIVE state	Huawei, HiSilicon, CBN

R1-2104338	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs		ZTE

R1-2104389	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			vivo

R1-2104444	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	Spreadtrum Communications

R1-2104493	Discussion on basic functions for MBS for RRC_IDLEINACTIVE UEs	CATT, CBN

R1-2104552	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

R1-2104634	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

R1-2104697	Views on group scheduling for Multicast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

R1-2104761	Discussion on support for IDLE and INACTIVE state Ues	OPPO

R1-2104867	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

R1-2104930	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

R1-2104986	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

Withdrawn

R1-2105043	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

Withdrawn

R1-2105130	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Apple

R1-2105180	Considerations on MBS functions for RRC_IDLE/RRC_INACTIVE UEs	Sony

R1-2105338	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

R1-2105383	Discussion on broadcast or multicast for RRC_IDLE or INACTIVE UEs	MediaTek Inc.

R1-2105439	Basic function for broadcast/multicast	LG Electronics

R1-2105602	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

R1-2105673	Discussion on MBS for RRC_IDLE/INACTIVE UE	Google Inc.

R1-2105722	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			NTT DOCOMO, INC.

R1-2105849	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs	CHENGDU TD TECH LTD.

R1-2105916	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

R1-2105994	Feature lead summary #2 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

[105-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs with checkpoints for agreements on May 24, May 27 – David (BBC)

Proposal:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use the bandwidth with the same frequency range as CORESET#0 to receive GC-PDCCH/PDSCH carrying MCCH.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MCCH.

### Others

R1-2104339	Consideration on performance enhancement for RRC_IDLE/INACTIVE UEs	ZTE

R1-2104390	Other issues for Rel-17 MBS	vivo

R1-2104494	Discussion on other issues in Rel-17 MBS	CATT

R1-2104762	PUCCH resource allocation for  NACK-only based HARQ-ACK feedback in MBMS	OPPO

R1-2105440	Other aspects for MBS	LG Electronics

R1-2105526	Impact from MCCH and MTCH on broadcast reception	Huawei, HiSilicon

R1-2105853	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

Withdrawn

R1-2105854	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

Withdrawn

R1-2105855	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

R1-2105917	Assumptions for performance evaluations of NR-MBS	Ericsson