## Supporting NR from 52.6GHz to 71 GHz

Please refer to RP-210862  for detailed scope of the WI

### Initial access aspects

Including SSB, initial BWP, PRACH, etc.

R1-2104210	Initial access for Beyond 52.6GHz	FUTUREWEI

R1-2104273	Initial access signals and channels for 52-71GHz spectrum	Huawei, HiSilicon

R1-2104348	Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz	vivo

R1-2104416	Discussion on initial access aspects for NR for 60GHz	Spreadtrum Communications

R1-2104452	Initial access aspects	Nokia, Nokia Shanghai Bell

R1-2104460	Initial Access Aspects	Ericsson

R1-2104507	Initial access aspects for up to 71GHz operation	CATT

R1-2104659	Initial access aspects for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104765	Discusson on initial access aspects	OPPO

R1-2104833	Discussion on the initial access aspects for 52.6 to 71GHz	ZTE, Sanechips

R1-2104894	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

R1-2104950	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105007	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105061	Considerations on initial access for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2105092	Discussion on Initial access signals and channels	Apple

R1-2105156	Considerations on initial access aspects for NR from 52.6 GHz to 71 GHz	Sony

R1-2105260	Discussion on initial access aspects supporting NR from 52.6 to 71 GHz	NEC

R1-2105297	Initial access aspects for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105370	Discussion on initial access of 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105419	Initial access aspects to support NR above 52.6 GHz	LG Electronics

R1-2105495	Initial access aspects for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105555	On initial access aspects for NR from 52.6GHz to 71 GHz	Xiaomi

R1-2105581	Discussions on initial access aspects	InterDigital, Inc.

R1-2105592	NR Initial Access from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105630	Initial access aspects	Sharp

R1-2105660	On the importance of inter-operator PCI confusion resolution and ANR support in 52.6 GHz and beyond	AT&T

R1-2105688	Initial access aspects for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105786	Further details of initial access for NR above 52.6 GHz	Charter Communications

R1-2105868	Discussion on initial access aspects for NR beyond 52.6GHz	WILUS Inc.

R1-2105977

[105-e-NR-52-71GHz-01] Email discussion/approval on initial access aspects with checkpoints for agreements on May-24, May-27 – Daewon (Intel)

### PDCCH monitoring enhancements

Void (not be handled during this e-meeting). No contributions please.

### Enhancements for PUCCH formats 0/1/4

R1-2104211	Enhancements of PUCCH formats for Beyond 52.6GHz	FUTUREWEI

R1-2104349	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

R1-2104417	Discussion on enhancements for PUCCH format 0/1/4 for above 52.6GHz	Spreadtrum Communications

R1-2104453	Enhanced PUCCH formats 0/1/4	Nokia, Nokia Shanghai Bell

R1-2104461	PUCCH enhancements	Ericsson

R1-2104508	Enhancements for PUCCH formats for up to 71GHz operation	CATT

R1-2104660	Enhancements for PUCCH for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104766	Discussion on enhancements for PUCCH format 0/1/4	OPPO

R1-2104834	Discussion on the PUCCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2104895	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2104951	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105008	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105093	Discussion on Enhancements for PUCCH formats 0/1/4	Apple

R1-2105157	Further considerations on enhancements for PUCCH formats 0/1/4	Sony

R1-2105298	Enhancements for PUCCH format 0/1/4 for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105369	On Enhancements for PUCCH formats 0/1	MediaTek Inc.

R1-2105420	Enhancements for PUCCH formats 0/1/4 to support NR above 52.6 GHz	LG Electronics

R1-2105496	Enhancements to PUCCH formats 0/1/4 for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105582	Discussions on enhancements for PUCCH formats 0/1/4	InterDigital, Inc.

R1-2105689	PUCCH format 0/1/4 enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105869	Discussion on PUCCH enhancement for PUCCH format 0/1/4	WILUS Inc.

R1-2105929	Enhancement on PUCCH formats	Huawei, HiSilicon

R1-2105961

[105-e-NR-52-71GHz-02] Email discussion/approval on PUCCH format 0/1/4 enhancements with checkpoints for agreements on May 24, May 27 – Steve (Ericsson)

Proposal:

For enahanced PF 0/1/4 for 120 kHz, all REs within each RB are mapped

- Note: PRB and sub-PRB interlaced mapping is not considered further

### Beam management for new SCSs

Including timing associated with beam-based operation

Void (not be handled during this e-meeting). No contributions please.

### PDSCH/PUSCH enhancements

Including defining maximum bandwidth for new SCSs, time line related aspects adapted to each of the new numerologies 480kHz and 960kHz, reference signals, scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, etc.

Only to handle multi-PDSCH/PUSCH with a single DCI and HARQ

- NO contributions please to other topics such as defining maximum bandwidth for new SCSs, timeline related aspects adapted to each of the new numerologies 480 kHz and 960 kHz and reference signals, etc.

R1-2105422	Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

R1-2104212	Enhancements to support PDSCH/PUSCH for Beyond 52.6GHz	FUTUREWEI

R1-2104274	PDSCH/PUSCH enhancements for 52-71GHz spectrum	Huawei, HiSilicon

R1-2104350	Discussions on multi-PDSCH/PUSCH scheduling for NR operation from 52.6GHz to 71GHz			vivo

R1-2104418	Discussion on PDSCH and PUSCH enhancements for above 52.6GHz	Spreadtrum Communications

R1-2104454	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

R1-2104462	PDSCH-PUSCH Enhancements	Ericsson

R1-2104509	PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

R1-2104661	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104767	Discussion on PDSCH/PUSCH enhancements	OPPO

R1-2104835	Discussion on the PDSCH/PUSCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2104896	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2104952	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105009	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105062	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2105094	Discussion on multi-PxSCH and HARQ Codebook Enhancements	Apple

R1-2105158	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Sony

R1-2105259	Discussion on PDSCH enhancements supporting NR from 52.6GHz to 71 GHz	NEC

R1-2105299	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105372	HARQ codebook design for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105396	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz	Panasonic Corporation

R1-2105421	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

R1-2105497	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105556	PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

R1-2105583	Enhancing PDSCH/PUSCH Scheduling for 52.6 GHz to 71 GHz Band	InterDigital, Inc.

R1-2105596	PDSCH Considerations for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105690	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105784	PDSCH-PUSCH Enhancement for NR beyond 52.6 GHz	Charter Communications

R1-2105870	Discussion on multi-PDSCH/PUSCH scheduling for NR from 52.6GHz to 71GHz	WILUS Inc.

[105-e-NR-52-71GHz-03] Email discussion/approval on scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, with checkpoints for agreements on May 25, May 27 – Seonwook (LGE)

### Channel access mechanism

R1-2104213	Channel access for shared spectrum Beyond 52.6 GHz	FUTUREWEI

R1-2104275	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

R1-2104351	Discussions on channel access mechanism for NR operation from 52.6GHz to 71 GHz	vivo

R1-2104419	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

R1-2104455	Channel access mechanism	Nokia, Nokia Shanghai Bell

R1-2104463	Channel Access Mechanisms	Ericsson

R1-2104510	Channel access mechanism for up to 71GHz operation	CATT

R1-2104662	Channel access mechanism for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104720	Discussions on channel access mechanism enhancements for 52.6G-71 GHz	CAICT

R1-2104768	Discussion on channel access mechanism	OPPO

R1-2104836	Discussion on the channel access for 52.6 to 71GHz	ZTE, Sanechips

R1-2104897	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

R1-2104953	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105010	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105063	Considerations on channel access mechanism for NR  from 52.6GHz to 71 GHz	Fujitsu

R1-2105095	Channel access mechanism	Apple

R1-2105145	Channel access for multi-beam operation	Panasonic

R1-2105159	Channel access mechanism for 60 GHz unlicensed spectrum	Sony

R1-2105261	Discussion on channel access mechanism supporting NR from 52.6 to 71GHz	NEC

R1-2105300	Channel access mechanism for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105371	On the channel access mechanisms for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105423	Channel access mechanism to support NR above 52.6 GHz	LG Electronics

R1-2105498	Channel access mechanisms for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105557	Discussion on channel access mechanism for NR on 52.6-71 GHz	Xiaomi

R1-2105584	Discussion on channel access mechanisms	InterDigital, Inc.

R1-2105597	On Channel Access Mechanism for NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105661	On receiver assisted channel access and directional LBT	AT&T

R1-2105691	Channel access mechanism for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105755	Discussion on multi-beam operation	ITRI

R1-2105785	Channel access mechanisms for above 52.6 GHz	Charter Communications

R1-2105871	Discussion on channel access mechanism for NR from 52.6GHz to 71GHz	WILUS Inc.

[105-e-NR-52-71GHz-04] Email discussion/approval on channel access mechanism with checkpoints for agreements on May 25, May 27 – Jing (Qualcomm)

### Others

Including analysis or recommendation to RAN#92e (June) on how to introduce the 52.6-71GHz frequency range

R1-2104352	Discussions on frequency range definition and sync raster for 52.6-71GHz	vivo

R1-2104456	Extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

R1-2104464	Introduction of the 52.6 to 71 GHz frequency range	Ericsson

R1-2104511	Views on Frequency Range for 52.6 to 71 GHz	CATT

R1-2104769	Discussion on other aspects	OPPO

R1-2104837	Discussion on the frequency range definition for above 52.6GHz	ZTE, Sanechips

R1-2104898	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

R1-2104954	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

Withdrawn

R1-2105011	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

Withdrawn

R1-2105096	On How to Introduce the 52.6-71GHz Frequency Range	Apple

R1-2105301	Discussion on defining frequency range for NR from 52.6 to 71 GHz	Samsung

R1-2105373	Frequency range naming discussion for 52.6-71 GHz	MediaTek Inc.

R1-2105424	Discussion on frequency range definition to support NR above 52.6 GHz	LG Electronics

R1-2105533	Discussion on the frequency range for bands in 52.6-71 GHz	Huawei, HiSilicon

R1-2105585	Efficient beam management for NR in 52.6 – 71 GHz	InterDigital, Inc.

R1-2105692	Views on how to introduce 52.6 - 71 GHz frequency range in NR	NTT DOCOMO, INC.

R1-2105787	Discussion on how to introduce the 52.6-71GHz frequency range	Charter Communications

[105-e-NR-52-71GHz-05] Email discussion/approval focusing on analysis or recommendation to RAN#92e (June) on how to introduce the 52.6-71GHz frequency range with checkpoints for agreements on May 25, May 27 – Alex (Lenovo)