## Supporting NR from 52.6GHz to 71 GHz

Please refer to RP-210862  for detailed scope of the WI

### Initial access aspects

Including SSB, initial BWP, PRACH, etc.

R1-2104210	Initial access for Beyond 52.6GHz	FUTUREWEI

R1-2104273	Initial access signals and channels for 52-71GHz spectrum	Huawei, HiSilicon

R1-2104348	Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz	vivo

R1-2104416	Discussion on initial access aspects for NR for 60GHz	Spreadtrum Communications

R1-2104452	Initial access aspects	Nokia, Nokia Shanghai Bell

R1-2104460	Initial Access Aspects	Ericsson

R1-2104507	Initial access aspects for up to 71GHz operation	CATT

R1-2104659	Initial access aspects for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104765	Discusson on initial access aspects	OPPO

R1-2104833	Discussion on the initial access aspects for 52.6 to 71GHz	ZTE, Sanechips

R1-2104894	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

R1-2104950	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105007	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105061	Considerations on initial access for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2105092	Discussion on Initial access signals and channels	Apple

R1-2105156	Considerations on initial access aspects for NR from 52.6 GHz to 71 GHz	Sony

R1-2105260	Discussion on initial access aspects supporting NR from 52.6 to 71 GHz	NEC

R1-2105297	Initial access aspects for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105370	Discussion on initial access of 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105419	Initial access aspects to support NR above 52.6 GHz	LG Electronics

R1-2105495	Initial access aspects for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105555	On initial access aspects for NR from 52.6GHz to 71 GHz	Xiaomi

R1-2105581	Discussions on initial access aspects	InterDigital, Inc.

R1-2105592	NR Initial Access from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105630	Initial access aspects	Sharp

R1-2105660	On the importance of inter-operator PCI confusion resolution and ANR support in 52.6 GHz and beyond	AT&T

R1-2105688	Initial access aspects for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105786	Further details of initial access for NR above 52.6 GHz	Charter Communications

R1-2105868	Discussion on initial access aspects for NR beyond 52.6GHz	WILUS Inc.

R1-2105977	Issue Summary for initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

R1-2105978

R1-2106082

[105-e-NR-52-71GHz-01] Email discussion/approval on initial access aspects with checkpoints for agreements on May-24, May-27 – Daewon (Intel)

Agreement:

For 480kHz/960kHz SSB, select one of the following alternatives:

- ALT 1) First symbols of the candidate SSB have index {X, Y} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

  - value of X and Y are identical for 480kHz and 960kHz

    - FFS: exact value of X and Y

- ALT 2) First symbols of the candidate SSB have index {4, 8, 16,20} + 28*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

- Values of n for 480kHz and 960kHz for ALT 1 and 2

  - FFS: whether number of values for ‘n’ depend on LBT operation (i.e. LBT vs no-LBT)

  - FFS: exact values of ‘n’ for each SCS

  - Values of ‘n’ for one mode of operation shall be strictly a subset of values for another mode of operation, if two mode of operation exist for number of candidate SSBs

  - FFS: whether values of ‘n’ shall not be all consecutive integer values (i.e. non-candidate SSB slots are positioned every few candidate SSB slots)

Proposal:

For 480kHz and 960kHz PRACH,

- The reference slot duration corresponds to 60 kHz SCS

- A PRACH slot index,  , corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot. Select one or more of the following alternatives

  - ALT 1) At least the same density (i.e. number of PRACH slots per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher PRACH slot density (number of PRACH slots per reference slot)

  - ALT 2) at least has the same RO density (i.e. number of RO PRACH slots per reference slot) as for 120kHz PRACH in FR2 is supported per reference slot

    - FFS: support for higher RO density for 480/960kHz PRACH is additionally supported

- FFS: supported values of the starting PRACH slot index  within reference slot and whether or not the ROs for a given PRACH configuration can span more than one PRACH slot if gaps between consecutive ROs are supported for LBT and/or beam switching purposes

- FFS: whether and how to account for LBT in RO configuration (if needed)

- FFS: whether and how to account for beam switching gap in RO configuration (if needed)

- An “example” illustration of PRACH slots for 480/960kHz is shown below:

Proposal:

In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints.

- Limited sync raster entry numbers

  - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n261 is 602). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

- only 480kHz CORESTE#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS.

- SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

Formal objection sustained by: Huawei, MediaTek (would like to discuss at next meeting)

Proposal:

In addition to 120kHz, support both 480 and 960 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints.

- Limited sync raster entry numbers

  - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n261 is 602). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

- only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS i.e., (480,480) and (960,960).

- SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

Formal objection sustained by: Huawei, MediaTek (object to 960 kHz)

Proposal:

To support ANR and PCI confusion detection for 480/960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 480 and 960kHz SSB

- FFS: additional method(s) to enable support to obtain neighbor cell PCI and SIB1 contents related to CGI reporting

- Only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (480,480) and (960,960).

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

- Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB.

- Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

Formal objection sustained by: Huawei

Agreement:

For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access)

- Support configuring CORESET#0/Type0-PDCCH for the purpose of ANR/PCI confusion detection by down selecting from the following two alternatives

  - Alt 1) Using dedicated signaling

  - Alt 2) Using configuration in MIB

    - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

### PDCCH monitoring enhancements

Void (not be handled during this e-meeting). No contributions please.

### Enhancements for PUCCH formats 0/1/4

R1-2104211	Enhancements of PUCCH formats for Beyond 52.6GHz	FUTUREWEI

R1-2104349	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

R1-2104417	Discussion on enhancements for PUCCH format 0/1/4 for above 52.6GHz	Spreadtrum Communications

R1-2104453	Enhanced PUCCH formats 0/1/4	Nokia, Nokia Shanghai Bell

R1-2104461	PUCCH enhancements	Ericsson

R1-2104508	Enhancements for PUCCH formats for up to 71GHz operation	CATT

R1-2104660	Enhancements for PUCCH for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104766	Discussion on enhancements for PUCCH format 0/1/4	OPPO

R1-2104834	Discussion on the PUCCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2104895	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2104951	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105008	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105093	Discussion on Enhancements for PUCCH formats 0/1/4	Apple

R1-2105157	Further considerations on enhancements for PUCCH formats 0/1/4	Sony

R1-2105298	Enhancements for PUCCH format 0/1/4 for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105369	On Enhancements for PUCCH formats 0/1	MediaTek Inc.

R1-2105420	Enhancements for PUCCH formats 0/1/4 to support NR above 52.6 GHz	LG Electronics

R1-2105496	Enhancements to PUCCH formats 0/1/4 for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105582	Discussions on enhancements for PUCCH formats 0/1/4	InterDigital, Inc.

R1-2105689	PUCCH format 0/1/4 enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105869	Discussion on PUCCH enhancement for PUCCH format 0/1/4	WILUS Inc.

R1-2105929	Enhancement on PUCCH formats	Huawei, HiSilicon

R1-2105961	FL Summary for Enhancements for PUCCH formats 0/1/4	Moderator (Ericsson)

R1-2106068

[105-e-NR-52-71GHz-02] Email discussion/approval on PUCCH format 0/1/4 enhancements with checkpoints for agreements on May 24, May 27 – Steve (Ericsson)

Agreement:

- For 120 kHz SCS:

  - Support at least Alt-1 for enhanced PF0/1 for both PUCCH resources before and after dedicated PUCCH resource configuration

  - FFS: Whether or not Alt-2 is additionally supported for PF0/1 for either or both of the following:

    - PUCCH resources before dedicated PUCCH resource configuration

    - PUCCH resources after dedicated PUCCH resource configuration

  - FFS: Supported RE mapping scheme(s) amongst {Alt-1, Alt-2} for enhanced PF4 including design details

- Notes:

  - Alt-1 = all REs within each RB are mapped

  - Alt-2 = a subset of REs within each RB are mapped (sub-PRB interlaced mapping)

  - Which RE mapping scheme(s) to support for PF0/1/4 to be concluded in RAN1#106

- Note: No further enhancements on RB shortage issue and frequecy hopping distance issue should be considered for PUCCH resource sets prior to RRC configuration.

### Beam management for new SCSs

Including timing associated with beam-based operation

Void (not be handled during this e-meeting). No contributions please.

### PDSCH/PUSCH enhancements

Including defining maximum bandwidth for new SCSs, time line related aspects adapted to each of the new numerologies 480kHz and 960kHz, reference signals, scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, etc.

Only to handle multi-PDSCH/PUSCH with a single DCI and HARQ

- NO contributions please to other topics such as defining maximum bandwidth for new SCSs, timeline related aspects adapted to each of the new numerologies 480 kHz and 960 kHz and reference signals, etc.

R1-2105422	Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

R1-2104212	Enhancements to support PDSCH/PUSCH for Beyond 52.6GHz	FUTUREWEI

R1-2104274	PDSCH/PUSCH enhancements for 52-71GHz spectrum	Huawei, HiSilicon

R1-2104350	Discussions on multi-PDSCH/PUSCH scheduling for NR operation from 52.6GHz to 71GHz			vivo

R1-2104418	Discussion on PDSCH and PUSCH enhancements for above 52.6GHz	Spreadtrum Communications

R1-2104454	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

R1-2104462	PDSCH-PUSCH Enhancements	Ericsson

R1-2104509	PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

R1-2104661	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104767	Discussion on PDSCH/PUSCH enhancements	OPPO

R1-2104835	Discussion on the PDSCH/PUSCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2104896	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2104952	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105009	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105062	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2105094	Discussion on multi-PxSCH and HARQ Codebook Enhancements	Apple

R1-2105158	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Sony

R1-2105259	Discussion on PDSCH enhancements supporting NR from 52.6GHz to 71 GHz	NEC

R1-2105299	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105372	HARQ codebook design for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105396	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz	Panasonic Corporation

R1-2105421	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

R1-2105497	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105556	PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

R1-2105583	Enhancing PDSCH/PUSCH Scheduling for 52.6 GHz to 71 GHz Band	InterDigital, Inc.

R1-2105596	PDSCH Considerations for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105690	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105784	PDSCH-PUSCH Enhancement for NR beyond 52.6 GHz	Charter Communications

R1-2105870	Discussion on multi-PDSCH/PUSCH scheduling for NR from 52.6GHz to 71GHz	WILUS Inc.

[105-e-NR-52-71GHz-03] Email discussion/approval on scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, with checkpoints for agreements on May 25, May 27 – Seonwook (LGE)

Agreement:

- Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling.

- Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI.

- Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI.

Proposal:

For enhancement of TDRA in a DCI that can schedule multiple PDSCHs or PUSCHs,

- Support a gap larger than 1 slot between consecutive PDSCHs or PUSCHs.

  - FFS: The maximum value of the gap

  - FFS: Details to inform the slot gap, e.g.,

    - K0/K2 per SLIV

    - A single value of K0/K2 per row and distance between PXSCHs per SLIV

    - A single value of k0/k2 per row and other methods

Proposal:

- At least for 480/960 kHz SCSs, at most one PDSCH can be scheduled in a slot by the DCI.

- At least for 480/960 kHz SCSs, at most one PUSCH can be scheduled in a slot by the DCI.

Conclusion:

For a DCI that can schedule multiple PUSCHs,

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Agreement:

- If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH.

  - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH.

  - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

### Channel access mechanism

R1-2104213	Channel access for shared spectrum Beyond 52.6 GHz	FUTUREWEI

R1-2104275	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

R1-2104351	Discussions on channel access mechanism for NR operation from 52.6GHz to 71 GHz	vivo

R1-2104419	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

R1-2104455	Channel access mechanism	Nokia, Nokia Shanghai Bell

R1-2104463	Channel Access Mechanisms	Ericsson

R1-2104510	Channel access mechanism for up to 71GHz operation	CATT

R1-2104662	Channel access mechanism for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104720	Discussions on channel access mechanism enhancements for 52.6G-71 GHz	CAICT

R1-2104768	Discussion on channel access mechanism	OPPO

R1-2104836	Discussion on the channel access for 52.6 to 71GHz	ZTE, Sanechips

R1-2104897	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

R1-2104953	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105010	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105063	Considerations on channel access mechanism for NR  from 52.6GHz to 71 GHz	Fujitsu

R1-2105095	Channel access mechanism	Apple

R1-2105145	Channel access for multi-beam operation	Panasonic

R1-2105159	Channel access mechanism for 60 GHz unlicensed spectrum	Sony

R1-2105261	Discussion on channel access mechanism supporting NR from 52.6 to 71GHz	NEC

R1-2105300	Channel access mechanism for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105371	On the channel access mechanisms for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105423	Channel access mechanism to support NR above 52.6 GHz	LG Electronics

R1-2105498	Channel access mechanisms for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105557	Discussion on channel access mechanism for NR on 52.6-71 GHz	Xiaomi

R1-2105584	Discussion on channel access mechanisms	InterDigital, Inc.

R1-2105597	On Channel Access Mechanism for NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105661	On receiver assisted channel access and directional LBT	AT&T

R1-2105691	Channel access mechanism for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105755	Discussion on multi-beam operation	ITRI

R1-2105785	Channel access mechanisms for above 52.6 GHz	Charter Communications

R1-2105871	Discussion on channel access mechanism for NR from 52.6GHz to 71GHz	WILUS Inc.

R1-2105986

R1-2106165

R1-2106193

[105-e-NR-52-71GHz-04] Email discussion/approval on channel access mechanism with checkpoints for agreements on May 25, May 27 – Jing (Qualcomm)

Proposal:

Confirm the following modified version of the prior working assumption:

For Pout in EDT determination during a COT, define Pout as the maximum of the mean EIRP of each transmission burst of the node determining EDT.

Agreement:

For energy measurement in 8us deferral period, continue down-selection between the following alternatives

- Alt 1. Two energy measurements are required, with one measurement in the first 3us and one measurement in the last 5us

- Alt 2. One measurement is required

  - FFS where the measurement is located

Note: By implementation, it is possible to support longer than 8us deferral period (Intend to cover Alt 3 as implementation choice for either Alt 1 or Alt 2)

Agreement:

On maximum gap within a COT to allow COT sharing without LBT, down-select or support both of the following two alternatives

- Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration

- Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

Agreement:

- Contention Exempt Short Control Signaling rules apply to the transmission of msg1 and/or msg3 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS.

  - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals)

  - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell

  - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msg3/msgA transmission from one UE perspective

- FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

### Others

Including analysis or recommendation to RAN#92e (June) on how to introduce the 52.6-71GHz frequency range

R1-2104352	Discussions on frequency range definition and sync raster for 52.6-71GHz	vivo

R1-2104456	Extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

R1-2104464	Introduction of the 52.6 to 71 GHz frequency range	Ericsson

R1-2104511	Views on Frequency Range for 52.6 to 71 GHz	CATT

R1-2104769	Discussion on other aspects	OPPO

R1-2104837	Discussion on the frequency range definition for above 52.6GHz	ZTE, Sanechips

R1-2104898	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

R1-2104954	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

Withdrawn

R1-2105011	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

Withdrawn

R1-2105096	On How to Introduce the 52.6-71GHz Frequency Range	Apple

R1-2105301	Discussion on defining frequency range for NR from 52.6 to 71 GHz	Samsung

R1-2105373	Frequency range naming discussion for 52.6-71 GHz	MediaTek Inc.

R1-2105424	Discussion on frequency range definition to support NR above 52.6 GHz	LG Electronics

R1-2105533	Discussion on the frequency range for bands in 52.6-71 GHz	Huawei, HiSilicon

R1-2105585	Efficient beam management for NR in 52.6 – 71 GHz	InterDigital, Inc.

R1-2105692	Views on how to introduce 52.6 - 71 GHz frequency range in NR	NTT DOCOMO, INC.

R1-2105787	Discussion on how to introduce the 52.6-71GHz frequency range	Charter Communications

[105-e-NR-52-71GHz-05] Email discussion/approval focusing on analysis or recommendation to RAN#92e (June) on how to introduce the 52.6-71GHz frequency range with checkpoints for agreements on May 25, May 27 – Alex (Lenovo)

R1-2106278

R1-2106276	DRAFT LS on how to introduce the 52.6-71GHz frequency range DRAFT LS on how to introduce the 52.6-71GHz frequency range

Final LS endorsed in R1-2106277

Conclusion:

Regarding the impact on specifications maintained by RAN1, there are only relatively small differences between potential options

Conclusion:

RAN1 can adapt to other groups' preferences on the notation for 52.6-71 GHz.

Conclusion:

Regardless of if the frequency range 52.6 to 71 GHz is an extension of FR2 or a new FR, the related UE capabilities and their applicability to the frequency range 52.6 to 71 GHz will have to be analyzed on a case by case basis

Conclusion:

From RAN1's perspective, an easy distinction between features, capabilities and other characteristics in the specifications that apply specifically to the following ranges may be useful:

- 24.25-52.6 GHz (NOTE: this is the current definition of FR2)

- 52.6-71 GHz

- 24.25-71 GHz

NOTE: The distinction can be realized with or without a designated frequency range (FRx) definition.

Include above conclusion with the note in the LS to RAN.

Conclusion:

Regardless of if the frequency range 52.6 to 71 GHz is an extension of FR2 or a new FR, the application of any of the UE feature introduced for 52.6-71 GHz to existing FR1/FR2 should be discussed case by case.

NOTE: This conclusion will not be included in the LS to RAN.