## NR Positioning Enhancements

Please refer to RP-210903 for detailed scope of the WI

[105-e-NR-ePos-LS-01] Email discussion/approval for the reply LS to R1-2102306 till 5/25 (Alex, Qualcomm)

### Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Including DL, UL and DL+UL positioning methods, and UE-based and UE-assisted positioning solutions.

R1-2104277	Enhancement to mitigate gNB and UE Rx/Tx timing error	Huawei, HiSilicon

R1-2104359	Discussion on  potential enhancements for RX/TX timing delay mitigating	vivo

R1-2104520	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays			CATT

R1-2104590	Positioning accuracy improvement by mitigating timing delay	ZTE

R1-2104611	Discussion on mitigation of gNB/UE Rx/Tx timing errors	CMCC

R1-2104671	Enhancements on Timing Error Mitigations for improved Accuracy	Qualcomm Incorporated

R1-2104739	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays		OPPO

R1-2104871	Discussion on accuracy improvements by mitigating timing delays	InterDigital, Inc.

R1-2104905	Mitigation of UE/gNB TX/RX Timing Errors	Intel Corporation

R1-2104961	Mitigation of UE/gNB TX/RX Timing Errors	Intel Corporation

Withdrawn

R1-2105018	Mitigation of UE/gNB TX/RX Timing Errors	Intel Corporation

Withdrawn

R1-2105105	Positioning accuracy enhancements under timing errors	Apple

R1-2105168	Discussion on mitigating UE Rx/Tx and gNB Rx/Tx timing delays	Sony

R1-2105310	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays			Samsung

R1-2105482	Discussion on accuracy improvement by mitigating UE Rx/Tx and gNB Rx/Tx timing delays	LG Electronics

R1-2105512	Views on mitigating UE and gNB Rx/Tx timing errors	Nokia, Nokia Shanghai Bell

R1-2105699	Discussion on mitigating UE and gNB Rx/Tx timing delays	NTT DOCOMO, INC.

R1-2105759	Mitigation of RX/TX timing delays for higher accuracy	MediaTek Inc.

R1-2105856	On methods for Rx/Tx timing delays mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2105859	Enhancements for mitigation of Tx/Rx Delays	Lenovo, Motorola Mobility

R1-2105908	Techniques mitigating Rx/Tx timing delays	Ericsson

R1-2105967

[105-e-NR-ePos-01] Email discussion/approval on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays with checkpoints for agreements on May 25, May 27 – Ren Da (CATT)

Agreement:

- For mitigating UE Tx timing errors for UL TDOA, support  one of the following options:

  - Option 1:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

    - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs

  - Option 2:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

    - Support the serving gNB to forward the association information provided by the UE to the LMF

    - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

- FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling

- Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

Proposal:

For mitigating UE Tx/Rx timing errors for DL+UL positioning, support a UE to provide the association information of a UE Rx-Tx time difference measurement with a UE RxTx TEG to LMF. The UE RxTx TEG is associated with one or more UE {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS.

- FFS: Whether RxTx TEG can be reported subject to UE capability

Proposal:

For mitigating UE Tx/Rx timing errors for DL+UL positioning, support a UE to provide the association information of a UE Rx-Tx time difference measurement with one or more of UE RxTx TEG, Rx TEG, Tx TEG to LMF. The UE RxTx TEG is associated with one or more UE {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to determine the Tx timing of the measurement.

### Accuracy improvements for UL-AoA positioning solutions

R1-2104278	Enhancement for UL AoA positioning	Huawei, HiSilicon

R1-2104360	Discussion on potential enhancements for UL-AoA method	vivo

R1-2104521	Discussion on accuracy improvements for UL-AoA positioning solutions	CATT

R1-2104591	Accuracy improvement for UL-AoA positioning solutions	ZTE

R1-2104612	Discussion on UL-AoA enhancements	CMCC

R1-2104672	Potential Enhancements on UL-AOA positioning	Qualcomm Incorporated

R1-2104740	Enhancements for UL AoA Positioning	OPPO

R1-2104872	Discussion on enhancements for UL-AoA positioning solutions	InterDigital, Inc.

R1-2104906	NR Positioning UL-AoA Enhancements	Intel Corporation

R1-2104962	NR Positioning UL-AoA Enhancements	Intel Corporation

Withdrawn

R1-2105019	NR Positioning UL-AoA Enhancements	Intel Corporation

Withdrawn

R1-2105106	Positioning Accuracy enhancements for UL-AoA	Apple

R1-2105169	Discussion on accuracy improvements for UL-AoA positioning method	Sony

R1-2105311	Discussion on accuracy improvements for UL-AoA positioning solutions	Samsung

R1-2105483	Discussion on accuracy improvement for UL-AoA positioning	LG Electronics

R1-2105513	Views on enhancing UL AoA	Nokia, Nokia Shanghai Bell

R1-2105658	Discussion on UL AoA positioning enhancements	PML

R1-2105700	Discussion on UL-AoA positioning enhancements	NTT DOCOMO, INC.

R1-2105857	UL-AoA positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2105909	Enhancements of UL-AoA positioning solutions	Ericsson

R1-2106021

[105-e-NR-ePos-02] Email discussion/approval on accuracy improvements for UL-AoA positioning solutions and with lower priority any issues related to the Others section, with checkpoints for agreements on May 24, May 27 – Alexey (Intel)

Agreement:

- Both GCS and LCS are supported for UL AoA/ZoA assistance information indication.

- Note: Existing signaling can be used for obtaining LCS to GCS translation information

Agreement:

- Granularity of 0.1 degrees is applied for the expected AoA (φAOA), expected ZoA (θZOA ) and the corresponding uncertainty values

Agreement:

LMF to gNB signaling of UL AoA/ZoA assistance information (expected value and uncertainty range) is supported for UL-TDOA and Multi-RTT positioning methods

Agreement:

Send an LS to RAN3 (potentially also to RAN2 at least as cc) capturing RAN1 agreements on UL AOA/ZOA assistance information (expected value and uncertainty range) and request them to define signaling

### Accuracy improvements for DL-AoD positioning solutions

Including UE-based and network-based (including UE-assisted) positioning solutions

R1-2104228	Accuracy improvements for DL-AoD positioning solutions 	BUPT

R1-2104279	Enhancement for DL AoD positioning	Huawei, HiSilicon

R1-2104361	Discussion on potential enhancements for DL-AoD method	vivo

R1-2104522	Discussion on accuracy improvements for DL-AoD positioning solutions	CATT

R1-2104592	Accuracy improvements for DL-AoD positioning solutions	ZTE

R1-2104613	Discussion on DL-AoD enhancements	CMCC

R1-2104673	Potential Enhancements on DL-AoD positioning	Qualcomm Incorporated

R1-2104741	Enhancements for DL-AoD positioning	OPPO

R1-2104842	Discussion on enhancements for DL-AoD positioning	CAICT

R1-2104844	Carrier Phase Based Downlink Angle of Departure Measurement 	DanKook University

Late submission

R1-2104873	Discussion on enhancements for DL-AoD positioning solutions	InterDigital, Inc.

R1-2104907	NR Positioning DL-AoD Enhancements	Intel Corporation

R1-2104963	NR Positioning DL-AoD Enhancements	Intel Corporation

Withdrawn

R1-2105020	NR Positioning DL-AoD Enhancements	Intel Corporation

Withdrawn

R1-2105107	Positioning Accuracy enhancements for DL-AoD	Apple

R1-2105170	Discussion on accuracy improvements for DL-AoD positioning method	Sony

R1-2105312	Discussion on accuracy improvements for DL-AoD positioning solutions	Samsung

R1-2105484	Discussion on accuracy improvement for DL-AoD positioning	LG Electronics

R1-2105514	Views on enhancing DL AoD	Nokia, Nokia Shanghai Bell

R1-2105563	Accuracy improvements for DL-AoD positioning solutions	Xiaomi

R1-2105701	Discussion on DL-AoD positioning enhancements	NTT DOCOMO, INC.

R1-2105858	DL-AoD positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2105860	DL-AoD Positioning Enhancements	Lenovo, Motorola Mobility

R1-2105910	Enhancements of DL-AoD positioning solutions	Ericsson

R1-2106079

[105-e-NR-ePos-03] Email discussion/approval on accuracy improvements for DL-AoD positioning solutions with checkpoints for agreements on May 25, May 27 – Florent (Ericsson)

Agreement:

For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) the PRS RSRP of the first path

- FFS: Details of measurement and reporting of PRS RSRP of the first path

### Latency improvements for both DL and DL+UL positioning methods

R1-2104280	Positioning latency enhancements	Huawei, HiSilicon

R1-2104362	Discussion on latency enhancement for NR positioning	vivo

R1-2104523	Discussion on latency improvements for both DL and DL+UL positioning methods	CATT

R1-2104593	Discussion on latency reduction for NR positioning	ZTE

R1-2104614	Discussion on latency improvement for positioning	CMCC

R1-2104674	Enhancements for Latency Improvements for Positioning	Qualcomm Incorporated

R1-2104742	Enhancements on Latency Reduction in NR Positioning	OPPO

R1-2104874	Discussion on latency improvements for DL and DL+UL positioning methods	InterDigital, Inc.

R1-2104908	NR Positioning Latency Reduction	Intel Corporation

R1-2104964	NR Positioning Latency Reduction	Intel Corporation

Withdrawn

R1-2105021	NR Positioning Latency Reduction	Intel Corporation

Withdrawn

R1-2105108	Views on Rel-17 positioning latency reduction	Apple

R1-2105171	Considerations on Latency Improvements for DL and DL+UL positioning methods	Sony

R1-2105313	Discussion on latency improvements for both DL and DL+UL positioning methods	Samsung

R1-2105485	Discussion on latency improvements for NR positioning	LG Electronics

R1-2105515	Views on PHY Latency Reductions	Nokia, Nokia Shanghai Bell

R1-2105564	Latency improvements for both DL and DL+UL positioning method	Xiaomi

R1-2105760	Aspects for physical latency improvement	MediaTek Inc.

R1-2105861	Positioning Latency Reduction Enhancements	Lenovo, Motorola Mobility

R1-2105911	Latency improvements for both DL and DL+UL positioning methods	Ericsson

R1-2105989

[105-e-NR-ePos-04] Email discussion/approval on latency improvements for both DL and DL+UL positioning methods with checkpoints for agreements on May 24, May 27 – Su (Huawei)

Proposal:

Single-sample PRS processing subject to UE capability is supported from RAN1 perspective.

- Send an LS to RAN4 informing that

  - Single-sample measurements corresponding to measurements performed within a single instance of the DL PRS resource set on a PRS resource are beneficial for reduction of measurement latency from RAN1 point of view.

  - RAN4 is requested to check the feasibility of measurements performed within a single instance of the DL PRS resource set and identify the impact on requirements/side condition.

- RAN1 to further study the following aspects

  - Details of UE capability

  - Signaling details, e.g., to indicate whether measurement is based on one or more samples

  - Whether the PRS sample processing time is defined and the relation with (N, T).

Proposal:

- PRS measurement without MGs subject to UE capability is supported in Rel-17.

- The following aspects are FFS

  - PRS processing prioritization window

  - Mechanism to trigger UE DL PRS measurements and report

  - Bandwidth/numerology relationship and potential switching from(to) active DL BWP to(from) DL PRS bandwidth

  - UE/gNB assumptions on processing of DL PRS and other DL physical channels / signals

  - Potential restrictions on gNB behavior

  - UE DL PRS processing capabilities

  - Impact on deployment scenarios, including

    - Single gNB with multiple TRPs

    - Serving gNB and multiple neighbor gNBs

### Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

R1-2104198	NLOS Mitigation Enhancements	FUTUREWEI

R1-2104281	Enhancements to support multi-path and NLOS mitigation	Huawei, HiSilicon

R1-2104363	Discussion on potential enhancements for multipath/NLOS mitigation	vivo

R1-2104524	Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	CATT

R1-2104594	Enhancements on NLOS mitigation for NR positioning	ZTE

R1-2104675	Multipath Reporting in NR Positioning	Qualcomm Incorporated

R1-2104743	Discussion on multipath/NLOS mitigation for NR positioning	OPPO

R1-2104856	Potential enhancements of information reporting from UE for multipath/NLOS mitigation	China Telecom

R1-2104875	Discussion on multipath/NLOS mitigation for positioning	InterDigital, Inc.

R1-2104909	Mitigation of NLOS Problem for NR Positioning	Intel Corporation

R1-2104965	Mitigation of NLOS Problem for NR Positioning	Intel Corporation

Withdrawn

R1-2105022	Mitigation of NLOS Problem for NR Positioning	Intel Corporation

Withdrawn

R1-2105109	Views on potential enhancements for NLOS mitigation in Rel-17 positioning	Apple

R1-2105172	Discussion on enhanced reporting from UE and gNB for Multipath/NLOS mitigation	Sony

R1-2105314	Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	Samsung

R1-2105486	Discussion on multipath/NLOS mitigation for positioning	LG Electronics

R1-2105516	Views on LoS/NLoS Identification and Mitigation	Nokia, Nokia Shanghai Bell

R1-2105565	Potential enhancements for multipath/NLOS mitigation	Xiaomi

R1-2105702	Discussion on multipath/NLOS mitigation for NR positioning	NTT DOCOMO, INC.

R1-2105862	Accuracy enhancements based on NLOS/Multipath Information Reporting	Lenovo, Motorola Mobility

R1-2105865	Potential positioning enhancements for multipath/NLOS mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2105912	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation		Ericsson

R1-2106043

[105-e-NR-ePos-05] Email discussion/approval on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation with checkpoints for agreements on May 25, May 27 – Ryan (Nokia)

Agreement:

- Study reporting of LoS/NLoS indicators for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

- Study the following options (or combinations of the following options) for LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

  - FFS: additional information or options

- FFS: LoS/NLoS indicators for UE-based positioning

- Agreement:

- Study multipath reporting enhancements for DL, UL, and DL+UL positioning to enable LoS/NLoS/multipath identification and mitigation at the LMF for UE-assisted positioning.

  - FFS: Details of the enhancements.

### Others

Including aspects for RAN2-led on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, and RAN2-led methods/measurements/signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions

R1-2104364	Discussion on inactive state positioning and on-demand PRS	vivo

R1-2104525	Discussion on on-demand transmission and reception of DL PRS and positioning solutions for UEs in RRC_ INACTIVE state	CATT

R1-2104595	Discussion on items led by RAN2 for NR positioning	ZTE

R1-2104615	Discussion on NW/UE efficiency enhancements	CMCC

R1-2104676	Enhancements Related to On Demand PRS And Positioning in RRC Inactive State	Qualcomm Incorporated

R1-2104744	Discussion on positioning for UE in RRC_INACTIVE and on-demand PRS	OPPO

R1-2104876	Discussion on on-demand PRS and INACTIVE mode positioning	InterDigital, Inc.

R1-2104880	Carrier/Subcarrier Phase Based Enhancement for 5G NR Positioning	DanKook University

Late submission

R1-2104910	Support of On-demand DL PRS and NR Positioning for UEs in RRC-INACTIVE state	Intel Corporation

R1-2104966	Support of On-demand DL PRS and NR Positioning for UEs in RRC-INACTIVE state	Intel Corporation

Withdrawn

R1-2105023	Support of On-demand DL PRS and NR Positioning for UEs in RRC-INACTIVE state	Intel Corporation

Withdrawn

R1-2105315	Discussion on positioning in inactive state	Samsung

R1-2105487	Discussion on other enhancements for positioning	LG Electronics

R1-2105517	Additional views on Inactive Mode Positioning and on-demand PRS	Nokia, Nokia Shanghai Bell

R1-2105534	Discussion on UL and DL+UL positioning in INACTIVE state	Huawei, HiSilicon

R1-2105566	On-demand PRS and positioning for in-active state UE	Xiaomi

R1-2105892	Potential physical layer impact to the RAN2-led topics	MediaTek Inc.

R1-2105913	On-demand transmission and reception of DL PRS for DL and DL+UL positioning	Ericsson