3GPP TSG RAN WG1 #105-e	R1-2106050

e-Meeting, May 19 - 27, 2021

Source:	Moderator (OPPO)

Title:	Summary#1 of email thread [105-e-NR-L1enh-URLLC-02]

Agenda Item:	7.2.5

Document for:	Discussion and Decision

# Introduction

In this paper, discussions under the following email thread in RAN1#105-e are summarized.

[105-e-NR-L1enh-URLLC-02] Email discussion/approval on remaining issues on UCI enhancements – Jia (OPPO):

- Issue #1: Correction for sub-slot based PUCCH

- Issue #2: Conflict between the first PUCCH repetition and semi-static configuration

- Issue #3: Clarification on reference point of sub-slot based PUCCH resource

- Issue #5 (editor CR): Editorial correction on timing for secondary cell activation/deactivation

- Discussion and decision by May 24, TPs by May 27

# Remaining issues

## Issue#1: Correction for sub-slot based PUCCH

## Inputs from Tdocs

This issue was discussed in the past two meetings. The following CR was proposed by end of last meeting. It was supported by 6 companies, but still receives concerns from 3 companies. The CR was not approved.

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH. *** Unchanged text is omitted *** ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

In this meeting, some companies re-submitted the proposal. The observation is: Not all the slots in section 9 should be replaced by sub-slot when a UE is provided subslotLength-ForPUCCH

Nokia proposal:

Proposal 1: Adopt the following draft TP from Samsung

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH. *** Unchanged text is omitted *** ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

CATT proposal:

Proposal 2: Adopt the following TP for section 9 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

<Unchanged text omitted>

In the remaining of this Clause except for section 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH.

----------------------------------------------------- End of text proposal ------------------------------------------------------

vivo proposal:

Proposal 1: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this Clause except for Clauses 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal -----------------------------------------------

## 1st round proposal and discussion

It is suggested to start the discussion from the status of last meeting for this issue. The following CR was supported by most of companies, but received concerns from some others in last meeting. Companies are encouraged to express your views:

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH. *** Unchanged text is omitted *** ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

| Company | Comments |
| --- | --- |
| HW/HiSi | We are ok with the intention of the TP. Our concern is the handling of multiple CSI PUCCHs which had been raised by vivo last meeting. Obviously, this should be done in slot basis, but with the proposed TP it is done in sub-slot basis. In the last meeting the “with HARQ-ACK information” was deleted due to a late comment. In our view, this part is needed, otherwise it could be misunderstood that multiple CSI PUCCH is handled on sub-slot basis. Therefore, we prefer to change the TP back to: “In the remaining of this Clause, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLength-ForPUCCH.” |

## Issue#2: Conflict between the first PUCCH repetition and semi-static configuration

## Inputs from Tdocs

This issue was discussed in the past two meetings. The latest status in last meeting was the following draft conclusion. But due to some question on clarification for R15 behavior, the conclusion was not approved.

Potential conclusion:

It is clarified that, according to the running R15 and R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with  SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

In this meeting, some companies re-submitted the contribution for clarify this issue.

Nokia proposal:

Proposal 2: Include in the Chairman’s Notes the clarification
“It is clarified that, according to the running R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with  SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.”

CATT proposal:

Proposal 1: It is clarified that, according to the running R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- There may be different understandings/implementations for Rel-15 if the first PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

vivo proposal:

First of all, we agree that the current specification does not preclude that the first PUCCH repetition of a scheduled PUCCH to collide with semi-static DL symbol(s) or SSB symbols. Then, some clarifications for the proposed conclusion:

- It is only applicable for the case that PUCCH is configured with repetition, i.e.,  If a PUCCH is configured with no repetition or repetition factor is 1, i.e. , the UE would not defer the PUCCH transmission, and it is meaningless to schedule a PUCCH which is to be cancelled. If PUCCH is configured with repetition, i.e., , gNB is allowed to schedule a PUCCH with the first repetition collided with semi-static DL symbol(s) or SSB symbols. In this case, the UE still can transmit the PUCCH in the subsequent slots.

- The postpone behaviour has no impact on HARQ-ACK codebook construction, i.e., the PUCCH repetition is postponed after the corresponding HARQ-ACK codebook is generated.

- This conclusion can apply to both Rel-15 and Rel-16.

Huawei proposal:

Proposal 1: Conclude that,

It is clarified that, according to the running R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

## 1st round proposal and discussion

It is suggested to start the discussion from the status of last meeting for this issue, which was the following draft conclusion. Companies are encouraged to express your views:

Potential conclusion:

It is clarified that, according to the running R15 and R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with  SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

| Company | Comments |
| --- | --- |
| HW/HiSi | We think that “UL slot” should be changed “slot”. The term “UL slot” means one slot consists of only UL symbols. But for the deferral of PUCCH repetition, it can also be deferred to a slot consisting of flexible symbols or DL symbols, as long as all the symbols of the PUCCH resource for the deferred PUCCH repetition are UL symbols. We therefore suggest to change the conclusion as follows: “It is clarified that, according to the running R15 and R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.” |

## Issue#3: Clarification on reference point of sub-slot based PUCCH resource

## Inputs from Tdocs

In last meeting, an issue for reference point of sub-slot based HARQ-ACK PUCCH resource was discussed [5]. The proponents think if HARQ-ACK information bits are multiplexed in a PUCCH resource in multi-CSI-PUCCH-ResourceList, the specification below is also applied to the PUCCH in multi-CSI-PUCCH-ResourceList. However, the specification intended to cover PUCCH resources configured for HARQ-ACK feedback only, but currently it also cover a PUCCH resource configured for CSI when HARQ-ACK is multiplexed in a PUCCH resource in multi-CSI-PUCCH-ResourceList or pucch-CSI-ResourceList. In addition, it also covers a PUCCH resource configured for SR when 1/2-bits HARQ-ACK is multiplexed in a PUCCH resource in SchedulingRequestResourceConfig.

From TS 38.213 Section 9: If a UE is provided subslotLengthForPUCCH in a PUCCH-Config, the first symbol of a PUCCH resource in PUCCH-Config for multiplexing HARQ-ACK in a PUCCH transmission is relative to the first symbol of the subslotLengthForPUCCH symbols [12, TS 38.331]. For the remaining cases, the first symbol of a PUCCH resource is relative to the first symbol of a slot with symbols [4, TS 38.211].

CATT proposal:

To guarantee that only PUCCH resources configured for HARQ-ACK reference sub-slot boundary, the cases that HARQ-ACK is multiplexed on a PUCCH resource in multi-CSI-PUCCH-ResoruceList, pucch-CSI-ResourceList and SchedulingRequestResourceConfig should be excluded in the specification. Regarding a PUCCH resource with a same PUCCH-ResourceId is configured for both CSI/SR and HARQ-ACK, we think a PUCCH resource configuration can be interpreted differently according to where the resource is used, but to make it clear, we agree the latest TP given by moderator at the end of last meeting considering how the PUCCH resource is provided.

Proposal 3: Adopt the following TP for section 9.2.1 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.2.1	PUCCH Resource Sets

<Unchanged text omitted>

If a UE is provided subslotLengthForPUCCH in a PUCCH-Config, the first symbol of a PUCCH resource in PUCCH-Config except when the PUCCH resource is provided by pucch-ResourceId in multi-CSI-PUCCH-ResoruceList, pucch-CSI-ResourceList and SchedulingRequestResourceConfig, for multiplexing HARQ-ACK in a PUCCH transmission is relative to the first symbol of the subslotLengthForPUCCH symbols [12, TS 38.331]. For the remaining cases, the first symbol of a PUCCH resource is relative to the first symbol of a slot with  symbols [4, TS 38.211].

<Unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

vivo proposal:

In the last meeting, the issue of multiplexing of SPS HARQ-ACK and CSI was solved, and one related issue was raised regarding reference of the first symbol of a PUCCH resource when sub-slot is configured Error: Reference source not found. There is a common understanding that the sub-slot reference applies only to initial HARQ-ACK PUCCH resource, i.e., PUCCH resources configured by PUCCH-ResourceSet, SPS-PUCCH-AN-List, or n1PUCCH-AN. For the PUCCH resources configured for CSI and SR, i.e., the PUCCH resource configured by multi-CSI-PUCCH-ResoruceList, pucch-CSI-ResourceList and SchedulingRequestResourceConfig, the slot reference is applied. However, in according to the current specification, it is possible that the same pucch-ResourceId is configured in PUCCH-ResourceSet, SPS-PUCCH-AN-List, n1PUCCH-AN and multi-CSI-PUCCH-ResoruceList, pucch-CSI-ResourceList and SchedulingRequestResourceConfig. In that case, when the PUCCH resource is used for CSI/SR transmission, the slot reference should be applied, and when the PUCCH resource is used for initial HARQ-ACK transmission, the sub-slot reference should be applied. Which means, for the PUCCH resource with the same pucch-ResourceId, its starting symbol can have different references when it is configured in different IEs. To make it clear, the following text proposal is made.

Proposal 1: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.Proposal 1: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.2.1	PUCCH Resource Sets

*** Unchanged text is omitted ***

If a UE is provided subslotLengthForPUCCH in a PUCCH-Config, the first symbol of a PUCCH resource provided by PUCCH-ResourceSet, SPS-PUCCH-AN-List, or n1PUCCH-AN in PUCCH-Config for multiplexing HARQ-ACK in a PUCCH transmission is relative to the first symbol of the subslotLengthForPUCCH symbols [12, TS 38.331]. For a PUCCH resource provided by multi-CSI-PUCCH-ResoruceList, pucch-CSI-ResourceList and SchedulingRequestResourceConfig in PUCCH-Config, the first symbol of a PUCCH resource is relative to the first symbol of a slot with  symbols [4, TS 38.211].

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal --------------------------------------------------------

Huawei proposal:

Regarding the TP proposed above, a concern that was raised is that it would prevent a PUCCH-Resource ID to be configured for both CSI and HARQ-ACK, which is allowed in Rel-15. We think it is still up to gNB’s implementation in Rel-16 to decide when sub-slot is configured. For example, if the startingSymbolIndex of one PUCCH resource is less than the subslotLengthForPUCCH and the PUCCH resource is confined within the sub-slot, such PUCCH resource can be configured for both CSI and HARQ-ACK. If the startingSymbolIndex of one PUCCH resource is equal to or larger than the subslotLengthForPUCCH or if one PUCCH resource is across the sub-slot boundary, such PUCCH resource can only be configured for CSI.

Based on the above discussion, we think whether to configure a PUCCH resource with the same PUCCH-Resource ID for both HARQ-ACK and CSI when sub-slot is configured can also be left for gNB’s implementation, and no spec change is needed.

Proposal 2: Conclude that

Whether to configure a PUCCH resource with the same PUCCH-Resource ID for both HARQ-ACK and CSI when sub-slot is configured can be left for gNB’s implementation, and no spec change is needed.

## 1st round proposal and discussion

The discussion can be started based on proposal from CATT. Companies are encouraged to express your views:

Proposal 3: Adopt the following TP for section 9.2.1 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.2.1	PUCCH Resource Sets

<Unchanged text omitted>

If a UE is provided subslotLengthForPUCCH in a PUCCH-Config, the first symbol of a PUCCH resource in PUCCH-Config except when the PUCCH resource is provided by pucch-ResourceId in multi-CSI-PUCCH-ResoruceList, pucch-CSI-ResourceList and SchedulingRequestResourceConfig, for multiplexing HARQ-ACK in a PUCCH transmission is relative to the first symbol of the subslotLengthForPUCCH symbols [12, TS 38.331]. For the remaining cases, the first symbol of a PUCCH resource is relative to the first symbol of a slot with  symbols [4, TS 38.211].

<Unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

| Company | Comments |
| --- | --- |
| HW/HiSi | We are ok with the proposed CR |

## Issue #5 (editor CR): Editorial correction on timing for secondary cell activation/deactivation

## Background

There is a typo error in the formula in section 4.3 of TS38.213, in which the multiplication sign is written as a subscript point. A correction is needed as below.

## 1st round proposal and discussion

Companies are encouraged to check the CR proposed by CATT for the editorial correction:

Proposal 4: Adopt the following TP for section 4.3 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissions each consisting of  symbols as defined in [4, TS 38.211], when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is  where slot n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in Clause 9.2.3 and  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission as defined in [4, TS 38.211].

With reference to slots for PUCCH transmissions each consisting of  symbols as defined in [4, TS 38.211], if a UE receives a deactivation command [11, TS 38.321] for a secondary cell ending in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot .

If the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in the first slot that is after slot  where  is the SCS configuration for PDSCH reception on the secondary cell.

----------------------------------------------------- End of text proposal ------------------------------------------------------

| Company | Comments |
| --- | --- |
| HW/HiSi | We are fine with the CR, to give it directly to the editor |

# References

- R1-2104311	Rel-16 URLLC/IIoT maintenance of UCI enhancements	Nokia, Nokia Shanghai Bell

- R1-2104480	Remaining issues on PUCCH transmission	CATT

- R1-2105466	Maintenance on UCI enhancement	vivo

- R1-2105531	Remaining issues on UCI enhancements and SPS	Huawei, HiSilicon

- R1-2104126, Final Summary of [104b-e-NR-L1enh-URLLC-05] on other aspects for URLLC and IIoT, Moderator (LG Electronics)