3GPP TSG RAN WG1 Meeting #106-e	R1-2106447

E-meeting, August 16th -27th, 2021

Agenda Item:	8.2.6

Source:	Huawei, HiSilicon

Title:	Channel access mechanism for 60 GHz unlicensed operation

Document for:	Discussion and Decision

Introduction

In the last plenary meeting RAN#92-e, the objectives for the physical layer procedures were not updated in the revised WID [1]. In this contribution, we discuss the channel access mechanisms with LBT and the channel access mechanisms without LBT for operation in the 60 GHz unlicensed band in light of the latest agreements and email discussions.

Channel access mechanisms with LBT

Enhancements to baseline Energy Detection Threshold

One objective of the WID [1] is to study, and if needed specify, energy detection threshold (EDT) enhancement.  In RAN1#104-e [2], it was agreed to update the baseline ED threshold formula based on the latest draft version of the HS EN 302 567 (v2.2.0) [3] as follows:

Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP

In RAN1#104bis-e [4], the following definition for Pout EIRP has been agreed as a working assumption:

Working assumption: For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

In subclause 4.2.2.1 of the HS EN 302 567, the following definition of the ‘RF output power’ is stated

4.2.2.1 Definition The RF output power is the mean equivalent isotropically radiated power (EIRP) for the equipment during a transmission burst.

As can be observed from the above definition, the RF output power is defined as the mean value of the EIRP over a transmission burst. It should be also understood from the following step 1) of the adaptivity requirements in subclause 4.2.5.3 that a ‘transmission burst’ contains a number of transmissions following the CCA check.

The LBT mechanism is as follows: 1) Before a single transmission or a burst of transmissions on an Operating Channel, the equipment that initiates transmission shall perform a Clear Channel Assessment (CCA) Check in the Operating Channel.

Nevertheless, in the last meeting RAN1#105-e, two issues were discussed without consensus and captured in the FL summary [6] for further discussion in this meeting:

- Whether to define Pout to be the maximum of mean EIRP of each transmission burst during the COT rather than the maximum EIRP during the COT

- Whether to consider the COT sharing node’s max EIRP

On the first issue, considering the difficulty of predicting/calculating the mean output power with dynamic scheduling, especially for a long transmission burst or multiple transmission bursts within a 5 ms COT, we think that adopting the “maximum of mean EIRP of each transmission burst” is not a practical approach.  This is due to the fact that it requires the gNB to know all scheduling decisions for up to 320 slots at 960 kHz before acquiring the COT. In contrast, defining Pout (EIRP) as the maximum output power over the transmission burst(s) of the COT is a more practical approach though more conservative.

Observation 1: Adopting the “maximum of mean EIRP of each transmission burst” is not a practical solution as it requires the gNB to know all scheduling decisions for up to 320 slots at 960 kHz before acquiring the COT.

On the second issue, we think that the maximum EIRP of the COT sharing node should not be considered in the calculation of Pout by the initiating device. This is because subclause 4.2.2.1 of  HS EN 302 567 [3] only considers the EIRP of the equipment during a transmission burst before which the equipment is required to perform the CCA check according to subclause 4.2.5.3. While transmissions from a responding equipment within the initiated COT are allowed by the same subclause (4.2.5.3), no impact of the responding device was stated on the determination of Pout in subclause 4.2.2.1. Given the above discussion, we propose to confirm the working assumption agreed in RAN1#104bis-e [4].

Proposal 1: For operation in NR-U-60, confirm the working assumptions on the definition of Pout in the previously agreed baseline EDT formula.

In the following, we discuss the remaining points that were left for further study.

Operating Channel Bandwidth: Although no specific value for the bandwidth of an operating channel is stated in the HS EN 302 567 [3], the definition of the term ‘operating channel’ is clearly stated in Section 3.1 of the same document as follows

operating channel: channel on which the RLAN equipment has started the Adaptivity mechanism to start transmissions

wherein the ‘Adaptivity mechanism’ is also defined as the LBT mechanism for spectrum sharing as follows

4.2.5.2 Definition Adaptivity (medium access protocol) is a mechanism designed to facilitate spectrum sharing with other devices in the wireless network. 4.2.5.3 Requirement Adaptivity (medium access protocol) shall be implemented by the equipment and shall be active under all circumstances. LBT is mandatory to facilitate spectrum sharing.

As such, it should be understood that the ‘operating channel bandwidth’ is the bandwidth of the channel on which LBT is performed by the equipment. We therefore propose to capture the definition of the term ‘Operating channel bandwidth’ in 3GPP specifications as the ‘LBT bandwidth,’ or following the terminology of TS 37.213; the ‘bandwidth on which a channel access procedure is performed in shared spectrum’.

Proposal 2: For operation in NR-U-60, the term ‘Operating Channel Bandwidth’ in the agreed baseline EDT formula is defined as the ‘LBT Bandwidth’ or the ‘bandwidth on which a channel access procedure is performed in shared spectrum’.

Further adjustment based on the transmission beam: As captured in the agreement above, this further adjustment to the agreed baseline formula is to be further studied under the constraint that the EDT regulatory requirements may not be violated. That is, the adjusted EDT level should not exceed the EDT level determined by the agreed baseline formula for the same Pout (EIRP) and Operating Channel BW (MHz). According to the agreed baseline formula, EDT decreases when the RF output power Pout (EIRP) increases. The devices with higher conducted transmit power but lower antenna gain will have larger impact area than the devices with lower conducted transmit power but higher antenna gain. The device with higher antenna gain should be thus encouraged due to less interference in un-targeted directions. However, the current EDT only reflects the impact from RF output power (EIRP) which cannot differentiate devices with different antenna gain but the same EIRP. Therefore, in line with the above agreement, we propose that the agreed baseline EDT formula should be adjusted by a term that is proportional to the effective beamforming gain of the subsequent transmission(s) such that if two antenna arrays have the same RF output power (EIRP), the antenna array with the higher beamforming gain also has a higher EDT.

Proposal 3: For operation in NR-U-60, the agreed baseline EDT formula should be adjusted such that, for a given RF output power (EIRP), the EDT proportionally increases with the effective beamforming gain of the potential following transmission(s) by the device.

Considering the constraint stated earlier on the adjusted EDT level not exceeding the regulatory level, we propose to adopt the following formula in Proposal 4 to capture the potential enhancement we discussed.

Proposal 4: For operation in NR-U-60, when LBT is used, adopt the following formula to capture the potential adjustment to the baseline EDT formula based on the transmit beamforming gain:

EDT=-80 dBm+10*log10PmaxPout+10*log10BW [MHz]+(1-a)(GTX -GTX,max)

  - GTX is the effective transmit antenna gain at the potential transmitter [dBi]

  - GTX,max is the maximum effective transmit antenna gain considered for the deployment [dBi]

  - a is a scaling factor such that  0≤ a≤ 1

Note that the effective transmit antenna gain GTX includes the overall gain of the antenna element and the antenna array (beamforming gain). The offset (1-a)GTX,max is subtracted to allow for proportional increase of the EDT, i.e. (1-a)GTX below the agreed regulatory level. Moreover, when a=1, the EDT is reduced to the agreed baseline EDT formula.

It can be easily observed from the adjusted EDT formula in Proposal 4 that, if a < 1, for two transmissions of the same LBT BW and the same Pout (EIRP), a higher EDT2 > EDT1 is used when GTX,2 > GTX,1:

EDT2= -80+10log10(BW [MHz])+Pmax [dBm]-Pout[dBm]  +(1-a) (GTX,2 - GTX,max )   >

EDT1= -80+10log10(BW [MHz])+Pmax [dBm]-Pout[dBm]  +(1-a) (GTX,1 - GTX,max )

During earlier email discussions, some companies argued that reducing the EDT below the regulatory level to achieve the proposed enhancement would reduce the channel access opportunities and thus would not be beneficial to the overall system performance. While we have explained above the benefits of differentiating between directional transmissions based on their potential interference footprint, it should be also noted that reducing the EDT below the regulatory level would not be a concern in light of the observations made by the same companies that the reference level (-47dBm) in the baseline formula is quite generous and could be often above the detected energy level during LBT.

Further adjustment based on the sensing beam: Finally, the agreed baseline formula does not account for whether the potential transmitter performs the channel sensing in a directional or an omni-directional manner. When a directional antenna is used in channel sensing, the received energy will be amplified at the main-lobe while attenuated at the side-lobes. As such, changing the beamforming gain of the sensing antenna pattern could dramatically change the detected energy level and thus the LBT result for the same interference instance. Note that it is defined in EN 301 893 v2.1.27 for the 5GHz band that “the received power shall be measured at the interface between the equipment and the antenna assembly” and EDT is related to the highest stated power level PH. The CCA in the 5GHz band is thus independent of the sensing antenna gain. Therefore, we propose to deduct the sensing beamforming gain of the LBT beam from the detected energy level when comparing it to the EDT.

Proposal 5: For operation in NR-U-60, when LBT is used, the sensing beamforming gain of the LBT beam is deducted from the detected energy level before comparing it to the EDT.

Detail design of the energy measurement periods

In RAN1#104-e, it was agreed to perform a single measurement within the 5us observation slot and to down select between three alternatives for performing either a single measurement or two measurements in the deferral period. In RAN1#104bis-e, it was agreed as a working assumption that the location of the single measurement within the 5us observation slot is implementation specific as follows:

Agreement: For energy measurement in 8us deferral period, down-select from the following: - Alt 1. Two energy measurements are required - Alt 2. One measurement is required - Alt 3. Extend the 8us to 10us and perform two measurements, one in each 5us segment For energy measurement in 5us observation slot, perform single measurement - FFS minimum duration of the measurement - FFS location of the measurement Working assumption: For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.

The duration of the single measurement within the 5us observation slot remained FFS but was not discussed in the previous meeting RAN1#105-e. Since typically the sensing duration of the CCA check procedure consists of a deferral period followed by a random number of observation slots, the observation slot should end with a sufficient turnaround time for the transceiver of the device to switch from sensing to transmission if the device can transmit immediately after the end of the last idle observation slot. It can be observed however that most of the corresponding parameters for 802.11ay (EDMG) have been left to implementation. Therefore, we propose that the duration of the single measurement within the 5us is also left to implementation for NR-U-60.

Proposal 6: Confirm the following WA reached in RAN1 #104bis-e:

“For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.”

Proposal 7: For operation in NR-U-60, when LBT is used, the measurement duration X us within the 5us observation is implementation specific.

In the previous meeting RAN1#105-e, it was agreed to continue down selecting between two energy measurements in the deferral period of 8us (Alt 1) and only one energy measurement (Alt 2), whereas Alt.3 in the agreement above from RAN1 #104-e could be supported by longer than 8us deferral period as an implementation choice:

Agreement: For energy measurement in 8us deferral period, continue down-selection between the following alternatives - Alt 1. Two energy measurements are required, with one measurement in the first 3us and one measurement in the last 5us - Alt 2. One measurement is required - FFS where the measurement is located Note: By implementation, it is possible to support longer than 8us deferral period (Intend to cover Alt 3 as implementation choice for either Alt 1 or Alt 2)

In our view, the deferral period, Td, should consist of a duration Tf immediately followed by one observation slot duration (5us). However, since Tf is only 3us long and even shorter than one observation slot, Tf should not include a measurement duration at the start of it. It is important to note that this is also the case with 802.11ad (DMG) and 802.11ay (EDMG) wherein the definition of the 3us SIFS does not include any sensing time. As such, Alt 2 in the above agreement should be supported. The channel is considered to be sensed idle for the deferral period Td if it is sensed idle during the observation slot duration in the last 5 us of the 8 us deferral period.

Given the majority support for a single measurement in the 8us deferral period (Alt 2), the group further discussed in RAN1#105-e following FL Proposal 2.3.3-1 as captured in [6]:

Proposal 2.3.3-1 (high priority) from [6]

For energy measurement in 8us deferral period, performs single measurement within 8us, the measurement duration is selected from one of the following alternatives:

- Alt 1: At least 3+X us (FFS X, such as X=1).

- Alt 2: At least X us, where X is the same as the minimum measurement duration in a 5 us observation slot

The proposal was motivated by the concern raised by some companies that the single measurement duration may fall in the 3us SIFS gap between two DMG/EDMG transmissions leading to a collision as such. However, it should be noted that according to the CCA check procedure in HS EN 302 567, the 8us deferral applies from the end of the last observation slot in which the channel is occupied as follows

4) CCA Check definition: a) A CCA check is initiated at the end of an operating channel occupied slot time. b) Upon observing that Operating Channel was not occupied for a minimum of 8 μs, transmission deferring shall occur.

Therefore, it is unlikely that the NR-U-60 device would only carry on the Xus measurement exactly within the 3us SIFS gap, let alone the fact that aa successful initial CCA during the deferral period is typically followed by subsequent observation slots in the eCCA phase in which the second DMG/EDMG transmission would be sensed.  We do not see a need as such for additional sensing in the deferral period beyond the X us measurement duration of the observation slot. Nevertheless, we think that Alt 2 in FL Proposal 2.3.3-1 is more aligned with our views than Alt 1 and can be supported for the sake of progress with the condition that any additional sensing to the X us of the observation slot is supported by implementation.

Proposal 8: For operation in NR-U-60, when LBT is used, support one energy measurement in the 8us deferral period.

- Td consists of a Tf duration of 3us immediately followed by a 5us observation slot duration

- Tf may include an additional measurement duration by implementation.

Enhancements to the LBT procedure

LBT bandwidth

The following agreement was made in the previous meeting RAN1#104bis-e in which some alternatives have been considered for further down selection in regards to the LBT bandwidth for both the single-carrier and the multi-carrier transmission cases:

Agreement: For LBT for single carrier transmission, continue down selection between - Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) - Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth For LBT for multi-carrier transmission in intra-band CA, continue down selection between - Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately - Alt CA.2. gNB/UE performs single LBT over all CCs - Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

In our view, LBT BW should be flexible and can be at least equal to the total transmission BW. This facilitates, for instance, performing only one LBT for the whole transmission BW instead of multiple parallel LBTs. This in fact would significantly reduce the computational complexity and energy consumption of the LBT in low density deployments.

Therefore, in the case of single-carrier transmission, we are supportive of Alt SC.1, i.e., gNB/UE performs LBT over the channel/BWP bandwidth. In contrast, we observe the following issues with Alt SC.3 in addition to the increased complexity and energy consumption of performing N parallel eCCA processes on N LBT BW units:

- Need to specify the guard bands between the LBT BW units (subbands) within the carrier.

- Need to specify the behavior upon LBT failure on a subset of the subbands, especially in UL.

- gNB indication to the UE of the LBT BW configuration if the LBT BW unit is configurable rather than fixed.

Similarly, a single LBT BW can span the total aggregated bandwidth of DL CA or UL CA or the total aggregated bandwidth of DL FDM transmissions to possibly multiple UEs. For instance, if a carrier BW of 400 MHz is used, and the transmissions are scheduled over 5 contiguous intra-band carriers (as CCs for a contiguous CA scheme to serve a single UE or as independent carriers serving multiple UEs), the LBT BW could span 2 GHz instead of performing 5 parallel LBT procedures with 400 MHz BW each.

Therefore, in the case of multi-carrier transmission in intra-band CA targeted in the above agreement, we think that performing single LBT over all CCs (Alt CA.2) should be supported. In contrast, Alt CA.5 would significantly increase the complexity/energy consumption, especially if a small fixed LBT BW unit, e.g. 100 MHz, is defined to suit different channel and CC BWs (from 100 to 1600 MHz). In addition, same issues observed above for Alt SC.3 are also applicable to Alt AC.5.

For more dense deployments where channel access probability would be reduced by such a wideband LBT, finer LBT BW granularities than that of Alt CA.2 should also be supported to achieve a better tradeoff between the channel access probability and complexity/energy savings. Hence, performing multiple LBTs, one for each channel bandwidth separately (Alt CA.1) should be also supported to achieve such a tradeoff.

Proposal 9: For a single-carrier transmission in NR-U-60, support performing a single LBT over the channel/BWP bandwidth, i.e. Alt SC.1.

Proposal 10: For a multi-carrier transmission in intra-band CA in NR-U-60, support both performing a single LBT over all CCs, and performing multiple LBTs, one for each channel bandwidth separately, i.e., Alt CA.2 and Alt CA.1, respectively.

Multi-channel Access

Following Rel-16 NR-U, multi-channel access options were also discussed and two alternatives were identified as per the following agreement in RAN1#104-e:

Agreement: Define Type A and Type B multi-channel channel access as: - Type A: Perform independent eCCA for each channel - Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot Down-selection between - Alt1: Support Type A multi-channel channel access only - Alt2: Support both Type A and Type B multi-channel channel access. Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

In our view, Type B multi-channel access procedure is more efficient and simpler to implement than Type A multi-channel access procedure. This is due to the fact that only one eCCA is performed on the ‘primary’ channel using one backoff counter whereas a short one-shot CAT2 LBT is used on all remaining channels as ‘secondary’ channels. However, such simplicity and efficiency come at the risk of losing the channel access opportunity on all the channels on which transmission(s) are scheduled/configured if the eCCA fails on the primary channel. This is not the case with Type A procedures wherein transmissions occur based on the independent eCCA results. Therefore, we support Alt2 in the above agreement, i.e., both Type A and Type B.

It was argued in previous meetings though that a device may randomly choose a carrier with narrower bandwidth as its primary channel and perform CAT2 LBT on the wider bandwidth secondary channels. We note that 802.11 ad/ay already uses a similar mechanism where, in fact, secondary channel BWs are integer multiple of the primary channel BW. In NR-U-60, how to define the BW of the primary channel should be further discussed. In our view, unlike in Rel-16 NR-U, there is no need to define a fixed BW for the primary channel. Neither is there a need for NR-U-60 to follow the 801.11ad/ay channel bonding sets if Type B multi-channel access is supported given the fact that the 2.16 GHz minimum channel BW is not mandated for NR-U-60 and thus the channelization may not be aligned with 801.11ad/ay anyway given the wide range of channel BWs agreed so far, i.e. from 100 MHz to 2000 MHz.

Proposal 11: For multi-channel access in NR-U-60, support both Type A and Type B procedures.

LBT beam

Given that LBT as a spectrum sharing mechanism is meant to assess whether or not the subsequent transmission(s) is allowed, whereas the impact of such transmission(s) varies significantly depending on the associated transmission beam(s), it is intuitive that determining the LBT beam should also be done in relation to these transmission beam(s). Otherwise, the LBT result would be unreliable and different sensing devices would have unpredictable behavior even under the same interference conditions. In our view, when LBT is mandated as the spectrum sharing mechanism in a given region, the LBT procedure has to be unified across networks of the same RAT to ensure fair inter-operator coexistence and thus should be well specified and not left for implementation. Moreover, specifying the LBT procedure would be also important for the purpose of inter-RAT coexistence evaluations. Therefore, as a general approach, we support specifying the spatial relationship between the LBT beam and the transmission beam(s) for operation in the 60GHz band.

In the previous RAN1#105-e, it was discussed how 3GPP specifications would define the relative relationship between all applicable sensing beams and the transmission beam(s) where at least sensing beam “covers” the transmission beam(s). Given the majority support of capturing the definition of such spatial relationship in RAN1 first rather than leaving it completely to RAN4, following FL Proposal 2.9.4-1 [6] was formulated considering two approaches to define “cover”; the first uses the physical properties of the beams involved and the second extends the beam correspondence framework and/or QCL/TCI framework:

Proposal 2.9.4-1 (high priority) from [6]

3GPP specification defines at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives

- Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam

  - FFS: This is handled in RAN1 and/or RAN4

  - FFS: The angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam.

  - FFS: The sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain

  - FFS: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak transmission beam gain. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

  - FFS: Sensing beam has the minimum [3]dB beamwidth which at least contains all beam peak directions of transmission beams

  - Other mechanisms not precluded

- Alt 2. Extending the beam correspondence framework and/or QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s)

  - FFS: Details on how to extend the beam correspondence framework and/or QCL/TCI/ SpatialRelationInfo framework

We agree with FL’s recommendation to consider Proposal 2.9.4-1 in [6] as a starting point for further discussion in this meeting and we propose to agree to the proposal and continue to discuss the details of the respective FFS points.

Proposal 12: For operation in the 60 GHz band, support FL Proposal 2.9.4-1 from RAN1#105-e to specify the spatial relation between the LBT beam and the transmission beam(s).

In fact, we observe that both approaches in Alt 1 and Alt 2 have technical merits, and in the following subsections, we discuss the cases in which Alt 1 and Alt 2 should be supported as well as our preferences in terms of the FFS options.

COT with a single transmission beam

For the case of initiating a COT in which the intended transmission(s) are performed using a single transmission beam, defining the correspondence between the LBT beam and the single corresponding transmission beam could be a straightforward task taking the approach in Alt 2 of FL Proposal 2.9.4-1 in [6]. For UL, the UE can be specified to use a spatial domain sensing filter that is the same as the spatial domain transmit filter used for the subsequent UL transmission(s) during the COT. Note that, a similar issue to above, the correspondence between UE’s spatial domain receive filter and the spatial domain transmission filter is already supported in Rel-15/16. For instance, a DL RS can be configured in spatialRelationInfo in SRS configuration in which case the UE shall transmit the SRS resource with the same spatial domain transmission filter used for the reception of the indicated DL RS.

Similarly, for DL, the gNB can use a spatial domain sensing filter that is the same as the spatial domain transmission filter used for the subsequent transmission(s) during COT, e.g. based on QCL information provided to the intended UE in association with a DL RS, or in association with any another DL-RS QCLed with that DL-RS.

Proposal 13: For a COT with a single transmission beam, the spatial domain sensing filter for the LBT beam at the beginning of the COT can be configured to be the same as the spatial domain filter used for the transmission during the COT.

- FFS: Further extending the framework to indicate a different spatial domain sensing filter to the UE

COT with multiplexed transmission beams

In RAN1#104-e, alternatives were discussed for both cases of spatial domain and time-domain multiplexing of the transmission beams to ensure that the LBT procedure is accounting for the footprint of the subsequent COT as captured in the following agreements:

Agreement: For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both) - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Agreement: Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations - Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of "cover" - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

The one-to-one correspondence between LBT beam and the transmission beam discussed in the previous subsection can be directly extended to support independent per-beam LBT sensing for either a COT with SDMed beams or a COT with TDMed beams.  Nevertheless, performing a single LBT at the beginning of the COT using a wide LBT beam covering all the subsequent SDMed or TDMed transmission beams is a low complexity alternative to performing multiple parallel LBTs. Moreover, it can be considered a fall back alternative when the one-to-one correspondence cannot be established.

Therefore, for a COT with spatial domain multiplexing or time domain multiplexing of different beams, both one wide LBT beam covering all transmission beams (Alt 1 in the agreements above), and multiple independent per-beam LBT beams (Alt 2 in the agreements above) should be supported. Given that the LBT beam(s) at the beginning of the COT already cover all the time multiplexed transmission beams by that device during the COT, there is no need to introduce CAT2 LBT before each beam switching in the middle of the COT as in Alt 3.

Proposal 14: For initiating a COT with SDM or TDM of different beams, support multiple independent per-beam LBTs, i.e. Alt 2.

Proposal 15: When gNB performs multiple independent per-beam LBTs, the spatial domain sensing filter for an LBT beam is the same as the spatial domain filter used for the corresponding transmission beam.

Proposal 16: For initiating a COT with SDM or TDM of different beams, support one LBT beam covering all transmission beams (Alt 1) as a fallback mechanism when the one-to-one correspondence between the LBT beams and transmission beams cannot be established.

In both cases of SDMed and TDMed transmission beams, we observe that specifying the spatial relationship between a wide LBT beam and multiple subsequent transmission beams is feasible, following the approach of Alt 1 in FL Proposal 2.9.4-1 [6], if spatial properties similar to those defined in TS 38.104 for a transmission beam are defined for the ‘wide’ LBT beam. The key spatial properties for a BS transmission beam are defined in TS 38.104 as follows:

  - Beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

    - NOTE: For certain BS antenna array, there may be more than one beam.

  - Beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

  - Beam peak direction: direction where the maximum EIRP is found

  - Beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

  - Beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

Once similar spatial properties are defined for the LBT beam, it could be specified that the gNB selects a spatial sensing filter that minimizes the resulting [3]dB sensing beamwidth which at least contains all beam peak directions of the subsequent DL transmission beams within the COT as captured among the FFS options under Alt 1 of FL Proposal 2.9.4-1 [6] and as shown in Fig. 1.

Fig. 1: Defining spatial properties of a wide sensing beam covering multiple potential transmission beams

Intuitively, minimizing the sensing beamwidth of the selected LBT beam is motivated by increasing the channel access probability and/or spatial reuse opportunities. Compared to the other criteria listed under Alt 1 of FL Proposal 2.9.4-1 [6], specifying that the sensing [3]dB beamwidth at least contains all beam peak directions of the subsequent DL transmission beams within the COT would simplify specifying the necessary RAN4 requirement/test procedure to guarantee that the sensing beam “covers” the transmission beam.  For instance, a margin, θ≥0, for the selected sensing beamwidth angle can be defined as an absolute or a relative sensing beamwidth accuracy requirement. If the largest relative angle between the beam peak directions, i.e., of the potential transmission beams towards UE1 and UE2 in Fig. 1, is denoted as β, then the selected LBT sensing beamwidth is required to be bounded between the values β and β+θ.

The other criteria, in contrast, involve measuring/testing the XdB beamwidth for each of the potential DL transmission beam in addition to the sensing beam, and/or measuring/testing the beamforming gain in these multiple directions.

Observation 2: specifying the spatial relationship between a wide LBT beam and multiple subsequent   transmission beams is feasible if spatial properties similar to those defined in TS 38.104 for a transmission beam are defined for the LBT beam, including beam peak direction, beam center direction and beamwidth.

Proposal 17: For initiating a COT with SDM or TDM of different beams using a single LBT beam that “covers” all the subsequent DL transmission beams, gNB selects a spatial sensing filter that minimizes the resulting [3]dB sensing beamwidth which at least contains all beam peak directions of the subsequent DL transmission beams within the COT.

Alternatives for performing independent per-beam LBTs

Assuming the support for independent per-beam LBTs for initiating either a COT with SDMed transmission beams or a COT with TDMed transmission beams, it was agreed in RAN1#104bis-e to down-select from the following alternatives on how to perform the independent per-beam LBTs in each case:

Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

In our view, for a COT with MU-MIMO (SDM) transmissions, it is intuitive that all per-beam LBTs should be performed simultaneously at the beginning of the COT (Alt B). For instance, using the same spatial transmit filters used for simultaneous transmissions for simultaneous sensing. In RAN1#105-e, following FL Proposal 2.7.1-2 was formulated capturing this position and was supported by almost all companies [6]. We propose to agree to the proposal in this meeting:

Proposal 2.7.1-2 from [6]

For a COT with MU-MIMO (SDM) transmission if Alt 2 is supported (independent per beam LBT), and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported.

Although for a COT with TDMed transmission beams it could be more straightforward to assume that the per-beam LBTs are also multiplexed in time as in Alt A, our preference is still Alt B for the following reasons:

- If the per-beam eCCAs are performed sequentially as in Alt A-1, the first eCCA in the sequence of eCCAs is far off from the beginning of the COT, thus rendering its sensing result irrelevant. Moreover, latency and LBT overhead are maximized compared to performing these eCCAs simultaneously.

- Alt A-2 in fact defeats the purpose of TDM of N transmission beams in one COT as it simply splits one COT with N TDM beams to N single-beam COTs each initiated with its own eCCA while the LBT overhead is the same as that of Alt A-1.

- Alt A-3 does not seem to be compliant with the regulations as for any given CCA engine/backoff counter a sensing slot cannot be skipped or blindly assumed idle based on the sensing result of another CCA engine/backoff counter.

- Based on the input from some companies during the email discussions, we think it should be also clarified that further partition an observation slot of 5us for different beams is impractical from the implementation perspective.

Therefore, we propose to support Alt B for a COT with TDMed transmission beams as well assuming that the node has the capability to simultaneously sense in different beams. In RAN1#105-e, following FL Proposal 2.7.4-2 was formulated capturing this position and was supported by almost all companies [6] as well. We propose to agree to the proposal in this meeting:

Proposal 2.7.1-4 from [6]

Within a COT with TDM of beams with beam switching, if Alt 2 or Alt 3 is supported (independent per beam LBT), and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported.

Otherwise, if the node is incapable of sensing simultaneously in different beams, the fallback alternative Alt-1 should be used, i.e., a single LBT with a wide beam covering the TDMed transmission beams within the COT.

Nevertheless, even if per-beam LBTs start simultaneously, some may end earlier than others. How to coordinate these parallel LBTs to avoid a large discrepancy among the LBT end times can be further investigated.

Proposal 18: For initiating a COT with SDM or TDM of different beams, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, support performing the per-beam LBTs simultaneously in parallel (Agree to FL Proposals 2.7.1-2 and 2.7.1-4 from RAN1#105-e).

- FFS: How to coordinate these parallel LBTs to align the start times of the SDMed transmissions, and how to determine the COT start time in the TDM case.

- If the node is incapable of sensing simultaneously in different beams, a single LBT beam covering the multiplexed transmission beams should be used.

CAT2 LBT and COT sharing aspects

It was discussed in RAN1#104-e whether or not to introduce CAT2 LBT in the 60GHz band and a list of its potential use cases was provides as captured in the following agreements:

Agreement: For Cat 2 LBT, down-select from the following alternatives - Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation - Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation Agreement: If Cat 2 LBT is introduced, the following use cases can be further studied: - Resume transmission after a gap Y: Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y) - COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT - Multi-Beam LBT: Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z) - Rx-Assistance: Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling Other use cases not precluded. FFS if Cat 2 LBT is mandated for each use case or not.

In the previous meeting RAN1#105-e, it was agreed, based on the companies’ support of the three alternatives for COT sharing, to further down-select between only Alt 1 and Alt 3 as per the following agreement:

Agreement: On maximum gap within a COT to allow COT sharing without LBT, down-select or support both of the following two alternatives - Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration - Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

In our view, COT sharing for transmission(s) by a responding device as specified in the HS EN 302 567 does not require additional LBT within the COT. Furthermore, no requirement on a max gap between transmissions within the COT has been stated. We thus do not see the need to restrict the scheduling within the COT by applying restrictions on the gap between transmissions, and to even further increase the LBT overhead and the complexity of the channel access procedures by introducing additional LBT instances within the COT.

Therefore, we propose to support COT sharing Alt 1 in the agreement above, i.e., no maximum gap defined and a later transmission can share the COT without LBT irrespective to the gap duration within the MCOT.  Furthermore, any gap duration should be counted in the COT duration.

Proposal 19: For COT sharing without LBT in NR-U-60, no maximum gap is defined and a later transmission from a responding device can share the COT without LBT irrespective to the gap duration within the MCOT (Alt 1).

- Any gap duration should be counted in the COT duration

Regarding the discussion on introducing CAT2 LBT in NR-U-60, our view is that introducing such short one-shot LBT is beneficial, especially for procedures related to COT initiation. This is due to its simplicity and LBT overhead reduction compared to eCCA. Note that the simplicity and the low overhead of CAT2 LBT compared to CAT4 LBT were also the major reasons for not agreeing in RAN1#105-e to replace the one-shot CAT2 LBT by CAT4 LBT with a deterministic counter as the latter procedure still relies on persistent deferral as long as the channel is sensed busy. As we explained earlier in Section 2.3.2, the benefits of Type B multi-channel access procedures cannot be realized without introducing CAT2 LBT to initiate a CO on a secondary channel. Furthermore, on initiating a CO using Rx-assisted LBT, CAT2 LBT can be used for energy measurement at the receiver and providing the Rx-assistance information from only the devices that pass the LBT as discussed in Section 2.5 in more details.

Proposal 20: Support introducing CAT2 LBT for the 60GHz unlicensed band operation.

Proposal 21: The following use cases of CAT2 LBT related to COT initiation should be prioritized in the discussion due to the low complexity and overhead of CAT2 LBT compared to eCCA:

- Starting transmission on a secondary channel in Type B multi-channel access, if supported

- Energy measurement and reporting of Rx-assistance information by the receiver in Rx-assisted LBT, if supported

Receiver-assisted LBT/Receiver-only LBT

Channel sensing only at the transmitter side is efficient when the assumption is valid that the interference sensed at the transmitter side is equivalent to that sensed at the receiving side. However, in the high frequency, e.g. the 60GHz band, due to the beamforming and large path-loss, the following issues are more pronounced when employing LBT only at the transmitter side: (1) Hidden node issue, due to the transmitter’s inability to detect the interference at the receiver. (2) Exposed node issue, due to the transmitter maybe detecting tolerable/harmless interference to the receiver.

Therefore, a general approach for NR-U transmission to avoid above issues should be supporting receiver-assisted LBT as a configurable handshake mechanism between the gNB and the UE as shown in Fig. 2.

For DL transmission, gNB performs CAT4 LBT to send DL assignments to K UEs out of M intra-cell UEs, e.g., based on the status of their respective buffers. As discussed in Section 2.3.3, this can be done using one sensing beam or multiple beams covering the transmission beams of the respective PDCCHs. Each DL assignment triggers an UL transmission of an idle indication (CTS) signal/channel on indicated resources upon a successful LBT by the intended receiving UE. The CTS could further include receiver-assistance information such as the interference level measured by the UE during LBT. In order to ensure no interference during PDSCH reception from other nodes, the UE could perform a directional LBT for a deterministic duration, i.e. CAT2 LBT in the direction of PDSCH reception as, e.g., indicated by PDCCH. As such, the gNB receives the CTS/receiver-assistance information from N (<=K) UEs to which it proceeds with the transmission of their respective PDSCHs. It should be clarified that, in contrast to the argument raised by some contributions in RAN1#105-e, a COT initiated as such is not limited to the maximum number of PDSCHs that can be scheduled by a single DCI. This is due to the fact that subsequent scheduling PDCCHs and their respective PDSCHs can be further transmitted within the remaining MCOT duration to any of the UEs that reported idle channel and thus the overhead caused by this handshake is insignificant.

Similarly, for UL transmission, gNB performs CAT4 LBT to send UL grants to K UEs. This can be also done using one sensing beam or multiple beams covering the transmission beams of the respective K PDCCHs. However, since gNB is also the intended receiver of the UL transmission, such an LBT could serve the purpose of receiver-side sensing and assessment of the interference level at reception. Note that, this mechanism would be efficient when the scheduling time offset of PUSCH (K2) is small enough such that the interference measurement by the gNB prior to the UL grant transmission is representative of the interference during PUSCH reception. Spatial reciprocity may be assumed in such case between the transmission beam of PDCCH and the reception beam of the associated PUSCH(s).  If the scheduling time offset of PUSCH is large, the interference measurement by the gNB prior to the UL grant transmission may not represent the interference during PUSCH reception. In such a case, as shown at the bottom of Fig. 2 (b), each UL grant can also trigger a DL transmission of an idle indication (CTS) signal on indicated resources, e.g., a number of symbols preceding the allocated PUSCH start, upon a successful LBT by the gNB. The gNB could perform the directional LBT for a deterministic duration prior to the CTS transmission, i.e. CAT2 LBT in the direction of PUSCH reception.

A UE that receives the UL grant in the first case, and receives as well the idle indication/CTS in the second case, proceeds with the transmission of the respective PUSCHs.

Observation 3：A COT initiated using receiver-assisted LBT is not limited to the maximum number of PDSCHs that can be scheduled by a single DCI. Subsequent scheduling PDCCHs and their respective PDSCHs can be further transmitted within the remaining MCOT duration to any of the UEs that reported idle channel.

- DL transmission

- UL transmission

Fig. 2: LBT with receiver-assisted mechanism for NR-U in the 60GHz unlicensed band

As can be observed from the simulation results discussed in Section 4, receiver-assisted LBT provides significant coverage gains relative to the transmitter–side omni-directionl and directional LBT mechanisms, especially at medium to high load in indoor scenarios. This attests to the fact that introducing the receiver-side directional LBT is an efficient solution to combat the interference from hidden nodes that cannot be avoided using the transmitter–side LBT mechanisms.

Receiver-only directional LBT:

- DL transmission

- UL transmission

Fig. 3: Receiver-only LBT mechanism for NR-U in the 60GHz unlicensed band

It can be noticed from the observations on simulation results captured in the TR [7] in SI phase that despite the relative performance gains of the receiver-assisted LBT with respect to the transmitter-side only directional and omni-directional LBT, the three mechanisms often suffered from performance loss compared to the No-LBT mechanism in a low or medium traffic. This is due to the fact that the transmitter-side LBT is common in these three mechanisms which introduces LBT overhead (Fixed CWS of 127 slots was assumed for our results in [7] as compared to the current results assuming 5 slots) and may undermine the potential spatial reuse without tangible gains from interference coordination. How to design a channel access mechanism to attain the benefits of the receiver-side LBT without undermining the spatial reuse gains is thus an important question. We therefore propose the receiver-only directional LBT as a simplified and efficient variant of receiver-assisted LBT mechanism, whereby only the receiver performs the directional LBT as shown in Fig. 3. It is noted that this receiver-side LBT could be also CAT4 LBT to initiate the CO instead of the transmitter and then shares its CO with the transmitter. Intuitively, the receiver-only directional LBT saves the LBT overhead associated with the transmitter-side LBT of the receiver-assisted LBT mechanism and provides an efficient trade-off as it aims at increasing the spatial reuse while mitigating the hidden node issue when present.

Observation 4：Receiver-only directional LBT saves the LBT overhead associated with the transmitter-side LBT of the receiver-assisted LBT mechanism and provides an efficient tradeoff as it aims at increasing the spatial reuse while mitigating the hidden node issue.

In Section 4, we highlight the key observations from our SLS results that are also provided in details in our contribution [8]. The SLS results consider the performance of the channel access mechanisms in a system using wider beam transmissions compared to the baseline scenario, as well as the impact of reducing the CW size for CAT4 LBT to only 5 slots, varying the EDT-Rx used at the receiver side, and the granularity of the receiver assistance information. In summary, compared to No-LBT, substantial coverage gains are achieved using Receiver-assisted LBT/Receiver-only LBT in the indoor scenario, especially at medium and high traffic load. Even higher gains are realized when wider beams are used for directional transmissions. Whereas, receiver-only LBT outperforms all the channel access mechanisms considered.

Receiver-side LBT vs. Interference measurement alternatives

In RAN1#104-e, alternatives were discussed for the receiver to perform the channel sensing and reporting of the Rx-assistance information to the transmitter as captured in the following agreement:

Agreement: For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion - Alt 1. Legacy RSSI measurement and reporting with possible enhancements - Alt 2. AP-CSI report with possible enhancements - Alt 3. LBT at receiver - Alt 3.1 eCCA - Alt 3.2 Cat2 LBT

Our understanding of Alt 1 is that all UEs in the cell, say M UEs, would be semi-statically configured with resources to perform the RSSI measurements periodically. All measurements would be reported periodically to the gNB and the gNB would have to process the measurements for all M UEs.

Whereas, when LBT is performed at the receiver, only K UEs (K≤M) who are dynamically scheduled by the gNB (based on their respective data buffers) would perform the LBT procedure including the actual interference measurement a few symbols before the potential DL transmission(s). Using a low EDT at the receiver, only N UEs (N≤K) whose LBTs are successful are required to feed back their respective Rx-assistance Information/Idle indication, and the gNB needs to process the feedback from only those N UEs. The SLS results in Section 3.2.2 of [8] clearly show that such limited feedback does not considerably change the UPT performance if gNB uses the reported interference levels to prioritize UE scheduling.

As such, the following issues can be observed in Alt 1 in comparison to performing LBT at the receiver:

- Legacy RSSI is periodic measurement and thus not representative of the experienced interference immediately prior to data reception.

- Legacy RSSI requires resources dedicated for measurements and the resources used by each of the M UEs to report the measurements in UL channels. This also incurs complexity at each UE to conduct and report the measurements periodically regardless of the gNB’s intent to schedule it, as well as the complexity at gNB to continuously process these reports.

- Legacy RSSI is less efficient in terms of resource overhead and complexity at both UE and gNB, especially at high load, compared  to only 1 or 2 UEs reporting Rx-assistance info upon passing LBT

- Configuring shorter periodicities for measurements and reporting of legacy RSSI further emphasizes the overhead and complexity savings of Receiver-side LBT.

Although Alt 2 could overcome some of the resource overhead and complexity incurred by Alt 1 since AP CSI-RS would be dynamically triggered for K UEs instead of M UEs, this mechanism would incur more latency and resource overhead and would introduce a more complex handshake for each CO compared to receiver-side LBT. This is due to the following facts

- An AP CSI-RS would have to be triggered first by each scheduling DL assignments for measurement, then followed by some processing delay before reporting CSI on PUCCH resources from all the candidate K UEs.

- During such a processing time the UE needs to perform a more complex measurement procedure, e.g. including FFT due to the sparse nature of the CSI-RS measurement resource in the frequency domain. This is in contrast with the simple energy detection as part of LBT,

- Based on the discussion on required synchronization accuracy in higher numerologies, CSI-RS detection is not immune to such requirement. However, energy detection as in receiver-side LBT does not nearly need such a level of timing accuracy.

- As acknowledged by the proponents of Alt 2, current processing delays for CSI reports in NR are rather long, which means that such a handshake would also result in an increased delay overhead within the CO compared to the receiver-side LBT.

- The latency between CSI-RS reception and CSI-RS report is a UE capability and it may be too long so that the reported CSI is not actually a representative of the experienced interference during the data reception.

- Finally, in terms of specification effort, there is no advantage of AP CSI reporting on PUCCH as compared to LBT at the receiver since AP CSI reporting on PUCCH is not a legacy mechanism supported in Rel15/16.

Nevertheless, it should be noted that both Alt 1 and Alt 2 are discussed for DL only. However, Alt 3 is also applicable to UL, e.g., when the scheduling time offset of PUSCH is large such that the interference measurement by the gNB prior to the UL grant may not represent the interference during PUSCH reception.

It was proposed in RAN1#105-e [6] for the receiver assistance to introduce L1-RSSI measurement to be sent as part of an enhanced AP-CSI report. However, it should be noted that L1-RSSI is not an enhancement of the ‘Legacy RSSI’ measurement and reporting in Alt 1which is a L3 measurement. Introducing L1-RSSI would require defining a new measurement quantity in L1 along with designing and specifying its measurement configuration, resources, trigger and associated timelines. Besides, the current AP-CSI reporting mechanism by itself needs several enhancements to resolve the issues listed above.

Observation 5：Enhancements and specification effort required to introduce L1-RSSI on AP-CSI feedback are substantial compared to those required to introduce receiver assistance based on receiver-side LBT.

A good starting point to continue the discussion in this meeting is following FL Proposal 2.6.3-1 in [6] which was formulated in the third round of the email discussion in RAN1#105-e:

Discussion 2.6.3-1 from [6]

For receiver to provide assistance Alt 3.1 (eCCA based), when gNB is the initiating device (UE is providing assistance), what is your view on this scheme

- Alt 3.1A: gNB schedules or triggers UL transmission (PUCCH, PUSCH, SRS etc) with DCI and indicating Cat 4 LBT in the DCI. UE performs Cat 4 LBT for the scheduled UL transmission. gNB detects the scheduled UL transmission to tell if UE passes the Cat 4 LBT

- Alt 3.1B: New RTS/CTS-like signaling introduced. gNB sends RTS-like signaling to UE. UE performs Cat 4 LBT and if LBT passes, transmits CTS-like signaling to explicitly indicate the LBT outcome. gNB detects the CTS-like signaling to identify if the UE passed Cat 4 LBT. After detecting the CTS-like signal, the data transmission happens

- Anything else?

Although the proposal intended to clarify what Alt 3.1 in the above agreement means exactly for receiver-assisted LBT, discussing the differences between the descriptions in Alt 3.1A and Alt 3.1B has been quite helpful to resolve the ambiguities about Alt 3 in general. While the majority views acknowledged that Alt 3.1B is a closer mechanism to Receiver-assisted LBT than Alt 3.1A, we have also clarified along with other companies that the CTS/RTS need not be new signals/channels and could be similar to those described in Alt 3.1A.

For example, when the target transmission is DL and UE is the receiver, the scheduling DL assignment itself resembles an RTS and triggers the LBT at the UE and the UL transmission of PUCCH (as CTS/receiver-assistance information) or SRS (as idle indication CTS only). The UE transmits the triggered UL only after the LBT has passed. This does not require introducing a new DCI format unless there is a consensus to support triggering the LBT at the UE, e.g., using a group common DCI, separately from the DL scheduling. Such solution may be discussed later on.

Therefore, we propose a more accurate description of Alt 3.1 as following Alt 3.1C for further discussion in this meeting. We also note that setting the context of the discussion point based on the target transmission direction provides more clarity than “gNB/UE is initiating device”.  This is due to the fact that the DL assignment DCI could be transmitted, for instance, in a previous gNB/UE COT.

Proposal 22：For UE to provide receiver-assistance when gNB intends to transmit in the DL, consider the following Alt 3.1C in FL Proposal 2.6.3-1 from RAN1#105-e to clarify Alt 3.1 based on eCCA at the receiver:

- Alt 3.1C: gNB schedules or triggers UL transmission (PUCCH, PUSCH, SRS etc) as CTS/Receiver-assistance information with the DL assignment DCI and indicating Cat 4 LBT in the DCI. UE performs Cat 4 LBT and if LBT passes, transmits the CTS/Receiver-assistance information to explicitly indicate the LBT outcome. gNB detects the CTS/Receiver-assistance information to identify if the UE passed Cat 4 LBT. After detecting the CTS/Receiver-assistance information, the data transmission happens

In our view, both Alt 3.1 and Alt 3.2 are beneficial in different cases and should be supported. For instance, considering DL, CAT2 LBT as in Alt 3.2 could be used at the UE if the gNB has performed CAT4 LBT (eCCA) before transmitting the scheduling DL assignment. Otherwise, CAT4 LBT as in Alt 3.1 could be used at the UE to initiate the CO and sharing it to the gNB.

Proposal 23：For operation in the 60 GHz band, receiver-side LBT should be supported (Alt 3 in the agreement made in the RAN1#104-e).

Channel access mechanisms without LBT

According to the following agreement captured in the TR [7], for operation where LBT is not required, it can be further discussed when specifications are developed whether to introduce additional restrictions, such as DFS, ATPC, long term sensing, certain duty cycle limitation, certain transmit power limitation, MCOT limits, etc., or leave the restriction for gNB implementation.

Agreement: Capture the following in the TR: For operation where LBT is not required, it can be further discussed when specifications are developed - If RAN1 should introduce additional conditions/mechanisms for no-LBT to be used, or leave it for gNB implementation - When no-LBT mode is used, if RAN1 should introduce additional restrictions, such as DFS needs to be applied, ATPC needs to be applied, long term sensing needs to be applied, certain duty cycle limitation, certain transmit power limitation, MCOT limits, etc, or leave the restriction for gNB implementation - When no-LBT mode is used, if RAN1 should introduce mechanism for the system to fallback to LBT mode, or leave it for gNB implementation

In our view, for regions wherein LBT is not mandated by regulations, No-LBT channel access can be performed to initiate a channel occupancy by a transmitter in conjunction with some means of interference mitigation scheme. The interference mitigation schemes such as ATPC or DFS would be implemented as specified by the region-specific regulations and do not need to be specified by 3GPP. This was in fact the approach taken by 3GPP when developing the specifications of Rel-16 NR-U: Despite that DFS requirements and procedures are specified in the ETSI BRAN HS for 5GHz (EN 301 893), NR-U specifications did not capture such requirements and procedures.

Also, according to the WID objectives in [1], channel access mechanisms assuming beam based operation are considered in order to comply with the applicable regulatory requirements. Among these objectives is to specify both LBT and No-LBT related procedures, whereas “for No-LBT case no additional sensing mechanism is specified”. As such, a sensing mechanism that has not been specified for channel access with LBT shall not be specified for channel access with No-LBT.

Proposal 24：For operation in the 60 GHz band, in regions where LBT is not mandated, a gNB/UE can initiate a channel occupancy access using a channel access mechanism without LBT if it is used in conjunction with an interference mitigation scheme.

- Interference mitigation schemes such as ATPC or DFS would be implemented as specified by the region-specific regulations and do not need to be specified by 3GPP.

- Additional sensing mechanisms such as long-term sensing shall not be specified for No-LBT in accordance with the WID.

For regions where LBT is not mandated, regarding how to indicate whether gNB-UE connection is operating in LBT mode or no-LBT mode, following agreement in RAN1 #104bis-e was reached [4]:

Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode. Down-select between - Alt 1. Support cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) gNB indication - Alt 2. Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication - FFS: Whether the indication of the decision on applying LBT mode or no-LBT mode is per beam (can be different for different UEs in different beams or can be different for different beam pairs between gNB and the UE) or per cell (can be different for different cells for a UE in carrier aggregation) - FFS: Whether a gNB and its UE(s) can have different mode - FFS: Whether L1 signalling can be used for both Alt 1 and Alt 2 for gNB indication

In RAN1#105-e [5], it was agreed to support both Alt 1 and Alt 2 as follows. In addition, each of the FFS points in the above agreement was discussed separately.

Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode - Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

Even though the group has agreed to support the flexibility of UE-specific indication, in our view, if two UEs in the same cell operate with two different channel access modes, the UE operating with LBT is consistently at a disadvantage compared to the UE operating without LBT. We thus think that further indicating the LBT/No-LBT mode in per-beam granularity would overcomplicate the signaling without a clear benefit to the system performance. Moreover, for a COT with multiplexed beams, a transmission on beam indicated with No-LBT would have to be deferred to allow for sensing by the same device before transmitting on another beam indicated with LBT mode; hindering as such the benefit of No-LBT. Therefore, we believe that indicating the LBT/No-LBT mode per beam should not be supported.

Proposal 25: For operation in the 60 GHz band, in regions where LBT is not mandated, indication of the decision on applying LBT mode or no-LBT mode per beam is not supported.

On the open FFS issue of whether L1 signaling can be used, we do not see the need for dynamic switching between LBT and No-LBT modes.  Also, during initial access the indication can be implicitly or explicitly derived from MIB/SIB1. Some proponents of introducing the L1 signaling explained though that the intention is to provide the indication to the UE before it reads SIB1. However, we do not see the need for such an optimization given that any such indication field would have to be always present in the fallback DCI, and the fact that even in regions where LBT is mandated, the transmission of Msg1/MsgA can be exempted from LBT.

Proposal 26: For operation in the 60 GHz band, in regions where LBT is not mandated, indication of the decision on applying LBT mode or no-LBT mode using L1 signaling is not supported.

In regions where LBT is not required, a serving cell can be configured to enable the LBT mode based on some performance criteria such as the level of experienced interference. For instance, serving cell can be configured for Rx-side LBT to mitigate the interference from hidden nodes and improve performance. We note that mitigating/avoiding the potential interference at the receiver caused by hidden nodes should be targeted to overcome this drawback of the No-LBT mechanism. This is corroborated by the significant throughput and coverage gains of receiver-only directional LBT over No-LBT observed in Section 4.

Proposal 27: For operation in the 60 GHz band, in regions where LBT is not mandated, the serving cell may enable Rx-side LBT using a higher layer configuration to mitigate high levels of interference experienced from hidden nodes.

It should be noted however that when the network allows enabling/disabling the LBT mode through cell-specific/UE-specific gNB configuration, coexistence issues would arise as the performance of the nodes operating with LBT mode would be adversely impacted by the nodes operating with No-LBT mode and occupying the channel resources for possibly unlimited time. Therefore, from the potential additional restrictions captured in the TR, COT durations should be limited for a channel occupancy initiated without LBT.

Observation 6: When network allows enabling/disabling the LBT mode, coexistence issues would arise as the performance of the nodes operating with LBT mode would be adversely impacted by the nodes operating with No-LBT on the channel without a time limit .

Proposal 28: For operation in the 60 GHz band, in regions where LBT is not mandated, COT should be limited when No–LBT is used.

Transmission of short control signalling without LBT: In RAN1#104bis-e [4], it was agreed that contention-exempt short control signalling (CESCS) rules can be applicable to the transmission of SS/PBCH while further studies are needed as to whether this applies to all SCSs and which DL channels/signals can be multiplexed with CESCS-based SSB transmission.:

Agreement: - Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH. - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule - FFS: Whether this can be applied to all supported SCS or specific SCS. - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval) - FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc Agreement: For contention exemption short control signalling based DL transmission of SS/PBCH, further consider if the following signals/channels can be multiplexed with SS/PBCH block transmission. - RMSI PDCCH and RMSI PDSCH - Other broadcast PDSCH - PDSCH without user-plane data - PDCCH - CSI-RS - PRS - Other signals/channels contained in Discovery Burst (i.e., exemption applies to Discovery Burst) Note: Total exempted signals/channels should meet the restriction of 10% over any 100ms interval. FFS: If contention exemption short control signalling based DL transmission is allowed when not multiplexed with SS/PBCH block transmission.

According to section 5.3.8.2 in EN 302 567, examples of short control signalling include ACK/NACK signals, beacon frames, other time synchronization frames and frames for beamforming. The same document sets a maximum duty cycle limit for the time synchronization and beamforming frames as 10 % within an observation period of 100 ms.

However, given the following agreement made in RAN1#104bis-e as well on supporting the discovery burst (DB) as defined for Rel-16 NR-U, we think that, in downlink, only channels/signals that can be multiplexed within the DB as defined for Rel-16 NR-U should be eligible for CESCS. That is, CORESET for PDCCH scheduling PDSCH with SIB1, PDSCH carrying SIB1 and non-zero power CSI-RS.

Agreement: - For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0 - FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements - PBCH payload size is no greater than that for FR2 - Duration of DBTW is no greater than 5 ms - Number of PBCH DMRS sequences is the same as for FR2 - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs - FFS: how to support UEs performing initial access that do not have any prior information on DBTW. - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands - FFS: details of how to inform UEs of the configuration of DBTW

Proposal 29: In regions where LBT is mandated, only channels/signals that can be multiplexed within the DB as defined for Rel-16 NR-U should be supported for contention exemption short control signalling based DL transmission.

For UL, CESCS eligibility was further discussed in RAN1 105-e  and following agreement was made [5] for transmission of msg1 for the 4 step RACH, and MsgA for the 2-step RACH:

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

In Alt 2 in the above agreement, if it is left to each individual UE to use CESCS for msg1 for the 4 step RACH and MsgA for the 2-step RACH, then the total time resources at which at least one UE within the cell transmits msg1/ MsgA can easily far exceed the 10% occupancy time for short control signaling exemption. In our view, this is a misuse of the exemption that is introduced in regulations for “short control signaling”. Therefore, our preference is to support Alt 1.

Proposal 30: In regions where LBT is mandated, contention-exempt short control signaling rules apply to the transmission of msg1 for 4 step RACH and msgA for 2-step RACH such that the 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured in a cell (Alt 1).

It follows also that, with the above agreement on supporting CESCS for masg1/msgA, it would be challenging for the network, if not infeasible, to ensure that the restrictions are maintained if other UL signals/channels are also allowed to be transmitted with the CESCS rule. We therefore propose that the CESCS based transmission is not supported for UL signals/channels other than msg1/msgA.

Proposal 31: In regions where LBT is mandated, contention-exempt short control signaling based transmission is not supported for UL signals/channels other than msg1/msgA.

Observations from system level simulation results

The simulation results in Fig. 4 show the mean value and the 5th percentile user perceived throughput (UPT) for the channel access mechanisms discussed earlier with both DL and UL FTP3 traffic loads with the file size of 27 Mbytes for the Indoor scenario A. A bandwidth of 2 GHz and a SCS of 960 kHz are assumed. It is assumed that the CW is set to a fixed value of 5 observation slots and MCOT equals 5ms. It is also assumed that the EDT used by the receiver for the LBT before sending CTS with interference level feedback in receiver-assisted/receiver-only LBT is the same as the baseline EDT (-47dBm). According to the baseline simulation scenario, 64 antenna elements for the gNB and 8 antenna elements for the UE were assumed.

The simulation results in Fig. 5 show the mean value and the 5th percentile UPT for the same channel access mechanisms in Indoor scenario A under the same set of assumptions used for the results in Fig 4 yet with 32 antenna elements assumed for the gNB and 4 antenna elements assumed for the UE to allow for studying the performance of these channel access mechanisms in a system where wider transmission beams are used, e.g., due to limited number of antennas on low-cost devices.

Details of the system level simulation scenarios for all indoor and outdoor deployments, respective parameters and simulation results are provided in our contribution [8].

Fig. 4. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-A with InH open office (64 antenna elements for gNB, 8 antenna elements for UE).

As can be observed from Fig. 4, receiver-assisted LBT provides coverage gains relative to the transmitter–side omnidirectional and directional LBT mechanisms, as well as No-LBT, in the indoor scenario A. Intuitively, such benefits are more significant under medium to high traffic load conditions. Since the UPT of the cell-edge users is more sensitive to interference, the coverage gains are even more significant when the probability of hidden nodes is increased due to wider beam transmissions as observed from Fig. 5. This attests to the fact that the receiver-assisted LBT is an efficient solution to combat the interference from hidden nodes that cannot be avoided using the transmitter–side LBT mechanisms and the No-LBT mechanism.

Fig. 5. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-A with InH open office (32 antenna elements for gNB, 4 antenna elements for UE).

According to the above evaluation results, receiver-only LBT has the best performance compared to other channel access types in fifth percentile DL and UL user throughput. The relative gains of receiver-only LBT to No-LBT shown in Table 1 are 177% in the DL and 75% in the UL for cell edge at high traffic load under the baseline assumption. Whereas, the gain of receiver-only LBT compared to No-LBT could be 291% in the DL and 103% in the UL for cell edge at high traffic load when half-reduced antenna elements are configured as shown in Table 2.

Table 1. Performance of various channel access type at high traffic load (64 antenna elements for gNB, 8 antenna elements for UE)

| Channel access type/User throughput(Mbps) | DL mean UPT | UL mean UPT | DL 5% UPT | UL 5% UPT | Gain of DL mean UPT w.r.t No-LBT | Gain of UL mean UPT w.r.t No-LBT | Gain of 5% DL UPT w.r.t No-LBT | Gain of 5% UL UPT w.r.t No-LBT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No-LBT | 2388.7 | 1521.8 | 85.7 | 86.6 |  |  |  |  |
| Directional LBT | 2446.7 | 1521.1 | 101.3 | 82.3 | 2% | 0% | 18% | -5% |
| Omni-dir LBT | 2452.9 | 1538 | 95.9 | 89.8 | 3% | 1% | 12% | 4% |
| Receiver-assisted LBT | 2494.2 | 1607.9 | 191.7 | 145.6 | 4% | 6% | 123% | 68% |
| Receiver-only LBT | 2715.7 | 1637.6 | 237.6 | 151.5 | 14% | 8% | 177% | 75% |

Table 2. Performance of various channel access type at high traffic load (32 antenna elements for gNB, 4 antenna elements for UE)

| Channel access type/User throughput(Mbps) | DL mean UPT | UL mean UPT | DL 5% UPT | UL 5% UPT | Gain of DL mean UPT | Gain of UL mean UPT | Gain of 5% DL UPT | Gain of 5% UL UPT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No-LBT | 1842.4 | 1208.4 | 34.7 | 44.5 |  |  |  |  |
| Directional LBT | 1852.7 | 1193.5 | 39.3 | 16.6 | 1% | -1% | 13% | -37% |
| Omni-dir LBT | 1964.1 | 1218.8 | 52.4 | 40.9 | 7% | 1% | 51% | -8% |
| Receiver-assisted LBT | 2033.6 | 1299.1 | 88.9 | 76.4 | 10% | 8% | 156% | 72% |
| Receiver-only LBT | 2215.7 | 1340 | 135.6 | 90.4 | 20% | 11% | 291% | 103% |

It can be observed from the above figures and tables that transmit-side directional and omni-directional LBT can provide gains relative No-LBT in the DL, and especially in terms of coverage, when the LBT overhead by the gNB is reduced via limiting the CW size.

Furthermore, the gains from receiver-assisted LBT/Receiver-only LBT over the transmitter-side LBT mechanisms, and also over No-LBT, especially in terms of coverage, suggest that when No-LBT is used in regions where LBT is not mandated by regulations, the hidden node issue would still persist.

Observation 7: When No-LBT is used in regions where LBT is not mandated by regulations, the hidden node issue would still persist.

Observation 8: Compared to No-LBT, substantial coverage gains are achieved using Receiver-assisted LBT/Receiver-only LBT in the indoor scenario, especially at medium and high traffic load.

- Even higher gains are realized when wider beams are used for directional transmissions

Impact of interference level feedback on DL Receiver-assisted/receiver-only LBT:

In the following set of results, for receiver-assisted/receiver-only LBT, receiver-side LBT using an EDT threshold (EDT_Rx) is performed at the UE side before sending CTS/assistance information reporting in the DL or at gNB side for directional sensing in direction of potential PUSCH reception from the corresponding UE. EDT_Rx = -71dBm + 10log10 (BW/2GHz) + offset_dB was assumed.

Table 3 shows the performance of the average and fifth percentile DL UPT if only CTS/idle indication is fed back when the interference level sensed by UE in certain directional beam is lower than the EDT_Rx using different EDT_Rx threshold values. It is noted that CTS/idle indication including the actual interference level would benefit the gNB’s scheduling of the best UEs among those who have experienced interference levels below the EDT_Rx. For low EDT_Rx threshold, the results show comparable performance in both mean and fifth percentile user throughput. For higher EDT_Rx threshold, i.e. as high as the baseline transmit –side EDT, due to only one bit information being reported to the gNB, it is observed that the performance of the fifth percentile degraded about 33%.

Considering a scenario wherein interference measurements are provided by all the candidate/triggered UEs to the gNB without the UEs passing LBT first, the results for the scheme ‘CTS/idle indication with actual interference level reporting’ at Rx_EDT ≥-47dBm (e.g 0dBm) provide an upper bound to the UPT performance in such a scenario. This is due to the fact that all UEs always pass the LBT at such high Rx_EDT values. However, the results of the same scheme at lower Rx_EDT values show insignificant performance losses while significant savings in resource overhead are expected when only the UEs that pass the LBT would transmit the CTS plus the interference level information.

Table 3. Simulation results with different EDT_Rx and feedback information in DL receiver assisted/receiver-only LBT in indoor scenario-A

| EDT_Rx(dBm) | 0 |  | -47 |  | -51 |  | -61 |  | -67 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| User perceived throughput(Mbps) | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT |
| CTS/idle indication with actual interference level reporting | 3873.5 | 559.5 | 3873.5 | 559.5 | 3875.8 | 566.6 | 3832.6 | 563.7 | 3857.0 | 555.1 |
| Only CTS/idle indication reporting |  |  | 3528.2 | 377.4 | 3551.1 | 419.5 | 3771.9 | 550.1 | 3781.0 | 549.7 |
| Relative Loss |  |  | -9% | -33% | -8% | -26% | -2% | -2% | -1.9% | -0.9% |

Observation 9: For Receiver-assisted LBT/Receiver-only LBT, if a high EDT_Rx threshold is used, the DL cell-edge performance degrades if only CTS/idle indication is fed back when interference level is lower than the EDT_Rx threshold.

Conclusions

Based on the discussions, the following proposals and observations were made:

Observation 1: Adopting the “maximum of mean EIRP of each transmission burst” is not a practical solution as it requires the gNB to know all scheduling decisions for up to 320 slots at 960 kHz before acquiring the COT.

Proposal 1: For operation in NR-U-60, confirm the working assumptions on the definition of Pout in the previously agreed baseline EDT formula.

Proposal 2: For operation in NR-U-60, the term ‘Operating Channel Bandwidth’ in the agreed baseline EDT formula is defined as the ‘LBT Bandwidth’ or the ‘bandwidth on which a channel access procedure is performed in shared spectrum’.

Proposal 3: For operation in NR-U-60, the agreed baseline EDT formula should be adjusted such that, for a given RF output power (EIRP), the EDT proportionally increases with the effective beamforming gain of the potential following transmission(s) by the device.

Proposal 4: For operation in NR-U-60, when LBT is used, adopt the following formula to capture the potential adjustment to the baseline EDT formula based on the transmit beamforming gain:

EDT=-80 dBm+10*log10PmaxPout+10*log10BW [MHz]+(1-a)(GTX -GTX,max)

  - GTX is the effective transmit antenna gain at the potential transmitter [dBi]

  - GTX,max is the maximum effective transmit antenna gain considered for the deployment [dBi]

  - a is a scaling factor such that  0≤ a≤ 1

Proposal 5: For operation in NR-U-60, when LBT is used, the sensing beamforming gain of the LBT beam is deducted from the detected energy level before comparing it to the EDT.

Proposal 6: Confirm the following WA reached in RAN1 #104bis-e:

“For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.”

Proposal 7: For operation in NR-U-60, when LBT is used, the measurement duration X us within the 5us observation is implementation specific.

Proposal 8: For operation in NR-U-60, when LBT is used, support one energy measurement in the 8us deferral period.

- Td consists of a Tf duration of 3us immediately followed by a 5us observation slot duration

- Tf may include an additional measurement duration by implementation.

Proposal 9: For a single-carrier transmission in NR-U-60, support performing a single LBT over the channel/BWP bandwidth, i.e. Alt SC.1.

Proposal 10: For a multi-carrier transmission in intra-band CA in NR-U-60, support both performing a single LBT over all CCs, and performing multiple LBTs, one for each channel bandwidth separately, i.e., Alt CA.2 and Alt CA.1, respectively.

Proposal 11: For multi-channel access in NR-U-60, support both Type A and Type B procedures.

Proposal 12: For operation in the 60 GHz band, support FL Proposal 2.9.4-1 from RAN1#105-e to specify the spatial relation between the LBT beam and the transmission beam(s).

Proposal 13: For a COT with a single transmission beam, the spatial domain sensing filter for the LBT beam at the beginning of the COT can be configured to be the same as the spatial domain filter used for the transmission during the COT.

- FFS: Further extending the framework to indicate a different spatial domain sensing filter to the UE

Proposal 14: For initiating a COT with SDM or TDM of different beams, support multiple independent per-beam LBTs, i.e. Alt 2.

Proposal 15: When gNB performs multiple independent per-beam LBTs, the spatial domain sensing filter for an LBT beam is the same as the spatial domain filter used for the corresponding transmission beam.

Proposal 16: For initiating a COT with SDM or TDM of different beams, support one LBT beam covering all transmission beams (Alt 1) as a fallback mechanism when the one-to-one correspondence between the LBT beams and transmission beams cannot be established.

Observation 2: specifying the spatial relationship between a wide LBT beam and multiple subsequent   transmission beams is feasible if spatial properties similar to those defined in TS 38.104 for a transmission beam are defined for the LBT beam, including beam peak direction, beam center direction and beamwidth.

Proposal 17: For initiating a COT with SDM or TDM of different beams using a single LBT beam that “covers” all the subsequent DL transmission beams, gNB selects a spatial sensing filter that minimizes the resulting [3]dB sensing beamwidth which at least contains all beam peak directions of the subsequent DL transmission beams within the COT.

Proposal 18: For initiating a COT with SDM or TDM of different beams, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, support performing the per-beam LBTs simultaneously in parallel (Agree to FL Proposals 2.7.1-2 and 2.7.1-4 from RAN1#105-e).

- FFS: How to coordinate these parallel LBTs to align the start times of the SDMed transmissions, and how to determine the COT start time in the TDM case.

- If the node is incapable of sensing simultaneously in different beams, a single LBT beam covering the multiplexed transmission beams should be used.

Proposal 19: For COT sharing without LBT in NR-U-60, no maximum gap is defined and a later transmission from a responding device can share the COT without LBT irrespective to the gap duration within the MCOT (Alt 1).

- Any gap duration should be counted in the COT duration

Proposal 20: Support introducing CAT2 LBT for the 60GHz unlicensed band operation.

Proposal 21: The following use cases of CAT2 LBT related to COT initiation should be prioritized in the discussion due to the low complexity and overhead of CAT2 LBT compared to eCCA:

- Starting transmission on a secondary channel in Type B multi-channel access, if supported

- Energy measurement and reporting of Rx-assistance information by the receiver in Rx-assisted LBT, if supported

Observation 3：A COT initiated using receiver-assisted LBT is not limited to the maximum number of PDSCHs that can be scheduled by a single DCI. Subsequent scheduling PDCCHs and their respective PDSCHs can be further transmitted within the remaining MCOT duration to any of the UEs that reported idle channel.

Observation 4：Receiver-only directional LBT saves the LBT overhead associated with the transmitter-side LBT of the receiver-assisted LBT mechanism and provides an efficient tradeoff as it aims at increasing the spatial reuse while mitigating the hidden node issue.

Observation 5：Enhancements and specification effort required to introduce L1-RSSI on AP-CSI feedback are substantial compared to those required to introduce receiver assistance based on receiver-side LBT.

Proposal 22：For UE to provide receiver-assistance when gNB intends to transmit in the DL, consider the following Alt 3.1C in FL Proposal 2.6.3-1 from RAN1#105-e to clarify Alt 3.1 based on eCCA at the receiver:

- Alt 3.1C: gNB schedules or triggers UL transmission (PUCCH, PUSCH, SRS etc) as CTS/Receiver-assistance information with the DL assignment DCI and indicating Cat 4 LBT in the DCI. UE performs Cat 4 LBT and if LBT passes, transmits the CTS/Receiver-assistance information to explicitly indicate the LBT outcome. gNB detects the CTS/Receiver-assistance information to identify if the UE passed Cat 4 LBT. After detecting the CTS/Receiver-assistance information, the data transmission happens

Proposal 23：For operation in the 60 GHz band, receiver-side LBT should be supported (Alt 3 in the agreement made in the RAN1#104-e).

Proposal 24：For operation in the 60 GHz band, in regions where LBT is not mandated, a gNB/UE can initiate a channel occupancy access using a channel access mechanism without LBT if it is used in conjunction with an interference mitigation scheme.

- Interference mitigation schemes such as ATPC or DFS would be implemented as specified by the region-specific regulations and do not need to be specified by 3GPP.

- Additional sensing mechanisms such as long-term sensing shall not be specified for No-LBT in accordance with the WID.

Proposal 25: For operation in the 60 GHz band, in regions where LBT is not mandated, indication of the decision on applying LBT mode or no-LBT mode per beam is not supported.

Proposal 26: For operation in the 60 GHz band, in regions where LBT is not mandated, indication of the decision on applying LBT mode or no-LBT mode using L1 signaling is not supported.

Proposal 27: For operation in the 60 GHz band, in regions where LBT is not mandated, the serving cell may enable Rx-side LBT using a higher layer configuration to mitigate high levels of interference experienced from hidden nodes.

Observation 6: When network allows enabling/disabling the LBT mode, coexistence issues would arise as the performance of the nodes operating with LBT mode would be adversely impacted by the nodes operating with No-LBT on the channel without a time limit .

Proposal 28: For operation in the 60 GHz band, in regions where LBT is not mandated, COT should be limited when No–LBT is used.

Proposal 29: In regions where LBT is mandated, only channels/signals that can be multiplexed within the DB as defined for Rel-16 NR-U should be supported for contention exemption short control signalling based DL transmission.

Proposal 30: In regions where LBT is mandated, contention-exempt short control signaling rules apply to the transmission of msg1 for 4 step RACH and msgA for 2-step RACH such that the 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured in a cell (Alt 1).

Proposal 31: In regions where LBT is mandated, contention-exempt short control signaling based transmission is not supported for UL signals/channels other than msg1/msgA.

Observation 7: When No-LBT is used in regions where LBT is not mandated by regulations, the hidden node issue would still persist.

Observation 8: Compared to No-LBT, substantial coverage gains are achieved using Receiver-assisted LBT/Receiver-only LBT in the indoor scenario, especially at medium and high traffic load.

- Even higher gains are realized when wider beams are used for directional transmissions

Observation 9: For Receiver-assisted LBT/Receiver-only LBT, if a high EDT_Rx threshold is used, the DL cell-edge performance degrades if only CTS/idle indication is fed back when interference level is lower than the EDT_Rx threshold.

- References

- RP-211584, Revised WID: Extending current NR operation to 71 GH, RAN#92-e

- Chairman notes, RAN1#104-e

- Draft EN 302 567 v2.2.0, "Multiple-Gigabit/s radio equipment operating in the 60 GHz band", ETSI

- Chairman notes, RAN1#104bis-e

- Chairman notes, RAN1#105-e

- R1-2106193, Contribution summary of channel access mechanism for 52.6GHz-71GHz band, ver03, Moderator (Qualcomm Incorporated)

- TR 38.808 v0.1.0, Study on supporting NR from 52.6 GHz to 71 GHz

- R1- 2103393, “Link level and System level evaluation for NR system operating in 52.6GHz to 71GHz,” Huawei, HiSilicon