3GPP TSG RAN WG1 Meeting #106-e	R1-2106461

E-meeting, August 16 – 27, 2021

Agenda Item:	8.6.1.3

Source:	Huawei, HiSilicon

Title:	Duplex operation for RedCap

Document for:	Discussion and Decision

Introduction

From RAN1#104-e to RAN1#105-e meeting, the duplex operation for RedCap UEs were discussed. The following agreements were made [1]:

Agreement: - For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), a HD-FDD RedCap UE is not required to monitor ULCI - No special handling on the priority rule for PDCCH carrying ULCI Agreement: - For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, down-select from the following options - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured PDCCH - Option 2: Leave to UE implementation whether to receive the configured PDCCH or transmit the PRACH on the valid RO - Option 3: If configured PDCCH is in a Type-2 CSS set, then PDCCH is prioritized; otherwise the valid RO is prioritized - Option 4: Configured PDCCH is prioritized over valid RO - Option 5: Configured by network, e.g. via a priority indicator - FFS: whether or not the set of symbols overlapping with PDCCH in CSS set includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD - FFS whether a valid RO follows TDD’s or FDD’s definition, and if so, the corresponding impact - FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported Agreement: - For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), down-select from the following options - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured DL - Option 2: Leave to UE implementation whether to receive the configured DL or transmit the PRACH on the valid RO - Option 5: Configured by network, e.g. via a priority indicator - Other options are not precluded. - FFS: whether or not the set of symbols overlapping with configured DL includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD - FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported Agreement: - For Case 8 of valid RO overlapping with dynamically scheduled DL reception, down-select from the following options - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD for operation on a single carrier /single cell in unpaired spectrum - Option 2: Leave to UE implementation whether to receive the DL or transmit the PRACH on a valid RO - Option 3: Follow the handling of Case 1 that when the cancellation timeline is satisfied, the UE cancels the PRACH transmission and receives the DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 2 in R1-2103809) - Option 4: Valid RO is prioritized over dynamic DL that UE performs PRACH transmission and does not perform the DL receptions (Interpretation 3 in R1-2103809) - Option 5: When the cancellation timeline is satisfied, the UE neither performs transmission nor receives any DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 1 in R1-2103809) - FFS: whether or not the set of symbols overlapping with dynamic DL reception includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD - FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

In this contribution, we focus on the remaining issues of collision handling of Case 8, i.e. the valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, with UE-dedicated configured DL reception, and with dynamically scheduled DL reception. The validation and mapping rules of ROs and corresponding PUSCH occasions for MsgA are also discussed.

RO/PRU validation for HD-FDD UEs

Before the discussion of Case 8: valid RO overlapping with DL resources, the RO validation rule for FDD bands (paired spectrum) for the RedCap UEs with half-duplex capability should be defined. Besides, in 2-step RACH, PUSCH occasions (PO) of MsgA is coupled with RO, so that the validation rule of PUSCH occasions and the mapping rule between PO and should be considered together with RO validation. The specified RO/PO validation rules in both TDD and FDD bands are referred as following [2]. In a word, in FDD band, all the ROs are valid, and all the POs that not overlapped in time with ROs are valid. In TDD band, the RO/PO validation should also take the SSB symbols and tdd-UL-DL-ConfigurationCommon configurations into consideration, which is stricter than that in FDD band.

TS 38.213

…

For paired spectrum or supplementary uplink band all PRACH occasions are valid.

For unpaired spectrum,

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon , as described in Clause 4.1

-	If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213]

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in Clause 4.1.

…

A PUSCH occasion is valid if it does not overlap in time and frequency with any valid PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2

-	if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	is within UL symbols, or

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

In a FDD cell, both the RO and PO resources are cell-level configured, which means the RO/PO validation rules, the RO/preamble-to-SSB mapping rules, and the RO-to-PRU mapping rules between the FD-FDD (full-duplex FDD) UEs and the HD-FDD UEs should be totally the same if the HD-FDD UEs share the RO/PRU configuration with FD-FDD UEs. Otherwise, if one of the rules mentioned above is different, e.g. using the validation rules in TDD bands, the gNB has to try two receiving alternatives for HD-FDD or FD-FDD UEs, which increases the receiving complexity and even achieve bad receiving performance if the receiving beams are formed in the analog domain.

One example is given as shown in Figure 1. There are 4 ROs and 8 PRUs in an association period, with 2 potential preambles in each RO and mapping to 4 SSBs in total. There are two FDMed ROs with red cross shown in Figure 1 overlapped with SSBs in time so that if following TDD’s definition, these two ROs are invalid. The RO/PO validation and mapping patterns following FDD’s and TDD’s definition are illustrated in Figure 1, respectively. It is obvious that the different valid ROs lead to different RO/preamble-to-SSB and RO/preamble-to-PRU mapping pattern, which means the network will increase the receiving complexity and/or with a bad receiving performance to receive RO/PRU.

Figure 1. An example of RO/PO validation and mapping follow FDD’s/TDD’s definition

Observation 1: The RO/PO validation and mapping rules of HD-FDD UEs following the TDD definition may cause the receiving complexity increasing and even bad receive performance when the receiving beams are formed in the analog domain, if the RO/PO configurations are shared between HD-FDD and FD-FDD UEs.

So we propose the HD-FDD UEs follow the FDD’s definition for RO/PO validation. Not only the RO/PO configurations but also the validation and mapping rules are shared between HD-FDD and FD-FDD UEs. As mentioned above, the SSB symbols and tdd-UL-DL-ConfigurationCommon configurations are the main difference between FDD’s and TDD’s definition of validation, and it is assumed no tdd-UL-DL-ConfigurationCommon configurations existent in FDD bands, the only factor to make the difference is the SSB symbols. When the HD-FDD UE choose an RO to transmit, the UE can implement to transmit preamble in RO and not to receive the overlapped SSB but wait for the following SSB period to receive the SSB. In this case, there is no impact for HD-FDD UE to follow the FDD’s definition of RO/PO validation and mapping rules.

Proposal 1: The RO/PO validation rules and RO/Preamble-to-PRU mapping rules of HD-FDD UEs follow the rules of FDD’s definition.

On the collision handling of Case 8

In this section, the three sub cases of Case 8 remained in last meeting are discussed, i.e. the valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, with UE-dedicated configured DL reception and with dynamically scheduled DL reception, respectively.

Valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set

For valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, some options are concluded for down-selection as following.

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured PDCCH

  - Option 2: Leave to UE implementation whether to receive the configured PDCCH or transmit the PRACH on the valid RO

  - Option 3: If configured PDCCH is in a Type-2 CSS set, then PDCCH is prioritized; otherwise the valid RO is prioritized

  - Option 4: Configured PDCCH is prioritized over valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

The UE’s behaviors in Option 1, Option 3 and Option 4 are fixed by the specification, which means no flexibility left to gNB and UE. For some special cases that the time resources (period and the slots/symbols in the period) of RO and CSS configurations are totally the same, there will be no available UL or DL resources for HD-FDD UEs. For example, if Option 4 is employed, and the time resources of RO and CSS are the same, all the ROs cannot be used due to lower priority and collision. Things are the same to Option 5 if the priority is configured in advance.

Option 2 brings much flexibility to UE and is more reasonable in this case. The DCIs in Type 0/0A/2 (searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace) CSS set are cell common DCI that will be broadcasted in the cell from time to time. It means the DCI Type 0/0A/2 set is not intended to the UEs who send the preamble in the RO overlapping with the CSS set. If Option 2 is employed, UE can choose a suitable RO to transmit preamble without worrying about the miss detection of PDCCH even though the RO is overlapped with Type 0/0A/2 PDCCH, since the DCIs in Type 0/0A/2 PDCCH are expected to be sent again afterwards and UE can choose to detect them then. Towards Type 1 CSS for DCI scrambled by RA-RNTI, MsgB-RNTI, or TC-RNTI, even though the network expects the specific UEs to receive the DCIs instead of broadcasting them, the preamble transmitting shall not overlap with these DCIs’ receiving during 4-step or 2-step RA procedure. For example, the UE detects the DCI scrambled by RA-RNTI or MsgB-RNTI within a RAR window after the transmission of preamble, and detects the DCI scrambled by TC-RNTI after the transmission of Msg3. So for all the PDCCH in Type 0/0A/1/2 CSS set overlapping with RO, it is feasible to leave to UE implementation whether to receive or transmit.

Proposal 2: For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave to UE implementation whether to receive the configured PDCCH or transmit the PRACH on the valid RO (Option 2).

Valid RO overlapping with UE-dedicated configured DL reception

For valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), some options are concluded for down-selection as following.

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured DL

  - Option 2: Leave to UE implementation whether to receive the configured DL or transmit the PRACH on the valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

Different to the case in Section 3.1, the DL resource in this case is UE-dedicated configured resource, which implies the network can configure the resources according to UE’s HD-FDD capability. The network expects the UEs to receive the UE-dedicated configured DL when network actually transmit, e.g. receive UE-specific DCI that scheduling DL/UL transmission. Or the network does not transmit to the UE when the UEs are allowed to transmit preambles in the overlapped RO. So it is reasonable to give network the flexibility to decide the priority configured with each dedicated DL resource. Thus both the network and UE can confirm whether the RO or DL configuration is with higher priority for each overlapped resource.

Proposal 3: For Case 8 of valid RO overlapping with UE-dedicated configured DL reception, the priority should be configured by network, e.g. via a priority indicator (Option 5).

Valid RO overlapping with dynamically scheduled DL reception

For valid RO overlapping with dynamically scheduled DL reception, some options are concluded for down-selection as following.

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD for operation on a single carrier /single cell in unpaired spectrum

  - Option 2: Leave to UE implementation whether to receive the DL or transmit the PRACH on a valid RO

  - Option 3: Follow the handling of Case 1 that when the cancellation timeline is satisfied, the UE cancels the PRACH transmission and receives the DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 2 in R1-2103809)

  - Option 4: Valid RO is prioritized over dynamic DL that UE performs PRACH transmission and does not perform the DL receptions (Interpretation 3 in R1-2103809)

  - Option 5: When the cancellation timeline is satisfied, the UE neither performs transmission nor receives any DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 1 in R1-2103809)

There are two cases for the dynamically scheduled DL reception, one is during the RA procedure, e.g. the Msg4 reception, and the other is in RRC_CONNECTED state. The previous one is discussed in Section 3.1, so in this section it can be assumed that the network knows the UE’s HD-FDD capability when UE is dynamically scheduled by the network in RRC_CONNECTED.

Since the network knows the RO configuration, if the RO is prior to dynamic DL, the HD-FDD UEs shall consider no dynamic DL reception overlapped with RO, and the network should not schedule the DL transmission overlapped with RO, i.e. this case can be considered as an error case if the RO is prior to dynamic DL reception. In other words, if this case is allowed, the dynamic DL should be prior to RO, i.e. the Option 3 should be supported.

Proposal 4: For Case 8 of valid RO overlapping with dynamically scheduled DL reception, when the cancellation timeline is satisfied, the UE cancels the PRACH transmission and receives the DL signal/channels on the symbols overlapping with PRACH occasion (Option 3).

Conclusions

Observation 1: The RO/PO validation and mapping rules of HD-FDD UEs following the TDD definition may cause the receiving complexity increasing and even bad receive performance when the receiving beams are formed in the analog domain, if the RO/PO configurations are shared between HD-FDD and FD-FDD UEs.

Proposal 1: The RO/PO validation rules and RO/Preamble-to-PRU mapping rules of HD-FDD UEs follow the rules of FDD’s definition.

Proposal 2: For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave to UE implementation whether to receive the configured PDCCH or transmit the PRACH on the valid RO (Option 2).

Proposal 3: For Case 8 of valid RO overlapping with UE-dedicated configured DL reception, the priority should be configured by network, e.g. via a priority indicator (Option 5).

Proposal 4: For Case 8 of valid RO overlapping with dynamically scheduled DL reception, when the cancellation timeline is satisfied, the UE cancels the PRACH transmission and receives the DL signal/channels on the symbols overlapping with PRACH occasion (Option 3).

- References

RAN1 Chair’s Notes for 3GPP TSG RAN WG1 Meeting #105-e.

3GPP TS38.213, Physical layer procedures for control.