3GPP TSG RAN WG1 Meeting #106-e	R1-2106484

E-meeting, Aug 16th – 27th, 2021

Agenda Item:	8.4.3

Source:	Huawei, HiSilicon

Title:	Discussion on HARQ enhancement for NTN

Document for:	Discussion and Decision

Introduction

In RAN1#105-e, a couple of agreements were reached to enhance HARQ for NTN [1]

Agreement:

For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2

Agreement:

For Type-2 HARQ codebook in NTN,

- For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes

- FFS: Whether DCI for SPS release and any other DCIs are included in counting of C-DAI and T-DAI

Agreement:

Confirm the previous working assumption for  X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH for the given HARQ process.

Agreement:

For enhancement on the HARQ process indication at least for DCI 0-1/1-1, the Option-1 and Option-1a are lower priority for further discussion.

Agreement:

Discussion of enhancement(s) on the aggregated transmission (including repetition) is prioritized to improve the performance in NTN.

In this contribution, we provide some further considerations on HARQ enhancement for NTN.

HARQ enhancement in NTN

HARQ process ID indication

It was agreed that the maximum supported HARQ process number for NTN shall be extended to 32, and in RAN1 105-e [1], it was further agreed that the indication filed of HARQ process number extend to 5 bits for DCI 0-2/1-2.

In the current specification, the field of HARQ process number for DCI format 0_0/1_0 and DCI format 0_1/1_1 are fixed to 4 bits in length [2]. The total length DCI format 0_1/1_1 is configurable as some fields depend on UE-specific configurations. Once the UE knows all the configurations, the size of DCI 0_1/1_1 is known so that UE can perform blind decoding correctly. The DCI format 0_0/1_0 only supports a limited functionality, and the total DCI size is small and fixed in length. To ensure the compatibility with existing specification, it is desirable not to introduce additional bits in fallback DCI with extended maximum HARQ process number. However, as it has been agreed that the indication field of HARQ process number extends to 5 bits for DCI 0_2/1_2, to keep the consistency of non-fallback format, the indication of HARQ process number for DCI 0_1/1_1 may also be extended to 5 bits for NTN, i. e. fixed 5 bits for NTN and 4 bits for TN.

Proposal 1: For DCI format 0_1/1_1, the field of HARQ process number is extended to 5 bits in NTN.

For DCI format 0_0/1_0, it is preferred not to extend the field of HARQ process number due to its static DCI size. Besides, since the maximum supported HARQ process number may depend on UE capability, it is not expedient to extend HARQ process number ID field by default when there are also UEs supporting only 16 HARQ processes. In addition, DCI 1_0 is used to schedule SIB1 which is expected to indicate the network type, i.e. TN or NTN, if the size of fallback DCI 0_0/1_0 is extended for NTN and remain unchanged for TN, the UE has to do blind decoding of two different DCI sizes for SIB1 PDSCH if the UE does not know the type of the network before SIB1. Consequently, it would introduce extra complexity at UE side.

Observation 1: For DCI format 0_0/1_0, extending the HARQ process ID field would introduce extra complexity at UE side.

According to the discussion in previous meetings, another solution is to reuse one bit from other bit field in the DCI. This option is more preferable, particularly now that enhancements on the aggregated transmission (which include repetition) have been agreed at RAN1#105-e in order to improve the NTN performance. Therefore, the idle bits of RV field can also be used to indicate the MSB/LSB of HARQ process number. Furthermore, due to limited link budget, high data rate is not required in NTN scenarios, which means that the existing 5 bits for MCS indication may be redundant, consequently 1~2 bits can be reused for other purpose, including extended HARQ process ID indication.

Proposal 2: For DCI format 0_0/1_0, keep the fixed 4 bits of HARQ process number field and support indication of HARQ process ID via reusing idle bits of DCI.

HARQ-ACK codebook enhancements

It was agreed that the enabling/disabling of HARQ feedback is configured per UE and per HARQ process, i.e. the disabled HARQ processes do not need HARQ-ACK feedback. Note that the concept of “disabling” means that network is not interested in receiving the feedback.

Type-1 HARQ-ACK codebook

In RAN1#105-e, enhancement to Type-1 HARQ-ACK codebook were discussed targeting two corner cases. The first one is when only disabled HARQ processes are scheduled and the other one is when all DCIs of enabled HARQ processes are not decoded at the UE. It was proposed to skip the HARQ-ACK feedback when these cases happen. However, it should be first clarified whether the two above cases are typical operations for NTN. It have been discussed PDSCH carrying important message, e.g. MAC CE, should be scheduled with enabled process to avoid potential misunderstanding between the gNB and UE. The MAC CE signalling such as TAC may need to be informed to UE frequently due to the high mobility of satellite in NTN, which means there is a high likelihood that most of the time the HARQ codebook will contain the feedback for this kind of MAC CE. As a result, the case of all scheduled processes being disabled occurs rarely.

Observation 2: The case where all the scheduled HARQ processes are disabled is a rare corner case as there is a high likelihood that the HARQ codebook contains the feedback for TAC.

It is important to note that the size of Type-1 codebook depends on the MA,C occasions for candidate PDSCH receptions. Within the configuration of a set of slot timing values K1 and the information provided by tdd-UL-DL-ConfigurationCommon, the MA,C occasions of any UL slot can be determined. The size of Type-1 codebook is not changed with the dynamic scheduling of DCI. This is why Type-1 codebook is semi-static. However, feedback disabling mechanism has been introduced in NTN, the semi-static HARQ-ACK codebook (Type-1) which assumes that all the HARQ processes need ACK/NACK feedback may result in redundant HARQ-ACK bits, especially when HARQ process number is extended to 32 but only a few HARQ processes are enabled.

Recalling the two cases discussed in last meeting, for the second case of miss-detected all DCIs scheduling enabled processes, gNB would never know whether UE can decode the DCI correctly or not, UL resources should always be reserved for feedback as long as one feedback enabled process is associated with PDSCH occasion. Note that even in the case that only a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, UE would determines a HARQ-ACK codebook only for the PDSCH reception [3] and gNB would reserve all feedback resource based on the MA,C occasions. Consequently, with disabling mechanism, if gNB still reserve all feedback resource based on the MA,C occasions, a large number of resources would be waste. However, if gNB only reserve feedback enabled resources for the two corner cases, which means that the Type-1 codebook size can be decided based on the scheduling of DCI. It changed the semi-static characteristic for Type-1 codebook. A more generic method targeting more typical cases to save the HARQ-ACK feedback payload is preferable.

Observation 3: With disabling mechanism, if gNB still reserve all feedback resources based on the MA,C occasions, a large number of resources would be waste.

Observation 4: As long as one enabled HARQ process is scheduled, the gNB has to reserve feedback resources anyway as UE miss-detection is unpredictable.

In fact, when a large number of HARQ processes are disabled, e.g. only one HARQ process is enabled, the HARQ-ACK payload size can be reduced by skipping the feedback for the disabled HARQ processes, which will reduce the overhead of UL transmission. Taking the maximum HARQ number of 32 for example, the signalling overhead can be reduced as large as 32x.

Observation 5: For Type-1 HARQ-ACK codebook, by skipping the feedback of PDSCH occasions from disabled HARQ process, the signalling overhead can be reduced as large as 32x.

The benefit to report a NACK for a disabled HARQ process is not clear as on one hand it violates the principle of HARQ disabling and on the other hand there is no need to use it as a channel state indication since gNB will not do re-transmission anyway. If channel state information is needed, using the latest CSI report is sufficient due to the property of NTN channel.

The HARQ-ACK codebook size can be determined based only on the enabled HARQ processes. One possible solution is to schedule the enabled/disabled HARQ processes together so that HARQ-ACK for the disabled HARQ processes can be removed from the HARQ-ACK codebook. The detailed scheme for optimizing Type-1 HARQ-ACK codebook is illustrated in Figure 1 (a). The ACK/NACK feedback for enabled HARQ processes are scheduled in slot 1~4, and the HARQ processes with feedback disabled are scheduled in slots 5~8. Consequently, the HARQ-ACK bits for potential PDSCH transmissions in slot 5~8 can be removed, i.e. HARQ-ACK codebook size is reduced from size A to size B.

Figure 1. Optimization for Type-1 HARQ-ACK codebook

To ensure a common understanding on HARQ-ACK codebook size between the UE and gNB, some assistant information, e. g. a bitmap can be introduced to indicate which of the PDSCH occasions need HARQ-ACK feedback. In fact, it has been agreed that enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signalling. Therefore, UE can also be configured to know which HARQ process is disabled regardless whether DCI is decoded or not. Furthermore, for the set of slot timing values K1 associated with the active UL BWP, the UE determines a set of MA,C occasions for candidate PDSCH receptions, and the size of Type-1 codebook is depend on the MA,C occasions. As an example, the occasions for candidate PDSCH receptions is MA,C = {1, 2, 3}, if the second occasion for PDSCH reception is configured as disabled, the PDSCH occasions that need HARQ feedback become {1, 3}. Consequently, UE generates ACK/NACK in position {1, 3} for corresponding PDSCH, and if any DCI is missed at the UE, NACK is generated. For the second PDSCH occasion, as gNB is not interested in the feedback, UE shall skip the transmission occasions regardless whether DCI is decoded or not.

Observation 6: UE can be configured to know whether a PDSCH occasion is for feedback disabled or enabled HARQ processes.

Actually, the more general option of skipping all feedback disabled processes has more benefits than the enhancement targeting corner cases. Suppose that the set of slot timing values K1 associated with the active UL BWP is K1 = {1,2,3,4,5,6,7,8} and the UL-DL configuration is set as Figure 2. For the UL slot 8, the associated candidate PDSCH occasions MA,C = {1, 2, 3, 4, 5, 6, 7, 8}. Suppose that only one feedback enabled process is scheduled, as shown in Table 1, only 1-bit size for HARQ-ACK codebook is sufficient with the scheme mentioned above. While with the existing specification or no feedback only if there is no HARQ enabled processes being scheduled, gNB would have to reserve 8-bit HARQ-ACK codebook resource. Obviously, the resource would be wasted no matter UE feedback NACK or nothing when UE miss-detect the only enabled scheduled DCI. Moreover, it should be noted that the all feedback disabled case is just a special case of the general option. In conclusion, instead of targeting corner cases, one should focus on enhancement applicable for more typical scenarios. Specifically, skipping the feedback of PDSCH occasions from disabled HARQ processes as enhancement can save the overall signalling overhead.

Figure 2. UL-DL Configuration

Table 1. UL resource reserved by gNB

|  | UL resource reserved by gNB |  |
| --- | --- | --- |
|  | Case 1 (only disabled processes are scheduled) | Case 2 (all enabled processes are miss-detected) |
| Existing technology | 8 bit | 8 bit |
| Corner case enhancement | 8 bit (No enhancement on semi-static feature) or 0 bit(enhancement on semi-static feature) | 8 bit (No enhancement on semi-static feature) or 1 bit(enhancement on semi-static feature) |
| General enhancement (Skip the feedback of all disabled processes) | 0 bit | 1 bit |

Observation 7: For Type-1 HARQ-ACK codebook, when the only enabled DCI miss-detected, the benefit of skipping feedback to reduce signalling overhead is marginal.

Proposal 3: For Type-1 HARQ-ACK codebook, a UE can be configured with a bitmap which indicates a set of PDSCH occasions for which HARQ-ACK feedback is provided.

Type-2 HARQ-ACK codebook

For Type-2 HARQ ACK codebook, the value of counter downlink assignment index (C-DAI) field in DCI denotes the accumulative number of PDSCH receptions up to the current serving cell and current PDCCH monitoring occasion [4]. And the value of total DAI (T-DAI) in DCI denotes the total number of PDSCH receptions up to the current PDCCH monitoring occasion. The value of T-DAI is the same for all serving cell at a same PDCCH monitoring occasion, and it is updated from PDCCH monitoring occasion to PDCCH monitoring occasion.

As an example, Figure 3 presented possible values of C-DAI and T-DAI. When disabled HARQ processes are configured, the redundancy caused by the disabled HARQ processes can be reduced by only including HARQ-ACK bits for enabled HARQ-ACK feedback. Consequently, the values of the DAI fields in a DCI format only accumulate when the HARQ-ACK is enabled for related PDSCH receptions. The value of C-DAI in DCI updates with the number of enabled HARQ process as shown in Figure 3, the value of T-DAI denotes the total number of enabled HARQ processes up to the current PDCCH monitoring occasion, and the value of T-DAI keeps the same for all serving cell at a same PDCCH monitoring occasion.

By the indication shown in Figure 3, values of C-DAI and T-DAI in the DCI of feedback-disabled HARO process can be utilized for UE to determine DCI miss detections. Suppose that UE misses DCIs as shown in Figure 3, by comparing the values of C-DAI and T-DAI of the disabled HARQ process with the previous decoded enabled HARQ process, UE would decode the T-DAI value as “7”. Because the indication bits of T-DAI unchanged but the indication bits of C-DAI changed. However, if the value of C-DAI reserved, as shown in Figure 4, UE would decode the T-DAI value in the feedback disabled process as “3”. And UE would feedback a codebook with size 3. This would result in misunderstanding between the gNB with UE. Although gNB may correctly detect the HARQ-ACK codebook by blind detection assuming different codebook sizes, there is no way for the gNB to know which DCI have been missed. One additional benefit by keeping the C-DAI

Observation 8: For Type-2 HARQ-ACK codebook, keeping the true values of C-DAI and T-DAI as the count of feedback-enabled HARQ processes can be useful to detect DCI missing at the UE.

Besides, when a UE is configured with one serving cell, sometimes only C-DAI field is present in the DCI, and the value of C-DAI of disabled HARQ process should be kept as the feedback enabled processes so that the UE can detect possible DCI missing which is possible if C-DAI field is reserved.

Observation 9: If one serving cell is configured and only C-DAI is present in the DCI, C-DAI for the disabled HARQ process should not be reserved in order to detect DCI missing.

Figure 3. Value of C-DAI and T-DAI for Type-2 HARQ codebook with disabled HARQ process

Figure 4. Reserved C-DAI value for Type-2 HARQ codebook with disabled HARQ process

Proposal 4: In Type-2 codebook, for the DCI of PDSCH with feedback-disabled HARQ process, give true values of C-DAI and T-DAI as the count of feedback-enabled processes.

Type-3 HARQ-ACK codebook

In NR Rel-16, the Type-3 HARQ codebook is introduced for unlicensed band in order to benefit from the HARQ-ACK retransmission due to listen-before-talk (LBT) failure [5]. In Type-3 HARQ-ACK codebook, UE would generate HARQ-ACK feedback for all HARQ processes for all the configured serving cells in a one shot manner. Furthermore, in the RAN1#103-e, it was agreed that Type-3 HARQ codebook is also applicable for licensed spectrum. As NTN is using the licensed band, and Type-3 codebook does not have ambiguity issue caused by DCI missing, there is no need to preclude the usage of Type-3 HARQ codebook in NTN. Given the feedback for the disabled HARQ processes may result in redundant HARQ-ACK bits, potential enhancement can be considered in NTN. As one alternative, the Type-3 HARQ-ACK codebook can be enhanced simply by skipping the disabled HARQ processes. Figure 1 (b) shows the enhancement that reduces codebook size from A to B. Similar to Type-1 HARQ-ACK codebook, UE can be configured to know the position of disabled/enabled HARQ process.

Figure 5. Optimization for Type-3 HARQ-ACK codebook

Proposal 5: For Type-3 HARQ-ACK codebook, skip the feedback of PDSCH occasions from disabled HARQ processes.

Enhancement on transmission

At the RAN1#105-e, enhancements on the aggregated transmission (include repetition) is prioritized to improve the performance in NTN. The aggregated data is transmitted to UE in multiple slots with configured redundancy versions, and soft-combining can be applied accordingly depending on aggregation scheme and UE buffer status. Correspondingly, the aggregation level, aggregation pattern or redundancy version sequence and the corresponding time and frequency resources may be indicated when aggregation level is larger than one (1). In Rel-16, it has been agreed that the maximum number of repetitions is 16 which is configured by repetitionNumber-r16 for DL and numberOfRepetitions-r16 for UL. For NTN, the performance of DL decoding with disabled HARQ processes can be ensured with repetition.

As a side note, when the data is transmitted following a default aggregation/redundancy pattern, the indication for aggregation pattern can be avoided while the aggregation/repetition number is also needed according to different channel qualities. In addition, with more data transmitted at one time, the corresponding time and frequency resource need to be extended, e. g. more symbols are needed for time resource allocation. Furthermore, as different satellite orbit heights has different RTD and link budget, the aggregation/repetition number and pattern can be decided based on different scenarios. For example, for GEO which often suffer worse link budget than LEO, gNB can schedule four aggregation versions one time by using same DCI to improve the decode performance, as shown in Figure 6.

Figure 6. Aggregation transmission scheme

Proposal 6: Aggregation/repetition transmission parameters can be configured depends on orbital height.

Compared with higher layer based indication, indication via DCI provides better flexibility. As listed in table 2, some HARQ related DCI fields may be redundant when a HARQ process is disabled. As introduction of extra DCI size is not expected considering the blind DCI detection complexity as the UE side, all these idle bits of HARQ disabled processes are preferred to be kept, and can be reused for other purpose, including  indication of transmission parameters.

Table 2. Idle bits in DCI when HARQ disabled

| HARQ related DCI fields | Format 1-0 （bits） | Format 1-1 （bits） | Format 0-0 （bits） | Format 0-1 （bits） |
| --- | --- | --- | --- | --- |
| New data indicator (NDI) | 1 | 1 | 1 | 1 |
| Redundancy version (RV) | 2 | 2 | 2 | 2 |
| Downlink assignment index (DAI) | 2 | 0,2,4 | 0 | 0,1,2 |
| TPC command for scheduled PUCCH | 2 | 2 | 0 | 0 |
| PUCCH resource indicator | 3 | 3 | 0 | 0 |
| PDSCH-to-HARQ feedback timing indicator | 3 | 0,1,2,3 | 0 | 0 |

Proposal 7: Aggregation/repetition transmission parameters can be indicated via DCI.

Proposal 8: Reinterpret idle bits in DCI for indicating transmission parameters.

In addition, some UE assistance information including channel status, UE buffer status and PDSCH decoding statistics, can also be considered for transmission enhancement, e.g.:

- Upon detecting channel status deteriorating or memory status varying to a pre-defined condition, UE can report this to the gNB so that the gNB can initiate an early retransmission

- With timely knowledge of channel status, gNB can dynamically re-configure the data transmission parameters e. g. aggregation or MCS for HARQ disabled processes.

Accordingly, resources for channel status indication can be scheduled in a semi-static fashion, dynamic fashion, or upon UE’s request, as shown in Figure 6. The long RTD of NTN will introduce large delay for the configuration if it is scheduled by UE’s request. Moreover, since the NTN channel is dominated by LOS model, the channel state will not likely change seriously before the updating of transmission parameters.

Figure 6. Resource configuration for reporting opportunities

Proposal 9: UE assistance information reporting in reserved resource can be considered for NTN.

Conclusions

In this contribution, we discussed enhancements for HARQ procedures in NTN. Then we get the following observations and proposals:

Observation 1: For DCI format 0_0/1_0, extending the HARQ process ID field would introduce extra complexity at UE side.

Observation 2: The case where all the scheduled HARQ processes are disabled is a rare corner case as there is a high likelihood that the HARQ codebook contains the feedback for TAC.

Observation 3: With disabling mechanism, if gNB still reserve all feedback resources based on the MA,C occasions, a large number of resources would be waste.

Observation 4: As long as one enabled HARQ process is scheduled, the gNB has to reserve feedback resources anyway as UE miss-detection is unpredictable.

Observation 5: For Type-1 HARQ-ACK codebook, by skipping the feedback of PDSCH occasions from disabled HARQ process, the signalling overhead can be reduced as large as 32x.

Observation 6: UE can be configured to know whether a PDSCH occasion is for feedback disabled or enabled HARQ processes.

Observation 7: For Type-1 HARQ-ACK codebook, when the only enabled DCI miss-detected, the benefit of skipping feedback to reduce signalling overhead is marginal.

Observation 8: For Type-2 HARQ-ACK codebook, keeping the true values of C-DAI and T-DAI as the count of feedback-enabled HARQ processes can be useful to detect DCI missing at the UE.

Observation 9: If one serving cell is configured and only C-DAI is present in the DCI, C-DAI for the disabled HARQ process should not be reserved in order to detect DCI missing.

Proposal 1: For DCI format 0_1/1_1, the field of HARQ process number is extended to 5 bits in NTN.

Proposal 2: For DCI format 0_0/1_0, keep the fixed 4 bits of HARQ process number field and support indication of HARQ process ID via reusing idle bits of DCI.

Proposal 3: For Type-1 HARQ-ACK codebook, a UE can be configured with a bitmap which indicates a set of PDSCH occasions for which HARQ-ACK feedback is provided.

Proposal 4: In Type-2 codebook, for the DCI of PDSCH with feedback-disabled HARQ process, give true values of C-DAI and T-DAI as the count of feedback-enabled processes.

Proposal 5: For Type-3 HARQ-ACK codebook, skip the feedback of PDSCH occasions from disabled HARQ processes.

Proposal 6: Aggregation/repetition transmission parameters can be configured depends on orbital height.

Proposal 7: Aggregation/repetition transmission parameters can be indicated via DCI.

Proposal 8: Reinterpret idle bits in DCI for indicating transmission parameters.

Proposal 9: UE assistance information reporting in reserved resource can be considered for NTN.

- References

- Chairman’s Notes RAN1#105-e.

- TS 3GPP, 38.212, “Multiplexing and channel coding”.

- TS 3GPP, 38.213, “Physical layer procedures for control”.

- RP-193234, “Solutions for NR to support non-terrestrial networks (NTN)”, Sitges, Spain, 9th-13th December, 2019.

- TR 38.821 V16.0.0, “Solution for NR to support non terrestrial networks (NTN) ”, 3GPP