3GPP TSG RAN WG1 #106-e		R1-2106526

e-Meeting, August 16th – 27th, 2021

Title: 	Remaining Issues of XR Traffic Model

Source: 	ZTE, Sanechips

Agenda item:	8.14.1

Document for:	Discussion and decision

Introduction

During RAN1#105-e meeting, the remaining issues for single stream, including e.g., the PDB of UL AR video and  DL traffic for separate packet arrival under dual-eye buffer, have been agreed in [1].

In this paper, we further discuss the remaining aspect, including the multiple stream model for I-stream and P-stream as well as FoV-stream and non-FoV stream.

Multiple Streams Modeling for DL Video

  - 2.1 I-frame & P-frame

During RAN1#105-e meeting, an initial tabulation has been proposed for two types of I/P-stream modeling [1] as follows,

Agreement: For the optional evaluation scenario, two streams of I-frame and P-frame for DL video stream (option 1), the traffic models described in the below table are assumed. - FFS: Parameter values of , A, B, C, D, E, F, G, H - Including the possibility of using multiple set of parameter values - For companies who are evaluating this option, it is recommended to evaluate at least the following scenario: AR/VR, 30Mbps, Dense Urban for FR1 and InH for FR2. It is encouraged to evaluate additional baseline/optional scenarios/configurations. Two data streams, i.e. M1 = 2; Option 1A: slice-based; Option 1B: GOP-based. - I-stream; - P-stream; - I-stream; - P-stream. Packet modeling; Slice-level; Frame-level. Traffic pattern; Both streams are periodic at 60 fps with the same jitter model as for single stream.; Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream.. Number of packets per stream at a time; - 1; - N-1; - I-frame: 1 or 0 - P-frame: 0 or 1 - At each time instant, there is either only one I-stream packet or only one P-stream packet. - N = 8: the number of slices per frame.. Average data rate per stream; RI=R*αN-1+α; RP=R*N-1N-1+α. - R: average data rate of a single stream video - : average size ratio between one I-frame/slice and one P-frame/slice, e.g. = 1.5, 2, 3. Packet size distribution; - Truncated Gaussian distribution. Mean =; Mean =; Mean =; Mean =. - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video. PER, PDB; [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms]; [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms].

However, the parameters including, e.g., average data rate per stream between I-stream and P-stream, PER of the two streams and PDB of the two streams, are still required for further study.

The average data rate per stream between I-stream and P-stream can be obtained by statistics [2] for SA4 input [3][4]. The VR2 configurations in [3][4] are listed in Table A in the Appendix.

The VR2 configurations thereof can be classified as:

- Slice-based multiple streams modeling: VR2-1, VR2-2, VR2-3, VR2-4, VR2-6, VR2-7, VR2-8.

- Group of Picture (GoP)-based multiple streams modeling: VR2-5.

For the analysis of slice-based multiple stream modeling, we calculate mean packet size based on all I-slices/P slices, respectively. The parameter for identifying I-slices and P-slices is the “importance” field in [3]. A summary of the packet size info. for slice-based multiple stream modeling within VR2 configurations is shown in Table 1.

Table 1 Summary of mean packet size ratio for slice-based traffic model

| Configuration | - slice Mean Packet size (Byte) | - slice Mean Packet size (Byte) | Average Packet size ratio |
| --- | --- | --- | --- |
| VR2-1 | 5991.3 | 3218.7 | 1.86 |
| VR2-2 | 5992.2 | 3218.8 | 1.86 |
| VR2-3 | 6328.7 | 3318.4 | 1.91 |
| VR2-4 | 6325.5 | 3317.5 | 1.91 |
| VR2-6 | 5990.8 | 3218.4 | 1.86 |
| VR2-7 | 8374.0 | 4623.3 | 1.81 |
| VR2-8 | 8374.2 | 4623.4 | 1.81 |

The average packet size ratio between I-slices and P-slices is ranging from 1.81 to 1.91 in VR2 configurations provided by SA4.

Accordingly, seems to be the reasonable ratio for slice-based multiple stream modeling.

For the analysis of GoP-based multiple streams modeling, method for obtaining average packet size ratio between I-frames and P-frames is the same as that of slice-based multiple streams. As a result, the summary of the GoP based multiple stream modeling within VR2 configurations is shown in Table 2.

Table 2 Summary of mean packet size ratio for GoP-based traffic model

| Configuration | - frame Mean Packet size (Byte) | P-frame Mean Packet size (Byte) | Mean Packet size ratio |
| --- | --- | --- | --- |
| VR2-5 | 55340 | 52637 | 1.05 |

The average packet size ratio between I-frames and P-frames is around 1.05 in VR2 configurations provided by SA4.

In addition, in RAN1#105-e meeting, three candidate values for , including e.g., ,  and have been proposed. Given 11 UEs per cell, the capacities of slice-based and GoP-based multiple streams modeling with these three values are shown in Table 3.

Table 3 Satisfied UE percentage for slice-based traffic model with different alpha values

| Alpha values | Satisfied UE percentage |
| --- | --- |
|  | 97.54% |
|  | 97.54% |
|  | 97.35% |

The capacity performance is unchangeable whatever the average packet size ratio changes for the sliced-based traffic model.

To minimize the simulation efforts and facilitate cross verification of the simulation results, the following is proposed.

The average packet size ratio is 2 as baseline and companies can optionally report the results with other ratios, including, e.g., 1.5 and 3.

In case of I-stream and P-stream DL traffic modeling, a UE is declared a satisfied UE if more than (100-A) (%) or (100-E) (%) of I-streams are successfully delivered within a given air interface PDB C or G ms as well as more than (100-B) (%) or (100-F) (%) of P-streams are successfully delivered within a given air interface PDB D or H ms. Hence, the following traffic characteristics for I-streams and P-stream should be determined.

- Slice-based multiple streams modeling:

  - I-slices: (A%, C ms)

  - P-slices: (B%, D ms)

- GoP-based multiple streams modeling:

  - I-frames: (E%, G ms)

  - P-frames: (F%, H ms)

For sliced-based traffic model, I-slices and P-slices arrive at the same time. Hence, we have C = D. According to the agreement in RAN1#104-e meeting, the baseline PDB is 10 ms for AR/VR, while the baseline PDB is 15ms for CG, which have already been well evaluated in single stream simulation. Hence, the optional PDB for AR/VR and CG in the agreement are able to be considered in multi-stream simulation, in order to obtain more simulation results in accordance with the current agreement. To this end, for the AR/VR traffic, we propose C = D = 20, while C = D = 30ms for CG traffic. Moreover, I/P slice should exhibit a certain level of reliability differentiation, A = 1 and B = 10 may be a reasonable packet error rate for I-slices and P-slices, respectively. Then we suggest a baseline KPI for I-slices and P-slices:

- AR/VR: I-slices: (1%, 20ms), P-slices: (10%, 20ms)

- CG: I-slices:(1%, 30ms), P-slices:(10%, 30ms)

For GoP-based traffic model, I-frames are more important than P-frames in a GoP so that higher reliability should be considered for I-frame to ensure its delivery. Therefore, for PER, we have E = 1 and F = 10. While for PDB, we have G = H = 10. Then the baseline KPI for I-frames and P-frames can be

- AR/VR: I-frames: (1%, 10ms), P-frames:(10%, 10ms)

- CG: I-frames: (1%, 15ms), P-frames:(10%, 15ms)

Table 4 Baseline and Option values from A to H

| Traffic Model | Parameter | Proposed baseline values | Optional values per RAN1 agreements |
| --- | --- | --- | --- |
| Slice-based model | A (PER_I) % | 1 | 0.5, 5, 10, >10 |
|  | B (PER_P) % | 10 | 5, >10 |
|  | C (PDB_I) ms | 20 for AR/VR, 30 for CG | 10 for AR/VR, 15 for CG |
|  | D (PDB_P) ms | 20 for AR/VR, 30 for CG | 10 for AR/VR, 15 for CG |
| GoP-based model | E (PER_I) % | 1 | 0.5, 5, 10, >10 |
|  | F (PER_P) % | 10 | 0.5, 5, 10, > 10 |
|  | G (PDB_I) ms | 10 for AR/VR, 15 for CG | 20, 30 |
|  | H (PDB_P) ms | 10 for AR/VR, 15 for CG | 20, 30 |

Based on the above discussion, we make the following proposal for I/P stream modeling:

Consider the parameters for I/P stream modeling in Table 5 as baseline.

Table 5 Summary of parameters for I/P stream modeling

| Application | AR/VR/CG |  |
| --- | --- | --- |
| Two stream data | Stream #1: I-frame Stream #2: P-frame |  |
|  | Option 1 Sliced-based | Option 2: Frame-based (GoP) |
| Structure | A frame consists of: Number of Stream #1: 1 Number of Stream #2: 7 | A GoP consists of: Number of Stream #1: 1 Number of Stream #2: 7 |
| Frame per second | Stream #1: 60FPS Stream #2: 60FPS | Stream #1 + Stream #2 = 60FPS |
| Average packet size ratio | Stream #1 : Stream #2 = 2:1 |  |
| (PSR, PDB) | AR/VR: Stream #1: (99%, 20ms) Stream #2: (90%, 20ms) CG: Stream #1: (99%, 30ms) Stream #2: (90%, 30ms) | AR/VR: Stream #1: (99%, 10ms) Stream #2: (90%, 10ms) CG: Stream #1: (99%, 15ms) Stream #2: (90%, 15ms) |

  - 2.2 FoV & Non-FoV

According to SA4 input [5], FoV and non-FoV traffic model is distinctive of the VR1 traffic. Non-FoV stream can be regarded as the background of 360°video in relatively low resolution, while FoV stream is the 18 tiles in high resolution to enhance the scene corresponding to user’s current viewpoint.

From our perspective, similar to the I/P-frame modeling, the FoV and non-FoV modeling has the two options as follow.

Option 1:   FoV stream and non-FoV stream are integrated in one frame for encoding and transmission as shown in Figure 1. Hence, this modeling is similar to sliced-based traffic model of I/P-stream modeling.

Figure 1 Option1 for FoV and non-FoV modeling

Option 2: FoV stream and non-FoV stream are two separate streams with different bit rate and periodicity. In detail, FoV stream is divided into multiple tiles to encode and transmission as shown in Figure 2.

Figure 2 Option2 for FoV and non-FoV modeling

For the FoV and non-FoV modeling, relevant enhancement on UL assistance information is elaborated in our companion contribution [6].The KPI differentiation of FoV and non-FoV stream are similar to those of I-frame and P-frame. FoV stream is the high resolution scene to enhance the scene of users’ current viewpoint. The quality of FoV stream transmission directly determines user experience. As a result, it seems that FoV stream has higher importance similarly to I-frame in multi-stream video traffic so that FoV stream may have more stringent KPI per UE than non FoV stream.

Therefore, we propose the following initial configuration for FoV and non-FoV modeling.

Table 6 Initial Parameters of FoV and non-FoV stream modeling

| Application | VR1 |  |
| --- | --- | --- |
| Two Stream Data | Stream #1: FoV stream Stream #2: Non-FoV stream |  |
|  | Option 1: Sliced based traffic model | Option 2: Two separate streams |
| Structure | A frame consists of: Stream #1: 1 (18 tiles) Stream #2: 1 | A Group of Tiles consist of: Stream #1: 18 tiles Stream #2: 1 |
| Frame Per Second | Stream #1: 30FPS Stream #2: 30FPS | Stream #1: 540 tiles per second Stream #2: 30FPS |
| Data Rate | Stream #1: 12.78 Mbps Stream #2: 8Mbps | Stream #1: 12.78Mbps (the aggregated data rate of the 18 tiles within a group of tiles) Stream #2: 8Mbps |
| (PSR, PDB) | Stream #1: (99%, 20ms) Stream #2: (90%, 20ms) | Stream #1: (99%, 10ms) Stream #2: (90%, 10ms) |

Based on the discussion above, we make the following proposal for FoV and non-FoV modeling.

Further discuss in RAN1 the parameters of FoV and non-FoV stream modeling for DL 360°video stream with parameters in Table 6 as a starting point.

Table 6 Initial Parameters of FoV and non-FoV stream modeling

| Application | VR1 |  |
| --- | --- | --- |
| Two Stream Data | Stream #1: FoV stream Stream #2: Non-FoV stream |  |
|  | Option 1: sliced based traffic model | Option 2: Two separate streams |
| Structure | A frame consists of: Stream #1: 1 (18 tiles) Stream #2: 1 | A Group of Tiles consist of: Stream #1: 18 tiles Stream #2: 1 |
| Frame Per Second | Stream #1: 30FPS Stream #2: 30FPS | Stream #1: 540 tiles per second Stream #2: 30FPS |
| Data Rate | Stream #1: 12.78 Mbps Stream #2: 8Mbps | Stream #1: 12.78Mbps (the aggregated data rate of the 18 tiles within a group of tiles) Stream #2: 8Mbps |
| (PSR, PDB) | Stream #1: (99%, 20ms) Stream #2: (90%, 20ms) | Stream #1: (99%, 10ms) Stream #2: (90%, 10ms) |

Conclusion

According to the discussion above, we prefer to discuss/adopt the following observations/proposals:

Observation 1: The average packet size ratio between I-slices and P-slices is ranging from 1.81 to 1.91 in VR2 configurations provided by SA4.

Observation 2: The average packet size ratio between I-frames and P-frames is around 1.05 in VR2 configurations provided by SA4.

Observation 3: The capacity performance is unchangeable whatever the average packet size ratio changes for the sliced-based traffic model.

Proposal 1: The average packet size ratio is 2 as baseline and companies can optionally report the results with other ratios, including, e.g., 1.5 and 3.

Proposal 2: Consider the parameters for I/P stream modeling in Table 5 as baseline.

Table 5 Summary of parameters for I/P stream modeling

| Application | AR/VR/CG |  |
| --- | --- | --- |
| Two stream data | Stream #1: I-frame Stream #2: P-frame |  |
|  | Option 1 Sliced-based | Option 2: Frame-based (GoP) |
| Structure | A frame consists of: Number of Stream #1: 1 Number of Stream #2: 7 | A GoP consists of: Number of Stream #1: 1 Number of Stream #2: 7 |
| Frame per second | Stream #1: 60FPS Stream #2: 60FPS | Stream #1 + Stream #2 = 60FPS |
| Average packet size ratio | Stream #1 : Stream #2 = 2:1 |  |
| (PSR, PDB) | AR/VR: Stream #1: (99%, 20ms) Stream #2: (90%, 20ms) CG: Stream #1: (99%, 30ms) Stream #2: (90%, 30ms) | AR/VR: Stream #1: (99%, 10ms) Stream #2: (90%, 10ms) CG: Stream #1: (99%, 15ms) Stream #2: (90%, 15ms) |

Proposal 3: Further discuss in RAN1 the parameters of FoV and non-FoV stream modeling for DL 360°video stream with parameters in Table 6 as a starting point.

Table 6 Initial Parameters of FoV and non-FoV stream modeling

| Application | VR1 |  |
| --- | --- | --- |
| Two Stream Data | Stream #1: FoV stream Stream #2: Non-FoV stream |  |
|  | Option 1: sliced based traffic model | Option 2: Two separate streams |
| Structure | A frame consists of: Stream #1: 1 (18 tiles) Stream #2: 1 | A Group of Tiles consist of: Stream #1: 18 tiles Stream #2: 1 |
| Frame Per Second | Stream #1: 30FPS Stream #2: 30FPS | Stream #1: 540 tiles per second Stream #2: 30FPS |
| Data Rate | Stream #1: 12.78 Mbps Stream #2: 8Mbps | Stream #1: 12.78Mbps (the aggregated data rate of the 18 tiles within a group of tiles) Stream #2: 8Mbps |
| (PSR, PDB) | Stream #1: (99%, 20ms) Stream #2: (90%, 20ms) | Stream #1: (99%, 10ms) Stream #2: (90%, 10ms) |

References

“Session notes for 8.14 (Study on XR Evaluations for NR)”, 3GPP TSG RAN WG1 Meeting #105-e, May 10th– 27th, 2021.

R1-2105603, “Remaining Issues of XR Traffic Model”, 3GPP TSG RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021.

http://dash.akamaized.net/WAVE/3GPP/XRTraffic/Traces/Candidate/VR2/

S4-210071. “[FS_XRTraffic] Proposed Additional Configurations, Traces and Traffic Models.” Qualcomm Incorporated. 3GPP TSG-SA4 Meeting #112e.

S4aV200614. “Proposed Updated Modeling for Viewport Dependent 3DoF Streaming”, Huawei, HiSilicon, 3GPP SA4-Post-111e, 22 December 2021.

R1-210652, “On Capacity and Power Working Areas for XR”, 3GPP TSG RAN WG1 #106-e e-Meeting, August 16th–27th, 2021

Appendix

Table A Various content type from SA4 Input

| Configuration | Basic Content Parameters |
| --- | --- |
| VR2-1 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-2 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |
| VR2-3 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, 1500 byte max packet size packets |
| VR2-4 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, unlimited packet size |
| VR2-5 | 1 slice per eye buffer, every 8th frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-6 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffers sent interleaved, 1500 byte max packet size |
| VR2-7 | 8 slices per eye buffer, 1 slice per frame is intra coded, 45Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-8 | 8 slices per eye buffer, 1 slice per frame is intra coded, 45Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |