3GPP TSG RAN WG1 #106-e		R1-2106573

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Further discussion on inter-cell MTRP operation

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

Introduction

In RAN1 #102-e meeting, the following agreements were made:

Agreement Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation. - Details on configuration of non-serving cell RS; - Allowed source and target RS types for RS transmitted from the non-serving cell TRP ; - Allowed QCL types for RS transmitted from the non-serving cell TRP ; - Measurement and reporting related to QCL /TCI enhancement except for that in 8.1.1, if any; - Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels; - Other details not precluded.

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

For future meetings

Consider rate matching behavior related to non-serving cell SSB.

In RAN1#103-e meeting, further agreements were made as below:

In RAN1#104-e meeting, further agreements were made as below:

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

- FFS: Whether the indication of PCI is implicit or explicit

Conclusion

Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel.

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Agreement

For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - FFS other non-serving cell information

- Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

  - FFS: how the flag is linked to non-serving cell

- Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

  - FFS: Each group is associated with a CORESETPoolIndex value.

  - FFS: how to link the group of TCI states to non-serving cell.

- Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

  - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, …

  - FFS: detailed re-indexing rule(s) of non-serving cell RSs

- Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

  - FFS: how the indicator is linked to non-serving cell

  - Note: when there is only one non-serving cell, it means the same as Option2.

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Conclusion

The UE may assume received DL transmission from multiple TRP within a CP in FR1 and FR2.

- Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.

In RAN1#104b-e meeting, further agreements were made as below:

Agreement

- For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC

  - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC.

  - Applicable at least for non-cross carrier QCL indication

    - FFS: Cross carrier scheduling QCL indication

- RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP

- Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

Conclusion

Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specifcation point of view

Agreement

For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e

- Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex

- Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex

- Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex

Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e

Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e

In this contribution, we discuss remaining issues on inter-cell MTRP including association of non-serving cell information with TCI state, rate matching etc. Most importantly, down selection among five options on TCI state association from RAN1#104-e, three alternatives on association with CORESETPoolIndex from RAN1#104b-e should be resolved in this meeting!

TCI state association with PCI different from serving cell PCI

There were different views among companies on association of PCI with CORESETPoolIndex for inter-cell multi TRP operation. Five options for associating TCI states with PCI different from the serving cell PCI were agreed in RAN1#104-e and three alternatives on association with CORESETPoolIndex were agreed in RAN1#104b-e. Since inter-cell multi TRP operation is assuming multi-DCI based multi-PDSCH reception based on Rel-15/16 TCI framework, CORESETPoolIndex will be configured to the UE supporting inter-cell multi TRP operation however from RRC signaling design point of view TCI state may not necessarily be directly associated with CORESETPoolIndex.

Among the 5 options agreed in RAN1#104-e, it can be noticed option 2 and 5 are similar when there is only one PCI which is different from the serving cell PCI. Option 1 is about associating TCI states with PCI different from the serving sell PCI, whether PCI is directly or indirectly associated requires further discussion. In option 3, the main point is grouping of TCI states corresponding to PCIs, which can be discussed in RAN1 together with three alternatives of CORESETPoolIndex association with TCI states. From the above analysis, the common part in options 1, 2, 3 and 5 is associating TCI with PCI and these options are details of how RRC signaling designed. In this sense, RAN1 needs to send LS to RAN2 to start working on signaling design.

A simple approach would be to introduce a new RRC IE which includes all information for the cell with PCI different from the serving cell PCI, which is then associated with TCI state, however it is up to RAN2 to design the detailed signaling. An example of RRC signaling is given below which avoids relationship with CORESETPoolIndex.

xxx_IE:

{

PCI,

SSB time domain position,

SSB transmission periodicity,

SSB transmission power,

}

And, the xxx_IE is associated the TCI state,

TCI-state:

{

Tci-stateID,

Qcl-type1,

Qcl-type2,

xxx_IE

}

The three alternatives on association with CORESETPoolIndex, which can be discussed in RAN1 and down selected. And, RAN1 specification can capture the agreement accordingly, for example, if RAN1 agrees on alt1 then something like “the TCI states associated with same PCI associated with one CORESETPoolIndex” can be captured in  38.214.

It was agreed in previous meeting that RAN1 to decide the maximum number of PCIs that can be configured to the UE, from flexibility point of view the maximum of configurable PCIs different than serving cell PCI can be larger than one however only 1 is active at a time.

Proposal1:

- Discuss and agree on maximum number of PCIs that can be configured to the UE to support inter-cell multi TRP operation

- Discuss and agree on the options (5 options from RAN1#104-e) for associating TCI state with PCI different from serving cell PCI, send LS to RAN2 on the agreements

- Discuss and agree on the alternatives (3 alternatives from RAN1#104b-e) for associating TCI states with CORESETPoolIndex, outcome of the agreements can be captured in RAN1 specification

QCL indication for CORESETs with common search space

It should be noted that it is also possible for the network to indicate the QCL source for the CORESETs with common search space, which is typically configured through MIB or SIBs. UE behaviour for these CORESETs should be clarified.

For example, if CORESET #0 TCI state is updated (through dedicated MAC CE signaling) to be associated with QCL source from SSB of another PCI, UE may begin to decode the SIB information of the target cell. It should be clarified whether such behavior is allowed or not in Rel-17 FeMIMO inter-cell multi-TRP operation.

Above clarification may not be limited for CORESET #0, but also needed for CORESETs associated with type 0/1/2 search spaces.

Proposal 2: Clarify UE behaviour when CORESETs with type 0/1/2 SS is configured/activated with TCI states associated with SSB of another PCI.

On rate matching of non-serving cell SSB

In RAN1#104-e, it was agreed that PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI. However, following two cases are left for further discussion:

- whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

gNB may configure rate matching pattern including SSB pattern(s) by implementation, hence for flexibility of implementation further restriction on rate matching behavior seems not necessary.

Proposal 3: PDSCH in non-serving cell is not rate matched around SSB from serving cell and PDSCH in serving cell is not rate matched around SSB from non-serving cell.

Another issue that needs discussion is on how to interpret “PDSCH/PDCCH from non-serving cell (PCI)” in the following agreement:

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

The straightforward understanding on “PDSCH/PDCCH from non-serving cell (PCI)” is the PDSCH/PDCCH QCL’ed indirectly with an SSB with different PCI from serving cell. Note that it has already been clarified in 8.1.1 that when RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel.

Proposal 4: Clarify that “PDSCH  from non-serving cell (PCI)” are those PDCH/PDCCH that use SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference.

- Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

Another aspect to clarify is that it might only be applicable for PDSCH.

Proposal 5: Update previous agreement on rate matching as following:

- PDSCH that uses SSB associated with a physical cell ID as an indirect QCL reference is rate matched around SSB with the same PCI as the indirect QCL reference of the PDSCH.

  - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

Conclusions

In this paper we discussed remaining issues on inter-cell multi-TRP operations, we have following proposals:

Proposal1:

- Discuss and agree on maximum number of PCIs that can be configured to the UE to support inter-cell multi TRP operation

- Discuss and agree on the options (5 options from RAN1#104-e) for associating TCI state with PCI different from serving cell PCI, send LS to RAN2 on the agreements

- Discuss and agree on the alternatives (3 alternatives from RAN1#104b-e) for associating TCI states with CORESETPoolIndex, outcome of the agreements can be captured in RAN1 specification

Proposal 2: Clarify UE behaviour when CORESETs with type 0/1/2 SS is configured/activated with TCI states associated with SSB of another PCI.

Proposal 3: PDSCH in non-serving cell is not rate matched around SSB from serving cell and PDSCH in serving cell is not rate matched around SSB from non-serving cell.

Proposal 4: Clarify that “PDSCH  from non-serving cell (PCI)” are those PDCH/PDCCH that use SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference.

- Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

Proposal 5: Update previous agreement on rate matching as following:

- PDSCH that uses SSB associated with a physical cell ID as an indirect QCL reference is rate matched around SSB with the same PCI as the indirect QCL reference of the PDSCH.

  - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

References

[1] RAN1#102e, Chairman’s notes

[2] RAN1#103e, Chairman’s notes

[3] RAN1#104e, Chairman’s notes

[4] RAN1#104be, Chairman’s notes