3GPP TSG RAN WG1 #106-e	R1-2106575

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Further discussion and evaluation on HST-SFN schemes

Agenda Item:	8.1.2.4

Document for:	Discussion and Decision

Introductions

In the previous RAN1 e-meetings, the following agreements and working assumption about enhancement on HST-SFN deployment and multi-TRP PDCCH were achieved and need further discussion.

Agreement Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - FFS: Details including UL RS enhancement - DL RS based Doppler feedback by UE - FFS: Details - FFS: Whether UE capability needs to be introduced - Whether to support one or both will be decided later Agreement When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations - Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB) - Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties - Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD) - Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting - Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation. Working Assumption For TRP-based pre-compensation, Variant A (based on RAN1#103-e meeting agreement) are supported as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states. - FFS: Additional support of Variant B Agreement - For TRP-based pre-compensation QCL assumptions is provided to the UE by using the existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type - FFS rule or signalling to determine which TCI state with dropped QCL parameters - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH. - FFS whether this restriction is per UE or per CC - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for different CORESETs. - FFS whether this restriction is per UE or per CC Agreement Enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is identified by the number of TCI states activated per CORESET and RRC parameter - FFS: Configuration detail of RRC parameter - Including whether the same RRC parameter is used for PDCCH and PDSCH Agreement If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP -based pre-compensation) is configured and a CORESET is activated with two TCI states and UE is configured with enableTwoDefaultTCI-States and time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, down-select rule to determine default beam(s) for Rel-17 SFN PDSCH reception in RAN1#106-e: - Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a - Alt 2: Introduce new rules to determine TCI states based on two TCI state(s) of the CORESET Agreement If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, support the following configuration of RS for BFD - Down-select one alternative for implicit configuration - Alt 1-2: RS of CORESETs with both single and two TCI states are used - Alt 1-3: RS of CORESETs with only two TCI states are used - Down-select one alternative for explicit configuration - Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS - FFS other details - Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration - Note: down-selection can be done separately for Rel-15/16 cell specific BFR and Rel-17 TRP-specific BFR, Rel-17 TRP-specific BFR to be discussed under AI 8.1.2.3

In this contribution, further analysis of QCL assumption of frequency offset pre-compensation would be provided. Link-level simulation results are given to show the performance of the potential enhancement. Additionally, the details about the indication for SFN transmission, default TCI states and BFD issues about SFN-based PDSCH/PDCCH are further discussed.

Discussion

HST-SFN deployment

In the HST scenario, due to frequent handover between the cells, SFN transmission scheme was introduced in LTE. Multiple TRPs with the same Cell ID are connected to one BBU to overcome frequent handover under HST-SFN deployment. However, one of the main problems of HST-SFN deployment is the opposite Doppler shifts from different TRPs. From UE’s perspective, the received signal is a combination of multiple delayed signals with different power and opposite high Doppler shifts in SFN manner as shown in Figure 1, which is quite challenging for UE demodulation.

HST-SFN deployment

To deal with opposite Doppler shifts, several potential solutions were proposed in RAN1#102 e-meeting, which basically can be classified as the UE-based solution and NW-based solution.

UE-based solution for HST-SFN

Scheme 1: SFN transmission with distributed TRS

In scheme 1, two TRS resource sets TRS1 and TRS2 are transmitted from TRP1 and TRP2 respectively, while PDSCH/PDCCH are transmitted in SFN manner, as shown in Figure 2. UE can estimate the Doppler shifts by measuring two TRS resource sets, and then apply the results for PDSCH/PDCCH DMRS estimation.

SFN transmission with distributed TRS

However, the channel of HST-SFN deployment is generally a LOS channel with multiple Doppler shifts, and the Doppler spread is typically not a U-shape spectrum. Therefore, after estimating the multiple Doppler shifts based on distributed TRS, UE should use them to reconstruct the Doppler spectrum, and then conduct time-domain interpolation for DMRS estimation by using the Wiener filter, instead of the U-shape Doppler spectrum assumption. That means a Rel-17 UE should simultaneously support the advanced time-domain interpolation designed for scheme 1 and the legacy U-shape spectrum time-domain interpolation for DMRS estimation, leading to the increasing of implementation complexity of Rel-17 UE.

Scheme 2: SFN transmission with distributed TRS and DMRS

In scheme 2, in addition to two TRS resource sets, two orthogonal PDSCH DMRS ports are also transmitted from TRP1 and TRP2 respectively, while PDSCH is still transmitted in SFN manner, as shown in Figure 3. UE can achieve estimating the channel of each TRP separately with orthogonal PDSCH DMRS ports, and then combine the channel of each TRP to reconstruct a composite channel for PDSCH demodulation.

SFN transmission with distributed TRS and DMRS

However, there are two main drawbacks of scheme 2 as follows.

- Complexity: Separate channel estimation of two TRPs needs a total of twice time-domain filtering to be operated for the LS channel estimation results from two orthogonal DMRS ports. Relatively, scheme 1 only needs once time-domain filtering with two Doppler shift values estimated by two TRS. Moreover,  the NW-based solution even has no requirement for UE to enhance its DMRS estimation processing. Therefore, from the perspective of UE, the complexity of scheme 2 is higher than other schemes.

- DMRS overhead: Two orthogonal PDSCH DMRS ports from two CDM groups separately would directly affect the overall throughput of PDSCH transmission in scheme 2.

Scheme 2 needs higher UE complexity and more DMRS overhead, compared to other proposed schemes for Rel-17 HST-SFN.

NW-based solution for HST-SFN

In the procedure of frequency offset pre-compensation, the key point is how to know the Doppler shifts that are used to process frequency pre-compensation in the NW side. There are two potential methods proposed in the previous meetings to handle this issue.

Frequency offset pre-compensation based on Doppler estimation by NW

Based on uplink signals, NW can estimate the uplink frequency offset based on the uplink signals such as SRS and PUSCH/PUCCH DMRS. The frequency offset pre-compensation process based on Doppler estimation using UL RS is provided in Figure 4.

Frequency offset pre-compensation based on Doppler estimation using UL RS

As described in Figure 4, UE transmits the UL RS signal to TRP1 and TRP2 at the uplink carrier frequency fc  + Δf, where Δf is the frequency adjusting value for uplink. Then TRP1 estimates the uplink frequency offset Δf + Δf1 and TRP2 estimates the uplink frequency offset Δf + Δf2. NW calculates the frequency pre-compensation value Δf1 - Δf2 for TRP2 by subtracting Δf + Δf2 from Δf + Δf1. It can be observed that the final frequency pre-compensation value Δf1 - Δf2 is independent of Δf. That means as long as satisfying the uplink frequency shifts estimate requirement, no matter what the exact value of Δf is, frequency offset pre-compensation can still work.

Therefore, the uplink carrier frequency can be adjusted based on UE implementation, regardless of the downlink frequency shift estimation by TRS. To avoid adjusting the oscillator frequently, after UE modulates the downlink carrier frequency based on TRS, it can transmit uplink signals at the same downlink carrier frequency by UE implementation. In this way, the frequency adjusting value Δf for uplink can be equal to Δf1, which achieves the same purpose as indicating the QCL-like association between uplink signals and TRS.

The frequency pre-compensation value can be derived based on implementation, regardless of the carrier frequency of UL RS.

Support implementation-based frequency offset pre-compensation using uplink signals for Doppler estimation, without defining QCL-like association between uplink signals and TRS.

Frequency offset pre-compensation based on Doppler feedback by UE

In the previous meetings, some companies had concerns about the feasibility and complexity when using UL RS for Doppler estimation at the NW side. Therefore, another potential way is UE to report the Doppler shifts estimated on TRSs to NW using the CSI framework. A feasible frequency offset pre-compensation process based on Doppler feedback is provided in Figure 5.

Frequency offset pre-compensation based on Doppler feedback

In the above procedure, firstly UE would adjust its downlink carrier frequency indicated by the TCI state associated with TRS1, then UE could directly feed back a pre-compensation value of the Doppler shift, rather than two TRP-specific Doppler values corresponding to each TRP. In this case, same as the pre-compensation based on Doppler estimation using UL RS in Figure 4, there is no need to perform frequency offset pre-compensation at TRP1.

In the current specification, TRS is used to track the Doppler shift which is no need to be reported. However, if the Doppler shift needs to be fed back by UE using the CSI framework, some mechanisms should be specified to support it. For instance, the direct or indirect associations of TRS, i.e. NZP-CSI-RS resource sets with trs-info and CSI report setting should be built. Subsequently, UCI mapping and quantization of the Doppler feedback value is a big issue. Moreover, when UE is far from one TRP, the power of received signals would be very low and the estimated Doppler shift corresponding to faraway TRPs would also be inaccurate. It would cause a certain degree of waste of feedback resources if the Doppler shift is still reported by UE in this case. Therefore, potential methods could be studied to save the CSI feedback resources.

If Doppler shift feedback is supported, instead of reporting two Doppler shift values separately associated with two TRPs, the difference value of two Doppler shifts should be reported.

Additionally, SFN-based PDSCH can be regarded as NC-JT transmission as well. If CSI-RS is also transmitted from two different TRPs in SFN manner, a UE will be able to estimate only the composite channel of two TRPs. As a result, the UE can only measure and report one PMI corresponding to the configured single CSI-RS resource set. However, the difference in the directions of the channels between the UE and two TRPs might cause performance degradation due to a single PMI derived from the mismatched composite channel. To avoid this negative impact, distributed CSI-RS can be introduced to measure respective PMI for different TRPs even for SFN transmission, as shown in Figure 6.

Distributed CSI-RS transmitted from two TRPs

In Figure 7, link-level simulation results for HST-SFN are given below to show the benefit of distributed CSI-RS configuration with enhanced SFN-based CSI feedback over a single CSI-RS configuration. In the evaluation, 8-port CSI-RS per TRP is assumed and frequency offset pre-compensation is applied to handle Doppler shifts that are prevalent in the HST-SFN scenario. It is observed that distributed CSI-RS can provide performance gain, compared to a single CSI-RS configuration. This is because two PMIs reported by UE are matching the propagation directions of two TRPs.

The comparison of SFN and distributed CSI-RS

Compared with SFN-based CSI-RS, distributed CSI-RS from different TRPs can provide further transmission gain for HST-SFN deployment.

If Doppler shift feedback is supported, it should be reported with 2PMI/RI/CQI in the same CSI report using Rel-17 MTRP CSI framework.

Timing offset pre-adjustment for HST-SFN

From the perspective of UE, the SFN channel is composed of multiple TRPs’ channels with respective received path power and delay. When UE is located at the middle point of two TRPs, the signals transmitted from two TRPs have the same path power and delay, but different arrival phases. Therefore, two TRP’s SFN signals might cancel out with each other at the middle point of two TRPs as shown in Figure 8 c), resulting in the low SNR of received signals. That would cause the degradation of UE demodulation performance. To solve the above problem, a small delay CDD can be introduced in a certain TRP. For instance, a fixed small delay CDD with the length of CP/16 can be added before signals are transmitted from the second TRP. After that, the relative delay of two LOS paths in the received SFN signal would be nearly CP/16, which can effectively avoid SFN signals canceling out with each out.

| a) Channel of TRP1 | b) Channel of TRP2 | c) Composed SFN channel at the UE side |
| --- | --- | --- |

TRP’s channel canceling out with each other when UE is located at 350m

Moreover, when UE is at some locations where the power of signals from two TRPs is similar, but the path delay difference is relatively large, the frequency-domain SFN channel would experience deep and more frequent fading as shown in Figure 9. That would also cause the performance degradation of SFN transmission. In this case, small delay CDD can also be used to reduce the path delay difference between two TRPs’ SFN signals, which can inhibit the deep and frequent fading of the composited SFN channel at the UE side.

| a) Channel of TRP1 | b) Channel of TRP2 | c) Composed SFN channel at the UE side |
| --- | --- | --- |

The deep and frequent fading of the composed SFN channel when UE is located at 250m

SFN signals from two TRPs might cancel out with each other at the middle point of two TRPs.

Lager delay between the SFN signals from two TRPs would cause deep and more frequent fading of the SFN channel, leading to performance degradation of SFN transmission.

However, a fixed small delay CDD at a certain TRP is not always optimal when UE is moving along the track. A potential way is to perform a dynamic small delay CDD corresponding to UE’s location. In this case, NW could adjust the Tx timing when transmitting the downlink signals to UE at each TRP. The main point is how does NW get the timing shift value for a certain TRP. Similar to Doppler shifts, timing shifts can also be estimated by UL RS or fed back from UE. To further enhance the performance of HST-SFN, TRP-specific timing offset pre-adjustment is an effective way to solve the mentioned drawbacks of HST-SFN and could be further studied.

| a) SNR=0dB | b) SNR=4dB |
| --- | --- |

Performance comparison of SFN w/s CDD and w/o CDD

Figure 10 shows the simulation results of SFN with small delay CDD and without small delay CDD using frequency offset pre-compensation. Type I codebook with PRG2 is applied to PDSCH which is mapped on 50 PRBs. According to the results, it is obvious that when UE is located at 350m, the performance of SFN without CDD is degraded by the cancellation of SFN signals. After introducing a fixed CDD with the length of CP/16, it can be found that the performance at 350m is improved significantly. However, a fixed CDD cannot improve the performance at every location and even would cause degradation at certain locations. Therefore, if the network can adjust the small delay CDD value dynamically based on UE’s location, the performance at other locations can also be improved. As shown in Figure 8 SFN with dynamic CDD to keep the received path delay difference of SFN signals at every location as CP/16 can generally improve the performance of UE demodulation.

Small delay CDD can improve the UE demodulation performance but is sensitive to delay offset added between the two TRPs.

TRP-specific timing offset pre-adjustment should be considered to further enhance the performance of HST-SFN transmission,  e.g. based on the feedback of timing offset between two different TRPs.

New QCL assumption for PDSCH/PDCCH DMRS

- Potential QCL reference for PDSCH/PDCCH DMRS

In the previous e-meetings, several QCL assumptions were proposed for the frequency offset pre-compensation scheme. They can mainly be divided into two types of realization as follows:

- Variant A: One of the TCI states can be associated with {average delay, delay spread} and another TCI state can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant B: One of the TCI states can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

- Variant C: One of the TCI states can be associated with {delay spread} and another TCI state can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

The main difference between Variant A and Variant C is whether another TCI state should be associated with average delay, which would lead to the difference of average delay estimation processing at the UE side as shown in Figure 11.

Comparison of the average delay estimation between Variant A and Variant C

In the existing specification, average delay is mainly measured by SSB and TRS and used as the downlink timing reference for UE. When only one TCI state is associated with average delay, it can be used as an anchor for UE to adjust downlink timing correctly, and help NW implement TRP-specific timing offset pre-adjustment.

For instance, in the previous procedure description of frequency offset pre-compensation, the downlink carrier frequency is assumed to be adjusted based on TRS1 as fc  +Δf1, where Δf1 is the Doppler shift estimate result corresponding to the transmission between TRP1 and UE, because TRS1 has stronger receiving power and can provide more precise Doppler shift estimation than TRS2. Similarly, the downlink timing adjustment can also refer to TRS1 with its average delay, then NW can implement a small delay CDD at the second TRP to further enhance the transmission performance.

Variant C is beneficial for TRP-specific timing offset pre-adjustment to enhance the performance of HST-SFN transmission.

- The dropping rule for QCL assumption

In the last meeting, the agreement that using the existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type has been achieved for TRP-based frequency offset pre-compensation. Assume that both TCI states associated with QCL-TypeA, i.e.,{average delay, delay spread, Doppler shift, Doppler spread} are indicated, UE should ignore some of the QCL properties of a certain TCI state.

However, before ignoring the QCL properties of a certain TCI state, UE should know which TCI state is the anchor to provide the complete QCL-TypeA reference or which TCI state is used to provide delay reference. Thus, the indication should be designed to make sure UE can know it. One potential solution is to specify a default dropping rule for the QCL assumption of TRP-based frequency offset pre-compensation, i.e. only refer to the delay parameter of the first/second TCI state associated with QCL-typeA. In this case, additional specific explicit indications for dropping the QCL parameters could be avoided.

Specify a default dropping rule that only refers to the delay parameter of the first or second TCI state associated with QCL-typeA for the frequency offset pre-compensation scheme.

Indication for SFN transmission

Considering the UE complexity, in the last meeting, we have achieved the agreement that UE does not expect to be configured with different SFN schemes, i.e. scheme 1 and frequency offset pre-compensation for both PDCCH and PDSCH, and different SFN schemes for different CORESETs. However, how to configure the SFN transmission for PDCCH and PDSCH by RRC parameters need further discussion.

Indication for SFN PDSCH

According to the agreements reached in the previous meetings, it has been agreed to support the semi-static switching of scheme 1 and frequency offset pre-compensation with R16 MTRP schemes for PDSCH. That implies specific RRC parameters would be specified to indicate scheme 1 and frequency offset pre-compensation for SFN PDSCH transmission.

Moreover, since scheme 1 and frequency offset pre-compensation would be both indicated with two TCI states associated with QCL-type A, UE should distinguish whether it is configured with frequency offset pre-compensation or not, before implementing the dropping rule of QCL assumption. Therefore, one feasible solution is to specify RRC parameters separately for scheme 1 and frequency offset pre-compensation, e.g. {sfnSchemeA, sfnSchemeB} respectively corresponding to these two SFN-based transmission schemes.

Support to specify RRC parameters separately, e.g. {sfnSchemeA, sfnSchemeB} as the indication for scheme 1 and frequency offset pre-compensation per BWP.

Indication for SFN PDCCH

There are several MTRP PDCCH schemes studied in the current R17 discussion, how to indicate the transmission scheme for SFN PDCCH is another important issue. In HST-SFN deployment, PDCCH and PDSCH are basically both transmitted in SFN manner. Since it is agreed to configure the same SFN scheme for both PDCCH and PDSCH for reducing the UE complexity, the same RRC parameter, e.g. {sfnSchemeA, sfnSchemeB}can also be used to indicate SFN PDCCH and PDSCH simultaneously to save the RRC overhead.

Regarding the multiple CORESETs configured for UE, different CORESETs can be configured with STRP-based transmission or SFN-based transmission based on the number of TCI states in the codepoint in MAC CE. Therefore,  it would be redundant to define an additional RRC parameter for indicating SFN PDCCH per CORESET.

Regarding SFN PDCCH indication, support to use the same RRC indication as SFN PDSCH, rather than a specific RRC parameter per CORESET.

Default TCI state for SFN transmission

Default beam for PDSCH

If a CORESET is indicated with two TCI states and the time offset between the reception of the DC and its scheduled PDSCH is less than the threshold timeDurationForQCL, UE would use the default beam for PDSCH reception. In the current specification, the default beam for PDSCH in some cases is related to the QCL assumption of the lowest CORESET ID in the last slot. Therefore, some new cases about the scheduling relationship between SFN-based PDCCH and corresponding PDSCH should be discussed. There are four potential situations as follows.

- Option1: SFN-based PDCCH scheduling PDSCH from STRP in Rel-15

- Option2: SFN-based PDCCH scheduling PDSCH from MTRP in Rel-16 (including scheme 1a,2a,2b,3,4), and UE is configured with enableTwoDefaultTCI-States

- Option3: SFN-based PDCCH scheduling PDSCH from M-TRP in Rel-16 (including scheme 3,4), but UE is not configured with enableTwoDefaultTCI-States

- Option4: SFN-based PDCCH scheduling SFN-based PDSCH from MTRP in Rel-17, and UE is configured with enableTwoDefaultTCI-States

Regarding option 2 and option 4, UE can receive two different beams simultaneously, meanwhile, at least one TCI codepoint indicates two TCI states. The default beam of these PDSCH transmissions are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states in MAC CE, regardless of the number of TCI states of the lowest CORESET ID.

Regarding option 1 and option 3, the default beam of PDSCH should refer to the lowest CORESET ID in the latest slot, in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. The case that the lowest CORESET ID configured with one TCI state has been supported in the existing specification. However, if the lowest CORESET ID is configured with two TCI states, how to determine the default beam of the PDSCH transmission from S-TRP may include the following alternatives.

- Alt1: gNB ensures the lowest CORESET ID in the latest slot only configured one TCI state by implementation

- Alt2: Modify the definition of the lowest CORESET ID in the latest slot, e.g. the lowest CORESET ID among the CORESETs associated with one TCI state in the latest slot.

- Alt3: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state

The pros and cons of these alternatives are listed in the below table.

The pros and cons of different solutions of default beam for PDSCH

|  | Pros | Cons |
| --- | --- | --- |
| Alt1 | No change in the current spec | There may be a certain restriction on RRC configuration. For example, gNB should ensure the lowest CORESET ID in every PDCCH monitoring occasion is always associated with one TCI state. |
| Alt2 | Small spec impact | There may be a certain restriction on RRC configuration, e.g. gNB should ensure at least one CORESET associated with one TCI state in every PDCCH monitoring occasion. |
| Alt3 | Small spec impact | Association with a certain TCI state should be defined. |

When the time offset between the reception of SFN-based PDCCH and its scheduled PDSCH is less than the threshold timeDurationForQCL and enableTwoDefaultTCI-States is not configured for UE, support to select the first or the second TCI state as the default TCI assumption for STRP and Rel-16 scheme 3/4 PDSCH.

When the TCI field is not present in the SFN-based PDCCH which schedules an SFN-based PDSCH transmission, but the time offset between the reception of the DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL, the default beam assumption should also be enhanced. In this case, regarding option 1 and option 3, the first or the second TCI state of the SFN-based PDCCH can be used as the default TCI assumption for STRP and Rel-16 scheme 3/4. Besides, regarding option 2 and option 4, two TCI states of the SFN-based PDCCH can be used as the default TCI assumption.

When the time offset between the reception of SFN-based PDCCH without TCI field and its scheduled PDSCH is equal or larger than the threshold timeDurationForQCL and enableTwoDefaultTCI-States is not configured for UE, support to select the first or the second TCI state as the default TCI assumption for STRP and Rel-16 scheme 3/4 PDSCH.

Default beam for CSI-RS

In Rel-16, for each AP-CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated with the QCL parameters through higher layer signaling of qcl-info which contains a list of TCI states for the AP-CSI-RS resources associated with the CSI triggering state. However, in case that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the AP-CSI-RS resources configured without higher layer parameter trs-Info is smaller than the beam-switching threshold which is indicated in reporting of UE capability, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET ID in the latest slot, where one or more CORESETs within the active BWP of the serving cell are monitored, and the CORESETs belong to the same CORESET pool index if configured. Therefore, the default beam assumption of AP-CSI RS can also refer to the lowest CORESET ID in the latest slot, which is the same as the default beam for PDSCH discussed above. For simplifying the spec impact, it is better to share the same design of the default beam for both AP-CSI RS and PDSCH.

The default TCI assumption for AP-CSI RS can be considered to apply a similar mechanism as PDSCH reception.

Current specification 38.214 defines the following: “For a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'on', the UE shall not expect to be configured with CSI-RS over the symbols during which the UE is also configured to monitor the CORESET, while for other NZP-CSI-RS-ResourceSet configurations, if the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that the CSI-RS and a PDCCH DM-RS transmitted in all the search space sets associated with CORESET are quasi co-located with 'typeD', if 'typeD' is applicable. This also applies to the case when CSI-RS and the CORESET are in different intra-band component carriers, if 'typeD' is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s).”

Therefore, for a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'off' and overlapping with a search space set associated with a CORESET in time, UE would assume the default beam for the CSI-RS refers to QCL assumption for the CORESET related to the PDCCH. However, if the CORESET is indicated with two TCI states, two alternatives should be considered to determine the default beam of the CSI-RS.

- Alt1: gNB ensures the CORESET ID only be configured one TCI states by implementation

- Alt2: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state

Alt1 may introduce some scheduling restrictions for CSI-RS but without any spec impact, while Alt2 is also a simple scheme to resolve the issue without restricting the scheduling flexibility of gNB.

If CSI-RS is overlapping in the time domain with CORESET associated with two TCI states, support to select the first or the second TCI state of the CORESET as the default TCI assumption for the CSI-RS.

BFD for SFN transmission

Beam failure detection is the first step of the beam failure recovery mechanism, in which UE monitors a set of q0  periodic CSI-RS resources to assess whether the beam failure trigger condition has been met or not. The CSI-RS can be explicitly configured by RRC signaling or implicitly associated by using the same QCL assumption with CORESETs. In the current specification, at most three CORESETs are configured for one active BWP and one of them is reserved as special CORESET-BFR, so UE only support up to a set q0 of two SSB or CSI-RS resources for BFD. Since UE can be configured with multiple CORESETs, the CORESET would be indicated with STRP-based transmission or SFN-based transmission. If the SSB or CSI-RS resources for BFD are implicit, and one of CORESETs is configured with two TCI states while the other CORESET is configured with one TCI state, UE would monitor three SSB or CSI-RS resources. If BFD RS is explicitly configured by RRC, the CSI-RS pair should be indicated corresponding to the CORESET with two TCI states, which needs extra specification effort.

Prefer implicit BFD RS configuration for both CORESETs with both single and two TCI states.

Furthermore, UE assesses the beam failure trigger condition according to SINR estimation based on SSB or CSI-RS against the SINR threshold of target PDCCH BLER. In the case of SFN-based PDCCH transmission, whether the SINR is estimated based on each BFD RS separately or based on two BFD RS together should be discussed. In order to reflect the performance of SFN-based PDCCH transmission more accurately, UE should calculate one SINR in SFN transmission assumption based on two BFD RS. UE can combine the channel estimation results based on two BDF RS in transmission assumption, and then estimate the SINR to identify the performance of PDCCH.

Support UE to calculate hypothetical BLER based on two BFD RS in SFN transmission assumption.

Conclusions

In summary, the following observations and proposals are made for Rel-17 HST-SFN enhancement.

- Scheme 2 needs higher UE complexity and more DMRS overhead, compared to other proposed schemes for Rel-17 HST-SFN.

- The frequency pre-compensation value can be derived based on implementation, regardless of the carrier frequency of UL RS.

Compared with SFN-based CSI-RS, distributed CSI-RS from different TRPs can provide further transmission gain for HST-SFN deployment.

SFN signals from two TRPs might cancel out with each other at the middle point of two TRPs.

Lager delay between the SFN signals from two TRPs would cause deep and more frequent fading of the SFN channel, leading to performance degradation of SFN transmission.

Small delay CDD can improve the UE demodulation performance but is sensitive to delay offset added between the two TRPs.

- Support implementation-based frequency offset pre-compensation using uplink signals for Doppler estimation, without defining QCL-like association between uplink signals and TRS.

- If Doppler shift feedback is supported, instead of reporting two Doppler shift values separately associated with two TRPs, the difference value of two Doppler shifts should be reported.

If Doppler shift feedback is supported, it should be reported with 2PMI/RI/CQI in the same CSI report using Rel-17 MTRP CSI framework.

TRP-specific timing offset pre-adjustment should be considered to further enhance the performance of HST-SFN transmission,  e.g. based on the feedback of timing offset between two different TRPs.

Variant C is beneficial for TRP-specific timing offset pre-adjustment to enhance the performance of HST-SFN transmission.

Specify a default dropping rule that only refers to the delay parameter of the first or second TCI state associated with QCL-typeA for the frequency offset pre-compensation scheme.

Support to specify RRC parameters separately, e.g. {sfnSchemeA, sfnSchemeB} as the indication for scheme 1 and frequency offset pre-compensation per BWP.

Regarding SFN PDCCH indication, support to use the same RRC indication as SFN PDSCH, rather than a specific RRC parameter per CORESET.

When the time offset between the reception of SFN-based PDCCH and its scheduled PDSCH is less than the threshold timeDurationForQCL and enableTwoDefaultTCI-States is not configured for UE, support to select the first or the second TCI state as the default TCI assumption for STRP and Rel-16 scheme 3/4 PDSCH.

When the time offset between the reception of SFN-based PDCCH without TCI field and its scheduled PDSCH is equal or larger than the threshold timeDurationForQCL and enableTwoDefaultTCI-States is not configured for UE, support to select the first or the second TCI state as the default TCI assumption for STRP and Rel-16 scheme 3/4 PDSCH.

The default TCI assumption for AP-CSI RS can be considered to apply a similar mechanism as PDSCH reception.

If CSI-RS is overlapping in the time domain with CORESET associated with two TCI states, support to select the first or the second TCI state of the CORESET as the default TCI assumption for the CSI-RS.

Prefer implicit BFD RS configuration for both CORESETs with both single and two TCI states.

Support UE to calculate hypothetical BLER based on two BFD RS in SFN transmission assumption.

References

RAN WG1 #105 e-meeting Chairman’s Notes.

Annex

- Simulation assumptions

Link-level simulation assumptions

| Parameter | Value |
| --- | --- |
| Duplexing | TDD |
| Carrier frequency | 3.5GHz |
| Subcarrier spacing | 30kHz |
| Propagation condition | CDL-based extension |
| TRP deployment | Ds=700m, Dmin=150m, TRP height=35m, UE height=1.5m |
| gNB antenna port configuration | 2 ports/8ports |
| UE antenna port configuration | 2 ports/4ports |
| Digital precoding method | Type I codebook, PRG=2 |
| TRS periodicity | 10ms, 2 slot TRS |
| DMRS type | Type 1 |
| Number of DMRS symbols | 1+1+1 |
| PDSCH mapping | Type A, Starting symbol 2, Duration 12 |
| Bandwidth | 10RB / 50RB |
| MCS | MCS adaptation |
| Rank | 1 |
| UE speed | 500km/h |