3GPP TSG RAN WG1 #106-e		R1-2106581

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

The following objective is included in this revised work item [1]:

- Physical layer aspects including [RAN1]:

  - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

In this contribution, we make some discussion about the number of RBs for enhanced PF0/1/4, including maximum number, configuration granularity, and the indication method. Additionally, we also discuss the RE mapping for PF0/1/4 with SCS 120kHz, the sequence type for PF0/1. Last, the rate-matching for PF4 is discussed.

- Discussion

Evaluation assumption and methodology

Our evaluation assumptions and reporting metrics are based on the baseline as agreed in RAN1#104-e[2]. There are some additional details we provided in the following Table 1. Furthermore, we list all the evaluation cases in Table 2 to express more clearly.

Table 1 The additional evaluation assumptions and reporting metrics

| Parameter | Notes | Values |
| --- | --- | --- |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) Note: If alternative RE allocation per PRB is used, companies to report details | For case 1/2, all REs are allocated per PRB. For case 3/4, there are 6 REs allocated per PRB. |
| Frequency hopping details | Frequency offset between hops, | If the PRB number of BWP is N_RB, then the frequency offset between the minimum PRB index in the first hop and the maximum PRB index in the second hop are N_RB – 1. |
| Number of RBs used per hop (N_RB) | N_RB contiguous RBs per hop | The contiguous RBs per hop is {1,2,3,4,5, 6….12} |
| Sequence construction details | Sequence type for PF0/1 Sequence type for DMRS of PF4 | For case 1/3, the sequence type is alt.1. For case 2/4, the sequence type is alt.2. |
| OCC configuration details | Applicable for PF1, PF4 | 1 user is configured by OCC index 0. OCC length is 2. |
| PUCCH payload encoder type | Reed Muller or Polar for PF4 | Reed Muller when payload is 4 bits or 11 bits. Polar when payload is 22 bits. |
| PUCCH encoding rate(s) | Applicable for PF4 If multiple payload sizes evaluated, encoding rates to be reported for each payload size | The encoding rates are listed in Table 3 for different payloads and different cases. |
| Number of multiplexed users, e.g., by code division, if applicable | 1 user Note: Companies to report if other cases if evaluated | Most cases are evaluated with 1 user while we also provide the evaluation with 2 users for PUCCH format 0. |
| Pmax (dBm) | Maximum allowed conducted power considering combined limit per region (from Table 3). Note: Companies should report if Pmax is considered per region or a combined limit is considered across multiple regions | Pmax is considered per region. |

Table 2 The cases of PUCCH evaluation

| Case | RE mapping: (X, Y) is used to state X contiguous PRBs per hop and Y REs are allocated per PRB for one user. | Sequence type and sequence length |
| --- | --- | --- |
| Case 0 | (1, 12) | Alt 1, 12 |
| Case 1 | (X≥2, 12) | Alt 1, X*12 |
| Case 2 | (X≥2, 12) | Alt 2, 12 |
| Case 3 | (X≥2, 6) with sub-PRB interlace | Alt 1, X*6 |
| Case 4 | (X≥2, 6) with sub-PRB interlace | Alt 2, 6 |

Table 3 The coding rates for PUCCH format 4

|  | Case 0 | Case 1 (2 PRB) | Case 1 (4 PRB) | Case 1 (8 PRB) | Case 1 (16 PRB) |
| --- | --- | --- | --- | --- | --- |
| 4 bits | 0.0278 | 0.0139 | 0.0069 | 0.0035 | 0.0017 |
| 11 bits | 0.0764 | 0.0382 | 0.0191 | 0.0095 | 0.0048 |
| 22 bits | 0.2292 | 0.1146 | 0.0573 | 0.0286 | 0.0143 |

For example, when X =2, the RE mapping and sequence type for evaluation cases mentioned above are as illustrated in Figure 1, the different colours show the sequence with cyclic shift across RBs, the same colour means a single sequence.

Figure 1 the RE mapping and sequence type for evaluation cases when X = 2

The number of RBs for enhanced PF0/1/4

Maximum number of RBs

In RAN1 104b-e meeting[3], the following agreement was made[2]:

Agreement: - The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least: - 12 RBs for 120 kHz SCS - 3 RBs for 480 kHz SCS - 2 RBs for 960 kHz SCS FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

RAN1 sent LS to RAN4 to ask whether or not RAN4 would specify UE power class(es) with larger values for maximum UE conducted power and maximum UE EIRP than the ones which were agreed in the RAN1 104-e meeting (maximum UE Conducted Power = 21 dBm, maximum UE EIRP = 25 dBm)[2][4]. RAN4 replied that their discussion is at an early stage so that they provided the FR2 minimum peak EIRP requirements for our reference[5] in Table 8 in the appendix.

We assume the UE_P = 23dBm, UE_EIRP = 43dBm or UE_P = 35dBm, UE_EIRP = 55dBm according to the FR2 EIRP requirements provided by RAN4 to calculate the maximum number of RBs for enhanced PF0/1/4, additionally the TxBF = 6dBi, CM = 2dB. The results are demonstrated in Table 4 and Table 5.

Table 4 The maximum number of RBs for enhanced PF0/1/4 when all REs within each RB are mapped

| Power class 1 UE_P = 35dBm, UE_EIRP = 55dBm | US | Europe | South Korea 1 (when an equipment is <300m from an astronomical antenna) | South Korea 2 (when an equipment is >=300m from an astronomical antenna) | Power class 2-5 UE_P = 23dBm, UE_EIRP = 43dBm | US | Europe | South Korea 1 | South Korea 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P_TX | P_TX = min(Pmax, 55– 6, 35 – 2) |  |  |  | P_TX | P_TX = min(Pmax, 43– 6, 23 – 2) |  |  |  |
| Pmax | 27 dBm – max(0, 10*log10(100 / BW)) = 33 | 23 dBm/MHz + max(0, 10*log10(BW)) - TxBF = 33 | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF = 33 | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF = 33 | Pmax | 27 dBm – max(0, 10*log10(100 / BW)) = 21 | 23 dBm/MHz + max(0, 10*log10(BW)) - TxBF = 21 | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF = 21 | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF = 21 |
| 120kHz | 277 | 28 | 277 | 277 | 120kHz | 18 | 2 | 18 | 18 |
| 480kHz | 70 | 7 | 70 | 70 | 480kHz | 5 | 1 | 5 | 5 |
| 960kHz | 35 | 4 | 35 | 35 | 960kHz | 3 | 1 | 3 | 3 |

Table 5 The maximum number of RBs for enhanced PF0/1/4 with comb-2 structure

| Power class 1 (35,55) | US | Europe | South Korea 1 | South Korea 2 | Power class 2-5 (23,43) | US | Europe | South Korea 1 | South Korea 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P_TX | P_TX = min(Pmax, 55– 6, 35 – 2) |  |  |  | P_TX | P_TX = min(Pmax, 43– 6, 23 – 2) |  |  |  |
| Pmax | 27 dBm – max(0, 10*log10(100 / BW)) = 33 | 23 dBm/MHz + max(0, 10*log10(BW)) - TxBF = 23 dBm/MHz – TxBF + 10*log10(N_RE *N_RB/N_ref)=33 | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF=13 dBm/MHz – TxBF + 10*log10(N_RE * N_RB/N_ref) =33 | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF=13 dBm/MHz – TxBF + 10*log10(N_RE * N_RB/N_ref) =33 | Pmax | 27 dBm – max(0, 10*log10(100 / BW)) = 21 | 23 dBm/MHz + max(0, 10*log10(BW)) - TxBF = 23 dBm/MHz – TxBF + 10*log10(N_RE *N_RB/N_ref)=21 | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF=13 dBm/MHz – TxBF + 10*log10(N_RE * N_RB/N_ref) =21 | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF=13 dBm/MHz – TxBF + 10*log10(N_RE * N_RB/N_ref) =21 |
| (N_ref=13/3, 25/12, 1 for SCS 120kHz/480kHz/960kHz) |  |  |  |  |  |  |  |  |  |
| 120kHz | 277 | 29 | 288 | 288 | 120kHz | 18 | 2 | 19 | 19 |
| 480kHz | 70 | 14 | 139 | 139 | 480kHz | 5 | 1 | 9 | 9 |
| 960kHz | 35 | 7 | 67 | 67 | 960kHz | 3 | 1 | 5 | 5 |

As shown in Table 4 and Table 5, the maximum number of RBs for two RE mapping methods are different. Furthermore, the maximum number of RBs needs to be revised if the above UE EIRP and conduct power for power class 2-5 UE are supported.

Observation 1: The maximum number of RBs for two different RE mapping methods is different.

For power class 1 UE, we furtherly provide the MIL results in Figure 2 to discuss whether should we support such a large RB number for enhanced PF0/1/4.  As shown in Figure 2, the MIL is limited by the EIRP of regulation power in US/Europe/South Korea 1 except for the UE_P and UE_EIRP when UE_P is 35dBm and UE_EIRP is 55dBm. Moreover, the MIL of large RB number is limited by the large CM value for case 2 in South Korea 2 when SCS is 120kHz or 480kHz.

Figure 2 The MIL of enhanced PF0 when SCS is {120kHz, 480kHz, 960kHz}, delay spread is 10ns, UE conduct power is 35dBm, UE_EIRP is 55dBm

Observation 2: The MIL is limited by EIRP of regulation power and affected by the sequence type when UE_P is 35dBm and UE_EIRP is 55 dBm.

Proposal 1: When considering the revision of maximum number of RBs, the coverage (maximum isotropic loss (MIL)) should be mainly considered, including the RE mapping method, sequence type, UE power class.

Configuration granularity on number of RB

In the RAN1 104b-e meeting[3], the following agreement was made[2]:

Agreement #RAN1 104b-e: Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4: - Alt-1: - For enhanced PF0/1 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS - For enhanced PF4 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers. - Alt-2: - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

We support alt-1 if the maximum RB number is not more than 16. The first reason is that the flexibility   RB indication would be increased if all integer values in the allowed range are supported. Furthermore, it would not cost more RRC signaling overhead compared with alt-2. Lastly, if we must choose some values from the allowed range, it is hard to identify the so-called effective values if we don’t align the MIL value for each RB number.

Proposal 2: For the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4, the alternative 1 is supported if the maximum RB number is not more than 16.

Indication of number of RBs

Agreement #RAN1 104-e: - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4)

Indication of NRB by cell-specific signaling (for PF0/1)

The indication of NRB for common PUCCH format 0/1 can be predefined in the table of PUCCH resource sets before dedicated PUCCH resource configuration or configured by SIB1. For the different regions, the number of RBs of PUCCH should be different which requires flexible indication. Even though the different values for different rows in the predefined table and SIB1 indication can both achieve the aim, the SIB1 indication method of increasing the number of bits would increase the complexity of UE decoding. Furthermore, the rows in the predefined table correspond to different coverage, so the different values of the number of RBs for each row is suitable.

In Rel.15, the PUCCH frequency resource allocation is decided once the first PRB index is decided because there is constant one PRB allocated for PUCCH 0/1. However, the PUCCH frequency resource for enhanced PUCCH 0/1 with multi-PRB should be dependent on not only the first PRB index but also the NRB. Moreover, the first PRB index is also dependent on the NRB if frequency hopping is supported for mutli-PRB PUCCH.

For example, when the NRB is 4, if rPUCCH8=0 ,	the UE determines the PRB index of the PUCCH transmission in the first hop as (RBBWPoffset+rPUCCHNCS)*4 and the PRB index of the PUCCH transmission in the second hop as NBWPsize-1-(RBBWPoffset-rPUCCHNCS)*4, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes. If rPUCCH8=1 ,	the UE determines the PRB index of the PUCCH transmission in the first hop as NBWPsize-1-(RBBWPoffset-rPUCCHNCS)*4 and the PRB index of the PUCCH transmission in the second hop as (RBBWPoffset+rPUCCHNCS)*4.

Proposal 3: The indication of NRB for common PUCCH format 0/1 can be predefined in the table of PUCCH resource sets before dedicated PUCCH resource configuration.

Proposal 4: The PUCCH frequency resource and the first PRB index are dependent on the NRB.

Indication of NRB by dedicated signaling (PF0/1/4)

For dedicated PUCCH resources, the frequency resource is RRC configured in Rel.15&Rel.16, so the NRB should be indicated by UE-specific RRC configuration. Similarly, the sub-PRB interlace RE mapping pattern can be also indicated by UE-specific RRC configuration. For example, the following IEs are listed for enhanced PF0. The PRBnumofmultiPRB indicates the number of RB, the comb and comboffset indicate the RE mapping structure.

PUCCH-format0 ::= SEQUENCE {

initialCyclicShift INTEGER(0..11),

nrofSymbols INTEGER (1..2),

startingSymbolIndex INTEGER(0..13)

comboffset                                    {0,1,2,3}

comb                                          {2,4}

PRBnumofmultiPRB                              {2,3,……16}

}

Proposal 5: The number of RBs for enhanced PF0/1/4 and RE mapping structure are indicated by dedicated RRC signaling.

Sequence construction for enhanced PF0/1

The sequence construction for enhanced PF4 is already agreed in the RAN1 104b-e meeting[3], in this section, we have some further discussion about the sequence construction for enhanced PF0/1.

The evaluation results for different sequence constructions

Considering the sequence type for enhanced PUCCH format 0/1, there are two alternatives agreed in the RAN1 104 e-meeting as the following agreement. Moreover, the maximum values for the NRB for enhanced PF0/1/4 have been agreed in the RAN1 104b-e meeting[3] according to the simulation assumption in the appendix. Firstly, the 95 percentile CM is provided in Table 6 and Figure 3 to compare the performance of the two sequence types.

Agreement #RAN1 104-e[2]: - For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives: - Alt-1: A single sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured. - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact Agreement #RAN1 104b-e[3]: - The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least: - 12 RBs for 120 kHz SCS - 3 RBs for 480 kHz SCS - 2 RBs for 960 kHz SCS - FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

Table 6 The 95-th percentile cubic metric [dB] for different number of RBs

| NRB | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alt1 (Case1) | 0.796 | 0.8147 | 1.309 | 2.111 | 2.145 | 2.202 | 2.249 | 1.601 | 2.205 | 1.771 | 2.295 | 2.031 |
| Alt2 (Case2) | 0.796 | 2.852 | 3.352 | 3.524 | 3.372 | 3.17 | 2.76 | 2.387 | 2.204 | 1.81 | 1.652 | 1.476 |
| Alt1 (Case3) | 1.444 | 0.7667 | 0.768 | 0.8029 | 2.394 | 1.305 | 2.089 | 2.123 | 2.202 | 2.144 | 1.61 | 2.211 |
| Alt2 (Case4) | 1.444 | 2.948 | 3.132 | 2.532 | 1.952 | 2.032 | 0.9752 | 1.364 | 1.956 | 2.178 | 2.276 | 2.54 |

Figure 3 the comparison of two sequence types of 95th percentile CM for different number of RBs

As we see from Table 6 and Figure 3, for case 1 and case 2, the CM of alt 2 is much larger than that of alt 1 when the PRB number is less than 9, and a bit lower when the PRB number is more than 10. For case 3 and case 4, the CM of alt 2 is higher than that of alt 1 when the PRB number is less than 5 or larger than 10 or equal to 6, while lower when the PRB number is equal to 5/7/8/9. Additionally, the trend of alt 1 is steadier than alt 2 with varying PRB numbers. In general, alt 1 has better CM performance than alt 2 if we support only one sequence type for enhanced PF0/1/4.

Observation 3: Alt 1 has better CM performance than alt 2 if we support only one sequence type for enhanced PF0/1/4.

Secondly, as we see from Figure 4 to Figure 6, the MIL performance is compared with different sequence types and RE mapping methods(for 120kHz SCS) varying with different numbers of RBs for enhanced PF0. As shown in Figure 4, when SCS is 120kHz, the MIL of the four cases we defined in Table 2 have minor differences. The MIL difference value is not more than 0.5dBm except when the UE_P is 23dBm and UE_EIRP is 43dBm and RB number is more than 15. While the SCS is 480kHz or 960kHz, the MIL of alt 1 is always larger than that of alt 2 no matter what the RB number is. The maximum MIL difference value between the two alternatives is more than 1 dBm. Similarly, Figure 7 to Figure 9 demonstrate the comparison of MIL performance for enhanced PF1. The MIL almost has the same trend for enhanced PF1 and PF0.

Observation 4: For the two alternatives of the sequence construction, the MIL has minor difference when SCS is 120kHz, and the alt 1 is better than alt 2 when SCS is 480kHz and 960kHz.

Figure 4 The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is 10ns with 1 OS, 1bit payload in the US

Figure 5 The MIL of enhanced PUCCH format 0 when SCS is 480kHz, delay spread is 10ns with 1 OS, 1bit payload in the US

Figure 6 The MIL of enhanced PUCCH format 0 when SCS is 960kHz, delay spread is 10ns with 1 OS, 1bit payload in the US

Figure 7 The MIL of enhanced PUCCH format 1 when SCS is 120kHz, delay spread is 10ns with 14 OS, 1bit payload in the US

Figure 8 The MIL of enhanced PUCCH format 1 when SCS is 480kHz, delay spread is 10ns with 14 OS, 1bit payload in the US

Figure 9 The MIL of enhanced PUCCH format 1 when SCS is 960kHz, delay spread is 10ns with 14 OS, 1bit payload in the US

Thirdly, in terms of the capacity of user multiplexing, the maximum supported number of multiplexing users of alt 1 is more than that of alt 2 if the cyclic shift of alt1 is enhanced as we demonstrated in section 2.3.2. Furthermore, in the current Rel-15/16 specification, the alt 1 is applied to PF3-DMRS, also applied to enhanced PF4-DRMS as agreed in the RAN1 104b-e meeting[3]. However, the alt 2 is applied only to UCI when the interlace is configured.

Observation 5: The capacity of user multiplexing of alt 1 is more than that of alt 2 if the cyclic shift of alt 1 is enhanced. Moreover, the specification impacts of the two alternatives are almost similar.

Proposal 6: For enhanced PUCCH format 0/1, the alt 1 of a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred.

The cyclic shift for a single long sequence

The cyclic shift α varies as a function of the symbol and slot number according to [8]

αl=2πNscRBm0+mcs+mint+ncsns,fμ,l+l' mod NscRB

For PUCCH format 0/1, where m0 is RRC configured (the value ranges from 0 to 11) for dedicated PUCCH resource and predefined for common PUCCH resource as in Table 7 -1. NscRB is 12 which is the total RE number within one PRB.  except for PUCCH format 0 when it depends on the information to be transmitted as Table 7-2 to Table 7-5 below.

As we discussed in section 2.3.1, in the current spec, the maximum number of multiplexing users of alt 1 and alt 2 are always 6 for PF0 with 1 bit HARQ-ACK no matter what the RB number is. However, if the cyclic shift is enhanced, the maximum number of multiplexing users of alt 1 would incease. For example, the sequence length is 24 when the PRB number is 2 for SCS 480kHz. To support more and better UE multiplexing, NscRB should be changed to 24. Additionally, the value range of m0 should change from 0～11 to 0～23 for dedicated PUCCH resources, the value candidates of m0 in Table 7-1 should also change for common PUCCH resources. Similarly, and the value of mcs in Table 7-2 to Table 7-5 should be changed depending on the length of the sequence.

Table 7 Copy of Table 9.2.1-1 and Table 9.2.3-3&4 and Table 9.2.5-1&2 from TS 38.213[9]

Table 7-1 Copy of Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration Index; PUCCH format; First symbol; Number of symbols; PRB offset; Set of initial CS indexes. 0; 0; 12; 2; 0; {0, 3}. 1; 0; 12; 2; 0; {0, 4, 8}. 2; 0; 12; 2; 3; {0, 4, 8}. 3; 1; 10; 4; 0; {0, 6}. 4; 1; 10; 4; 0; {0, 3, 6, 9}. 5; 1; 10; 4; 2; {0, 3, 6, 9}. 6; 1; 10; 4; 4; {0, 3, 6, 9}. 7; 1; 4; 10; 0; {0, 6}. 8; 1; 4; 10; 0; {0, 3, 6, 9}. 9; 1; 4; 10; 2; {0, 3, 6, 9}. 10; 1; 4; 10; 4; {0, 3, 6, 9}. 11; 1; 0; 14; 0; {0, 6}. 12; 1; 0; 14; 0; {0, 3, 6, 9}. 13; 1; 0; 14; 2; {0, 3, 6, 9}. 14; 1; 0; 14; 4; {0, 3, 6, 9}. 15; 1; 0; 14; {0, 3, 6, 9}. Table 7-2 Copy of Table 9.2.3-3: Mapping of values for one HARQ-ACK information bit to sequences for PUCCH format 0 HARQ-ACK Value; 0; 1. Sequence cyclic shift. Table 7-3 Copy of Table 9.2.3-4: Mapping of values for two HARQ-ACK information bits to sequences for PUCCH format 0 HARQ-ACK Value; {0, 0}; {0, 1}; {1, 1}; {1, 0}. Sequence cyclic shift. Table 7-4 Copy of Table 9.2.5-1: Mapping of values for one HARQ-ACK information bit and positive SR to sequences for PUCCH format 0 HARQ-ACK Value; 0; 1. Sequence cyclic shift. Table 7-5 Copy of Table 9.2.5-2: Mapping of values for two HARQ-ACK information bits and positive SR to sequences for PUCCH format 0 HARQ-ACK Value; {0, 0}; {0, 1}; {1, 1}; {1, 0}. Sequence cyclic shift.

Proposal 7：For a single sequence of length equal to the total number of mapped REs of the PUCCH resource, the cyclic shift should be adapted with the length of the sequence.

RE mapping for enhanced PF0/1/4 for 120 kHz SCS

In RAN1 #105-e[6], it is agreed that supporting at least Alt-1(all REs within each RB are mapped) for enhanced PF0/1, whether or not Alt-2 is additionally supported for PF0/1 and the supported RE mapping scheme(s) amongst {Alt-1, Alt-2} for enhanced PF4 need further discussion.

Agreement: - For 120 kHz SCS: - Support at least Alt-1 for enhanced PF0/1 for both PUCCH resources before and after dedicated PUCCH resource configuration - FFS: Whether or not Alt-2 is additionally supported for PF0/1 for either or both of the following: - PUCCH resources before dedicated PUCCH resource configuration - PUCCH resources after dedicated PUCCH resource configuration - FFS: Supported RE mapping scheme(s) amongst {Alt-1, Alt-2} for enhanced PF4 including design details - Notes: - Alt-1 = all REs within each RB are mapped - Alt-2 = a subset of REs within each RB are mapped (sub-PRB interlaced mapping) - Which RE mapping scheme(s) to support for PF0/1/4 to be concluded in RAN1#106 - Note: No further enhancements on RB shortage issue and frequency hopping distance issue should be considered for PUCCH resource sets prior to RRC configuration.

RE Mapping for enhanced PF0/1

We evaluate the user multiplexing cases shown in Figure 10. Figure 11 and Figure 12 illustrate the MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns} for the cases 1&3, and PRB number equals 2. In Figure 11, the power between the two UEs is the same called “power balance”. Owing to the power control tolerance in [7], 3dB power difference is considered to model power imbalance in Figure 12 called “power imbalance”. For alt 1-1 and alt 2-1, the power imbalance does not have an impact because the two users are multiplexed in different subcarriers. While for alt 1-2, the two users are multiplexed (CDM) with a single long sequence in the same RE, so, the power imbalance impact should be considered.

Figure 10 The user multiplexing cases for the two RE mapping alternatives

Figure 11 The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns}, and  power is balance

Figure 12 The MIL of enhanced PUCCH format 0 when SCS is 120kHz, delay spread is {5ns, 10ns, 20ns},  and power is imbalance

As we see from Figure 11, when power is balanced, the MIL of alt 1-1 is worse than those of alt 1-2 and 2-1 in US and South Korea, better in Europe. In Europe, the PSD is 23dBm/MHz, so the power is limited by the conduct power 21dBm. As we see from Figure 12, when power is imbalanced, the MIL of alt 2-1 is better than those of alt 1-1 and 1-2 in the US and South Korea as well as when delay spread is 10ns in Europe.

Observation 6: The alt 1-1 has MIL performance than those of alt 1-2 and 2-1. The alt 2-1 has better MIL performance when power is imbalanced.

On the other side, according to the current spec, as discussed in section 2.3.2, the maximum number of multiplexing users of alt 1-1 and alt 2-1 is double that of alt 1-2. For PUCCH resources before dedicated PUCCH resource configuration, if the alt 2 is supported, UE must support the two RE mapping modes which are not beneficial for initial access. However, for PUCCH resources after dedicated PUCCH resource configuration, the alt 2 should be supported especially for the worse coverage UE and for the high load cell.

Proposal 8: For enhanced PUCCH format 0/1, for 120 kHz SCS, we additionally support alt 2 for RE mapping for PUCCH resource after dedicated PUCCH resource configuration.

RE Mapping for enhanced PF4

For UCI of PUCCH format 4, the Fourier basis can be used for the OCC spreading code {[1,1,1,1], [1, j, -1, -j], [1, -1, 1, -1], [1, -j, -1, j]} to multiplex 4 UEs, and {[1,1], [1, -1]} to multiplex 2 UEs, according to [7], we can conclude that the pre-DFT-OCC user multiplexing is equivalent to the comb-based user multiplexing. So, we think it is not necessary to discuss additional sub-PRB interlace RE mapping for UCI of enhanced PUCCH format 4.

Figure 13 Example of Rel.15 PUCCH format 4 using DFT-precoder.

Proposal 9: For UCI of enhanced PUCCH format 4, all REs within each RB are mapped is preferred for 120kHz SCS as in Rel.15.

For DMRS of PUCCH format 4, the two alternatives of RE mapping are shown in Figure 14. As we see the evaluation results from Figure 15 to Figure 17, alt 2 has better MIL performance when the payload is 4 bits, 11  bits, and 22 bits,  delay spread is 10 ns, with 14 symbols whatever the UE_P, UE_EIRP, and the number of RBs assumptions are.

Figure 14 the RE mapping of alt 1 and alt 2 for DMRS of PF4

Figure 15 The comparison of two alternatives of RE mapping for DMRS of PF4, SCS is120Khz, delay spread is 10ns, payload =4 bits, 14 OS

Figure 16 The comparison of two alternatives of RE mapping for DMRS of PF4, SCS is120Khz, delay spread is 10ns, payload =11 bits, 14 OS

Figure 17 The comparison of two alternatives of RE mapping for DMRS of PF4, SCS is120Khz, delay spread is 10ns, payload =22 bits, 14 OS

Observation 7: Alt 2 has better MIL performance than Alt 1 for all evaluated payload size.

Proposal 10: For DMRS of PUCCH format 4, the sub-PRB interlaced mapping should be supported.

Rate matching for enhanced PF4

In the RAN1 104-e meeting[2], it was agreed that for PF4, the actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload. Then there are two questions discussed in the RAN1 105-e meeting[6].

Q1: How to make a limitation on the UCI payload for enhanced PF4?

- Alt-1a: Support the same restriction (upper limit) on the UCI payload as in Rel-15/16 PF4

- Alt-1b: Support different restrictions (upper limit).

Q2: How to do rate matching for enhanced PF4?

- Alt-2a: Rate matching to the configured number of RBs N_RB (similar to Rel-16 rate matching to the fixed number of RBs N = 10/11 of an interlace for PF3)

- Alt-2b: New rate matching mechanism, e.g., some form of repetition.

For question 1, we can observe from our evaluation, the MIL gain of larger RBs for enhanced PF4 is mainly from the coding rate gain. The UCI payload limitation is calculated by 12(OS except 2 DMRS symbol) * 12(RE) * 0.8(maxcoderate) =115. Since multiple PRBs are supported for enhanced PF4, the UCI payload limitation should multiply the number of RB if the same coverage is required. However, better coverage is required for enhanced PF4 so that the factor should depend on the evaluations. Otherwise, we should simply support the same restriction on the UCI payload as in Rel-15/16 PF4. For a larger UCI payload, the PF3 can bear.

For question 2, we prefer alt-2a. The repetition performance is not better than the low code rating gain of alt-2a. Moreover, the rate matching to the configured number of RBs is applied to the R16 interlace PF3.

Proposal 11: The rate matching to the configured number of RBs should be supported for enhanced PF4.

- Conclusion

In this contribution, we focus on the evaluation methodology, analysis of results, and the enhancement methods on the increasing of the number of RBs of PUCCH format 0/1/4, and have the following observations and proposals:

Observation 1: The maximum number of RBs for two different RE mapping methods is different.

Observation 2: The MIL is limited by EIRP of regulation power and affected by the sequence type when UE_P is 35dBm and UE_EIRP is 55 dBm.

Observation 3: Alt 1 has better CM performance than alt 2 if we support only one sequence type for enhanced PF0/1/4.

Observation 4: For the two alternatives of the sequence construction, the MIL has minor difference when SCS is 120kHz, and the alt 1 is better than alt 2 when SCS is 480kHz and 960kHz.

Observation 5: The capacity of user multiplexing of alt 1 is more than that of alt 2 if the cyclic shift of alt 1 is enhanced. Moreover, the specification impacts of the two alternatives are almost similar.

Observation 6: The alt 1-1 has MIL performance than those of alt 1-2 and 2-1. The alt 2-1 has better MIL performance when power is imbalanced.

Observation 7: Alt 2 has better MIL performance than Alt 1 for all evaluated payload size.

Proposal 1: When considering the revision of maximum number of RBs, the coverage (maximum isotropic loss (MIL)) should be mainly considered, including the RE mapping method, sequence type, UE power class.

Proposal 2: For the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4, the alternative 1 is supported if the maximum RB number is not more than 16.

Proposal 3: The indication of NRB for common PUCCH format 0/1 can be predefined in the table of PUCCH resource sets before dedicated PUCCH resource configuration.

Proposal 4: The PUCCH frequency resource and the first PRB index are dependent on the NRB.

Proposal 5: The number of RBs for enhanced PF0/1/4 and RE mapping structure are indicated by dedicated RRC signaling.

Proposal 6: For enhanced PUCCH format 0/1, the alt 1 of a single sequence of length equal to the total number of mapped REs of the PUCCH resource is preferred.

Proposal 7：For a single sequence of length equal to the total number of mapped REs of the PUCCH resource, the cyclic shift should be adapted with the length of the sequence.

Proposal 8: For enhanced PUCCH format 0/1, for 120 kHz SCS, we additionally support alt 2 for RE mapping for PUCCH resource after dedicated PUCCH resource configuration.

Proposal 9: For UCI of enhanced PUCCH format 4, all REs within each RB are mapped is preferred for 120kHz SCS as in Rel.15.

Proposal 10: For DMRS of PUCCH format 4, the sub-PRB interlaced mapping should be supported.

Proposal 11: The rate matching to the configured number of RBs should be supported for enhanced PF4.

- Appendix

Table 8. FR2 minimum peak EIRP requirements

| Power class | Max TRP [dBm] | FR2 band | Min peak EIRP [dBm] | Max EIRP [dBm] |
| --- | --- | --- | --- | --- |
| Power class 1 Fixed wireless access UE | 35 | n257/n258/n261 | 40.0 | 55 |
|  |  | n260 | 38.0 |  |
| Power class 2 Vehicular UE | 23 | n257/n258/n261 | 29.0 | 43 |
| Power class 3 Handheld UE | 23 | n257/n258/n261 | 22.4 | 43 |
|  |  | n260 | 20.6 |  |
|  |  | n259 | 18.7 |  |
|  |  | n262 | 16.0 |  |
| Power class 4 High-power non-handheld UE | 23 | n257/n258/n261 | 34.0 | 43 |
|  |  | n260 | 31.0 |  |
| Power class 5 Fixed wireless access UE | 23 | n257 | 30.0 | 43 |
|  |  | n258 | 30.4 |  |

- References

- RP-210862, Revised WID:  Extending current NR operation to 71 GHz, RAN #91e, Electronic Meeting, March 16-26, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 #104-e meeting.

- Chairman Notes, 3GPP TSG RAN WG1 #104b-e meeting.

- R1-2104060, [DRAFT] LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band, Ericsson, 3GPP TSG RAN WG1#104bis-e.

- R4-2107950, LS on maximum UE EIRP and conducted power, Intel, 3GPP TGS-RAN WG4 Meeting #99e.

- Chairman Notes, 3GPP TSG RAN WG1 #105-e meeting.

- 3GPP TS 38.101-2 V17.1.0 (2021-03), User Equipment (UE) radio transmission and reception, Part 2: Range 2 StandaloneR1-1912937, 3GPP TSG RAN WG1 Meeting #99, November 18th – November 22nd, 2019

- 3GPP TS 38.211 V16.5.0 (2021-03), Physical channels and modulation.

- 3GPP TS 38.213 V16.5.0 (2021-03), Physical layer procedures for control.