3GPP TSG RAN WG1 #106-e                                       R1-2106623

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs

Agenda Item:	8.12.1

Document for:  Discussion and Decision

- Introduction

In RAN#86 meeting, the work item of NR support of Multicast and Broadcast Services has been approved [1]. The followings have been identified as the objectives related with physical layer of the work item.

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [RAN1, RAN2]

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]:

  - Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1].

Note: the possibility of receiving Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Broadcast/Multicast service while in RRC CONNECTED state beforehand, is subject to verification of service subscription and authorization assumptions during the WI.

In RAN1 #105 e-meeting, the following agreements and conclusion were made regarding mechanisms to support group scheduling for RRC_CONNECTED UEs [2].

Agreement:

For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, Alt 2 is supported:

- Alt 2: support a Type-x CSS

  - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS.

- FFS: Whether the Type-x CSS is a Type-3 CSS

Agreement:

For PTP retransmission of SPS group-common PDSCH, CS-RNTI is used for CRC scrambling of PDCCH with the NDI bit set to 1.

Agreement:

As a baseline, reuse existing fields in DCI format 1_0 with CRC scrambled by C-RNTI for the fields of first DCI format with CRC scrambled with G-RNTI.

- FFS: how to determine the bitlength of FDRA field.

- FFS: Whether ‘Identifier for DCI formats’, ‘TPC command for scheduled PUCCH’ are needed.

- FFS: How to perform DCI size alignment

- FFS: Whether to include new DCI fields

- Note: All of the fields may not be reused and the size of the fields may not be the same

Working assumption:

Option 2B for CFR associated with UE active BWP other than initial BWP is supported at least for multicast of RRC-CONNECTED UEs.

- FFS: CFR associated with initial BWP

- FFS: CFR larger than initial BWP

Agreement:

For multicast of RRC_CONNECTED UEs, further study

- How the LBRM (Limited buffer rate-matching) for GC-PDSCH TBS is determined.

- how the xOverhead for GC-PDSCH TBS determination is configured.

- whether MAC-CE over GC-PDSCH is needed for activation/deactivation of semi-persistent ZP CSI-RS resource set if the semi-persistent ZP CSI-RS resource set is configured in PDSCH-Config in CFR.

Agreement:

Confirm the working assumption:

Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

- FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

Agreement:

For Rel-17 MBS UE, the UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.

- Note:   Group-common PDSCH(s) are counted as unicast PDSCH(s).

Agreement:

For reliability of the group-common PDCCH activation of SPS group-common PDSCH, support at least one of the following alternatives.

- Alt 1: retransmit the activation command via group-common PDCCH.

- Alt 2: retransmit the activation command via UE-specific PDCCH.

- Alt 3: retransmit the activation command via MAC-CE.

- FFS other details.

- Note: Down-selection can take into account the HARQ-ACK feedback scheme for SPS activation

Working assumption:

The maximum number of CORESETs per BWP is not increased for support of MBS, and the number of CORESETs configured within the CFR is left to gNB implementation.

Agreement:

As a baseline, reuse existing fields in DCI format 1_1 for the fields of the second DCI format with CRC scrambled with G-RNTI.

- FFS: whether ‘Identifier for DCI formats’, ‘TPC command for scheduled PUCCH’, ‘Carrier indicator’ and ‘Bandwidth part indicator’ are needed.

- FFS: How to perform DCI size alignment

- FFS: Whether to include new DCI fields for the second DCI format

- Note: All of the fields may not be reused and the size of the fields may not be the same

Agreement:

For HARQ process management, further study whether/how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

In this contribution, we will make discussions on mechanisms to support group scheduling for Broadcast/Multicast service for RRC_CONNECTED UEs.

- Discussion

Common frequency resource for group-common PDSCH

In the last e-meeting, it was agreed that Option 2B for CFR associated with UE active BWP other than initial BWP is supported at least for multicast of RRC-CONNECTED UEs.

- Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

Agreement: (in RAN1 #104e) - If Option 2B is supported for common frequency resource for multicast of RRC-CONNECTED UEs, the starting PRB and the length of PRBs of the MBS frequency region within a dedicated unicast BWP are configured via UE-specific RRC signaling. - The starting PRB is referenced to one of the two options: - Option 1: Point A - Option 2: the starting PRB of the dedicated unicast BWP - FFS the detailed signaling - If Option 2A is supported for common frequency resource for multicast of RRC-CONNECTED UEs, the configurations of the starting PRB and the length of PRBs of the MBS frequency resource reuse the legacy BWP configuration. Agreement: (in RAN1 #104e) From RAN1 perspective, the CFR (common frequency resource) for multicast of RRC-CONNECTED UEs, which is confined within the frequency resource of a dedicated unicast BWP and using the same numerology (SCS and CP), includes the following configurations: - Starting PRB and the number of PRBs - One PDSCH-config for MBS (i.e., separate from the PDSCH-Config of the dedicated unicast BWP) - One PDCCH-config for MBS (i.e., separate from the PDCCH-Config of the dedicated unicast BWP) - SPS-config(s) for MBS (i.e., separate from the SPS-Config of the dedicated unicast BWP) - FFS: Other configurations and details including whether signaling of starting PRB and the length of PRBs is needed when CFR is equal to the unicast BWP - FFS: Whether a unified CFR design is also used for broadcast reception for RRC_IDLE/INACTIVE and RRC_CONNECTED - FFS: Whether Coreset(s) for CFR in addition to existing Coresets in UE dedicated BWP is needed - Note: The terminology of CFR is only aiming for RAN1 discussion, and the detailed signaling design is up to RAN2 - Note: This agreement does not negate any previous agreements made on CFR

One remaining issue for option 2B is the reference point for the starting PRB of CFR. It depends on how to configure CFR. For example, if CFR is configured per BWP per UE, then it would be better to use the starting PRB of the dedicated unicast BWP as the reference point. If CFR is configured per UE rather than per BWP. It is needed to be referred to point A considering that two BWPs with different starting PRBs can contain the same CFR at the same time. From the configuration perspective, we think the former option is better.

Proposal 1: For CFR for multicast of RRC-CONNECTED UEs, the starting PRB of the CFR within a dedicated unicast BWP configured via UE-specific RRC signaling is referenced to the starting PRB of the dedicated unicast BWP.

Regarding whether multicast can be supported or not in a dedicated unicast BWP when no CFR is configured for that BWP. We think the CFR has to be configured. Otherwise, UE can’t identify whether it should receive multicast in this dedicated unicast BWP. However, it doesn’t mean all parameters included in the CFR has to be configured. For some parameters, it can default to reuse the same configuration as that of dedicated unicast BWP. For example, when the frequency resource of CFR is equal to the unicast BWP, for the signaling of starting PRB and the length of PRBs, it can be optional. If the configuration of starting PRB and the length of PRBs is not provided, the configuration of starting PRB and length of PRBs of dedicated unicast BWP is reused. For the other configurations, such as PDCCH/PDSCH, can also be reused. For the configuration of SPS, it may depend on how to distinguish SPS PDSCH for unicast and SPS PDSCH for multicast as we discussed in the following section 2.4.

It was agreed in the last meeting that for multicast of RRC_CONNECTED UEs, further study

- How the LBRM (Limited buffer rate-matching) for GC-PDSCH TBS is determined.

- how the xOverhead for GC-PDSCH TBS determination is configured.

- whether MAC-CE over GC-PDSCH is needed for activation/deactivation of semi-persistent ZP CSI-RS resource set if the semi-persistent ZP CSI-RS resource set is configured in PDSCH-Config in CFR.

All of these are relative with PDSCH TBS determination, it should be kept the same among a group of UEs to guarantee the TBS is the same. For LBRM for GC-PDSCH TBS, it should be calculated based on the bandwidth of CFR rather than dedicated unicast BWP. For xOverhead, if it is configured in PDSCH-config in CFR, xOverhead configured in PDSCH-config in CFR should be used. Otherwise, xOverhead in PDSCH-config in dedicated unicast BWP is used. For semi-persistent ZP CSI-RS, if it is configured in PDSCH-config in CFR, it can be up to gNB to guarantee the semi-persistent ZP CSI-RS is activated/deactivated with MAC-CE over unicast PDSCH at the same time. For MAC-CE over GC-PDSCH, it may not be reliable enough and will casue some UE miss detects the activation/deactivation command.

Proposal 2: For multicast of RRC_CONNECTED UEs,

- LBRM (Limited buffer rate-matching) for GC-PDSCH TBS is determined based on bandwidth of CFR.

- xOverhead in PDSCH-config in CFR is used for GC-PDSCH TBS determination if it is configured.

- It is up to gNB to guarantee the activation/deactivation of semi-persistent ZP CSI-RS resource set at the same time if the semi-persistent ZP CSI-RS resource set is configured in PDSCH-Config in CFR.

It was agreed that one CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs. Regarding the FFS whether to support more than one CFR per UE / per dedicated unicast BWP subjected to UE capabilities, we think it is needed to support considering a scenario illustrated in the following Figure 1, in which UE1 and UE2 receive MBS service 1 in group 1 and UE2 and UE3 receive MBS service 2 in another group 2. In this scenario, UE2 may need to be configured with 2 CFRs in a BWP, otherwise, gNB has to configure a wide CFR for MBS service 1 and MBS service 2 for UE2. Of course, this wide CFR is also configured for UE1 and UE3. Thus, it will significantly increase the power consumption of UE1 and UE3. From terminal vendor’s perspective, this is not acceptable.

Figure 1 Example for CFR for different MBS services

Based on the above analysis, the following proposals are made.

Observation 1: When considering whether to support more than one CFR per UE / per dedicated unicast BWP subjected to UE capabilities, the issue of power consumption should be considered.

Proposal 3: More than one CFR is supported based on UE capability per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.

Support on simultaneous operation with unicast reception

According to the WID, one objective is to specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service and this objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception. The following agreements were made in previous meetings for FDMed PDSCHs.

Agreements: [3] For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. - FFS: TDM or SDM in a slot. Agreements: [4] Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

Regarding the following cases:

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

•	Case 5: support FDM among multiple group-common PDSCHs in a slot

- FFS: maximum number of PDSCHs in a slot simultaneous received per UE

Considering that FDM between unicast PDSCH and group-common PDSCH is supported and multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot is also supported, case 4 can also be supported.

For case 5, since FDM between unicast PDSCH and group-common PDSCH is supported and there is no difference between reception of unicast PDSCH and reception of multicast PDSCH, it can also be supported. In addition, it would be beneficial to support this case for a UE which is interested in multiple MBS services. Regarding the number of FDMed group-common PDSCHs in a slot can be supported by a UE, it can be discussed further, for example, two can be considered as a starting point.

Proposal 4: For simultaneous reception of unicast PDSCH and group-common PDSCH in a slot for RRC_CONNECTED UEs, support the following cases.

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 5: support FDM among multiple group-common PDSCHs in a slot

Group scheduling mechanism for MBS

PTM transmission scheme 1 vs PTM transmission scheme 2

In the previous e-meetings, it was agreed that for RRC_CONNECTED UEs, at least support PTM transmission scheme 1 for both intial transmission and retransmssion. In our view, using a UE-specific PDCCH to schedule a group-common PDSCH, i.e., PTM transmission scheme 2 should also be considered.

- For PTM transmission scheme 1 where group-common PDSCH is scheduled by a group-common PDCCH. A UE can monitor the DCI format scrambled with g-RNTI in a common search space.

  - This mechanism can benefit from PDCCH overhead consumption,

  - However, it is not flexible for the gNB to have UE-specific indication with group-common DCI. For example, HARQ-ACK feedback timing indication (i.e., k1 indication), PUCCH resource indication, TPC, etc.

    - For k1, if it is indicated in group-common PDCCH, all UEs in an MBS group will feed back HARQ-ACK in the same PUCCH slot, the HARQ-ACK payload may be very large within one PUCCH slot, which would result in PUCCH resource overload and collision.

    - For PUCCH resource, it may be required to configure different resources for UEs in an MBS group to gurantee the indicated PUCCH resources for these UEs are orthogonal.

    - For PUCCH transmision power control, it is not possible to have close loop power control for ACK/NACK feedback for different UEs.

  - DCI size alignment for group-common PDCCH for MBS may impact other DCIs

    - So far, DCI size budget only supports 3+1. By using group-common PDCCH for MBS, other group-common DCI need to align the DCI payload size with it, which may reduce the performance of other group-common DCI.

  - Signficant imapcts when considering simultaneous receptions of group-common PDSCH and unicast PDSCH

- For PTM transmission scheme 2 where group-commmon PDSCH is scheduled by a UE-specific PDCCH,  DCI 1_1/1_2 can be reused and the indication of PDSCH scrambling initialization. i.e., C-RNTI or g-RNTI is needed.

  - This mechanism can benefit from minimum impact on HARQ-ACK feedback.

    - gNB can indicate different HARQ-ACK feedback timings for different UEs in an MBS group to offload HARQ-ACK payload within a certain slot.

    - The PUCCH resources for HARQ-ACK for group-cmmon PDSCH can be shared with that for HARQ-ACK for unicast PDSCH. gNB can indicate different PUCCH resources and different TPC for different UEs.

    - HARQ-ACK priority for mulitcast can be lower than, higher than or equal to the priority of unicast. However, the relative priority between multicast and unicast may be different for different in the same MBS group. For example, for UE1, multicast priority is lower than unicast priority, but for UE2, multicast priority is higher than unicast priority. Then, it would be unable to correctly indicate the priority in a group-common DCI.

    - In general, it is same as that for unicast PDSCH, gNB can make it all under flexible control.

  - No additional imapct when when considering simultaneous receptions of group-common PDSCH and unicast PDSCH

The comparisons of group-common PDCCH and UE-specific PDCCH are summarized in following Table 1.

Table 1 Comparisons of PTM transmission scheme 1 and PTM transmission scheme 2

|  |  | PTM transmission scheme 1 | PTM transmission scheme 2 |
| --- | --- | --- | --- |
| Common frequency resource |  | Need to down-select for option 2A/2B and further details needed. Refer to issue#1 in [3]. | Reuse the legacy mechanism, no additional issue. No need to define CFR, the scheduling DCI used for scheduling MBS PDSCH can reuse the same DCI size and FDRA/TDRA table for unicast. |
| Search space set |  | New CSS type is needed to be introduced, or modification for existing CSS type is needed. Refer to issue #3 [3]. | Reuse the legacy mechanism, no additional issue |
| Impact on HARQ-ACK feedback | PUCCH resource | Need to address whether separate HARQ-ACK resource configuration from unicast resources is needed and how to configure and indicate these resources. Refer to Proposal 2.2.1.1 in [4]. | Reuse the legacy mechanism, no additional issue |
|  | HARQ-ACK feedback timing | Same timing, all UEs in an MBS group will feed back HARQ-ACK in the same slot, resulting PUCCH overload and collision | Reuse the legacy mechanism, no additional issue |
|  | HARQ-ACK priority indication | Same indication. Unable to accommodate different prioritity services of different UEs | HARQ-ACK priority of the group-common can be indicated per UE. |
|  | TPC | Difficult to indicate different UEs’ TPC using one single DCI | Reuse the legacy mechanism, no additional issue |
| Support of simultaneous receptions of MBS and unicast Or support of multiple MBS service | DAI | Separating DAI counting for group-common PDSCH and unicast PDSCH or separating DAI counting for different MBS services New mechanism is needed. Refer to Proposal 2.3.2.1 in [4]. | Reuse the legacy mechanism, no additional issue |
|  | CORESET | If a UE supports multiple MBS services which potentially needs multiple common frequency resources, there would be no enough CORESETs considering the limitations of max of 3 CORESET per serving cell. Refer to issue #3 proposal 3-1 in [3]. | Reuse the legacy mechanism, no additional issue |
|  | DCI size budget | Since the FDRA is dependent on the CFR of each MBS service, DCI size for different for different MBS services may be different which will bring additional problem on DCI size budget of “3+1” Refer to issue #3 proposal 3-3 in [3]. | Reuse the legacy mechanism, no additional issue |
|  | BD/CCE | Blind decoding may be increased. Refer to issue #3 proposal 3-2 in [3]. | Reuse the legacy mechanism, no additional issue |
|  | Overbooking | Priority for SS set for MBS needs to be discussed for overbooking | Reuse the legacy mechanism, no additional issue |

Considering the significant pros of UE-specific PDCCH, the following is proposed.

Proposal 5: For RRC_CONNECTED UEs, support PTM transmission scheme 2 for multicast.

Retransmission of MBS service

In the previous meetings, it was agreed that for initial transmission with PTM scheme 1, support PTM transmission scheme 1 (i.e., named g-RNTI based retransmission scheme) and PTP (i.e., named C-RNTI based retransmision scheme) for retransmission(s). The former scheme can get benefits from resource utilization perspective. For the latter scheme, using unicast PDCCH scheduling a unicast PDSCH to send the MBS TB to each UE independently for the MBS retransmission would improve the transmission reliability of the MBS service due to UE specific beamforming and MCS selection and save the power of the other UEs which have already received an MBS TB successfully. One related issue is whether it is needed for a UE to support both of these two retransmission schemes simultaneously or a UE can be configured with only one of these two retransmission schemes. If a UE supports both retransmission schemes simultaneously, it would be possible that the gNB dynamically selects to use PTP or PTM shceme 1 for the retransmission of a particular MBS HARQ by implementation. This issue will have impact on how to determine a downlik alignment is for a new TB or for retransmision:

- If a UE supports only PTM transmission scheme 1 for the retranmission of group-common PDSCH with PTM scheme 1. Then when the UE detects a downlink alignment with g-RNTI, the UE determines the downlink alignment schedules a new TB or retansmission of a TB by comparing the NDI with the previous downlink alignment with g-RNTI. When UE detects a downlink alignment with other RNTI for unicast transmission, such as C-RNTI/TC-RNTI, etc, the UE determines the downlink alignment schedules a new TB or retansmission of a TB by comparing the NDI with the previous downlink alignment with the other RNTI(s) for unicast transmission in the same way as NR Rel-15/16. And there is no interaction between group-common downlink alignment and unicast downlink alignment.

- If a UE supports only PTP for the retranmission of group-common PDSCH with PTM scheme 1. Then when the UE detects a downlink alignment with g-RNTI, the UE determines the downlink alignment schedules a new TB. When the UE detects a downlink alignment with other RNTI for unicast transmission, such as C-RNTI/TC-RNTI, etc, the UE determines the downlink alignment schedules a new TB or retansmission of a TB by comparing the NDI with the previous downlink alignment, including the dowlink alignment with g-RNTI and other RNTI(s) for unicast transmission. A group-common downlink would schedule a new TB without considering its NDI value (this field may be absent).

- If a UE supports both PTM transmission scheme 1 and PTP for the retranmission of group-common PDSCH with PTM scheme 1. Then when the UE detects a downlink alignment with g-RNTI, the UE determines the downlink alignment schedules a new TB or retansmission of a TB by comparing the NDI with the previous downlink alignment with g-RNTI. When the UE detects a downlink alignment with other RNTI for unicast transmission, such as C-RNTI/TC-RNTI, etc, the UE determines the downlink alignment schedules a new TB or retansmission of a TB by comparing the NDI, including the dowlink alignment with g-RNTI and other RNTI(s) for unicast transmission.

To evaluate the performance of different retransmission schemes, i.e. with C-RNTI, g-RNTI, or C-RNTI/g-RNTI dynamically selected by gNB implementation, system-level simulation was conducted. The evaluation results in terms of spectral efficiency (SE) and resource utilization (RU) are shown in Table 2. The video traffic with 720p and 30fps is evaluated. UEs belong to same beam generate a UE group. For retransmission scheme of C-RNTI/g-RNTI dynamically selected by gNB implementation, g-RNTI is selected when the UEs number needing retransmission belong to same beam is more than one, otherwise the C-RNTI is used. Other detailed simulation assumptions are provided in Appendix.

Note that, in our analysis and simulation, for a particular retransmission, either PTP or PTM scheme 1 is used. Meanwhile, for the first retransmission and the second retransmission, different retransmission schemes can be used. From a UE perspective, for a particular retransmission, either PTM scheme 1 or PTP is used, so that UE does not need to receive both unicast PDSCH and group-common PDSCH for the same TB after it transmits NACK for the previous transmission of this TB.

Table 2 Evaluation results for MBS HARQ retransmission schemes

| HARQ re-transmission scheme | g-RNTI only | C-RNTI only | C-RNTI/g-RNTI |
| --- | --- | --- | --- |
| Cell average spectral efficiency (bps/Hz/TRP) | 1.6420 | 1.6372 | 1.6427 |
| Resource utilization | 0.5535 | 0.5248 | 0.5190 |

Based on the results shown above, the followings can be observed.

- Firstly, the C-RNTI only scheme and C-RNTI/g-RNTI scheme both outperform than the g-RNTI only scheme in term of RU with 5.18% and 6.23% gains respectively. This result is reasonable, since for both C-RNTI involved schemes, the higher MCS level in retransmission can be used based on UE-specific CSI feedback. Secondly, the retransmission scheme with C-RNTI/g-RNTI dynamically selected has about 1.11% gains in term of RU than the C-RNTI only scheme. The slight gains of C-RNTI/g-RNTI scheme can be attributed to the group-common retransmission when there are multiple UEs belong to same beam need retransmission, which spends only one slot for retransmission while multi slots are required in C-RNTI only scheme.

- Furthermore, for the cell spectral efficiency, the performance of the three HARQ retransmission schemes is similar. The slight difference in SE of the three schemes mainly results from the different MCS levels of initial transmission, as well as the different packet loss probability due to the different interference condition in each retransmission scheme.

Consequently, the following observations and proposal are conducted.

Observation 2: The retransmission scheme with dynamically selected C-RNTI/g-RNTI brings about 6.23% and 1.11% gain in term of RU compared to the g-RNTI only and C-RNTI only retransmission scheme respectively.

Observation 3: For the cell spectral efficiency, the performances of the three kinds of MBS HARQ retransmission schemes are similar.

Proposal 6: For the retransmission of group-common PDSCH for MBS service, the retransmission scheme(s) is configured:

- Only PTM scheme 1 is supported, or

- Only PTP is supported, or

- Both PTM scheme 1 and PTP are supported

Whether to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast

In the last meeting, one issue about PTP retransmission for PTM initial transmission was discussed. The issue is shown in the following Figure 2. For the same HARQ process, different UEs in an MBS group have different PTP scheduling, thus the NDI states are different for UE-a and UE-b. When PTM scheme 1 is used for multicast initial transmission, its NDI toggle should be relative to the NDI in a previous DCI with the same g-RNTI. Then PTP is used for the retransmission of PTM1 with the same NDI value as initial transmission. However, for UE-b, the initial PTM1 transmission was miss detected. When UE-b detects a unicast DCI with NDI=1, it may make incorrect soft-combination with previous unicast transmission.

This issue can be avoided to some extent by gNB’s scheduling. For example, when gNB knows the miss-detection of UE-b, gNB can set NDI=0 for the PTP retransmission for UE-b. Another option to solve this issue is to indicate whether a PTP transmission is for PTM retransmission or not in the scheduling DCI. However, it will increase the DCI size.

Figure 2 Example for issue of PTP retransmission

Proposal 7: For HARQ process management, there is no need differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

SPS Group-common PDSCH for MBS

In the previous meeting, it was agreed that for RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability. Then some remaining issues are identified as followed.

- How to allocate a SPS PDSCH between MBS and unicast

In NR Rel-16, UE can be configured with up to 8 SPS PDSCHs. The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS. Then it is up to gNB to allocate the number of SPS PDSCH for unicast and MBS. Since SPS unicast PDSCH and SPS group-common PDSCH are scrambled with different RNTIs. It is needed to indicate an SPS PDSCH is unicast SPS or group-common SPS. Then two alternaitives can be considered:

- Alt 1: an SPS PDSCH is unicast or group-common is configured by RRC

- Alt 2: an SPS PDSCH is unicast or group-common is indicated by corresponding activation DCI

For Alt 1, when an SPS PDSCH is configured, one parameter can be introduced to indcate it is group-common PDSCH. Considering that it was agreed to include SPS-config in a CFR configuration, it is straightforward that the SPS-config configured in a CFR is a group-common SPS, otherwise, it is unicast SPS. This implies the configurtation of CFR is a prerequisite of the configuration of SPS group-common PDSCHs. Or as above proposed, a group-common RNTI can be configured in each SPS-config for MBS, then, if there is a RNTI configured in an SPS-config, the SPS-config is for a group-common SPS, otherwise, it is unicast SPS.

For Alt 2, for each SPS PDSCH, it can be dynamically determined it is for MBS or unicast based on pracitical service reqiurement. However, more complexity is found considering indication methods may be needed to be dicussed and may need to increase DCI size. In addition, an SPS PDSCH is configured for a certain service usually. There is no clear motivation to support dynamic indication betwwen MBS and unicast.

Proposal 8: For an SPS PDSCH configuration, it is indicated as a group-common SPS by RRC configuration.

- Using different RNTIs or the same RNTI for differet SPS group-common PDSCHs

One issue needs to be discussed is that when a UE is configured with multiple SPS group-common PDSCHs, for these different SPS group-common PDSCHs, only one RNTI (i.e.,CS-RNTI) is used for these PDSCHs scrambling or different RNTIs can be used. Considering that different SPS group-common PDSCHs may be configured for the same or different MBS services, and each MBS service is assocated with a RNTI, it shoud be supported that the multiple SPS group-common PDSCHs can be scrambled with different RNTIs. For example, the group-common RNTI is configured per SPS group-common PDSCH.

Proposal 9: When a UE is configured with multiple SPS group-common PDSCHs, it should be supported to configure group-common RNTI for each SPS group-common PDSCH.

- Joint activation/deacvation of SPS group-common PDSCH

In NR Rel-16, when multiple SPS PDSCHS are configured, joint deactivation is supported. Then for MBS in RRC connected state, considering that it is possible that UE is configured with multiple SPS group-common PDSCHs for one MBS service, joint deactivation can also be considered and same mechanism as that of unicast PDSCH can be reused.

- Using UE-specific PDSCCH for Activation/deactivation of SPS group-common PDSCH

It was agreed that for activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, at least group-common PDCCH is supported. It is necessary to support UE-specific PDCCH for activation/deactivation of SPS group-common PDSCH for MBS. Because we have to consider the case that after a SPS PDSCH transmission has been activated, then a new UE is interested in the MBS service or a UE does not want to receive this MBS data, the activation/deactivation PDCCH should not be group-common in that case, it should be UE-specific.

Proposal 10: For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, UE-specific PDCCH is supported

- Reliability of the group-common PDCCH activation of SPS group-common PDSCH

In the last meeting, the issue of reliability of the group-common PDCCH activation of SPS group-common PDSCH was discussed, and it was agreed to support at least one of the following alternatives.

- Alt 1: retransmit the activation command via group-common PDCCH.

- Alt 2: retransmit the activation command via UE-specific PDCCH.

- Alt 3: retransmit the activation command via MAC-CE.

- FFS other details.

- Note: Down-selection can take into account the HARQ-ACK feedback scheme for SPS activation

When ACK/NACK is supported for the activation of SPS group-common PDSCH, gNB can know whether a UE detects an activation DCI correctly in some cases. When gNB can resend the activation DCI before the PDSCH transmission occasion in the next period. For Alt 1 and Alt 2, it is up to gNB to guarantee that the PDSCH are scheduled in the same slot as that of the initial activation, considering that HARQ process is determined based on the slot index of PDSCH transmission occasion. Note that, cross-slot scheduling is supported, thus it is flexible enough for gNB to resend the activation PDCCH in different slots and schedule PDSCH in the same slot. If UE-specific PDCCH activation/deactivation is supported, both Alt 1 and Alt 2 are transparent from UE perspective, since UE does not need to know the activation DCI is for retransmission or is initial transmission. Thus, there is no any spec impact for these two alternatives. Comparing with Alt 1, UE-specific PDCCH will be more reliable considering UE-specific beam forming can be used for Alt 2. For Alt 3, gNB can resend the activation command at any time before the PDSCH transmission occasion in the next period. However, it will have large impact on MAC CE design, which is RAN2’s work.

Proposal 11: For reliability of the group-common PDCCH activation of SPS group-common PDSCH, support Alt 1and Alt 2.

- Alt 1: retransmit the activation command via group-common PDCCH.

- Alt 2: retransmit the activation command via UE-specific PDCCH.

CORESET/search space for MBS

In the previous meetings, some agreements were made regarding COSERT and search space set for multicast in RRC_CONNECTED state. It was agreed that for PTM transmission scheme 1, the CORESET for group-common PDCCH is configured within the CFR for group-common PDSCH. In NR Rel-15/16, the number of CORESET(s) within each BWP is limited as 3. For the number of CORESET(s) for group-common PDCCH within the CFR for group-common PDSCH, it may depend on the discussion of how to define/configure the common frequency resource, i.e., if the current BWP framework is reused, it can follow the current limitation for number of CORESETs configuration per BWP, i.e. 3; Otherwise, if the CFR is defined as an MBS frequency region, the number of CORESET(s) for group-common PDCCH within the CFR should be counted in the number of CORESET configured in its associated dedicated unicast BWP. Then if the limitation of 3 CORESETs within each BWP can’t provide enough flexibility for gNB’s scheduling, it can be considered to support more than 3 CORESETs within a BWP which is configured with an MBS frequency region.

In addition, it was agreed that if a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

- Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

Note that whether CORESET can be shared between unicast and multicast is not only dependent on the CORESET frequency range, there are some parameters configured in ControlResourceSet, such as tci-StatesPDCCH, pdcch-DMRS-ScramblingID, etc. For tci-StatesPDCCH, it may be different for PDCCHs for MBS and unicast. For pdcch-DMRS-ScramblingID, it is UE-specific if it is configured for unicast, thus the CORESET configured in PDCCH-config for unicast can only be used for MBS when pdcch-DMRS-ScramblingID is not configured. However, it can be up to gNB’s implementation to configure these parameters and the mapping between search space to CORESET. If these parameters are common among a group of UE, the CORESET configured for unicast can be used for MBS. In addition, from scheduling perspective, among these four options, Option 1 can provide most flexibility.

Proposal 12: If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, option 1 is supported.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, it was agreed in the last meeting that Alt 2 is supported.

- Alt 2: support a Type-x CSS

  - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS.

- FFS: Whether the Type-x CSS is a Type-3 CSS

Considering PDCCH overbooking issue, the monitoring priority of legacy type-3 CSS is always has higher than USS. Thus, it is better to define the type-x search space set for multicast is a new type CSS other than type-3. If it is type-3 CSS, it has to make distinguish between legacy type-3 CSS with legacy monitoring priority and type-3 CSS with new monitoring priority. For example,

- Option 1: introducing a flag for each CSS set to indicate whether the CSS set has new monitoring priority

- Option 2: defining that a CSS set only associated with g-RNTI has new monitoring priority

- Option 3: a CSS set configured in PDCCH-config in CRF has new monitoring priority.

No matter it is type-3 CSS with new monitoring priority or it is type-x CSS, there is no much different in essence.

One remaining issue is whether other group-common DCI formats, such as DCI format 2_x can be configured to be monitored in the type-x CSS. If some other group-common DCI formats other than MBS DCI formats can also be configured in this type-x CSS, the monitoring priority for the other group-common DCI formats in type-x CSS would be different than that in the legacy type-3 CSS. Considering that, it is better to clarify that only DCI formats for MBS group scheduling can be configured in the type-x CSS.

Proposal 13: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, only DCI formats with CRC scrambled with g-RNTI for multicast scheduling can be monitored in the search space.

DCI format for multicast scheduling

In the last meeting, it was agreed that for group-common PDCCH of Rel-17 MBS, support at least two DCI formats. One DCI format is based on DCI format 1_0 and the second DCI format is based on DCI format 1_1.

For the fields of first DCI format and second DCI format with CRC scrambled with g-RNTI, the bitlength of FDRA field should be determined based on the configuration of CFR. ‘Identifier for DCI formats’ is not needed since there is no UL scheduling for MBS. ‘TPC command for scheduled PUCCH’ can be included in the DCI format, e.g, for the case with NACK only feedback, it is useful to do close loop power control for NACK-only PUCCH transmission.

For the fields of second DCI format with CRC scrambled with g-RNTI, ‘Carrier indicator’ and ‘Bandwidth part indicator’ may not be needed, but considering forward compatibility, it can leave to gNB to configure these fields.

Proposal 14: For the fields of first DCI format and second DCI format with CRC scrambled with G-RNTI,

- FDRA field is determined based on the configuration of CFR.

- ‘Identifier for DCI formats’ is not needed.

- ‘TPC command for scheduled PUCCH’ can be included in the DCI format for NACK only feedback.

- ‘Carrier indicator’ and ‘Bandwidth part indicator’ can leave to gNB to configuration.

One open issue is that from a UE perspective, whether these two MBS DCI formats can be configured simultaneously for a UE or not. Considering that a UE can be configured with multiple g-RNTIs for multiple MBS services, it is possible that one MBS service uses the first DCI format for small DCI size and another MBS service uses the second DCI format for more scheduling flexibility. Thus, it is possible that a UE is configured with the two DCI formats for MBS scheduling simultaneously when multiple g-RNTIs are configured. For the case with only one g-RNTI, we don’t see the need to configure two DCI formats for MBS scheduling simultaneously.

Proposal 15: When UE is configured with multiple g-RNTIs, it is supported to configured two DCI formats with CRC scrambled different g-RNTIs. Otherwise, there is no need to configure two DCI formats for a UE simultaneously.

Another issue is how to do DCI size alignment considering the DCI size budget is kept as “3+1”. One option is that the g-RNTI is counted as “C-RNTI”. For the first DCI format, its size can be aligned with the size of DCI 0_0/1_0 in CSS.For the second DCI format, its size should be aligned with DCI format 1_1/0_1 or 1_2/0_2 in USS. In addition, since the size of DCI format 1_1/0_1 or 1_2/0_2 in USS is UE-specific, it should align the size of DCI format 1_1/0_1 or 1_2/0_2 in USS with the second DCI format by zero padding.

Another option is the g-RNTI is counted as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH. Then it would be easier to do DCI size alignment. In that case, there is no need to configure two different MBS DCI formats with the same size for a UE simultaneously.

Proposal 16: For the DCI size alignment, g-RNTI is counted as “C-RNTI”.

- For the first DCI format, its size is aligned with the size of DCI 0_0/1_0 in CSS

- For the second DCI format, the size of DCI format 1_1/0_1 or 1_2/0_2 in USS is aligned with the second DCI format by zero padding.

- Conclusion

In this contribution, we make discussions on the mechanisms to support group scheduling for Multicast and Broadcast Services for RRC_CONNECTED UEs, and have the following observations and proposals.

Observation 1: When considering whether to support more than one CFR per UE / per dedicated unicast BWP subjected to UE capabilities, the issue of power consumption should be considered.

Observation 2: The retransmission scheme with dynamically selected C-RNTI/g-RNTI brings about 6.23% and 1.11% gain in term of RU compared to the g-RNTI only and C-RNTI only retransmission scheme respectively.

Observation 3: For the cell spectral efficiency, the performances of the three kinds of MBS HARQ retransmission schemes are similar.

Proposal 1: For CFR for multicast of RRC-CONNECTED UEs, the starting PRB of the CFR within a dedicated unicast BWP configured via UE-specific RRC signaling is referenced to the starting PRB of the dedicated unicast BWP.

Proposal 2: For multicast of RRC_CONNECTED UEs,

- LBRM (Limited buffer rate-matching) for GC-PDSCH TBS is determined based on bandwidth of CFR.

- xOverhead in PDSCH-config in CFR is used for GC-PDSCH TBS determination if it is configured.

- It is up to gNB to guarantee the activation/deactivation of semi-persistent ZP CSI-RS resource set at the same time if the semi-persistent ZP CSI-RS resource set is configured in PDSCH-Config in CFR.

Proposal 3: More than one CFR is supported based on UE capability per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.

Proposal 4: For simultaneous reception of unicast PDSCH and group-common PDSCH in a slot for RRC_CONNECTED UEs, support the following cases.

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

•	Case 5: support FDM among multiple group-common PDSCHs in a slot

Proposal 5: For RRC_CONNECTED UEs, support PTM transmission scheme 2 for multicast.

Proposal 6: For the retransmission of group-common PDSCH for MBS service, the retransmission scheme(s) is configured:

- Only PTM scheme 1 is supported, or

- Only PTP is supported, or

- Both PTM scheme 1 and PTP are supported

Proposal 7: For HARQ process management, there is no need differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

Proposal 8: For an SPS PDSCH configuration, it is indicated as a group-common SPS by RRC configuration.

Proposal 9: When a UE is configured with multiple SPS group-common PDSCHs, it should be supported to configure group-common RNTI for each SPS group-common PDSCH.

Proposal 10: For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, UE-specific PDCCH is supported

Proposal 11: For reliability of the group-common PDCCH activation of SPS group-common PDSCH, support Alt 1and Alt 2.

- Alt 1: retransmit the activation command via group-common PDCCH.

- Alt 2: retransmit the activation command via UE-specific PDCCH.

Proposal 12: If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, option 1 is supported.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

Proposal 13: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, only DCI formats with CRC scrambled with g-RNTI for multicast scheduling can be monitored in the search space

Proposal 14: For the fields of first DCI format and second DCI format with CRC scrambled with G-RNTI,

  - FDRA field is determined based on the configuration of CFR.

  - ‘Identifier for DCI formats’ is not needed.

  - ‘TPC command for scheduled PUCCH’ can be included in the DCI format for NACK only feedback.

  - ‘Carrier indicator’ and ‘Bandwidth part indicator’ can leave to gNB to configuration.

Proposal 15: When UE is configured with multiple g-RNTIs, it is supported to configured two DCI formats with CRC scrambled different g-RNTIs. Otherwise, there is no need to configure two DCI formats for a UE simultaneously.

Proposal 16: For the DCI size alignment, g-RNTI is counted as “C-RNTI”.

  - For the first DCI format, its size is aligned with the size of DCI 0_0/1_0 in CSS

  - For the second DCI format, the size of DCI format 1_1/0_1 or 1_2/0_2 in USS is aligned with the second DCI format by zero padding.

# References

- RP-193248, ‘New Work Item on NR support of Multicast and Broadcast Services’, Huawei.

- RAN WG1 #105 e-meeting Chairman’s Notes

- RAN WG1 #102 e-meeting Chairman’s Notes

- RAN WG1 #103 e-meeting Chairman’s Notes

- R1-2102171, ‘Summary#10 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS’, Moderator (CMCC).

- R1-2102134, ‘FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs’, Moderator (Huawei)

# Appendix

Table 3  Simulation assumptions for evaluation of MBS service

| Parameters | Value |
| --- | --- |
| Layout | UMa in TR38.901 |
| Inter-BS distance | 500 m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 20 MHz, 8.2% Guard Band |
| Frame structure | DDDDDDDSUU, 10:2:2 for S |
| SCS / Cyclic Prefix | 30 KHz / NCP |
| Channel model | UMa in TR 38.901 |
| BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8) (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| UE antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| BS beam set | For direction of BS analog beam steering (in LCS): Azimuth angle φi = [0] Zenith angle θj = [90 95 100 105] degree NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. |
| Traffic model | Traffic mode: video Traffic frequency: 30fps Traffic Image resolution: 720*1280p |
| Transmit power | 49 dBm at BS |
| Antenna height | assign with TR38.901 |
| Antenna gain | 8 dBi at BS and 0 dBi at UE |
| Noise figure | 9 dB at UE |
| UE distribution | 80% indoor, with 3km/h 20% outdoor, with 3 km/h |
| Number of UEs per cell | 10 |
| Schedule method | Beam round robin and TDM scheduled |
| Schedule granularity | Unicast: subband; Groupcast: wideband |
| PMI CQI feedback granularity | Unicast: subband; Groupcast: wideband |
| Max retransmission time | 4 |
| MCS table | EMBB_CP_OFDM_64QAM |
| Receiver | MMSE-IRC |