3GPP TSG RAN WG1 #106-e 						      						R1-2106625

e-Meeting, August 16th – August 27th, 2021

Source:	vivo

Title:	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Agenda Item:	8.12.3

Document for:  Discussion and Decision

Introduction

According to the WID [1] of the NR MBS, the MBS service of NR is to be provided to both the RRC IDLE/INACTIVE UEs and the RRC CONNECTED UEs. In this contribution, we provide our considerations on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, including MBS common frequency resource, HARQ feedback, and semi persistent scheduling.

Discussion

MBS common frequency resource

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, further study the following cases of a configured/defined specific common frequency resource (CFR) for group-common PDCCH/PDSCH, and identify which case(s) will be supported:

- [Case E] the case where a CFR is defined based on a configured BWP.

  - In particular, study the following:

    - whether a configured BWP for MBS is needed or not.

    - whether BWP switching is needed or not.

  - In this study, the configured BWP has the following properties:

    - The configured BWP is different than the initial BWP where the frequency resources of this initial BWP are configured smaller than the full carrier bandwidth.

    - The CFR has the frequency resources identical to the configured BWP.

    - The configured BWP needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

  - Note: The configured BWP is not larger than the carrier bandwidth

- the case where the initial BWP fully contains the CFR in the frequency domain.

  - In this study the following sub-cases are considered:

    - [Case B] A CFR with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

    - [Case D] A CFR with smaller size than the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with smaller size than the initial BWP are needed or not for MBS.

- the case where the initial BWP has same size as the CFR in the frequency domain.

  - In this study the following two sub-cases are considered:

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with the same size as the initial BWP are needed or not for MBS.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MCCH.

- Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MTCH.

- Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling.

In RAN 1 #104bis e-meeting, five cases are defined for further study. In Case A and Case C, a CFR has the same size as CORESET 0 and the initial BWP configured by SIB1, respectively. We suppose that Case A and Case C are supported by default if a specific CFR is not configured according to the agreements in [2]. Furthermore, in RAN 1 #105 e-meeting, it has been agreed that a CFR with the same size as CORESET0 can be configured/defined to receive GC-PDCCH/PDSCH carrying MCCH and MTCH.

In Case B and D, a CFR has smaller size than CORESET 0 and the initial BWP configured by SIB1, respectively. As RRC IDLE/INACTIVE UE camps on the initial BWP of a cell, the benefit of defining a CFR with smaller size is not clear but causing additional signalling overhead, and less frequency resources for MBS services can be achieved by scheduling. And thus, we believe Case B and Case D are not necessary. If gNB wants to schedule MBS service only in a CFR smaller than initial DL BWP configured by CORESET0 or SIB1, it can be up to gNB’s implementation but no need to configure a smaller CFR additionally.

For Case E, a CFR with a larger size than initial BWP has been defined. Considering multiple MBS services are to be provided for RRC IDLE/INACTIVE UE in a cell, large bandwidth is required for transmitting MTCH. Furthermore, in order to ensure RRC idle/inactive UEs to monitor both MCCH and SI/paging without BWP switching, CORESET0 shall be included in the configured/defined CFR, and thus, we support that one configured/defined CFR fully contains the CORESET0 in frequency domain and has the same SCS and CP as CORESET0. On the other hand, when a certain MBS service has large periodicity, UEs can switch from one configured/defined CFR with large bandwidth to a small one, i.e., CORESET0, between two receptions for power saving purpose. UEs can also switch to CORESET0 after receiving the session stop of interested service, and monitor new session start via MCCH in CORESET0. When new session start is detected, UEs can switch back to the configured/defined CFR. The timing of switching between one configured/defined CFR and CORESET0 is up to UEs’ implementation, so that MBS service can be received timely. An illustrative example for CFR switching is shown in figure 1.

Figure 1: CFR switching

Furthermore, among multiple MBS services, some RRC IDLE/INACTIVE UEs may be interested in only a subset of services while some other UEs are interested in another subset of services, thus, transmitting all MBS services in one CFR for RRC IDLE/INACTIVE UEs is not friendly to power saving purpose. If several CFRs are defined and only a subset of MBS services are transmitted in one CFR, UE can select one unique CFR depending on its interested services. As shown in figure 2, suppose there are four services: service 1-4. When only one CFR is defined, UE has to perform RF tuning to the bandwidth of the CFR, no matter what the interested service is. While multiple CFRs are defined with each fully containing CORESET0 in frequency domain, UE can perform RF tuning to only the bandwidth of certain CFR, and thus, it is beneficial to define multiple CFRs for power saving purpose.

Figure 2: MBS common frequency resource

Proposal 1: For RRC_IDLE/RRC_INACTIVE UEs, support that one configured/defined CFR fully contains the initial BWP in frequency domain and has the same SCS and CP as the initial BWP, where the initial BWP has the same frequency resources as CORESET0, to receive GC-PDCCH/PDSCH carrying MTCH.

Proposal 2: For RRC_IDLE/RRC_INACTIVE UEs, more than one common frequency resource can be defined/configured.

HARQ feedback

Regarding the HARQ feedback for the MBS reception, we consider that the RRC IDLE/INACTIVE UE should not be required to provide the HARQ feedback at least for broadcast reception, as the received UEs are not known by gNB. Furthermore, they may even not be uplink synchronized with the network. And the reliability can be improved by PDSCH repetition if there is a need. Therefore, at least for broadcast reception, HARQ feedback is not supported.

Proposal 3: For RRC_IDLE/RRC_INACTIVE UEs, at least for broadcast reception, HARQ feedback is not supported.

Semi-persistent scheduling

Semi-persistent scheduling (SPS) is beneficial for periodic transmissions. However, in broadcast, the received UEs are not known by gNB and HARQ feedback is not expected to be supported. If UE fails to detect the activation DCI for SPS MBS PDSCH, it will miss all the subsequent transmissions. Therefore, for RRC_IDLE/RRC_INACTIVE UEs, SPS PDSCH with DCI activation/deactivation is not supported at least for broadcast reception. On the other hand, SPS PDSCH without dynamic activation/deactivation which is similar to uplink configured grant type 1 can be considered instead.

Proposal 4: For RRC_IDLE/RRC_INACTIVE UEs, at least for broadcast reception, SPS PDSCH with DCI activation/deactivation is not supported.

- FFS: SPS PDSCH without DCI activation/deactivation.

Conclusions

For RRC IDLE/INACTIVE UEs, this contribution provides our consideration on MBS common frequency resource, HARQ feedback, and semi persistent scheduling, and the following proposals are proposed:

Proposal 1: For RRC_IDLE/RRC_INACTIVE UEs, support that one configured/defined CFR fully contains the initial BWP in frequency domain and has the same SCS and CP as the initial BWP, where the initial BWP has the same frequency resources as CORESET0, to receive GC-PDCCH/PDSCH carrying MTCH.

Proposal 2: For RRC_IDLE/RRC_INACTIVE UEs, more than one common frequency resource can be defined/configured.

Proposal 3: For RRC_IDLE/RRC_INACTIVE UEs, at least for broadcast reception, HARQ feedback is not supported.

Proposal 4: For RRC_IDLE/RRC_INACTIVE UEs, at least for broadcast reception, SPS MBS PDSCH with DCI activation/deactivation is not supported.

FFS: SPS PDSCH without DCI activation/deactivation.

Reference

[1] RP-201038, Huawei, “WID revision: NR Multicast and Broadcast Services”.

[2] RAN WG1 #103 e-meeting Chairman’s Notes