3GPP TSG-RAN WG1 Meeting #106-e              	R1-2106673

e-Meeting, August 16th – 27th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UE Procedures for UCI Multiplexing and Prioritization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The overlapping resolution procedure between HP and LP channels has been discussed during last few meetings and resulted in the following working assumption that is captured in the current specifications. Working assumption (RAN1#102-e) - Multiplexing/overriding/etc. is performed similar to Rel.15 as if HP channels do not exist; this means that LP operations, multiplexing/overriding/etc., are performed before cancellation. - A UE cancels the transmission of a LP channel including any intermediate scheduled LP transmission that does not overlap with any LP channel, if any DCI schedules an overlapping HP transmission with the LP channel, before performing multiplexing/overriding HP channels if any. - Multiplexing/overriding of HP channels is performed as if LP channels do not exist. - A final HP channel is prioritized if it overlaps with a final LP channel, after performing multiplexing of HP channels However, it is observed that the overlapping resolution procedure is ambiguous and unnecessarily complicated. At the time the Working assumption was made, it was not well understood that the requirement in Step 2 of WA to perform cancellation “before” overriding/ multiplexing, is in contradiction with the principle of operation based on fulfilling timeline requirements that limits the implementation flexibility and results in unexpected outcomes. Another issue with the requirement in Step 2 of WA is that it can lead the UE to perform unnecessary cancellation operation without clear motivation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Maintain the cancellation timelines required for intermediate cancellation in Step 2 while not performing the intermediate cancellation in Step 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The overlapping resolution procedure between HP and LP channels is ambiguous and allows to be interpreted differently as per implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 9	UE procedure for reporting control information

< Unchanged parts are omitted >

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of a same priority index as described in clauses 9.2.5 and 9.2.6. Then,

-	if a transmission of a first PUCCH of larger priority index corresponding to a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission

-	if a transmission of a first PUSCH of larger priority index corresponding to a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission

where

-	the UE expects that the transmission of a larger priority index PUCCH or PUSCH scheduled by a DCI in a PDCCH reception that would overlap in time with transmission of a smaller priority index PUSCH/PUCCH or PUCCH, respectively, would not start before Tproc,2 after a last symbol of the corresponding PDCCH before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6

-	any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1

Tproc,2

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability

< Unchanged parts are omitted >