3GPP TSG-RAN WG1#106-e	Tdoc R1-2106682

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.3.4

Source:	Ericsson

Title:	Propagation Delay Compensation Enhancements for Time Synchronization

Document for:	Discussion, Decision

# 1	Introduction

SA2 Rel-17 calls for a new use case for TSN Grandmaster clocks supported within the context of TSN-5GS interworking wherein TSN Grandmaster clocks are located at end stations connected to UE/DS-TTs. This new Rel-17 use case involves two Uu interfaces in the 5GS path (i.e. 5GS ingress to the 5GS egress) over which a TSN Grandmaster clock is relayed. Considering that up to 540ns of uncertainty can be introduced by a single Uu interface when using the legacy Timing Advance method to determine the downlink propagation delay as indicated by [1], supporting a 5GS path that includes two Uu interfaces will be problematic when the maximum allowed uncertainty allowed over the 5GS path is limited to 900ns per Table 5.6.2-1 of [2] (see Appendix also). For the Rel-17 URLLC/IIoT WI, RAN1 is directed to investigate possible enhancements in the following area:

- Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

Possible enhancements to the legacy Timing Advance method as well as the possible use of an enhanced RTT method to determine the total uncertainty introduced per Uu interface were discussed in [7]. In this contribution, we investigate further the accuracy achievable via the TA-based method and the RTT-based method for propagation delay estimation.

# 2	Discussion

An example of the Two Uu interface use case is illustrated in Figure 1 below, wherein two UEs can be connected to different gNBs, thereby introducing the potential for increased uncertainty compared to the case where each UE is connected to the same gNB.

The 5GS synchronicity budget requirement can be as low as 900ns in what is currently the most demanding TSN – 5GS integration use case (see Appendix and [6]) and represents the portion of the end-to-end synchronicity budget applicable between the ingress and egress of the 5G system (see Figure 1). The per Uu interface synchronization error represents a portion of the end-to-end synchronicity budget and consists of the uncertainty introduced when (a) sending the 5G reference time from gNB antenna to the UE antenna by including ReferenceTimeInfo in either a DLInformationTransfer RRC message or SIB9 and then (b) adjusting the 5G reference time to reflect the downlink propagation delay. These uncertainties are further discussed below.

Figure 1: TSN E2E Timing delivery case 2 – ingress at UE

In RAN1#102e, it was agreed that both control-to-control and smart-grid use cases are used as representative cases for study, where:

- One Uu interface is assumed for smart grid.

- Two Uu interfaces are assumed for control-to-control.

A first approach is to pursue a single set of Rel-17 PDC (propagation delay compensation) enhancements that can adopted for supporting both use cases above. Following this approach RAN1 only needs to come up with a single PDC method that satisfies the most stringent Rel-17 use case requirements, where the same method can be applied to support all other use cases as well.

A second approach is to use the legacy TA-based method in support of use cases having a less demanding PDC uncertainty requirement and supplement it with a new method for supporting use cases having the most demanding PDC uncertainty requirements.

The range of uncertainty for a single Uu interface shown in Table 1 below was agreed at 3GPP TSG-RAN WG2 #113-e. Following the second approach above, RAN1 should allow for supplementing the legacy TA-based method by studying the enhancements needed to realize the indicated range of uncertainty for the most demanding control-to-control use case (i.e., 900ns 5GS uncertainty budget with two Uu interfaces), with the sanity check that the same enhancements work for smart grid case as well.

Table 1 – Range of Uncertainty for a Single Uu interface

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |
| Smart Grid | ±795ns to ±845ns |

RAN1 to pursue Rel-17 PDC enhancements that satisfy the most demanding PDC accuracy requirements as a supplement to the legacy TA-based method for determining PDC.

## 2.1	Time Synchronization Error if using the TA-based propagation delay estimation

In general, for both the TA-based method and the RTT-base method, the total uncertainty introduced for a single Uu interface consists of the following sources of error:

- Error Source 1: Uncertainty related to the gNB transmission of the 5G reference time having a value linked to a specific point in downlink frame transmissions. This is reflected by ErrBS,DL,Tx+ ErrUE,DL,Rx.

- Error Source 2: Uncertainty related to the determination of downlink propagation delay (PD) value identified as a result of performing the TA procedure.  This is a major contribution to the total uncertainty when the 5G system clock is adjusted to reflect the downlink PD.

Error Source 1 can be seen as having a value that is independent of propagation delay since the procedure that introduces this error (i.e., sending UE a value for 5G reference time) can be performed at a point in time different from when the PD estimation procedure (i.e., sending UE a TA command) is performed. This results in allowing errorUE, DL,RX to be included in both Error Source 1 and Error Source 2.

When using the TA-based propagation delay method to determine the downlink PD value, the propagation delay is estimated as half of the Timing Advance. The total uncertainties when using TA-based method is described by the equation below [7].

Errorclock_sync=Error Source 1+Error Source 2, where:

Error Source 1= ErrBS,DL,Tx+ ErrUE,DL,Rx
Error Source 2 =ErrUE,DL,rx+ErrUE,UL,tx+ErrBS,UL,rx+ErrTAG+ErrTAAA2
=Te+ErrBS,UL,rx+ErrTAG+ErrTAAA2

In the above, it is assumed that Te=ErrUE,DL,rx+ErrUE,UL,tx, i.e., Te as defined in 38.133 includes downlink frame timing detection error as a component.

At RAN WG1#103e the following agreements were reached:

- For evaluation of the overall time synchronization error for the control-to-control use case companies are to use 65ns as the assumption for the  ErrBS,DL,Tx component (i.e. the BS transmit timing error).

- For evaluation of the overall time synchronization error for smart grid companies can use either 200ns (option 1) or 65ns (option 2) as the assumption for BS transmit timing error.

- Asymmetry between downlink and uplink channels for smart grid scenario is not to be considered.

- TA adjustment accuracy is not to be considered for the evaluation of time synchronization error (see ErrTAAA error component below).

At RAN WG1#104e the following agreements were reached:

- Take ±100 ns as the assumption for downlink frame timing detection error (ErrUE,DL,rxerrorUE, DL, RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately.

The two error sources comprise the following error components:

(a) ErrBS,DL,tx: The uncertainty due to the value of the 5G reference time indicated by the gNB as being applicable to the end of SFNx not reflecting the actual 5G reference time value when the end of SFNx occurs at the gNB Antenna Reference Point (ARP).

- ±65ns (for control-to-control, and for Option 2 of smart grid, per agreement at RAN WG1#103e)

(b) ErrUE,DL,rx:  The uncertainty associated with UE downlink frame timing detection. As a worst case, a UE synchronizes to the DL using Sync Signal Block (SSB) received within the last 160 ms, where SSB contains information identifying specific DL frame and slot numbering. As described in [13] and [14], the minimum DL frame timing detection error (without any margin) is inverse of the DL BW of the signals used for timing estimation. When using SSB as DL signal for timing estimation, the minimum error is calculated in Table 2 below.

Table 2. DL frame timing detection error at UE, based on SSB detection

| #PRB of PBCH (=240 subcarrier) | SCS (kHz) | PBCH BW (MHz) = 240 * SCS | Min Timing Error (sec) = 0.5 / (PBCH BW) |
| --- | --- | --- | --- |
| 20 | 15 | 3.6 | 0.139 us = 4.27 Ts = 273 Tc |
| 20 | 30 | 7.2 | 0.069 us = 2.13 Ts = 137 Tc |
| 20 | 60 | 14.4 | 0.035 us = 1.07 Ts = 69 Tc |
| 20 | 120 | 28.8 | 0.017 us = 0.53 Ts = 34 Tc |

Thus, the following can be assumed for PD estimation:

- ErrUE,DL,rx = ±139ns for 15 kHz DL SCS;

It should be emphasized that the values in Table 2 are only theoretically possible minimum timing detection error, and very optimistic. For example, while the UE performance in AWGN channel may be close to these values, the timing detection error caused by multipath channel is not taken into account, even though the error by multipath likely dominates.  Thus the values in Table 2 provide a reference value for RAN1 estimation, with the understanding that actual values, if they are to be specified, are up to RAN4 discussion.

(c) Te: The error when a UE performs transmission of UL frames after acquiring the first detected path of the corresponding downlink frame and applying the most recently received TA information. This error is denoted as Te in TS 38.133. Among the values tabulated in TS 38.133, the following is selected considering that 15 kHz SCS has been agreed for the selected scenarios.

- ±12*64*Tc = 768*Tc = ±391ns (assuming 15kHz SCS for SSB signals, 15kHz SCS for uplink signals)

(d) ErrBS,UL,rx: The uncertainty with which a gNB acquires UL frame timing, which affects how accurately it determines the difference between when an UL slot has been received and when that slot should have been received if the UE was perfectly time aligned (i.e. this uncertainty affects the value the gNB sends within the MAC TA command). According to agreement from RAN1#102e, the following value is assumed:

- ±100ns  (per agreement at RAN WG1#102e)

(e) ErrTAG (also known as errorTA_indication):  The uncertainty due to timing advance (TA) command granularity. Maximum value of this uncertainty is half of TA command granularity in the existing NR specification where µ represents the applicable SCS:

- ±8*64*Tc/2µ = 512*Tc = ±260ns (µ = 0 for 15 kHz SCS)

- A Timing Delta MAC CE has been introduced [TS 38.213 Section 14, TS 38.321] which serves the purpose of enhancing DL PD estimation accuracy. It is FFS whether the value of this component can be further reduced, similar to the Timing Delta.

(f) ErrTAAA: The uncertainty due to “Timing Advance adjustment accuracy” performed by a UE, see section 7.3.2.2 of TS 38.133. For 15 kHz SCS, the accuracy requirement is:

- ±256 * Tc = 130ns if included in the evaluation, however a value of 0ns is used below since at RAN WG1#103e it was agreed to leave this component out of the evaluation

In the above, time unit Tc=1480∙103∙4096 (sec). Plugging in the value of each error components we obtain:

Errorclock_sync (total uncertainty) = 65 + 139 + 0.5*(391 + 100 + 260 + 0) = ±579.5 (ns)

Using the TA-based propagation delay method to determine the downlink PD value and then adjusting the 5G reference time according to the determined PD introduces over 579ns of uncertainty for a single Uu interface (i.e. when using ErrTAAA = 0ns, Te = 391ns and ErrTAG = 260ns).

- For the smart grid use case the TA-based method is sufficient, due to the larger Uu interface budget of ±795ns to ±845ns (see Table 1).  Note that this is true even if considering the larger, Option 1 value of ErrBS,DL,tx=200ns.

- However, the TA-based method is inadequate for supporting the control-to-control use case, due to the much smaller Uu interface budget of ±145ns to ±275ns as shown in Table 1.

In Table 3, the achievable accuracy for each option of the TA-based method (see agreement from RAN1#102e) is listed. Compared to the most demanding Uu interface error budget range of ±145ns to ±275ns as shown in Table 1 for the control-to-control use case, the requirement cannot be satisfied under any of the options.

None of the TA-based options can satisfy the most demanding PDC accuracy requirement range for the control-to-control use case when agreed values for the error components are used.

Table 3. Clock synchronization achievable by the 3 options of TA-based method

| Option 1: TA-based propagation delay | Clock synchronization achievable |
| --- | --- |
| Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity). | - 579.5 ns if assuming the existing Te and TA indication granularity - 449.3 + 0.5*ErrTAG (ns) if assuming existing Te, and considering enhanced TA indication granularity |
| Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te) | - 254 + 0.5*(Te+ErrTAG) (ns) if considering both enhanced Te and enhanced TA indication granularity - 384.2+ 0.5*Te (ns) if assuming existing TA indication granularity, and considering enhanced Te |
| Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected) | - 449.3 + 0.5*ErrTAG (ns) if assuming existing Te, and considering enhanced TA indication granularity |

Considering the case where the two largest uncertainty components (i.e. Te = 391 ns and ErrTAG = 260 ns) were to be reduced by 50%, i.e., assuming (Te' = 195.3 ns and Err'TAG = 130 ns), the total uncertainty would be 416.8 ns which is still above the 275ns upper limit shown in Table 1 for the control-to-control use case. In addition, reducing these same two components by 75%, i.e., assuming (Te'' = 97.65 ns and Err''TAG = 65ns), results in a total uncertainty of 335 ns which is still above the 275ns upper limit shown in Table 1 for the control-to-control use case.

Reducing Te from 391ns to a substantially smaller value for the TA-based method is expected to require the UE to make use of a specialized downlink Reference Signal for clock synchronization (e.g. PRS)

Reducing ErrTAG from 260ns to a substantially smaller value for the TA-based method is expected to require the gNB to make use of a specialized uplink Reference Signal (e.g. a wideband SRS).

The introduction of specialized reference signals for determining PD values with a substantially reduced uncertainty effectively calls for a new procedure that is distinct from the TA-based propagation delay method for determining PD.

None of the TA-based options can satisfy the high end of the accuracy requirement range for the control-to-control use case even if values for the two largest error components are aggressively reduced from their current values.

The introduction of reference signals needed to determine PD values with acceptable uncertainty within the context of a TA-based method effectively calls for a procedure that is new and distinct from existing TA-based methods.

Based on the analysis and observations above, we propose:

RAN1 does not adopt the current TA-based method for determining propagation delay compensation for the most demanding PDC accuracy requirement since even a 75% reduction of the two largest uncertainty components (i.e. Te = 391 ns and ErrTAG = 260 ns for 15 kHz SCS) does not result in a total uncertainty ±275ns or less for a single Uu interface.

## 2.2	Time Synchronization Error if using the RTT-based propagation delay estimation

The RTT-based method for PDC determination is investigated in this section to estimate the time synchronization error it introduces when used to adjust the value of the 5G system clock. The overall uncertainty consists of the error due to signaling the 5G reference time to the UE (i.e. ErrBS,DL,Tx+ ErrUE,DL,Rx) and the error due to adjusting the value of the 5G reference time with estimated PDC. The component of ErrBS,DL,tx is the same as that of TA-based method. In the following, we focus on the error due to propagation delay compensation via RTT-based method.

For RTT-based method, the timing relationship of transmission and reception is illustrated in Figure 3, at both the gNB side and the UE side.

Figure 3: Timing relationship of DL transmission and UL transmission

With reference to Figure 3, the following relationship exists:

Trx,UE=Ttx,BS+d+ErrUE,DL,rx

Trx,BS=Ttx,UE+d+ErrBS,UL,rx

The propagation delay is estimated as half of TADV, where:

TADV=Trx,BS-Ttx,BS+Trx,UE-Ttx,UE

Expressed another way:

TADV=Trx,BS-Ttx,UE+Trx,UE-Ttx,BS

As described in TS 38.215, the measurement of gNB Rx-Tx time difference provides Trx,BS-Ttx,BS, and the measurement of UE Rx-Tx time difference provides Trx,UE-Ttx,UE.

When using d=TADV2 to estimate propagation delay, the total uncertainties for PD when using RTT-based method is captured in the equation below:

Errorclock_sync=Error Source 1+Error Source 2, where:

Error Source 1= ErrBS,DL,Tx+ ErrUE,DL,Rx
Error Source 2=ErrgNB,RxTxDiff+ErrUE, RxTxDiff+ErrRxTxDiff,report2

(2)

In the equation above, ErrgNB,RxTxDiff and ErrUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively. ErrRxTxDiff,report reflects the error due to the granularity of reporting the Rx-Tx time difference.  Component ErrRxTxDiff,report can be the error due to the report mapping table granularity of the UE Rx-Tx time difference,  or the error due to the report mapping table granularity of gNB Rx-Tx time difference, depending on the direction of reporting.  For example, if the UE reports its measured value of UE Rx-Tx time difference to gNB, then ErrRxTxDiff,report corresponds to the error due to the report mapping table granularity of the UE Rx-Tx time difference. Note that, in contrast to the TADV estimation for positioning, where both gNB and UE report RxTxTimeDiff to the location server, for clock synchronization, reporting is only necessary in one direction. Hence report mapping error needs to be included once only.

Currently, measurement inaccuracy requirements (i.e., ErrgNB,RxTxDiff and ErrUE, RxTxDiff) are not yet available in 38.133. In 38.133 V16.4.0, report mapping tables are provided for gNB Rx-Tx time difference. However the report mapping tables are not yet defined for UE Rx-Tx time difference in 38.133. Thus, estimation has to be taken in the analysis below.

For the control-to-control use case, to evaluate the time synchronization error for the RTT-based method we assume the following uncertainty values:

- ErrBS,DL,Tx: 	±65ns, the same as that of TA-based method.

- ErrUE,DL,rx: 	±116 ns. This is obtained using the method shown in Table 1, but with minimum PRS bandwidth of 24 PRB. It is noted that the PRS bandwidth can be as large as 272 PRBs. In general, the larger the bandwidth of the DL reference signal used for timing detection, the smaller the DL timing detection error.

- ErrgNB,RxTxDiff and ErrUE, RxTxDiff: Due to lack of accuracy requirements in 38.133, the maximum value of ±90*Tc as proposed in [11] is taken for the estimation below. That is, assume  ErrgNB,RxTxDiff = ErrUE, RxTxDiff=±90*Tc = ±45.8 ns [11].

- ErrRxTxDiff,report:  In 38.133, report mapping tables are provided for gNB Rx-Tx time difference where the granularity ranges from 1*Tc to 32*Tc (where Tc = 0.509ns). Assuming the largest granularity of 32*Tc, the uncertainty due to the granularity of reporting the Rx-Tx time difference = 32*Tc = 16 ns.

Using the values above, the total clock synchronization error for the 5G reference time is estimated as:

Errorclock_sync=65+116+45.8*2+162=234.8 (ns)

Compared to the most demanding per Uu interface error budget requirement of 145ns to 275ns (see Table 1) for the control-to-control use case, the RTT-based method can satisfy the requirements according to the analysis above. Furthermore, the total error margin associated with the RTT-based method can be further improved by taking into account the possible use of better reference signals (RS) on the downlink and/or uplink.

RTT-based propagation delay estimation can satisfy the most demanding per Uu interface uncertainty budget of ±145ns to ±275ns for control-to-control use case.

In the analysis above, the values for ErrgNB,RxTxDiff, ErrUE, RxTxDiff, ErrRxTxDiff,report come from RAN4 study or specification for positioning. Thus, specialized downlink signals (e.g., PRS) and uplink reference signals (e.g., SRS) are assumed for achieving such accuracy requirements. The need of specialized reference signals means that a set of signals and related procedures should be introduced for clock synchronization on the Uu interface.

The need of reference signals for determining propagation delay with acceptable uncertainty for the RTT-based method calls for the introduction of a set of signals and procedures for clock synchronization.

RAN1 adopts an RTT-based procedure for determining propagation delay compensation in support of the most demanding PDC accuracy requirements in Rel-17.

# Conclusion

In the previous sections we discussed the problem and potential solutions related to clock synchronization. The following observations are made:

Observation 1	None of the TA-based options can satisfy the accuracy requirement range for the control-to-control use case when agreed values for the error components are used.

Observation 2	Reducing Te from 391ns to a substantially smaller value for the TA-based method is expected to require the UE to make use of a specialized downlink Reference Signal for clock synchronization (e.g. PRS)

Observation 3	Reducing ErrTAG from 260ns to a substantially smaller value for the TA-based method is expected to require the gNB to make use of a specialized uplink Reference Signal (e.g. a wideband SRS).

Observation 4	The introduction of specialized reference signals for determining PD values with a substantially reduced uncertainty effectively calls for a new procedure that is distinct from the TA-based propagation delay method for determining PD.

Observation 5	None of the TA-based options can satisfy the high end of the accuracy requirement range for the control-to-control use case even if values for the two largest error components are aggressively reduced from their current values.

Observation 6	The introduction of reference signals needed to determine PD values with acceptable uncertainty within the context of a TA-based method effectively calls for a procedure that is new and distinct from existing TA-based methods.

Observation 7	RTT-based propagation delay estimation can satisfy the tighter Uu interface budget of ±145ns to ±275ns for control-to-control use case.

Observation 8	The need of reference signals for determining propagation delay with acceptable uncertainty for the RTT-based method calls for the introduction of a set of signals and procedures for clock synchronization.

And the following proposals are made:

Proposal 1	RAN1 to pursue Rel-17 PDC enhancements that satisfy the most demanding PDC accuracy requirements as a supplement to the legacy TA-based method for determining PDC.

Proposal 2	RAN1 does not adopt the current TA-based method for determining propagation delay compensation for the most demanding PDC accuracy requirement since even a 75% reduction of the two largest uncertainty components (i.e. Te = 391 ns and ErrTAG = 260 ns for 15 kHz SCS) does not result in a total uncertainty ±275ns or less for a single Uu interface.

Proposal 3	RAN1 adopts an RTT-based procedure for determining propagation delay compensation in support of the most demanding PDC accuracy requirements in Rel-17.

# References

R2-2010837, Reply LS on propagation delay compensation enhancements, 3GPP TSG-RAN WG2 Meeting #112 Electronic.

R1-1901470, Reply LS on TSN requirements evaluation, RAN1, 3GPP TSG-RAN WG1 Ad-Hoc Meeting 1901 Taipei, Taiwan, January 21-25, 2019

3GPP TS 22.104 V17.3.0, Service requirements for cyber-physical control applications in vertical domains; Stage 1, SA1, 2020-07

TR 38.825 V16.0.0, Study on NR Industrial Internet of Things (IoT); Technical Report, RAN1, 2019-03

TS 36.214

TS 38.215

TS 22.104

R1-2005517, Ericsson, Propagation Delay Compensation Enhancements for Time Synchronization, 3GPP TSG-RAN WG1#102e, August 17th – 28th 2020.

R2-2006906, Qualcomm, Propagation Delay Compensation for Reference Timing Delivery, 3GPP TSG-RAN WG2#111-e, August 17th – 28th 2020.

R2-2008880, Ericsson, Propagation Delay Compensation Enhancements, 3GPP TSG-RAN WG2 #112-e, November 2nd – 13th, 2020.

R4-2016406, Ericsson, “On UE Rx-Tx measurement accuracy,” 3GPP TSG-RAN WG4 Meeting #97-e, Electronic Meeting, 2nd – 13 November, 2020.

R4-2016154, Ericsson, “gNB Positioning UL SRS Link Level Simulation Results, ” 3GPP TSG-RAN WG4 Meeting #97-e, Electronic Meeting, 2nd – 13 November, 2020.

R4-1711514, Qualcomm, Initial timing error and timing advance adjustment accuracy in NR.

R4-1713648, Ericsson, “Even Further Analysis of UE Initial Transmit Timing Requirement,” 3GPP TSG RAN WG4 Meeting #85, Reno, USA, 27th November - 1st December, 2017.