3GPP TSG RAN WG1 #106-e		R1-2106690

e-Meeting, August 16th – 27th, 2021

Agenda Item:  8.1.3

Source:	Spreadtrum Communications

Title:	Considerations on SRS enhancement

Document for:	Discussion and decision

Introduction

Diverse aspects of requirements can be figured out during NR implementation. This contribution mainly discuss the issues concerning SRS enhancement in Rel-17.

Flexible triggering and DCI overhead reduction

Agreement A list of t values is configured in RRC for each SRS resource set. Adopt at least one of the following for DCI indication of t. - In DCI format 0_1/0_2 without data and without CSI request, - Alt 1-1: Reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 1-2: Re-purpose unused DCI field to indicate t - Alt 1-3: t is indicated by a configurable DCI field, where the DCI field may contain bits from unused fields and additional bits configured by gNB - FFS design details with other potential field(s) - FFS: whether t can be slot offset - In DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 2-1: t is indicated by adding a new configurable DCI field - Alt 2-2: t is indicated without adding DCI payload - Note: The size of DCI payload does not change dynamically - Note: RAN1 should strive for unified solution for different DCI formats. - FFS: The number of RRC configured t values per SRS resource set and DCI bit field size. Agreement Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples - CAT A: Time-domain parameters - A-1: Indication of available slot position, i.e., the t values - A-2: Indication of slot offset - A-3: Indication of SRS symbol-level offset - A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting - CAT B: Frequency-domain parameters - B-1: Indication of a group of CCs for SRS transmission - B-2: Indication of frequency domain resource in a BWP for SRS transmission - B-3: Indication of whether DL/UL BWP is applied for SRS transmission - CAT C: Power control parameters - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’ - FFS impact on power control, impact from triggering a group of CCs for SRS - C-2: Indication of open loop power control parameter e.g., p0. - CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming - CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states - Other examples are not precluded

A unified solution is preferred for indication of t either for the case without data and without CSI request or with a scheduled PDSCH or PUSCH. Therefore, a new configurable DCI field is needed for indication t.

In Rel-15, usage is configured per SRS resource set by RRC signaling, and the association with trigger state per AP SRS resource set is also configured by RRC signaling. DCI is utilized to trigger AP SRS resource set by SRS request field where each codepoint of the field other than ‘0’ corresponds to one trigger state. In light of the current specification, irrespective of which usage, the AP SRS resource set(s) associated to the trigger state in the DCI will triggered and should be transmitted by UE. However, it is not necessary to trigger all the usages simultaneously. For example, in some occasions, gNB only wants to achieve the channel information for DL by utilizing channel reciprocity, then only AP SRS resource sets configured with usage as antennaSwitching should be triggered. The overhead could be reduced, and power saving for UE could be gained. Based on this, we prefer to utilize unused fields for DCI format 0_1/0_2 without data and without CSI request to select/trigger SRS resource sets with some target usage(s) where the target usage(s) could be indicated by unused field(s).

Proposal 1: For DCI format 0_1/0_2 without data and without CSI request, we can reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH.

Proposal 2: For DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH, a new configurable DCI field is needed for indication t.

Proposal 3: For DCI format 0_1/0_2 without data and without CSI request, support to utilize unused field(s) to indicate the target usage(s), and only AP SRS resource set(s) associated with the trigger state corresponding to the codepoint of SRS request in the DCI and configured with usage as antennaSwitching could be triggered.

Increasing Rx number for antenna switching

Agreement On aperiodic SRS configuration for antenna switching with > 4Rx, support the following N_max values - 1T6R: N_max = 3 - 1T8R: N_max = 4 - 2T6R: N_max = 3 - 2T8R: N_max = 4 - [4T8R: N_max = 2] - The support of N_max value does not imply the support of N value that is smaller than N_max. This is FFS. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed Agreement On supported values of N for Rel-17 aperiodic SRS antenna switching with >4Rx, down-select at least one of the following alternatives in RAN1#105e - Alt 1: All the non-zero integer values <= N_max are supported for N - Alt 2: Support N=N_max only - Alt 3: Support specific N values <= N_max - FFS whether different alternatives may be selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot - FFS: whether different alternatives may be selected for different xTyR configuration

From our view, there is no need to introduce both two parameters N_max and N. As done in Rel-15/16, only parameter N is supported. Therefore, we prefer supporting N=N_max only. Besides, for 4T8R, N=2 seems to be a natural choice and can be confirmed.

Besides, during the discussion of Rel-16 TEI session, a UE supporting 1T4R could also support 1T1R, 1T2R and 1T4R. Similar to the TEI discussion, a UE support 2T8R may also support 1T4R, 1T2R, 1T1R, etc. Therefore, new capabilities including multiple combinations of xiTyjR should be considered

Proposal 4: For 4T8R, support N_max = 2.

Proposal 5: Support N_max=N regardless of xTyR;

Proposal 6: Combinations of newly introduced xiTyjR should be considered as new UE capability.

Enhance SRS capacity and/or coverage

Agreement For increased repetition in Rel-17, support the following N_symbol (number of OFDM symbols in one SRS resource) and R (repetition factor) values - N_symbol = 8, R = {1, 2, 4, 8} - N_symbol = 12, R = {1, 2, [3], 4, 6, 12} - FFS the following configurations - N_symbol = 10, R = {1, 2, 5, 10} - N_symbol = 14, R = {1, 2, 7, 14} - FFS options to reduce SRS BW for R>1

More symbols in one SRS resource can bring performance enhancement on coverage. From the perspective of scheduling flexibility, multiple lengths of SRS symbols is beneficial. Therefore, we support N_symbol = 10, R = {1, 2, 5, 10} and N_symbol = 14, R = {1, 2, 7, 14}.

Proposal 7: Support N_symbol = 10, R = {1, 2, 5, 10} and N_symbol = 14, R = {1, 2, 7, 14}.

Agreement For RB-level partial frequency sounding (RPFS) in Rel-17 - The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1} - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern - Support to determine PF and Noffset at least via RRC configuration per SRS resource. - FFS whether to introduce DCI and/or MAC CE in addition Agreement - For RPFS in Rel-17, support PF = {2, 4}. - FFS 3, 8, 12, 16 or fractional numbers - Support at least one of the following alternatives (to be decided in RAN1#105-e) - Alt 1: 1PFmSRS, BSRS is an integer value - Alt 2: 1PFmSRS, BSRS is an integer value with minimum value 4 - Alt 3: is a multiple of 4 - Alt 4: Round 1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2 Agreement Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points - Alt 1: The maximum number of CSs for Comb-8 is 6 - Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

For partial sounding, we think PF = {2, 4} is sufficient and adopting other PF values will probably introduce more specification efforts. Besides, network should guarantee 1PFmsrs,Bsrs is an integer value. If only PF = {2, 4} is supported and considering msrs,Bsrs is multiple of 4 , the integer constraint can be naturally satisfied. The benefit of enabling different Noffset for different SRS occasions is unclear, and we prefer to link only one Noffset to one SRS resource. RRC configuration is sufficient for PF and Noffset.

For positioning SRS, 6 CSs has already been supported. No need to further enhance the maximum number of CSs for Comb-8.

Proposal 8: No need to introduce DCI and/or MAC CE in addition for determining PF and Noffset.

Proposal 9: No need to support start RB location (Noffset) hopping in different SRS occasions.

Proposal 10: 1PFmsrs,Bsrs is a multiple of 4.

Proposal 11: The maximum number of CSs for Comb-8 is 6.

Conclusions

In this contribution, we have the following observation and proposals:

Proposal 1: For DCI format 0_1/0_2 without data and without CSI request, we can reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH.

Proposal 2: For DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH, a new configurable DCI field is needed for indication t.

Proposal 3: For DCI format 0_1/0_2 without data and without CSI request, support to utilize unused field(s) to indicate the target usage(s), and only AP SRS resource set(s) associated with the trigger state corresponding to the codepoint of SRS request in the DCI and configured with usage as antennaSwitching could be triggered.

Proposal 4: For 4T8R, support N_max = 2.

Proposal 5: Support N_max=N regardless of xTyR;

Proposal 6: Combinations of newly introduced xiTyjR should be considered as new UE capability.

Proposal 7: Support N_symbol = 10, R = {1, 2, 5, 10} and N_symbol = 14, R = {1, 2, 7, 14}.

Proposal 8: No need to introduce DCI and/or MAC CE in addition for determining PF and Noffset.

Proposal 9: No need to support start RB location (Noffset) hopping in different SRS occasions.

Proposal 10: 1PFmsrs,Bsrs is a multiple of 4.

Proposal 11: The maximum number of CSs for Comb-8 is 6.

References

Draft Report of 3GPP TSG RAN WG1 #105e